3GPP TSG RAN WG1 #112bis-e		R1-2302705

e-Meeting, April 17th – 26th, 2023

Source:	CATT, GOHIGH

Title:	Discussion on physical channel design framework for sidelink on unlicensed spectrum

Agenda Item:	9.4.1.2

Document for:	Discussion and Decision

Introduction

In RAN#94-e meeting, a new WID on NR sidelink evolution was approved, and further updated in RAN#99-e meeting, which includes one objective to study and specify sidelink operation on unlicensed spectrum [1].

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - Focus on FR1 unlicensed bands (n46 and n96/n102). - Note: In sidelink unlicensed operation, the gNB does not perform Type 1 channel access to initiate and share a channel occupancy, neither Type 2 channel access to share an initiated channel occupancy, nor semi-static channel access procedures to access an unlicensed channel.

In this contribution, we will further discuss the issues on physical channel framework for sidelink operation on unlicensed spectrum.

Physical channel framework of SL-U

Bandwidth part and resource pool

In RAN1#110bis-e meeting, the issue on how to use guard band PRBs was discussed and one agreement was achieved with some FFS parts [2]. And in RAN1#112 meeting, whether additional candidate S-SSB occasions are excluded from resource pool was discussed with an agreement where two opposite options were given [3].

Agreement in RAN1#110bis-e Agreement Regarding usage of PRBs within intra-cell guard band of two adjacent RB sets: - Such PRBs can be used for PSSCH transmission if and only if a UE can transmit on the respective LBT channels after performing channel access procedure in multi-channel case and the UE uses both of these two RB sets for PSSCH transmission - FFS details, e.g., handling of potential unequal sub-channel size, for interlaced RB based transmission, whether the PRB(s) in the intra-cell guard band have the same interlace index(s) as the PRBs for PSSCH transmission in these two RB sets - Such PRBs are not used for PSCCH transmission - FFS: whether or not such PRBs are used for PSFCH/S-SSB transmission Agreement in RAN1#112 Agreement Down-select one or support both of the followings: - Option 1: Additional candidate S-SSB occasions are excluded from resource pool - Option 2: Additional candidate S-SSB occasions belong to resource pool - Note: Companies are encouraged to consider aspects including: S-SSB resource overhead, Tx/Rx UE behavior (e.g., whether any blind detection in Option 2), applicable scenarios, etc.

Regarding whether the PRBs within intra-cell guard band can be used for PSFCH or S-SSB transmission, this issue is correlated with the details design of PSFCH or S-SSB.

- For S-SSB transmission, the FFS point is mainly due to the fact that the guard band PRBs need to be used if interlaced RB based transmission is supported. However, since the guard band is used for mitigating interference from adjacent RB set transmission, the reliability of S-SSB transmissions in such PRBs cannot be guaranteed. Therefore, it is preferred that S-SSB transmission on guard band PRBs is not supported.

- For PSFCH transmission, besides the reliability issue in guard band PRBs, the unified design for PSFCH resource determination for both multiple RB sets or single RB set should be supported. So it is preferred that the intra-cell guard band PRBs are not used for PSFCH.

Proposal 1: The PRBs within intra-cell guard band are not used for PSFCH/S-SSB transmission.

Considering that the PRB number within one RB set may not be an integral multiple of IRB number for IRB-based PSCCH/PSSCH transmissions, which will further cause the PRB number difference for different sub-channels. For example, one sub-channel includes 11 PRBs and another one only includes 10 PRBs caused by guard band. The further raised question is how to determine a harmonized TBS if different PRBs are applied for the initial transmission and re-transmissions. The simplest way is to pre-define a PRB number of one sub-channel used for TBS determination, such as the minimum PRB number of one sub-channel, and then UE can determine the total PRBs allocated for PSSCH that can be used in the procedure of TBS determination.

Proposal 2: Considering the difference of PRBs contained in different sub-channels for IRB-based structure, a pre-defined PRB number of one sub-channel is needed for determining a harmonized TBS for initial transmission and re-transmissions.

Regarding the issue on whether the additional candidate S-SSB occasions are excluded from resource pool. From our understanding, the additional candidate S-SSB occasions are also used for synchronization searching for SL-U UEs, if these S-SSB occasions are included into resource pool, there is a potential scenario that a hidden-node UE may transmit PSCCH/PSSCH in that occasion which is collided with S-SSB transmission. In this scenario, when a Rx UE performs S-SSB reception, the S-SSB reception performance will be degraded due to the interference by the hidden-node UE’s PSCCH/PSSCH transmission. Since the synchronization is the most important thing for sidelink communication, hence we prefer that the additional S-SSB occasions should be excluded from resource pool to ensure the performance of synchronization searching.

Proposal 3: The additional S-SSB occasions should be excluded from resource pool, i.e., only option 1 should be supported.

Slot structure of SL-U

TBS determination

In RAN1#112 meeting, TBS determination issue caused by 2 candidate starting symbols was discussed and the following working assumption was achieved [3].

Working assumption If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - At least for COT initiation, TBS is determined based on a reference number of symbols as follows: - Option 4: The reference number of symbols is determined by (pre-)configuration - FFS: value range - FFS: whether a different reference number of symbols is needed for transmission in a shared COT

For the general scenario of PSCCH/PSSCH transmission, considering that the duration of type 1 LBT is uncertain, UE cannot identify predictably which staring symbol will be finally used for the actual PSCCH/PSSCH transmission, so the reference number of symbols for TBS determination should be identified firstly, where the (pre-)configuration method is the most reasonable way to balance both spectrum efficiency and coding rate.  But the starting symbol can be identified in advance at least for the following two cases:

- PSSCH transmissions within slots with PSFCH resources

If all the slots related to PSCCH/PSSCH transmissions of one TB are configured with PSFCH resources, which means only the 1st starting symbol can be used, the reference symbol length can be identified only based on the 1st starting symbol, which means even the reference number of symbols for TBS determination is (pre-)configured, it will be neglected for such case.

- PSSCH transmissions within COT

If it can be guaranteed that all the PSCCH/PSSCH transmissions of one TB will start from the 1st starting symbol, such as all the PSSCH transmissions are within the shared remaining COT and CPE is applied to avoid type 2 LBT failure, then the reference symbol length can be identified only based on the 1st starting symbol, which means the (pre-)configured parameter of reference number of symbols will also be neglected.

Proposal 4: At least for the following two cases, the reference number of symbols for TBS determination is identified based on the 1st staring symbol:

- All the slots related to PSCCH/PSSCH transmissions of one TB are configured with PSFCH resources

- PSCCH/PSSCH transmissions of one TB are within a COT.

AGC training

In RAN1#111 meeting, the issues related to AGC training were discussed and the following agreement was achieved [4].

Agreement For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - Regarding Tx UE behaviour: - If PSCCH/PSSCH transmission starts from 1st starting symbol, down-select one of the followings - Option 1: The PSCCH/PSSCH transmission has 2 symbols for AGC purpose - Option 2: The PSCCH/PSSCH transmission has only 1 symbol for AGC purpose - Option 3: The PSCCH/PSSCH transmission has 1 or 2 symbol(s) for AGC purpose depending on conditions, FFS details - If PSCCH/PSSCH transmission starts from 2nd starting symbol, the PSCCH/PSSCH transmission has only 1 symbol for AGC purpose - Regarding Rx UE behaviour, down-select one of the followings: - Option A: The Rx UE always monitors two AGC symbols in such slot - Option B: The Rx UE monitors two AGC symbols in such slot by default, but could drop monitoring the 2nd AGC symbol at least if it detects a PSCCH/PSSCH transmission starting from the 1st starting symbol - FFS details - Option C: The Rx UE monitors two AGC symbols in such slot by default, but it is up to UE implementation whether to drop monitoring the 2nd AGC symbol - Option D: It is up to UE implementation to monitor 1 or 2 AGC symbol(s) in such slot

Regarding AGC issue, the hidden-node issue should be considered before AGC design work. As shown in Figure 1, the distance between UE-A and UE-B is large enough and they cannot sense each other, where UE-A’s transmission will not interrupt UE-B’s transmission and UE-B may start PSCCH/PSSCH transmission from the 2nd starting symbol based on LBT result, so it is a valid case that Rx UE needs to perform PSCCH/PSSCH receptions starting from different starting symbols in one slot. In such case, the additional AGC adjustment at the second starting symbol is necessary, and Rx UE should always perform AGC adjustment twice in such slot.

Figure 1: PSCCH/PSSCH transmissions start from different starting symbols in hidden-node case

Observation 1: Considering hidden-node issue, the additional AGC adjustment at the 2nd starting symbol is needed.

Proposal 5: If PSCCH/PSSCH transmission starts from the 1st starting symbol, 2 AGC symbols for AGC purpose are needed. And Rx UE always monitors two AGC symbols in such slot.

PSCCH/PSSCH transmission of SL-U

PSCCH/PSSCH transmission for multiple RB sets

In RAN1#110bis-e meeting, the related issues about PSCCH/PSSCH transmissions across multiple RB sets were discussed and the following agreements were achieved [2].

Agreement Regarding frequency domain resource indication for interlace RB-based PSSCH transmission: - When more than one RB set is used for transmissions, down-select one of the followings - Option A: Support that the used interlace index(s) in different RB sets are always the same - Option B: Support that the used interlace index(s) in different RB sets can be different - FFS details Agreement For PSCCH and PSSCH in SL-U: - PSCCH is transmitted within 1 sub-channel - At least support Option 1 below - Option 1: PSCCH locates in the lowest sub-channel of lowest RB set of corresponding PSSCH - Note: the lowest sub-channel may not be entirely contained in the lowest RB set - FFS whether/how to handle the case where UEs supporting different bandwidths can use the same resource pool to communicate with each other, e.g., whether/how to additionally support Option 2 below - Option 2: PSCCH locates in every RB set of corresponding PSSCH - Note: the above options do not imply any restriction on the mapping of sub-channels to PRBs. - FFS other details

When more than one RB set is used for PSSCH transmission, regarding the issue on whether or not the used interlaced RB index(s) in different RB sets are always the same, considering the following aspects, it is preferred to use the same interlaced RB index(s) in different RB sets.

- One aspect for using different interlaced RB index is that the PAPR increment due to uneven PRB interval when different interlaced RB index is used.  The CM results for these two options are evaluated, as shown in Figure 2. It can be observed that these two options have similar CM performance, therefore the PAPR impacts will not be an essential point for this issue.

- Another aspect for using different interlaced RB index is that the PSCCH/PSSCH power reduction due to PSD limit restriction, we think this is a valid point. As shown in Figure 3, if the different RB sets can use different interlaced RB index, there is a potential 3 dB power reduction for PSSCH transmission due to PSD limit restriction.

- The definition of sub-channel has already been identified in RAN1#112 meeting where one sub-channel is defined within one RB set and is periodically indexed. So from the perspective of reducing signaling overhead of resource indication (the details are discussed in section 2.3.3), option A is preferred.

Figure 2: CM results for same IRB index or different IRB index

Figure 3: Power reduction due to different IRB index in different RB sets

Observation 2: When more than one RB set is used for PSSCH transmission, whether or not the used interlaced RB index(s) in different RB sets are always the same has small difference for CM results.

Observation 3: When more than one RB set is used for PSSCH transmission, if different interlaced RB index is used for different RB sets, there is a potential 3dB power reduction for PSSCH transmission due to PSD limit restriction and will cause bigger signaling overhead of resource indication.

Proposal 6: When more than one RB set is used for PSSCH transmission, it is preferred to use same interlaced RB index(s) in different RB sets, i.e., only option A is supported.

Additionally, considering the PSD limit restriction, if more than one RB set is used for PSSCH transmission, the number of used sub-channels in each RB set should be the same, otherwise there is a potential power reduction due to PSD limit restriction. Besides, only the contiguous RB sets can be used for PUSCH in NR-U, the same principle should be applied in R18 SL-U, where one PSCCH/PSSCH can only transmit within one RB set or spanning over multiple contiguous RB sets. And in order to reduce the signaling overhead, the RB set number for different PSSCH transmissions of one TB should be the same and RIV-like indication method can be used to indicate RB set related information. The details are discussed in section 2.3.3.

Proposal 7: When more than one RB set is used for PSSCH transmission, the number of used sub-channel(s) per RB set should be the same.

Proposal 8: When more than one RB set is used for PSSCH transmission, the RB sets should be contiguous for one PSSCH transmission and the RB set number for different PSSCH transmissions of one TB should be the same.

There is a FFS on the case that whether/how to handle the case where UEs support different bandwidths can use the same resource pool to communicate with each other. Based on current agreement, only one sidelink BWP is supported for SL-U operation, this case requires UEs supporting multiple BWPs, therefore, it is not a valid case in R18 SL-U.

Proposal 9: It is not a valid case in R18 SL-U where UEs supporting different bandwidths can use the same resource pool to communicate with each other

There is an RB set(s) selection issue when a resource pool is configured with multiple RB sets. For example, when a UE performs resource selection, it can either select more sub-channels within one RB set for one TB transmission, or select more than one RB set with smaller number of sub-channels in each RB set for one TB transmission. This issue should be further studied.

Proposal 10: When a resource pool is configured with multiple RB sets, the resource selection for PSCCH/PSSCH transmission on multiple RB sets or single RB set should be further studied.

The definition of candidate single-slot resource

In RAN1#112 meeting, the related issues about sub-channel definition were fully discussed and the following agreements were achieved respectively for contiguous RB-based structure and IRB-based structure [3].

Agreement For contiguous RB-based PSCCH/PSSCH transmission in SL-U: - Regarding mapping between sub-channel and PRBs, down-select one of the followings during RAN1#112: - Option 1 (sub-channel aligns with resource pool boundary): Same as in legacy NR SL, i.e., the mapping of sub-channel starts from the first PRB of the resource pool and mapped sequentially within the resource pool according to the sub-channel size - FFS: how to deal with the remaining PRBs, e.g. for meeting OCB requirements Agreement For interlace RB-based PSCCH/PSSCH transmission in SL-U: - Regarding mapping between sub-channel and interlace, 1 sub-channel is defined and indexed within 1 RB set, and is periodically indexed across different RB sets within the resource pool

- Contiguous RB-based structure

In R16/R17 NR SL, one candidate single-slot resource is defined as LsubCH contiguous sub-channels within one slot. One issue is whether such definition can be inherited for R18 SL-U, where the guard band between adjacent RB sets is existed and such guard band PRB can only be used if corresponding PSCCH/PSSCH transmission spans over two adjacent RB sets. As shown in Figure 4, based on current agreements, single sub-channel#4 or single sub-channel#5 cannot be used for PSCCH/PSSCH transmissions, but the combination of sub-channel#4 and sub-channel#5 including all the guard band PRBs can be used for PSCCH/PSSCH transmissions. So the definition of Rx,y needs to be slightly updated, where only LsubCH  contiguous sub-channels without any guard band PRBs (such as sub-channel#1+sub-channel#2) or LsubCH contiguous sub-channels with all guard band PRBs (such as sub-channel#4+sub-channel#5) within one slot can be treated as candidate single-slot resource. Inversely, LsubCH contiguous sub-channels with only part guard band PRBs cannot be used as candidate single-slot resource (such as sub-channel#3+sub-channel#4 or sub-channel#5+sub-channel#6).

Observation 4: Based on current agreements on the restrictions of the usage of guard band PRBs, current definition on candidate single-slot resource of R16/R17 NR SL cannot be used directly for R18 SL-U.

Proposal 11: For contiguous RB-based structure for PSCCH/PSSCH transmission, candidate single-slot resource (Rx,y) is defined within one slot as follows:

- LsubCH contiguous sub-channels within a RB set without any guard band PRBs

- LsubCH contiguous sub-channels within multiple contiguous RB sets with all guard band PRBs

Figure 4: Sub-channel index for contiguous RB-based structure

- IRB-based structure

For IRB-based structure, considering that sub-channel is defined within one RB set and is periodically indexed across different RB sets, so the definition of candidate single-slot resource of R16/R17 NR SL is not fully applicable for R18 SL-U, such as PSCCH/PSSCH transmission can occupy two sub-channels with the same sub-channel index, which are located within two adjacent RB sets. Besides, as discussed in section 2.3.1, the used interlace index in different RB sets should always be the same, so the definition of candidate single-slot for IRB-based structure should be updated as follows:

- Spanning over N contiguous RB sets, and M contiguous sub-channels per RB set, where M*N= LsubCH and the used sub-channel index within different RB sets should be the same (as the candidate single-slot resource in the first slot of Figure 5)

  - Especially, when N equals to 1, candidate single-slot resource is defined as LsubCH contiguous sub-channels within one RB set (as the candidate single-slot resource in the second slot of Figure 5)

Proposal 12: For IRB-based structure for PSCCH/PSSCH transmission, candidate single-slot resource (Rx,y) is defined within one slot as follows:

- Spanning over N contiguous RB sets, and M contiguous sub-channels per RB set, where M*N= LsubCH and the used sub-channel index within different RB sets should be the same

  - Especially, when N equals to 1, candidate single-slot resource is defined as LsubCH contiguous sub-channels within one RB set

Figure 5: Sub-channel index for IRB-based structure

Frequency resource indication for PSCCH/PSSCH transmission

Regarding the frequency resource indication for PSCCH/PSSCH transmission, one essential issue is the sub-channel indexing especially when multiple RB sets are included in a resource pool. Based on the discussion in section 2.3.1, when a PSSCH transmission spans over multiple RB sets, it is preferred to use the same sub-channel number and the same IRB index(s) in each RB set. In order to reuse the FRIV indication in Rel-16 NR sidelink and provide an efficient resource indication method, the sub-channels are indexed independently per RB set and the sub-channel index for a given interlaced RB index(s) is same in different RB sets as shown in Figure 6. That means a sub-channel is determined by a parameter pair {RB_Set_index, Sub-channel_index}.

Figure 6: Sub-channel is defined within one RB set and is periodically indexed per RB set

- The indication of sub-channel

Based on the sub-channel definition and sub-channel indexing above, there will be an implicit correspondence between the used sub-channel(s) within different RB sets, where both the sub-channel index and IRB index within different RB sets are same. Thus, we don’t need to indicate all the sub-channels explicitly, only these sub-channels within the reference RB set need to be indicated, then the sub-channels within other RB set(s) can be identified implicitly. The FRIV indication method can be reused for the sub-channel indication of the reference RB set. The reference RB set can be the lowest RB set per PSCCH/PSSCH transmission. The FRIV determination is shown as follows:

If sl-MaxNumPerReserve is 2 then FRIV=nsubCH,1start+i=1LsubCH-1N subchannel RBset+1-i If sl-MaxNumPerReserve is 3 then FRIV=nsubCH,1start+nsubCH,2start⋅N subchannelRBset+1-LsubCH+i=1LsubCH-1N subchannel RBset+1-i2 where - nsubCH,1start denotes the starting sub-channel index for the second resource within the corresponding reference RB set - nsubCH,2start denotes the starting sub-channel index for the third resource within the corresponding reference RB set - LsubCH denotes the sub-channel number per PSCCH/PSSCH transmission within the corresponding reference RB set - N subchannel RBset is the number of sub-channels of one RB set

As shown in Figure 7, the initial transmission can indicate further two re-transmissions. For the 1st re-transmission, the reference RB set is RB set#0, and the used sub-channel index of 1st re-transmission is sub-channel#2 and sub-channel#3; For the 2nd re-transmission, the reference RB set is RB set#0, and the sub-channel within RB set#0 is sub-channel#3 and sub-channel#4. Then,

FRIV=2+3×5+1-2+(5+1-1)2=39

Figure 7: Resource indication for PSCCH/PSSCH transmission across multiple RB sets

- The indication of RB set

In NR-U, only the contiguous RB sets are used for PUSCH indicated by DCI. The similar principle should be applied for PSCCH/PSSCH transmissions. Considering that PSCCH can indicate at most two or three transmissions, so the RB set number for different transmissions of one TB should be same and the similar resource indication method as R16/R17 SL FRIV can be treated as the starting point for RB set indication design, which is called RRIV (RB set resource indication value). The details are as follows:

If sl-MaxNumPerReserve is 2 then RRIV=nRBset,1start+i=1LRBset-1N RBset +1-i If sl-MaxNumPerReserve is 3 then RRIV=nRBset,1start+nRBset,2start⋅N RBset +1-LRBset+i=1LRBset-1N RBset +1-i2 where - nRBset,1start denotes the starting RB set index for the second resource - nRBset,2start denotes the starting RB set index for the third resource - N RBset is the number of RB set(s) in a resource pool - LRBset is the number of RB set(s) used for one PSCCH/PSSCH transmission

As shown in Figure 8, the first PSCCH can indicate at most two re-transmissions, where the 1st re-transmission spans over RB set#0 and RB set#1, the 2nd re-transmission spans over RB set#1 and RB set#2. Then, the RRIV is determined as follows:

RRIV=0+1×3+1-2+(3+1-1)2=11

Figure 8: PSCCH/PSSCH transmission across multiple RB sets

Proposal 13: In order to indicate the frequency domain resource efficiently, it is preferred that the sub-channel index for a given interlaced RB index(s) is same in different RB sets.

Proposal 14: The frequency resource indication of PSCCH/PSSCH transmissions across over multiple RB sets shall be indicated by explicit sub-channel index(s) and RB set index(s).

- For sub-channel indication, R16 NR SL FRIV is reused to indicate the sub-channel(s) within the reference RB set

  - The reference RB set is the lowest RB set per PSCCH/PSSCH transmission

- For RB set indication, RRIV is used to indicate the RB set(s) as follows

If sl-MaxNumPerReserve is 2 then RRIV=nRBset,1start+i=1LRBset-1N RBset +1-i If sl-MaxNumPerReserve is 3 then RRIV=nRBset,1start+nRBset,2start⋅N RBset +1-LRBset+i=1LRBset-1N RBset +1-i2 where - nRBset,1start denotes the starting RB set index for the second resource - nRBset,2start denotes the starting RB set index for the third resource - N RBset is the number of RB set(s) in a resource pool - LRBset is the number of RB set(s) used for one PSCCH/PSSCH transmission

S-SSB transmission of SL-U

S-SSB structure

In RAN1#112 meeting, S-SSB structure related issues were discussed and the following agreement was achieved [3].

Agreement For S-SSB transmission within 1 RB set, down-select to one or more of the following for 15 kHz and 30 kHz SCS: - Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH - FFS: whether/how to handle the case when each interlace has only 10 PRBs in a RB set, e.g. whether 1 or 2 interlaces will be used for S-SSB - Option 3-1: Transmit S-PSS/S-SSS/PSBCH N times by repetition in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement - FFS details, e.g., the length of gap between repetitions is (pre-)configured or pre-defined, value of N (e.g., N=2), how to reduce PAPR, etc. - FFS gap of 0 - Option A: Apply OCB exemption to all of S-PSS/S-SSS/PSBCH - Continue studying how to meet the minimum 2 MHz requirements under 15 kHz SCS.

Since option 3-1 can reuse most of the design aspects in R16/R17 NR sidelink and can provide repetition gain for S-SSB reception, option 3-1 is preferred for S-SSB transmission to meet the OCB requirement. Further considering the LBT failure impacts, it is preferred to confine the S-SSB transmission within one RB set.

Proposal 15: For S-SSB channel structure under 15kHz and 30kHz, if OCB and PSD requirement are needed, the frequency repetition structure (Option 3-1) is preferred and the repetition should be within one RB set.

The (pre-)configuration of S-SSB occasions

In RAN1#111 meeting, the related issues about the location of addition candidate S-SSB occasions were discussed and the following agreement was achieved [4].

Agreement Regarding the number and location(s) of additional candidate S-SSB occasions, RAN1 further study the followings: - Option 1: Reuse legacy NR SL design, and increase the available values in sl-NumSSB-WithinPeriod for each SCS - Option 2: Each R16/R17 NR SL S-SSB slot has K corresponding additional candidate S-SSB occasion, and the gap between them is (pre-)configured - FFS details, e.g., value of K, details on gap length, etc. - Option 3: The number and location(s) of additional candidate S-SSB occasions are separately (pre-)configured - Option 4: Introduce M contiguous candidate S-SSB occasions in one S-SSB period - Option 5: the number of candidate S-SSB occasions is (pre-)configured, and locations are determined based on the (pre-)configured number

In Rel-16 NR sidelink, S-SSB occasions are uniformly distributed in time domain within a synchronization period, where the same duration between two adjacent S-SSB occasions is configured by higher layer parameter sl-TimeInterval-r16, the first S-SSB occasion and the number of S-SSB occasions are configured by sl-TimeOffsetSSB-r16 and sl-NumSSB-WithinPeriod-r16 respectively. For the enhanced designs in time domain for SL-U S-SSB transmissions, the analysis is given as follows:

The advantage of option 1 is that it can work with less specification changes. But the impact on the legacy S-SSB designs should be avoided when introducing additional S-SSB occasions so that the legacy mechanism and legacy parameters can be reused directly. Based on TS 38.331, sl-NumSSB-WithinPeriod is used to indicate the number of sidelink SSB transmission within one sidelink SSB period, which can guarantee the synchronization performance under certain SCS configuration, so this parameter should not be changed. The additional S-SSB occasions should be configured additionally, i.e., there should be explicit distinguish between the legacy S-SSB occasions and additional S-SSB occasions, so option 1 should be deprioritized.

sl-NumSSB-WithinPeriod Indicates the number of sidelink SSB transmissions within one sidelink SSB period. The applicable values are related to the subcarrier spacing and frequency as follows: FR1, SCS = 15 kHz: 1 FR1, SCS = 30 kHz: 1, 2 FR1, SCS = 60 kHz: 1, 2, 4 FR2, SCS = 60 kHz: 1, 2, 4, 8, 16, 32 FR2, SCS = 120 kHz: 1, 2, 4, 8, 16, 32, 64

The advantage of option 4 is that UE can perform continuous S-SSB transmission after successful channel access, but the following issues will be raised:

- Considering the characteristics of Wi-Fi system where UE will continuously occupy the channel after successful channel access, SL-U UEs still cannot use the channel if the transmission window of Wi-Fi UE and the S-SSB transmission window are fully overlapped.

- In AI 9.4.1.1, type 2A channel access for S-SSB transmission is discussed, the duty cycle requirement should also be considered for S-SSB occasion configuration, so the continuous S-SSB occasions are not preferred which may violate duty cycle restriction.

- As discussed in section 2.1, all the S-SSB slots should be excluded from the resource pool, so option 4 will cause larger latency for PSCCH/PSSCH transmission.

For the other options, because S-SSB occasions are non-continuous, UE can has more opportunities to perform channel access, duty cycle restriction and PSCCH/PSSCH transmission latency will be less effected, so option 4 should also be deprioritized.

Option 5 is unclear on how to determine the starting occasion of the first additional S-SSB occasions, there is no relationship between the legacy S-SSB occasions and additional S-SSB occasions. Thus, option 5 is not feasible.

The left option 2 and option 3 has similar functionality. For option 2, two additional parameters are needed:

- the number of additional S-SSB occasions corresponding to one legacy S-SSB occasion

- the gap between legacy S-SSB occasion and additional S-SSB occasion

For option 3, three additional parameters are needed:

- The number of additional S-SSB occasions

- The starting point of the first S-SSB occasion of additional S-SSB occasions

- The offset between the adjacent additional S-SSB occasions

From the perspective of reducing specification change, option 2 as shown in Figure 9 is the preference.

Figure 9: Each R16/R17 NR SL S-SSB slot has one corresponding additional candidate S-SSB occasion

Observation 5: If continuous S-SSB transmission window is configured, the following issues will be raised:

- The channel availability gain is limited considering the Wi-Fi system characteristics

- The duty cycle requirement of type 2A channel access may not be met

- The PSCCH/PSSCH transmission latency will be increased

Proposal 16: Regarding how to (pre-)configure additional S-SSB occasions, option 2 should be supported, i.e., each R16/R17 NR SL S-SSB slot has K corresponding additional candidate S-SSB occasion, and the gap between them is (pre-)configured

The behaviors of S-SSB transmissions and receptions

In RAN1#111 meeting, the S-SSB transmission behavior on additional candidate S-SSB occasion(s) was discussed and one agreement was achieved comprised by several candidate options [4]. Besides, it has been achieved consensus in RAN1#112 meeting that RAN1 should further S-SSB behaviors in multiple RB sets case [3].

Agreement in RAN1#111 Agreement Regarding additional candidate S-SSB occasions: - In the same S-SSB period, RAN1 further study the followings: - Alt 1: UE attempts to transmit on all or some of additional candidate S-SSB occasion(s) only when it fails to transmit on R16/R17 S-SSB occasion(s) - Alt 2: UE attempts to transmit on all additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s) - Alt 3: UE can attempt to transmit on all or some of additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s) - Alt 4: upon LBT failure on a (candidate) S-SSB occasion, a UE attempts to transmit on the subsequent additional candidate S-SSB occasion if within a period S-SSB transmission has not been transmitted in any prior occasions - FFS details Agreement in RAN1#112 Agreement RAN1 further study the followings: - Whether/how to maintain a COT when the COT contains multiple RB sets and includes S-SSB slot(s), e.g., whether to transmit S-SSB repetitions in more than one RB set, etc.

Regarding S-SSB transmission behavior on additional S-SSB occasion(s), the following two issues should be further discussed:

- Maximum successful S-SSB transmission number

The configured number of transmission occasions of each R16/R17 S-SSB resource is to ensure the sidelink synchronization performance. Therefore, the total number of successful S-SSB transmissions within a synchronization period in SL-U shall not exceed the configured number of R16/R17 S-SSB occasion(s).

- Relationship between R16/R17 S-SSB transmissions and additional candidate S-SSB transmissions

Based on above discussion, in order to ensure the synchronization performance, the number of successful S-SSB transmission occasions is to meet the configured number of R16/R17 S-SSB occasion(s) regardless of the S-SSB transmission is transmitted on legacy S-SSB occasions or additional S-SSB occasions. Based on this principle, when the total successful S-SSB transmissions are less than the number of R16/R17 S-SSB occasions, UE will attempt to transmit on the subsequent candidate S-SSB occasion, the candidate S-SSB occasion can be either legacy R16/R17 S-SSB occasion or additional candidate S-SSB occasion.

Proposal 17:  When the total number of successful S-SSB transmissions within a S-SSB period is less than the number of R16/R17 S-SSB occasions, UE attempts to transmit on the subsequent candidate S-SSB occasion, the candidate S-SSB occasion can be either legacy R16/R17 S-SSB occasion or additional candidate S-SSB occasion.

One additional remaining FFS issue is how to perform S-SSB transmission or reception within a COT containing multiple RB set and S-SSB slot. If the UE obtaining the COT is the S-SSB transmission UE, then considering the motivation that UE should try to maintain a COT after type1 channel access, so additional transmissions (such as repetitive S-SSB transmission or dummy signal transmission) can be performed within such COT especially when S-SSB occasions are excluded from resource pool, otherwise, no SL transmission is performed within some RB sets included in COT and then such COT may be interrupted by other RATs.  But if the UE obtaining the COT is not the S-SSB transmission UE, then it is not needed to perform additional transmissions in S-SSB slot to avoid half-duplex issue, because UE needs to perform S-SSB receptions in such slot. As shown in Figure 10, the location of normal S-SSB transmissions are fixed and the additional transmissions are performed based on the obtained COT of different UEs, where all the UEs are S-SSB transmission UEs, RX UE only need to decode these normal S-SSB transmissions, i.e., the additional S-SSB transmissions are only used to maintain COT and the reception behaviors are same as R16/R17 NR sidelink.

Proposal 18:  For S-SSB transmission UE, additional transmissions (such as repetitive S-SSB transmissions or dummy signal transmissions) over multiple RB sets should be supported to maintain COT that contains multiple RB sets and includes S-SSB slot(s). And S-SSB receptions only be performed for the normal S-SSB transmission.

Figure 10: S-SSB transmissions within multiple RB sets

PSFCH and SL HARQ

In previous RAN1 meeting, the PSFCH design aspects of SL-U were discussed and achieved following agreements [3] [5].

Agreements in RAN1#110 Agreement To meet OCB and PSD requirement for PSFCH transmission, at least RB-based interlace is supported at least for 15 kHz and 30 kHz SCS, FFS details. Agreements in RAN1#112 Agreement Regarding PSFCH transmission with 15 kHz and 30 kHz SCS, RAN1 down-select one of followings, or support the combination of followings: - Alt 1-1a: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s) - FFS: value of K3 - Alt 2-2a: each PSFCH transmission occupies 1 interlace, and further apply PRB-level cyclic shift - A UE transmits dedicated cyclic shift on K1 dedicated PRB(s) within this interlace, and transmits common cyclic shift on other PRBs of this interlace - FFS: value of K1 - Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace - Alt 2-4a: each PSFCH transmission occupies 1 dedicated interlace and adopt PRB-level cyclic shift hopping as in NR-U - Alt 3-2a: each PSFCH transmission occupies K4 dedicated PRB(s) and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set - FFS: value of K2, K4 - FFS: the impact of PSD limit, e.g., whether/how to handle the case when common PRB and dedicated PRB locate within the same 1 MHz bandwidth, e.g., drop common PRB or reduce power on common PRB in such case - FFS: whether/how to reduce PAPR of PSFCH transmission

PSFCH structure

According to the agreement, in order to meet OCB and PSD requirement, at least interlaced RB transmission for PSFCH is applied for 15kHz and 30kHz SCS. Based on this assumption, alt 3-2a should not be supported for PSFCH transmissions under 15KHz and 30KHz SCS, since such alternative is not interlaced RB based transmission.

For alt 1-1a, if the number of dedicated PRB is zero, i.e., each PSFCH transmission only shares a common interlace, the PSFCH resource capacity will be problematic, and it will also increase the interference for PSFCH transmissions. Besides, if other number of dedicated PRB(s) is configured, the dedicated PRB(s) are used for carry the corresponding SL HARQ information, but the transmission power of the dedicated PRB(s) will be shared by the common interlace, which will largely reduce the coverage and reliability of PSFCH transmission. Therefore, alt 1-1a is not our preference.

For the left options, in order to increase the PSFCH capacity, the code domain enhancements should be studied which need more specification work. Thus, alt 2-2a and alt 2-4a can be further studied.

Proposal 19: For PSFCH transmission under 15 KHz and 30KHz SCS, alt 2-2a and alt 2-4a can be further studied.

Regarding the PSFCH transmission for 60KHz SCS, since there is no interlaced structure in 60KHz, alt 3-2a could be considered as a starting point. From our understanding, in order to meet the OCB requirement, at least 2 common RBs are necessary which are located at the two edges of a RB set. The dedicated RB(s) can be used to carry the SL HARQ information.

Proposal 20: For PSFCH transmission with 60 KHz SCS, alt 3-2a can be further studied.

PSFCH dropping due to LBT failure

In RAN1#112 meeting, the PSFCH transmission behavior was discussed and the following agreement was achieved [3].

Agreement To address PSFCH transmission dropping due to LBT failure: - Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission - Down-select one or support both of the followings - Option 1: Such PSFCH occasion(s) are (pre-)configured - Option 2: Such PSFCH occasion(s) are (pre-)configured and dynamically indicated - FFS applicable scenarios, e.g., considering the applicability of COT sharing, MCSt, etc. - FFS other details

Regarding the issue of PSFCH transmission dropping due to LBT failure, two alternatives are proposed regarding the PSFCH occasions in last meeting. In Rel-16 NR sidelink design, due to the distributed resource allocation mechanism (mode 2), the PSFCH resource associated with its corresponding PSSCH is determined by an implicit manner to avoid PSFCH resource collision. Since the distributed resource allocation mechanism shall also be used for SL-U, hence it is naturally to reuse this principle in SL-U PSFCH transmission design.

Proposal 21: The PSFCH resource associated with its corresponding PSSCH is determined by an implicit manner, i.e., option 1 should be supported.

Regarding option 2 that PSFCH resources are dynamically indicated, such design is mainly targeted for the COT sharing case, and PSFCH resource would be indicated by COT initiating UE. Such design is highly correlated with the details design of COT sharing operation. However, the details design of COT sharing is still unclear currently. Therefore, the alternative that PSFCH resources are dynamically indicated should be deferred until the details design of COT sharing operation is clear.

Proposal 22: Whether/how to support option 2 (such PSFCH occasion(s) are (pre-)configured and dynamically indicated) should be deferred until the details design of COT sharing operation is clear.

Additionally, considering that more than 1 PSFCH occasion per PSCCH/PSSCH transmission has already been endorsed, there is some impacts on HARQ RTT restriction among PSSCH resources. As in Rel-16 NR sidelink, if the PSFCH is presented in a resource pool, the intervals between any two PSSCH resources of the same TB shall meet the HARQ RTT restriction. Since there is more than 1 PSFCH occasion per PSCCH/PSSCH transmission, how to meet the HARQ RTT restrictions shall be further studied. Besides, considering that PSCCH/PSSCH transmission spanning over multiple RB set is supported, RAN1 should further study that how to (pre-)configure and determine PSFCH resource, such as every RB set should be (pre-)configured with PSFCH resources to guarantee that PSSCH transmission and related PSFCH transmission are located within the same RB set

Proposal 23: The following issues should be further studied:

- How to meet the HARQ RTT restriction considering that more than 1 PSFCH occasion per PSCCH/PSSCH transmission has already been endorsed

- How to (pre-)configure PSFCH resources, e.g., whether every RB set should be (pre-)configured with PSFCH resources

- How to determine PSFCH resource in response to a PSSCH reception especially when PSSCH transmission spanning over multiple RB sets is performed

Conclusion

In this contribution, the physical channel structure of SL-U is discussed. Partially, we have following proposals and observations.

Proposal 1: The PRBs within intra-cell guard band are not used for PSFCH/S-SSB transmission.

Proposal 2: Considering the difference of PRBs contained in different sub-channels for IRB-based structure, a pre-defined PRB number of one sub-channel is needed for determining a harmonized TBS for initial transmission and re-transmissions.

Proposal 3: The additional S-SSB occasions should be excluded from resource pool, i.e., only option 1 should be supported.

Proposal 4: At least for the following two cases, the reference number of symbols for TBS determination is identified based on the 1st staring symbol:

- All the slots related to PSCCH/PSSCH transmissions of one TB are configured with PSFCH resources

- PSCCH/PSSCH transmissions of one TB are within a COT.

Observation 1: Considering hidden-node issue, the additional AGC adjustment at the 2nd starting symbol is needed.

Proposal 5: If PSCCH/PSSCH transmission starts from the 1st starting symbol, 2 AGC symbols for AGC purpose are needed. And Rx UE always monitors two AGC symbols in such slot.

Observation 2: When more than one RB set is used for PSSCH transmission, whether or not the used interlaced RB index(s) in different RB sets are always the same has small difference for CM results.

Observation 3: When more than one RB set is used for PSSCH transmission, if different interlaced RB index is used for different RB sets, there is a potential 3dB power reduction for PSSCH transmission due to PSD limit restriction and will cause bigger signaling overhead of resource indication.

Proposal 6: When more than one RB set is used for PSSCH transmission, it is preferred to use same interlaced RB index(s) in different RB sets, i.e., only option A is supported.

Proposal 7: When more than one RB set is used for PSSCH transmission, the number of used sub-channel(s) per RB set should be the same.

Proposal 8: When more than one RB set is used for PSSCH transmission, the RB sets should be contiguous for one PSSCH transmission and the RB set number for different PSSCH transmissions of one TB should be the same.

Proposal 9: It is not a valid case in R18 SL-U where UEs supporting different bandwidths can use the same resource pool to communicate with each other

Proposal 10: When a resource pool is configured with multiple RB sets, the resource selection for PSCCH/PSSCH transmission on multiple RB sets or single RB set should be further studied.

Observation 4: Based on current agreements on the restrictions of the usage of guard band PRBs, current definition on candidate single-slot resource of R16/R17 NR SL cannot be used directly for R18 SL-U.

Proposal 11: For contiguous RB-based structure for PSCCH/PSSCH transmission, candidate single-slot resource (Rx,y) is defined within one slot as follows:

- LsubCH contiguous sub-channels within a RB set without any guard band PRBs

- LsubCH contiguous sub-channels within multiple contiguous RB sets with all guard band PRBs

Proposal 12: For IRB-based structure for PSCCH/PSSCH transmission, candidate single-slot resource (Rx,y) is defined within one slot as follows:

- Spanning over N contiguous RB sets, and M contiguous sub-channels per RB set, where M*N= LsubCH and the used sub-channel index within different RB sets should be the same

  - Especially, when N equals to 1, candidate single-slot resource is defined as LsubCH contiguous sub-channels within one RB set

Proposal 13: In order to indicate the frequency domain resource efficiently, it is preferred that the sub-channel index for a given interlaced RB index(s) is same in different RB sets.

Proposal 14: The frequency resource indication of PSCCH/PSSCH transmissions across over multiple RB sets shall be indicated by explicit sub-channel index(s) and RB set index(s).

- For sub-channel indication, R16 NR SL FRIV is reused to indicate the sub-channel(s) within the reference RB set

  - The reference RB set is the lowest RB set per PSCCH/PSSCH transmission

- For RB set indication, RRIV is used to indicate the RB set(s) as follows

If sl-MaxNumPerReserve is 2 then RRIV=nRBset,1start+i=1LRBset-1N RBset +1-i If sl-MaxNumPerReserve is 3 then RRIV=nRBset,1start+nRBset,2start⋅N RBset +1-LRBset+i=1LRBset-1N RBset +1-i2 where - nRBset,1start denotes the starting RB set index for the second resource - nRBset,2start denotes the starting RB set index for the third resource - N RBset is the number of RB set(s) in a resource pool - LRBset is the number of RB set(s) used for one PSCCH/PSSCH transmission

Proposal 15: For S-SSB channel structure under 15kHz and 30kHz, if OCB and PSD requirement are needed, the frequency repetition structure (Option 3-1) is preferred and the repetition should be within one RB set.

Observation 5: If continuous S-SSB transmission window is configured, the following issues will be raised:

- The channel availability gain is limited considering the Wi-Fi system characteristics

- The duty cycle requirement of type 2A channel access may not be met

- The PSCCH/PSSCH transmission latency will be increased

Proposal 16: Regarding how to (pre-)configure additional S-SSB occasions, option 2 should be supported, i.e., each R16/R17 NR SL S-SSB slot has K corresponding additional candidate S-SSB occasion, and the gap between them is (pre-)configured

Proposal 17:  When the total number of successful S-SSB transmissions within a S-SSB period is less than the number of R16/R17 S-SSB occasions, UE attempts to transmit on the subsequent candidate S-SSB occasion, the candidate S-SSB occasion can be either legacy R16/R17 S-SSB occasion or additional candidate S-SSB occasion.

Proposal 18:  For S-SSB transmission UE, additional transmissions (such as repetitive S-SSB transmissions or dummy signal transmissions) over multiple RB sets should be supported to maintain COT that contains multiple RB sets and includes S-SSB slot(s). And S-SSB receptions only be performed for the normal S-SSB transmission.

Proposal 19: For PSFCH transmission under 15 KHz and 30KHz SCS, alt 2-2a and alt 2-4a can be further studied.

Proposal 20: For PSFCH transmission with 60 KHz SCS, alt 3-2a can be further studied.

Proposal 21: The PSFCH resource associated with its corresponding PSSCH is determined by an implicit manner, i.e., option 1 should be supported.

Proposal 22: Whether/how to support option 2 (such PSFCH occasion(s) are (pre-)configured and dynamically indicated) should be deferred until the details design of COT sharing operation is clear.

Proposal 23: The following issues should be further studied:

- How to meet the HARQ RTT restriction considering that more than 1 PSFCH occasion per PSCCH/PSSCH transmission has already been endorsed

- How to (pre-)configure PSFCH resources, e.g., whether every RB set should be (pre-)configured with PSFCH resources

- How to determine PSFCH resource in response to a PSSCH reception especially when PSSCH transmission spanning over multiple RB sets is performed

References

- RP-230077, “WID revision: NR sidelink evolution”, OPPO, Mar. 2023.

- 3GPP TSG RAN WG1 Meeting RAN1 #110bis-e Chairman's Notes, Oct. 2022.

- 3GPP TSG RAN WG1 Meeting RAN1 #112 Chairman's Notes, Mar. 2023.

- 3GPP TSG RAN WG1 Meeting RAN1 #111 Chairman's Notes, Nov. 2022.

- 3GPP TSG RAN WG1 Meeting RAN1 #110 Chairman's Notes, Aug. 2022.