3GPP TSG RAN WG1 #112bis-e	                                            R1-2302706

e-Meeting, April 17th – 26th, 2023

Source:	CATT, GOHIGH

Title:	Discussion on co-channel coexistence for LTE sidelink and NR sidelink

Agenda Item:	9.4.2

Document for:	Discussion and Decision

Introduction

At RAN1#112 meeting, several agreements and one working assumption were achieved related to co-channel coexistence for LTE sidelink and NR sidelink as follows [1].

Agreement In NR SL resource (re)selection procedure, option 1 is adopted for how to determine candidate resource set for NR SL considering the LTE SL reserved resources by other LTE SL UE - Option 1: The PHY layer of NR SL module excludes NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE when the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold, where the SL RSRP threshold is derived based on LTE SL priority of other LTE SL UE and NR SL priority for NR SL transmission - For the list of the above initial SL RSRP threshold, down-select one of followings: - Alt 1: NR SL RSRP threshold list (pre)configured in a NR SL resource pool - Alt 2: LTE SL RSRP threshold list (pre)configured in a LTE SL resource pool - Alt 3: SL RSRP threshold list separately (pre)configured for dynamic resource pool sharing - Alt 4: SL RSRP threshold list separately (pre)configured for dynamic resource pool sharing - A different SL RSRP threshold list may be (pre)configured for selecting single slot resources in NR SL slots with NR PSFCH - For the LTE SL periodic reserved resources by other LTE SL UE, - For determining the above LTE SL reserved resources, the time-and-frequency resources of LTE SL reserved resources by other LTE SL UE are repeated Q times according to the LTE SL resource reservation period - FFS details including at least whether the formula of Q in Section 14.1.1.6 in TS 36.213 or the formula of Q in Section 8.1.4 in TS 38.214 is used - The PHY layer of NR module applies the above procedure in Step 6 in Section 8.1.4 of TS 38.214 - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of which NR SL periodic resources are overlapping with LTE SL reserved resources by other LTE SL UE when the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold according to condition c in Step 6 in Section 8.1.4 of TS 38.214 - Note: It is assumed that the information relevant to LTE SL reserved resources by other LTE SL UE used in the above procedure is shared from LTE SL module to NR SL module - FFS whether/how LTE SL RSRP is applied Working assumption - For dynamic resource pool sharing, select one option between 1-1 and 1-2, and select one option between 3-1 and 4-1-a, and define how the two selected options can be used in the operation: - Option 1-1: Avoiding PSFCH transmissions overlapping with LTE SL resources reserved for LTE SL transmissions in the time domain is performed by the UE transmitting PSFCH - FFS whether/how to define condition(s) under which the UE transmitting PSFCH drops its PSFCH transmission overlapping with the LTE SL resources reserved for LTE SL transmission - Option 1-2: Avoiding PSFCH transmissions overlapping with LTE SL resources reserved for LTE SL transmissions in the time domain is ensured by the UE transmitting PSSCH - FFS whether/how to define condition(s) of selecting NR SL candidate resources for NR SL PSCCH/PSSCH - Option 3-1: Additional PSFCH periodicity(ies) is introduced - Alt 3-1-a: 10 logical slots - Alt 3-1-b: 8 logical slots - Alt 3-1-c: 5 logical slots - FFS: Alignment between PSFCH periodicity and LTE logical subframes should be ensured by proper configuration - Option 4-1: PSFCH resources and LTE SL resources are TDMed - Alt 4-1-a: PSFCH resources is confined within the guard symbol of the LTE SL subframe in the time domain - Note: selecting a single option is not precluded Agreement In NR SL resource (re)selection procedure, down-select one of followings for how to determine candidate resource set for NR SL considering the LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission - Option 1: The PHY layer of NR SL module excludes NR SL candidate resources in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission - For the LTE SL periodic resources selected to be used for LTE SL module’s own LTE SL transmission, - For determining the above LTE SL selected resources, the time-and-frequency resources of LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission are repeated according to the LTE SL resource reservation period and LTE SL resource reselection count - The PHY layer of NR module applies the above procedure in Step 5 in Section 8.1.4 of TS 38.214 - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of which NR SL periodic resources are in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission according to Step 5 in Section 8.1.4 of TS 38.214 - Note: When the PHY layer of NR module cancels the above procedure according to Step 5a in Section 8.1.4 of TS 38.214, UE selects either LTE SL transmission or NR SL transmission according to Rel-16 NR SL in-device coexistence rule - FFS: whether the above procedure is applied based on the priority of LTE SL transmission and the priority of NR SL transmission - Note: It is assumed that the information relevant to LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission used in the above procedure is shared from LTE SL module to NR SL module - Option 2: The PHY layer of NR SL module determines NR SL candidate resources regardless of whether the NR SL candidate resources are in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission - UE selects either LTE SL transmission or NR SL transmission according to Rel-16 NR SL in-device coexistence rule

At RAN#99 [7] meeting, the objective for NR sidelink evolution for co-channel coexistence for LTE sidelink and NR sidelink has been updated as follows：

- Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any [RAN1, RAN2, RAN4] - Reuse the in-device coexistence framework defined in Rel-16 as much as possible - Note, RAN1 continues the work on dynamic resource pool sharing based on existing agreements and WID with high priority for Type A devices and operating combination A - RAN1 is tasked to support only 15 and 30 kHz SCSs for dynamic resource pool sharing. Existing RAN1 agreements for dynamic resource pool sharing apply to support of 30 kHz. - For NR PSCCH/PSSCH transmissions in 30kHz SCS, NR SL UE selects in MAC layer at least the first of NR SL slots overlapping with a LTE SL subframe, and can select the subsequent overlapping NR SL slot in MAC layer - No change to the R16/17 resource allocation procedure in PHY due to this restriction - The existing SL slot structure from Rel-16 is unchanged - The starting symbol of the first of the overlapping NR SL slots is assumed to be aligned with the first symbol of the LTE SL subframe - For NR SL with 15/30kHz SCSs, NR SL UE avoids selecting resources for PSCCH/PSSCH transmissions where the corresponding PSFCH transmission occasions overlap with LTE SL reservations in time domain - Note, this is inline with Option 1-2 in the working assumption made in RAN1#112. No other options from the working assumption need to be considered. - Mode 2 operation only

In this contribution, we continue to discuss remaining open issues for co-channel coexistence for LTE sidelink and NR sidelink and propose related solutions. Section 2 discusses the design principles of co-channel coexistence mechanism. Section 3 discusses the priority of SRPP and DRPS schemes. Section 4 discusses the types of devices which should be supported for co-channel coexistence. Section 5 discusses the combinations of operational modes which should be considered for co-channel coexistence. Section 6 discusses the semi-static resource pool partition for LTE sidelink and NR sidelink and Section 7 discusses the dynamic resource pool sharing for LTE sidelink and NR sidelink. Section 8 summarizes the proposals and observations with conclusions.

Requirements and design principles of co-channel coexistence mechanism

In Rel-16, sidelink communication was developed in RAN mainly to support advanced V2X applications. In Rel-17, SA2 studied and standardized Proximity based service including public safety and commercial related service. As part of Rel-17, power saving solutions (e.g., partial sensing, DRX) and inter-UE coordination had been developed in RAN1 and RAN2 to improve power consumption for battery limited terminals and reliability of sidelink transmissions.

One important aspect to consider is the V2X deployment scenario where both LTE V2X and NR V2X devices are to coexist in the same frequency channel. For the two different types of devices to coexist while using a common carrier frequency, it is important that there is a mechanism to efficiently utilize resource allocation by the two technologies without negatively impacting the operation of each technology.

This requirement was also mentioned as part of the input from 5G Automotive Association (5GAA) to the Rel-18 RAN Workshop, and it was listed as the second highest priority among the 8 requirement proposals by 5GAA [2], as shown in the Table 1 below.

Table 1: 5GAA list of proposals according to priority ranking

| No. | 5GAA features and requirements proposals | Priority ranking |
| --- | --- | --- |
| 1 | Positioning enhancements | 1 |
| 2 | LTE/NR-V2X sidelink co-channel coexistence | 2 |
| 3 | Sidelink carrier aggregation (CA) | 3 |
| 4 | Enhancements to sidelink power saving | 4 |
| 5 | Predictive QoS | 5 |
| 6 | UE-to-UE relay | 6 |
| 7 | NR-V2X sidelink adjacent-channel coexistence with non-3GPP technologies | 7 |
| 8 | Enhancements for vehicular distributed antenna system (DAS) UE transmission | 8 |

According to the requirements from 5GAA, the motivations for co-channel coexistence mechanism include [2]:

- Dedicated V2X spectrum is scarce in some regions and LTE/NR-V2X need to co-exist even in the same channel;

- Important across all basic and advanced use cases;

- Improves overall system performance, increases deployment flexibility, enables technology migration path from IEEE technology.

In order to meet the requirements from 5GAA and realize co-channel coexistence for LTE sidelink and NR sidelink, the design principles of the co-channel coexistence mechanism between LTE sidelink and NR sidelink should be discussed first. In our point of view, the basic design principle should be ensuring backward compatible of R14/R15 LTE sidelink and R16/R17 NR sidelink. It is also important to reuse the in-device coexistence framework defined in Rel-16 as much as possible. In addition, we prefer no or limited performance degradation of LTE sidelink.

Proposal 1: Design principles of co-channel coexistence mechanism for LTE sidelink and NR sidelink should be considered as follows:

- Ensuring backward compatible of R14/R15 LTE sidelink and R16/R17 NR sidelink

- Reuse the in-device coexistence framework defined in Rel-16 as much as possible

- No or limited performance degradation of LTE sidelink

Priority of SRPP and DRPS schemes

At RAN#99 meeting, the following revised objective related to co-channel coexistence for LTE sidelink and NR sidelink in the updated WID was agreed [7].

Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any [RAN1, RAN2, RAN4] - Reuse the in-device coexistence framework defined in Rel-16 as much as possible - Note, RAN1 continues the work on dynamic resource pool sharing based on existing agreements and WID with high priority for Type A devices and operating combination A - RAN1 is tasked to support only 15 and 30 kHz SCSs for dynamic resource pool sharing. Existing RAN1 agreements for dynamic resource pool sharing apply to support of 30 kHz. - For NR PSCCH/PSSCH transmissions in 30kHz SCS, NR SL UE selects in MAC layer at least the first of NR SL slots overlapping with an LTE SL subframe, and can select the subsequent overlapping NR SL slot in MAC layer - No change to the R16/17 resource allocation procedure in PHY due to this restriction - The existing SL slot structure from Rel-16 is unchanged - The starting symbol of the first of the overlapping NR SL slots is assumed to be aligned with the first symbol of the LTE SL subframe - For NR SL with 15/30kHz SCSs, NR SL UE avoids selecting resources for PSCCH/PSSCH transmissions where the corresponding PSFCH transmission occasions overlap with LTE SL reservations in time domain - Note, this is inline with Option 1-2 in the working assumption made in RAN1#112. No other options from the working assumption need to be considered. - Mode 2 operation only

In our view, such revised objective just adds a sub-objective to explain that RAN1 is tasked to support only 15 and 30 kHz SCSs for dynamic resource pool sharing, so it doesn’t mean Semi-static Resource Pool Partition (SRPP) scheme do not need further discussion. In fact, considering the good progress on dynamic resource pool sharing at the previous meetings, we prefer that RAN1 should continue the work on semi-static resource pool partition at RAN1#112bis-e and RAN1#113 meetings and finish the design of semi-static resource pool partition by June 2023.

Proposal 2: RAN1 should continue the work on semi-static resource pool partition at RAN1#112bis-e and RAN1#113 meetings and finish the design of semi-static resource pool partition scheme by June 2023.

Types of devices

The following working assumption was achieved in RAN1#110 related to device type A [10]:

Working assumption Co-channel coexistence between LTE SL and NR SL is supported for device type A. Device type A contains both LTE SL and NR SL modules. For device type A, the NR SL module may use the sensing and resource reservation information shared by the LTE SL module.

Device type A should be the most important target device type for co-channel coexistence between LTE SL and NR SL in Rel-18. According to the discussion in previous meetings, device type A should contain both LTE SL and NR SL modules. However, the presence of both the LTE SL and NR SL modules alone cannot exactly define the device type A. The relationship and interaction between the LTE SL and NR SL modules should also be defined. Therefore, The definition of device type A should include the capability of the LTE SL module to share the sensing and resource reservation information to the NR SL module within the device, and the capability of the NR SL module to include this sensing and resource reservation information in its resource selection procedure. Then, in addition to device type A should contain both LTE SL and NR SL modules, for device type A, the NR SL module may use the sensing and resource reservation information shared by the LTE SL module.

Proposal 3: Confirm the working assumption in RAN1#110 as follows:

- Co-channel coexistence between LTE SL and NR SL is supported for device type A. Device type A contains both LTE SL and NR SL modules. For device type A, the NR SL module may use the sensing and resource reservation information shared by the LTE SL module.

The following proposal was discussed in RAN1#110 related to device type B, but failed to reach a consensus [4]:

Proposal 1-2a (II): - For co-channel coexistence solutions in Rel-18, RAN1 is to study device type B which contains: - Alt 1: Only an NR SL module. - Alt 2: a co-located LTE SL and NR SL module, but the LTE SL module does not share any LTE sensing and resource reservation information.

The motivation of introduction of device type B into co-channel coexistence investigation in Rel-18 is that device type B may potentially have lower cost and complexity compared to device type A. Regarding the definition of device type B, in our point of view, device type B can include two kinds of sub-types: device type B1 and device type B2. Both device type B1 and device type B2 are the possible device types in the real deployment.

First, device type B1 should only contain an NR SL module, this definition of device type B1 aligns with the discussion in the past RAN1 meetings. Some companies think that the absence of LTE SL module for device type B1 will lead to the device type B1 cannot receive the basic safety messages (BSM) then it cannot coordinate with LTE SL devices. However, the device type A can share the BSM to device type B1 via IUC messages.

Second, device type B2 should contain a co-located LTE SL and NR SL module, but the LTE SL module does not share any LTE sensing and resource reservation information. In this case, the LTE SL module of device type B2 supports the Rel-16 in-device coexistence framework, but it does not support the new mechanism of sharing LTE sensing and resource reservation information to the NR SL module. With the aid of legacy LTE SL module, device type B2 can transmit/receive BSM to/from other legacy LTE UEs.

We prefer RAN1 to study both device type B1 and device type B2 for co-channel coexistence solutions in Rel-18.

Proposal 4: For co-channel coexistence solutions in Rel-18, RAN1 should study both device type B1 and device type B2 as follows:

- Device type B1 contains only an NR SL module.

- Device type B2 contains a co-located LTE SL and NR SL module, but the LTE SL module cannot share any LTE sensing and resource reservation information to NR SL module.

Combinations of operational modes

At RAN1#109-e, the following combinations of operational modes to be considered for the study of co-channel coexistence in Rel-18 were discussed:

- Combination A: Mode 2 NR SL + Mode 4 LTE SL

- Combination B: Mode 1 NR SL + Mode 4 LTE SL

- Combination C: Mode 2 NR SL + Mode 3 LTE SL

The following agreement was achieved in RAN1#109-e related to combinations of operational modes [3]:

Agreement For the study of co-channel coexistence solutions in Rel-18, the combination of operational modes Mode 2 NR SL with Mode 4 LTE SL (Combination A) is considered with high priority. - FFS: Whether/how to support Mode 1 NR SL + Mode 4 LTE SL (Combination B) and/or Mode 2 NR SL + Mode 3 LTE SL (Combination C).

In the above agreement, the combination of operational modes Mode 2 NR SL with Mode 4 LTE SL (Combination A) is considered with high priority. We admit that the agreed Combination A is the most challenging scenario for co-channel coexistence of NR SL and LTE SL in Rel-18. However, Combination C is similar with Combination A, since both of them will focus on the enhancements of NR sidelink and LTE sidelink specification shall not be impacted, so Combination C should be considered. In addition, Combination B is also one possible deployment scenario for NR SL and LTE SL and it should also be considered. Therefore, for the study of co-channel coexistence solutions in Rel-18, the combinations of operational modes Mode 1 NR SL + Mode 4 LTE SL (Combination B) and Mode 2 NR SL + Mode 3 LTE SL (Combination C) should be considered.

For the following FL proposal raised and discussed in RAN1#110 [4]:

Conclusion 3-1 (I): - For co-channel coexistence solutions in Rel-18, the combination of operational modes Mode 1 NR SL + Mode 4 LTE SL (Combination B) and Mode 2 NR SL + Mode 3 LTE SL (Combination C) can be revisited once Mode 2 NR SL with Mode 4 LTE SL (Combination A) has been specified.

Regarding the above proposal, there is still a concern as it would be better to consider the Combination B and Combination C at the earlier stage, otherwise the design agreed may be incompatible with the combinations needs to be considered at later stage.

Proposal 5: For the study of co-channel coexistence solutions in Rel-18, the combinations of operational modes Mode 1 NR SL + Mode 4 LTE SL (Combination B) and Mode 2 NR SL + Mode 3 LTE SL (Combination C) should also be considered at the earlier stage.

Semi-static resource pool partition between LTE sidelink and NR sidelink

In this section, remaining issues on semi-static resource pool partition between LTE sidelink and NR sidelink are discussed, including FDM based semi-static resource pool partition and synchronization issue.

FDM based semi-static resource pool partition

The following feature lead proposal was discussed but failed to reach a consensus in RAN1#110 related to FDM based semi-static resource pool partition between LTE sidelink and NR sidelink [4]:

Proposal 2-2 (II): - For co-channel coexistence in Rel-18, RAN1 assumes that the FDM-based semi-static resource pool partitioning can be used based on Rel-16/17 specifications, and can be studied with the following constraints: - NR SL resource pool is configured with only 15 kHz SCS. - FFS other solutions to overcome the AGC issue caused by the differing SCSs between the NR SL and LTE SL resource pools. - Transmission/reception Configuration of PSFCH in resources overlapping with LTE SL subframes is not permitted. - FFS other solutions to overcome the AGC issues caused due to PSFCH being configured in NR SL resource pools. - FFS other constraints whether a guard band is required requirement. - Note: The LTE and NR resource pools do not overlap in frequency with each other in the FDM-based semi-static resource pool partitioning.

For FDM based semi-static resource pool partition, two separate resource pools that do not overlap in frequency domain are (pre)-configured for LTE sidelink and NR sidelink.

There is AGC issue for LTE SL receiving UE if the sub-carrier spacing of LTE sidelink is different from that of NR sidelink, i.e. 15kHz and 30kHz, respectively. Even if the carrier spacing of LTE SL and NR SL is same, there is also AGC issue because NR sidelink supports PSFCH transmission feature which LTE sidelink does not support.

According to the proposal 2-2 (II) raised in RAN1#110, there is a solution that configuration of PSFCH in resources overlapping with LTE SL subframes is not permitted. This solution implies that regardless of whether the PSFCH resource of NR SL is configured every 1, 2, or 4 slots, a large number of slots will be excluded from the LTE resource pool, which will result in undesirable spectral inefficiencies.

In the same way, a guard band will also lead to spectral inefficiencies in case of the available spectrum is already limited.

In our point of view, HARQ based retransmissions cannot be supported for NR SL if we take the FDM based semi-static resource pool partition solution, so this solution is not adopted.

Proposal 6: FDM based semi-static resource pool partition solution is not adopted for co-channel coexistence mechanism between LTE sidelink and NR sidelink in Rel-18.

Synchronization issue

The following conclusion was achieved in RAN1#110 related to TDM based semi-static resource pool partition between LTE sidelink and NR sidelink [10]:

Conclusion For co-channel coexistence in Rel-18, RAN1 concludes that the TDM-based semi-static resource pool partitioning based on Rel-16/17 specifications is one possible solution to ensure co-channel coexistence between LTE-V UEs and NR-V UEs. - Note: The LTE and NR resource pools do not overlap in time with each other in the TDM-based semi-static resource pool partitioning. - Note 2: Rel-16 in-device coexistence framework can ensure alignment between the slot boundary of the NR SL time slot and the subframe boundary of the LTE SL subframe - FFS: potential enhancements for synchronization can be further investigated

In the above conclusion, TDM-based semi-static resource pool partitioning was agreed as one possible solution to ensure co-channel coexistence between LTE-V UEs and NR-V UEs. And there is one FFS on whether potential enhancements for synchronization should be further investigated.

The following section in TS 38.213 [5] described the operation for in-device coexistence:

- 16.7 Operation for in-device coexistence If a UE would transmit or receive a first channel/signal using E-UTRA radio access and a second channel/signal using NR radio access, when - the first channel/signal and the second channel/signal are time-division multiplexed, and - the UE knows the frame indexes of the first channel/signal and the frame indexes of the second channel/signal, the UE transmits or receives each channel/signal so that the subframe boundary of the second channel/signal is aligned with the subframe boundary of the first channel/signal where the subframe boundary alignment is achieved by UE implementation means.

According to the above conclusion achieved in RAN1#110, RAN1 noted that Rel-16 in-device coexistence framework can ensure alignment between the slot boundary of the NR SL time slot and the subframe boundary of the LTE SL subframe.

Moreover, according to section 16.7 in TS 38.213 above, the UE transmits or receives each channel/signal so that the subframe boundary of the NR channel/signal is aligned with the subframe boundary of the LTE channel/signal where the subframe boundary alignment is achieved by UE implementation means.

Therefore, it can be observed that the synchronization issue can be solved with Rel-16 in-device coexistence framework. At RAN1#110, some companies proposed that both the NR SL module and the LTE SL module should select the same synchronization source, or suggested that the NR SL module can use the LTE SL synchronization procedure and sync source [4]. We think there is no need to further study the potential enhancements for synchronization since the existing Rel-16 in-device coexistence framework can ensure alignment between the slot boundary of the NR SL time slot and the subframe boundary of the LTE SL subframe, and how to achieve the alignment is up to UE implementation.

As shown in section 4, we believe RAN1 should also study both device type B1 and device type B2 for co-channel coexistence solutions in Rel-18. Device type A and device type B2 can be able to achieve this synchronization based on Rel-16 in-device coexistence framework, but device type B1 cannot achieve it. The synchronization issue in Rel-18 is how to achieve the synchronization among device type A, device type B1/B2 and device type C (device type D and device type E also need to be considered). In fact, there is also synchronization issue in Rel-16/17 that how to achieve the synchronization among device type C, device type D and device type E, and we think the synchronization issue in Rel-18 is same as the synchronization issue in Rel-16/17, because device type B1 and device type D are similar in that they contain only NR SL modules, and device type A, device type B2 and device type E are similar in that they contain both LTE SL and NR SL modules. Therefore, we consider synchronization issue to be a low priority for co-channel coexistence investigations in Rel-18. Moreover, such synchronization issue exists not only for semi-static resource pool partition, but also for Dynamic resource pool sharing.

Proposal 7: There is no need to further study the potential enhancements for synchronization or consider it as low priority since the existing Rel-16 in-device coexistence framework can ensure alignment between the slot boundary of the NR SL time slot and the subframe boundary of the LTE SL subframe.

Dynamic resource pool sharing between LTE sidelink and NR sidelink

In this section, the following issues on dynamic resource pool sharing between LTE sidelink and NR sidelink are discussed:

- Higher SCS configuration for NR sidelink is used in dynamic resource pool

- Resource selection enhancements

  - The timeline of the information shared by LTE sidelink module

  - How NR sidelink module used the information shared by LTE sidelink module

  - Enhancements of Inter-UE coordination(IUC)

At RAN#99 [7] meeting, the objective for NR sidelink evolution for co-channel coexistence for LTE sidelink and NR sidelink has been revised, the scope of which includes：

- Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any [RAN1, RAN2, RAN4] - Reuse the in-device coexistence framework defined in Rel-16 as much as possible - Note, RAN1 continues the work on dynamic resource pool sharing based on existing agreements and WID with high priority for Type A devices and operating combination A - RAN1 is tasked to support only 15 and 30 kHz SCSs for dynamic resource pool sharing. Existing RAN1 agreements for dynamic resource pool sharing apply to support of 30 kHz. - For NR PSCCH/PSSCH transmissions in 30kHz SCS, NR SL UE selects in MAC layer at least the first of NR SL slots overlapping with a LTE SL subframe, and can select the subsequent overlapping NR SL slot in MAC layer - No change to the R16/17 resource allocation procedure in PHY due to this restriction - The existing SL slot structure from Rel-16 is unchanged - The starting symbol of the first of the overlapping NR SL slots is assumed to be aligned with the first symbol of the LTE SL subframe - For NR SL with 15/30kHz SCSs, NR SL UE avoids selecting resources for PSCCH/PSSCH transmissions where the corresponding PSFCH transmission occasions overlap with LTE SL reservations in time domain - Note, this is inline with Option 1-2 in the working assumption made in RAN1#112. No other options from the working assumption need to be considered. - Mode 2 operation only

Configuration of higher SCS

According to the revised objective for co-channel coexistence for NR sidelink evolution in RAN#99 [7], only 15kHz and 30kHz SCSs were supported for dynamic resource pool sharing, and NR SL UE selects in MAC layer at least the first of NR SL slots overlapping with a LTE SL subframe, and can select the subsequent overlapping NR SL slot in MAC layer.

Firstly, the revised objective means NR SL UE can only select the first of NR SL slots overlapping with a LTE SL subframe and resource efficiency is reduced if NR SL UE works in unicast or groupcast, because if NR SL UE selects the subsequent overlapping NR SL slot, it does not ensure the minimum time gap between any two selected resources in case that PSFCH is configured for this pool of resources, and receiving NR SL UE cannot feedback in time. RAN1 needs to study the impact of the restriction of update WID.

Secondly, no change to the R16/17 resource allocation procedure in PHY with the restriction that NR SL UE selects in MAC layer at least the first of NR SL slots overlapping with a LTE SL subframe base on the revised objective, but that means the number of actual available resources in the set SA reported to MAC layer by PHY layer may be smaller than X⋅Mtotal when NR SL UE works in unicast or groupcast. So when SCS is 30kHz for NR SL, as shown in following Figure 1, the resource should be not involved in the total number of candidate resource Mtotal and should be removed from initial SA if it meets any following conditions:

- The resource overlapping with a LTE SL subframe configured within the LTE SL resource pool, and the first slot in a subframe which contains the resource is not in NR SL resource pool in NR SL resource selection window, like the yellow resources and blue resources in Figure 1.

- The resource overlaps with the subframe configured for LTE SL SLSS, like the purple resources in Figure 1.

Figure 1: the resources should be removed from Mtotal and the initial SA

Thirdly, when SCS is 30kHz for NR SL, as shown in Figure 1, the UE shall exclude any candidate resource from SA after step 6) in Section 8.1.4 in TS 38.214 and the exclusion procedure considering the LTE SL reserved resources by other LTE SL UE if it meets following condition:

- The resource overlapping with a LTE SL subframe configured within the LTE SL resource pool and resource in the same frequency domain as the resource in first slot in a subframe which contains the resource has been excluded from SA, like the green resources in Figure 1.

Observation 1: The revised objective means NR SL UE can only select the first of NR SL slots overlapping with a LTE SL subframe and resource efficiency is reduced if NR SL UE works in unicast or groupcast.

Observation 2: The number of actual available resources in the set SA reported to MAC layer by PHY layer may be smaller than X⋅Mtotal when NR SL UE works in unicast or groupcast.

Proposal 8: RAN1 needs to study the impact of the restriction of update WID.

Proposal 9: When SCS is 30kHz for NR SL, the resource should be not involved in the total number of candidate resource Mtotal and the resource should be removed from initial SA if it meets any following conditions:

- The resource overlapping with a LTE SL subframe configured within the LTE SL resource pool, and the first slot in a subframe which contains the resource is not in NR SL resource pool in NR SL resource selection window, like the yellow resources and blue resources in Figure 1.

- The resource overlaps with the subframe configured for LTE SL SLSS.

Proposal 10: When SCS is 30kHz for NR SL, the UE shall exclude any candidate resource from SA after step 6) in Section 8.1.4 in TS 38.214 and the exclusion procedure considering the LTE SL reserved resources by other LTE SL UE if it meets following condition:

- The resource overlapping with a LTE SL subframe configured within the LTE SL resource pool, and  resource in the same frequency domain as the resource in first slot in a subframe which contains the resource has been excluded from SA.

Resource selection enhancements

Timeline of information shared by the LTE SL module

For the timeline of the information shared by LTE SL module in the dynamic resource pool, the following agreement was achieved in RAN1#111 meeting[9] and the following latest  FL’s proposal was discussed in RAN1#112 meeting [8]:

Agreement Based on the agreement in RAN1#110bis-e, the value of Tmax = 4 ms. Proposal 4-1(I): - For dynamic resource pool sharing, the starting LTE SL subframe used to determine the information shared with the NR SL module is at most T_valid1 ms prior to the time (n-T) where the shared information is known by the NR SL module. - Down-select one of followings for the value of T_valid1: - Option 1: T_valid1 is defined as a fixed value - Alt 1-1: T_valid1 = 1000ms - Alt 1-2: T_valid1 = 2000ms - Alt 1-3: T_valid1 = 5000ms - Option 2: T_valid1 is (pre)configured - Option 3: T_valid1 is up to UE implementation Proposal 4-2(I): - For dynamic resource pool sharing, the ending LTE SL subframe used to determine the information shared with the NR SL module is at the latest T_valid2 ms prior to the time (n-T) where the shared information is known by the NR SL module. - Down-select one of followings for the value of T_valid2: - Alt 1: T_valid2 is defined as a fixed value - FFS: Details - Alt 2: T_valid2 is up to UE implementation

Figure 2: Sensing window shared by LTE SL module

For above proposal 4-2(I), the ending LTE SL subframe used to determine the information shared with the NR SL module should be determined and restricted to ensure the shared information is useful, for example, as shown in Figure 2, the ending LTE SL subframe is at most T_valid2 ms prior to the time n, the ending LTE SL subframe and the time of sharing information are in the duration [n- T_valid2, n-T]. For above proposal 4-1(I), The starting LTE SL subframe used to determine the information shared with the NR SL module could be confirmed by the ending LTE SL subframe and a sensing window size T0. For the sensing window size, it could be a (pre-)configuration value, i.e. 1000ms, or up to UE implementation.

Proposal 11: The sensing window of LTE SL module shared information could be determined by the ending LTE SL subframe and a sensing window size T0. The ending LTE SL subframe used to determine the information shared with the NR SL module is at most T_valid2 ms prior to the time n, the sensing window size T0 could be a (pre-)configuration value or up to UE implementation.

Use of information shared by the LTE SL module

For the use of sharing information shared by the LTE SL module in the dynamic resource pool, the following agreement was achieved in RAN1#112 meeting [1] and the following latest  FL’s proposal was discussed in RAN1#111 meeting [6] and in RAN1#112 meeting [8]:

Proposal 1-3 (II): - For dynamic resource pool sharing, the information shared by the LTE SL module to the NR SL module contains at least the following parameters: - Time and frequency locations of reserved resources by other LTE UEs, determined based on decoded SCIs - SL RSRP measurement results - Resource reservation periods based on decoded SCI and for own LTE SL transmissions - Priority based on decoded SCI and for own LTE SL transmissions - Time and frequency location of resources used for own LTE SL transmissions - LTE logical subframe related information - Resources corresponding to half-duplex subframes which are not monitored by the LTE SL UE - SL RSSI measurement results - It is RAN1’s understanding that the NR SL module is aware of the LTE SL resource pool configurations as per Rel-16 in-device coexistence. Agreement In NR SL resource (re)selection procedure, option 1 is adopted for how to determine candidate resource set for NR SL considering the LTE SL reserved resources by other LTE SL UE - Option 1: The PHY layer of NR SL module excludes NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE when the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold, where the SL RSRP threshold is derived based on LTE SL priority of other LTE SL UE and NR SL priority for NR SL transmission - For the list of the above initial SL RSRP threshold, down-select one of followings: - Alt 1: NR SL RSRP threshold list (pre)configured in a NR SL resource pool - Alt 2: LTE SL RSRP threshold list (pre)configured in a LTE SL resource pool - Alt 3: SL RSRP threshold list separately (pre)configured for dynamic resource pool sharing - Alt 4: SL RSRP threshold list separately (pre)configured for dynamic resource pool sharing - A different SL RSRP threshold list may be (pre)configured for selecting single slot resources in NR SL slots with NR PSFCH - For the LTE SL periodic reserved resources by other LTE SL UE, - For determining the above LTE SL reserved resources, the time-and-frequency resources of LTE SL reserved resources by other LTE SL UE are repeated Q times according to the LTE SL resource reservation period - FFS details including at least whether the formula of Q in Section 14.1.1.6 in TS 36.213 or the formula of Q in Section 8.1.4 in TS 38.214 is used - The PHY layer of NR module applies the above procedure in Step 6 in Section 8.1.4 of TS 38.214 - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of which NR SL periodic resources are overlapping with LTE SL reserved resources by other LTE SL UE when the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold according to condition c in Step 6 in Section 8.1.4 of TS 38.214 - Note: It is assumed that the information relevant to LTE SL reserved resources by other LTE SL UE used in the above procedure is shared from LTE SL module to NR SL module - FFS whether/how LTE SL RSRP is applied Agreement In NR SL resource (re)selection procedure, down-select one of followings for how to determine candidate resource set for NR SL considering the LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission - Option 1: The PHY layer of NR SL module excludes NR SL candidate resources in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission - For the LTE SL periodic resources selected to be used for LTE SL module’s own LTE SL transmission, - For determining the above LTE SL selected resources, the time-and-frequency resources of LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission are repeated according to the LTE SL resource reservation period and LTE SL resource reselection count - The PHY layer of NR module applies the above procedure in Step 5 in Section 8.1.4 of TS 38.214 - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of which NR SL periodic resources are in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission according to Step 5 in Section 8.1.4 of TS 38.214 - Note: When the PHY layer of NR module cancels the above procedure according to Step 5a in Section 8.1.4 of TS 38.214, UE selects either LTE SL transmission or NR SL transmission according to Rel-16 NR SL in-device coexistence rule - FFS: whether the above procedure is applied based on the priority of LTE SL transmission and the priority of NR SL transmission - Note: It is assumed that the information relevant to LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission used in the above procedure is shared from LTE SL module to NR SL module - Option 2: The PHY layer of NR SL module determines NR SL candidate resources regardless of whether the NR SL candidate resources are in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission - UE selects either LTE SL transmission or NR SL transmission according to Rel-16 NR SL in-device coexistence rule Proposal 2-3(II): - In NR SL resource (re)selection procedure, down-select one of followings for how to determine candidate resource set for NR SL considering the LTE SL resources associated with non-monitored subframe of LTE SL module - Option 1: - The PHY layer of NR SL module excludes NR SL candidate resources overlapping with LTE SL resources associated with non-monitored subframe of LTE SL module - For determining the LTE SL resources associated with non-monitored subframe of LTE SL module, - All the LTE SL resources in the non-monitored subframes are assumed to be repeatedly reserved Q times for each LTE SL resource reservation period (pre)configured in a LTE SL resource pool - FFS details including at least whether the formula of Q in Section 14.1.1.6 in TS 36.213 or the formula of Q in Section 8.1.4 in TS 38.214 is used - The PHY layer of NR module applies the above procedure in Step 5 in Section 8.1.4 of TS 38.214 - Note: For periodic reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of which NR SL periodic resources are overlapping with LTE SL resources associated with non-monitored subframe of LTE SL module according to Step 5 in Section 8.1.4 of TS 38.214 - Note: It is assumed that the information relevant to LTE SL resources associated with non-monitored subframe of LTE SL module used in the above procedure is shared from LTE SL module to NR SL module - Option 2: - The PHY layer of NR SL module does not perform NR SL candidate resource exclusion based on the information regarding LTE SL resources associated with non-monitored subframe of LTE SL module

Issue1: Information shared from the LTE SL module to the NR SL module within a device

For device type A, the NR SL module can obtain some information from the LTE SL module. For candidate information shared by LTE SL module, in addition to the reserved resources information by other LTE UEs and own LTE SL module, the NR SL module is aware of the LTE SL resource pool configurations as per Rel-16 in-device coexistence which at least contains the following information:

- LTE logical subframe related information

- Sub-channel configuration of shared LTE SL resource pool (including the number of sub-channel and the number of PRB per sub-channel)

- Available resource reservation periods in shared LTE SL resource pool

- LTE SLSS subframe related information

- Pstep

Additionally, the count of resource reservation of LTE SL module also needs to be shared by LTE SL module to NR SL module then NR SL module can accurately avoid TX/TX collision between LTE SL module and NR SL module according to the transmission time, period and count of resource reservation. The ending LTE SL subframe used to determine the information shared with the NR SL module also needs to be shared, which can be used to determine the value of Q by  NR SL module.

Issue 2: how to determine candidate resource set for NR SL considering the LTE SL reserved resources by other LTE SL UE

For the list of the above initial SL RSRP threshold used to exclude NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE, Alt2 or Alt3 is applicable because the exclusion operation is related to LTE SL resources and NR SL RSRP threshold list may not work. For Alt 4, NR SL UE avoids selecting resources for PSCCH/PSSCH transmissions where the corresponding PSFCH transmission occasions overlap with LTE SL reservations in time domain based on revised WID, NR SL UE transmits PSCCH/PSSCH in the first 10 symbols in the slot overlapping with LTE SL transmission will not lead to AGC issue for LTE SL UE receiving LTE SL transmission.

For determining the above LTE SL reserved resources, the time-and-frequency resources of LTE SL reserved resources by other LTE SL UE are repeated Q times according to the LTE SL resource reservation period.  The formula of Q in Section 8.1.4 in TS 38.214 is properly to be used because the resource selection window of NR SL is possible large than 100ms. But some modifications are still needed of the formula of Q in Section 8.1.4 in TS 38.214, for example, the definition of  tn'SL should be redefined by subframe tp'SL which is the first subframe after the ending LTE SL subframe used to determine the information shared with the NR SL module belonging to the LTE SL set t0SL, t1SL,..., tTmaxSL, and the definition of Tscal  should be reset to the duration between the ending LTE SL subframe p and the subframe which contains slot n+T2 converted to units of msec. Because if the definitions of tn'SL  and  Tscal in Section 8.1.4 in TS 38.214 are reused, the yellow LTE SL reserved resources by other LTE SL UE shown as Figure 3 are only repeated one time, which leads to the candidate single-slot resources overlapping with LTE SL reserved resources by other LTE SL UE will be not excluded by NR SL module.

Figure 3: LTE SL reserved resources by other LTE SL UE are repeated Q times

The formula of Q for the repeat times of the time-and-frequency resources of LTE SL reserved resources by other LTE SL UE as follows:

- the LTE SL reserved resource in  tmSL is assumed to be reserved in subframe(s) tm+q×Pstep×Prsvp_RXSL which overlaps with  Rx, y+j×Prsvp_TX' for q=1, 2, …, Q and j=0, 1, …, cresel-1. Here, Q=Tscal100×Prsvp_RX if 100×Prsvp_RX< Tscal and p‘-m≤Pstep×Prsvp_RX, where tp'SL is the subframe p if the subframe p belongs to the LTE SL set t0SL, t1SL,..., tTmaxSL, otherwise subframe tp'SL is the first subframe after subframe p belonging to the LTE SL set t0SL, t1SL,..., tTmaxSL; otherwise Q=1. Tscal is set to the duration between subframe p and subframe which contains slot n+T2 converted to units of msec.The subframe p is  set to the ending LTE SL subframe used to determine the information shared with the NR SL module.

Issue 3: how to determine candidate resource set for NR SL considering the LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission

Firstly, all LTE SL resources in the subframe m which contains LTE SL resources selected to be used for LTE SL module are repeated Q times according to the LTE SL resource reservation period. The value of Q equals to LTE SL resource reselection count if m +count ×Pstep×Prsvp_RX ≥ d, subframe d is the subframe containing slot n+T2 and belonging to the LTE SL set t0SL, t1SL,..., tTmaxSL, otherwise subframe d is the first subframe before the subframe containing slot n+T2 and belonging to the LTE SL set t0SL, t1SL,..., tTmaxSL; otherwise  the formula of Q  in above bullet that NR SL module determines candidate resource set considering the LTE SL reserved resources by other LTE SL UE is used.

Secondly, this procedure should be set to step5-1) and legacy step5) in Section 8.1.4 of TS 38.214 should be set to step5-2).  if the number of candidate single-slot resources Rx,y remaining in the set SA after step5-1) is smaller than X⋅Mtotal, the set SA is initialized to the set of step 4) and the procedure skips to step 6), if the number of candidate single-slot resources Rx,y remaining in the set SA after step5-2) is smaller than X⋅Mtotal, the set SA is initialized to the set of step 5-1) and the procedure skips to step 6).

Issue 4: how to determine candidate resource set for NR SL considering the LTE SL resources associated with non-monitored subframe of LTE SL module

All LTE SL resources in the subframe m of associated with non-monitored subframe of LTE SL module are repeated Q times according to any resource reservation period configured in LTE SL resource pool. The formula of Q in above bullet that NR SL module determines candidate resource set considering the LTE SL reserved resources by other LTE SL UE is used.

Secondly, this procedure should be set to step5-3). If the number of candidate single-slot resources Rx,y remaining in the set SA after step5-3) is smaller than X⋅Mtotal, the set SA is initialized to the set of step 5-2) and the procedure skips to step 6).

The formula of Q for the repeat times of the time-and-frequency resources of LTE SL associated with non-monitored subframe of LTE SL module as follows:

Observation 3: If the definitions of tn'SL and Tscal in Section 8.1.4 in TS 38.214 are reused, some candidate single-slot resources overlapping with LTE SL reserved resources by other LTE SL UE, or overlapping in time with LTE SL reserved resources by LTE SL module, or overlapping in time with LTE SL resources associated with non-monitored subframe of LTE SL module will be not excluded by NR SL module.

Proposal 12: In addition to the information in the agreement, the following information should be also transmitted from LTE SL module to NR SL module：

- The count value of resource reservation of LTE SL module

- The ending LTE SL subframe used to determine the information shared with the NR SL module

Proposal 13: The formula of Q in Section 8.1.4 in TS 38.214 is properly to be used.

Proposal 14: The definition of  tn'SL should be redefined by subframe tp'SL which is the first subframe after subframe p belonging to the LTE SL set t0SL, t1SL,..., tTmaxSL, the subframe p is set to the ending LTE SL subframe used to determine the information shared with the NR SL module..

Proposal 15: The definition of Tscal  should be reset to the duration between the ending LTE SL subframe p and the subframe which contains slot n+T2 converted to units of msec.

Proposal 16: The value of Q equals to LTE SL resource reselection count if m +count ×Pstep×Prsvp_RX ≥ d, when UE determine candidate resource set for NR SL considering the LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission,  subframe d is the subframe containing slot n+T2 and belonging to the LTE SL set t0SL, t1SL,..., tTmaxSL, otherwise subframe d is the first subframe before the subframe containing slot n+T2 and belonging to the LTE SL set t0SL, t1SL,..., tTmaxSL.

Enhancements of Inter-UE coordination (IUC)

Inter-UE coordination schemes are supported to overcome hidden node, half duplex, exposed node problems between NR SL UEs and improve reliability of NR SL transmissions in Rel-17 NR SL. In the scenario of dynamic resource pool sharing for co-channel coexistence between LTE SL and NR SL in Rel-18, there are also hidden node, half duplex, exposed node problems between NR SL UE and LTE SL UE, and Inter-UE coordination schemes (both scheme 1 and scheme 2) could be enhanced to overcome these problems to increase reliability of NR SL transmissions  and LTE SL transmissions. For example, in scheme 1, a type A device which contains both LTE SL module and NR SL module can determine preferred/non-preferred resources set according to sensing information of LTE SL module and/or NR SL module itself; in scheme 2, a type A device can transmit conflict information if there is a detection of expected/potential resource conflict(s) between LTE SL and NR SL.

On the other hand, if type B device which cannot obtain LTE SL resource reservation information from itself is supported in dynamic resource pool sharing, enhanced inter-UE coordination scheme 1 is an approach that provides LTE SL resource reservation to type B device to avoid expected/potential resource conflict with other LTE SL UE.

There are some aspects to be enhanced in inter-UE coordination schemes in Rel-18:

- For IUC scheme 1:

- If UE B receives a preferred resource set which is determined according to LTE SL module shared information and NR SL module sensing information of UE A, and there are no resources within the intersection of the received preferred resource set and SA, UE B selects the time and frequency resources for sidelink transmission(s) from SA in Rel-17 scheme 1 and there may still be collision between LTE SL and NR SL.

- If UE B receives a non-preferred resource set which is determined according to LTE SL module shared information and NR SL module sensing information of UE A, and it is not possible to meet the requirement that the number of candidate single-slot resources remaining in the set SA be at least X⋅Mtotal after excluding resource(s) overlapping with the received non-preferred resource set, it is up to UE B implementation whether or not to take into account the received non-preferred resource set to meet such requirement in Rel-17 scheme 1, and there may still be collision between LTE SL and NR SL.

- If UE B which is a type B device receives preferred/non-preferred resource set from UE A,  UE B should be able to identify the second UE is a type A device or type B device.

- For IUC scheme 2:

- If UE B receives the conflict information from UE A, UE B should be able to identify whether the conflict information indicates there is collision between LTE SL and NR SL or collision between NR SLs.

- If UE B receives the conflict information which indicates a collision between LTE SL and NR SL, and higher SCS configuration for NR SL resource pool, the resources which should be reported to higher layers by physical layer should be determined. For example, all resources in a subframe should be reported, the subframe refers to the one contains the slot where resource conflict will occur.

Proposal 17: Both IUC scheme 1 and scheme 2 should be enhanced to support to alleviate expected/potential resource conflict(s) between LTE SL and NR SL in dynamic resource pool sharing.

Conclusion

In this contribution, issues and enhancements of co-channel coexistence for LTE sidelink and NR sidelink are discussed. Our observations and proposals are given as follows:

Observation 1: The revised objective means NR SL UE can only select the first of NR SL slots overlapping with a LTE SL subframe and resource efficiency is reduced if NR SL UE works in unicast or groupcast.

Observation 2: The number of actual available resources in the set SA reported to MAC layer by PHY layer may be smaller than X⋅Mtotal when NR SL UE works in unicast or groupcast.

Observation 3: If the definitions of tn'SL and Tscal in Section 8.1.4 in TS 38.214 are reused, some candidate single-slot resources overlapping with LTE SL reserved resources by other LTE SL UE, or overlapping in time with LTE SL reserved resources by LTE SL module, or overlapping in time with LTE SL resources associated with non-monitored subframe of LTE SL module will be not excluded by NR SL module.

Proposal 1: Design principles of co-channel coexistence mechanism for LTE sidelink and NR sidelink should be considered as follows:

- Ensuring backward compatible of R14/R15 LTE sidelink and R16/R17 NR sidelink

- Reuse the in-device coexistence framework defined in Rel-16 as much as possible

- No or limited performance degradation of LTE sidelink

Proposal 2: RAN1 should continue the work on semi-static resource pool partition at RAN1#112bis-e and RAN1#113 meetings and finish the design of semi-static resource pool partition scheme by June 2023.

Proposal 3: Confirm the working assumption in RAN1#110 as follows:

- Co-channel coexistence between LTE SL and NR SL is supported for device type A. Device type A contains both LTE SL and NR SL modules. For device type A, the NR SL module may use the sensing and resource reservation information shared by the LTE SL module.

Proposal 4: For co-channel coexistence solutions in Rel-18, RAN1 should study both device type B1 and device type B2 as follows:

- Device type B1 contains only an NR SL module.

- Device type B2 contains a co-located LTE SL and NR SL module, but the LTE SL module cannot share any LTE sensing and resource reservation information to NR SL module.

Proposal 5: For the study of co-channel coexistence solutions in Rel-18, the combinations of operational modes Mode 1 NR SL + Mode 4 LTE SL (Combination B) and Mode 2 NR SL + Mode 3 LTE SL (Combination C) should also be considered at the earlier stage.

Proposal 6: FDM based semi-static resource pool partition solution is not adopted for co-channel coexistence mechanism between LTE sidelink and NR sidelink in Rel-18.

Proposal 7: There is no need to further study the potential enhancements for synchronization or consider it as low priority since the existing Rel-16 in-device coexistence framework can ensure alignment between the slot boundary of the NR SL time slot and the subframe boundary of the LTE SL subframe.

Proposal 8: RAN1 needs to study the impact of the restriction of update WID.

Proposal 9: When SCS is 30kHz for NR SL, the resource is not considered to be involved in the total number of candidate resource Mtotal and the resource should be removed from initial SA if it meets any following conditions:

- The resource overlapping with a LTE SL subframe configured within the LTE SL resource pool, and the first slot in a subframe which contains the resource is not in NR SL resource pool in NR SL resource selection window, like the yellow resources and blue resources in Figure 1.

- The resource overlaps with the subframe configured for LTE SL SLSS.

Proposal 10: When SCS is 30kHz for NR SL, the UE shall exclude any candidate resource from SA after step 6) in Section 8.1.4 in TS 38.214 and the exclusion procedure considering the LTE SL reserved resources by other LTE SL UE if it meets following condition:

- The resource overlapping with a LTE SL subframe configured within the LTE SL resource pool, and resource in the same frequency domain as the resource in first slot in a subframe which contains the resource has been excluded from SA.

Proposal 11: The sensing window of LTE SL module shared information could be determined by the ending LTE SL subframe and a sensing window size T0. The ending LTE SL subframe used to determine the information shared with the NR SL module is at most T_valid2 ms prior to the time n, the sensing window size T0 could be a (pre-)configuration value or up to UE implementation.

Proposal 12: In addition to the information in the agreement, the following information should be also transmitted from LTE SL module to NR SL module：

- The counter value of resource reservation of LTE SL module.

- The ending LTE SL subframe used to determine the information shared with the NR SL module.

Proposal 13: The formula of Q in Section 8.1.4 in TS 38.214 is properly to be used.

Proposal 14: The definition of  tn'SL should be redefined by subframe tp'SL which is the first subframe after subframe p belonging to the LTE SL set t0SL, t1SL,..., tTmaxSL, the subframe p is set to the ending LTE SL subframe used to determine the information shared with the NR SL module.

Proposal 15: The definition of Tscal  should be reset to the duration between the ending LTE SL subframe p and the subframe which contains slot n+T2 converted to units of msec.

Proposal 16: The value of Q equals to LTE SL resource reselection count if m +count ×Pstep×Prsvp_RX ≥ d, when UE determine candidate resource set for NR SL considering the LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission,  subframe d is the subframe containing slot n+T2 and belonging to the LTE SL set t0SL, t1SL,..., tTmaxSL, otherwise subframe d is the first subframe before the subframe containing slot n+T2 and belonging to the LTE SL set t0SL, t1SL,..., tTmaxSL.

Proposal 17: Both IUC scheme 1 and scheme 2 should be enhanced to support to alleviate expected/potential resource conflict(s) between LTE SL and NR SL in dynamic resource pool sharing.

References

  - RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #112.

  - RWS-210360, “5GAA input to 3GPP Rel.18 Workshop”, 3GPP TSG RAN e-Meeting #92-e.

  - RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #109-e.

  - R1-2208096, “FL Summary EoM of AI 9.4.2 - Co-Channel Coexistence for LTE and NR Sidelink”, 3GPP TSG RAN WG1 Meeting #110.

  - 3GPP TS 38.213, “NR; Physical layer procedures for control (Release 17)”, v17.5.0, 2023-03.

  - R1-2212540, “FL Summary EoM of AI 9.4.2 - Co-channel coexistence for LTE sidelink and NR sidelink”, 3GPP TSG RAN WG1 Meeting #111.

  - RP-230077, “WID revision: NR sidelink evolution”, RAN#99 meeting.

  - R1-2301855, “FL Summary EoM of AI 9.4.2 - Co-Channel Coexistence for LTE and NR Sidelink”, 3GPP TSG RAN WG1 Meeting #112.

  - RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #111.

  - RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #110.