3GPP TSG RAN WG1 #112bis-e	                      R1-2302708

e-Meeting, April 17th – 26th, 2023

Source:	CATT, GOHIGH

Title:	Further discussion on SL positioning reference signal

Agenda item:	9.5.1.1

Document for:	Discussion and Decision

Introduction

At RAN1#112 meeting, a lot of agreements were achieved related to SL positioning reference signal as follows [1].

Agreement SL PRS sequence is generated based on Gold sequence: rn=121-2⋅c2n+j121-2⋅c2n+1, where c(i) is a pseudo-random sequence as defined in Clause 5.2.1 of TS 38.211. Agreement - For SL PRS sequence generation, the pseudo-random sequence c(i) initialization equation is defined as a function of at least: slot number, symbol number, and a parameter nID,seqSL-PRS. - The pseudo-random sequence c(i) initialization equation is based on initialization equation as for DL PRS Agreement For SL PRS sequence generation, consider at least the following options to define the parameter nID,seqSL-PRS, and select one option: - Option 1: nID,seqSL-PRS is a higher layer configured parameter - Option 2: nID,seqSL-PRS is based on 12 bits CRC of PSCCH associated with the SL PRS transmission - Option 3: based on a combination of higher layer configured parameter from a configured ID list and 12 bits of CRC of PSCCH associated with the SL PRS transmission - Option 5: nID,seqSL-PRS is based on 12bits LSB of destination ID - Option 6: nID,seqSL-PRS is based on 8 bits of source ID + 4 zero bits - Option 7: nID,seqSL-PRS is based on the CRC field of the 2nd SCI associated with SL PRS transmission, if there is a 2nd SCI defined. Agreement Range of the parameter nID,seqSL-PRS is: nID,seqSL-PRS∈0,1,…,4095 Agreement A SL PFL is not defined. SL positioning RS are defined directly with respect to and contained within a single SL BWP and carrier. Agreement Support SCS values for SL PRS include: - 15 kHz, 30 kHz, 60 kHz for FR1, and 60 kHz, 120 kHz for FR2 - Which SCS values are required, and which ones are optional follow Rel-16 UE capabilities. Agreement For RE-offset sequence for SL PRS, the RE-offset sequences specified for DL PRS are considered as a starting point. - FFS: Exact RE-offset sequences Agreement - Comb-based multiplexing of SL PRS from different UEs in a slot is supported at least for dedicated resource pools. - FFS: Comb-based multiplexing of SL PRS from different UEs in a slot for shared resource pools. - For comb-based multiplexing of SL PRS from different UEs, support at least the case wherein a single (M,N) value is possible . - FFS: Whether to support comb-based multiplexing of SL PRS from different UEs in a slot using multiple (M,N) values. - FFS: additional restrictions (if any) due to e.g. the impact of synchronization and IBE interference between UEs Agreement For SL PRS in shared or dedicated resource pools, - at least comb sizes (N) 2, 4 are supported. - Comb size 6 is supported at least in dedicated resource pool - FFS: comb size 6 in shared resource pool - Comb size 1 is supported at least in shared resource pool - FFS: comb size 1 in dedicated resource pool - comb sizes (N) > 12 are not supported. - FFS: support of comb sizes (N) of 8, 12. Agreement For SL PRS in shared and dedicated resource pools, - SL PRS patterns with full staggering are supported. - FFS: whether (M,N)=(6,6) is supported - SL PRS patterns with partial staggering are supported at least for the following (M,N) pairs: - (M, 2) with M = {1} - (M, 4) with M = {2} - FFS: constraints on maximum effective comb size - FFS: support of partial staggering for other comb sizes - FFS: Support of SL PRS patterns with M > N at least with full staggering. Agreement TDM-based multiplexing of SL PRS from different UEs in a slot is supported at least for dedicated resource pools. - FFS: TDM-based multiplexing of SL PRS from different UEs in a slot for shared resource pools. - FFS: Details, including resource granularity and relationship to SCI/PSCCH associated with the SL PRS resources, additional AGC symbols. - FFS: restrictions for the configuration of TDM-based multiplexing of SL PRS from different UEs in a slot, if any - FFS: which resource allocation schemes are applicable - FFS: whether or not this is a separate UE capability Agreement The OLPC framework defined for PSSCH/PSCCH is considered as a starting point for OLPC for SL PRS.

This contribution discusses sidelink positioning reference signal. Section 2 discusses sequence design of SL-PRS. Section 3 discusses pattern design of SL-PRS. Section 4 discusses slot structure of SL-PRS. Section 5 discusses power control for SL-PRS and associated PSCCH. Section 6 summarizes the proposals with conclusions.

Sequence design of SL-PRS

The following agreement was achieved in RAN1#112 related to sequence design of SL-PRS [1]:

Agreement SL PRS sequence is generated based on Gold sequence: rn=121-2⋅c2n+j121-2⋅c2n+1, where c(i) is a pseudo-random sequence as defined in Clause 5.2.1 of TS 38.211. Agreement - For SL PRS sequence generation, the pseudo-random sequence c(i) initialization equation is defined as a function of at least: slot number, symbol number, and a parameter nID,seqSL-PRS. - The pseudo-random sequence c(i) initialization equation is based on initialization equation as for DL PRS. Agreement For SL PRS sequence generation, consider at least the following options to define the parameter nID,seqSL-PRS, and select one option: - Option 1: nID,seqSL-PRS is a higher layer configured parameter - Option 2: nID,seqSL-PRS is based on 12 bits CRC of PSCCH associated with the SL PRS transmission - Option 3: based on a combination of higher layer configured parameter from a configured ID list and 12 bits of CRC of PSCCH associated with the SL PRS transmission - Option 5: nID,seqSL-PRS is based on 12bits LSB of destination ID - Option 6: nID,seqSL-PRS is based on 8 bits of source ID + 4 zero bits - Option 7: nID,seqSL-PRS is based on the CRC field of the 2nd SCI associated with SL PRS transmission, if there is a 2nd SCI defined. Agreement Range of the parameter nID,seqSL-PRS is: nID,seqSL-PRS∈0,1,…,4095

According to the above agreements, SL PRS sequence is generated based on Gold sequence:

rn=121-2⋅c2n+j121-2⋅c2n+1,

where c(i) is a pseudo-random sequence as defined in Clause 5.2.1 of TS 38.211.

For SL-PRS sequence generation, the pseudo-random sequence c(i) initialization equation is defined as a function of at least: slot number, symbol number, and a parameter nID,seqSL-PRS. In addition, the pseudo-random sequence c(i) initialization equation is based on initialization equation as for DL PRS.

The Gold sequence generator initialization function cinit of the SL-PRS can distinguish the SL-PRS from different UEs, and maintains the interference randomization and lower sequence correlation between the SL-PRS from different UEs. With regard to the values of the initialization function cinit for pseudo-random sequence cm of SL-PRS, according to the above agreements, the existing initialization function cinit for the sequence of DL-PRS should be used as a starting point.

In addition, DL-PRS sequence generation equation and initialization function are the extensions to the DL CSI-RS sequence generation equation and initialization function to facilitate the use of the same equation to generate both DL-PRS and DL CSI-RS, so as to achieve RE-level reuse between DL-PRS and DL CSI-RS and reduce the workload of standardization and the complexity of UE implementation. Therefore, SL-PRS sequence initialization function should also be an extension to the SL CSI-RS sequence initialization function for the same purpose.

Let us look back the design of initialization function cinit of DL CSI-RS, DL-PRS and SL CSI-RS as follows [2]:

For DL CSI-RS, the pseudo-random sequence generator shall be initialized with:

cinit=210Nsymbslotns,fμ+l+12nID+1+nIDmod231

- at the start of each OFDM symbol where ns,fμ is the slot number within a radio frame,  is the OFDM symbol number within a slot, and  equals the higher-layer parameter scramblingID or sequenceGenerationConfig.

For DL-PRS, the pseudo-random sequence generator shall be initialized with:

cinit=222nID,seqPRS1024+210Nsymbslotns,fμ+l+12nID,seqPRS mod 1024+1+nID,seqPRS mod 1024 mod 231

- where ns,fμ is the slot number, the downlink PRS sequence ID nID,seqPRS∈0,1,…,4095 is given by the higher-layer parameter dl-PRS-SequenceID, and l is the OFDM symbol within the slot to which the sequence is mapped.

For SL CSI-RS, the pseudo-random sequence generator shall be initialized with:

cinit=210Nsymbslotns,fμ+l+12nID+1+nID mod 231

- at the start of each OFDM symbol where ns,fμ is the slot number within a radio frame, l is the OFDM symbol number within a slot, and NID=NIDX mod 210 where the quantity NIDX equals the decimal representation of CRC for the sidelink control information mapped to the PSCCH associated with the CSI-RS according to NIDX=i=0L-1pi∙2L-1-i with p and L given by clause 7.3.2 in TS 38.212 [3].

Based on the above existing design of initialization function cinit of DL CSI-RS, DL-PRS and SL CSI-RS, for SL-PRS generator initialization function cinit, it is reasonable to replace the parameter nID,seqPRS with nID,seqSL-PRS in the above cinit equation of DL-PRS.

Therefore, for SL-PRS, the pseudo-random sequence generator shall be initialized with:

cinit=222nID,seqSL-PRS1024+210Nsymbslotns,fμ+l+12nID,seqSL-PRS mod 1024+1+nID,seqSL-PRS mod 1024 mod 231

- where ns,fμ is the slot number, the SL-PRS sequence ID nID,seqPRS∈0,1,…,4095, and l is the OFDM symbol within the slot to which the sequence is mapped.

Regarding the SL-PRS sequence ID nID,seqPRS, it can be given by the higher-layer parameter (i.e., Option 1 in the above agreement), which just likes DL-PRS, or it can also be calculated from the CRC for the sidelink control information mapped to the PSCCH associated with the SL-PRS (i.e., Option 2 in the above agreement), which likes SL CSI-RS. However, Option 1 is only applicable to the in-coverage scenario, and Option 2 is applicable to the in-coverage and out-of-coverage scenarios, therefore, we prefer to Option 2, i.e., nID,seqSL-PRS is based on 12 bits CRC of PSCCH associated with the SL-PRS transmission.

In addition, the detailed calculation method of nID,seqSL-PRS is given as follows:

nID,seqSL-PRS=NIDY mod 212 where the quantity NIDY equals the decimal representation of CRC for the sidelink control information mapped to the PSCCH associated with the SL-PRS according to NIDY=i=0L-1pi∙2L-1-i with p and L given by clause 7.3.2 in TS 38.212 [3].

Proposal 1: The pseudo-random sequence generator shall be initialized with:

cinit=222nID,seqSL-PRS1024+210Nsymbslotns,fμ+l+12nID,seqSL-PRS mod 1024+1+nID,seqSL-PRS mod 1024 mod 231

- where ns,fμ is the slot number, the SL-PRS sequence ID nID,seqPRS∈0,1,…,4095, and l is the OFDM symbol within the slot to which the sequence is mapped.

Proposal 2: SL-PRS sequence ID nID,seqSL-PRS is calculated from the CRC for the sidelink control information mapped to the PSCCH associated with the SL-PRS.

- nID,seqSL-PRS=NIDY mod 212 where the quantity NIDY equals the decimal representation of CRC for the sidelink control information mapped to the PSCCH associated with the SL-PRS according to NIDY=i=0L-1pi∙2L-1-i with p and L given by clause 7.3.2 in TS 38.212 [3].

Pattern design of SL-PRS

The following agreements were achieved in RAN1#112 related to pattern design of SL-PRS [1]:

Agreement For RE-offset sequence for SL PRS, the RE-offset sequences specified for DL PRS are considered as a starting point. - FFS: Exact RE-offset sequences Agreement - Comb-based multiplexing of SL PRS from different UEs in a slot is supported at least for dedicated resource pools. - FFS: Comb-based multiplexing of SL PRS from different UEs in a slot for shared resource pools. - For comb-based multiplexing of SL PRS from different UEs, support at least the case wherein a single (M,N) value is possible . - FFS: Whether to support comb-based multiplexing of SL PRS from different UEs in a slot using multiple (M,N) values. - FFS: additional restrictions (if any) due to e.g. the impact of synchronization and IBE interference between UEs Agreement For SL PRS in shared or dedicated resource pools, - at least comb sizes (N) 2, 4 are supported. - Comb size 6 is supported at least in dedicated resource pool - FFS: comb size 6 in shared resource pool - Comb size 1 is supported at least in shared resource pool - FFS: comb size 1 in dedicated resource pool - comb sizes (N) > 12 are not supported. - FFS: support of comb sizes (N) of 8, 12. Agreement For SL PRS in shared and dedicated resource pools, - SL PRS patterns with full staggering are supported. - FFS: whether (M,N)=(6,6) is supported - SL PRS patterns with partial staggering are supported at least for the following (M,N) pairs: - (M, 2) with M = {1} - (M, 4) with M = {2} - FFS: constraints on maximum effective comb size - FFS: support of partial staggering for other comb sizes - FFS: Support of SL PRS patterns with M > N at least with full staggering.

With regards to the frequency and time domain pattern within a SL-PRS resource, there are three kinds of patterns as follows:

- Fully staggered SL-PRS pattern: M symbols of SL-PRS with comb-N with M=N and, at each symbol a different RE offset is used, N>1;

- Partially staggered SL-PRS pattern: M symbol(s) of SL-PRS with comb-N, with M<N, at each symbol a different RE offset is used, N>1;

- Unstaggered SL-PRS patterns: M symbol(s) of SL-PRS with comb-N, at each symbol a same RE offset is used, N>1.

Note: N=1 corresponds to full RE mapping pattern.

In Rel-16, the full staggered pattern is supported for the design of DL-PRS and SRS-Pos, in order to enhance the hearability of positioning reference signal. For SL-PRS, its transmission distance is shorter than DL-PRS or SRS-Pos, so the partially staggered pattern should be used as SL-PRS frequency domain pattern, i.e., M symbol(s) of SL-PRS with comb-N, with M<N, at each symbol a different RE offset is used. The number of Comb-N is bigger than the number of symbols-M, so that more users can be can accommodated in one slot. In addition, the full staggered pattern can also be supported to improve the flexibility of SL-PRS configuration.

Comb-based multiplexing of SL-PRS from different UEs in a slot had been agreed to be supported at least for dedicated resource pools in RAN1#112. In our view, the comb-based multiplexing of SL-PRS from different UEs in a slot should not be supported for shared resource pools, in order to simplify the design of SL-PRS and associated PSCCH and ensure the backward compatibility with legacy Rel-16/17 UEs. In addition, for comb-based multiplexing of SL PRS from different UEs, it had been agreed that supporting at least the case wherein a single (M,N) value is possible in RAN1#112. In our view, for comb-based multiplexing of SL-PRS from different UEs in a slot, it should be supported that using multiple (M,N) values for dedicated resource pools, in order to improve the flexibility of SL-PRS configuration.

Proposal 3: Comb-based multiplexing of SL-PRS from different UEs in a slot is not supported for shared resource pools.

Proposal 4: For comb-based multiplexing of SL-PRS from different UEs in a slot, support using multiple (M,N) values for dedicated resource pools.

Regarding the numbers of symbols of SL-PRS within a slot (i.e., M), the numbers of symbols of DL-PRS and SRS-Pos within a slot are defined as {2, 4, 6, 12} and {1, 2, 4, 8, 12} in Rel-16, respectively. We prefer to use the numbers of symbols of DL-PRS within a slot as the baseline, the symbol number 1 can be added to support the configuration of 1-symbol SL-PRS and the symbol number 12 can be removed from the candidate values as the transmission distance of SL-PRS is shorter than DL-PRS. On the other hand, several symbols should be reserved for the transmission of SCI. According to the above discussion, for SL-PRS in shared or dedicated resource pools, the candidate values of number of symbols of SL-PRS within a slot should be M={1, 2, 4, 6}.

Proposal 5: For SL-PRS in shared or dedicated resource pools, symbol numbers of SL-PRS within a slot M={1, 2, 4, 6} should be supported.

Regarding the candidate values of comb size of SL-PRS (i.e., N), the comb sizes of DL-PRS and SRS-Pos are defined as {2. 4, 6, 12} and {2, 4, 8} in Rel-16, respectively. It had been agreed that at least the values N = {2,4} are supported for SL-PRS in shared or dedicated resource pools, comb size 6 is supported at least in dedicated resource pool and comb size 1 is supported at least in shared resource pool in RAN1#112. We prefer to keep the same design on comb size in shared or dedicated resource pools, then for SL-PRS in shared resource pools, comb size 6 should be supported, and for SL-PRS in dedicated resource pools, comb size 1 should be supported. The comb size 8 can be removed from the candidate values as the comb size 6 had been supported. In addition, the comb size 12 can be removed from the candidate values as the transmission distance of SL-PRS is shorter than DL-PRS/SRS-Pos. According to the above discussion, for SL-PRS in shared or dedicated resource pools, comb sizes N={1, 2, 4, 6} should be supported.

Proposal 6: For SL-PRS in shared resource pools, comb size 6 should be supported.

Proposal 7: For SL-PRS in dedicated resource pools, comb size 1 should be supported.

Proposal 8: For SL-PRS in shared or dedicated resource pools, comb sizes of 8, 12 are not supported.

Proposal 9: For SL-PRS in shared or dedicated resource pools, comb sizes N={1, 2, 4, 6} should be supported.

In addition, For SL-PRS in shared and dedicated resource pools, SL-PRS pattern with full staggering of (M,N)=(6,6) should be supported, in order to improve the flexibility of SL-PRS configuration. For SL-PRS in shared and dedicated resource pools, there is no need to support other SL-PRS patterns with partial staggering, except for (M,N)=(1,2) and (2,4), which will align with the legacy design of DL-PRS.

Proposal 10: For SL-PRS in shared and dedicated resource pools, SL-PRS pattern with full staggering of (M,N)=(6,6) should be supported.

Proposal 11: For SL-PRS in shared and dedicated resource pools, there is no need to support other SL-PRS patterns with partial staggering, except for (M,N)=(1,2) and (2,4).

Regarding the RE-Offset sequence within a SL-PRS resource, according to the agreement in RAN1#112, the RE-offset sequences specified for DL PRS are considered as a starting point. Hence, corresponding to M={1, 2, 4, 6} and N={1, 2, 4, 6}, RE-Offset sequence within a SL-PRS resource across the symbols should follow the Table 1 below. An example of the frequency and time domain pattern of SL-PRS resources (M=4 and N=4) can be found in Figure 1.

Table 1: The RE-Offset sequence koffset0, …, koffsetMsymbSL-PRS-1within a SL-PRS resource.

| NcombSL-PRS | koffset0, …, koffsetMsymbSL-PRS-1 |  |  |  |
| --- | --- | --- | --- | --- |
|  | MsymbSL-PRS=1 | MsymbSL-PRS=2 | MsymbSL-PRS=4 | MsymbSL-PRS=6 |
| 1 | 0 | 0, 0 | - | - |
| 2 | 0 | 0, 1 | 0, 1, 0, 1 | 0, 1, 0, 1, 0, 1 |
| 4 | - | 0, 2 | 0, 2, 1, 3 | - |
| 6 | - | - | - | 0, 3, 1, 4, 2, 5 |

Figure 1: Frequency and time domain pattern of SL-PRS resources

Proposal 12: For SL PRS in shared and dedicated resource pools, RE-Offset sequence within a SL-PRS resource across the symbols should follow the table below:

| NcombSL-PRS | koffset0, …, koffsetMsymbSL-PRS-1 |  |  |  |
| --- | --- | --- | --- | --- |
|  | MsymbSL-PRS=1 | MsymbSL-PRS=2 | MsymbSL-PRS=4 | MsymbSL-PRS=6 |
| 1 | 0 | 0, 0 | - | - |
| 2 | 0 | 0, 1 | 0, 1, 0, 1 | 0, 1, 0, 1, 0, 1 |
| 4 | - | 0, 2 | 0, 2, 1, 3 | - |
| 6 | - | - | - | 0, 3, 1, 4, 2, 5 |

It has been supported that the symbols of a SL-PRS resource within a slot are consecutive symbols, and there is an open issue in RAN1#110bis-e that whether consecutive and/or non-consecutive symbols should be supported for shared resource pool. In our view, even for shared resource pool, the symbols of a SL-PRS resource within a slot should also be consecutive symbols, in order to keep the unified design of SL-PRS resource pattern for both dedicated resource pool and shared resource pool.

Proposal 13: The symbols of a SL-PRS resource within a slot should be consecutive symbols for shared resource pool, in order to keep the unified design of SL-PRS resource pattern for both dedicated resource pool and shared resource pool.

Slot structure of SL-PRS

Dedicated resource pool

The following agreement was achieved in RAN1#109-e related to slot structure of SL-PRS [4]:

Agreement For a potential new SL PRS, study further the following - Number of symbol(s) for AGC and/or Rx-Tx turnaround time - Conditions under which AGC training and/or Rx-Tx turnaround time are needed

The following agreement was achieved in RAN1#112 related to slot structure of SL-PRS [1]:

Agreement TDM-based multiplexing of SL PRS from different UEs in a slot is supported at least for dedicated resource pools. - FFS: TDM-based multiplexing of SL PRS from different UEs in a slot for shared resource pools. - FFS: Details, including resource granularity and relationship to SCI/PSCCH associated with the SL PRS resources, additional AGC symbols. - FFS: restrictions for the configuration of TDM-based multiplexing of SL PRS from different UEs in a slot, if any - FFS: which resource allocation schemes are applicable - FFS: whether or not this is a separate UE capability

In NR V2X, an AGC symbol occupies the first or middle symbol of a sidelink slot for the UE to do AGC training to adopt the dynamic range of sidelink signals. In addition, a GP symbol occupies the last or middle symbol of a sidelink slot for the UE to switch between sidelink transmission and sidelink reception. For the SL-PRS, due to the same motivation as SL data transmission/reception, one AGC symbol is needed before the SL-PRS transmission/reception, and one GP symbol is needed after the SL-PRS transmission/reception. For the out-of-coverage scenario, UE-A should allocate the resources to SL-PRS resource #1 based on the sensing results, which including the resource allocation information of SL-PRS resource #2 from UE-B. In order to indicate the resource allocation information of SL-PRS resource #2, UE-B should use one or multiple symbols of SCI before the SL-PRS resource #2 transmission, so as to UE-A can sense the resource allocation information of SL-PRS resource #2 from UE-B via decoding SCI and allocate the resources to its own SL-PRS resource #1. Therefore, SCI which indicates the resource allocation of a SL-PRS resource should occupied at least one symbol before the transmission of the SL-PRS resource.

Regarding the conditions under which AGC training and/or Rx-Tx turnaround time are needed, we prefer to always allocate one AGC symbol before every SL-PRS transmission/reception or SCI+SL-PRS transmission/reception. And one GP symbol should be always allocated after every SL-PRS transmission/reception or SCI+SL-PRS transmission/reception.

As shown in the following Figure 2, the slot is used for SL-PRS transmission. The associated SCI carried by PSCCH occupies four symbols and indicate the resource allocation information of SL-PRS resource #1. SL-PRS resource #1 occupies eight symbols with one AGC symbol before the SCI+SL-PRS resource and one GP symbol after the SL-PRS resource, respectively.

Figure 2: SL-PRS slot structure

Proposal 14: For the SL-PRS in dedicated resource pool, one AGC symbol is needed before the SL-PRS transmission/reception, and one GP symbol is needed after the SL-PRS transmission/reception.

Proposal 15: For dedicated resource pool, the SCI which indicates the resource allocation of a SL-PRS resource should occupy at least one symbol before the transmission of the SL-PRS resource.

Proposal 16: For dedicated resource pool, one AGC symbol should be always allocated before every SL-PRS transmission/reception or SCI+SL-PRS transmission/reception. And one GP symbol should be always allocated after every SL-PRS transmission/reception or SCI+SL-PRS transmission/reception.

In the SCI part, as the frequency-domain resource is sufficient, the structure of PSCCH transmission along with PSSCH transmission is a better choice than that of PSCCH transmission only. Besides, two-stage SCI also should be considered for supporting different cast type in SL positioning. The SCI carried by PSCCH can be used for scheduling the associated PSSCH/SL-PRS transmission and decoding of the associated PSSCH. The PSSCH can carry some information for positioning such as SL measurement reports, assistant information or 2nd-stage SCI. In order to reduce the complexity of blind detection, the number of PRBs for PSCCH or PSCCH/PSSCH should not be larger than the subchannel size and should be fixed in the dedicated resource pool, starting from the lowest PRB of each subchannel as shown in following Figure 3. Besides, the time domain resource set of the PSCCH for multiple UEs should also be unique and fixed in a slot. If multiple positions of PSCCH are introduced in one slot, which will increase the complexity of blind detection.

Proposal 17: In order to reduce the complexity of blind detection:

- The number of PRBs for PSCCH or PSCCH/PSSCH should not be larger than the subchannel size, starting from the lowest PRB of each subchannel.

- The time domain resource set of PSCCH should also be unique and fixed in a slot.

For the restrictions for configuration of TDM-based multiplexing of SL PRS from different UEs in a slot, to reduce resource fragmentation, different UEs should have the same bandwidth and frequency resource location of SL PRS. Both scheme 1 and scheme 2 resource allocation should be applicable for this kind of TDM-based multiplexing, where scheme 2 includes sensing-based RA and random selection. TDM-based multiplexing of SL PRS from different UEs in a slot may need more sophisticated device capabilities, so it should be a separate UE capability.

Proposal 18: To reduce resource fragmentation, TDM-based multiplexing of SL PRS from different UEs in a slot should have the same bandwidth and frequency resource location.

In the SL-PRS resources part, each SL-PRS resource is used for one UE to send SL-PRS, the whole structure may be more flexible than the structures shown in Figure 3. The structure of SL-PRS resource is more like a PSFCH, because of the sequence-based characteristic, the first symbol is a duplication of the second symbol which also can be used for AGC handling. In fact, no matter which symbol the first symbol duplicates, it also can be used for estimating Doppler frequency offset. Therefore, it is better to leave this to UE implementation.

Figure 3: Multiple SL-PRS resources in single slot

Proposal 19: For a SL-PRS transmission, the first symbol is used for AGC handling. The duplication mechanism of first symbol is left to UE implementation.

If the SL-PRS and associated PSCCH are restricted in the same slot in time domain, which may lead to the half-duplex problems because multiple different PSCCHs occupy the same time domain resource set, the UEs only can transmit or receive SL-PRS in a single slot, so the latency will be increased. The standalone PSCCH and cross-slot scheduling for SL-PRS can solve this problem, if cross-slot scheduling is supported, the UEs can operate the TX/RX turnaround of SL-PRS in the same slot as shown in Figure 4. UE3 can receive the SL-PRS scheduled by UE2 in slot 0, and also can transmit its own SL-PRS in slot N.

Figure 4: Cross-slot scheduling for SL-PRS

Proposal 20: For dedicated resource pool, the UE can transmit the SL-PRS in the same or different slot as the associated PSCCH.

Regarding PSCCH/SL-PRS time division multiplexing, if the RB configuration (location and/or size) is different between the PSCCH and SL-PRS, the power spectral density for PSCCH is higher than that for SL-PRS, which could result in a different MPR and AMPR for PSCCH and SL-PRS [5], so a transient period should be needed. Besides, one symbol should be used for Tx-Rx switching time right after PSCCH transmission, because the PSCCH and SL-PRS may be transmitted by different UEs. Therefore, a gap symbol should be introduced for both the transient period and Tx-Rx switching time.

Proposal 21: For dedicated resource pool, one symbol should be used for gap right after PSCCH transmission.

Shared resource pool

For the shared resource pool, the backward compatibility should be ensured. The slot structure design of SL-PRS will not have any influence on the resource allocation of legacy UE(s), so the SL-PRS should be mapped on the resources of PSSCH. Besides, the details of mapping rule for SL-PRS should be discussed further, such as the time-domain location, the pattern design, frequency-domain location and bandwidth.

Figure 5: Slot structure of SL-PRS in shared resource pool

To minimize the impact on resource allocation of legacy UE(s) in the shared resource pool, the PSCCH design for R18 SL positioning UE(s) should keep the same with that of legacy UE(s), and SL-PRS cannot be interfered by other signals. For the time-domain resources location restriction, SL-PRS symbols are not mapped to PSSCH symbols carrying PSSCH DMRS, SCI REs and CSI REs, as shown in Figure 5. The pattern design of SL-PRS may follow the content in section 3. In terms of the frequency-domain resource location and bandwidth, SL-PRS should at least belong to the PSSCH in frequency domain considering the backward compatibility. If a larger bandwidth of SL-PRS than that of PSSCH is supported, this may introduce more issues for the physical layer structure.

For TBS determination in shared resource pool, the mapping rule between data and SL-PRS may lead to different calculation methods, so the overhead of SL-PRS should be considered. If SL-PRS and data can be comb-multiplexed in the same symbol in frequency domain, the resource utilization efficiency will be increased, but the positioning accuracy will be influenced. So it is better to support that SL-PRS is TDMed with PSSCH in symbol-level or slot-level.

Proposal 22: Considering the backward compatibility of SL-PRS in shared resource pool:

- The impact on the resource allocation of legacy UE(s) should be minimized.

- The bandwidth of SL-PRS should be equal to that of associated PSSCH.

- SL-PRS should be TDMed with PSSCH in symbol-level or slot-level.

The following agreement was achieved in RAN1#112 related to slot structure of SL-PRS [1]:

Agreement TDM-based multiplexing of SL PRS from different UEs in a slot is supported at least for dedicated resource pools. - FFS: TDM-based multiplexing of SL PRS from different UEs in a slot for shared resource pools. - FFS: Details, including resource granularity and relationship to SCI/PSCCH associated with the SL PRS resources, additional AGC symbols. - FFS: restrictions for the configuration of TDM-based multiplexing of SL PRS from different UEs in a slot, if any - FFS: which resource allocation schemes are applicable - FFS: whether or not this is a separate UE capability

In our view, the TDM-based multiplexing of SL-PRS from different UEs in a slot should not be supported for shared resource pools, in order to simplify the design of SL-PRS and associated PSCCH and ensure the backward compatibility with legacy Rel-16/17 UEs.

Proposal 23: TDM-based multiplexing of SL PRS from different UEs in a slot is not supported for shared resource pools.

Power control for SL-PRS and associated PSCCH

In this section, the power control for SL-PRS and associated PSCCH will be discussed.

The following agreement was achieved in RAN1#112 related to power control for SL-PRS [1]:

Agreement The OLPC framework defined for PSSCH/PSCCH is considered as a starting point for OLPC for SL PRS.

Power control for SL-PRS

According to the agreement in RAN1#112, the OLPC framework defined for PSSCH/PSCCH is considered as a starting point for OLPC for SL PRS. Therefore, the open-loop power control mechanism of PSSCH in NR V2X in Rel-16 can be reused for SL-PRS, i.e., both downlink pathloss and sidelink pathloss should be considered as the compensated pathloss for the SL-PRS open-loop power control, as shown in Figure 6. There is no need to introduce the closed loop power control for SL-PRS.

Figure 6: SL-PRS power control

Proposal 24: For SL-PRS open-loop power control in Rel-18:

- Both DL pathloss and SL pathloss should be considered as the compensated pathloss for the SL-PRS open-loop power control.

The transmission power of SL-PRS should be determined based on the minimum value between the power calculated according to DL pathloss and the power calculated according to SL pathloss for fractional pathloss compensation, and the transmission power of SL-PRS should be limited by UE maximum output power. If CBR measurement for SL-PRS was introduced, the UE maximum output power based on measured CBR also needs to be considered.

For the open-loop power control mechanism of PSSCH in NR V2X in Rel-16, a UE determines a power PPSSCH(i) for a PSSCH transmission on a resource pool in symbols where a corresponding PSCCH is not transmitted in PSCCH-PSSCH transmission occasion i on active SL BWP b of carrier f as [6]:

PPSSCH(i)=minPCMAX,PMAX,CBR,minPPSSCH,Di,PPSSCH,SL(i) [dBm]

where

-	PCMAX is defined in [TS 38.101-1]

-	PMAX,CBR is determined by a value of sl-MaxTxPower based on a priority level of the PSSCH transmission and a CBR range that includes a CBR measured in slot i-N [TS 38.214]; if sl-MaxTxPower is not provided, then PMAX,CBR=PCMAX;

-  if dl-P0-PSSCH-PSCCH is provided

-	PPSSCH,D(i)=PO,D+10log102μ⋅MRBPSSCHi+αD⋅PLD [dBm]

-	else

-	PPSSCH,D(i)=minPCMAX,PMAX,CBR [dBm]

-	if sl-P0-PSSCH-PSCCH is provided and if a SCI format scheduling the PSSCH transmission includes a cast type indicator field indicating unicast or is SCI format 2-C

-	PPSSCH,SL(i)=PO,SL+10log102μ⋅MRBPSSCHi+αSL⋅PLSL [dBm]

-	else

-	PPSSCH,SL(i)=minPCMAX,PPSSCH,D(i) [dBm]

Based on the above existing open-loop power control mechanism of PSSCH in NR V2X, it is reasonable to obtain the open-loop power control mechanism of SL-PRS as follows:

A UE determines a power PSL-PRS(i) for a SL-PRS transmission on a resource pool in symbols where a corresponding PSCCH is not transmitted in PSCCH-SL-PRS transmission occasion i on active SL BWP b of carrier f as:

PSL-PRS(i)=minPCMAX,PMAX,CBR,minPSL-PRS,Di,PSL-PRS,SL(i) [dBm]

where

-	PCMAX is defined in [TS 38.101-1];

-	PMAX,CBR is determined based on measured CBR of SL-PRS, the details are FFS;

-	if a new higher layer parameter dl-P0-SL-PRS -PSCCH is provided

-	PSL-PRS,D(i)=PO,D+10log102μ⋅MRBSL-PRSi+αD⋅PLD [dBm],

where, PO,D and αD are the parameters of SL-PRS fractional power control according to DL pathloss PLD, MRBSL-PRS(i) is a number of resource blocks for PSCCH-SL-PRS transmission occasion i and μ is a SCS configuration, the details are FFS;

-	else

-	PSL-PRS,D(i)=minPCMAX,PMAX,CBR [dBm]

-	if a new higher layer parameter sl-P0-SL-PRS-PSCCH is provided and if a SCI format scheduling the SL-PRS transmission includes a cast type indicator field indicating unicast

-	PSL-PRS,SL(i)=PO,SL+10log102μ⋅MRBSL-PRSi+αSL⋅PLSL [dBm]

where, PO,SL and αSL are the parameters of SL-PRS fractional power control according to SL pathloss PLSL, MRBSL-PRS(i) is a number of resource blocks for PSCCH-SL-PRS transmission occasion i and μ is a SCS configuration, the details are FFS;

-	else

-	PSL-PRS,SL(i)=minPCMAX,PSL-PRS,D(i) [dBm]

Proposal 25: A UE determines a power PSL-PRS(i) for a SL-PRS transmission on a resource pool in symbols where a corresponding PSCCH is not transmitted in PSCCH-SL-PRS transmission occasion i on active SL BWP b of carrier f as:

PSL-PRS(i)=minPCMAX,PMAX,CBR,minPSL-PRS,Di,PSL-PRS,SL(i) [dBm]

where

-	PCMAX is defined in [TS 38.101-1];

-	PMAX,CBR is determined based on measured CBR of SL-PRS, the details are FFS;

-	if a new higher layer parameter dl-P0-SL-PRS -PSCCH is provided

-	PSL-PRS,D(i)=PO,D+10log102μ⋅MRBSL-PRSi+αD⋅PLD [dBm],

where, PO,D and αD are the parameters of SL-PRS fractional power control according to DL pathloss PLD, MRBSL-PRS(i) is a number of resource blocks for PSCCH-SL-PRS transmission occasion i and μ is a SCS configuration, the details are FFS;

-	else

-	PSL-PRS,D(i)=minPCMAX,PMAX,CBR [dBm]

-	if a new higher layer parameter sl-P0-SL-PRS-PSCCH is provided and if a SCI format scheduling the SL-PRS transmission includes a cast type indicator field indicating unicast

-	PSL-PRS,SL(i)=PO,SL+10log102μ⋅MRBSL-PRSi+αSL⋅PLSL [dBm]

where, PO,SL and αSL are the parameters of SL-PRS fractional power control according to SL pathloss PLSL, MRBSL-PRS(i) is a number of resource blocks for PSCCH-SL-PRS transmission occasion i and μ is a SCS configuration, the details are FFS;

-	else

-	PSL-PRS,SL(i)=minPCMAX,PSL-PRS,D(i) [dBm]

Power control for associated PSCCH

For the open-loop power control mechanism of PSCCH in NR V2X in Rel-16, a UE determines a power PPSCCH(i) for a PSCCH transmission on a resource pool in PSCCH-PSSCH transmission occasion i as

PPSCCH(i)=10log10MRBPSCCHiMRBPSSCHi+PPSSCH(i) [dBm]

where

-	PPSSCH(i) is specified in clause 16.2.1

-	MRBPSCCH(i) is a number of resource blocks for the PSCCH transmission in PSCCH-PSSCH transmission occasion i

-	MRBPSSCH(i) is a number of resource blocks for PSCCH-PSSCH transmission occasion i

Based on the above existing open-loop power control mechanism of PSCCH in NR V2X, it is reasonable to obtain the open-loop power control mechanism of PSCCH associated with SL-PRS transmission as follows:

A UE determines a power PPSCCH, SL-PRS(i) for a PSCCH associated with the SL PRS transmission on a resource pool in PSCCH-PSSCH transmission occasion i as

PPSCCH, SL-PRS(i)=10log10MRBPSCCH, SL-PRSiMRBSL-PRSi+PSL-PRS(i) [dBm]

where

-	PSL-PRS(i) is described in section 5.1.

-	MRBPSCCH, SL-PRS(i) is a number of resource blocks for the PSCCH transmission associated with SL-PRS transmission in PSCCH-SL-PRS transmission occasion i

-	MRBSL-PRS(i) is a number of resource blocks for PSCCH-SL-PRS transmission occasion i.

Proposal 26: A UE determines a power PPSCCH, SL-PRS(i) for a PSCCH associated with the SL PRS transmission on a resource pool in PSCCH-PSSCH transmission occasion i as

PPSCCH, SL-PRS(i)=10log10MRBPSCCH, SL-PRSiMRBSL-PRSi+PSL-PRS(i) [dBm]

where

-	PSL-PRS(i) is described in section 5.1.

-	MRBPSCCH, SL-PRS(i) is a number of resource blocks for the PSCCH transmission associated with SL-PRS transmission in PSCCH-SL-PRS transmission occasion i.

-	MRBSL-PRS(i) is a number of resource blocks for PSCCH-SL-PRS transmission occasion i.

Conclusions

In this contribution, we discuss the design of SL positioning reference signal, and give the following proposals:

Proposal 1: The pseudo-random sequence generator shall be initialized with:

cinit=222nID,seqSL-PRS1024+210Nsymbslotns,fμ+l+12nID,seqSL-PRS mod 1024+1+nID,seqSL-PRS mod 1024 mod 231

- where ns,fμ is the slot number, the SL-PRS sequence ID nID,seqPRS∈0,1,…,4095, and l is the OFDM symbol within the slot to which the sequence is mapped.

Proposal 2: SL-PRS sequence ID nID,seqSL-PRS is calculated from the CRC for the sidelink control information mapped to the PSCCH associated with the SL-PRS.

- nID,seqSL-PRS=NIDY mod 212 where the quantity NIDY equals the decimal representation of CRC for the sidelink control information mapped to the PSCCH associated with the SL-PRS according to NIDY=i=0L-1pi∙2L-1-i with p and L given by clause 7.3.2 in TS 38.212 [3].

Proposal 3: Comb-based multiplexing of SL-PRS from different UEs in a slot is not supported for shared resource pools.

Proposal 4: For comb-based multiplexing of SL-PRS from different UEs in a slot, support using multiple (M,N) values for dedicated resource pools.

Proposal 5: For SL-PRS in shared or dedicated resource pools, symbol numbers of SL-PRS within a slot M={1, 2, 4, 6} should be supported.

Proposal 6: For SL-PRS in shared resource pools, comb size 6 should be supported.

Proposal 7: For SL-PRS in dedicated resource pools, comb size 1 should be supported.

Proposal 8: For SL-PRS in shared or dedicated resource pools, comb sizes of 8, 12 are not supported.

Proposal 9: For SL-PRS in shared or dedicated resource pools, comb sizes N={1, 2, 4, 6} should be supported.

Proposal 10: For SL-PRS in shared and dedicated resource pools, SL-PRS pattern with full staggering of (M,N)=(6,6) should be supported.

Proposal 11: For SL-PRS in shared and dedicated resource pools, there is no need to support other SL-PRS patterns with partial staggering, except for (M,N)=(1,2) and (2,4).

Proposal 12: For SL PRS in shared and dedicated resource pools, RE-Offset sequence within a SL-PRS resource across the symbols should follow the table below:

| NcombSL-PRS | koffset0, …, koffsetMsymbSL-PRS-1 |  |  |  |
| --- | --- | --- | --- | --- |
|  | MsymbSL-PRS=1 | MsymbSL-PRS=2 | MsymbSL-PRS=4 | MsymbSL-PRS=6 |
| 1 | 0 | 0, 0 | - | - |
| 2 | 0 | 0, 1 | 0, 1, 0, 1 | 0, 1, 0, 1, 0, 1 |
| 4 | - | 0, 2 | 0, 2, 1, 3 | - |
| 6 | - | - | - | 0, 3, 1, 4, 2, 5 |

Proposal 13: The symbols of a SL-PRS resource within a slot should be consecutive symbols for shared resource pool, in order to keep the unified design of SL-PRS resource pattern for both dedicated resource pool and shared resource pool.

Proposal 14: For the SL-PRS in dedicated resource pool, one AGC symbol is needed before the SL-PRS transmission/reception, and one GP symbol is needed after the SL-PRS transmission/reception.

Proposal 15: For dedicated resource pool, the SCI which indicates the resource allocation of a SL-PRS resource should occupy at least one symbol before the transmission of the SL-PRS resource.

Proposal 16: For dedicated resource pool, one AGC symbol should be always allocated before every SL-PRS transmission/reception or SCI+SL-PRS transmission/reception. And one GP symbol should be always allocated after every SL-PRS transmission/reception or SCI+SL-PRS transmission/reception.

Proposal 17: In order to reduce the complexity of blind detection:

- The number of PRBs for PSCCH or PSCCH/PSSCH should not be larger than the subchannel size, starting from the lowest PRB of each subchannel.

- The time domain resource set of PSCCH should also be unique and fixed in a slot.

Proposal 18: To reduce resource fragmentation, TDM-based multiplexing of SL PRS from different UEs in a slot should have the same bandwidth and frequency resource location.

Proposal 19: For a SL-PRS transmission, the first symbol is used for AGC handling. The duplication mechanism of first symbol is left to UE implementation.

Proposal 20: For dedicated resource pool, the UE can transmit the SL-PRS in the same or different slot as the associated PSCCH.

Proposal 21: For dedicated resource pool, one symbol should be used for gap right after PSCCH transmission.

Proposal 22: Considering the backward compatibility of SL-PRS in shared resource pool:

- The impact on the resource allocation of legacy UE(s) should be minimized.

- The bandwidth of SL-PRS should be equal to that of associated PSSCH.

- SL-PRS should be TDMed with PSSCH in symbol-level or slot-level.

Proposal 23: TDM-based multiplexing of SL PRS from different UEs in a slot is not supported for shared resource pools.

Proposal 24: For SL-PRS open-loop power control in Rel-18:

- Both DL pathloss and SL pathloss should be considered as the compensated pathloss for the SL-PRS open-loop power control.

Proposal 25: A UE determines a power PSL-PRS(i) for a SL-PRS transmission on a resource pool in symbols where a corresponding PSCCH is not transmitted in PSCCH-SL-PRS transmission occasion i on active SL BWP b of carrier f as:

PSL-PRS(i)=minPCMAX,PMAX,CBR,minPSL-PRS,Di,PSL-PRS,SL(i) [dBm]

where

-	PCMAX is defined in [TS 38.101-1];

-	PMAX,CBR is determined based on measured CBR of SL-PRS, the details are FFS;

-	if a new higher layer parameter dl-P0-SL-PRS -PSCCH is provided

-	PSL-PRS,D(i)=PO,D+10log102μ⋅MRBSL-PRSi+αD⋅PLD [dBm],

where, PO,D and αD are the parameters of SL-PRS fractional power control according to DL pathloss PLD, MRBSL-PRS(i) is a number of resource blocks for PSCCH-SL-PRS transmission occasion i and μ is a SCS configuration, the details are FFS;

-	else

-	PSL-PRS,D(i)=minPCMAX,PMAX,CBR [dBm]

-	if a new higher layer parameter sl-P0-SL-PRS-PSCCH is provided and if a SCI format scheduling the SL-PRS transmission includes a cast type indicator field indicating unicast

-	PSL-PRS,SL(i)=PO,SL+10log102μ⋅MRBSL-PRSi+αSL⋅PLSL [dBm]

where, PO,SL and αSL are the parameters of SL-PRS fractional power control according to SL pathloss PLSL, MRBSL-PRS(i) is a number of resource blocks for PSCCH-SL-PRS transmission occasion i and μ is a SCS configuration, the details are FFS;

-	else

-	PSL-PRS,SL(i)=minPCMAX,PSL-PRS,D(i) [dBm]

Proposal 26: A UE determines a power PPSCCH, SL-PRS(i) for a PSCCH associated with the SL PRS transmission on a resource pool in PSCCH-PSSCH transmission occasion i as

PPSCCH, SL-PRS(i)=10log10MRBPSCCH, SL-PRSiMRBSL-PRSi+PSL-PRS(i) [dBm]

where

-	PSL-PRS(i) is described in section 5.1.

-	MRBPSCCH, SL-PRS(i) is a number of resource blocks for the PSCCH transmission associated with SL-PRS transmission in PSCCH-SL-PRS transmission occasion i.

-	MRBSL-PRS(i) is a number of resource blocks for PSCCH-SL-PRS transmission occasion i.

References

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #112.

- TS 38.211, “NR; Physical channels and modulation (Release 17)”, V17.4.0 (2022-12).

- TS 38.212, “NR; Multiplexing and channel coding (Release 17)”, V17.5.0 (2023-03).

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #109-e.

- R4-1902514, “Reply LS on NR UE transient period in sidelink channels”, 3GPP TSG RAN WG4 Meeting #90.

- TS 38.213, “NR; Physical layer procedures for control (Release 17)”, V17.5.0 (2023-03).