3GPP TSG RAN WG1 #112bis-e                                          R1-2302712

e-meeting, April 17th – April 26th, 2023

Source:	CATT

Title:	Further discussion on Low Power High Accuracy Positioning

Agenda Item:	9.5.3

Document for:	Discussion and Decision

Introduction

In RAN#98 meeting, a new work item on expanded and improved NR positioning was approved [1]. In RAN1#112, a good progress was achieved for the LPHAP objective as shown in the following agreements [2].

Agreement For SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state, support the following: - An SRS positioning validity area consists of cells configured in the same band and the same carrier, and the following parameters with respect to BWP information of SRS for positioning configuration are commonly applied across cells within the validity area: - BWP parameters - locationAndBandwidth - subcarrierSpacing - cyclicPrefix Agreement For SRS for positioning configuration in multiple cells for a UE in RRC_INACTIVE state, at least the following parameters in SRS for positioning configuration are commonly configured across cells within the validity area: - srs-PosConfig - SRS-PosResourceSet - srs-PosResourceSetId - srs-PosResourceIdList - resourceType - SRS-PosResource - srs-PosResourceId - transmissionComb - resourceMapping - freqDomainShift - freqHopping - groupOrSequenceHopping-r16 - resourceType - FFS: whether sequenceId is configured commonly across cells or per cell Agreement From RAN1 perspective, it is feasible to configure SRS positioning validity area-specific TA timer (e.g., with larger values) for a UE in RRC_INACTIVE state. Details can be up to RAN2. - For TA validation, use of area-specific RSRP change threshold is feasible - FFS: which RS is the reference RS for the RSRP change threshold Agreement For the determination of UL timing to transmit SRS for positioning by UEs in RRC_INACTIVE state within the SRS positioning validity area: - For the UL timing advance, further study the following options, including the DL reference timing for each option: - Option 1: UE maintains the TA obtained from the last serving cell within the validity area - Option 2: UE autonomously adjusts the TA - FFS: how the UE adjusts the TA, e.g. up to UE implementation, or based on the TA from the last serving cell and the DL time difference measurement of SSBs from the last serving cell and the new camping cell. - FFS: whether there is RAN1 specification impact - Option 3: UE maintains multiple TA values, e.g. UE obtains TA using RACH Agreement For the spatial relation of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state, further study the following options: - Option 1: Spatial relation information is absent in the configuration - FFS: different approaches for down selection at least include the following: - 1a: UE transmits SRS for positioning resources using different spatial domain transmission filter - 1b: UE transmits SRS for positioning resource(s) using a fixed spatial domain transmission filter - FFS criterion on UE determination of the fixed spatial domain transmission filter (e.g., up to UE implementation, based on a selected SSB of the camping cell, based on the configured path-loss RS such as SSB, etc.) - Option 2: Spatial relation information is provided in the configuration - FFS details on the configuration and corresponding UE behavior, including whether the information is configured for all or subset of cells - FFS signaling to configure the spatial relation information, e.g., via SRS activation message. - Note: UE power consumption needs to be taken into account - FFS validity criteria of spatial relation for the configured RS and UE behavior if it determines that the validity criteria of spatial relation for the configured RS is not met, if any, to avoid frequent RRC connection for SRS (re)configuration. Agreement For the power control of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state, further study the following options: - Option 1: Pathloss RS is absent in the configuration. - FFS criterion on UE determination of pathloss RS (e.g., up to UE implementation, UE selects an SSB as the pathloss RS based on DL measurements from multiple SSBs of cells within the validity area, etc.). - Option 2: Pathloss RS is provided in the configuration - FFS details on configuration (e.g., pathloss RS is configured as a cell-agnostic DL RS, pathloss RS is configured as a fixed SSB within the validity area, etc.), including whether the information is configured for all or a subset of cells within the validity area - FFS signaling to configure the pathloss RS, e.g., via SRS activation message. - Note: UE Power consumption needs to be taken into account - FFS: Whether p0 and alpha can be commonly configured across cells within the validity area. - FFS validity criteria of pathloss RS and UE behavior if it determines that the validity criteria of pathloss RS is not met., if any, to avoid frequent RRC connection for SRS (re)configuration.

In this contribution, we further discuss potential enhancements for LPHAP.

SRS in multiple cells

For LPHAP, area-specific SRS-Pos configuration, i.e., pre-configuring SRS-Pos for multiple cells, is to be introduced. In our opinion, a cell list could be pre-configured by the serving cell as the validity area. In this section, we discuss some key aspects related to the SRS-Pos configuration, including the interferences due to SRS collision, UL timing, spatial relation information, and power control.

Interferences by SRS collision

In Rel-17, UE-specific SRS-Pos configuration in the serving cell is supported by RRC release signaling. Similarly, the above multiple SRS-Pos configurations could also be UE-specific. SRS-Pos resource allocation would be coordinated among multiple cells within the same validity area. And these SRS-Pos resources would be preserved by the NW. Therefore, the interference between the pre-configured SRS-Pos and UL transmission in the new camped-on cell could be avoided. In addition, if the SRS-Pos receiving timing misalignment between the serving cell and the new camped-on cell is within one CP, there will be no inter-symbol interference on UL transmission of the new camped-on cell. This depends on the accuracy of TA.

Observation 1: If multiple cells within the valid area coordinate to preserve the pre-configured SRS resources, the UL transmission interference can be avoided provided that the timing misalignment between the serving cell and the new camped-on cell is within one CP.

UL timing

In RAN1#112 agreement, the agreement was made to study three options for the determination of UL timing to transmit SRS-Pos.

Agreement For the determination of UL timing to transmit SRS for positioning by UEs in RRC_INACTIVE state within the SRS positioning validity area: - For the UL timing advance, further study the following options, including the DL reference timing for each option: - Option 1: UE maintains the TA obtained from the last serving cell within the validity area - Option 2: UE autonomously adjusts the TA - FFS: how the UE adjusts the TA, e.g. up to UE implementation, or based on the TA from the last serving cell and the DL time difference measurement of SSBs from the last serving cell and the new camping cell. - FFS: whether there is RAN1 specification impact - Option 3: UE maintains multiple TA values, e.g. UE obtains TA using RACH

According to TS 22.104 [3], LPHAP UEs are applicable in a flexible modular assembly area, i.e. tracking of workpiece (indoor and outdoor) in assembly area and warehouse. For these scenarios, the geometric coverage of different cells is probably to be similar. For example, in InF scenario used in the Rel-17 simulation evaluation, gNBs are equally distributed in the rectangle area. For such deployment scenarios, UE could maintain the TA obtained from the last serving cell within the validity area (Option 1). However, it is too restrictive to require all the cells within the validity area to share the same geometric coverage. For those cells with different geometric coverage, it should allow a UE to autonomously adjust the TA (Option 2).  For UE in RRC_INACTIVE state, SSBs from multiple cells are available.  UE would adjust the TA value by itself according to the RSTD of SSBs of the two cells. In general, the NW has the knowledge of the geometric coverage of each cell within the validity area. For more flexibility and the supportive of more general scenario, a combination of Option 1 and Option 2 should be supported for the TA determination, where each cell is pre-configured with a TA acquisition identifier. The TA acquisition identifier corresponds to Option 1 or Option2. Alternatively, it could also be a label related to the geometric coverage. Taking Figure 1 as an example, cells 3/6/11 have different geometric coverage to the other cells. If the last serving cell is cell 1 and the new camping cell is cell 2, UE would maintain the TA obtained from the last serving cell. If the last serving cell is cell 2 and the new camping cell is cell 6, UE autonomously adjusts the TA.

Figure 1：TA acquisition identifier configuration for each cell within the validity area

Proposal 1: For the determination of UL timing to transmit SRS for positioning by UEs in RRC_INACTIVE state within the SRS positioning validity area, support both of the following options:

- Option 1: UE maintains the TA obtained from the last serving cell within the validity area

- Option 2: UE autonomously adjusts the TA

A TA acquisition identifier can be included in cell-specific pre-configuration to specify which option should be used.

Spatial relation information

In RAN1#112, the following agreement was made on the spatial relation of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state.

Agreement For the spatial relation of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state, further study the following options: - Option 1: Spatial relation information is absent in the configuration - FFS: different approaches for down selection at least include the following: - 1a: UE transmits SRS for positioning resources using different spatial domain transmission filter - 1b: UE transmits SRS for positioning resource(s) using a fixed spatial domain transmission filter - FFS criterion on UE determination of the fixed spatial domain transmission filter (e.g., up to UE implementation, based on a selected SSB of the camping cell, based on the configured path-loss RS such as SSB, etc.) - Option 2: Spatial relation information is provided in the configuration - FFS details on the configuration and corresponding UE behavior, including whether the information is configured for all or subset of cells - FFS signaling to configure the spatial relation information, e.g., via SRS activation message. - Note: UE power consumption needs to be taken into account - FFS validity criteria of spatial relation for the configured RS and UE behavior if it determines that the validity criteria of spatial relation for the configured RS is not met, if any, to avoid frequent RRC connection for SRS (re)configuration.

Spatial relation information is determined by the location of UE. In Rel-16, multiple SRS-Pos resources could be configured for a UE, where different resources are associated with different SpatialRelationInfo. In this way, SRS beam sweeping could be achieved to guarantee the reception of SRS at both serving cell and neighboring cells. But for Rel-18 LPHAP, if multiple SRS-Pos resources are configured for each cell, a lot of SRS-Pos resources need to be preserved by the NW, which results in inefficient resource utilization. In our opinion, when UE moves and/or performs cell reselection, spatial relation may change accordingly and the spatial relation information needs to be updated. Some enhancements on spatial relation configuration have to be further studied.

If the UE moves to a new cell and multiple SRSs are configured, the UE can transmit multiple beams in a beam scanning manner, which is received by the base station in a fixed direction, so spatial relation information can be decided.

Proposal 2: Support “Option 1a: Spatial relation information is not configured, and UE transmits multiple SRS for positioning resources using different transmission filters”.

Power control

In RAN1#112, the agreement was made to study two options for the determination of configurations of power control related parameters.

Agreement For the power control of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state, further study the following options: - Option 1: Pathloss RS is absent in the configuration. - FFS criterion on UE determination of pathloss RS (e.g., up to UE implementation, UE selects an SSB as the pathloss RS based on DL measurements from multiple SSBs of cells within the validity area, etc.). - Option 2: Pathloss RS is provided in the configuration - FFS details on configuration (e.g., pathloss RS is configured as a cell-agnostic DL RS, pathloss RS is configured as a fixed SSB within the validity area, etc.), including whether the information is configured for all or a subset of cells within the validity area - FFS signaling to configure the pathloss RS, e.g., via SRS activation message. - Note: UE Power consumption needs to be taken into account - FFS: Whether p0 and alpha can be commonly configured across cells within the validity area. - FFS validity criteria of pathloss RS and UE behavior if it determines that the validity criteria of pathloss RS is not met., if any, to avoid frequent RRC connection for SRS (re)configuration.

For Option 1, pathloss RS is absent in the configuration, UE selects an SSB as the pathloss RS based on DL measurements from multiple SSBs of cells within the validity area. But each cell corresponds to multiple SSBs, which SSB should be determined as the pathloss RS needs further discussion. The difference of the pathloss RS is small in neighboring cells, and there is no need to get multiples SSB from each cell, which may cause increased power consumption. For Option 2, it is difficult for multiple cells to use one pathloss RS if there are many cells in the validity area. The validity area could be further divided into sub areas and then configure the same pathloss RS within each sub area. For example, the 16 cells shown in the figure below are divided into 4 sub regions, and each region is configured with the same pathloss RS. In our view, this option should be supported.

Figure 2: Pathloss RS configuration for each cell within the validity area

Proposal 3: For the power control of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state, the following option can be supported:

- (Option 2) Pathloss RS is provided in the configuration. In addition, a validity area can be divided into multiple sub-areas and a pathloss RS can be configured for each sub-area.

SRS request procedures

In RAN1#111, the following agreement was achieved [4]:

Agreement - For the conclusion section of the TR: - For UL and DL+UL positioning for UEs in RRC_INACTIVE state, the enhancements on SRS for positioning in order to avoid frequent RRC connection for SRS (re)configuration is recommended for normative work. - For the potential specification impact section of the TR: - For UL and DL+UL positioning for UEs in RRC_INACTIVE state, the details of solutions for enhancements on SRS for positioning to avoid frequent RRC connection for SRS (re)configuration can be further discussed during normative work, which may include but are not limited to one or combinations of the following: - SRS for positioning configurations in multiple cells. - Note: Details including issues such as interference, timing advance, spatial relation information, pathloss reference and common SRS parameters across multiple cells can be further discussed during normative work. - Pre-configuration of one or multiple SRS for positioning configurations. - SRS for positioning activation/request procedure(s).

As mentioned above, when a UE moves out of the validity area, the SRS configuration would be released. Then, the UE has to request new SRS-Pos configurations for the subsequent transmission. The SRS-Pos request could be sent by a new type of RACH procedure. One alternative is to define new preambles and corresponding RACH Occasions (ROs) to discriminate with that of legacy PRACH. When the gNB receives such new preamble, SRS-Pos configurations would be indicated to the UE in Msg2 for 4-steps RACH procedure or MsgB for 2-steps RACH procedure. Another alternative with less spec impact is to carry SRS-Pos request in Msg3 or MsgA. After the gNB decodes the message, new SRS-Pos configurations would be indicated to the UE in Msg4 or MsgB. The validity area needs to be configured to UE together with the SRS-Pos configuration. However, before performing the positioning procedure, UE may have no valid location information. Hence the validity area can only be defined by the cell information, e.g. a list of cell or Tracking Area Code (TAC). When cell reselection happens, UE will compare the serving cell ID with the cell information of the valid area. If the serving cell ID is not contained, the UE may need to request SRS-Pos from the network.

Proposal 4: Support introducing a new RACH procedure for a UE to obtain SRS-Pos configurations:

- E.g., new SRS-Pos configurations could be indicated to the UE in Msg2, Msg 4 or Msg B.

Conclusions

In this contribution, we discuss the SRS configuration enhancements for LPHAP, and have the following observations and proposals:

Observation 1: By preserving the pre-configured SRS resources, which are determined by coordinating among multiple cells within the validity area, interference to UL transmission could be eliminated as long as the timing misalignment between the serving cell and the new camped-on cell is within one CP.

Proposal 1: For the determination of UL timing to transmit SRS for positioning by UEs in RRC_INACTIVE state within the SRS positioning validity area, support both of the following options:

- Option 1: UE maintains the TA obtained from the last serving cell within the validity area

- Option 2: UE autonomously adjusts the TA

A TA acquisition identifier can be included in cell-specific pre-configuration to specify which option should be used.

Proposal 2: Support “Option 1a: Spatial relation information is not configured, and UE transmits multiple SRS for positioning resources using different transmission filters”.

Proposal 3: For the power control of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state, the following option can be supported:

- (Option 2) Pathloss RS is provided in the configuration. In addition, a validity area can be divided into multiple sub-areas and a pathloss RS can be configured for each sub-area.

Proposal 4: Support introducing a new RACH procedure for a UE to obtain SRS-Pos configurations:

- E.g., new SRS-Pos configurations could be indicated to the UE in Msg2, Msg 4 or Msg B.

References

- RP-223549, “New WID on Expanded and Improved NR Positioning”, 3GPP TSG RAN Meeting #98.

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #112.

- 3GPP TS 22.104, Service requirements for cyber-physical control applications in vertical domains, V19.0.0, March, 2023.

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #111.