3GPP TSG RAN WG1 #112bis-e		R1-2302717

e-Meeting, April 17th – April 26th, 2023

Source:	CATT

Title:	DTX/DRX for Network Energy Saving

Agenda Item:	9.7.2

Document for:	Discussion and Decision

Introduction

During the study in the network energy saving SI phase, the techniques in time domain mainly aim to reduce the power consumption for dynamic part by trying to shut down more symbols on one or more carriers to achieve BS micro sleep, and the static power part by enlarging the interval between the contiguous active transmission/reception occasions to achieve BS light/deep sleep. From the study report of network energy saving in TR 38.864 [1], the potential enhancement technology in time domain, e.g. cell DTX/DRX, can reduce power consumption, in which the cell may have no transmission/reception or only keep limited transmission/reception.

The scope of Work Item “Network energy savings for NR” [2] includes the enhancement technology in time domain, i.e. cell DTX/DRX in the following items:

Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3] - Note: No change for SSB transmission due to cell DTX/DRX. - Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided.

In RAN1#112, the following agreements were made [3].

Agreement - RAN1 continues discussion on the at least following physical layer related aspects of cell DTX/DRX aspects - physical layer signals/channels and procedures expected to be impacted during non-active periods of cell DTX/DRX - consider impact to at least KPIs from the SI when physical layers/signals/channels are impacted by cell DTX/DRX - Further discussions on other aspects are not precluded Agreement At least the following candidate signals/channels for connected mode UEs, which the UE may be expected to not transmit or receive during non-active periods of cell DTX/DRX, are considered from RAN1 perspective for further discussion. The exact set of signals/channels that the UE may be expected to not transmit or receive is FFS. - DL - Periodic/Semi-persistent CSI-RS (including TRS) - PRS - PDCCH scrambled with UE specific RNTI - PDCCH in Type-3 CSS - SPS-PDSCH - UL - SR - Periodic/Semi-persistent CSI report - Periodic/Semi-persistent SRS - CG-PUSCH Other signals/channels are not precluded

In this contribution, we mainly discuss the alignment of cell DTX/DRX and UE C-DRX in Section 2. The gNB and UE behaviors in cell DTX/DRX are provided in Section 3. The detailed mechanism for cell DTX/DRX is proposed in Section 4. Finally, the cell DTX/DRX for CA is discussed in Section 5.

The alignment of cell DTX/DRX transmission and UE C-DRX

The cell DTX/DRX technique can keep the cell in sleeping mode for a relatively long time without frequent waking-up, which is beneficial to network energy saving. When system load is low and less number of UEs access the system, the staggering C-DRX configuration for system load balancing becomes unnecessary. Instead, the cell could reduce the energy consumption with the DTX transmission in low system load state by allocating same set of C-DRX configuration for all UEs, which including DTX active time and DTX non-active time. During DTX non-active time, the cell could go to sleep to reduce energy consumption.

For Rel-18 RRC CONNECTED mode UEs, DTX parameters, such as the DTX cycle, the DTX active offset and the active duration, etc. can be configured by gNB through high layer parameters. The cell DTX cycle could be aligned with that of UE C-DRX. In order not to affect the reception of the DCI format 2_6 outside the active time of UE C-DRX, the cell DTX active time should start earlier than ps-offset the time where the UE monitoring DCI format 2_6 [4], even earlier than its preparation time for the detection of DCI format 2_6. The cell DTX active time would completely cover the DRX-ON of UEs as shown in Figure 1.

Figure 1: The cell DTX/DRX structure

Proposal 1: The cell DTX/DRX parameters could be configured to ensure the alignment between the cell DTX/DRX and UE C-DRX, i.e. the cell DTX active time should completely capture the DRX-ON of UEs and at least cover the reception window of DCI format 2_6 in order not to affect the legacy UEs.

For the legacy UE, if C-DRX parameters are configured, UE stays in the active time during the drx-onDurationTimer running, which is configured by high layer signaling. The cell DTX active time should at least capture the UE DRX-ON duration configured by drx-onDurationTimer. However, if a UE is scheduled a new UL/DL data or expects a retransmission data, the UE still stays in the active time when drx-InactivityTimer, drx-RetransmissionTimerDL or drx-RetransmissionTimerUL is running, the start point of these timers is related to dynamic scheduling. If the UE’s active time is extended by any of drx-InactivityTimer, drx-RetransmissionTimerDL or drx-RetransmissionTimerUL, the cell may enter DTX non-active time before these timers expired. Therefore, if UE DRX-ON is extended beyond the end of cell DTX active time, UE behaviors should be specified.

Proposal 2: If the cell DTX/DRX is applied, the UE behaviors should be specified when the cell DTX active time ends earlier than the UE DRX-ON extended by any of drx-InactivityTimer, drx-RetransmissionTimerDL or drx-RetransmissionTimerUL.

The gNB/UE behaviors in cell DTX/DRX transmission

The cell DTX/DRX mechanism should not affect the IDLE/INACTIVE UEs. The gNB could transmit SSB for UE to perform synchronization and RRM measurement in order that the IDLE/INACTIVE UEs can camp on the cell. In RAN2#121 [5], it was agreed that there will be no impact to RACH, paging and SIBs in IDLE/INACTIVE for both gNB and Rel-18 and legacy UEs. The IDLE/INACTIVE UEs can perform paging and RACH procedure to access the cell, and the paging and RACH procedure are not affected by the cell DTX/DRX since the paging occasions and RACH resources are configured by the network.

For legacy RRC_CONNECTED mode UE, the UE performs the PDCCH monitoring with the CRC scrambled by the specific RNTI (e.g., P-RNTI, SI-RNTI, etc.), reception of the configured PDSCH, transmission and reception without dynamic scheduling and performing channel tracking operation outside the C-DRX active time. The cell DTX/DRX behavior definitions may affect the RRC_CONNECTED UE behavior and transmission performance within cell DTX/DRX non-active time. The cell DTX/DRX behavior definition has been discussed in RAN2 [6] and four examples for defining the cell DTX/DRX were agreed in RAN2#119bis-e. One example 1 is that gNB is expected to turn off all transmissions and receptions for data traffic and reference signals during cell DTX/DRX non-active time. In this example, the reference signal contains SSB, which conflicts with the WID and should be excluded. The remaining three examples can be further discussed. For the convenience of discussion in this contribution, we use option 1/2/3 to represent the remaining three examples and the cell DTX/DRX behavior definitions are as following:

- Cell DTX/DRX active time: gNB performs the same transmission/reception behaviors as Rel-17.

- Cell DTX/DRX non-active time:

  - Option 1: gNB is expected to turn off its transmission/reception only for data traffic during cell DTX/DRX non-active time (i.e., gNB will still transmit/receive reference signals).

  - Option 2: gNB is expected to turn off its dynamic data transmission/reception during cell DTX/DRX non-active time (i.e., gNB is expected to still perform transmission/reception in periodic resources, including SPS, CG-PUSCH, SR, RACH, and SRS).

  - Option 3: gNB is expected to only transmit reference signals (e.g., CSI-RS for measurement).

The main difference between option 1 and option 2 is whether to support transmission/reception in periodic resources, the option 2 only turns off dynamic scheduling, so the network energy saving gain of option 2 is lower than that of option 1. Compared to option 1, the network energy saving gain of option 3 is larger, and the UE only can perform synchronization and measurement by discontinuously detecting SSB and CSI-RS. The gNB/UE behaviors in cell DTX/DRX non-active time are summarized in Table 1.

Table 1: gNB/ RRC_CONNECTED UE behaviours during cell DTX/DRX non-active time

| Options | gNB behaviors | RRC_CONNECTED UE behaviors |
| --- | --- | --- |
| Option 1 | gNB is expected to turn off its transmission/reception only for data traffic during cell DTX/DRX non-active time(i.e., gNB will still transmit/receive reference signals). | UE transmits/receives reference signals for synchronization, measurement and so on. Legacy/Rel-18 UE cannot transmit/receive data in periodic resources. |
| Option 2 | gNB is expected to turn off its dynamic data transmission/reception during cell DTX/DRX non-active time(i.e., gNB is expected to still perform transmission/reception in periodic resources, including SPS, CG-PUSCH, SR, RACH, and SRS). | UE transmits/receives reference signals for synchronization, measurement and so on. Legacy/Rel-18 UE transmits/receives data in periodic resources. |
| Option 3 | gNB is expected to only transmit reference signals (e.g., CSI-RS for measurement). | UE performs synchronization and measurement by detecting SSB and CSI-RS. |

For legacy RRC_CONNECTED UEs, the UE can perform transmission/reception in periodic resources outside the C-DRX active time. If the option 1 of cell DTX/DRX behavior is supported, the legacy RRC_CONNECTED UE cannot recognize the cell DTX/DRX parameters. The UE can wake up any time and will continue to transmit/receive data in periodic resources. Thus, the legacy RRC_CONNECTED UE’s power consumption and transmission performance are degraded. In order not to affect the performance of legacy RRC_CONNECTED UE in UE power consumption and network access, one possible solution is to have the full subset of the cell DTX/DRX active time overlapped with the UE’s periodic resources based on gNB implementation. Another possible solution is that option 2 of cell DTX/DRX behavior is supported, i.e. the legacy RRC_CONNECTED UE can perform the transmission and reception in periodic resources without the restriction by cell DTX/DRX.

Proposal 3: If the cell DTX/DRX is applied, the following options can be considered in order not to impact on the transmission/reception in periodic resources for legacy RRC_CONNECTED UE:

- Option 1: The periodic resources outside the C-DRX active time are overlapped with the cell DTX/DRX active time based on gNB implementation.

- Option 2: Specify that transmission and reception in periodic resources are not affected by cell DTX/DRX.

For Rel-18 RRC_CONNECTED UEs, if the option 1 of cell DTX/DRX behavior is supported, Rel-18 RRC_CONNECTED UE will not transmit/receive data in periodic resources outside the C-DRX active time when the cell DTX/DRX parameters are configured. The delay of data transmission/reception in periodic resources could increase. If the option 2 of cell DTX/DRX behavior is supported, the data transmission/reception in periodic resources will not be impacted but the network energy saving gain will be reduced. If the service of Rel-18 RRC_CONNECTED UE is not periodic or delay-sensitive service such as XR or URLLC, the delay of data transmission/reception in periodic resources may be acceptable, and then the cell DTX/DRX prioritizes the transmission/reception in periodic resources to maximize the network energy saving gain, i.e. data transmission/reception in periodic resources is not allowed in DTX/DRX non-active time.

Proposal 4: If the service of Rel-18 RRC_CONNECTED UE is not periodic or delay-sensitive service such as XR or URLLC, the data should not be transmitted or received in periodic resources outside the C-DRX active time during cell DTX/DRX non-active time.

If the service of Rel-18 RRC_CONNECTED UE is periodic or delay-sensitive service such as XR or URLLC, the service should not be affected or interrupted by cell DTX/DRX, and the cell should provide the XR or URLLC service for these UEs, and the following options are considered.

- Option 1: Cell DTX/DRX is not applied.

- Option 2: The periodic resources outside the C-DRX active time are overlapped with the cell DTX/DRX active time based on gNB implementation.

- Option 3: Specify that transmission and reception in periodic resources are not affected by cell DTX/DRX.

If there is only one UE has XR or URLLC service but other UEs are not delay-sensitive, option 1 affects the network energy saving gain. Option 2 and option 3 may be preferred in this situation.

Proposal 5: If the service of Rel-18 RRC_CONNECTED UE is periodic or delay-sensitive service such as XR or URLLC, the following options can be considered in order not to impact on the transmission/reception in periodic resources outside the C-DRX active time:

- Option 1: Cell DTX/DRX is not applied.

- Option 2: The periodic resources outside the C-DRX active time are overlapped with the cell DTX/DRX active time based on gNB implementation.

- Option 3: Specify that transmission and reception in periodic resources are not affected by cell DTX/DRX.

The mechanism for cell DTX/DRX transmission

Configuration of cell DTX/DRX

Regarding the configuration of cell DTX/DRX, cell DTX and cell DRX could be configured jointly or separately and should be discussed firstly. In current NR system, the UL and DL transmission of UE may be related or paired, in particular with HARQ operation. If cell DTX and cell DRX are jointly configured, the cell DTX active time and cell DRX active time can be aligned and gNB services both UL data reception and DL data transmission and the respective control signaling during the aligned gNB active time. The reception of UL PUSCH would need to have the DL PDCCH for scheduling the UL transmission. DL data transmission would require the UL control signaling, such as CSI for link adaptation and HARQ-ACK feedback for HARQ operation. In addition, the RRC_CONNECTED UE can perform transmission/reception in periodic resources outside the C-DRX active time, e.g. UL CG and DL SPS. The cell DTX and cell DRX can be configured separately that the cell DTX active time and cell DRX active time flexibly capture CG/SPS periodic resources to avoid affecting UE transmission performance. Since the cell DTX/DRX is applied in low system load, the signaling overhead of separate configuration of cell DTX and cell DRX is acceptable. Considering that the aligned cell DTX active time and cell DRX active time can also be implemented by separate cell DTX and DTX configurations, separate cell DTX and DTX configurations are flexible and preferred.

Proposal 6: The cell DTX and cell DRX can be configured and operated separately by higher layer signaling to adapt to the individual characteristics of DL and UL operations.

For the cell DTX/DRX configuration, periodic cell DTX/DRX pattern can be considered. The potential configuration methods related to the cell DTX/DRX can be considered as following:

- Option 1: The cell DTX/DRX parameters including the DTX/DRX cycle, the starting offset of DTX/DRX active time and the DTX/DRX active time duration, are semi-static configured.

- Option 2: The cell DTX/DRX configuration only including the DTX/DRX cycle and the starting offset of DTX/DRX active time, the end point of cell DTX/DRX active time may be related the UE’s C-DRX active time.

For option 1, the cell DTX/DRX parameters may include the DTX/DRX cycle, the starting offset of DTX/DRX active time and the DTX/DRX active time duration, and then the periodic cell DTX/DRX is determined. For option 2 that cell DTX/DRX configuration only includes the DTX/DRX cycle and the starting offset of DTX/DRX active time, the end point of cell DTX/DRX active time may be related the UE’s C-DRX active time, i.e. UE expects that the end point of cell DTX/DRX active time is later than its C-DRX active time. The offset between the end of cell DTX/DRX active time and the end of C-DRX active time could be configured via the high layer signaling.

Proposal 7: The potential configuration methods related to the cell DTX/DRX can be considered as following:

- Option 1: The cell DTX/DRX parameters including the DTX/DRX cycle, the starting offset of the DTX/DRX active time and the DTX/DRX active time duration, are semi-static configured.

- Option 2: The cell DTX/DRX configuration only including the DTX/DRX cycle and the starting offset of the DTX/DRX active time, the end point of cell DTX/DRX active time may be related the UE’s C-DRX active time.

Multiple cell DTX/DRX configurations

As for whether to support multiple cell DTX/DRX configurations, the traffic arrival may vary and robust over time. It may not be appropriate to use single cell DTX/DRX configuration for a long time. The cell DTX/DRX can be reconfigured by RRC reconfiguration. However, one semi-static configuration may be not sufficient to cover all different traffic characteristics at a time. Therefore, multiple cell DTX/DRX can be configured and at least one cell DTX/DRX configuration can be activated/deactivated/switched by L1/L2 signaling or RRC signaling. Moreover, multiple DTX/DRX configurations can be activated simultaneously, and the multiple DTX/DRX configurations can be used to capture different semi-static periodic resources by gNB implementation, such as CG, SPS, etc.

Proposal 8: Multiple cell DTX/DRX configurations would be supported.

Proposal 9: The cell DTX/DRX is configured to Rel-18 CONNECTED UEs via RRC signaling, and L1/L2 signaling or RRC signaling is used to activate at least one cell DTX/DRX configuration.

Activation and deactivation of cell DTX/DRX

In RAN2#120 [7], it was agreed that the periodic cell DTX/DRX pattern is configured by UE specific RRC signaling. The periodic cell DTX/DRX can be activated/deactivated by L1/L2 signaling and UE specific RRC signaling. When considering the activation and deactivation of cell DTX/DRX, the principle of this design should be identified firstly. The cell DTX/DRX is a technology mainly applied to the low system load scenario. The gNB can turn off some transmissions or receptions to achieve network energy saving. Therefore, the cell DTX/DRX should be a long-term and semi-static procedure. The cell DTX/DRX configurations should not be activated or deactivated frequently from the perspective of network energy saving but also the UE network access performance. In addition, the cell DTX/DRX technology should consider UE power consumption while maximizing network energy saving gains. The activation and deactivation of cell DTX/DRX which increases the UE power consumption should be avoided. Thus the activation and deactivation of cell DTX/DRX should consider the following aspects:

- The cell DTX/DRX is a semi-static procedure and is not activated or deactivated frequently.

- The activation and deactivation of cell DTX/DRX should reduce the impact to the UE power consumption.

Proposal 10: The activation and deactivation of cell DTX/DRX should consider the following aspects:

- The cell DTX/DRX is a semi-static procedure and is not activated or deactivated frequently.

- The activation and deactivation of cell DTX/DRX should reduce the impact to the UE power consumption.

While the cell DTX/DRX is activated or deactivated by UE specific RRC signaling, the cell DTX/DRX can be activated immediately once configured by RRC message and deactivated by RRC released messages. Furthermore, there may be additional indication for activation or deactivation of cell DTX/DRX in RRC message. Since the activation or deactivation of cell DTX/DRX is not so dynamic procedure and cell DTX/DRX is applied in the scenario of low system load. The latency of RRC configuration or reconfiguration and the corresponding RRC signaling overhead are acceptable. The gNB can transmit reliable RRC signaling for activation or deactivation of cell DTX/DRX when the gNB wants to perform or stop network energy saving mode.

For the cell DTX/DRX is activated or deactivated by L1 signaling, the L1 signaling can be UE-specific DCI or group common DCI. After the cell DTX/DRX is configured by RRC signaling, the L1 signaling can be used to activate or deactivate the cell DTX/DRX configuration, which is similar to the procedure of SPS and Type 2 CG. The advantage of L1-based activation and deactivation is dynamic and quick. The cell DTX/DRX is applied to all UEs in a cell. Group common L1 signaling can be used to expedite the transition and reduce the overhead of RRC signaling, which may be an efficient way for cell DTX/DRX activation and deactivation. However, UE may need to frequently monitor the common search space of PDCCH in multiple PDCCH monitoring occasions to determine whether the cell TX/DRX is activated or deactivated. Thus, L1 signaling is used to activate and deactivate the cell DTX/DRX to reduce the impact of L1 signaling monitoring on UE power consumption should be considered.

Proposal 11: The activation and deactivation of cell DTX/DRX can consider the following options:

- Option 1: The cell DTX/DRX is activated or deactivated by RRC signaling.

- Option 2: The cell DTX/DRX is activated or deactivated by L1/L2 signaling.

Considering the multiple cell DTX/DRX configurations may be supported and cell DTX and cell DRX can be configured separately, the L1/L2 signaling or RRC signaling for activation and deactivation of cell DTX/DRX should contain the contents such as activation or deactivation, cell DTX identification and cell DRX identification.

Proposal 12: The L1/L2 signaling or RRC signaling for activation and deactivation of cell DTX/DRX should at least contain the following contents:

- Activation or deactivation indication.

- Cell DTX identification.

- Cell DRX identification.

If L1 signaling is used to activate or deactivate the cell DTX/DRX, UE may miss the detection of the DCI even if gNB has transmitted the DCI in PDCCH monitoring occasion. The UE and gNB should have same understanding on the current cell DTX/DRX state. Thus, the confirmation of L1 signaling in activation/deactivation of cell DTX/DRX should be supported, e.g. HARQ-ACK feedback or cell DTX/DRX confirmation MAC CE.

Proposal 13: If L1 signaling is used to activate or deactivate the cell DTX/DRX, the confirmation of L1 signaling such as HARQ-ACK feedback or cell DTX/DRX confirmation MAC CE should be supported.

The cell DTX/DRX transmission for CA

For operation with carrier aggregation (CA), the number of active carriers used for data should be as few as possible in case of low system load. From UE power saving perspective, the unused SCells can be deactivated or indicated switching to the dormant BWP for reducing PDCCH monitoring. When the SCell is deactivated or is in the dormant BWP, UE would not receive/transmit the data traffic and reference signals except the sparse CSI-RS/normal SSB or periodic CSI report. In this case, the cell would still be activated for transmitting the CSI-RS/normal SSB or receiving the periodic CSI report. When the system is in the low load, the cell DTX/DRX would be considered to increase gNB light/deep sleep for power saving purpose.

In RAN2#120 [7], it was agreed that the cell DTX/DRX could be configured per serving cell and can be applicable for different cells in CA.

Agreements in RAN2#120 [7]: …… 5 Cell DTX/DRX can be configured per serving cell and can be applicable for different cells in CA. No additional RAN2 impacts or enhancements are foreseen. ……

For the cell DTX/DRX separately configured and applied for the different serving cells is the most flexible method in CA scenario. When multiple carriers are used for the data transmission, the active times of the SCells are different for the separate activation/deactivation operations for the SCells. Thus, the cell DTX/DRX for the different SCells would not necessarily be the same, thus the cell DTX/DRX for the SCells could be separately configured based on their desired active time and duty cycle.

The transmission parameters of cell DTX/DRX, such as the DTX/DRX cycle, the starting offset of DTX/DRX active time, the DTX/DRX active time duration etc., can be semi-static configured per serving cell via the UE-specific RRC signaling. For the cell DTX/DRX of each serving cell, it can be activated or deactivated via the UE-specific RRC signalling or L1/L2 signaling for its flexibility. For activation and deactivation of the cell DTX/DRX via UE-specific RRC signaling, the cell DTX/DRX of each serving cell would be activated or deactivated based on the RRC configuration or RRC re-configuration procedure, which is the semi-static indication especially for the scenario of low system load. In addition, the activation and deactivation of the cell DTX/DRX via L1/L2 signaling could be more flexible and efficient indication. When the serving cell has been indicated into the DTX/DRX for energy saving purpose, the cell would be deactivated and switch to the normal state for the urgent traffic transmission via the L1/L2 signaling. Thus, the cell DTX/DRX of each serving cell can be activated or deactivated via the UE-specific RRC signalling or L1/L2 signaling for expediting the transition between cell DTX/DRX active time and non-active time.

Considering that the same carrier for the different UE may be the different cells, such as PCell for the UE#1 and SCell for the UE#2, the L1/L2 signaling based the activation and deactivation signaling can be received at either PCell or SCell.

Proposal 14: The cell DTX/DRX could be applied for the different serving cells in CA scenario, in which the cell DTX/DRX can be activated or deactivated via the UE-specific RRC signalling or L1/L2 signaling for expediting the transition between cell DTX/DRX active time and non-active time.

Conclusion

In this contribution, we discuss the DTX/DRX for network energy saving and have the following proposals.

Proposal 1: The cell DTX/DRX parameters could be configured to ensure the alignment between the cell DTX/DRX and UE C-DRX, i.e. the cell DTX active time should completely capture the DRX-ON of UEs and at least cover the reception window of DCI format 2_6 in order not to affect the legacy UEs.

Proposal 2: If the cell DTX/DRX is applied, the UE behaviors should be specified when the cell DTX active time ends earlier than the UE DRX-ON extended by any of drx-InactivityTimer, drx-RetransmissionTimerDL or drx-RetransmissionTimerUL.

Proposal 3: If the cell DTX/DRX is applied, the following options can be considered in order not to impact on the transmission/reception in periodic resources for legacy RRC_CONNECTED UE:

- Option 1: The periodic resources outside the C-DRX active time are overlapped with the cell DTX/DRX active time based on gNB implementation.

- Option 2: Specify that transmission and reception in periodic resources are not affected by cell DTX/DRX.

Proposal 4: If the service of Rel-18 RRC_CONNECTED UE is not periodic or delay-sensitive service such as XR or URLLC, the data should not be transmitted or received in periodic resources outside the C-DRX active time during cell DTX/DRX non-active time.

Proposal 5: If the service of Rel-18 RRC_CONNECTED UE is periodic or delay-sensitive service such as XR or URLLC, the following options can be considered in order not to impact on the transmission/reception in periodic resources outside the C-DRX active time:

- Option 1: Cell DTX/DRX is not applied.

- Option 2: The periodic resources outside the C-DRX active time are overlapped with the cell DTX/DRX active time based on gNB implementation.

- Option 3: Specify that transmission and reception in periodic resources are not affected by cell DTX/DRX.

Proposal 6: The cell DTX and cell DRX can be configured and operated separately by higher layer signaling to adapt to the individual characteristics of DL and UL operations.

Proposal 7: The potential configuration methods related to the cell DTX/DRX can be considered as following:

- Option 1: The cell DTX/DRX parameters including the DTX/DRX cycle, the starting offset of the DTX/DRX active time and the DTX/DRX active time duration, are semi-static configured.

- Option 2: The cell DTX/DRX configuration only including the DTX/DRX cycle and the starting offset of the DTX/DRX active time, the end point of cell DTX/DRX active time may be related the UE’s C-DRX active time.

Proposal 8: Multiple cell DTX/DRX configurations would be supported.

Proposal 9: The cell DTX/DRX is configured to Rel-18 CONNECTED UEs via RRC signaling, and L1/L2 signaling or RRC signaling is used to activate at least one cell DTX/DRX configuration.

Proposal 10: The activation and deactivation of cell DTX/DRX should consider the following aspects:

- The cell DTX/DRX is a semi-static procedure and is not activated or deactivated frequently.

- The activation and deactivation of cell DTX/DRX should reduce the impact to the UE power consumption.

Proposal 11: The activation and deactivation of cell DTX/DRX can consider the following options:

- Option 1: The cell DTX/DRX is activated or deactivated by RRC signaling.

- Option 2: The cell DTX/DRX is activated or deactivated by L1/L2 signaling.

Proposal 12: The L1/L2 signaling or RRC signaling for activation and deactivation of cell DTX/DRX should at least contain the following contents:

- Activation or deactivation indication.

- Cell DTX identification.

- Cell DRX identification.

Proposal 13: If L1 signaling is used to activate or deactivate the cell DTX/DRX, the confirmation of L1 signaling such as HARQ-ACK feedback or cell DTX/DRX confirmation MAC CE should be supported.

Proposal 14: The cell DTX/DRX could be applied for the different serving cells in CA scenario, in which the cell DTX/DRX can be activated or deactivated via the UE-specific RRC signalling or L1/L2 signaling for expediting the transition between cell DTX/DRX active time and non-active time.

References

  - 3GPP TR 38.864, Study on network energy savings for NR, v1.0.0.

  - RP-223540,	 New WID: Network energy savings for NR, Huawei, RAN#98-e, December 12nd - 16th, 2022.

  - Chairman’s Notes RAN1#112, RAN1#112, Athens, Greece, February 27th - March 3rd, 2023.

  - TS38.213, Physical layer procedures for control, v16.5.0.

  - Report of 3GPP TSG RAN WG2 meeting #121, February 27th - March 3rd, 2023.

  - Report of 3GPP TSG RAN WG2 meeting #119bis-e, October 10th -19th, 2022.

  - Report of 3GPP TSG RAN WG2 meeting #120, Toulouse, France, November 14th -18th, 2022.