3GPP TSG RAN WG1 #112bis-e		R1-2302718

e-Meeting, April 17th– April 26th, 2023

Source:	CATT

Title:	Design of Multiple CG Occasions

Agenda Item:	9.8.1

Document for:	Discussion and Decision

Introduction

In Rel-17 “Study on XR Evaluation for NR” [1], the video stream in eXtended Reality (XR) and Cloud Gaming (CG) application, such as VR/AR and CG, are periodically generated based on the source codec frame generation rate, e.g. 60 FPS. For DL traffic in VR/CG, the packet inter-arrival time at the gNB would be varied caused by the delay jitter of network transport, while the packet arrival of the UL XR traffic would be periodic from the UE application layer to MAC/physical layer for scheduled UL transmission also with non-negligible delay jitter. The pose/control information as the classical UL XR traffic is usually generated with the 4ms periodicity with a small packet size, e.g. 100 bytes. Both DL and UL traffic are also characterized by relatively strict packet delay budget (PDB).

A UE can transmit UL data using CG resources after configuration (of a CG Type 1) or activation (of a CG Type 2), without the need of receiving UL grant from the gNB. The UL configured grant also has a fixed amount of resource allocation at each periodicity. The reserved resource allocation could be based on average packet size or minimum packet size of XR service with consideration of the system resource utilization. The larger the resource is allocated for CG, the less number of CG users is supported. According to the TR 38.835[2], the enhanced CG transmission for XR-specific traffic could improve the UL system capacity. In the CG enhancement, multiple PUSCH occasions per CG period are pre-configured with the semi-statically configured periodicity not relying on the SR.

The study results of “Study on XR Enhancements for NR” [3] has the conclusion that the XR-specific capacity enhancements would mainly focus on the enhancements of CG PUSCH transmission, including multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration and dynamic indication of unused CG PUSCH occasion(s) based on UCI by the UE.

The main potential technology points had been summarized in RAN1#112 meeting as following with detailed issues to be further discussed [4].

Conclusion For convenience in discussion, the term "multi-PUSCHs CG" refers to "a CG PUSCH configuration with multiple CG PUSCH transmission occasions within a period of the CG PUSCH configuration". Agreement - Multi-PUSCHs CG is supported for Type-1 configured grant. - Multi-PUSCHs CG is supported for Type-2 configured grant. Agreement For determination of the time domain resource allocation of CG PUSCHs associated to a multi-PUSCHs CG, the following alternatives for further study: - Alt-A: TDRA determination based on repetition framework. - Alt-A1: Follow the time domain resource mapping of Type A repetition - N configured by higher layers or indicated by activation DCI - Single SLIV is determined from TDRA - The same SLIV in N PUSCH in consecutive slots per CG period - FFS for non-consecutive slots - FFS details, including related RRC parameters - Alt-A2: Follow the time domain resource mapping of Type B repetition - N configured by higher layers or indicated by activation DCI - Single SLIV is determined from TDRA - The SLIV used for 1st PUSCH per CG period. - N consecutive nominal PUSCHs with same duration per CG period - Note: N is not necessarily the repetition factor. FFS details, including related RRC parameters - Alt-B: TDRA determination based on NR-U framework - N and M configured by higher layers - Single SLIV is determined from TDRA. - The SLIV used for 1st PUSCH per CG period. - M consecutive PUSCH TOs with same duration in slot. The M PUSCH TOs are used in N consecutive slots per CG period - Note: N and M are configured independently from cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, respectively. M and N configuration is independent from cgRetransmissionTimer configuration. - FFS details, including related RRC parameters - Alt-C: TDRA determination based on single DCI scheduling multiple PUSCHs - Alt-C1: Follow Rel-16 single DCI scheduling multiple PUSCHs - TDRA configured by pusch-TimeDomainAllocationListForMultiPUSCH-r16 with k2-r16 - A row of TDRA with N entries determines the time domain resources allocation of N PUSCH TOs per period - Note: N PUSCH TOs should be consecutive PUSCH TOs in consecutive slots. - FFS details, including related RRC parameters - Alt-C2: Follow Rel-17 single DCI scheduling multiple PUSCHs - TDRA configured by pusch-TimeDomainAllocationListForMultiPUSCH-r16 with extendedK2-r17 - A row of TDRA with N entries determines the time domain resources allocation of N PUSCH TOs per period - Note: N PUSCH TOs can be non-consecutive PUSCHs and/or in non-consecutive slots. - FFS details, including related RRC parameters Agreement For the PUSCHs parameters in a multi-PUSCHs CG configuration, the configuration/indication parameters except MCS and FDRA of CG PUSCHs in a multi-PUSCHs CG configuration are the same - FFS: For MCS and FDRA, study further to decide whether/how to be different. - FFS: Applicability to type-1 and type-2 - Note: TDRA and HP ID are not in this scope of the above statement. Conclusion RAN1 discusses to decide how to determine the HARQ process ID of CG PUSCHs of a multi-PUSCHs CG. Agreement For determination of HARQ process IDs associated to PUSCHs in multi-PUSCHs CG assuming one TB per PUSCH, consider the following alternatives: - Alt. 1: The HARQ process ID for the first configured/valid PUSCH in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured, and applying "the period duration divided by X instead of the period duration. - The HARQ process ID of the remaining PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period. - Alt 1-1; X = 1 - Alt 1-2: X is the number of configured PUSCHs in a period - Alt 1-3: X is provided by RRC configuration. - FFS details - Alt. 2: Support that UE can decide, as in NR-U, the HARQ IDs for the multiple CG PUSCH transmission occasions and indicate the decided HARQ IDs to gNB if multiple HARQ processes are used for the multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration - FFS details - Alt. 3: The HARQ process ID for the configured PUSCHs in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured. - FFS on potential enhancements different from previous alternatives - Alt 3-1: Note: Same HP ID would be used for all PUSCHs within a period. - FFS details - Alt 3-2: Note: Different HP ID could be used for all PUSCHs within a period. - FFS details - Alt. 4: The HARQ process ID for the first configured/valid PUSCH in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured. - The HARQ process ID of the remaining PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period - FFS on potential enhancements different from previous alternatives - Alt 5: Support that UE can decide, as in NR-U, the HARQ IDs for the first CG PUSCH transmission occasions and indicate the decided HARQ IDs to gNB if multiple HARQ processes are used for the multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration - The HARQ process ID of the remaining PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period - FFS details - Alt 6: FFS other solutions Note: The case of one TB map to multiple PUSCHs is not considered here. Agreement The physical channel that carries the UCI that provides information about unused CG PUSCH transmission occasions is CG PUSCH. Agreement Encoding and multiplexing for “the UCI that provides information about unused CG PUSCH transmission occasions” in a CG PUSCH applies encoding and multiplexing procedures for CG-UCI as baseline. - FFS on details Agreement Consider the following alternatives for “the UCI that provides information about unused CG PUSCH transmission occasions” for down-selection or revision - Alt. 1: “The UCI that provides information about unused CG PUSCH transmission occasions” is defined as a new UCI. - FFS on details - Alt. 2: “The UCI that provides information about unused CG PUSCH transmission occasions” is added as new field(s) to the CG-UCI. - FFS on details - Alt. 3: “The UCI that provides information about unused CG PUSCH transmission occasions” replaces/re-purposes some field(s) of the CG-UCI. - FFS on details Agreement For dynamic indication of unused CG PUSCH occasion(s) based on a UCI, the following options for further down-scoping with possible revision, are considered for the transmission occasion of the UCI: - Option 1: A transmitted CG PUSCH, includes the UCI. - FFS details - Option 2: A transmitted CG PUSCH includes the UCI, if it is transmitted in an occasion determined by RRC. - FFS details - Option 3: A transmitted CG PUSCH includes the UCI, if it is transmitted in a pre-defined transmission occasion. - FFS details - Example of a pre-determined occasion: 1st configured PUSCH TO in a CG period or 1st configured PUSCH TO in a multiple CG periods - Option 4: A transmitted CG PUSCH includes the UCI, if it is transmitted in a transmission occasion determined satisfying given condition(s). - FFS details - Examples of a condition: A first transmitted PUSCH in a CG period, or a first PUSCH transmission within a multiple of CG periods. Other options are not precluded. Proponent companies to provide details. Agreement For dynamic indication of unused CG PUSCH transmission occasion(s) based on a UCI, the following options for further down-scoping, are considered for the information provided by the UCI: - Option 1: The UCI determines the consecutive CG PUSCH TO(s) that are indicated as “unused” - Option 1-1: The UCI provides the number of consecutive TO(s) in time domain. - Applicable numbers can be determined from information obtained from configuration. - FFS details - Option 1-2: The UCI provides a time duration/range that includes the consecutive TO(s) in time domain. - Applicable time duration/range can be determined from information obtained from configuration - FFS details - Option 2: The UCI determines the CG PUSCH TO(s) that are indicated as “unused” (consecutive/non-consecutive TO(s) in time domain) - Option 2-1: The UCI provides a bitmap where a bit corresponds to a TO within a time duration/range. The bit indicates whether the TO is “unused”. - Applicable time duration/range can be determined from information obtained from configuration - FFS details - Option 2-2: The UCI provides a bitmap where a bit corresponds to TOs within a time duration/range. The bit indicates whether all TOs within the time duration/range are “unused”. - Applicable time duration/range can be determined from information obtained from configuration - FFS details - FFS whether/how the unused TO(s) can be associated to multiple CG configuration. - Other options are not precluded. Proponent companies to provide details.

In this contribution, we discuss the TDRA/FDRA/MCS determination based on the repetition framework comparing to the alternatives of NR-U framework and single DCI scheduling multiple PUSCHs in Section 2. The determination of the HARQ process ID in a multi-PUSCHs CG configuration is discussed in Section 3. The encoding and multiplexing for the UCI with the unused CG TOs indication in PUSCH and UCI design are provided in Section 4 and Section 5, respectively. The detailed procedure of indicating the unused TOs by UCI is described in Section 6.

The TDRA/FDRA/MCS determination

From RAN1#112[4], three different frameworks were discussed for the TDRA determination of multi-PUSCHs CG resources. We compare these three frameworks and list the differences in Table 1.

Table 1. The differences of the three frameworks for the TDRA 
determination of multi-PUSCHs CG resources

|  | Alt-A: TDRA determination based on repetition framework.[5] | Alt-B: TDRA determination based on NR-U framework[5] | Alt-C: TDRA determination based on single DCI scheduling multiple PUSCHs [5] |
| --- | --- | --- | --- |
| The baseline of the TDRA determination framework | CG transmission | CG transmission | Dynamic scheduling transmission |
| The SLIV configuration | Alt-A1 (Type A repetition): The same SLIV in N PUSCH in consecutive slots per CG period Alt-A2 (Type B repetition): Single SLIV used for 1st PUSCH per CG period, same duration for the N consecutive nominal PUSCHs in a CG period | Single SLIV is determined from TDRA. • The SLIV used for 1st PUSCH per CG period. | Alt-C1: Multiple SLIVs for consequent PUSCH Alt-C2: Multiple SLIVs for non-consequent PUSCH |
| The consecutive slots for PUSCH | Alt-A1: Consecutive slots FFS for non-consecutive slots Alt-A2: Consecutive nominal PUSCHs | Consecutive PUSCH TOs | Alt-C1: Consecutive TOs Alt-C2: Non-consecutive TOs |

The main differences proposed among these three methods focus on:

- Whether to use the same or different SLIV for a multi-PUSCHs CG;

- Whether to use consecutive or non-consecutive slots.

The motivation of the different SLIV for a multi-PUSCHs CG is to have the flexible configuration of different CG PUSCH transmission occasions in a CG period. The XR traffic is characterized with variation in packet size and the inter-arrival time with delay jitter in both DL and UL. The different SLIV values in TDRA determination might not be perfectly adapted to these variables and would increase the gNB and UE processing complexity. Furthermore, the objective of Rel-18 XR WID [3] includes the UCI indication of the unused CG occasions for increasing the resource utilization.

When the consecutive CG PUSCH occasions are configured for multi-PUSCHs CG in a XR packet generation period, UE can transmit the large size XR packet within interval in a timely matter. However, the UCI indication of unused CG occasions piggyback on CG PUSCH would not give enough time to the gNB to allocate the remaining unused TOs after receiving the UCI with processing time, especially for the FDD system. Since the jitter is expected for UL XR packet arrival from application layer to MAC/physical layer, the consecutive CG PUSCH occasions would not adapt to the variation of the XR packet arrival time within a XR packet generation period. When the CG PUSCH occasions are not configured successively in a multi-PUSCHs CG period, UE would have buffer between CG occasions to adapt to the variation of XR traffic arrival from application layer based on the jitter information of UL XR traffic. The time gap between the adjacent CG PUSCHs of non-consecutive CG occasions can be configured by higher signaling. Thus, the consecutive and non-consecutive CG PUSCH occasions should both be supported in the specification to allow gNB implementation to allocate the resource of the CG occasions in adaptation to the variation of XR traffic arrivals from application.

Furthermore, the TB mapping to the CG occasion(s) should be specified with the flexibility with consideration of resource allocation and system capacity in the gNB implementation for handling of different XR traffics. When the XR PDU size is larger than the reserved resource of the single CG occasion, UE could either segment the XR packets to multiple TBs or extend the mapping of the packet to the adjacent CG transmission occasions, in which the TBS is determined over the multiple slots. The number of sequent transmission occasions is configured by high layer signaling. Moreover, when the XR PDUs are associated with the same PDU set, the XR PDUs within a PDU set should be mapped to a TB in minimizing the processing delay of segmentation/reassembly at RLC. If the TB mapping to one or more CG occasions is supported in the enhanced CG transmission, a HARQ processing can be associated with a TB mapping to the multiple transmission occasions, which can reduce the budget of the number of HARQ processing. The TB mapping to one or more CG occasions should also be supported in the enhanced CG transmission for the larger XR packet transmission, which has been supported in CG repetition structure.

Above all, the Alt-A: TDRA determination based on repetition framework should be supported for the SLIV determination, in which N PUSCH occasions in a CG period with the same SLIV can be configured by higher layers or indicated by activation DCI. The consecutive and non-consecutive CG PUSCH occasions would both be supported, which is up to gNB implementation. The time gap between the adjacent CG PUSCH occasions can be configured by higher signaling.

Proposal 1: The Alt-A: TDRA determination based on repetition framework should be supported for the SLIV determination, in which

- N PUSCH occasions in a CG period with the same SLIV can be configured by higher layers or indicated by activation DCI;

- The configuration of consecutive and non-consecutive CG PUSCH occasions should both be supported to give the flexibility of gNB implementation for the adaptation of different XR traffic;

- The time gap between the adjacent CG PUSCH occasions could be configured by higher signaling;

- The single TB transmission over multiple CG occasions should be supported.

Since the frequency selective scheduling would not apply to CG, the parameters related to the frequency domain resource allocation, i.e. FDRA, could be the same as those in Rel-16 CG transmission for minimizing the specification impact. The other parameters related to the channel state information, such as MCS, would not be effective for the link adaptation gain for CG. Thus, the other parameters including MCS and FDRA in a multi-PUSCHs CG configuration should be the same.

Proposal 2: The other parameters including MCS and FDRA in a multi-PUSCHs CG configuration should be the same.

The determination of the HARQ process ID in a multi-PUSCHs CG configuration

When the multiple PUSCH occasions are configured for XR in a CG period, the determination of HARQ process ID (HPID) of CG PUSCHs would be one critical issue for further discussion. There are two alternatives in determining the HPID s based on the summary of discussion in RAN1#112 [4], which are the procedures of determination based on the legacy CG procedure and reported by UE as that in NR-U.

Since the HPID reported by UE might conflict with that of dynamic scheduling PUSCH for other traffic, such as eMBB, in the same UE, the HPID for the multi-PUSCHs in a CG period determined based on the legacy CG procedure in licensed band would minimize the impact to the specification. The HPID of the first configured PUSCH in a CG period is determined based on the legacy CG procedure as the Alt-1 agreed in RAN1#112 when cg-RetransmissionTimer is not configured, assuming that one TB per PUSCH. The HPID of the remaining PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period. When the configured number of multi-PUSCH occasions is larger than the maximum number of the HARQ process would be an issue ass shown in Figure 1. Assuming that the maximum number of HARQ process is 8 and the configured multiple CGPUSCH occasions in an XR packet period is 10, the HPIDs of the multiple CG-PUSCHs are identified at each UL slots. From Figure 1, the HPID of the last PUSCH in the previous XR packet cycle is near the slot that of the first CG-PUSCH at the next XR packet cycle,

Figure 1. The HPID determined as Alt-1

In order to resolve the problem of the same HARQ processing ID folding over colliding with the beginning of HARQ processing ID of next XR packets,  the HPID of the multiple CG-PUSCHs in a XR packet period should be enhanced. The HPID of first CG occasion can be calculated as the legacy method and the HARQ process ID of the remaining PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period by an offset, e.g. 2. The HPID enhancement is that, the reserved HPIDs in the adjacent CG periods can be different among adjacent XR packet periods, e.g. the odd HPID set for one XR packet period and even HPID set for the next XR packet period. The enhanced HPID determination method based on Alt-1 is shown in Figure 2.

Figure 2. The enhanced HPID determination method based on Alt-1

Furthermore, if one TB mapping to more than one CG PUSCH occasions with one HPID, the number of reserved HPID in a XR packet period could be reduced. Since the single TB transmission over more than one CG-PUSCH occasions had supported in the CG repetition transmission. The multiple CG-PUSCH occasions in a XR packet period can be further studied based on the repetition framework. For instance, the packets belonging to the same PDU set with small packet size could be multiplexed as a single TB and mapped to over the multiple CG occasions to increase the channel coding gain. The adjacent CG occasions could be used for transmitting a single TB in order to reduce the HARQ process ID in a XR packet period.

Proposal 3: The following HPID determination method could be supported:

- The HPID for the multiple CG PUSCHs in a CG period determined and reported by UE should not be supported in Rel-18 XR work item.

- The HARQ process ID based on the Alt-1 would be enhanced that the HPID of first CG occasion can be calculated as the legacy method and the HARQ process ID of the remaining PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period by an offset, e.g. 2.

- The single TB transmission mapping to one or more CG occasions could reduce the number of HARQ process ID and should be further studied based on the repetition framework.

The encoding and multiplexing for the UCI with the unused CG TOs indication in PUSCH

It was agreed in RAN1#112 [4] that the physical channel that carries the UCI that provides information about unused CG PUSCH transmission occasions is CG PUSCH. Based on the procedure of the UCI reporting in PUSCH, the multiplexing control information including HARQ-ACK, CSI part 1, CSI part 2 and CG-UCI can be multiplexed with the coded bits for UL-SCH as in Figure 3. As shown in Figure 3, each  control information has its beta-offset value for determining the number of resources in a PUSCH. In the existing procedure, the CG-UCI is jointly coded with HARQ-ACK or separately coded when there is not HARQ-ACK. The RE allocation is the same as that of the HARQ-ACK information bits with more than 2 bits.

Figure 3. The UCI multiplexing in PUSCH

For the UCI for indicating the unused CG occasions (denoted as UTO-UCI) could be cascaded with HARQ-ACK and/or CG-UCI information bits. When UTO-UCI is configured with the multiplexing with the cg-RetransmissionTimer configured, the UCI bit sequence of HARQ-ACK can be determined as OACK= OHARQ+OCG-UCI+OUTO-UCI, where OHARQ, OCG-UCI and OUTO-UCI are the bit sequences of HARQ-ACK, CG-UCI and UTO-UCI, respectively. Otherwise, if cg-RetransmissionTimer is not configured, the UCI bit sequence of HARQ-ACK can be determined as OACK= OHARQ+ OUTO-UCI. The RE allocation is the same as that of the HARQ-ACK information bits with more than 2 bits. The beta-offset for the UTO-UCI can be configured separately with the same value as that of HARQ-ACK and/or CG-UCI.

Proposal 4:  For the UCI carried the unused CG TOs information (UTO-UCI), it could be multiplexed in PUSCH as following:

- The information bit of the UCI can be cascaded with HARQ-ACK and/or CG-UCI information bits for the simplicity.

- The beta-offset value of UTO-UCI could be the same as that of HARQ-ACK and/or CG-UCI.

UCI design on the unused Tx occasions

According to the agreements in RAN1#112 meeting [4], there were three alternatives for the design of the UCI that provides information about unused CG PUSCH transmission occasions. The Alt.2 and Alt.3 were proposed based on the CG-UCI, in which the Alt.2 was adding a new filed in the CG-UCI, while the Alt.3 was replaces/re-purposes some field(s) of the CG-UCI. When the higher layer parameter cg-RetransmissionTimer is configured, the CG-UCI bits would be transmitted on a CG PUSCH. The information in CG-UCI is provided in Table 6.3.2.1.3-1 in TS38.212 [6] as following.

Table 6.3.2.1.3-1: Mapping order of CG-UCI fields in TS38.212[6]

| Field | Bitwidth |
| --- | --- |
| HARQ process number | 5 if nrofHARQ-Processes-v1700 in ConfiguredGrantConfig is configured; 4 otherwise. |
| Redundancy version | 2 |
| New data indicator | 1 |
| Channel Occupancy Time (COT) sharing information | log2C if both higher layer parameter ul-toDL-COT-SharingED-Threshold and higher layer parameter cg-COT-SharingList are configured, or if both higher layer parameter semiStaticChannelAccessConfigUE and higher layer parameter cg-COT-SharingList are configured, or if higher layer parameter cg-COT-SharingList is configured in frequency range 2-2, where C is the number of combinations configured in cg-COT-SharingList; 1 if higher layer parameter ul-toDL-COT-SharingED-Threshold is not configured, and if higher layer parameter semiStaticChannelAccessConfigUE is not configured, and if higher layer parameter cg-COT-SharingOffset is configured; 0 otherwise. If a UE indicates COT sharing other than "no sharing" in a CG PUSCH within the UE's initiated COT, the UE should provide consistent COT sharing information in all the subsequent CG PUSCHs, if any, occurring within the same UE's initiated COT such that the same DL starting point and duration are maintained. |

Considering the information including the HARQ process number, RV, NDI and COT sharing information related to the CG PUSCH transmission in un-license band, there would not be the relationship with the CG transmission in license band. If the cg-RetransmissionTimer is configured, Alt.2 and Alt.3 of unused CG occasion indication would also confuse the gNB for the unused CG occasion indication in un-license band or license band. Thus, “The UCI that provides information about unused CG PUSCH transmission occasions” would be defined as a new UCI. The Alt. 1 should be supported.

Proposal 5: The Alt.1:“The UCI that provides information about unused CG PUSCH transmission occasions would be defined as a new UCI” should be supported.

The procedure of indicating the unused TOs by UCI

When the multiple CG-PUSCH transmission occasions are configured in a CG period in the CG configuration, UE can transmit the UL XR packets at some or all of the CG transmission occasions. UE can send UCI with the information about the utilization of the transmission occasions to inform gNB which CG occasions are used and unused. From RAN1#112 [4], the UCI carried the unused CG TOs information is multiplexed in PUSCH. Thus, the overhead of UCI related to the utilization of the transmission occasions would not the bottle neck for the UCI design.

Considered that the unused CG TOs can be indicated clearly for the consecutive and the non-consecutive TOs in the time domain, the bitmap indication for the utilization of each transmission occasion within a time duration could be supported as shown in Figure 4-(a) and Figure 4-(b).

(a) The UCI is sent at each PUSCH

(b) The UCI is sent at the configured PUSCH occasion

Figure 4. The bitmap indication carried in UCI

In Figure 4-(a), the UCI always multiplexes in a transmitted CG PUSCH. The size of information field in the UCI is N bits associated with the number of CG TOs or CG TO groups within a time duration, where the time duration can be configured by RRC, such as a CG period or the duration of a group of CG occasions. A “1” value for a bit in UCI indicates a used occasion (group) and a “0” value for a bit indicates an unused occasion (group). When the CG periodicity is aligned with the periodicity of the XR traffic with offset for MAC/physical layer processing IDs, the indication could be used to indicate the CG occasion utilization in a CG period, e.g. the indication is sent at the beginning of a CG period since the MAC/physical layer would receive the XR packet from application layer at the UE in advance in preparation of CG-PUSCH transmission. When the packet arrives before the unused CG occasion, the indication in UCI can update the utilization state of CG occasions based on the actual transmission.

In the Figure 4-(b), the UCI can be multiplexed at the configured CG PUSCH occasion. The size of information field in the UCI is N bits associated with the number of CG TOs or CG TO groups within a time duration, where the time duration associated with the time gap of the CG PUSCH occasions with the UCI. Similar to the indication shown in Figure 4-(a), a “1” value for a bit in UCI indicates a used occasion (group) and a “0” value for a bit indicates an unused occasion (group). In this case, the overhead of the UCI multiplexing with PUSCH can be reduced to obtain more resource to transmit the UL data.

The procedure of the UCI reported is provided as following:

- When the packet is not arrived before the first CG occasions due to the delay jitter, the UCI is not transmitted on PUSCH and does not need to be detected. gNB would continue to detect the CG PUSCH until the first CG-PUSCH, assuming that the CG periodicity is aligned with the periodicity of the XR traffic with offset for MAC/physical layer process.

- When the UL packet arrives, UE would send the UCI at each CG PUSCH or configured PUSCH occasions based on the buffer state at the first occasion within the certain time duration.

- The UCI multiplexing with PUSHC can be used to indicate the utilization of CG occasion within a time duration, such as the bitmap of these CG occasion with value “1” for used occasion or value “0” for unused occasion.

- If the packet size is large, the CG occasions within the time duration would all be used. gNB would continue to detect the subsequent CG occasions within the configured time duration.

- If the packet size is small, the bitmap of a part of CG occasions could be indicated as “0”. gNB would reused the unused CG occasions for UL transmission, such as dynamic scheduling for other users.

Whatever the UCI indication in Figure 4-(a) and Figure 4-(b), there will be a problem that another packet arrives at the middle of PUSCH occasions due to the delay jitter, which had been indicated as “unused” CG-PUSCH occasions. How to update the utilization of CG PUSCH occasions by the UCI should be further discussed.

Proposal 6: Either of following alternatives could be supported:

- Alt-1: A UCI would multiplex with each transmitted CG PUSCH occasions, which provides a bitmap where a bit corresponds to a TO or a TO group within a time duration/range. The bit indicates whether the TO or TO group is “unused”.

- Alt-2: A UCI would multiplex with the configured CG PUSCH occasions, which provides a bitmap where a bit corresponds to a TO or a TO group within a time duration/range. The bit indicates whether the TO or TO group is “unused”.

Conclusion

In this contribution, we discuss the CG enhancement scheme for XR-specific traffic and have the following proposals.

Proposal 1: The Alt-A: TDRA determination based on repetition framework should be supported for the SLIV determination, in which

- N PUSCH occasions in a CG period with the same SLIV can be configured by higher layers or indicated by activation DCI;

- The configuration of consecutive and non-consecutive CG PUSCH occasions should both be supported to give the flexibility of gNB implementation for the adaptation of different XR traffic;

- The time gap between the adjacent CG PUSCH occasions could be configured by higher signaling;

- The single TB transmission over multiple CG occasions should be supported.

Proposal 2: The other parameters including MCS and FDRA in a multi-PUSCHs CG configuration should be the same.

Proposal 3: The following HPID determination method could be supported:

- The HPID for the multiple CG PUSCHs in a CG period determined and reported by UE should not be supported in Rel-18 XR work item.

- The HARQ process ID based on the Alt-1 would be enhanced that the HPID of first CG occasion can be calculated as the legacy method and the HARQ process ID of the remaining PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period by an offset, e.g. 2.

- The single TB transmission mapping to one or more CG occasions could reduce the number of HARQ process ID and should be further studied based on the repetition framework.

Proposal 4:  For the UCI carried the unused CG TOs information (UTO-UCI), it could be multiplexed in PUSCH as following:

- The information bit of the UCI can be cascaded with HARQ-ACK and/or CG-UCI information bits for the simplicity.

- The beta-offset value of UTO-UCI could be the same as that of HARQ-ACK and/or CG-UCI.

Proposal 5: The Alt.1:“The UCI that provides information about unused CG PUSCH transmission occasions would be defined as a new UCI” should be supported.

Proposal 6: Either of following alternatives could be supported:

- Alt-1: A UCI would multiplex with each transmitted CG PUSCH occasions, which provides a bitmap where a bit corresponds to a TO or a TO group within a time duration/range. The bit indicates whether the TO or TO group is “unused”.

- Alt-2: A UCI would multiplex with the configured CG PUSCH occasions, which provides a bitmap where a bit corresponds to a TO or a TO group within a time duration/range. The bit indicates whether the TO or TO group is “unused”.

References

  - RP-220285, Revised SID : Study on XR Enhancements for NR, Nokia, RAN#95-e, March 17th-23rd, 2022.

  - TR 38.835, Study on XR enhancements for NR, v1.0.0.

  - RP-223502, New WID on XR Enhancements for NR, Nokia, Qualcomm, RAN#98-e, December 12nd-16th, 2022.

  - Chairman’s Notes RAN1#112, RAN1#112, Athens, Greece, February 27th - March 3rd, 2023.

  - TS 38.214, Physical layer procedures for data, v17.4.0.

  - TS 38.212, Multiplexing and channel coding, v17.4.0.