3GPP TSG RAN WG1 #112bis-e                                            R1-2302721

# e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.9.3

Source:	CATT

Title:	         Discussion on remaining issues of disabling of HARQ feedback for IoT NTN

Document for:	Discussion and Decision

Introduction

For the enhancements on IoT NTN HARQ, there were some discussions on Disabling/enabling HARQ feedback, and transmission enhancement. Moreover, a few agreements on HARQ operation were concluded [1] as follows in last RAN1 meeting:

Conclusion

For eMTC HD-FDD single TB scheduled by single DCI, UE is not expected to receive a DCI with “HARQ-ACK bundling flag” field set to 1 in case the corresponding HARQ process is configured with HARQ feedback disabled by RRC signaling.

Agreement

For a DL HARQ process with disabled HARQ feedback in eMTC, UE is not expected to receive another MPDCCH carrying a DCI scheduling a PDSCH for a given HARQ process or to receive another PDSCH without corresponding MPDCCH for the given HARQ process that starts at a BL/CE DL subframe until X=3 (ms) have passed after the end of the reception of the last PDSCH for that HARQ process.

Agreement

For HARQ feedback for eMTC SPS PDSCH, at least the following is supported: UE follows the per-process HARQ feedback enabled/disabled configuration for the associated HARQ process except for the first SPS PDSCH after activation

- for the first SPS PDSCH after activation,

  - Option 1: If HARQ feedback for SPS activation is additionally enabled, ACK/NACK is reported by UE for the first SPS PDSCH after activation regardless of network configuration of enabled/disabled for this HARQ process, and follow per-process HARQ feedback enabled/disabled configuration otherwise.

Conclusion

For DCI indicating SPS PDSCH release, HARQ-ACK report is performed as legacy in eMTC, regardless of HARQ feedback enabled/disabled configuration.

Agreement

For DCI-based overridden mechanism/indication in single TB scheduled by DCI, down select one of the following alternatives based on the criteria DCI overhead, PDCCH monitoring/power consumption, HARQ timer, impact on scheduling flexibility, UE implementation complexity

- Alternative 1: applies to both semi-statically HARQ enabled and disabled processes

- Alternative 2: only applied to semi-statically HARQ disabled processes

- Alternative 3: only applied to semi-statically HARQ enabled processes

Agreement

Confirm the following working assumption with the following update:

Working assumption

For NB-IoT NTN and eMTC NTN for CE Mode B, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission:

- Support Option 1 in case only per-HARQ process bitmap signaling is configured

- Support Option 3 DCI direct indication of HARQ feedback enable/disable in case only DCI solution enabling/disabling signaling is configured

- Support Option 3 DCI indication to override Option 1 configuration for corresponding transmission in case both per-HARQ process bitmap and DCI solution enabling/disabling signaling are configured

  - FFS #1: Option 3 DCI-based overridden mechanism is applied to both semi-statically HARQ feedback enabled and disabled processes or only applied to semi-statically HARQ feedback disabled processes or only applied to semi-statically HARQ feedback enabled processes.

  - FFS #2: whether/how to support Option 3 overriding Option 1 configuration for corresponding transmission for multiple TBs scheduled by single DCI

  - FFS#3：Option 3 DCI-based overridden mechanism is DCI signaling to reverse the HARQ feedback enable/disable for the corresponding transmission from per-HARQ process RRC configuration or DCI signaling to directly indicate the HARQ feedback enable/disable for the corresponding transmission regardless of per-HARQ process RRC configuration.

RAN1 strives to have a common design (in terms of DCI design, PDCCH monitoring, etc.) for “Option 3” and “Option 3 + Option 1”.

For eMTC NTN, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, take Option 1 for CE Mode A.

Agreement

For DCI-based overridden/direct indication, down select one of the following based on the criteria DCI overhead, PDCCH monitoring behavior, impact on scheduling flexibility, UE implementation complexity, etc

- Option 1: Indication by adding one field in DCI

- Option 2: Indication by reusing/reinterpreting existing field in DCI

In this contribution, we further analyze remaining technical issues of NB-IoT/eMTC HARQ operation and provide our views on these issues.

Discussion

  - The indication/configuration of disabling HARQ feedback

Since it has been agreed in NR NTN that enabling/disabling of HARQ feedback for downlink transmission is configurable per HARQ process via UE specific RRC signaling, the reuse of HARQ feedback enabling/disabling configuration is at least supported in IoT NTN. In addition, the other options have been proposed at the last meeting.

It has been agreed that for NB-IoT NTN and eMTC NTN for CE Mode B, Option 3 can override the default RRC configuration. Although the scenario of dynamically switching from 'disable' to 'enable' is clear, and it can provide more flexibility by dynamically switching from 'enable' to 'disable' on the eNB side. DCI-based overridden mechanism is applied to both semi-statically HARQ enabled and disabled processes since the DCI can directly indicate HARQ enable/disable.

Proposal 1: DCI-based overridden mechanism can be applied to both semi-statically HARQ enabled and disabled processes.

For DCI-based overridden/direct indication, down select one of the following based on the criteria DCI overhead, PDCCH monitoring behavior, impact on scheduling flexibility, UE implementation complexity, etc - Option 1: Indication by adding one field in DCI - Option 2: Indication by reusing/reinterpreting existing field in DCI

For DCI-based overridden/direct indication, above two options were proposed in the last meeting. Compared with option 2, which would reduce the scheduling flexibility, option 1 is a simple way since DCI-based overridden and direct indication could have a consistent design. Therefore, it is better to add a field in DCI to indicate the HARQ disable/enable.

Proposal 2: A new DCI field can be defined for overriding the default RRC configuration or direct indication.

And how the new field is used, there may be three options as following:

- Option 1: the field indicates the HARQ feedback state directly. For example, if the field is set 1, the HARQ feedback of the corresponding HARQ process is enabled. Otherwise, the HARQ feedback of the corresponding HARQ process is disabled.

- Option 2: the field indicates to reverse the current HARQ feedback state. For example, if the field is set 1, the HARQ feedback state of the corresponding HARQ process will be reversed, from enabling to disabling, and vice versa. Otherwise, the HARQ feedback state of the corresponding HARQ process will not be reversed.

- Option 3: the field indicates to reverse the current HARQ feedback state by the toggling of this field, like NDI. For example, if the field is toggled, the HARQ feedback state of the corresponding HARQ process will be reversed, from enabling to disabling, and vice versa. Otherwise, the HARQ feedback state of the corresponding HARQ process will not be reversed.

Proposal 3: RAN1 to discuss how to override the default RRC configuration by DCI, considering the three options as follows:

- Option 1: the field indicates the HARQ feedback state directly

- Option 2: the field indicates to reverse the current HARQ feedback state

- Option 3: the field indicates to reverse the current HARQ feedback state by the toggling of this field

For multiple TBs scheduled by single DCI, Option 3 overriding default configuration and the DCI-based direct indication can all be supported. Since a single DCI can schedule up to 4 TB, if one bit is used for per HARQ process, there would need 4bit to indicate HARQ feedback. In order to save DCI overhead and consistent design, we prefer the following option proposed in last meeting.

- DCI-based overridden mechanism/indication applied TBs is determined by the per-HARQ RRC configuration (e.g., all HARQ enabled, all HARQ disabled or mixed HARQ enabled/disabled configuration)

Specially, for DCI-based HARQ enabling/disabling direct indication, we assuming that all HARQ processes are configured as enabled. And how to support needs further study.

Proposal 4: For multiple TBs scheduled by single DCI, DCI-based overridden mechanism and the DCI-based direct indication can be supported both.

  - SPS PDSCH

Regarding the SPS PDSCH, some companies proposed that, for IoT NTN, the considered scenarios are not latency sensitive. The gNB can activate at a time where the first SPS PDSCH has a HARQ process with enabled HARQ-ACK report, so the additional HARQ feedback for SPS activation is not needed. However, it has not been agreed that at least one HARQ process with feedback is configured, the method is not applied.

Since SPS PDSCH in eMTC is same as NR NTN, it is better to reuse the same mechanism for IoT NTN as follows.

For HARQ feedback for eMTC SPS PDSCH, UE follows the per-process HARQ feedback enabled/disabled configuration for the associated HARQ process, except for the first SPS PDSCH after activation if HARQ feedback for SPS activation is additionally enabled. - If HARQ feedback for SPS activation is additio nally enabled, ACK/NACK is reported by UE for the first SPS PDSCH after activation regardless of network configuration of enabled/disabled for this HARQ process. For DCI indicating SPS PDSCH release, HARQ-ACK report is performed as legacy in eMTC.

Proposal 5:Reuse the SPS PDSCH mechanism of NR NTN for IoT NTN.

  - HARQ feedback for scheduling multiple TB

If one TB is scheduled by DCI and Multi-TB is scheduled by a Multi-TB DCI and higher layer parameter harq-AckBundling is not configured, it is nature to reuse the type1 codebook mechanism in NR NTN as follows.

For HARQ codebook in IoT over NTN and higher layer parameter harq-AckBundling is not configured, the UE can consistently report NACK-only for the feedback-disabled HARQ process regardless of decoding results of corresponding PDSCH.

If Multi-TB is scheduled by a Multi-TB DCI and higher layer parameter harq-AckBundling is configured, how to generate the codebook needs further study when HARQ feedback disabling is introduced. In eMTC, when the all TB in a TB bundle for a HARQ process with disabled HARQ-ACK information, the UE reports a NACK value for a HARQ-ACK information bit corresponding for the bundle. If the feedback of some TBs are enabled, and others are disabled, the UE shall generate one HARQ-ACK bit by performing a logical AND operation of HARQ-ACKs across all TBs in each TB bundle, where ACK is assumed for TB of the HARQ process with disabled HARQ-ACK information. To achieve a unified design, similar solutions can be used as eMTC for NB-IoT.

Proposal 6: ACK value is assumed for TB of the HARQ process with disabled HARQ-ACK information, when Multi-TB is scheduled and harq-AckBundling is configured.

  - HARQ bundling for eMTC HD-FDD

In eMTC HD-FDD, the UE can’t transmit and receive the TB at the same time. HARQ bundling is introduced in Rel.14 and enhanced in Rel.17 for more throughput. Similar as solutions of Multi-TB, ACK is assumed When HARQ feedback disabled is introduced.

Proposal 7: ACK value is assumed for TB of the HARQ process with disabled HARQ-ACK information for HARQ bundling in eMTC HD-FDD.

Conclusion

In this contribution, we analzyed potential issues of HARQ impact due to longer propogation delay in NB-IoT and eMTC over satellite. In sum, the observations and proposals are listed as follows:

Proposal 1: DCI-based overridden mechanism is applied to both semi-statically HARQ enabled and disabled processes.

Proposal 2: A new DCI filed can be defined for overriding the default RRC configuration or direct indication.

Proposal 3: RAN1 to discuss how to override the default RRC configuration by DCI, considering the three options as follows:

- Option 1: the field indicates the HARQ feedback state directly

- Option 2: the field indicates to reverse the current HARQ feedback state

- Option 3: the field indicates to reverse the current HARQ feedback state by the toggling of this field

Proposal 4: For multiple TBs scheduled by single DCI, DCI-based overridden mechanism and the DCI-based direct indication can be supported both.

Proposal 5: Reuse the SPS PDSCH mechanism of NR NTN for IoT NTN.

Proposal 6: ACK value is assumed for TB of the HARQ process with disabled HARQ-ACK information, when Multi-TB is scheduled and harq-AckBundling is configured.

Proposal 7: ACK value is assumed for TB of the HARQ process with disabled HARQ-ACK information for HARQ bundling in eMTC HD-FDD.

References

- Chairman’s note, 3GPP RAN1 #112.

- 3GPP TS 36.213: "LTE; Physical layer procedures".