3GPP TSG-RAN WG1 Meeting #112bis-e                                   R1-2302726

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.1.3.1

Source:	Lenovo

Title:	Discussion of increased number of orthogonal DMRS ports

Document for:	Discussion

# Introduction

In RAN1 #112 meeting, the following agreement is made for increasing the number of orthogonal DMRS ports for DL/UL MU-MIMO:

Conclusion

Dynamic switching between R15 DMRS port and R18 DMRS port by a scheduling DCI is not supported in Rel-18

Agreement

For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, support at least support the following rows:

- For 1 CW,

  - 1) Rows 0-2, 12-14, 24-25 (rows with Number of DMRS CDM group(s) without data = 1)

Agreement

For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, at least support the following rows:

- For 1 CW,

  - 2) Row 9-11

    - For the above rows, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group)

Working Assumption

For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, for 2 CWs,

- Alt.3-1: Support at least row 0-3 for 2 CWs in Table 4-0.

Table 4-0: DMRS ports for 2CWs.

| Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 2 | 0,1,2,3,8 |
| 1 | 2 | 0,1,2,3,8,10 |
| 2 | 2 | 0,1,2,3,8,9,10 |
| 3 | 2 | 0,1,2,3,8,9,10,11 |

Agreement

For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH for S-DCI based M-TRP, support at least the following row(s):

- For one CW, support at least row 30 in the following table.

  - For the above row, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group).

- FFS: other rows are not precluded

Table 7.3.1.2.2-1A-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=1

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |
| --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| … | … | … |
| 30 | 2 | 0,2,3 |

Agreement

For Rel.18 eType1/eType2 DMRS ports for PDSCH/PUSCH, support Alt.1 for PTRS RE mapping.

- Alt 1: Different RE offsets set for different Rel.18 DMRS port indexes as shown in Table 4

Table 4 Different RE offsets set for different Rel.18 DMRS port indexes

| DM-RS antenna port p (p for PUSCH, p+1000 for PDSCH) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DM-RS Configuration type 1 |  |  |  | DM-RS Configuration type 2 |  |  |  |
|  | resourceElementOffset |  |  |  | resourceElementOffset |  |  |  |
|  | offset00 | offset01 | offset10 | offset11 | offset00 | offset01 | offset10 | offset11 |
| 0 | 0 | 2 | 6 | 8 | 0 | 1 | 6 | 7 |
| 1 | 2 | 4 | 8 | 10 | 1 | 6 | 7 | 0 |
| 2 | 1 | 3 | 7 | 9 | 2 | 3 | 8 | 9 |
| 3 | 3 | 5 | 9 | 11 | 3 | 8 | 9 | 2 |
| 4 | - | - | - | - | 4 | 5 | 10 | 11 |
| 5 | - | - | - | - | 5 | 10 | 11 | 4 |
| 8 | 4 | 6 | 10 | 0 | - | - | - | - |
| 9 | 6 | 8 | 0 | 2 | - | - | - | - |
| 10 | 5 | 7 | 11 | 1 | - | - | - | - |
| 11 | 7 | 9 | 1 | 3 | - | - | - | - |
| 12 | - | - | - | - | 6 | 7 | 0 | 1 |
| 13 | - | - | - | - | 7 | 0 | 1 | 6 |
| 14 | - | - | - | - | 8 | 9 | 2 | 3 |
| 15 | - | - | - | - | 9 | 2 | 3 | 8 |
| 16 | - | - | - | - | 10 | 11 | 4 | 5 |
| 17 | - | - | - | - | 11 | 4 | 5 | 10 |

Working Assumption

To support PUSCH with rank = 5-8, support the following for enhancement of DMRS port allocation tables.

- Option 1: Separate DMRS ports tables for rank 5,6,7,8 for each of eType1/eType2 and maxLength=1/2 (similar to the current UL DMRS ports table).

  - FFS: whether/how to reuse the reserved field in antenna ports field for other purposes can be discussed in AI9.1.4.2 [or AI9.1.3.1].

Agreement

For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PUSCH, following Table 7.3.1.1.2-8-X, Table 7.3.1.1.2-9-X, Table 7.3.1.1.2-10-X, and Table 7.3.1.1.2-11-X are supported.

- FFS: Whether to increase the size of antenna ports field in DCI format 0_1/0_2 or not.

Table 7.3.1.1.2-8-X: Antenna port(s), transform precoder is disabled, dmrs-Type=eType1, maxLength=1, rank = 1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 1 | 0 |
| 1 | 1 | 1 |
| 2 | 2 | 0 |
| 3 | 2 | 1 |
| 4 | 2 | 2 |
| 5 | 2 | 3 |
| 6 | 1 | 8 |
| 7 | 1 | 9 |
| 8 | 2 | 8 |
| 9 | 2 | 9 |
| 10 | 2 | 10 |
| 11 | 2 | 11 |
| 12-15 | Reserved | Reserved |

Table 7.3.1.1.2-9-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 1 | 0,1 |
| 1 | 2 | 0,1 |
| 2 | 2 | 2,3 |
| 3 | 2 | 0,2 |
| 4 | 1 | 8,9 |
| 5 | 2 | 8,9 |
| 6 | 2 | 10,11 |
| [7] | [2] | [8,10] |
| 8-15 | Reserved | Reserved |

Table 7.3.1.1.2-10-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-2 |
| [1] | [2] | [8-10] |
| 2 | 1 | 0,1,8 |
| 3 | 2 | 0,1,8 |
| 4 | 2 | 2,3,10 |
| 5-15 | Reserved | Reserved |

Table 7.3.1.1.2-11-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-3 |
| [1] | [2] | [8-11] |
| 2 | 1 | 0,1,8,9 |
| 3 | 2 | 0,1,8,9 |
| 4 | 2 | 2,3,10,11 |
| 5-15 | Reserved | Reserved |

Agreement

For full-coherent PUSCH with rank 5-8 with one port PTRS, support Alt.1 in the RAN1#111 agreement with the following update

- Alt.1: the size of PTRS-DMRS association field is 2bit in DCI format 0_1/0_2.

  - FFS: Association with The CW with the higher MCS is selected in case of two CWs.

  - If the MCS is the same for two CWs, the PTRS port is associated with the first CW.

Table 7.3.1.1.2-25B: PTRS-DMRS association for UL PTRS port 0

| Value | DMRS port |
| --- | --- |
| 0 | 1st scheduled DMRS port with the CW with the higher MCS |
| 1 | 2nd scheduled DMRS port the CW with the higher MCS |
| 2 | 3rd scheduled DMRS port the CW with the higher MCS |
| 3 | 4th scheduled DMRS port the CW with the higher MCS |

In this contribution, we provide our views on increasing number of orthogonal DMRS ports for UL/DL MU-MIMO and 8 Tx UL SU-MIMO.

# Discussion on increasing the number of orthogonal DMRS ports

## Increasing the number of orthogonal DMRS ports for DL/UL MU-MIMO

In RAN1 #110b meeting, the working assumption is confirmed that at least Opt.1 (introduce new larger FD-OCC length (e.g. 4 or 6)) is supported to increase the number of DMRS ports for PDSCH/PUSCH. Based on the new agreements in RAN1 #110b meeting and in RAN1 #111 meeting, we make further discussion on the design for enhanced FD-OCC DMRS.

Orphan RE/RB

In RAN1#110b meeting, the method to resolve the orphan RB issue is agreed except for two FFS. The first FFS is on scheduling restriction of different UEs in case of MU-MIMO. We think if 1) The number of consecutively scheduled PRBs for PDSCH is even, and 2)  The number of PRBs offset of scheduled PDSCH from point A (common resource block 0) is even, are satisfied by each co-scheduled UE, the offset between starting RB of co-scheduled UEs is already even. Therefore, we support no additional restriction for MU-MIMO. The second FFS is related with M-TRP PDSCH transmission with FDM 2a or FDM 2b scheme. For these two schemes, the first ⌈nPRB/2 ⌉ PRBs are transmitted from a TRP and the remaining ⌊nPRB/2 ⌋ PRBs are transmitted from a another TRP, wherein the nPRB is the total number of allocated PRBs for a UE.  Even if the number of scheduled RBs of a PDSCH/PUSCH is even, there may also be orphan RB issue for each TRP. Therefore, the restriction shall be introduced for each TRP, that is the number of consecutively scheduled PRBs for PDSCH is even for each TRP. Therefore we propose:

Proposal 1: For FD-OCC length 4 in Rel.18 eType 1 DMRS for PDSCH:

- No restriction on scheduling of different UEs is needed in case of MU-MIMO

- The number of consecutively scheduled PRBs for PDSCH transmitted from each TRP is even.

MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports

In RAN1#110 meeting, it is agreed that MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports are supported. It can be used to improve scheduling flexibility. With MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports, we need design DMRS to guarantee good channel estimation performance for both legacy Rel-15/16/17 UEs and new Rel-18 UEs. For MU-MIMO by different CDM groups, there is no MU-MIMO scheduling restriction of PUSCH/PDSCH since DMRS ports in different CDM groups are orthogonal. For MU-MIMO within a CDM group, more discussion is needed for whether and how to support multiplexing between Rel.15 DMRS ports and Rel.18 DMRS ports. To enhance scheduling flexibility, Rel.15 DMRS ports and Rel.18 DMRS ports can be designed to support within a CDM group. But the enhanced schemes should be considered to reduce interference between Rel.15 DMRS ports and Rel.18 DMRS ports. For example, only subset of OCC sequences, such as [+1 +1 +1 +1] and [+1 -1 +1 -1], can be used for Rel.18 UEs. The gNB needs to choose the DMRS ports carefully to maintain orthogonality between R15 and R18 DMRS ports.

Proposal 2: Support multiplexing Rel.15 and Rel.18 DMRS ports in a same CDM group.

DMRS port indication

For demodulation of PDSCH/PUSCH with SU-MIMO or MU-MIMO transmission, the used DMRS port index is required to be indicated to UE. For legacy design, the number of DMRS CDM group(s) without data, DMRS port(s), number of front-load symbols are indicated by DCI signalling. With increasing the number of orthogonal DMRS ports, the enhanced signalling is required to indicate the used DMRS port(s). In RAN1 #111, it was agreed three categories of port combination can be indicated, where the port combination of Cat.1 consists of legacy port indexes; the port combination of Cat.2 consists of new port indexes; the port combination of Cat.3 consists of legacy port indexes and new port indexes.

The PDSCH DMRS port indication table for eType1 DMRS with maxLength=1 in single TRP case is agreed except that some entries in bracket are still FFS. In RAN1 #112, it is agree for row 0-2, 9-11, 12-14, 24-25, but there is still FFS for row 21-23.  In our view, dynamic switching between SU-MIMO and MU-MIMO transmission is used for PDSCH transmission and the new DMRS port indication table is used for both SU-MIMO and MU-MIMO transmission. Rows 21-22 can be supported to increase scheduling flexibility similar to row 12-13. For row 23, it can also be support to increase scheduling flexibility based on agreed row 11.. Thus, we prefer to include these entries in the DMRS port indication table.

Proposal 3: Support bracket entries in the row 21-23of the agreed PDSCH DMRS port indication table for eType1 DMRS with maxLength=1.

For PDSCH DMRS port indication table for eType1 or eType2 DMRS in single TRP case, the port combination of Cat.3 can be further discussed in case of 2 front-load DMRS symbols. The similar DMRS overhead can be achieved by length 4 FD OCC and combination of length 2 TD OCC and length 2 FD OCC. As an example to support 4 layers MU-MIMO, similar overhead is achieved for DMRS port combination {0, 1, 8, 9} or {0, 1, 4, 5}. Therefore, we prefer not to introduce entry to indicate port combination of Cat.3 in case of 2 front-load DMRS symbols.

Proposal 4: Do not introduce entry to indicate port combination of Cat.3 for eType1 or eType2 DMRS with 2 front-load DMRS symbols.

For single DCI based M-TRP case, it is agreed in RAN1 #112 to support row 30, i.e. {0, 2, 3} for Rel.18 eType1 DMRS ports with maxLength = 1. It is still FFS for other values, such as DMRS port combination {8,10,11}. On account of possible MU-MIMO in M-TRP case, we prefer to support DMRS port combination {8,10,11} to increase the scheduling flexibility but not to introduce additional DCI bit. There is no reserved entry in case of dmrs-Type1 and maxLength=2 for legacy DMRS port indication table 7.3.1.2.2-2A. If all the possible DMRS port combination of Cat.2 is supported, there is no available entry for indicating DMRS port combination of Cat.3 if only 1 additional bit (i.e. total 6 bits) is used for DMRS port indication. Thus, further discussion is needed for the detail of supported DMRS port combination if only 1 additional bit is introduced for M-TRP case.

Proposal 5: Support to indicate DMRS port combination {8,10,11} for eType1 DMRS ports with maxLength = 1 in case of single DCI based M-TRP transmission.

Proposal 6: Further discuss the supported DMRS port combination in case of dmrs-Type1 and maxLength=2 for M-TRP case.

eType1 DMRS for MsgA PUSCH

For MsgA PUSCH, only type 1 DMRS is supported and maximum 4/8 orthogonal uplink DMRS ports in case of maxLength=1/maxLength=2 are supported in the current release. In detail, DMRS ports associated with a PUSCH occasion are configured by parameters msgA-PUSCH-DMRS-CDM-group, msgA-PUSCH-NrofPorts, msgA-Maxlength in MsgA-DMRS-Config-r16. For eType1 DMRS, the maximum 8/16 orthogonal uplink DMRS ports can be supported for single and double-symbol Type1 DMRS in Rel.18, respectively. If eType1 DMRS with increasing number of DMRS ports is introduced for MsgA PUSCH, it can provide benefits such as higher multiplexing efficiency (i.e. more candidate UEs in one PUSCH occasion), more than 2 Msg-A PUSCH resource configuration identified by more than 2 sub-sets of DMRS port/sequence combination, lower DMRS overhead with more DMRS ports in one CDM group. It can be useful for the scenario with higher UE load with 2 step RACH procedure.

Proposal 7: Support eType1 DMRS for MsgA PUSCH.

### Power boosting for PDSCH transmission

In RAN1 #112 meeting, PTRS power boosting for PUSCH was discussed. Power boosting is also needed for PDSCH transmission. Power boosting of PTRS is only supported for up to six-layer PDSCH transmission in Rel-17, since a UE is not expected to be scheduled with DMRS with TD-OCC and PTRS in the same slot. In Rel-18, with the increased number of DMRS ports, single-symbol DMRS can support up to 8 or 12 DMRS ports for DMRS type 1 and DMRS type 2 respectively. Therefore, PTRS power boosting should be enhanced to support seven-layer or eight-layer PDSCH transmission.

Proposal 8: Study power boosting for PDSCH transmission.

## Increasing the number of orthogonal DMRS ports for more than 4 layer UL SU-MIMO

Since increasing orthogonal DMRS ports is introduced in Rel.18 as discussed in section 2.1, the newly introduced DMRS pattern can be also used for DMRS resource mapping for more than 4 layers uplink SU-MIMO transmission. It has advantage of lower DMRS overhead since Rel.18 DMRS can support 8-port with single symbol DMRS. For enhanced DMRS defined in Rel.18, the enhanced DMRS port allocation table can be used to determine DMRS port index. It can reduce standard effort to design enhanced legacy DMRS port allocation table for supporting more than 4 layers SU-MIMO. In RAN1 #111, it was also agreed to define new antenna ports tables with new DMRS port combinations for rank = 5,6,7,8. But the detail is FFS.

In Rel-17, different DMRS tables were specified for different ranks transmission to reduce the antenna port(s) field overhead, since the UE can determine the transmission rank according to the SRI or TPMI field. It means the UE shall determine a DMRS table for ‘antenna port(s)’ field interpretation based on the rank determined by the SRI/TPMI field. However, in DL, the PDSCH rank is determined by antenna port(s) field in DCI format 1_1/1_2 by combining the DMRS port(s) for different rank in a same Table. The same mechanism can be considered for 8Tx UE. Larger number of precoders will be specified for 8Tx UE with different coherent capability, according to the current progress in AI9.1.4.2, lots of companies proposed to support separate TRI and TPMI indication to simplify the precoder indication. One solution is to introduce a TRI field in the UL grant to indicate the transmission rank for the scheduled PUSCH transmission. Considering the PDSCH rank determination mechanism, TRI of the scheduled PUSCH transmission can be indicated by the antenna port(s) field by combining the DMRS port(s) for different rank in a same table as that for PDSCH. The up to 8 ports DMRS indication table can be reused for 8Tx UL transmission.

Proposal 9:  Consider to indicate the TRI of the scheduled PUSCH by the antenna port(s) field in the UE grant as that for PDSCH.

PTRS is an essential feature to support FR2 operation for phase noise estimation, one or two PTRS ports can be configured for 4TX PUSCH transmission and a DMRS port is indicated to be associated with a PTRS for phase noise estimation. Two issues should be discussed on the PTRS for 8TX PUSCH transmission, the first one is whether more than two PTRS ports are needed for 8TX with more than two coherent antenna groups. For example, two coherent antenna group corresponding to antenna layout case 2 and four coherent antenna groups corresponding to antenna layout case 3 shall be supported for 8Tx. Similar with Rel-15 PTRS design principle, it’s necessary to assign separate PTRS port to different coherent antenna group. Therefore, up to 2 PTRS ports can be configured for 8Tx UE with two antenna groups, and up to 4 PTRS ports can be required for 8Tx UE with four antenna groups.

Proposal 10: Support the following PTRS configuration for 8Tx UE in FR2

- Up to 2 PTRS ports can be configured for a UE with two antenna groups

- Up to 4 PTRS ports can be configured for a UE with four antenna groups

The other issue is the indication for the PTRS and DMRS association when more than 4 layers are scheduled for the PUSCH transmission. For example, one of the more than 4 DMRS ports should be indicated to be associated with a PTRS port when a single PTRS port is configured for a full coherent UE. In this case, the PTRS-DMRS-association field should be extended to select one of the up to 8 DMRS port(s) for the PTRS port.

When more than one PTRS ports are transmitted by the UE, the PTRS-DMRS association field can be extended similar with the table in alt.2 for single PTRS port case. An example is provided for the UE has two antenna groups.

For the case of two coherent groups, each coherent group contains 4 antenna ports, e.g., the 1st coherent antenna group contains antenna ports {0, 1, 4, 5} and the 2nd coherent antenna group contains antenna ports {2, 3, 6, 7}. Then, PUSCH antenna port 1000, 1001, 1004, 1005 share PT-RS port 0, and PUSCH antenna port 1002, 1003, 1006, 1007 share PT-RS port 1.

UL PT-RS port 0 is associated with the UL layer 'x' of layers which are transmitted with PUSCH antenna port 1000, PUSCH antenna port 1001, PUSCH antenna port 1004 and PUSCH antenna port 1005 in indicated TPMI(s), and UL PT-RS port 1 is associated with the UL layer 'y' of layers which are transmitted with PUSCH antenna port 1002, PUSCH antenna port 1003, PUSCH antenna port 1006 and PUSCH antenna port 1007 in indicated TPMI(s), where 'x' and/or 'y' are given by DCI parameter 'PTRS-DMRS association' as shown in DCI format 0_1 and DCI format 0_2 according to Table 3:

Table 3 PTRS-DMRS association for UL PTRS ports 0 and 1

| Value of the first 2 bits | DMRS port |  | Value of the last 2 bits | DMRS port |
| --- | --- | --- | --- | --- |
| 0 | 1st scheduled DMRS port which shares PTRS port 0 |  | 0 | 1st scheduled DMRS port which shares PTRS port 1 |
| 1 | 2nd scheduled DMRS port which shares PTRS port 0 |  | 1 | 2nd scheduled DMRS port which shares PTRS port 1 |
| 2 | 3rd scheduled DMRS port which shares PTRS port 0 |  | 2 | 3rd scheduled DMRS port which shares PTRS port 1 |
| 3 | 4th scheduled DMRS port which shares PTRS port 0 |  | 3 | 4th scheduled DMRS port which shares PTRS port 1 |

Proposal 11:  Extend the PTRS-DMRS association field to indicate the DMRS port associated with each PTRS port.

In RAN1 #112 meeting, PTRS power boosting for PUSCH with more than 4 layers transmission was discussed and several principles was proposed. We think the principles are well aligned with legacy PTRS power boosting and should be supported. And the factor related to PUSCH to PT-RS power ratio with PTRS to PUSCH power ratio should be derived based on these principles. For partial coherent TPMIs, it was proposed that the PTRS to PUSCH power ratio for PTRS port x is min{10log10(Lx)+10log10(Qp),10log10(L)}, where Lx is the number of PUSCH layers which are coherent with the PUSCH layer where PTRS port x is associated with. From this principle, the PTRS to PUSCH power ratio for different PTRS ports may be different. However, in the legacy system, the PTRS to PUSCH power ratio is same for each PTRS port. For example, for a three-layer PUSCH transmission, suppose the precoder is 12100100100001, even though PTRS port 0 is associated with one layer (i.e., layer 0) and PTRS port 1 is associated with two layers (i.e., layer 1 and layer 2), the PTRS to PUSCH power ratio for each PTRS port is 3*Qp-3, which is determined by assumption of one layer. Therefore, we think the Lx should be the minimum number of PUSCH layers which are precoded coherently with the PUSCH layer where different PTRS ports are associated with.

Proposal 12: Support the principles on power boosting for more than 4-layer PUSCH transmission proposed in RAN1 #112 meeting for full coherent and noncoherent case.

# Conclusion

Base on above discussion, our observations and proposals are summarized as follows:

Proposal 1: For FD-OCC length 4 in Rel.18 eType 1 DMRS for PDSCH:

- No restriction on scheduling of different UEs is needed in case of MU-MIMO

- The number of consecutively scheduled PRBs for PDSCH transmitted from each TRP is even.

Proposal 2: Support multiplexing Rel.15 and Rel.18 DMRS ports in a same CDM group.

Proposal 3: Support bracket entries in the row 21-23of the agreed PDSCH DMRS port indication table for eType1 DMRS with maxLength=1.

Proposal 4: Do not introduce entry to indicate port combination of Cat.3 for eType1 or eType2 DMRS with 2 front-load DMRS symbols.

Proposal 5: Support to indicate DMRS port combination {8,10,11} for eType1 DMRS ports with maxLength = 1 in case of single DCI based M-TRP transmission.

Proposal 6: Further discuss the supported DMRS port combination in case of dmrs-Type1 and maxLength=2 for M-TRP case.

Proposal 7: Support eType1 DMRS for MsgA PUSCH.

Proposal 8: Study power boosting for PDSCH transmission.

Proposal 9:  Consider to indicate the TRI of the scheduled PUSCH by the antenna port(s) field in the UE grant as that for PDSCH.

Proposal 10: Support the following PTRS configuration for 8Tx UE in FR2

- Up to 2 PTRS ports can be configured for a UE with two antenna groups

- Up to 4 PTRS ports can be configured for a UE with four antenna groups

Proposal 11:  Extend the PTRS-DMRS association field to indicate the DMRS port associated with each PTRS port.

Proposal 12: Support the principles on power boosting for more than 4-layer PUSCH transmission proposed in RAN1 #112 meeting for full coherent and noncoherent case.

# References

- 3GPP TSG RAN WG1 #112, "Draft Meeting Report," Athens, Greece, February 27th – March 3rd, 2023.