3GPP TSG RAN WG1 #112bis-e		R1- 2302727

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.1.3.2

Source:	Lenovo

Title:	Discussion of SRS enhancement

Document for:	Discussion

# Introduction

In Rel-18, enhancements on downlink MIMO that facilitate the use of large antenna array, for both FR1 and FR2, are needed to fulfil the demand for evolution of NR deployments. As coherent joint transmission (CJT) improves coverage and average throughput in commercial deployments with high-performance backhaul and synchronization, enhancement on CSI acquisition for FDD and TDD, targeting FR1, can be beneficial in expanding the utility of multi-TRP deployments. SRS enhancement targeting TDD coherent JT is agreed as part of MIMO enhancement, where the corresponding object is as highlighted as following in the approved WID [1]:

- Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows: - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

- Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices

  - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

In Rel-18, it is important to identify and specify necessary enhancements for uplink MIMO. As advanced UEs (e.g. CPE, FWA, vehicle, industrial devices) become more relevant, introducing necessary enhancements to support 8 antenna ports as well as 4 and more layers for UL transmission can offer the needed improvement for UL coverage and average throughput. SRS enhancement for 8Tx is agreed as part of MIMO enhancement, where the corresponding object is as highlighted as following in the approved WID [1]:

In this contribution, we provide our views on SRS enhancement targeting TDD CJT and 8 TX operation.

# Discussion on SRS enhancement targeting TDD CJT

In RAN1 #112 meeting, the following agreements were made for SRS enhancement for CJT [2]:

Agreement For SRS interference randomization, support: - Opt. 3: Both cyclic shift hopping and comb offset hopping. - At least the two features can be separately configured - FFS: Combined cyclic shift hopping and comb offset hopping for a UE - FFS: Separate or combined with SRS sequence group hopping / sequence hopping - FFS: Associated UE capability Agreement For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port, the hopping pattern is determined based on the pseudo-random sequence c(i), initialized with one of the following IDs. - Option 1: Reuse the SRS sequence identity nIDSRS. - Option 2: Introduce new ID(s). - FFS: the value range, one new ID or two separate new IDs, default ID(s) Agreement For SRS comb offset hopping and/or cyclic shift hopping, the time-domain hopping behavior depends on at least the slot index ns,fμ within a radio frame and OFDM symbol index l', and select at least one of the following options: - Option 1: Within a slot, hopping based on the repetition factor R and symbol index that is the same across the R repetitions. - Option 2: Within a slot, hopping based on only the symbol index l'. - Option 3: No intra-slot hopping. - FFS: Time domain hopping behaviour further depends on system frame number (SFN) nf. - FFS: reinitialization periodicity of N radio frames or reinitialization based on system frame number. - FFS: Whether to adopt the same option(s) for comb offset hopping and cyclic shift hopping (if supported separately) - FFS: At least support reinitialization at the beginning of each radio frame. Conclusion No consensus to support SRS TD OCC for TDD CJT SRS enhancement in Rel-18. Conclusion No consensus to support the following for TDD CJT SRS enhancement in Rel-18: - Further enhancements to frequency hopping - Sequence hopping/randomization, per-hop sequence from a long SRS sequence - Enhanced configuration of SRS transmission to enable more efficient SRS parameter assignment - Precoded SRS for DL CSI acquisition - Pseudo-random muting of SRS transmission for periodic and semi-persistent SRS - Configuration of v (sequence index within a group) per SRS resource - Multiplying mask sequence to the legacy SRS sequence

## Scenario and SRS coordination assumption for TDD CJT

For TDD systems, SRS can be used to obtain downlink CSI by exploiting channel reciprocity. In general, SRS-based DL CSI acquisition schemes have the benefit of lower CSI feedback overhead and higher CSI precision, compared with quantized PMI feedback. For cell-edge UEs, the uplink SINR and channel quality could be too low to perform SRS-based channel measurement with sufficient resolution, especially for power-limited UEs. Thus, it is important for gNB to enhance the coordination on SRS resources and manage inter-TRP cross-SRS interference.

One typical cross-SRS interference scenario is shown in Fig.1. For a UE that is associated with CJT transmission, i.e., UE1, the received SRS at the candidate TRPs for CJT, e.g., TRP1, TRP2, will be used to obtain channel estimates by one coordinated TRP, e.g. TRP1, and another coordinated TRP, e.g. TRP2. However, a concurrent SRS transmission from a neighboring UE2 to one of the candidate TRPs , e.g., TRP2, may use the same or overlapped time-frequency resources as the SRS resource for the CJT-based UE without any coordination. Under this setup, the estimated channel accuracy corresponding to the CJT-based UE1 will be degraded by TRP2, especially if the neighboring UE2 is closer to TRP2 than the CJT-based UE1 is. Therefore, SRS enhancement is needed for SRS resources from candidate TRPs to improve SRS channel estimation accuracy.

Fig.1 Example of cross-SRS interference for CJT UE

## Discussion on SRS enhancement for TDD CJT

In this section, we focus on SRS interference randomization and/or capacity enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT.

### Interference randomization for SRS enhancement

With randomized resources for SRS transmission, the inter-TRP cross-SRS interference can be randomized, which helps reduce the interference level when some resources with better orthogonality are used for cross-SRS. It can also help avoid persistent interference and provide good channel estimation performance for some transmission opportunity with smaller cross-SRS, without increasing the actual SRS overhead. This type of schemes can be used to obtain accurate downlink CSI especially when the UE load is not high.

Comb hopping and CS hopping for SRS transmission

For legacy systems, the cyclic shift nSRScs∈0,1,…,nSRScs,max-1 and transmission comb number KTC∈2,4,8 are configured by the higher-layer parameter transmissionComb. For Comb hopping and CS hopping, hopping is based on Comb value(s) and CS value(s), respectively, in different transmission occasions. Specifically, the Comb value(s) for the SRS port(s) can hop in one set, such as 0,1,…,KTC-1 for Comb hopping, and the CS value(s) for the SRS port(s) can hop in one set, such as 0,1,…,nSRScs,max-1 for CS hopping. In principle, CS hopping scheme and Comb hopping scheme are similar. In RAN1#112 meeting, it was agreed to support both CS (cyclic shift) hopping and Comb hopping for SRS interference randomization. At least CS hopping and Comb hopping can be separately configured. For supporting combined CS hopping and Comb hopping, there are different views. In our view, better interference randmomization effect can be achieved due to larger degrees-of-freedom for hopping if combined CS hopping and Comb hopping are supported. It is useful for scenarios with larger number of UEs. Also, it is more compatible for 4Tx SRS, where different Comb values and different CS values can be used for different SRS ports. For combined hopping scheme, we prefer to have simple design to make CS hopping and Comb hopping separately as much as possible. For example, for 4Tx SRS, combined CS and Comb hopping are used for the first SRS port, where there are total 16 possible hopping values (including 8 possible CS value and 2 possible Comb). For CS and Comb values for other SRS ports, they can be derived based on the specified scheme in section 6.4.1.4.2 and 6.4.1.4.3 of TS 38.211. In this way, the standard effort is expected to be limited. Thus, we prefer to support combined CS and Comb hopping due to the potential performance benefit. Regarding the concern on the impact to legacy UEs, we believe it can be handled by gNB implementation such as configuring SRS for CJT UEs and legacy UEs in different Comb and CS subsets or different OFDM symbols.

Support combined CS hopping and Comb offset hopping.

- For UE capability, the most straightforward approach would be configuring the UE to report 5 capability types as an optimized scheme, i.e. {1. Not support CS hopping or Comb hopping; 2. Support CS hopping only; 3. Support Comb hopping only; 4. Support both CS hopping and Comb hopping (without simultaneous CS and Comb hopping); 5. Support combined CS hopping and Comb hopping}. In our opinion, the difference between UE capability 4 and UE capability 5 is not significant, and hence can be unified in the same UE capability for supporting both CS hopping and Comb hopping to reduce gNB configuration complexity, i.e. {1. Not support CS hopping or Comb hopping; 2. Support CS hopping only; 3. Support Comb hopping only; 4. Support both CS hopping and Comb hopping} as UE capability candidates.

Prefer 4 UE capability types i.e. {1. Not support CS hopping or Comb hopping; 2. Support CS hopping only; 3. Support Comb hopping only; 4. Support both CS hopping and Comb hopping}  for SRS interference randomizing hopping.

In RAN1 #112 meeting, it was agreed that the hopping pattern for SRS comb offset hopping and/or cyclic shift hopping are determined based on the pseudo-random sequence c(i) initialized with a network-configured ID, where two options were listed for down selection. For option 1, SRS sequence identity nIDSRS is reused, which is a straightforward solution. For option 2, new ID(s) are introduced for Comb offset hopping and CS hopping. For one UE, only one hopping scheme is used, including separate or combined CS hopping and Comb hopping, and hence it is feasible to use only one UE specific ID. Thus, we prefer option 1 for simplicity.

Support to reuse SRS sequence identity nIDSRS as initial value of pseudo-random sequence for SRS comb offset hopping and/or cyclic shift hopping.

For time domain hopping, the granularity can be per OFDM symbol, per OFDM symbol group for one repetition in one SRS resource, per one SRS resource, or per SRS resource set. In RAN1 #112 meeting, three options were agreed for down selection on time domain hopping within a slot. For option 1, the hopping can be made per R OFDM symbols for one repetition in a SRS resource with repetition. It can maintain the same interference level for the R OFDM symbols with repetition. For option 2, Comb hopping and CS hopping can be made per OFDM symbol. For option 3, the intra-slot hopping is not supported. Among the three alternatives, we are fine with either option 1 or option 2 but prefer option 2 since it is simple and can achieve interference randomization in each SRS symbol. For the scheme of reinitializing the CS hopping and/or Comb offset hopping, a similar scheme to that of sequence and group hopping can be reused, where reinitialization is made at the beginning of each radio frame.

Support option 2, i.e. hopping based on only the symbol index within a slot.

Reinitialize the CS hopping and/or Comb offset hopping at the beginning of each radio frame.

For the hopping value set, the specific set can be configured by the network for interference coordination between TRPs. Also, it can be used for facilitating gNB realization to handle multiplexing between legacy UEs and Rel-18 UEs with Comb and/or CS hopping. The specific set can be indicated by bitmap based on maximum comb value KTC for Comb hopping and/or maximum CS value nSRScs,max for CS hopping.

Support configuring a subset of CS and/or Comb offset for CS hopping and/or Comb hopping for interference coordination between TRPs and for multiplexing with legacy UE.

- For the supported time domain behaviour of SRS with CS and/or Comb hopping, periodic and semi-persistent SRS are supported, however aperiodic SRS is still FFS. In general, CS and/or Comb hopping schemes can be used for aperiodic SRS transmission on SRS resource(s) in one or multiple SRS resource sets. It can achieve interference randomization effect for overlapping SRS transmission from different coordination area. Therefore, we support CS hopping and/or Comb hopping for aperiodic SRS.

Support CS hopping and/or Comb hopping for aperiodic SRS.

### Capacity enhancement for SRS enhancement

Enhanced power control for SRS

Proposal 2.5C: For per-TRP power control and/or power control of one or multiple SRS transmission occasions towards to multiple TRPs, for an SRS resource set, down select one from the following options:

- Option 0 (For legacy TRP-specific SRS using multiple SRS resource sets; no enhancement in Rel-18):

  - M (M >= 1) power control processes for M SRS resource sets where each of the M power control processes is based on a different UL power control parameter set (P0, alpha, and closed loop state) associated with a different DL pathloss RS

  - Each of the M SRS resource sets is for one of the M TRPs. M is generally at most 2 per current standards.

- Option 1 (For TRP-common SRS using one SRS resource set):

  - Same power control process for all SRS resources of an SRS resource set where the power control process is based on one P0 value and one closed loop state and jointly on more than one DL pathloss RS and/or more than one alpha

  - Each transmission occasion of the SRS resource is towards multiple TRPs

- Option 2 (For TRP-specific SRS using one SRS resource set):

  - M (M >= 1) power control processes for the SRS resource set where each of the M power control processes is based on a different UL power control parameter set (P0, alpha, and closed loop state) associated with a different DL pathloss RS

  - Different transmission occasions of the SRS resource can be towards different TRPs

  - Only for P/SP SRS

- Option 3 (For TRP-common and/or TRP-specific SRS using one SRS resource set):

  - M (M >= 1) power control processes for the SRS resource set where each of the M power control processes is based on one P0 value, one closed loop state, and jointly on N (N >= 1) DL pathloss RS(s) and/or N alphas, where N can be different for different power control processes

  - A transmission occasion of the SRS resource can be towards N (i.e., one or multiple) TRP(s) based on which power control process is used

  - FFS: Only for P/SP SRS or for P/SP/AP

Based on the discussion in RAN1#110b and RAN1#111, 4 options are listed under proposal 2.5C in the text box below for further study and down selection, however, no agreement has been reached as of RAN1#112.

For option 0, there is no enhancement on SRS power control scheme for CJT, where multiple SRS resource sets are used for legacy TRP-specific SRS. This has a large SRS overhead and can serve as a baseline for SRS power control enhancement for CJT. For option 2, different transmission occasions of the SRS resource can be intended to different TRPs, where a subset of an SRS resource of an SRS resource set is associated with one power control process. We do not see clear benefit compared with per-TRP power control realized by legacy scheme (i.e. option 0) under this approach. For option 1, each transmission of the SRS resource is sent towards multiple TRPs. It has lower SRS overhead than multiple SRS towards multiple TRPs. Based on the agreement in RAN1#109-e meeting, the pathloss between UE and TRPs can be different by at least {3, 6, 10}dB when SRSs sent by a UE are utilized by multiple TRPs for channel estimation. Since such large pathloss difference between the UE and multiple TRPs is possible, enhanced power control mechanism for CJT UE is needed to guarantee channel estimation quality for both TRPs when one SRS is received by multiple TRPs simultaneously. In detail, the pathloss used for SRS TX power computation is derived from a combination of multiple pathloss to multiple TRPs. The combination coefficient can be RRC signalled or fixed in the 3GPP specification. For example, the combination coefficient can be 1/M, where M is the number of configured pathloss reference RS for combination. Alternatively, the combination coefficient can be 1 for maximum pathloss among multiple pathloss and 0 for other pathloss. Based on above discussion, we support option 1 as enhanced SRS power control scheme for CJT.

Support option 1 as enhanced SRS power control scheme for CJT.

Support combination of multiple pathloss with RRC configured or fixed  combination  coefficients to derive pathloss for SRS TX power computation.

# Discussion on SRS enhancement for 8Tx UL operation

The following agreements were reached in RAN1#112:

Agreement For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s, support the 8 ports equally partitioned into s subsets with each subset having 8/s different ports. - At least s = 2 - FFS: s = 4, s = 8. - m = 2,4,8, 10,12,14, and m is a multiple of s. - Each of the m OFDM symbols has only one subset. Reuse the existing resource mapping designed for 8/s ports on each OFDM symbol. - Including frequency-domain resource allocation and mapping to cyclic shifts. FFS port indexing within the subset of 8/s ports. - FFS: down selection from existing resource mapping designs - FFS: which subset of 8/s ports are mapped onto each OFDM symbol. - FFS: the TDM factor s is configured as an explicit RRC parameter or determined implicitly from other parameters. Agreement For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy non-TDMed schemes (repetition, frequency hopping, partial sounding, or a combination thereof), - Option 2: For comb 4, do not support 4 comb offsets. Agreement For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s ≥ 2, the m OFDM symbols are adjacent, and select one of the following options regarding the TDM pattern: - Option 2-1: the s subsets of ports are mapped cyclically as {1, 2, …, s,1, 2, …, s} on the m OFDM symbols. - Option 2-2: the s subsets of ports are mapped sequentially as {1, …, 1, 2, …, 2, s, …, s} on the m OFDM symbols.

In this section, we provide our views on the enhancements on 8Tx SRS.

## 8Tx SRS for codebook

It was agreed that all the 8 ports of a SRS resource can be sounded in one or more symbols. For the case than all 8 ports are sounded in a symbol without repetition, all 8 ports are multiplexed using different comb offset and different CS values. The key issue is how to determine the CS and/or Comb offset parameter for different SRS ports. Some configuration examples are provided with different comb sizes and coherent groups. In the following examples, we assume that antenna port 1000, 1001, 1004, 1005 are within a same coherent group, and port 1002, 1003, 1006, 1007 are within another coherent group; or port 1000, 1002 are within a 1st coherent group, port 1001, 1003 are within a 2nd coherent group, port 1004, 1005 are within a 3rd coherent group and port 1006, 1007 are within a 4th coherent group:

- For KTC=2 with nSRScs,max=8, 2 Comb offset + 4 CS for 8 Tx ports are required for a SRS resources, e.g.,

  - Ports 1000, 1001, 1004, 1005 are transmitted using {nSRScs, (nSRScs+2) mod nSRScs,max, (nSRScs+4) mod nSRScs,max, (nSRScs+6) mod nSRScs,max} with kTC

  - Ports 1002, 1003, 1006, 1007 are transmitted using {nSRScs, (nSRScs+2) mod nSRScs,max, (nSRScs+4) mod nSRScs,max, (nSRScs+6) mod nSRScs,max} with (kTC+1) mod KTC

- For KTC=4 with nSRScs,max=12, the following two schemes can be used

  - Scheme 1: 2 Comb offset + 4 CS for 8 Tx ports, e.g.,

    - Ports 1000, 1001, 1004, 1005 are transmitted using {nSRScs, (nSRScs+3) mod nSRScs,max, (nSRScs+6) mod nSRScs,max, (nSRScs+9) mod nSRScs,max} with kTC

    - Ports 1002, 1003, 1006, 1007 are transmitted using {nSRScs, (nSRScs+3) mod nSRScs,max, (nSRScs+6) mod nSRScs,max, (nSRScs+9) mod nSRScs,max} with (kTC+2) mode KTC

  - Scheme 2: 4 Comb offset +4 CS for 8 Tx ports, e.g.,

    - If nSRScs < 6,

Ports 1000, 1001, 1004, 1005 are transmitted using {nSRScs, (nSRScs+3) mod nSRScs,max, (nSRScs+6) mod nSRScs,max, (nSRScs+9) mod nSRScs,max} with kTC

Ports 1002, 1003, 1006, 1007 are transmitted using {nSRScs, (nSRScs+3) mod nSRScs,max, (nSRScs+6) mod nSRScs,max, (nSRScs+9) mod nSRScs,max} with (kTC+2) mode KTC

    - If nSRScs >= 6,

Ports 1000, 1002 are transmitted using {nSRScs, (nSRScs+6) mod nSRScs,max} with kTC

Ports 1001, 1003 are transmitted using {(nSRScs+3) mod nSRScs,max, (nSRScs+9) mod nSRScs,max} with (kTC+1) mode KTC

Ports 1004, 1006 are transmitted using {nSRScs, (nSRScs+6) mod nSRScs,max} with (kTC+2) mode KTC

Ports 1005, 1007 are transmitted using {(nSRScs+3) mod nSRScs,max, (nSRScs+9) mod nSRScs,max} with (kTC+3) mode KTC

- For KTC=8 with nSRScs,max=6, 4 Comb offset+ 2 CS for 8 Tx ports, e.g.,

  - Ports 1000, 1002 are transmitted using {nSRScs, (nSRScs+3) mod nSRScs,max} with kTC

  - Ports 1001, 1003 are transmitted using {nSRScs, (nSRScs+3) mod nSRScs,max} with (kTC+2) mode KTC

  - Ports 1004, 1006 are transmitted using {nSRScs, (nSRScs+3) mod nSRScs,max} with (kTC+4) mode KTC

  - Ports 1005, 1007 are transmitted using {nSRScs, (nSRScs+3) mod nSRScs,max} with (kTC+6) mode KTC

For a SRS resource configured with 8 SRS port, considering the configurations

      - For KTC=2 with nSRScs,max=8, 2 Comb offset + 4 CS

      - For KTC=4 with nSRScs,max=12

        - 2 Comb offset + 4 CS

        - 4 Comb offset +2 CS

      - For KTC=8 with nSRScs,max=6, 4 Comb offset + 2 CS

When all 8 ports are sounded by different OFDM symbols, different SRS ports can be sounded on different OFDM symbols, e.g., by 2 or 4 OFDM symbols. For example, 8 ports are sounded by 2 OFDM symbols, each SRS resource transmission shall occupy two adjacent OFDM symbols, where SRS transmission in each of the two OFDM symbols is transmitted with 4 SRS ports, e.g., pi=0, 1, 2, 3 in the first symbol and pi=4,5,6,7 in the second symbol, and the SRS ports for the SRS transmission in the first OFDM symbol and in the second OFDM symbol are associated with different UE antenna ports. pi=0, 1, 2, 3 in the first symbol and pi=4,5,6,7 in the second symbol occupy the same comb offset and CS combinations, respectively. The UE shall determine 4 comb offset and CS values for each of the 4 ports in a OFDM symbol as that for 4 ports SRS resource.

When 8 ports are sounded by 4 OFDM symbols, each SRS resource transmission shall occupy four adjacent OFDM symbols, where SRS transmission in each of the four OFDM symbols is transmitted with 2 SRS ports, e.g., pi=0, 1 in the first OFDM symbol, pi=2, 3 in the second OFDM symbol, pi=4, 5 in the third OFDM symbol, and pi=6, 7 in the fourth OFDM symbol. SRS ports for the SRS transmission in each OFDM symbol are associated with different UE antenna ports. pi=0, 1 in the first OFDM symbol, pi=2, 3 in the second OFDM symbol, pi=4, 5 in the third OFDM symbol, and pi=6, 7 in the fourth OFDM symbol occupy the same comb offset and CS combinations, respectively. The UE shall determine 2 comb offset and CS combinations for each of the 2 ports in a OFDM symbol as that for 2 ports SRS resource.

8 ports of a SRS resource for codebook or antennaSwitch can be sounded by 2 or 4 adjacent OFDM symbol.

- When 8 ports are sounded by 2 OFDM symbols, the SRS resource shall determine 4 comb offset and CS combinations for 4 ports in each OFDM symbol as that for 4Tx SRS, and SRS ports in different OFDM symbol occupy the same comb offset and CS combinations.

- When 8 ports are sounded by 4 OFDM symbols, the SRS resource shall determine 2 comb offset and CS combinations for 2 ports in each OFDM symbol as that for 2Tx SRS, and SRS ports in different OFDM symbol occupy the same comb offset and CS combinations.

Another possible solution is to sounding 8 antenna ports in multiple SRS resources. For example, all 8 antenna ports can be sounded by two SRS resources transmitted on adjacent OFDM symbols at lease for partial-coherent and non-coherent UE. Compared with sounding 8 antennas as a single SRS resource, this offers more SRS resource allocation flexibility, especially for partial coherent UEs. Since the phase difference between different antennas within different coherent antenna groups are random, the channels corresponding to different coherent antenna groups can be sounded by different SRS resources in different symbols. The channel condition corresponding to different antennas within a same coherent antenna group are correlated and the phase difference between different antennas within a same coherent antenna group are fixed for a channel. Therefore, the channels for antennas within a same coherent antenna group should be sounded together by a same SRS resource. One example is provided as follows.

Example of sounding 8 antenna port by two SRS resource sets:

SRS resource#0: SRS antenna port 1000, 1001, 1004, 1005

SRS resource#1: SRS antenna port 1002, 1003, 1006, 1007

SRS antenna port 1000, 1001, 1004, 1005 belong to the 1st coherent antenna group are sounded by SRS resource#0 and SRS antenna port 1002, 1003, 1006, 1007 belong to the 2nd coherent antenna group are sounded by SRS resource#1. SRS ports of SRS resource 0 and SRS ports of SRS resource 1 share the same Comb offset and CS. In other words, the SRS port 0, 1, 2, 3 of SRS resource 0 are mapped to SRS antenna port 1000, 1001, 1004, 1005, and the SRS port 0, 1, 2, 3 of SRS resource 1 are mapped to SRS antenna port 1002, 1003, 1006, 1007. It means that SRS resource 0 and SRS resource 1 are associated with different antenna ports. Different from SRS for antenna switching, a gap is not need between different SRS resources and the antenna ports within a same coherent antenna group should be associated with a same SRS resource.

To quickly obtain the complete channel matrix, SRS resources for all 8 antennas sounding are better transmitted on adjacent OFDM symbols. Further, all the SRS resources for all 8 antennas sounding should be configured with the same frequency hopping pattern and repetition parameters and should perform frequency hopping and/or repetition together. For the SRI indication for PUSCH scheduling, each SRI codepoint can be associated with multiple SRS resources for all 8 antenna sounding.

Support to sound 8 antenna port with more than one SRS resources.

## 8Tx SRS for non-codebook

To support 8 Tx non-codebook based PUSCH transmission, 8 single port SRS resources shall be supported as agreed in RAN1#109. If all the 8 single-port SRS resources can be transmitted in a slot, they shall be configured within a same SRS resource set as in Rel-17. However, if the UE can only transmit SRS in the last 6 OFDM symbols in a slot and only one SRS resource can be transmitted in one OFDM symbol, 8 OFDM symbols are required for all the 8 SRS resource transmission. Then all the SRS resources within an SRS resource set cannot be transmitted in a slot. For this case, all the 8 SRS resources can be configured in two or more SRS resource sets and transmitted in different slots. All of the 8 SRS resources should be configured with a same UL TCI state and all the SRS resource sets should be associated with a same NZP CSI-RS for precoding calculation. Besides, for aperiodic SRS resource sets, all the SRS resource sets shall be triggered by a same DCI.

All the 8 single port SRS resources can be configured within one or more SRS resource sets according to UE capability.

- All the SRS  resource sets are configured with a same UL TCI state, or

- All the SRS resoruce sets have the same associatedCSI-RS resource.

The next issue is how to indicate the SRI(s) for the scheduled PUSCH transmission. One or more SRIs corresponding to one or more SRS resources can be indicated by the SRS resource indicator field similar in Rel-15. When all the 8 SRS resources are configured within a same SRS resource set, if the UE only support up to rank 4 UL transmission, 162 SRI combinations shall be indicated with Rel-15 SRI indication mechanism by define multiple SRI indication tables. To minimize the s standardization effort, bitmap based SRI indication can be introduced. For example, a 8-bit bitmap can be used for SRI indication, where each bit of the bitmap corresponds to a SRS resource.

Support bitmap based SRI indication in DCI  for non-codebook based PUSCH scheduling.

# Conclusion

In this contribution document, we discussed our views on SRS enhancement targeting CJT scenarios, as well as 8Tx UL operation. Based on the discussion points in the document, we have the following proposals:

- Support combined CS hopping and Comb offset hopping.

Prefer 4 UE capability i.e. {1. Not support CS hopping or Comb hopping; 2. Support CS hopping only; 3. Support Comb hopping only; 4. Support both CS hopping and Comb hopping}  for SRS interference randomizing hopping.

Support to reuse SRS sequence identity nIDSRS as initial value of pseudo-random sequence for SRS comb offset hopping and/or cyclic shift hopping.

Support option 2, i.e. hopping based on only the symbol index within a slot.

Reinitialize the CS hopping and/or Comb offset hopping at the beginning of each radio frame.

Support configuring a subset of CS and/or Comb offset for CS hopping and/or Comb hopping for interference coordination between TRPs and for multiplexing with legacy UE.

Support CS hopping and/or Comb hopping for aperiodic SRS.

Support option 1 as enhanced SRS power control scheme for CJT.

Support combination of multiple pathloss with RRC configured or fixed  combination  coefficients to derive pathloss for SRS TX power computation.

For a SRS resource configured with 8 SRS port, considering the configurations

      - For KTC=2 with nSRScs,max=8, 2 Comb offset + 4 CS

      - For KTC=4 with nSRScs,max=12

        - 2 Comb offset + 4 CS

        - 4 Comb offset +2 CS

      - For KTC=8 with nSRScs,max=6, 4 Comb offset + 2 CS

8 ports of a SRS resource for codebook or antennaSwitch can be sounded by 2 or 4 adjacent OFDM symbol.

- When 8 ports are sounded by 2 OFDM symbols, the SRS resource shall determine 4 comb offset and CS combinations for 4 ports in each OFDM symbol as that for 4Tx SRS, and SRS ports in different OFDM symbol occupy the same comb offset and CS combinations.

- When 8 ports are sounded by 4 OFDM symbols, the SRS resource shall determine 2 comb offset and CS combinations for 2 ports in each OFDM symbol as that for 2Tx SRS, and SRS ports in different OFDM symbol occupy the same comb offset and CS combinations.

Support to sound 8 antenna port with more than one SRS resources.

All the 8 single port SRS resources can be configured within one or more SRS resource sets according to UE capability.

- All the SRS  resource sets are configured with a same UL TCI state, or

- All the SRS resoruce sets have the same associatedCSI-RS resource.

Support bitmap based SRI indication in DCI  for non-codebook based PUSCH scheduling.

# References

| [1] | RP-213598, Samsung, "New WID: MIMO Evlution for Downlink and Uplink," Online, Dec. 6-17, 2021. |
| --- | --- |
| [2] | 3GPP TSG RAN WG1 #112, "Draft Meeting Report," Athens, Greece, February 27th – March 3rd, 2023. |