3GPP TSG RAN WG1 Meeting #112bis-e		R1-2302735

e-Meeting, April 17th – 26th, 2023

Agenda Item:                9.3.1

Source:                         MediaTek Inc.

Title: 	                           Discussion on evaluation of NR duplex evolution

Document for:              Discussion and Decision

# Introduction

A study item on evolution of NR duplex operation was approved in RAN#94e with the following objective ‎[1]:

- Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

In this contribution, we examine the Subband Full Duplexing (SBFD) evaluation results in comparison to the legacy Time Division Duplexing (TDD) and make some important observations from these evaluations.

# SBFD Evaluation Results

An analysis of the performance of SBFD compared to TDD is conducted based on Deployment Case 1 of SBFD. The FR1 UMa scenario is used for this comparison. Full buffer and FTP3 traffic are taken into consideration to make key observations.

## Full Buffer Traffic

### SLS validation with TDD and SBFD

Some subbands of TDD DL slots are allocated as UL resources to enable SBFD. This forms the SBFD format as ‘XXXXU’, where 20% of the RBs are allocated to UL, with the subband configuration < ND, NU, NG > = <104, 54, 5>. Thus, some DL resources are sacrificed to provide UL resources, leading to a theoretical calculation of resource gain for UL and resource loss for DL. This can be compared with the expected gain/loss of spectral efficiency of SBFD compared to TDD. This evaluation can be considered as a good validation method for the system level simulator (SLS).

Table 1: Comparison of resource gain and spectral efficiency gain

From Table 1, the DL and UL resource gain of SBFD (XXXXU) compared to TDD (DDDSU) are examined in relation to their corresponding spectral efficiency gain. As expected, a clear correlation is seen between resource gain and spectral efficiency.

- The resource gain/loss in UL and DL, as well as the spectral efficiency gain/loss in UL and DL for SBFD over the legacy TDD, are closely aligned.

### Impact of inter-UE CLI in Downlink

In the legacy TDD, UL and DL transmissions take place in distinct time slots, meaning there is no risk of interference between UL and DL signals from neighbouring UEs. However, with SBFD, UL and DL transmissions occur during the same time slot at the gNB, leading to Inter-UE CLI, as the UL transmission of one UE negatively affects the reception of DL signal at another nearby UE.

In traditional simulations, UEs are uniformly and randomly distributed, leading to a scenario where the distances between UEs are relatively large. As a result, the effect of Inter-UE CLI is not captured, as the separation distance is too big to account for it. This uniform random distribution also does not represent a real-world scenario, where users with multiple mobile devices typically group together in places like meeting rooms, restaurants and public transport, with much smaller separation distances than those depicted in the simulation. Thereby, clustered UE dropping is agreed [2] to study the inter-UE CLI.

The UE clustering distribution for the baseline setup is M=20 and X=2, while the optional setup is M=10 and X=1. R is the cluster radius, which is 25m for the evaluation and the results for other values of R may be submitted. Dmacro-to-cluster is the minimum distance between a macro gNB and the cluster, which is 35m+ R, and Dinter-cluster is the minimum distance between two clusters, which is 2R m. UEs within the clusters are indoors, with a mobility of 3km/h, while UEs outside the clusters are outdoors in a car with a mobility of 30km/h. The UE height is 1.5m for all UEs.

In this section, the DL interferences such as co-channel interference and inter-UE cross link interference are analysed.

Fig. 1: CCI over CLI ratio CDFs for UEs’ deployment with and without clustering

Fig. 1 illustrates the ratio of CCI to Inter-UE CLI for clustered (with radius of 25m and 10m) and uniformly distributed deployments. The CDF plots demonstrate that the addition of the clusters leads to an increase in CLI over the CCI.

- Inter-UE CLI has significant impact to the DL performance in clustered UE scenario.

### Impact of intra/inter-gNB CLI and SI in UL

In the uplink interference analysis, co-channel interference (CCI) and total leakage powers with two capabilities (RCIC and RSIC) are compared. RCIC is the ratio of co-site interference cancellation, while RSIC is the ratio of self-interference cancellation. Furthermore, the frequency isolation in all cases is modelled by the adjacent channel interference model (ACIR) with an ACLR of 45dB and ACS of 43dB. The RSIC capability is assumed to be 102 dB, which is considered the best value when accounting for a 1dB receiver sensitivity loss.

Fig. 2: Comparison of CCI and total leakage power in UL

From Fig. 2, it is evident that the UL reception is significantly impacted by the RCIC capability. In this analysis, the RSIC capability is assumed as 102 dB which is considered to be the best value. It is derived by assuming 1dB receiver sensitivity loss.

- Intra-site gNB CLI has significant impact to the UL performance.

### Spectral Efficiency

The spectral efficiency of both DL and UL are compared in the following cases.

- Baseline case: without modelling of any leakage power (ideal SBFD)

- With modelling of only inter-site gNB CLI in UL and inter-UE CLI in DL

- With modelling of inter/intra-site gNB CLI and gNB SI in UL, and inter-UE CLI in DL

  - RSIC = 0dB and RSIC = 0dB

  - RSIC = 102dB and RSIC = 75dB

  - RSIC = 102dB and RSIC = 100dB

Table 2 shows the absolute values of spectral efficiency (SE) and Fig. 3 shows the SE loss of SBFD due to the different cross-link interferences and gNB self-interference.

Table 2: Spectral Efficiency (absolute values)

|  | w/o CLI and SI (Baseline) | with inter-site gNB CLI in UL and inter-UE CLI in DL | with inter-site-CLI + co-site CLI + gNB-SI in UL and inter-UE CLI in DL |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | RSIC = 0 dB, RCIC = 0 dB | RSIC = 102 dB, RCIC = 75 dB | RSIC = 102 dB, RCIC = 100 dB |
| DL | 4.14 | 3.27 | 3.35 | 3.35 | 3.31 |
| UL | 2.70 | 2.26 | 1.17 | 1.63 | 2.17 |

Fig. 3: Spectral Efficiency loss comparison for different cases

Fig. 3 reveals that the DL SE is degraded by 20% due to the high inter-UE CLI, and UL SE is degraded by 16% due to the inter-site gNB CLI. Additionally, the UL spectral efficiency is heavily influenced by the RSIC and RCIC capabilities. Since DL reception at the UE is unaffected by these gNB capabilities, the DL SE remains unchanged for the other cases.

- Intra-site isolation has a major influence on UL performance, and it is observed that 75 dB of isolation is insufficient, and 100 dB is preferred in addition to the frequency isolation.

- RSIC of 102 dB in addition to the frequency isolation is required to mitigate the gNB self-interference.

## FTP3 Traffic

### UPT and Latency Performances of SBFD over TDD

The user perceived throughput (UPT) performances of the SBFD and the legacy TDD are analysed in all 3 RU conditions namely low (~10%), Medium (~27%) and High (~50%). This is done using FTP model 3 with 0.5Mbyte and 0.125Mbyte as DL and UL packet size. Here, the best gNB capabilities (RSIC=102dB, RCIC=100dB) are assumed. Table 3 shows the absolute values of UPT and Fig. 4 shows the UPT gain of SBFD over TDD.

When comparing DL UPT and packet latency of SBFD and TDD, SBFD suffers due to the DL resource loss and inter-UE CLI. On the other hand, UL UPT and packet latency of SBFD has a better performance than TDD, due to the UL resource gain with the best RSIC and RCIC capabilities of gNB. To further improve the UPT performance, a good scope of studying the solutions for the inter-site gNB CLI management with minimal or no impact to the UE can be done.

Fig. 4: UPT gain of SBFD over TDD

Table 3: UPT (absolute values)

- DL UPT of SBFD suffers (~43% loss in mean UPT of medium RU) when compared to TDD due to DL resource loss and inter-UE CLI.

- UL UPT of SBFD is gained (~69% gain in mean UPT of medium RU) when compared to TDD due to UL resource gain and with the best RSIC and RCIC capabilities at gNB.

- Good scope of studying the solutions for the inter-site gNB CLI management with minimal or no impact to the UE.

Table 4 shows the absolute values of packet latency and Fig. 5 shows the latency reduction of SBFD over TDD.

Fig. 5: Latency reduction of SBFD over TDD

Table 4: Packet Latency (absolute values)

From Fig. 5, when comparing DL latency of SBFD and TDD, SBFD is worse off due to the DL resource loss and inter-UE CLI. In contrast, UL latency of SBFD has an advantage compared to TDD, due to the UL resource gain with the best RSIC and RCIC capabilities of gNB.

- DL latency of SBFD suffers (~124% loss in mean packet latency of medium RU) when compared to TDD due to DL resource loss and inter-UE CLI.

- UL latency of SBFD is gained (~43% loss in mean packet latency of medium RU) when compared to TDD due to UL resource gain and with the best RSIC and RCIC capabilities at gNB.

# Conclusion

In this contribution, we discussed the SBFD evaluation results in comparison to the legacy TDD. We have the following observations:

- The resource gain/loss in UL and DL, as well as the spectral efficiency gain/loss in UL and DL for SBFD over the legacy TDD, are closely aligned.

- Inter-UE CLI has significant impact to the DL performance in clustered UE scenario.

- Intra-site gNB CLI has significant impact to the UL performance.

- Intra-site isolation has a major influence on UL performance, and it is observed that 75 dB of isolation is insufficient, and 100 dB is preferred.

- RSIC of 102 dB in addition to the frequency isolation is required to mitigate the gNB self-interference.

- DL UPT of SBFD suffers (~43% loss in mean UPT of medium RU) when compared to TDD due to DL resource loss and inter-UE CLI.

- UL UPT of SBFD is gained (~69% gain in mean UPT of medium RU) when compared to TDD due to UL resource gain and with the best RSIC and RCIC capabilities at gNB.

- Good scope of studying the solutions for the inter-site gNB CLI management with minimal or no impact to the UE.

- DL latency of SBFD suffers (~124% loss in mean packet latency of medium RU) when compared to TDD due to DL resource loss and inter-UE CLI.

- UL latency of SBFD is gained (~43% loss in mean packet latency of medium RU) when compared to TDD due to UL resource gain and with the best RSIC and RCIC capabilities at gNB.

# References

- RP-213591, “New SI: Study on evolution of NR duplex operation”, CMCC, RAN#94e, December 2021.

- Chairman notes, RAN1#110

# Appendix: Simulation assumptions in SLS evaluation

Table 5: Simulation assumptions in SLS for SBFD evaluation.

| Parameter set |  | Urban Macro (FR1) |
| --- | --- | --- |
| A. General | Carrier frequency | 4 GHz |
|  | System bandwidth | 100 MHz |
|  | Numerology | 14 OFDM symbol slot, SCS = 30kHz |
| B. Layout and UE distribution | BS Layout | Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around |
|  | Wrap-round | distance-based |
|  | Inter-BS (2D) distance | 500m |
|  | BS antenna height | 25m |
|  | UE distribution | UE clustering distribution - (Baseline) M=20, X=2 - R' = 25m - 8 UE per cluster per direction - Dmacro-to-cluster = 35m+R' - Dinter-cluster = 2R' m (Optional) Uniform UE distribution -10 users per macro TRP per direction |
|  | Minimum BS-UE (2D) distance | 35m |
|  | Minimum UE-UE (2D) distance | 1m |
|  | UE outdoor/indoor proportion | 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h |
|  | UE height (m) | For UE clustering distribution: 1.5m For Uniform UE distribution: Outdoor UE : 1.5 m; Indoor UE : 3(nfl – 1) + 1.5; nfl ~ uniform(1, Nfl) where Nfl ~ uniform(4,8) |
| C. Interference modelling | gNB self-interference - αSI | based on 1 dB UL desense |
|  | Co-site inter-sector co-channel inter-subband CLI | Spatial isolation: 75dB (RAN4 typical value) / 93dB (RAN4 best value) / 100dB Digital cancelation: companies to report, e.g., 10dB |
|  | BS ACLR | 45 dBc |
|  | BS ACS | 46 dBc |
|  | UE ACLR | 30 dBc |
|  | UE ACS | 33 dBc |
|  | UE IBE | Refer to Annex A.2.X in TR 38.858 |
|  | UE ICS | Not Modelled |
| D. SBFD subband and slot configurations | SBFD slot configuration | Legacy TDD: {DDDSU}, where S=[10D:2G:2U] SBFD: Frame structure#2 (XXXXU) |
|  | SBFD Subband configuration | {DUD}: < ND, NU, NG > = <104, 54, 5> |
|  | Guard symbols |  |
| E. Tranmit power | BS transmit power for legacy TDD | 53 dBm for 100MHz |
|  | BS transmit power for SBFD | Power boosting is not assumed for SBFD symbols compared to DL-only symbols (as in legacy systems) |
|  | UE Tx power | 23dBm |
| F. Antenna configurations | BS antenna configuration for legacy TDD | (M,N,P,Mg,Ng;Mp,Np) = (8,8,2,1,1;2,8) , (dH,dV) = (0.5, 0.8)λ, +45°/-45° polarization |
|  | BS antenna configuration for SBFD | Twice area&same TxRUs (higher priority): SBFD antenna configuration Option 2 |
|  | BS antenna radiation pattern | Table 9 in Report ITU-R M.2412 |
|  | UE antenna configuration | 2Tx: (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1), (dH,dV) = (N/A, N/A)λ, 0°,90° polarization 4Rx: (M,N,P,Mg,Ng;Mp,Np) = (1,2,2,1,1;1,2), (dH,dV) = (0.5, N/A)λ, 0°,90° polarization |
| G. Traffic model | DL/UL traffic assignment for the same UE | Option 2: Each UE is assigned both UL traffic and DL traffic |
|  | DL/UL FTP packet size | 0.5Mbyte for DL and 0.125 Mbytes for UL |
|  | DL/UL traffic load for legacy TDD | Low load (Type-2 RU: < 10%) Medium load (Type-2 RU: 20%-40%) High load (Type-2 RU: >= 50%) |
| H. Channel model | gNB-UE | Refer to Annex A.3 in TR 38.858 |
|  | gNB-gNB | Option2: Large scale fading only |
|  | UE-UE | Refer to Annex A.3 in TR 38.858 Option2: Large scale fading only |
|  | UE-UE details | TR 38.901 |
| I. Others | BS receiver noise figure | 5dB |
|  | UE receiver noise figure | 9 dB |
|  | Open loop power control parameters | P0= -80 dBm, alpha = 0.8 |
|  | UE receiver | MMSE-IRC |
|  | Feedback assumption | Realistic |
|  | Channel estimation | Ideal |
|  | UE processing capability | UE processing capability 1 as baseline |
|  | Handover margin (dB) | 3 dB |
|  | UE attachment | Based on RSRP from port 0 |
|  | Transmission scheme | SU-MIMO, maximum layers for SU-MIMO = 2 |
|  | Polarized antenna model | Model-1 in clause 7.3.2 in TR 38.901 |
|  | DL/UL Modulation | Up to 256QAM |
|  | Scheduling | PF |
|  | Overhead | DL: 27.38% UL: 14.9% |