3GPP TSG-RAN WG1 Meeting #112b	R1-2302740

e-meeting, April May 17 – 26, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.211 | CR | xxxx | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of dynamic spectrum sharing enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_DSS_enh-Core |  |  |  |  |  | Date: |  |  | 2023-03-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introduction of dynamic spectrum sharing enhancements |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of additional rate matching parameters |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete support for Rel-18 enhancements to dynamic spectrum sharing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.2.2, 7.4.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 7.3.2.2	Control-resource set (CORESET)

A control-resource set consists of NRBCORESET resource blocks in the frequency domain and NsymbCORESET∈1,2,3 symbols in the time domain.

A control-channel element consists of 6 resource-element groups (REGs) where a resource-element group equals one resource block during one OFDM symbol. Resource-element groups within a control-resource set are numbered in increasing order in a time-first manner, starting with 0 for the first OFDM symbol and the lowest-numbered resource block in the control resource set.

A UE can be configured with multiple control-resource sets. Each control-resource set is associated with one CCE-to-REG mapping only.

The CCE-to-REG mapping for a control-resource set can be interleaved or non-interleaved and is described by REG bundles:

-	REG bundle  is defined as REGs  where L is the REG bundle size, i=0,1,…,NREGCORESETL-1, and NREGCORESET=NRBCORESETNsymbCORESET is the number of REGs in the CORESET

-	CCE  consists of REG bundles  where  is an interleaver

For non-interleaved CCE-to-REG mapping, L=6 and fx=x.

For interleaved CCE-to-REG mapping, L∈2,6for NsymbCORESET=1 and L∈NsymbCORESET,6 for NsymbCORESET∈2,3. The interleaver is defined by

fx=rC+c+nshift mod NREGCORESETL
x=cR+r
r=0,1,…,R-1
c=0,1,…,C-1
C=NREGCORESET(LR)

where .

The UE is not expected to handle configurations resulting in the quantity C not being an integer.

For a CORESET configured by the ControlResourceSet IE:

-	NRBCORESET is given by the higher-layer parameter frequencyDomainResources;

-	NsymbCORESET is given by the higher-layer parameter duration, where NsymbCORESET=3 is supported only if the higher-layer parameter dmrs-TypeA-Position equals 3;

-	interleaved or non-interleaved mapping is given by the higher-layer parameter cce-REG-MappingType;

-	L equals 6 for non-interleaved mapping and is given by the higher-layer parameter reg-BundleSize for interleaved mapping;

-	R is given by the higher-layer parameter interleaverSize;

-	nshift∈0,1,…,274 is given by the higher-layer parameter shiftIndex if provided, otherwise nshift=NIDcell;

-	for both interleaved and non-interleaved mapping, the UE may assume

-	the same precoding being used within a REG bundle if the higher-layer parameter precoderGranularity equals sameAsREG-bundle;

-	the same precoding being used across the all resource-element groups within the set of contiguous resource blocks in the CORESET, and that no resource elements in the CORESET overlap with an SSB or LTE cell-specific reference signals as indicated by the higher-layer parameter lte-CRS-ToMatchAround, lte-CRS-PatternList1, lte-CRS-PatternList2, lte-CRS-PatternList3, or lte-CRS-PatternList4 if the higher-layer parameter precoderGranularity equals allContiguousRBs.

For CORESET 0 configured by the ControlResourceSetZero IE:

-	NRBCORESET and NsymbCORESET are defined by clause 13 of [5, TS 38.213];

-	the UE may assume interleaved mapping

-	L=6;

-	R=2;

-	nshift=NIDcell;

-	the UE may assume normal cyclic prefix when CORESET 0 is configured by MIB or SIB1;

-	the UE may assume the same precoding being used within a REG bundle.

##### 7.4.1.1.2	Mapping to physical resources

The UE shall assume the PDSCH DM-RS being mapped to physical resources according to configuration type 1 or configuration type 2 as given by the higher-layer parameter dmrs-Type.

The UE shall assume the sequence  is scaled by a factor  to conform with the transmission power specified in [6, TS 38.214] and mapped to resource elements k,lp,μ according to

where , , and Δ are given by Tables 7.4.1.1.2-1 and 7.4.1.1.2-2 and the following conditions are fulfilled:

-	the resource elements are within the common resource blocks allocated for PDSCH transmission

The reference point for  is

-	subcarrier 0 of the lowest-numbered resource block in CORESET 0 if the corresponding PDCCH is associated with CORESET 0 and Type0-PDCCH common search space and is addressed to SI-RNTI;

-	otherwise, subcarrier 0 in common resource block 0

The reference point for  and the position  of the first DM-RS symbol depends on the mapping type:

-	for PDSCH mapping type A:

-	 is defined relative to the start of the slot

-	if the higher-layer parameter dmrs-TypeA-Position is equal to 'pos3' and  otherwise

-	for PDSCH mapping type B:

-	 is defined relative to the start of the scheduled PDSCH resources

-

The position(s) of the DM-RS symbols is given by  and duration ld where

-	for PDSCH mapping type A, ld is the duration between the first OFDM symbol of the slot and the last OFDM symbol of the scheduled PDSCH resources in the slot

-	for PDSCH mapping type B, ld is the duration of the scheduled PDSCH resources

and according to Tables 7.4.1.1.2-3 and 7.4.1.1.2-4.

For PDSCH mapping type A

-	the case dmrs-AdditionalPosition equals to 'pos3' is only supported when dmrs-TypeA-Position is equal to 'pos2';

-	ld=3 and ld=4 symbols in Tables 7.4.1.1.2-3 and 7.4.1.1.2-4 respectively is only applicable when dmrs-TypeA-Position is equal to 'pos2';

-	single-symbol DM-RS, l1=11 except if all of the following conditions are fulfilled in which case l1=12:

-	the higher-layer parameter lte-CRS-ToMatchAround, lte-CRS-PatternList1, lte-CRS-PatternList2, lte-CRS-PatternList3, or lte-CRS-PatternList4 is configured; and

-	the higher-layer parameter dmrs-AdditionalPosition is equal to 'pos1' and l0=3; and

-	the UE has indicated it is capable of additionalDMRS-DL-Alt

For PDSCH mapping type B

-	if the PDSCH duration ld ∈2,3,4,5,6,7,8,9,10,11,12,13 OFDM symbols for normal cyclic prefix or ld∈2,4,6 OFDM symbols for extended cyclic prefix, and the front-loaded DM-RS of the PDSCH allocation collides with resources reserved for a search space set associated with a CORESET,  shall be incremented such that the first DM-RS symbol occurs immediately after the CORESET and until no collision with any CORESET occurs, and

-	if the PDSCH duration ld is 2 symbols, the UE is not expected to receive a DM-RS symbol beyond the second symbol;

-	if the PDSCH duration ld is 5 symbols and if one additional single-symbol DMRS is configured, the UE only expects the additional DM-RS to be transmitted on the 5th symbol when the front-loaded DM-RS symbol is in the 1st symbol of the PDSCH duration, otherwise the UE should expect that the additional DM-RS is not transmitted;

-	if the PDSCH duration ld is 7 symbols for normal cyclic prefix or 6 symbols for extended cyclic prefix:

-	if one additional single-symbol DM-RS is configured, the UE only expects the additional DM-RS to be transmitted on the 5th or 6th symbol when the front-loaded DM-RS symbol is in the 1st or 2nd symbol, respectively, of the PDSCH duration, otherwise the UE should expect that the additional DM-RS is not transmitted;

-	if the PDSCH duration ld ∈5,6,7,8,9,10,11,12,13 OFDM symbols, the UE is not expected to receive the front-loaded DM-RS beyond the 4th symbol;

-	if the PDSCH duration ld is 12 or 13 symbols, the UE is not expected to receive DM-RS mapped to symbol 12 or later in the slot;

-	for all values of the PDSCH duration ld other than 2, 5, and 7 symbols, the UE is not expected to receive DM-RS beyond the (ld-1):th symbol;

-	if the PDSCH duration ld is less than or equal to 4 OFDM symbols, only single-symbol DM-RS is supported.

-	if the higher-layer parameter lte-CRS-ToMatchAround, lte-CRS-PatternList1, lte-CRS-PatternList2, lte-CRS-PatternList3, or lte-CRS-PatternList4 is configured, the PDSCH duration ld=10 symbols for normal cyclic prefix, the subcarrier spacing configuration μ=0, single-symbol DM-RS is configured, and at least one PDSCH DM-RS symbol in the PDSCH allocation collides with a symbol containing resource elements as indicated by the higher-layer parameter lte-CRS-ToMatchAround, lte-CRS-PatternList1, lte-CRS-PatternList2, lte-CRS-PatternList3, or lte-CRS-PatternList4, then l shall be incremented by one in all slots.

The time-domain index l' and the supported antenna ports p are given by Table 7.4.1.1.2-5 where

-	single-symbol DM-RS is used if the higher-layer parameter maxLength in the DMRS-DownlinkConfig IE is not configured

-	single-symbol or double-symbol DM-RS is determined by the associated DCI if the higher-layer parameter maxLength in the DMRS-DownlinkConfig IE is equal to 'len2'.

In absence of CSI-RS configuration, and unless otherwise configured, the UE may assume PDSCH DM-RS and SS/PBCH block to be quasi co-located with respect to Doppler shift, Doppler spread, average delay, delay spread, and, when applicable, spatial Rx parameters. Unless specified otherwise, the UE may assume that the PDSCH DM-RS within the same CDM group are quasi co-located with respect to Doppler shift, Doppler spread, average delay, delay spread, and spatial Rx (when applicable). The UE may assume that DMRS ports associated with a TCI state as described in clause 5.1.6.2 of [6, TS 38.214] of a PDSCH are QCL with QCL Type A, Type D (when applicable) and average gain.

The UE may assume that no DM-RS collides with the SS/PBCH block.

Table 7.4.1.1.2-1: Parameters for PDSCH DM-RS configuration type 1.

| p | CDM group | Δ |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1000 | 0 | 0 | +1 | +1 | +1 | +1 |
| 1001 | 0 | 0 | +1 | -1 | +1 | +1 |
| 1002 | 1 | 1 | +1 | +1 | +1 | +1 |
| 1003 | 1 | 1 | +1 | -1 | +1 | +1 |
| 1004 | 0 | 0 | +1 | +1 | +1 | -1 |
| 1005 | 0 | 0 | +1 | -1 | +1 | -1 |
| 1006 | 1 | 1 | +1 | +1 | +1 | -1 |
| 1007 | 1 | 1 | +1 | -1 | +1 | -1 |

Table 7.4.1.1.2-2: Parameters for PDSCH DM-RS configuration type 2.

| p | CDM group | Δ |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1000 | 0 | 0 | +1 | +1 | +1 | +1 |
| 1001 | 0 | 0 | +1 | -1 | +1 | +1 |
| 1002 | 1 | 2 | +1 | +1 | +1 | +1 |
| 1003 | 1 | 2 | +1 | -1 | +1 | +1 |
| 1004 | 2 | 4 | +1 | +1 | +1 | +1 |
| 1005 | 2 | 4 | +1 | -1 | +1 | +1 |
| 1006 | 0 | 0 | +1 | +1 | +1 | -1 |
| 1007 | 0 | 0 | +1 | -1 | +1 | -1 |
| 1008 | 1 | 2 | +1 | +1 | +1 | -1 |
| 1009 | 1 | 2 | +1 | -1 | +1 | -1 |
| 1010 | 2 | 4 | +1 | +1 | +1 | -1 |
| 1011 | 2 | 4 | +1 | -1 | +1 | -1 |

Table 7.4.1.1.2-3: PDSCH DM-RS positions  for single-symbol DM-RS.

| ld in symbols | DM-RS positions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PDSCH mapping type A |  |  |  | PDSCH mapping type B |  |  |  |
|  | dmrs-AdditionalPosition |  |  |  | dmrs-AdditionalPosition |  |  |  |
|  | pos0 | pos1 | pos2 | pos3 | pos0 | pos1 | pos2 | pos3 |
| 2 | - | - | - | - |  |  | l0 | l0 |
| 3 |  |  |  |  | l0 | l0 | l0 | l0 |
| 4 |  |  |  |  |  |  | l0 | l0 |
| 5 |  |  |  |  | l0 | l0,4 | l0,4 | l0,4 |
| 6 |  |  |  |  |  |  | l0,4 | l0,4 |
| 7 |  |  |  |  |  |  | l0,4 | l0,4 |
| 8 |  | , 7 | , 7 | , 7 | l0 | l0, 6 | l0,3, 6 | l0,3, 6 |
| 9 |  | , 7 | , 7 | , 7 | l0 | l0,7 | l0, 4, 7 | l0, 4, 7 |
| 10 |  | , 9 | , 6, 9 | , 6, 9 | l0 | l0, 7 | l0, 4, 7 | l0, 4, 7 |
| 11 |  | , 9 | , 6, 9 | , 6, 9 | l0 | l0, 8 | l0, 4, 8 | l0, 3, 6, 9 |
| 12 |  | , 9 | , 6, 9 | , 5, 8, 11 | l0 | l0,9 | l0,5, 9 | l0, 3, 6, 9 |
| 13 |  | , l1 | , 7, 11 | , 5, 8, 11 | l0 | l0,9 | l0,5, 9 | l0, 3, 6, 9 |
| 14 |  | , l1 | , 7, 11 | , 5, 8, 11 | - | - | - | - |

Table 7.4.1.1.2-4: PDSCH DM-RS positions  for double-symbol DM-RS.

| ld in symbols | DM-RS positions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PDSCH mapping type A |  |  | PDSCH mapping type B |  |  |
|  | dmrs-AdditionalPosition |  |  | dmrs-AdditionalPosition |  |  |
|  | pos0 | pos1 | pos2 | pos0 | pos1 | pos2 |
| <4 |  |  |  | - | - |  |
| 4 |  |  |  | - | - |  |
| 5 |  |  |  | l0 | l0 |  |
| 6 |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |
| 8 |  |  |  | l0 | l0,5 |  |
| 9 |  |  |  | l0 | l0,5 |  |
| 10 |  | , 8 |  | l0 | l0,7 |  |
| 11 |  | , 8 |  | l0 | l0,7 |  |
| 12 |  | , 8 |  | l0 | l0,8 |  |
| 13 |  | , 10 |  | l0 | l0,8 |  |
| 14 |  | , 10 |  | - | - |  |

Table 7.4.1.1.2-5: PDSCH DM-RS time index l' and antenna ports p.

| Single or double symbol DM-RS | l' | Supported antenna ports p |  |
| --- | --- | --- | --- |
|  |  | Configuration type 1 | Configuration type 2 |
| single | 0 | 1000 – 1003 | 1000 – 1005 |
| double | 0, 1 | 1000 – 1007 | 1000 – 1011 |