3GPP TSG-RAN1 #112-bis-e	              R1-2302746

17th–26th April 2023

Source:	NEC

Title:	Discussion on subband non-overlapping full duplex

Agenda Item:	9.3.2

Document for: 	Discussion and Decision

# Introduction

In RAN #94-e meeting, a new SI on evolution of NR duplex operation was approved. The objective of the SI is listed as follows [1].

The objective of this study is to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum. In this study, the followings are assumed: - Duplex enhancement at the gNB side - Half duplex operation at the UE side - No restriction on frequency ranges The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4). Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

Further in RAN1#109-e [2], RAN1#110 [3] and RAN1#110-bis-e [4], following agreements were made for Sub-band Non-Overlapping Full Duplex (SBFD):

In this contribution, we discuss and provide views for subband non-overlapping full duplex.

# Discussion

## SBFD resource configuration

In Rel-18, for SBFD operation, it is assumed that gNB would be able to schedule transmissions with different UL and DL directions in non-overlapped sub-bands for different UEs in a cell at a time. This SBFD operation is expected to improve UL coverage, latency reduction and system capacity as compared to legacy TDD systems. However, to implement SBFD operation, various resource configuration aspects of SBFD operation need to be clarified.

For traditional NR TDD, the time domain resources are split between downlink and uplink for both gNB and UE either by semi-static TDD configuration (e.g., tdd-UL-DL-ConfigurationCommon/tdd-UL-DL-ConfigurationDedicated) and/or by dynamic TDD configuration (e.g., SFI in DCI format 2_0). Since, the TDD configuration can be dedicatedly provided to UE, gNB can potentially schedule DL and UL for different UEs at the same time achieving functionality of SBFD without requiring any signaling enhancements.

However, there are certain advantages of indicating SBFD resources to the UEs which can support SBFD operation. Using the SBFD resource configuration, many UL-DL resource conflicts can be resolved at the UE side with relatively simple implementations. For e.g., semi-static DL receptions (like CSI-RS) can be ignored by UE in the UL sub-band resources which are indicated by the gNB using SBFD configuration, to improve channel estimation. In absence of the SBFD resource indication to the UE, gNB would need to use DCI based preemption/puncturing to resolve any such DL/UL conflict during SBFD occasions. This would not only result in higher PDCCH overhead but would also lead to reduced UE/gNB performance in case UEs are not able to timely/correctly decode DCI indications.

Observation 1 Full duplex operation can be currently achieved in NR by providing different tdd-UL-DL-ConfigurationDedicated or SFI to different UEs in a cell, however, higher DL/UL performance gains are expected if UEs are indicated of SBFD time/frequency resources

Although it was agreed in RAN1#110-bis-e meeting to support configuration of at least UL subband for SBFD-aware UEs, it is still not agreed whether dynamic configuration of SBFD slot/symbols is supported. We believe that semi-static configuration of SBFD resources cannot fully address the requirement of dynamic traffic variations. The network should have the flexibility to reduce/increase the number of SBFD symbols in the cell based on the UL/DL traffic arrival. Hence, supporting dynamic configuration of SBFD slots/symbols is beneficial from network performance perspective.

Observation 2 Dynamic configuration of SBFD slots/symbols is beneficial for improving network performance when considering dynamic DL/UL traffic requirements

To achieve the dynamic SBFD configuration, there seem to be 2 possible mechanisms:

- Option-1: gNB to dynamically convert semi-statically configured UL subband time/frequency into DL transmissions. This would be beneficial when transient conditions require high DL resources and there are no scheduled/pending UL transmissions.

- Option-2: gNB to dynamically indicate additional UL subband resources (in addition to semi-statically configured UL subband resources) when there is a need for urgent UL scheduling (e.g. for low latency services or for UL scheduling a UE with coverage issues)

From our understanding, the functionality of the second option can be accomplished by using first option i.e., network can configure high number of semi-static UL subband occasions initially (to cater for increased UL traffic) and then reduce the number of actual UL subband occasions dynamically using the methodology described in the first option to cater for increase in DL traffic. So, we believe that Option-1 is sufficient for addressing all the use cases to be addressed in Rel-18 for SBFD scheduling.

Moreover, there was significant discussion in RAN1#112 on validity of DL transmissions outside DL subband for a symbol configured as DL in TDD-UL-DL-ConfigCommon and whether UL (or DL) transmissions can be performed outside UL (or DL) subband for a symbol configured as flexible in TDD-UL-DL-ConfigCommon. Both points are addressing the same issue mentioned above and hence, we provide the following proposal to address the discussion.

Proposal 1:

- gNB can convert a SBFD symbol configured as DL in the TDD-UL-DL-ConfigCommon dynamically to a DL-only symbol

  - An SBFD-aware UE which receives this indication assumes that UL subband is not present in the symbol

- gNB can convert a SBFD symbol configured as F in the TDD-UL-DL-ConfigCommon dynamically to a DL-only or an UL-only symbol

  - An SBFD-aware UE which receives this indication assumes that UL subband is not present in the symbol

Another agreement made in RAN1#110-bis is the study of SBFD operation during SSB occasions:

Study whether SBFD operation in SSB symbols is supported or not.

Given that SSBs are required by UEs not only for initial access but also for link recovery procedures, although the proponents of the scheme indicate that supporting SBFD within the SSB symbols can allow UL transmissions for the UEs which are not monitoring SSBs. Although we sympathize with the argument but many network implementations are not aware of the UE behavior for SSB reception, i.e., whether at a SSB occasion which UEs are trying to receive the SSB and which UEs are not. Hence, to be safe network in such a case should assume that UEs are monitoring SSBs to avoid any UE performance issues. Therefore, it does not seem beneficial to support SBFD operation during SSB symbols as UEs would not be able to perform UL transmission simultaneously with SSB reception. Thereby we need to ensure that SBFD operation do not conflict with UE’s SSB reception.

Proposal 2:

- Scheduled/configured SBFD transmissions should not overlap with SSB occasions

Given that SSB periodicity (20/40ms) is expected to be less frequent than TDD periodicity (<=10ms), if we configure SBFD occasions within tdd-UL-DL-ConfigurationCommon such that they avoid SSB occasions, then we might end up with a significant number of DL slots where we cannot schedule SBFD occasions. Consider the example below:

- SSBs transmitted within a time window of 2ms with repetition of 20ms

- TDD periodicity of 5ms with DDDUU configuration (15kHz SCS)

- In this case, if we try to avoid SSB occasions by using network configuration of SBFD (with same periodicity as TDD), then we end of with only 4ms duration out of 20ms where UL subband can be configured leaving 8ms duration of DL slots which cannot be used for SBFD (refer to figure below)

Figure 1 SBFD time occasion configuration to avoid SSB occasions.

Above example indicates that DL slots/symbols cannot be fully utilized for SBFD occasions using legacy network configuration. Hence, we need to define mechanisms on how to avoid configuration of SBFD transmissions during SSB occasions but also maximizing the number of available SBFD occasions. One natural mechanism to accomplish this is by specifying the UE behavior that any SBFD slot/symbol which overlaps with SSB occasions should be considered invalid by the UE. In this case, network can configure SBFD/UL subband occasions which may be overlapping with SSB occasions, but UE intelligently can discard any SBFD occasions which conflict with its SSB reception.

Proposal 3:

- Specify UE behavior for discarding SBFD occasions which partially/fully overlap with SSB occasions of the UE

In RAN1#111, we agreed to support indication of UL/DL subband using common resource blocks as baseline as shown below:

Agreement

For the purpose of RAN1 study, the understanding is that for semi-static configuration of subband frequency locations for SBFD operation, frequency location of UL/DL subband is with reference to CRB grid.

This agreement raises some questions with respect to the operation with multiple BWP operation. For e.g., when UE is configured with different BWPs, then UE will need to determine the DL and UL frequency allocation based on UL/DL subband CRB configuration and individual BWP configuration which is active at UE. For this, we may need to consider different scenarios of subband configuration and how does the configuration translate into different BWP pairs. Consider Figure 2 below for 3 such scenarios. Here, in scenario-1, UL subband is constrained within the middle BWP pair (BWP2), while in scenario-2, UL subband extends beyond the middle BWP (BWP2). The scenario-3 provides the case where each BWP has either UL or DL subband but not both.

Figure 2 Subband frequency locations wrt configured BWP pairs

From 3GPP standards perspective, each scenario should be studied, and potential resource allocation issues should be resolved. For instance, in scenario-2 and scenario-3, we need to enhance physical layer procedures where UE may perform only UL in legacy DL symbol when the UE is operating in BWP2. In such cases, if DL transmission needs to be performed to UE, then BWP may need to be switched. Further, in scenario-1 and scenario-2, we also need to consider the minimum required subband frequency resource availability in the BWP for the communication in the given subband to be feasible. For e.g., if UL subband resource in BWP1 for scenario-2 is very small (e.g., a few PRBs) then it may not be possible to schedule UL for the UE in UL subband. Ideally, network configuration should avoid such scenario (i.e., available subband frequency resources are a few PRBs within a BWP), however whether any specification support is required for this needs further discussion.

Proposal 4:

- Study the impact of different subband resource allocation scenarios with respect to multiple BWP configuration to UE

Note that scenario-3 itself covers the aspect of SBFD under multiple BWP operation, which was earlier discussed, in RAN1#110 (as included below), for further study. Our understanding here is that center frequencies for a DL and UL BWP pair are aligned.

For SBFD operation within a TDD carrier, study SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies as baseline.

- FFS feasibility and potential benefit of SBFD scheme within a single configured DL and UL BWP pair with unaligned center frequencies

- FFS feasibility and potential benefit of SBFD scheme with more than one configured DL and UL BWP pair with aligned/unaligned center frequencies for a DL and UL BWP pair

## Support of Multiple Antenna Panels

RAN1#109e also discussed the concept of multiple antenna panels for SBFD operation. For this it is assumed that a gNB operates with multiple antenna panels. It is argued that during SBFD operation, if separate antenna panels are used for UL receptions and DL transmission at gNB (i.e., one antenna panel for UL and other antenna panel for DL), then it would result in reduced gNB self-interference. The interference is expected to reduce further with greater spatial separation between the antenna panels. Whereas, during normal DL-only or UL-only time occasions, both antenna panels can operate in single transmission direction. We believe that this scheme can provide higher performance gains.

Proposal 5:

- Study application of multiple antenna panels for SBFD operation at gNB, where different antenna panels are used for UL reception and DL transmission by gNB

Considering that many operators and network vendors currently use single antenna panel for NR operation, the concept of antenna elements-based isolation can also be extended for the case of single antenna panel. For e.g., a single antenna panel with L antenna elements can be visualized as 2 separate logical antenna panels with L/2 elements each and then we can use the same set of approaches as used for multiple panels to reach the desired isolation. Although, the feasibility of such a scenario needs to be evaluated in evaluation agenda, but such a mechanism will allow deployment of SBFD with smaller cost.

Figure 3 SBFD operation using single antenna panel

Proposal 6:

- For SBFD slot/symbols, study simultaneous Tx and Rx using different sets of antenna elements belonging to single antenna panel

There are different mechanisms discussed on how to use different antenna panels for simultaneous Tx and Rx, some of them were captured in evaluation agenda item in RAN1#110 where 3 such options were discussed. Here, we will mainly concentrate on Option-1 which is described below for reference:

SBFD antenna configuration option-1 (same as Opt 1 in RAN1#109 agreement): The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD. The total number of TxRUs of the antenna array for SBFD is the same as the total number of TxRUs of the antenna array for legacy TDD.

Figure 4 SBFD antenna configuration option-1

As compared to other options specified in the evaluation study, this option allows utilization of all antenna elements even when gNB is operating in DL-only or UL-only slots/symbols, hence this option seems more efficient.

Observation 3 Among different options considered for SBFD operation using multiple antenna panels, Option-1 most efficiently utilizes the available antenna elements.

To support this option, we need to keep in mind that different number of antenna elements are used for DL in DL-only slot/symbol and SBFD slot/symbol and same is also true for UL. Therefore, the transmission parameters (e.g., codebook, channel estimates) are expected to be different between SBFD slots/symbols and UL-only/DL-only slots/symbols. From our understanding, this is a crucial point to be studied in 3GPP as it might impact the CSI-RS transmission and reporting procedures to account for the differences between UL-only/DL-only slots/symbols and SBFD slots/symbols. For e.g. we cannot implicitly assume that channel estimates determined assuming full set of antenna elements in DL-only slots would be equally applicable for SBFD slots/symbols due to reduction in number of antenna elements for SBFD slots/symbols.

Moreover, there has been similar discussion ongoing for Network Energy Savings (NES) WI, where spatial adaptation configuration is being considered. The rational is to have different set of antenna elements available for different spatial adaptation configuration. This is like what is being discussed for SBFD multi-panel operation, hence similar fundamentals are also applicable (for e.g., CSI-RS enhancements). Hence, it is preferable to also align the discussions as much as possible and identify the difference which should be covered for SBFD.

Observation 4 Availability of different number of antenna elements between DL-only and SBFD symbols is expected to cause channel estimation issues between the symbol types and hence may require enhancements to CSI-RS configuration and measurements.

Proposal 7:

- Study enhancements on CSI including measurement and report, and signaling to enable SBFD operation using multiple antenna panels

Further, reduction in number of antenna panels during SBFD slots/symbols would also imply that PUSCH/PUCCH decoding performance might suffer as compared to UL-only slots/symbols. This can occur due to variety of reasons (e.g., change in receiving beamforming pattern, reduction in receiving diversity) dependent on how the antenna elements are used in network implementations. Hence, this signifies the requirement to have more robust UL receiving procedures for SBFD slots/symbols as compared to UL-only slots/symbols. One potential solution can be that different power control parameters to be used between SBFD slots/symbols and UL-only slots and symbols to ensure that PUSCH/PUCCH performance remains stable irrespective of the slot/symbol type.

Observation 5 Availability of different number of antenna elements between UL-only and SBFD symbols is expected to cause gNB receive power difference for UL transmissions between the two symbol types and hence UL power control mechanism should be studied for SBFD.

Proposal 8:

- Study different power control parameters between SBFD and non-SBFD symbols to improve UL reception performance during SBFD symbols

## Timing Alignment Issues for SBFD

In legacy TDD system, a UE is provided a cell-specific TA offset NTA,offset. It can be provided in SIB1 and if not, UE assumes TA offset of 25600Tc or 39936Tc defined in Table 7.1.2-2 in 38.133. The cell-specific TA offset is used to reserve enough time for UL/DL switching as illustrated below in Figure 5.

Figure 5 Cell specific TA offset operation in NR

For SBFD symbols, if UE applies this TA offset value, then it will cause inter-symbol/inter-slot interference as shown in below figure. To address this issue, several companies have indicated configuring TA offset as 0 for UL transmission performed during SBFD symbols. However, as shown in ‘b’ of Figure 6, we can see that keeping 0 TA offset also does not solve the problem as it allows no margin for transition from UL to DL for the gNB. Hence, to solve this problem, there is a need to provide a guard period after UL transmission in an SBFD symbol (in addition to keeping TA offset as 0) to allow sufficient margin time for gNB to switch its RF components from UL to DL appropriately.

Observation 6 Even if timing alignment offset for UL transmission is set as 0 for the UL transmissions in the SBFD symbol, gNB may not get sufficient margin to switch from UL in SBFD to DL in the next symbol.

Figure 6 Different scenarios for TA offset for SBFD

Proposal 9:

- Study SBFD specific timing alignment offset to mitigate inter-symbol interference

- A guard period may need to be defined after the end of UL transmission in an SBFD symbol to allow sufficient margin time for gNB to switch from UL to DL

## Impact on Physical Layer Procedures due to SBFD

### 2.4.1 Semi-static physical channels/signal

In existing NR specification, UEs assume that entire cell/BWP bandwidth is available for either UL or DL transmission direction during TDD UL/DL slots indicated. However, this assumption is not valid for the case of SBFD where the cell bandwidth is split into UL and DL sub-bands in RB-set based SBFD operation. For any dynamic transmission e.g., PDSCH/PUSCH, gNB can appropriately allocate resource in the DL or UL sub-band based on existing procedure and there is not expected to be any conflict in receiving/transmitting these channels. However, for the case of semi-static physical channels (like CSI-RS/PUCCH/SRS), existing procedures (rate matching/puncturing/pre-emption) are not sufficiently suitable to avoid conflicts where UL (or DL) is performed by UE within DL (or UL) sub-band.

Observation 7 For semi-static physical channels following conflicts may occur during SBFD time/frequency resources: 
a) UE may try to receive DL receptions (e.g., CSI-RS) within UL sub-bands resulting in incorrect channel estimation or reduced DL performance; 
b) UE may try to perform UL transmissions (e.g., SRS) within DL sub-bands resulting in increased interference to nearby UE receiving DL

Current semi-static procedures for FDRA may not work to address the above conflict for the case when same transmission resource (e.g., CG/CSI-RS/SRS) can occur either in SBFD slots/symbols or in legacy UL-only/DL-only slots/symbols because of different radio resource available between the slot/symbol types. And we should not rely on dynamic pre-emption indications due to frequent occurrences of SBFD slot/symbol types.

There was discussion in RAN#112 whether for some of these configured transmissions, the transmissions should be restricted to SBFD symbols or non-SBFD symbols only or whether transmissions should be allowed to be performed in both SBFD and non-SBFD symbols. Please see the relevant discussion below for reference:

Agreement:

For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols)

- Study the following options for SBFD-aware UEs:

  - Option 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only

  - Option 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols

- UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols include the following:

  - PDSCH/PUSCH/PUCCH repetitions

  - SPS PDSCH/CG PUSCH

  - TBoMS

  - Multi-PUSCH/PDSCH scheduled by a single DCI

  - Periodic/semi-persistent SRS/CSI-RS/PUCCH

  - PDCCH

Given that SBFD operation (once implemented) shall occupy significant proportion of the slots/symbols, hence if certain configured transmissions (like CSI-RS, SRS, PUCCH) are only restricted to certain slots/symbol types then we would not be utilizing these channels and SBFDS properly. Consider the case slots format DXXXU and if PUCCH is only allowed to be transmitted to UL-only symbols then we are not properly utilizing the SBFD potential. Hence, our preference is to allow transmission/reception of configured channels to both SBFD and non-SBFD symbols.

Proposal 10:

- For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols)

  - For SBFD-aware UEs, the transmissions/receptions can be in SBFD symbols and non-SBFD symbols

Further to support such procedure without causing any conflicts, we need to support the mechanism of SBFD specific radio resource configuration. That is, such channels/signals use one set of frequency resources when using UL-only/DL-only slots/symbols and use a second set of frequency resources for SBFD slots/symbols. The need for such enhancement becomes even more prudent considering that we may eventually agree on dynamic update of slot/symbol type between SBFD and DL-only and hence physical channels/signals should use appropriate configuration based on slot/symbol type.

Proposal 11:

- Study support of 2-set of frequency resource configurations for semi-static UL/DL channels/signals where the first set is used during UL-only/DL-only slots/symbols and the second set is used for SBFD slots/symbols. Semi-static UL/DL channels/signals include at least:

  - CSI-RS

  - PUCCH

  - SRS

  - Configured grant and SPS

### 2.4.2 PUSCH/PDSCH transmission

Similar issue is also applicable for the case of multi-slot PUSCH/PDSCH transmission which span both SBFD slots(s)/symbol(s) and DL-only/UL-only slot(s)/symbol(s). Here, the multi-slot PUSCH/PDSCH transmission includes the PUSCH/PDSCH repetition transmission and one DCI scheduling multiple PUSCH/PDSCH transmission and TBoMS cases. For multi-slot PUSCHs scheduled by single DCI, UE determines whether the corresponding PUSCH is transmitted based on the time domain resource allocation of the PUSCH and TDD configuration. In case the PUSCH overlaps with invalid symbol, e.g., DL symbol configured by tdd-UL-DL-ConfigurationCommon/tdd-UL-DL-ConfigurationDedicated, the PUSCH transmission is cancelled. If subband non-overlapping full duplex is configured, as shown in the figure below, when multiple PUSCHs scheduled by a single DCI are indicated to be transmitted cross symbols/slots with different duplex type, e.g., UL symbols/slot only and SBFD symbols/slot with UL sub-band, how to perform the UL transmission should be clarified. This issue has also been agreed to study in RAN1#111.

Figure 7 Example for UL transmission scheduled on SBFD symbol with UL sub-band

Given that SBFD is mainly introduced to improve the UL coverage aspects and hence it is important that we allow multi-slot PUSCH to use the SBFD slots for UL transmission even if the span of multi-slot PUSCH may contain both UL-only and SBFD slots. However, to not introduce any complex behaviors, it should be under network’s control whether UL transmission for a multi-slot PUSCH can also be performed during SBFD slots or not.

Proposal 12:

- Multi-slot PUSCH/PDSCH spanning SBFD and UL-only/DL-only slots can be transmitted on both SBFD slots and UL-only/DL-only slots

  - FFS whether additional network control is required to disable/enable transmission on SBFD slots

Another aspect which needs to be discussed for SBFD is how to perform FDRA for various UL and DL channels. Currently, Type-1 PDSCH and CORESET frequency configuration can have the optional functionality of interleaving of frequency resources, while PUSCH allows for frequency hopping mechanism. Both set of procedures are crucial for robust network operation as they ensure necessary diversity in transmission/reception and this improving radio performance. We think that these procedures should be equally applicable to SBFD operation as well. However, given that interleaving and frequency hopping procedure can result in allocation of frequency resources which fall outside the intended subband (UL or DL) resource configuration, we need to define enhanced mechanism on how these procedures can work properly within SBFD slots/symbols.

Given that RAN1#112 already made an agreement to study frequency hopping mechanism for SBFD, we think that frequency interleaving mechanism should also be discussed.

Proposal 13:

- Study how to support frequency interleaving procedures for PDSCH and CORESTET for SBFD slots/symbols

One option considered during RAN1#112 is to configure an offset value specific for SBFD symbols. However, even with considering different offset based enhancements, it is still possible that the frequency resources selected for UL transmission may fall outside the UL subband bandwidth. Consider the RSstart calculation for intra-slot frequencybelow defined in TS 38.214. Similar calculation is also applicable for inter-slot hopping.

If we follow the same calculation for SBFD operation, then (RBstart +RBoffset) mode NBWP can still fall outside the UL subband bandwidth as shown in Figure 4 irrespective of value chosen for frequency offset value. Hence, even with frequency offset enhancement there would still be a need to

Figure 8 Frequency hoppping issues for SBFD

One potential way to address this by configuring an RB offset if the frequency allocation or hopping fall into the other subband resource or has some frequency overlapping with the other subband. Other option is to update the calculation of RBstart to also consider the UL subband bandwidth size.

Proposal 14:

- Study enhancements to frequency hopping offset and starting RB calculation for performing UL transmission in an SBFD symbol

Furthermore, for PUSCH/PDSCH Type 0 FDRA, as some RBs of the allocated RBG may overlap with the DL subband or UL subband or guardband, then it may need to support fractional RBGs allocation, otherwise some new RBG division method to ensure the allocated RBG boundary aligned with the DL/UL subband or guardband boundary should be studied. There was discussion in RAN1#112 for this issue, where two set of options were considered:

  - Option-1:

    - Part of the DL RBG inside the DL subband can be used

    - Part of the UL RBG inside the UL subband can be used

  - Option 2:

    - Part of the DL RBG inside the DL subband cannot be used

    - Part of the UL RBG inside the UL subband cannot be used

From our understanding, Option-2 does not provide any benefit as it reduces radio resource utilization. Also, there do not seem to be much implementation gain as well from Option-2 as NR currently supports partial RBG allocation.

Proposal 15:

- For SBFD-aware UEs, study the at least following options for resource allocation in frequency-domain in case of unaligned boundaries between RBG and SBFD subbands. For an RBG that overlaps the subband boundary,

  - Part of the DL RBG inside the DL subband can be used

  - Part of the UL RBG inside the UL subband can be used

### 2.4.3 PDCCH/CORESET

The frequency allocation issues are also applicable for CORESET configuration. Although, CORESET configuration in NR is flexible where network can configure the frequency resources to be used for a CORESET, there is still a need for enhancement in CORESET frequency allocation for the case of SBFD. In absence of any CORESET frequency allocation enhancements, network vendor will always need to configure the CORESET frequency resources within the DL subband portion to avoid CORESET overlap with the UL subband in SBFD slots/symbols. This is inefficient as network will not be able to utilize the frequency resources within the UL subband potion during non-SBFD slots hence reducing the PDCCH capacity. From our understanding, we can increase CORESET efficiency by keeping SBFD specific and non-SBFD specific CORESET configuration at the same time such that UE selects the CORESET frequency resources depending on the symbol type. For example,  two CORESET/SS groups/sets can be configured for a UE, and a new parameter, such as CORESETGroupIdList  can give a list of CORESET/SS for each CORESET/SS group, and the CORESET/SS in the first CORESET/SS group with the allocated frequency resource can overlap with the UL subband and guardband, and the CORESET/SS in the second CORESET/SS group with the allocated frequency resource cannot overlapped with the UL subband and guardband, and UE can switch or adapt the PDCCH monitoring  based on the DCI indication or based on whether the symbols is SBFD symbols or not.

Proposal 16:

- Support of 2-set of frequency resource configurations for CORESET associated with UE specific search space for SBFD-aware UEs, where the first set is used during UL-only/DL-only slots/symbols and the second set is used for SBFD slots/symbols.

However, such enhancements may not be applicable for CORESETs associated with common search space which is shared with legacy UEs. For such CORESETs, we can only rely on Rel-17 CORESET configuration and may need to specify behavior from network/UE side to configure CORESET resources during SBFD operation. Hence, independent study is required to specify solutions for supporting CORESETs associated with common search space for SBFD.

Proposal 17:

- Study UE/network behavior for configuring CORESETs associated to common search space during SBFD operation.

### 2.4.4 PUCCH

For PUCCH, how to configure the PUCCH resource not overlapping with DL subband in SBFD symbols should also be discussed. And how to ensure the frequency hopping does not overlap with DL subband can also be studied if inter/intra slot frequency hopping is also supported for SBFD. There are two options for PUCCH configuration for SBFD symbols.

Option1: The configured PUCCH resource set and PUCCH resource list in each set is common or shared between SBFD symbols and non-SBFD symbols, and additional configurations (such as power control, FDRA, PHY priority, spatialRelationInfo) is added for each PUCCH resource ID used for PUCCH transmission on SBFD symbols.

Option 2: A special PUCCH resource set, such as PUCCH-ResourceSet-SBFD can be configured for the SBFD aware UE, and a list of PUCCH resource are included in this set for UE to determine the PUCCH resource on the SBFD symbols.

For option 2, if separate PUCCH resource is configured for SBFD symbols, then some enhancement for the UCI multiplexing and HARQ-ACK codebook generation can also be considered for SBFD symbols. Therefore, further study or evaluation are needed to determine which option is adopted for PUCCH configuration in SBFD symbols.

Besides, for PUCCH repetition transmission, two allocation parameters set (including TPC, PRI, and beam) can be included in the DCI/RRC for PUCCH transmission on SBFD symbols and non-SBFD symbols, otherwise some defer or dropping rule should be defined if the SBFD resource is not available or overlapped with DL subband for the duration of the PUCCH repetition transmission.

Proposal 18:

- Study the PUCCH resource configuration for SBFD symbols should consider the following aspects

  - Inter/inter-slot frequency hopping

  - UCI multiplexing and HARQ-ACK codebook generation

### 2.4.5 CSI-RS

For CSI-RS, there are two set of issues being discussed in 3GPP, one is related to CSI-RS frequency configuration considering UL subband frequency resources and other is related to CSI report transmission during SBFD operation.

For CSI-RS resource configuration, the argument is that due to UL subband placement, a set of frequency resources are not available for CSI-RS resources, but UEs assume that CSI-RS resources are continuous is frequency domain. To address this problem, two options are being discussed:

  - Option 1: Two contiguous CSI-RS resources that are linked

  - Option 2: One CSI-RS resource

    - Option 2-1: Non-contiguous CSI-RS resource allocation

    - Option 2-2: One contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL subband (s)

Having one CSI-RS resource for the two DL subbands avoids the complicated discussion of number of CSI-RS resources that UE can support measuring whose rules would need to be updated based on whether SBFD operation is running. Hence, we support Option-2 to make the discussion simple. Also, between Option-2-1 and Option-2-2, we have a slight preference towards Option-2-2 as the UE would be expected to be aware of DL subband frequency resources and hence it can autonomously determine the valid CSI-RS resources based on legacy CSI-RS resource configuration and configuration of UL/DL subband.

Proposal 19:

- Study the frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs considering one CSI-RS resource as baseline

Further, for CSI report, the CSI/PMI values included within the CSI report, when the corresponding measurement was performed during SBFD symbol, should only include the measurements corresponding to DL subband resources to reduce CSI report overhead. Hence, UE should transmit the CSI report by removing any entries corresponding to the UL subband when CSI measurement if performed during SBFD symbols. If UE and gNB are in sync on SBFD symbol location, we have an understanding that gNB would be aware of the CSI transmitted by the UE and hence no other enhancement would be required.

Proposal 20:

- UE should only include the reporting quantities (e.g. CSI/PMI) associated with the DL subband resources when the corresponding CSI measurement are performed during SBFD symbol

### 2.4.6	SBFD operation for initial access

It has been agreed to study SBFD operation for RRC_CONNECTED state, and whether to support SBFD operation in RRC_IDLE and RRC_INACTIVE states such as supporting the SBFD operation for initial access are still being discussed. For this issue, we also observe that SBFD operation for initial access (e.g., 4-step or 2-step RACH) may offer some potential benefits. For example, if PRACH and PUSCH for msg3/msgA transmitted on the UL subband in the SBFD symbols, then the RACH capacity can be increased and the initial access latency can be reduced. Besides, if the PRACH and PUSCH for msg3/msgA repetition transmission can cross the SBFD symbols and non-SBFD symbols, then the UL coverage can be improved. Therefore, we propose to study potential enhancement of initial access for SBFD operation. The study include at least the following aspects.

- How to configure the RO on SBFD symbols and how to determine the validity of the RO.

- The association relationship between SSB index and the RO in SBFD symbols.

- Power ramping for the PRACH re-transmission on the SBFD symbols.

- Msg3 PUSCH frequency hopping.

- PRACH/PUSCH repetition transmission across SBFD symbols and non-SBFD symbols.

Proposal 21:

- Study potential enhancements of initial access for SBFD operation.

### 2.4.7 Other aspects

Lastly, we also need to study the UE behavior for SBFD slots/symbols, where UE can potentially receive both UL as well as DL transmissions. For example, the UE may be scheduled with CSI-RS resource in the time occasion where it is also scheduled a PUSCH using DCI. Note that many conflicts like these can be addressed using careful network configurations, but there may be some scenarios specifically where a configured DL (or UL) transmission can conflict with DCI scheduled UL (or DL) transmission. We need to at least identify these scenarios and specify UE behavior to resolve these conflicts properly.

Proposal 22:

- Identify the scenarios for UE where configured DL (or UL) transmissions may conflict with dynamic DCI scheduled UL (or DL) transmissions and specify UE behavior to resolve such conflicts.

## CLI Impacts for sub-band non-overlapping full duplex operation

In NR Rel-16, CLI handling, and RIM were introduced for interference management. While for SBFD, different interference issues may occur. For example, for a cell with SBFD, cross link interference between UEs using different subbands in a cell may happen, which has not been handled in prior studies. We need to study different mechanisms on how to address these interference issues based on frequency isolation or using enhanced interference measurement mechanism or using intelligent scheduling decisions (e.g. UL and DL UEs to be scheduled in different beams).

Further, there is an issue for self-interference at gNB arising from DL transmission from the gNB interfering with its UL receptions in adjacent sub-band. This is mainly arising due to significant power difference between DL and UL near gNB which is likely to degrade the gNB’s receiver sensitivity. There is a need to study whether available self-interference mitigation techniques at gNB are sufficient to resolve this interference issue or any standard based changes are required for enabling efficient interference management.

In addition, when gNB with SBFD mode and another gNB with DL-dominated TDD configuration are adjacently deployed for different application scenarios, there may be cross link interference from gNB #2 to gNB #1, as shown in the figure. To achieve the benefits of sub-band non-overlapping full duplex, the inter-UE CLI in a cell and inter-gNB CLI should be studied in Rel-18 and mechanisms need to be identified to mitigate the interference. When gNB #1 with sub-band full duplex mode is deployed in a factory for URLLC service, while gNB #2 with DL-dominant TDD configuration is deployed outside the factory for eMBB service, considering the requirements of different services are different, the traffic characteristics served by gNBs should be taken into the interference management to improve system performance.

Observation 8 Following interference scenarios are possible for sub-band non-overlapping full duplex operation: 
a) CLI between UEs of same cell i.e. UL transmission of one UE interfering with the DL reception of nearby UE
b) gNB experiencing interference in UL reception due to its DL transmission in adjacent sub-band
c) gNB experiencing interference in UL reception due to DL transmission from nearby gNB

Figure 9 Example for CLI between gNBs with different duplex modes

Proposal 23:

- In Rel-18, for sub-band non-overlapping full duplex at the gNB side, study interference management for

  - inter-UE CLI in a cell

  - self-interference management for gNB

  - inter-gNB CLI

Proposal 24:

- In Rel-18, for sub-band non-overlapping full duplex at the gNB side, traffic characteristics served by gNBs should be taken into the interference management.

Also, to mitigate CLI effectively for gNB-gNB CLI, it is beneficial to study information exchange between gNBs. There are already discussions for dynamic TDD which are based on assumption that coordinated scheduling is possible if gNBs are aware of contentious slots/symbols. While in semi-static case, nearby gNBs shall support common SBFD configuration but given that 3GPP is discussing different aspects of dynamic SBFD scheduling (e.g., by DL transmission in UL subband or dynamic UL subband scheduling), there are expected to be frequent cases where we observe co-subband gNB DL to gNB UL CLI.

In such cases, CLI can be properly mitigated if gNBs share between each other the SBFD configuration used by each of the gNBs. This can be used to identify the slots/symbols and frequency resources where interference is likely to deteriorate the radio performance and take appropriate action based on the mechanisms being considered in dynamic TDD section.

Proposal 25:

- For inter-gNB CLI mitigation, gNBs exchange with each other the UL subband frequency resource configuration and SBFD time occasions

For inter-subband CLI, besides the guardband configuration to suppress the CLI, some measurement and reporting scheme like for the case of intra-subband CLI can also be studied. Besides, as the inter-subband CLI is non-uniform, i.e., CLI is stronger at the edge of the subband that is adjacent to another subband and weaker for RBs that are further away from an adjacent subband, different measured bandwidths can be adopted for different frequency area to mitigate or suppress the CLI.

Furthermore, UE-to-UE inter/intra subband CLI measurement report size for SBFD operation can also be studied. For example, for subband CSI-RS e.g., RSRP/RSSI/SINR report, the size of the report subband can equal to the configured SBFD subband size. Or RB set based CLI-CSI report can be considered to report the inter-subband CLI for different RB set in the DL subband for help gNB to determine the schedule bandwidth for PDSCH. Wherein, the RB set size for the RB set based CLI report can be configured based on the DL subband size. Moreover, different type of CLI-CSI report can be defined for intra-cell inter-subband UL-DL interference, the inter-cell inter-subband UL-DL interference and the inter-cell intra-subband UL-DL interference, and the report bit sequence in the UCI with the priority rules for mapping should be defined.

Proposal 26:

- Study the CSI report enhancement for SBFD operation and different type of CLI interference

- Consider the non-uniform CLI bandwidth in inter-subband CLI measurement/report

# Conclusion

In this contribution, we discussed SBFD for NR duplex operation. Observation and Proposals are summarized as following:

Observation 1 Full duplex operation can be currently achieved in NR by providing different tdd-UL-DL-ConfigurationDedicated or SFI to different UEs in a cell, however, higher DL/UL performance gains are expected if UEs are indicated of SBFD time/frequency resources

Observation 2 Dynamic configuration of SBFD slots/symbols is beneficial for improving network performance when considering dynamic DL/UL traffic requirements

Observation 3 Among different options considered for SBFD operation using multiple antenna panels, Option-1 most efficiently utilizes the available antenna elements.

Observation 4 Availability of different number of antenna elements between DL-only and SBFD symbols is expected to cause channel estimation issues between the symbol types and hence may require enhancements to CSI-RS configuration and measurements.

Observation 5 Availability of different number of antenna elements between UL-only and SBFD symbols is expected to cause gNB receive power difference for UL transmissions between the two symbol types and hence UL power control mechanism should be studied for SBFD.

Observation 6 Even if timing alignment offset for UL transmission is set as 0 for the UL transmissions in the SBFD symbol, gNB may not get sufficient margin to switch from UL in SBFD to DL in the next symbol.

Observation 7 For semi-static physical channels following conflicts may occur during SBFD time/frequency resources: a) UE may try to receive DL receptions (e.g., CSI-RS) within UL sub-bands resulting in incorrect channel estimation or reduced DL performance; b) UE may try to perform UL transmissions (e.g., SRS) within DL sub-bands resulting in increased interference to nearby UE receiving DL

Observation 8 Following interference scenarios are possible for sub-band non-overlapping full duplex operation:  a) CLI between UEs of same cell i.e. UL transmission of one UE interfering with the DL reception of nearby UE b) gNB experiencing interference in UL reception due to its DL transmission in adjacent sub-band c) gNB experiencing interference in UL reception due to DL transmission from nearby gNB

Proposal 1:

- gNB can convert a SBFD symbol configured as DL in the TDD-UL-DL-ConfigCommon dynamically to a DL-only symbol

  - An SBFD-aware UE which receives this indication assumes that UL subband is not present in the symbol

- gNB can convert a SBFD symbol configured as F in the TDD-UL-DL-ConfigCommon dynamically to a DL-only or an UL-only symbol

  - An SBFD-aware UE which receives this indication assumes that UL subband is not present in the symbol

Proposal 2:

- Scheduled/configured SBFD transmissions should not overlap with SSB occasions

Proposal 3:

- Specify UE behavior for discarding SBFD occasions which partially/fully overlap with SSB occasions of the UE

Proposal 4:

- Study the impact of different subband resource allocation scenarios with respect to multiple BWP configuration to UE

Proposal 5:

- Study application of multiple antenna panels for SBFD operation at gNB, where different antenna panels are used for UL reception and DL transmission by gNB

Proposal 6:

- For SBFD slot/symbols, study simultaneous Tx and Rx using different sets of antenna elements belonging to single antenna panel

Proposal 7:

- Study enhancements on CSI including measurement and report, and signaling to enable SBFD operation using multiple antenna panels

Proposal 8:

- Study different power control parameters between SBFD and non-SBFD symbols to improve UL reception performance during SBFD symbols

Proposal 9:

- Study SBFD specific timing alignment offset to mitigate inter-symbol interference

- A guard period may need to be defined after the end of UL transmission in an SBFD symbol to allow sufficient margin time for gNB to switch from UL to DL

Proposal 10:

- For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols)

- For SBFD-aware UEs, the transmissions/receptions can be in SBFD symbols and non-SBFD symbols

Proposal 11:

- Study support of 2-set of frequency resource configurations for semi-static UL/DL channels/signals where the first set is used during UL-only/DL-only slots/symbols and the second set is used for SBFD slots/symbols. Semi-static UL/DL channels/signals include at least:

  - CSI-RS

  - PUCCH

  - SRS

  - Configured grant and SPS

Proposal 12:

- Multi-slot PUSCH/PDSCH spanning SBFD and UL-only/DL-only slots can be transmitted on both SBFD slots and UL-only/DL-only slots

  - FFS whether additional network control is required to disable/enable transmission on SBFD slots

Proposal 13:

- Study how to support frequency interleaving procedures for PDSCH and CORESTET for SBFD slots/symbols

Proposal 14:

- Study enhancements to frequency hopping offset and starting RB calculation for performing UL transmission in an SBFD symbol

Proposal 15:

- For SBFD-aware UEs, study the at least following options for resource allocation in frequency-domain in case of unaligned boundaries between RBG and SBFD subbands. For an RBG that overlaps the subband boundary,

  - Part of the DL RBG inside the DL subband can be used

  - Part of the UL RBG inside the UL subband can be used

Proposal 16:

- Support of 2-set of frequency resource configurations for CORESET associated with UE specific search space for SBFD-aware UEs, where the first set is used during UL-only/DL-only slots/symbols and the second set is used for SBFD slots/symbols.

Proposal 17:

- Study UE/network behavior for configuring CORESETs associated to common search space during SBFD operation.

Proposal 18:

- Study the PUCCH resource configuration for SBFD symbols should consider the following aspects

  - Inter/inter-slot frequency hopping

  - UCI multiplexing and HARQ-ACK codebook generation

Proposal 19:

- Study the frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs considering one CSI-RS resource as baseline

Proposal 20:

- UE should only include the reporting quantities (e.g. CSI/PMI) associated with the DL subband resources when the corresponding CSI measurement are performed during SBFD symbol

Proposal 21:

- Study potential enhancement of initial access for SBFD operation.

Proposal 22:

- Identify the scenarios for UE where configured DL (or UL) transmissions may conflict with dynamic DCI scheduled UL (or DL) transmissions and specify UE behavior to resolve such conflicts.

Proposal 23:

- In Rel-18, for sub-band non-overlapping full duplex at the gNB side, study interference management for

  - inter-UE CLI in a cell

  - self-interference management for gNB

  - inter-gNB CLI

Proposal 24:

- In Rel-18, for sub-band non-overlapping full duplex at the gNB side, traffic characteristics served by gNBs should be taken into the interference management.

Proposal 25:

- For inter-gNB CLI mitigation, gNBs exchange with each other the UL subband frequency resource configuration and SBFD time occasions

Proposal 26:

- Study the CSI report enhancement for SBFD operation and different type of CLI interference

- Consider the non-uniform CLI bandwidth in inter-subband CLI measurement/report

# References

- RP-213591, ‘New SI: Study on evolution of NR duplex operation,’ CMCC.

- Draft_Minutes_report_RAN1#109-e_v030

- Draft_Minutes_report_RAN1#110_v020

- Draft_Minutes_report_RAN1#110-bis-e_v010

- Draft_Minutes_report_RAN1#111_v020

- Draft_Minutes_report_RAN1#112_v010

# Previous Agreements

RAN1#112

Agreement For dynamic SBFD, - For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) are allowed or not in a symbol configured as DL in TDD-UL-DL-ConfigCommon based on the following options: - Option 1 (semi-static): DL receptions outside semi-statically configured DL subband(s) are not allowed - Option 2: DL receptions outside semi-statically configured DL subband(s) are allowed - For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) and UL transmissions outside semi-statically configured UL subband are allowed or not in the symbol configured as flexible in TDD-UL-DL-ConfigCommon based on the following options: - Option 1 (semi-static): DL receptions outside semi-statically configured DL subband(s) are not allowed and UL transmissions outside semi-statically configured UL subband are not allowed - Option 2: DL receptions outside semi-statically configured DL subband(s) are allowed - UL transmissions outside the semi-statically configured UL subbands are not allowed - Option 3: DL receptions outside semi-statically configured DL subband(s) are allowed - UL transmissions outside the semi-statically configured UL subbands are allowed Dynamic SBFD should be compared with dynamic TDD and/or semi-static SBFD in terms of performance, implementation complexity, switching latency. For each option, additional conditions may apply to determine whether the option is applicable. Agreement Study whether or not a slot can consist of both SBFD and non-SBFD symbols including - Benefits - Use cases - Scheduling flexibility - Implementation complexity - Compatibility with legacy TDD DL/UL configuration Agreement For inter-UE inter-subband CLI measurement, study at least the following methods: - Method#1: victim UE measures RSSI within DL subband - FFS: Whether SINR can be measured - Method#2: victim UE measures RSRP of aggressor UE within UL subband - Method#3: victim UE measures RSSI within UL subband - Note: the restriction in Rel-16 that CLI is only measured within DL BWP does not forbid UE to measure CLI in UL subband when UL subband is confined within DL BWP. Agreement: For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) - Study the following options for SBFD-aware UEs: - Option 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only - Option 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols - UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols include the following: - PDSCH/PUSCH/PUCCH repetitions - SPS PDSCH/CG PUSCH - TBoMS - Multi-PUSCH/PDSCH scheduled by a single DCI - Periodic/semi-persistent SRS/CSI-RS/PUCCH - PDCCH Agreement For SBFD-aware UEs, study the at least following options for resource allocation in frequency-domain in case of unaligned boundaries between RBG and SBFD subbands. For an RBG that overlaps the subband boundary, - Option 1: - Part of the DL RBG inside the DL subband can be used - Part of the UL RBG inside the UL subband can be used - Option 2: - Part of the DL RBG inside the DL subband cannot be used - Part of the UL RBG inside the UL subband cannot be used FFS: The part of the RBG outside. Agreement For SBFD-aware UEs, study at least the following issues for PDSCH: - PRG(s) with size of 2 and 4 that overlaps with subband boundary - Wideband precoder in case of non-contiguous DL subbands Agreement: Study the frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs considering the following options: - Option 1: Two contiguous CSI-RS resources that are linked - Option 2: One CSI-RS resource - Option 2-1: Non-contiguous CSI-RS resource allocation - Option 2-2: One contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL subband (s) Agreement: For SBFD-aware UEs, study the following options for CSI report associated with periodic/semi-persistent CSI-RS, at least, across SBFD symbols and non-SBFD symbols in different slots (each CSI-RS resource within a slot has either all SBFD or all non-SBFD symbols): - Option 1: separate CSI reporting for SBFD symbols and non-SBFD symbols - Option 2: same CSI reporting for SBFD symbols and non-SBFD symbols Agreement: Study at least the followings for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols in different slots: - Whether/how to have separate resources - Whether/how to have separate FH parameters - Whether/how to have separate UL power control parameters - Whether/how to have separate beam/spatial relation

RAN1#111

Agreement For a SBFD aware UE semi-statically configured with UL subband in a SBFD symbol configured as DL in TDD-UL-DL-ConfigCommon, the following is agreed as baseline in the RAN1 study: - UL transmissions within UL subband are allowed in the symbol - UL transmissions outside UL subband are not allowed in the symbol - Frequency locations of DL subband(s) are known to the SBFD aware UE - The frequency location of DL subband(s) can be explicitly indicated or implicitly derived - DL receptions within DL subband(s) are allowed in the symbol - Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol Agreement For the purpose of RAN1 study, the understanding is that for semi-static configuration of subband frequency locations for SBFD operation, frequency location of UL/DL subband is with reference to CRB grid. Agreement Study impact and potential enhancements for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols, including at least the following: - PDCCH, scheduled/configured PUCCH/PUSCH/PDSCH, without repetition in SBFD symbols and non-SBFD symbols - Scheduled/configured SRS/CSI-RS in SBFD symbols and non-SBFD symbols - Scheduled/configured TBoMS across SBFD symbols and non-SBFD symbols with or without repetition - Multi-PUSCH/PDSCH scheduled by a single DCI in SBFD symbols and non-SBFD symbols - Scheduled/configured PDSCH/PUSCH/PUCCH with repetitions across SBFD symbols and non-SBFD symbols Note: Inter-slot/intra-slot/inter-repetition/inter-group frequency hopping with DMRS bundling of PUSCH/PUCCH, if applicable, is considered. Examples of potential enhancements include: - Resource allocation in frequency domain including frequency hopping - Resource allocation in time domain - Power domain - Spatial domain FFS: If the PUCCH/PUSCH/PDSCH/PDCCH can be mapped to SBFD and non-SBFD in the same slot if configured. Agreement For SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon, study the following options for SBFD aware UEs, Option 1: - UL transmissions within UL subband are allowed in the symbol - UL transmissions outside UL subband are not allowed in the symbol - Frequency locations of DL subband(s) are known to the SBFD aware UE - DL receptions within DL subband(s) are allowed in the symbol - FFS: Whether DL receptions outside DL subband(s) are allowed or not in the symbol Option 2: - UL transmissions within UL subband are allowed in the symbol - The RBs outside the UL subband can be used as either UL, or DL excluding guardband(s) if used, in the symbol from gNB’s perspective, and the transmission direction for all those RBs is the same - FFS: SBFD aware UE behaviours - FFS: Whether or not signalling of guardband(s) is needed - FFS: Whether or not the symbol can be converted to a DL-only symbol - Frequency locations of DL subband(s) are known to the SBFD aware UE - DL receptions within DL subband(s) are allowed in the symbol Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol for both options. For all RBs outside the UL subband, UE cannot use separate RBs for DL and UL simultaneously Agreement Study the impact and benefits of potential enhancements to resource allocation in frequency-domain for SBFD operation, considering unaligned boundaries between resource block group(s)/reporting subband(s) and SBFD subbands, including at least the following: - RBG for PDSCH RA type 0 - CSI reporting configuration - CSI-RS resource configuration - PRG of PDSCH

RAN1#110-bis-e

Agreement For SBFD operation at least for RRC_CONNECTED state, it is agreed that SBFD operation Alt 4 is the baseline. - SBFD operation Alt 4: - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs. - UE behaviors for non-SBFD aware UEs follow existing specifications. - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation. Agreement For semi-static configuration of subband frequency locations for SBFD operation, at least explicit indication of frequency location of UL subband is required. - FFS: Whether frequency location of other subbands types is explicitly indicated or implicitly determined. Agreement Study impact and potential enhancements of CSI-RS resource set frequency domain resource allocation and CSI reporting configuration across non-contiguous DL subbands. Agreement Identify if there are any cases of time domain conflict of UE’s UL and DL operation in the same SBFD symbol for SBFD aware UE - If there are, whether/how to avoid/handle such collision cases (as second step) Agreement Study impact/potential enhancements for UE-to-UE CLI-RSSI measurement/report considering non-contiguous measurement resource in frequency. Agreement Study whether SBFD operation in SSB symbols is supported or not. Agreement For SBFD operation within a TDD carrier, it is agreed that SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies is the baseline. Agreement The maximum number of UL subbands for SBFD operation in an SBFD symbol (excluding legacy UL symbol) within a TDD carrier is one for the study in RAN1. - The UL subband can be located at one side of the carrier. - The UL subband can be located at the middle part of the carrier, subject to RAN4’s study and conclusion Note: RAN1 considers the above two possibilities unless RAN4 concludes that any one is infeasible. Note: Two UL subbands for SBFD operation in an SBFD symbol within a TDD carrier due to SBFD operation in legacy UL symbols is subject to further RAN1 discussions which is 2nd priority as per RAN guidance. Send an LS to RAN4 to inform the above agreement. If RAN4 has response, it will be taken into account but in the meanwhile, RAN1 work will continue based on the above. LS on maximum number of UL subbands for duplex evolution to RAN4 is endorsed. Final LS in R1-2210671. Agreement For semi-static configuration of subband time locations for SBFD operation, it is agreed that explicit configuration of SBFD subband time locations within a period is the baseline.

RAN1#110

Agreement: Study the following alternatives with Alt 4 prioritized, for SBFD operation at least for RRC_CONNECTED state. - SBFD operation Alt 1: - Time and frequency locations of subbands for SBFD operation are not known to UEs. - UE behaviors follow existing specifications without introducing new UE behaviors for SBFD operation at gNB side. - SBFD operation Alt 2: - Time and frequency locations of subbands for SBFD operation are not known to UEs. - UE behaviors for non-SBFD aware UEs follow existing specifications. - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs - SBFD operation Alt 3: - Only time location of subbands for SBFD operation is known to SBFD aware UEs. - UE behaviors for non-SBFD aware UEs follow existing specifications. - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time location of subbands for SBFD operation - SBFD operation Alt 4: - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs. - UE behaviors for non-SBFD aware UEs follow existing specifications. - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation. UE capability discussion is held in work item phase. Agreement: For indication of subband locations for SBFD operation, study semi-static configuration of subband time and frequency location as baseline. Agreement For semi-static configuration of subband location, consider same subband frequency resources across different SBFD symbols as baseline. Working Assumption For SBFD operation within a TDD carrier, study SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies as baseline. - FFS feasibility and potential benefit of SBFD scheme within a single configured DL and UL BWP pair with unaligned center frequencies - FFS feasibility and potential benefit of SBFD scheme with more than one configured DL and UL BWP pair with aligned/unaligned center frequencies for a DL and UL BWP pair Agreement For SBFD operation Alt 4, for an SBFD aware UE configured with an UL subband in an SBFD symbol, study the following options: - Option 1: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband or to be scheduled with DL reception within the UL subband in the SBFD symbol - Option 2: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband and may be scheduled with DL reception within the UL subband in the SBFD symbol - Option 3: The SBFD aware UE does not expect to be scheduled with DL reception within the UL subband and may be scheduled with UL transmission outside the UL subband in the SBFD symbol - Option 4: The SBFD aware UE may be scheduled with UL transmission outside the UL subband or DL reception within the UL subband in the SBFD symbol Agreement Study the feasibility and potential benefit of UE-to-UE co-channel CLI measurement and reporting, which can be specific for SBFD, at least includes: - Measurement resource/reporting configuration - Measurement/reporting details (including UE processing delay) - Relevant information exchange (between gNBs) if needed - Usage of measurement at gNB

RAN1#109e

Agreement Study whether/how to inform the UE of the time and/or frequency location of subbands that gNB would use for SBFD operation. Agreement Study the impact/potential enhancements of resource allocation in symbols with subbands that gNB would use for SBFD operation. Agreement At least study SBFD operation within a TDD carrier. Agreement The time and frequency location of subbands within a TDD carrier are not fixed in the specification. - Subject to any RAN4 guidance on minimum or maximum subband and guardband size and subband location within TDD carrier. - Note that whether the time and/or frequency location of subbands are informed to UE is separately discussed. Agreement For study of potential enhancement to dynamic/flexible TDD and/or SBFD, followings are considered as candidates of potential enhancement method of gNB-to-gNB CLI handling, where further prioritization/down-scoping of candidate schemes for study can be done in the future meetings: - gNB-to-gNB CLI measurement and reporting - Coordinated scheduling - Spatial domain enhancements - Advanced receiver - UE and gNB transmission and reception timing - Power control based solution - Potential enhancements to Rel-16 RIM - Sensing based mechanism - Note: Whether or not a particular scheme requires OTA or backhaul information exchange should be identified - Note: Any other scheme(s) for inter-gNB CLI handling is/are not precluded. - Note: For potential enhancements to dynamic/flexible TDD and/or SBFD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion. - Note: Potential enhancements specific for SBFD will be discussed in 9.3.2 Agreement For study of potential enhancement to dynamic/flexible TDD and/or SBFD, followings are considered as candidates of potential enhancement method of UE-to-UE CLI handling, where further prioritization/down-scoping of candidate schemes for study can be done in the future meetings: - Potential enhancements to UE-to-UE CLI measurement/reporting - Coordinated scheduling - Spatial domain enhancements, - Advanced Receiver - UE and gNB transmission and reception timing - Power control based solution - Sensing based mechanism - Note: Whether or not a particular scheme requires OTA or backhaul information exchange should be identified - Note: Any other scheme(s) for UE-to-UE CLI handling is/are not precluded. - Note: For potential enhancements to dynamic/flexible TDD and/or SBFD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion. - Note: Potential enhancement specific for SBFD will be discussed in 9.3.2