3GPP TSG RAN WG1 #112bis-e		    R1-2302751

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.7.1

Source:	NEC

Title:              	Discussion on network energy saving techniques in spatial and power domains

Document for:	Discussion and Decision

# Introduction

In RAN#98-e, a new work item on network energy savings was approved [1].

The objectives of the work item are the following:

- Specify enhancement on cell DTX/DRX mechanism including the alignment of cell DTX/DRX and UE DRX in RRC_CONNECTED mode, and inter-node information exchange on cell DTX/DRX [RAN2, RAN1, RAN3]

- Note: No change for SSB transmission due to cell DTX/DRX.

- Note: The impact to IDLE/INACTIVE UEs due to the above enhancement should be avoided.

- Specify the following techniques in spatial and power domains

- Specify necessary enhancements on CSI and beam management related procedures including measurement and report, and signaling to enable efficient adaptation of spatial elements (e.g. antenna ports, active transceiver chains) [RAN1, RAN2]

- Specify necessary enhancements on CSI related procedures including measurement and report, and signaling to enable efficient adaptation of power offset values between PDSCH and CSI-RS [RAN1, RAN2]

- Note: Above objectives are only for UE specific channels/signals

Note: Legacy UE CSI/CSI-RS capabilities applies when considering total number of CSI reports and requirements

The following agreements have been made in RAN1#112 [2]:

Agreement

For the purpose of further discussions in RAN1 on NES spatial domain adaptations, consider the following cases

- Type 1: all antenna elements associated to a logical antenna port is disabled/enabled

- Type 2: part/subset of antenna elements associated to a logical antenna port is disabled/enabled

Agreement

For spatial element adaptation, further study the following

- A1-1) Each CSI-RS resource/resource set/resource setting can be associated with only one spatial adaptation pattern

  - FFS: Details on how the association is done

- A1-2) Each CSI-RS resource/resource set/resource setting can be associated with one or more spatial adaptation patterns

  - FFS: Details on how the association is done

- FFS: Details on the definition of “spatial adaptation patterns”

In this contribution, we provide our views on the network energy saving techniques in spatial and power domains.

# Discussion

To support efficient adaptation of spatial elements for spatial domain network energy saving, hypotheses of antenna On/Off may be configured for a group of UEs to determine the antenna configuration for network energy saving. The different set of ports such as 64/32/8/4 and their associated CSI-RS configurations may be determined from the results of these hypothesis. The gNB could thus identify the best of the directional beams to turn On/Off. Antenna configuration for the network energy saving may then be configured by mapping the selected TRX ports setting to an associated TrxPoolIndex. The TrxPoolIndex field can be used to select each group of NZP-CSI-RS configuration and measurement reporting in reportConfig. Whenever the network enters into the energy saving mode, the corresponding spatial domain configuration can then be determined from the TRX pool index.

Proposal 1: Consider using an associated TRX pool index to address the spatial domain configuration whenever the network enters into the energy saving mode.

CSI-RS is a flexible multi-purpose signal and is widely used by e.g. beam management. During the energy saving mode, beam refinement may not be supported. The reduced density can be part of a network energy saving configuration, which would be applied autonomously each time when gNB enters into the network energy saving mode until the configuration is updated. Supported density setting is currently {0.5, 1, 3, spare} as in NZP-CSI-RS-Resource. When the network energy saving configuration is enabled, a reduced CSI-RS density may be applicable, such that 31 or 10.5. Assumption here is that the transmitting RE power is fixed, and the overall transmit power will be reduced due to reduced number of transmitting REs.

Proposal 2: Reduced CSI-RS density for network energy saving should be considered.

The network energy saving activation/deactivation may be, for example, determined based on semi-static signaling. Specifically, different combinations may be used to activate/deactivate different configuration types at the UE. It is also possible that two or more different configurations (e.g., time, spatial, power) may be activated simultaneously. Activation/deactivation may be triggered after expiry of a timer representing a delay between the configuration of network energy saving mode and the start timing for activation/deactivation of the network energy saving configuration.

The network energy saving configuration may also be supported for a predetermined duration or time of day. No explicit signaling for deactivation is then needed. Suitable for scenarios where there is known/recurring user traffic only for a certain period.

Proposal 3: Consider the activation of different network energy saving techniques (e.g., time, spatial, power) via semi-static network energy saving configuration.

To support efficient adaption of power and spatial elements for network energy saving, signaling may need to be provided via MAC CE or DCI. In this case, related indication may be frequent, at least compared to the legacy RRC signaling. Although for DCI-based indication, the indicated value may be applied in one-time manner, we still prefer the value can be valid till next indication.

At least for power domain network energy saving, two indication methods can be considered, one is absolute value indication, and the other is accumulative value indication. Absolute value means that the indicated value is with respect to a reference or configured value. Accumulative value means that the indicated value is with respect to the latest applied value. Both indication methods can be supported, and the applied method can be further configured.

Proposal 4: Support both accumulative value indication and absolute value indication for efficient adaption of power and spatial elements for network energy saving.

To assist data transmission in NES mode, additional hypothetical CSI/beam report is needed, it is hypothetical because it is based on the assumption that indicated power offset adaptation or spatial elements adaptation would be applied in NES mode. For power domain network energy saving, in addition to indicated power offset and hypothetical CSI/beam report, UE could also report its tolerance of potential PDSCH power reduction. For example, in a CSI/beam report, UE could indicate a hypothetical RSRP or hypothetical CQI which is only achievable when the PDSCH power reduction fulfills the reported tolerance.

Proposal 5: Support UE to report its tolerance of potential PDSCH power reduction.

For power domain network energy saving, PDSCH may be transmitted with power reduction but PDCCH and CSI-RS may still be transmitted with the configured power. It may cause inaccuracy when UE assesses the link quality via BFD-RS. For example, UE may overestimate the channel condition for data transmission if still assuming 0 dB offset between PDCCH EPRE to NZP CSI-RS EPRE. Having a scaled BFD threshold may be helpful. Alternatively, assuming hypothetical X dB offset between PDCCH and BFD-RS for hypothetical PDCCH BLER estimation may have the same effect.

On the other hand, for candidate beam identification, some candidate beams may be skipped even they can work in NES mode since the PDSCH would be transmitted with a reduce power anyway. Having a scaled candidate beam threshold may be helpful to identify these beams and accelerate BFR procedure.

Proposal 6: Support scaling the threshold of beam failure detection and threshold of candidate beam identification for power domain network energy saving.

For spatial domain network energy saving, applied spatial elements of a CSI-RS resource may be adaptive. It may cause issues if the CSI-RS resource is used as a reference signal for other purposes, including the cases when CSI-RS resource is configured as the reference RS in TCI state, QCL info, spatial relation, and pathloss reference signal. The change of spatial elements may cause large scale properties measured and stored by the UE based on “old” CSI-RS out-of-date or at least not that accurate. UE may need longer time to measure “new” CSI-RS after the change of spatial elements, which may have impacts on the application timing of the indicated TCI state, QCL info, spatial relation and pathloss reference signal. Furthermore, it may have impacts on the path loss estimation based on higher layer filtered RSRP.

Proposal 7: Study the impact of spatial elements adaption if the CSI-RS resource is configured as reference RS in TCI state, QCL info, spatial relation, and pathloss reference signal.

# Conclusion

In this contribution, we discussed the potential techniques for network energy saving, the following proposals are made:

Proposal 1: Consider using an associated TRX pool index to address the spatial domain configuration whenever the network enters into the energy saving mode.

Proposal 2: Reduced CSI-RS density for frequency domain network energy saving should be considered.

Proposal 3: Consider the activation of different network energy saving techniques (e.g., time, frequency, spatial, power) via semi-static network energy saving configuration.

Proposal 4: Support both accumulative value indication and absolute value indication for efficient adaption of power and spatial elements for network energy saving.

Proposal 5: Support UE to report its tolerance of potential PDSCH power reduction.

Proposal 6: Support scaling the threshold of beam failure detection and threshold of candidate beam identification for power domain network energy saving.

Proposal 7: Study the impact of spatial elements adaption if the CSI-RS resource is configured as reference RS in TCI state, QCL info, spatial relation, and pathloss reference signal.

# References

- RP-223540, New WI: Network energy savings for NR, Huawei, RAN Meeting #98e, December 12-16, 2022

- Chairman’s notes for 3GPP TSG RAN WG1 #112