3GPP TSG-RAN WG1 Meeting #112bis-e	R1-2302753

e-Meeting, April 17th – April 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | DRAFT | rev | - | Current version: | V17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on editorial correction on epoch time |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Thales |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2023-04-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Use epochTime in TS 38.213 as defined in TS 38.331 |  |  |  |  |  |  |  |  |
| Summary of change: |  | In current specification 38.213, it is stated that tepoch is the epoch time of TACommon, TACommonDrift, and TACommonDriftVariant [12, TS 38.331]. However, the TACommon, TACommonDrift, and TACommonDriftVariant are not defined in TS 38.331. Indeed, these variables (determined by the UE) are provided by the ta-Common, ta-CommonDrift, and ta-CommonDriftVariant which are defined in TS 38.331. To solve this inconsistency, we may reword the text as follows: tepoch is the epoch time of ta-Common, ta-CommonDrift, and ta-CommonDriftVariant [12, TS 38.331] or simply use the epochTime as defined in 38.331 as follows: and tepoch is provided by epochTime [12, TS 38.331]. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

4.2	Transmission timing adjustments

<Unchanged parts are omitted>

Using higher-layer ephemeris parameters for a serving satellite, if provided, a UE pre-compensates the two-way transmission delay on the service link based on NTA,adjUE that the UE determines using the serving satellite position and its own position. To pre-compensate the two-way transmission delay between the uplink time synchronization reference point and the serving satellite, the UE determines NTA,adjcommon [4, TS 38.211] based on one-way propagation delay Delaycommont that the UE determines as:

Delaycommont= TACommon2+ TACommonDrift2×t-tepoch+TACommonDriftVariant2×t-tepoch2

where TACommon, TACommonDrift, and TACommonDriftVariant are respectively provided by ta-Common, ta-CommonDrift, and ta-CommonDriftVariant and tepoch is provided by epochTime TACommonTACommonDriftTACommonDriftVariant[12, TS 38.331]. Delaycommon(t) provides a distance at time t between the serving satellite and the uplink time synchronization reference point divided by the speed of light. The uplink time synchronization reference point is the point where DL and UL are frame aligned with an offset given by NTA,offset.

<Unchanged parts are omitted>