3GPP TSG RAN WG1 #112b	R1-2302757

e-Meeting, April 17th – April 26th, 2023

Title: 	Discussion of subband non-overlapping full duplex

Source: 	ZTE

Agenda item:	9.3.2

Document for:	Discussion/Decision

Introduction

In RAN#96 meeting, a revised SID on NR duplex evolution has been endorsed. In this contribution, we provide our analysis for subband non-overlapping full duplex for possible solutions, feasibility and impact to legacy operation assuming co-existence in co-channel and adjacent channels.

SBFD framework and configuration

In RAN1#110 meeting, the following agreement was reached. In RAN1#110bis-e meeting, it has been agreed that SBFD operation Alt 4 is the baseline.

Agreement Study the following alternatives with Alt 4 prioritized, for SBFD operation at least for RRC_CONNECTED state. - SBFD operation Alt 1: - Time and frequency locations of subbands for SBFD operation are not known to UEs. - UE behaviors follow existing specifications without introducing new UE behaviors for SBFD operation at gNB side. - SBFD operation Alt 2: - Time and frequency locations of subbands for SBFD operation are not known to UEs. - UE behaviors for non-SBFD aware UEs follow existing specifications. - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs - SBFD operation Alt 3: - Only time location of subbands for SBFD operation is known to SBFD aware UEs. - UE behaviors for non-SBFD aware UEs follow existing specifications. - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time location of subbands for SBFD operation - SBFD operation Alt 4: - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs. - UE behaviors for non-SBFD aware UEs follow existing specifications. - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation. UE capability discussion is held in work item phase.

Regarding SBFD operation Alt 4, companies discussed RB-set based solution, BWP based solution and CA based solution. In RAN1#110bis-e meeting, it is agreed that SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies is the baseline. Basically, it implies that RB-set based solution (or subband based solution) is the baseline.

Agreement For SBFD operation within a TDD carrier, it is agreed that SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies is the baseline.

BWP-based solution: SBFD schemes across BWP(s)

In addition to the subband-based solution, the BWP-based solution can also be further studied. Starting from NR Rel-15, the framework of BWP is introduced. Each BWP corresponds to a specific SCS and CP type, and is configured with a set of transmission parameters, such as PDCCH-config, PDSCH-config for DL BWP, PUCCH-config and PUSCH-config for UL BWP, etc. Similarly, BWP can be reused to implement subband full duplex. For example, each subband is defined as one BWP and different frame structures can be configured for different BWPs. In this way, the existing BWP framework of NR can be used.

Take Figure 1 as an example, three BWP pairs are configured, i.e., each BWP pair contains one DL BWP and one UL BWP. BWP1 and BWP3 are the BWP pairs at the two edges of the carrier, and BWP2 is the BWP pair located in the middle. BWP switching delay may be an issue to be considered for crossing operation between different BWP pairs (i.e., across subbands). For that, a UE capability on supporting dual active BWP pair can be defined. For the UE with the dual active BWP pair capability, although it cannot work on both BWP pairs with different traffic directions at the same time, the UE can switch between different active BWP pairs with tolerable or even without delay. Thus, the data transmission/feedback delay can be effectively reduced, which is a major objective of subband full duplex. For example, if UE is activated on BWP1 and BWP2 simultaneously, then network can determine which BWP is used for transmission based on the real time traffic.

Figure 1: BWP based subband full duplex

During the previous RAN1 meetings, companies discussed at least the following different options for BWP-based SBFD.

- Option1: SBFD scheme within a single configured DL and UL BWP pair with unaligned center frequencies

- Option2: More than one configured DL and UL BWP pair with aligned center frequencies

- Option3: More than one configured DL and UL BWP pair with unaligned center frequencies

A high-level comparison among these three options can be summarized in the following table.

| Option | Pros | Cons |
| --- | --- | --- |
| Option1: SBFD scheme within a single configured DL and UL BWP pair with unaligned centre frequencies | Only one BWP pair | Unaligned center frequencies, which may cause longer Tx-Rx switching delay |
| Option2: More than one configured DL and UL BWP pair with aligned centre frequencies | Aligned centre frequency | UE may need to perform BWP switching between SBFD symbol and non-SBFD symbol |
| Option3: More than one configured DL and UL BWP pair with unaligned centre frequencies |  | - Unaligned center frequencies, which may cause longer Tx-Rx switching delay - UE may need to perform BWP switching between SBFD symbol and non-SBFD symbol |

In summary, in addition to the RB-set based solution, RAN1 can further study the BWP-based solution for SBFD configuration.

Proposal 1: In addition to the RB-set based solution, RAN1 further studies the BWP-based solution for SBFD configuration.

Subband configuration/indication

In the RAN1#111 meeting, the following agreement was reached for the frequency domain configuration of DL/UL subband.

Agreement For the purpose of RAN1 study, the understanding is that for semi-static configuration of subband frequency locations for SBFD operation, frequency location of UL/DL subband is with reference to CRB grid.

Flexible subband/resources

In the current discussion, UL subband and DL subband are proposed for UL transmission and DL transmission respectively. Companies also discussed “flexible subband” during RAN1#111 meeting. However, the definition of flexible subband is not aligned among companies. There are at least the following three different alternatives to implement flexible subband or flexible resources:

- Alt.1: subband that can be used to transmit UL and DL is considered as flexible subband. In this case, flexible subband can be configured in DL symbols and/or F symbols without configuring the transmission direction. It can transmit UL or DL according to the schedule from the base station. Its function is similar to the F symbol in the current specification, which can be used to dynamically balance DL and UL resources or as a gap for switching between UL and DL.

- Alt.2: subband that configured in the flexible symbols is considered as flexible subband. The flexible subband can be used to transmit UL or DL according to the schedule from the base station.

- Alt.3: part of the resources within the UL subband is configured as flexible resource, DL transmission and UL transmission are allowed within the flexible resources

For Alt.1 and Alt.2, a flexible subband is explicitly introduced. A flexible subband can be configured independently if the UL subband is not configured. However, if a UL subband is configured and a flexible subband is also configured simultaneously, then the flexible subband should follow the frequency domain location of the UL subband, and the time domain locations of the flexible subband can be configured independently. An example is shown in Figure 2. It can also be understood that the flexible resource of a UL subband can be configured in some symbols of the UL subband and has the same frequency domain resources as the UL subband.

For Alt.3, a flexible subband is not introduced explicitly. Instead, flexible resource within the UL subband can be configured. The flexible resource serves the same purpose as flexible subband.

From our point of view, the flexible subband/resource is beneficial, which can be used as a supplementary resource of the UL subband. Flexible subband/resource should be studied for subband full duplex. However, if DL transmission is allowed to be transmitted in the UL subband, then even without introducing the concept of “flexible subband/resource”, DL transmission may also be allowed in UL subband. In this sense, UL subband can serve as the “flexible subband/resource”.

Figure 2: An example for a flexible subband/resource (in the UL subband)

Proposal 2: RAN1 studies the flexible subband/resource with the following potential alternatives.

- Alt.1:  subband that can be used to transmit UL and DL is considered as flexible subband.

- Alt.2: subband that configured in the flexible symbols is considered as flexible subband.

- Alt.3: part of the resources within the UL subband is configured as flexible resource, within which DL transmission and UL transmission are allowed.

Cell-common/UE-specific UL subband configuration

Companies discussed whether the UL subband configuration is cell-common or UE-specific. The cell-common UL subband is supported by more companies due to simple resource allocation, such as configuring a continuous set of RB in the frequency domain. From the base station perspective, the UL subband is a continuous frequency domain resource, which is conducive to receiving UL transmissions and eliminating interference.  However, even if cell-common UL subband configuration is applied, UE-specific configuration is still needed, e.g., different PUSCH/PUCCH/SRS/RACH configurations for transmission in UL subband for different UEs.

In addition to the cell-common UL subband, it is worth studying whether UE-specific UL subband is needed. If UE-specific UL subband is introduced, then the UE-specific UL subband has to be confined within the cell-common UL subband. Meanwhile, the UE-specific configurations related to the UL subband can also be configured sub-level configuration for the UE-specific UL subband. For example, a UE dedicated UL transmission can be scheduled in the UE-specific UL subband, and a PUSCH resource and SCS and/CP used by the UL transmission should be determined based on the configured PUSCH candidate resource set and SCS and/CP of the UE-specific UL subband.

The cell-common UL subband should also be configured with the corresponding PUCCH candidate resource set, PUSCH candidate resource set, SCS and CP, etc. The configuration information can be used for the UL transmission in a random-access process or when the configuration information corresponding to UE-specific is not provided.

Meanwhile, there may also be some scenarios requireing UE-specific UL subband frequency/time configuration. For example, for UEs with restricted bandwidth capability (e.g., Redcap UE), they may only support UL transmission on partial bandwidth of the UL subband.  This can be further studied.

Proposal 3: Regarding the UL suband configuration, both cell-common configurations and UE-specific configurations are needed.

- Example of cell-common UL subband configurations: frequency/time resource of the UL subband, PUSCH/PUCCH/RACH /SCS/CP configuration for IDLE UE or for UE without UE-specific configuration, etc

- Example of UE-specific UL subband configurations: PUSCH/PUCCH/RACH/SCS/CP configuration for CONNECTED UE

- FFS: Whether UE-specific bandwidth configuration of UL subband is needed or not.

Time-domain configuration of subband

In RAN1#110bis-e meeting, the following agreement about explicit configuration of SBFD subband time locations within a period was made:

Agreement For semi-static configuration of subband time locations for SBFD operation, it is agreed that explicit configuration of SBFD subband time locations within a period is the baseline.

The following two options are provided for the configuration of subband time locations.

Option 1

A time domain window is defined for SBFD operation. And the time domain window can be configured via RRC parameters, such as, period, offset and length. As an example shown in Figure 3, the time domain window with 3 slots length will occur with a periodicity of 10 slots. And the starting point is the second slot within each window, which is determined according to the configuration of offset = 1. For a TDD slot format configuration with 5 slots periodicity, there will be one time-domain window for every two periods of TDD slot format. Then, the SBFD operation can be applied within each time domain window.

For legacy UEs not supporting subband full duplex, network can schedule them outside the window for better compatibility. Similarly, resource for some important signals (e.g., SSB) which have been well designed in legacy procedures should also be considered during the time domain window configuration, for which the minimum impact can be expected.

Symbol-level configuration will also be needed considering some special signals (e.g., SSB) only occupy partial symbols in one slot.

Figure 3: Time domain window for full duplex

Option 2

The period for the SBFD operation is determined by the periodicities of TDD UL-DL pattern(s) configured in TDD-UL-DL-ConfigCommon and TDD-UL-DL-ConfigDedicated. In case two TDD UL-DL patterns are provided, the SBFD subband time locations can be separately configured within the periodicity of each TDD UL-DL pattern.

However, if the period for the SBFD operation is determined by the TDD pattern, it may not be compatible with the SSB configuration. For example, if the period SBFD operation is equal to 5ms (the same as the period of TDD pattern) and the SSB period is 20ms, then some special handlings are needed for the SSB. For example, the symbols configured with SSB are automatically converted to non-SBFD symbol. In any case, as long as the special handlings for SSB are clearly defined, this option2 is also workable.

Proposal 4: For semi-static configuration of subband time locations for SBFD operation, the following two options are studied:

- Option 1: the time domain window (e.g., period, offset and length) can be configured by RRC for SBFD operation for better compatibility with less impact on legacy UE and procedures.

- Option 2: the period for subband is determined by the periodicities of TDD UL-DL pattern(s) configured in TDD-UL-DL-ConfigCommon and TDD-UL-DL-ConfigDedicated

Frequency-domain configuration of subband

In RAN1#110bis-e meeting, it has been agreed to at least indicate the frequency location of UL subband explicitly. Another controversial issue is whether to configure guard band between DL subband and UL subband explicitly.

Agreement For semi-static configuration of subband frequency locations for SBFD operation, at least explicit indication of frequency location of UL subband is required. - FFS: Whether frequency location of other subbands types is explicitly indicated or implicitly determined.

Although the detailed requirements and assumptions for guard band need RAN4 input, according to some preliminary simulation results submitted under AI 9.3.1, guard band is useful for alleviating gNB self-interference. Overall, UE needs to know the information about guard band either explicitly or implicitly. UE is not expected to transmit uplink overlapping with the guard band. If the gNB self-interference is serve, gNB should not schedule DL transmission in the guard band to alleviate the self-interference. However, if gNB realizes that there is no uplink transmission in the UL subband or gNB schedules DL transmission in the UL subband, then DL should also be allowed in the guard band. That is, the frequency domain resource of the guard band should be outside the frequency domain of UL subband, but it can be within the frequency domain of DL subband.

Guard band can also be implemented via base station implementation. For example, several RBs at the boundary of DL subband are reserved, if these RBs are not scheduled for DL or UL by base station, then these RBs realize the function of guard band.

Proposal 5: For SBFD operation, guard band can be reserved between DL subband and UL subband explicitly or implicitly.

- UE is not expected to transmit uplink transmission in the guard band.

- UE is allowed to receive DL transmission in the guard band.

Relationship between UL subband and BWP

As agreed in RAN1#110bis-e meeting, configuring only one UL subband for SBFD operation within a TDD carrier is a baseline assumption.

Agreement The maximum number of UL subbands for SBFD operation in an SBFD symbol (excluding legacy UL symbol) within a TDD carrier is one for the study in RAN1. - The UL subband can be located at one side of the carrier. - The UL subband can be located at the middle part of the carrier, subject to RAN4’s study and conclusion Note: RAN1 considers the above two possibilities unless RAN4 concludes that any one is infeasible. Note: Two UL subbands for SBFD operation in an SBFD symbol within a TDD carrier due to SBFD operation in legacy UL symbols is subject to further RAN1 discussions which is 2nd priority as per RAN guidance. Send an LS to RAN4 to inform the above agreement. If RAN4 has response, it will be taken into account but in the meanwhile, RAN1 work will continue based on the above. Final LS is endorsed in R1-2210671

There are two ways to configure the UL subband.

- Alt.1: Each UL BWP can be associated with up to one UL subband. In this case, there may be more than one UL subbands for different UL BWPs configured in one carrier with different SCS and/or different frequency resources. However, there is only up to one UL subband activated at one time per UE.

- Alt.2: One UL subband is configured in one carrier. If the UL subband is within the activated UL BWP, then this UL subband is activated. Otherwise, the UL subband is deactivated for this UE.

Both of the above two alternatives are workable. RAN1 may need further study on them.

Proposal 6: RAN1 further studies the following alternatives for UL subband configuration.

- Alt.1: Each UL BWP can be associated with up to one UL subband. More than one UL subband for different UL BWPs configured in one carrier can be configured, while up to one UL subband can be activated at one time per UE.

- Alt.2: One UL subband is configured in one carrier. The UL subband is activated only if the UL subband is confined within the activated UL BWP.

Dynamic indication of subband location

It is worth studying dynamic update of the subband time-domain configuration as long as the subband frequency domain location is fixed. One of the main benefits of allowing dynamic update of the subband time-domain configuration is to adapt to the traffic load. It can also balance sub-band and non sub-band resources to avoid resource shortage or waste. In case of sudden burst of UL traffic, network can indicate more slot/symbols as UL subband. For example, UL subband is configured from slot 2 to slot 4. In case of sudden burst of UL traffic, network can indicate UE to extend the UL subband to slot 1.

Two options can be considered for indicating the dynamic change of subband time-domain location:

- Option 1: Use a group-common DCI without scheduling DL/UL transmission.

- Option 2: Use a UE dedicated DCI with scheduling UL transmission, e.g., DCI scheduling PUSCH/PUCCH. If more UL resources are needed for the PUSCH/PUCCH, the UL subband can be extended if the interference is controllable.

Option1 is applicable to update/adjust the time-domain location of the cell-common subband. Its function is similar to SFI. After the time domain location of the cell-common subband is updated, it is valid until it is updated again.

Option 2 is applicable to update/adjust the time-domain location of the UE-specific subband, which is based on DCI with scheduling a PUSCH/PDSCH. For example, if the interference is controllable and if a PUSCH/PUCCH transmission is scheduled by the DCI in a UL subband in an SBFD slot, but the symbol of the PUSCH/PUCCH transmission exceeds the SBFD symbol of the UL subband in the SBFD slot, then the SBFD symbol of the UL subband can be automatically expanded to include the symbol of the PUSCH/PUCCH transmission. An example is shown in Figure 4.

Similarly, for the UL subband with semi-static configuration, its SBFD symbol can also be dynamically reduced by canceling the UL subband in the SBFD symbol. For example, if a PDSCH reception is scheduled in the UL subband of the SBFD symbol by a DCI, then the UL subband in the SBFD symbols where the PDSCH reception is located is dynamically cancelled for the PDSCH reception, that is, the SBFD symbols where the PDSCH reception is located is dynamically converted into DL symbols. An example is shown in Figure 5 in section 2.2.7.

Proposal 7: In addition to the semi-static UL subband configuration, RAN1 supports explicitly/implicitly updating the SBFD symbols of the UL subband dynamically.

Figure 4: An example of dynamically extending the SBFD symbol of the UL subband

Transmission direction

In RAN1#111 and RAN1#112 meeting, the following agreements were reached. However, there are still some remaining issues to be discussed.

Agreement For a SBFD aware UE semi-statically configured with UL subband in a SBFD symbol configured as DL in TDD-UL-DL-ConfigCommon, the following is agreed as baseline in the RAN1 study: - UL transmissions within UL subband are allowed in the symbol - UL transmissions outside UL subband are not allowed in the symbol - Frequency locations of DL subband(s) are known to the SBFD aware UE - The frequency location of DL subband(s) can be explicitly indicated or implicitly derived - DL receptions within DL subband(s) are allowed in the symbol - Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol Agreement For SBFD operation in a symbol configured as flexible in TDD-UL-DL-ConfigCommon, study the following options for SBFD aware UEs, Option 1: - UL transmissions within UL subband are allowed in the symbol - UL transmissions outside UL subband are not allowed in the symbol - Frequency locations of DL subband(s) are known to the SBFD aware UE - DL receptions within DL subband(s) are allowed in the symbol - FFS: Whether DL receptions outside DL subband(s) are allowed or not in the symbol Option 2: - UL transmissions within UL subband are allowed in the symbol - The RBs outside the UL subband can be used as either UL, or DL excluding guardband(s) if used, in the symbol from gNB’s perspective, and the transmission direction for all those RBs is the same - FFS: SBFD aware UE behaviours - FFS: Whether or not signalling of guardband(s) is needed - FFS: Whether or not the symbol can be converted to a DL-only symbol - Frequency locations of DL subband(s) are known to the SBFD aware UE - DL receptions within DL subband(s) are allowed in the symbol Note: UL transmissions are within active UL BWP and DL receptions are within active DL BWP in the symbol for both options. For all RBs outside the UL subband, UE cannot use separate RBs for DL and UL simultaneously Agreement For dynamic SBFD, - For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) are allowed or not in a symbol configured as DL in TDD-UL-DL-ConfigCommon based on the following options: - Option 1 (semi-static): DL receptions outside semi-statically configured DL subband(s) are not allowed - Option 2: DL receptions outside semi-statically configured DL subband(s) are allowed - For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) and UL transmissions outside semi-statically configured UL subband are allowed or not in the symbol configured as flexible in TDD-UL-DL-ConfigCommon based on the following options: - Option 1 (semi-static): DL receptions outside semi-statically configured DL subband(s) are not allowed and UL transmissions outside semi-statically configured UL subband are not allowed - Option 2: DL receptions outside semi-statically configured DL subband(s) are allowed - UL transmissions outside the semi-statically configured UL subbands are not allowed - Option 3: DL receptions outside semi-statically configured DL subband(s) are allowed - UL transmissions outside the semi-statically configured UL subbands are allowed Dynamic SBFD should be compared with dynamic TDD and/or semi-static SBFD in terms of performance, implementation complexity, switching latency. For each option, additional conditions may apply to determine whether the option is applicable.

In RAN1#111 meeting, there were some discussions about whether DL reception is allowed in the UL subband configured in the DL symbol, but for the sake of progress, "FFS where DL reception is within the UL subband are allowed or not in the symbol" in the Proposed agreement was finally deleted. In addition, we also noticed that "FFS where/now the symbols can be converted to DL-only symbols in the symbol" was also deleted from the Proposed Agreement for the same reason. But we think they should still be discussed. A joint approach is provided below.

For the functions of the two deleted FFSs mentioned above, they can be implemented simultaneously based on their combination. For example, SBFD symbol is first dynamically converted to DL symbol in the DL slot, that is, the UL subband is dynamically cancelled in the DL symbol configured with the UL subband, then in the DL symbol converted from the SBFD symbol, DL reception can be allowed in the resource corresponding to the cancelled UL subband. An example is shown in Figure 5.

In the above approach, DL reception is essentially implemented in the DL symbol converted from the SBFD symbol configured with the UL subband, but it is based on the fact that the UL subband can be dynamically cancelled in the DL symbol. So, if the SBFD symbol can be dynamically converted to the DL symbol, then DL reception can also be naturally supported in the DL symbol converted from the SBFD symbol.

Proposal 8: For a SBFD aware UE semi-statically configured with UL subband in a SBFD symbol configured as DL in TDD-UL-DL-ConfigCommon, the following is agreed as baseline in the RAN1 study:

- The SBFD symbol can be converted to the DL symbol and DL receptions are allowed in the DL symbol converted from the SBFD symbol

Figure 5: An example of dynamically canceling the UL subband in the SBFD symbol

Based on similar reasons, if the UL subband is configured in the F symbol, then DL reception can also be allowed in the UL subband. Similar approach can also be reused, for example, SBFD symbol is first dynamically converted to F symbol, that is, the UL subband is dynamically cancelled in the F symbol configured with the UL subband, then in the F symbol converted from the SBFD symbol, DL reception can be allowed in the resource corresponding to the cancelled UL subband.

So, if the SBFD symbol can be dynamically converted to the F symbol, then DL reception can also be naturally supported in the F symbol converted from the SBFD symbol.

Proposal 9: For a SBFD aware UE semi-statically configured with UL subband in a SBFD symbol configured as F in TDD-UL-DL-ConfigCommon, the following is agreed as baseline in the RAN1 study:

- The SBFD symbol can be converted to the F symbol and DL receptions are allowed in the F symbol converted from the SBFD symbol

In RAN1 # 112 meeting, it was discussed whether DL reception is allowed outside semi-statically configured DL subband, and the following options were reached.

For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) are allowed or not in a symbol configured as DL in TDD-UL-DL-ConfigCommon based on the following options:

    - Option 1 (semi-static): DL receptions outside semi-statically configured DL subband(s) are not allowed

    - Option 2: DL receptions outside semi-statically configured DL subband(s) are allowed

For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) and UL transmissions outside semi-statically configured UL subband are allowed or not in the symbol configured as flexible in TDD-UL-DL-ConfigCommon based on the following options:

    - Option 1 (semi-static): DL receptions outside semi-statically configured DL subband(s) are not allowed and UL transmissions outside semi-statically configured UL subband are not allowed

    - Option 2: DL receptions outside semi-statically configured DL subband(s) are allowed

      - UL transmissions outside the semi-statically configured UL subbands are not allowed

    - Option 3: DL receptions outside semi-statically configured DL subband(s) are allowed

      - UL transmissions outside the semi-statically configured UL subbands are allowed

From our perspective, for simplicity, the same processing mechanism should be supported for DL receptions outside semi-statically configured DL subband (s) in DL symbols and in F symbols.

We prefer DL receptions outside semi-statically configured DL subband (s) in DL symbols and in F symbols are allowed. It can provide flexibility and load balancing between uplink and downlink. For example, DL reception can be received from the semi-statically configured UL subband if the base station identifies that the interference is controllable. This operation does not require additional hardware enhancements for UE. However, if UL transmission outside the semi-statically configured UL subbands is allowed, it requires hardware enhancement of the UE, such as the design of filters for the UL subband. And as long as one of the DL reception outside semi-statically configured DL subband(s) and UL transmission outside the semi-statically configured UL subbands can be allowed, the flexibility of the system can be enhanced. Therefore, UL transmission outside the semi-statically configured UL subbands should not be allowed.

Proposal 10: For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) are allowed or not in a symbol configured as DL in TDD-UL-DL-ConfigCommon based on the following option 2:

    - Option 2: DL receptions outside semi-statically configured DL subband(s) are allowed

For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) and UL transmissions outside semi-statically configured UL subband are allowed or not in the symbol configured as flexible in TDD-UL-DL-ConfigCommon based on the following option 2:

    - Option 2: DL receptions outside semi-statically configured DL subband(s) are allowed

      - UL transmissions outside the semi-statically configured UL subbands are not allowed

When some urgent needs or necessary measurements are triggered, the CSI-RS/SSB can be transmitted in a resource including the UL subband, and at least this mechanism should be provided. For example, if a necessary measurement triggered by UE or initiated by the base station needs to be executed in a resource containing the UL subband, the base station can send the SSB/CSI-RS in the resource without scheduling UL transmission in the resource.

Therefore,  RAN1 considers supporting some urgent and necessary DL transmissions (including reference signals) to be transmitted in resources containing the UL subband.

Proposal 11: RAN1 considers supporting some urgent and necessary DL transmissions (including reference signals and SSB) to be transmitted in the resource containing the UL subband.

UE collision handling between DL and UL

In the RAN1 # 110b-e meeting, the UE collision handling between DL and UL was discussed. For discussion purpose, the following definitions for UE collision handling between DL and UL in same SBFD symbol are provided by FL.

Definitions:

  - Semi D: symbols configured as DL in tdd-UL-DL-ConfigurationCommon and/or tdd-UL-DL-ConfigurationDedicated

  - Semi U: symbols configured as UL in tdd-UL-DL-ConfigurationCommon and/or tdd-UL-DL-ConfigurationDedicated

  - UL: dynamic and semi-static PUCCH/PUSCH/SRS transmissions

  - DL: dynamic and semi-static PDCCH/PDSCH/CSI-RS receptions

Based on companies’ contributions, the following collision cases were discussed.

  - Case 1: UL vs. DL

  - Case 2: Semi D vs. UL

  - Case 3: SSB vs. UL

  - Case 4: Type 0 CSS vs. UL

  - Case 5: Semi D vs. valid RO

  - Case 6: valid RO vs. DL

  - Case 7: Semi U vs. DL

  - Case 8: Semi U vs. SSB

  - Case 9: Semi U vs. CORESET 0

Among the cases, Case 7/8/9 target for SBFD operation in legacy UL symbols and should be de-prioritized according to RAN decision.

Case 1: UL vs. DL

To resolve DL and UL collision, several options are provided as follows. These options or a combination of these options are applicable to all collisions between dynamic/semi-static PUCCH/PUSCH/SRS transmissions and dynamic/semi-static PDCCH/PDSCH/CSI-RS receptions.

- Option 1: When DL transmission and UL transmission have the same priority and both of them are DG or CG, if DL transmission is scheduled/configured in DL subband and UL transmission is scheduled in UL subband, UL transmission scheduled/configured in UL subband is always prioritized. If both DL transmission and UL transmission are scheduled/configured in the UL subband, DL transmission is always prioritized because DL transmission is scheduled in the UL subband only in case of emergency.

- Option 2: When DL transmission and UL transmission have different priorities, DL/UL transmission with high priority always survives.

Option 1 and option 2 are straightforward methods. They can be applied to all the collisions between DL and UL, but they are not appropriate for collisions between dynamic and semi-static, such as dynamic UL transmissions vs. semi-static DL receptions, semi-static UL transmissions vs. dynamic DL receptions.

When dynamic UL transmissions and semi-static DL receptions have the same priority, the dynamic UL transmissions should be prioritized. The base station generally knows the location of semi-static DL receptions, but the base station still schedules dynamic UL transmissions, which causes the collision. That is because the dynamic UL transmissions are urgent. Similar reasons and rules apply to conflicts between semi static UL transmissions and dynamic DL receptions.

Basically, option1 is considered for the case where both DL and UL are dynamic or semi-static transmission. For the case where DL and UL are dynamic or semi-static respectively, or vice versa, option 3 below is considered.

- Option 3: When dynamic DL transmission and semi-static UL transmission have the same priority and conflict in the time domain, the dynamic DL transmission is prioritized; when dynamic UL transmission and semi-static DL transmission have the same priority and conflict in the time domain, the dynamic UL transmission is prioritized.

Proposal 12: Regarding confliction of Case 1: UL vs. DL, consider the following potential solution.

- Option 1: When DL transmission and UL transmission have the same priority and both of them are DG or CG, if DL transmission is scheduled in DL subband and UL transmission is scheduled in UL subband, UL transmission scheduled in UL subband is always prioritized. If both DL transmission and UL transmission are scheduled in the UL subband, DL transmission is always prioritized.

- Option 2: When DL transmission and UL transmission have different priorities, DL/UL transmission with high priority always survives.

- Option 3: When dynamic DL (or UL) transmission and semi-static UL (or DL) transmission have the same priority and conflict in the time domain, the dynamic transmission is prioritized.

Case 2: Semi D vs. UL

For legacy behaviour, any UL signals or channels will be dropped in a symbol configured as semi D, which is clearly not desired for SBFD operation where UL subband can be configured in semi-static DL symbols. So at least in symbols configured as DL in TDD-UL-DL-ConfigCommon in which UL subband is configured, UL transmission in UL subband should be supported.  Overall, for Case 2: Semi D vs. UL, the UL subband should be supported in the DL symbol, so that dynamic/semi-static PUCCH/PUSCH/SRS can be supported in the UL subband in Semi D.

Case 3: SSB vs. UL

For legacy behavior, any UL signals or channels will be dropped in an SSB symbol. For SBFD operation, it is obvious that the simple and safe approach is that UL subband are not allowed to be configured in the SSB symbol. However, this may lead to discontinuous UL subband configuration in time domain.  To avoid this, the feasibility and benefits of configuring/scheduling UL transmissions in the SSB symbol can be studied.

Case 4: Type 0 CSS vs. UL

Different from SSB, UL transmission in symbols configured with Type 0 CSS is not prevented in the current specification. Thus, UL transmission should also be allowed in the UL subband in the Type 0 CSS symbols.

Proposal 13: RAN1 studies UE collision handling at least for the following cases.

- Case 1: UL vs. DL

- Case 2: Semi D vs. UL

- Case 3: SSB vs. UL

- Case 4: Type 0 CSS vs. UL

Transmission across SBFD symbols and non-SBFD symbols

In RAN1#111 and RAN1#112 meeting, the following agreements for scheduled/configured UL transmissions and scheduled/configured DL receptions across SBFD symbols and non-SBFD symbols were made.

Agreement Study impact and potential enhancements for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols, including at least the following: - PDCCH, scheduled/configured PUCCH/PUSCH/PDSCH, without repetition in SBFD symbols and non-SBFD symbols - Scheduled/configured SRS/CSI-RS in SBFD symbols and non-SBFD symbols - Scheduled/configured TBoMS across SBFD symbols and non-SBFD symbols with or without repetition - Multi-PUSCH/PDSCH scheduled by a single DCI in SBFD symbols and non-SBFD symbols - Scheduled/configured PDSCH/PUSCH/PUCCH with repetitions across SBFD symbols and non-SBFD symbols Note: Inter-slot/intra-slot/inter-repetition/inter-group frequency hopping with DMRS bundling of PUSCH/PUCCH, if applicable, is considered. Examples of potential enhancements include: - Resource allocation in frequency domain including frequency hopping - Resource allocation in time domain - Power domain - Spatial domain FFS: If the PUCCH/PUSCH/PDSCH/PDCCH can be mapped to SBFD and non-SBFD in the same slot if configured. Agreement Study whether or not a slot can consist of both SBFD and non-SBFD symbols including - Benefits - Use cases - Scheduling flexibility - Implementation complexity - Compatibility with legacy TDD DL/UL configuration Agreement For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) - Study the following options for SBFD-aware UEs: - Option 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only - Option 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols - UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols include the following: - PDSCH/PUSCH/PUCCH repetitions - SPS PDSCH/CG PUSCH - TBoMS - Multi-PUSCH/PDSCH scheduled by a single DCI - Periodic/semi-persistent SRS/CSI-RS/PUCCH - PDCCH Agreement Study at least the followings for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols in different slots: - Whether/how to have separate resources - Whether/how to have separate FH parameters - Whether/how to have separate UL power control parameters - Whether/how to have separate beam/spatial relation

It seems to imply that PUCCH/PUSCH/PDSCH/PDCCH configurations in SBFD symbols/slot and non-SBFD symbols/slot are separately configured. From our perspective, both the shared configuration and separate configuration for SBFD slot/symbol and non-SBFD slot/symbol can be studied. Separate configuration for SBFD slot/symbol and non-SBFD slot/symbol may increase the configuration/scheduling flexibility, while it will also complicate the system design, especially for HARQ-ACK feedback. Most of the configurations for HARQ-ACK codebook/feedback are associated with PDSCH/PUCCH configuration, if separate PUCCH configurations are configured for SBFD slot/symbol and non-SBFD slot/symbol, HARQ-ACK codebook/feedback mechanism may also need to be updated accordingly.

In addition, even if separate configurations for SBFD slot/symbol and non-SBFD slot/symbol are configured, since the UL subband configuration (e.g., the occupied symbols) in each slot may be different(e.g., X slot and DL slot), UE still needs to check the availability for each configured PUCCH/PUSCH/PDSCH/PDCCH.

Based on the above analysis, there are two options to configure PUCCH/PUSCH/PDSCH/PDCCH resources for the SBFD symbol/slot and non-SBFD symbol/slot.

- Option1: Separate PUCCH/PUSCH/PDSCH/PDCCH resources are configured for the SBFD symbol/slot and non-SBFD symbols/slot respectively.

- Option2: The PUCCH/PUSCH/PDSCH/PDCCH resources corresponding to the non-SBFD symbol/slot are shared for the SBFD symbol/slot.

So, for simplicity and low overhead, we believe that PUCCH/PUSCH/PDSCH/PDCCH resources configured for DL/UL BWP in non-SBFD slot can be shared with SBFD slots. In other words, the existing PUCCH/PUSCH/PDSCH/PDCCH configuration mechanism is reused, and UE detects whether the scheduled/configured PUCCH/PUSCH/PDSCH/PDCCH resource is valid based on the slot type.

Proposal 14: The SRS/PUCCH/PUSCH/PDSCH/PDCCH resources on SBFD symbols and non-SBFD symbols in different slots can be shared or configured separately.

Due to the UL subband being configured in the DL symbol and the UL BWP being configured in the UL symbol, UL transmission is subjected to interference of different intensities. Therefore, independent power control should be used for UL transmission in the UL subband and in the UL BWP, respectively. However, shared power control should also be supported for both UL transmissions if the interference is within the control capability of the base station or within a controllable range. Based on the above analysis, in order to be flexible and efficient, shared or independent power control parameters should be supported for the two UL transmissions in terms of signaling. For example, if one of the two UL transmissions is not configured with corresponding power control parameters, then the UL transmission can share the power control parameters of the other UL transmission.

For similar reasons, in order to be flexible and efficient, shared and independent beam/spatial relationships should be supported for both UL transmissions at the signaling level.

Proposal 15: Regarding SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols in different slots, UL power control parameters and beam/spatial relation can be shared or configured separately.

In case of frequency hopping, without any specific handling, the UL transmission hop may be mapped to the resource out of the UL subband. To address this issue, at least the following two alternatives can be studied.

- Alt.1: Separate frequency hopping parameters are configured for UL transmission in SBFD symbol and non-SBFD symbol

- Alt.2: An additional frequency hopping offset is introduced for the UL transmission in the UL subband to adjust the UL transmission hop location such that it can be confined with the UL subband.

Proposal 16: Regarding UL transmission hopping across SBFD symbol and non-SBFD symbol, the following alternatives can be studied to configure the UL transmission hop within UL subband.

- Alt.1: Separate frequency hopping parameters are configured for UL transmission in SBFD symbol and non-SBFD symbol

- Alt.2: An additional frequency hopping offset is introduced for the UL transmission in the UL subband to adjust the UL transmission hop location such that it can be confined with the UL subband.

In Figure 6, the UL subband is configured in the DL symbol in a slot, and a PUSCH/PUCCH is scheduled in the UL subband, also in the frequency domain of UL BWP. The starting symbol of the PUSCH/PUCCH is in the UL subband, but its ending symbol exceeds the UL subband and is in the UL BWP in the UL symbol. That is, the PUCCH/PUSCH is mapped to SBFD and non-SBFD in the same slot.  From our point of view, the PUSCH/PUCCH should be allowed to be transmitted for higher efficiency. There are similar problems with PDSCH/PDCCH. Therefore, FFS in the above agreement should be studied.

Figure 6: PUCCH/PUSCH mapped to SBFD and non-SBFD in the same slot

Proposal 17: Study impact and potential enhancements for scheduled/configured UL transmissions and scheduled/configured DL receptions across SBFD symbols and non-SBFD symbols, including at least the following:

- PUCCH/PUSCH/PDSCH/PDCCH can be mapped to SBFD symbols and non-SBFD symbols in the same slot if configured.

Regarding whether different receptions for UL transmissions or DL receptions with receptions can use SBFD symbols/slot and non SBFD symbols/slot in different slots, the following two options 2 are provided.

Option 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only.

Option 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols.

One of the main purposes of introducing the UL subband is to increase the opportunity for UL transmission and reduce the latency of UL transmission. The above Option 1 seems to contradict this purpose as it may result in a UL transmission with repetitions not being completed as quickly as possible. Therefore, from our understanding, for a UL transmission with repetition, in order to reduce latency, the different repetitions of the UL transmission can be transmitted in the SBFD symbol/slot and in the non SBFD symbol/slot.

For similar reasons, the different repetitions of the DL reception can be received in the SBFD symbol/slot and in the non SBFD symbol/slot.

The UL transmission and DL reception across SBFD symbol/slot and non-SBFD symbol/slot include the following:

- PDSCH/PUSCH/PUCCH repetitions

- SPS PDSCH/CG PUSCH

- TBoMS

- Multi-PUSCH/PDSCH scheduled by a single DCI

- Periodic/semi-persistent SRS/CSI-RS/PUCCH

- PDCCH

Proposal 18: The different repetitions of the UL transmission/DL reception can be transmitted/received in the SBFD symbol/slot and in the non SBFD symbol/slot.

- The UL transmission and DL reception include the following:

  - PDSCH/PUSCH/PUCCH repetitions

  - SPS PDSCH/CG PUSCH

  - TBoMS

  - Multi-PUSCH/PDSCH scheduled by a single DCI

  - Periodic/semi-persistent SRS/CSI-RS/PUCCH

  - PDCCH

In addition to the transmission across SBFD symbols and non-SBFD symbols, the periodic transmission across SBFD symbols and non-SBFD symbols should also be studied, including CG PUSCH, SPS PDSCH, periodic PUCCH, etc. A periodic transmission occasion is in the SBFD symbols while another occasion is in the non-SBFD symbols. Currently, frequency resource is determined based on the active BWP. Taking the UL transmission as example, it was agreed that the transmission is confined in the UL sub-band or UL BWP. Therefore, how to ensure this for periodic transmission in the SBFD symbols and non-SBFD symbols should be studied when the SBFD symbols and non-SBFD symbols have different bandwidths. In our view, there are two main alternatives.

- Alt.1: Separate indication of the frequency resource for the periodic transmission occasion in SBFD symbols and non-SBFD symbols

- Alt. 2: Shared indication of the frequency resource for the periodic transmission occasion in SBFD symbols and non-SBFD symbols

  - Alt 2-1: The same frequency resource is indicated

  - Alt 2-2: The different frequency resources are indicated

For Alt.1, separate indication may be possible for the frequency resource configured by RRC signaling, e.g., Type-1 CG PUSCH, PUCCH resource. However, it is a bit impossible for the frequency resource indicated by DCI since there is only one FDRA field in the DCI. For Alt.2, it can be applied to the dynamic scheduling without changing anything on DCI. We can obtain both the same frequency resource and different resources. If the same frequency resource for periodic transmission is always used, configuring different bandwidths for UL subband and UL BWP seems meaningless from periodic transmission perspective. In addition, there are also some scheduling limitations introduced. Therefore, different resources indicated by the shared indication based on the different bandwidth of UL BWP and UL subband seems better. For example, the design concept of FDRA field under dynamic BWP switching can be considered as a starting point.

Proposal 19: The frequency resource determination for the periodic transmission occasions across SBFD symbols and non-SBFD symbols should be studied and the following alternatives can be considered.

- Alt.1: Separate indication of the frequency resource for the periodic transmission occasion in SBFD symbols and non-SBFD symbols

- Alt. 2: Shared indication of the frequency resource for the periodic transmission occasion in SBFD symbols and non-SBFD symbols

- Alt 2-1: The same frequency resource is indicated

- Alt 2-2: The different frequency resources are indicated

Whether or not to include both SBFD and non-SBFD symbols in a slot was discussed in the RAN1#112 meeting, but the conclusion was not reached. From our point of view, a slot can consist of both SBFD and non-SBFD symbols. Some reasons are provided as follows.

First of all, this kind of slot is inevitable, such as S slot, which contains DL symbols, F symbols and UL symbols. For the slot, the UL subband can be configured in the DL symbol to reduce the latency of the UL transmission.

Second, parts of the DL symbol in the DL slot can be configured with the UL subband in order to perform fast HARQ-ACK feedback and to send SRS for fast measurement.  For example, some end symbols of DL slot can be configured with the UL subband. For example, to simplify design and complexity, SSB/PDCCH symbols are not configured with  the UL subband.

In addition, the legacy TDD slot format configuration is symbol-level. To be compatible with the legacy TDD slot configuration, it is critical to have symbol-level subband configuration instead of slot-level.

The above methods can improve the flexibility of the system without significant complexity. For some cases, supporting the above methods can effectively reduce complexity, such as skipping SSB and PDCCH symbols. Therefore, we believe that symbol level configuration for the UL subband is necessary. A slot containing both SBFD and non SBFD symbols should also be supported.

Proposal 20: Symbol level configuration for the UL subband should be supported, and a slot containing both SBFD and non SBFD symbols should also be supported.

Initial access

In the RAN1 #111 meeting, the initial access in UL subband was discussed, but no agreement was reached due to the divergence of company views. In our opinion, if initial access is supported in the UL subband, it is obviously beneficial to improve the initial access capability from at least two aspects, such as shortening the initial access time and providing more initial access opportunities. Some further studies are necessary on this aspect.

However, some companies believe that the initial access based on SBFD operation is for IDLE mode, which is not in the current SID [1]. But we think it can still be considered as triggering events related to RRC_CONNECTED mode, such as DL/UL data arrival, contention-free random access, etc.

Proposal 21: RAN1 further studies the potential enhancements for initial access in the UL subband at least for the triggered events related to the RRC_CONNECTED mode.

CLI handling

During RAN1#109-e meeting, discussion scope division among different agenda items were discussed [2]. More specifically, the potential schemes for gNB-to-gNB and UE-to-UE CLI handling and gNB self-interference handling that are specific for SBFD will be discussed under AI 9.3.2.

In RAN1#110, the following agreement about CLI handling specific for SBFD was obtained [3]:

Agreement Study the feasibility and potential benefit of UE-to-UE co-channel CLI measurement and reporting, which can be specific for SBFD, at least includes: - Measurement resource/reporting configuration - Measurement/reporting details (including UE processing delay) - Relevant information exchange (between gNBs) if needed - Usage of measurement at gNB Note: other enhancement(s) for gNB-to-gNB and UE-to-UE CLI handling specific for SBFD are not precluded.

In this section, we provide our further analysis on CLI handling schemes that are specific for SBFD.

gNB self-interference handling

TA offset determination for avoiding inter-slot interference

In RAN1#112 meeting, the following proposal about enhancements of potential unaligned slot/symbol boundary for DL and UL subbands was discussed [7]:

Proposed Agreement: Study impact/potential enhancements of unaligned slot/symbol boundary for DL and UL subbands in SBFD symbols.

In legacy TDD system, UE is provided a TA offset in SIB1, i.e.,  for a serving cell to reserve enough time for UL/DL switching. Further, it is typically configured with a value larger than zero in current TDD network, e.g., 25600 or 39936.

For operating SBFD shown in Figure 7, if a non-zero TA offset is also configured for UL transmission within the UL subband, inter-symbol and inter-slot interference will be generated as the symbols in DL subband and UL suband are not aligned. One way for avoiding this interference is to set  for all UL transmission no matter it is located with the UL subband or UL slot. However, potential backward compatibility problem exists, that is, legacy UE may not support to set  to zero under the TDD system.

Observation 1: The potential backward compatibility problem exists if  is set for all UL transmission.

Another way is to define two TA offset values for a SBFD capable UE for UL transmission in UL subband and UL slot, respectively. More specifically, one is zero for UL transmission in UL subband and another value is larger than zero for UL transmission in UL slot. Then, UL transmission of the legacy UE can also be scheduled or configured within UL slot. For SBFD capable UE, it should support two different TA offset values for UL subband and UL slot.

Figure 7: Inter-slot interference for SBFD

Proposal 22: For avoiding inter-slot and inter-symbol self-interference under SBFD, two values of  can be defined for UL transmission in UL subband and UL slot, respectively.

- for UL transmission in UL subband

- for UL transmission in UL slot

Power control based solution

DL transmissions need to be filtered before transmitting over the air so that power can be restricted within the desired frequency range. Ideally, after filtering, there should be no leakage signal out of the desired frequency range. However, due to the limitation of technology and implementation complexity, there are always leakage signals out of the desired frequency range. For SBFD, the transmission power of DL transmission within DL suband will leak to UL subband, which is decreasing with the increase of frequency distance. The uplink transmissions in the UL subband are subject to different interference levels, depending on the frequency domain isolation between it and the DL subband. As an example shown in Figure 8, the UL transmission located within frequency Area 1, Area 2 and Area 3 will suffer inter-subband interference with highest, medium and lowest level, respectively.

Figure 8: gNB self-interference with different levels on different frequency areas

Observation 2: The uplink transmissions in the UL subband are subject to different interference levels due to different frequency domain isolation between the uplink transmission and the DL subband

Different interference levels require different power control parameters for improving the performance of UL transmission. One way is to divide resources with different interference levels into multiple areas and make each area mapped with a dedicated power control parameter set. The resources contained in each area can be semi-statically configured via RRC or indicated by DCI. The DCI for indicating the area is similar to DCI format for UL cancellation, where multiple resource indication fields correspond to multiple resource sets one by one respectively. The power control parameters set can be either open-loop power control parameter (e.g., P0, alpha) or closed-loop power control parameter (e.g., TPC table). The transmission power of an UL transmission will be determined according to the power control parameters set corresponding the area where the UL transmission is located.

Proposal 23: UL subband resources can be divided into multiple areas and each area is mapped with a dedicated power control parameter set for compensating the different levels of inter-subband interference.

- The resources contained in each area can be semi-statically configured via RRC or indicated by DCI.

UE-to-UE CLI handling

UE-to-UE CLI measurement and reporting

According to discussion in RAN1#110bis-e meeting, L1/L2 based CLI measurement/report for UE-to-UE CLI handling is to be discussed in AI 9.3.3. However, considering that UE-to-UE inter-subband CLI as shown in Figure 9 has a non-uniform feature, which is specific for SBFD, some UE-to-UE CLI measurement and reporting mechanisms focus on this feature should also be discussed in AI 9.3.2.

Figure 9: UE-to-UE inter-subband CLI with different levels on different frequency areas

Proposal 24: UE-to-UE CLI measurement and reporting mechanisms for non-uniform inter-subband interference should be discussed in AI 9.3.2.

In RAN1#112 meeting, the following agreement about UE-to-UE CLI measurement/report considering non-uniform CLI was reached [8]:

Agreement For inter-UE inter-subband CLI measurement, study at least the following methods: - Method#1: victim UE measures RSSI within DL subband - FFS: Whether SINR can be measured - Method#2: victim UE measures RSRP of aggressor UE within UL subband - Method#3: victim UE measures RSSI within UL subband - Note: the restriction in Rel-16 that CLI is only measured within DL BWP does not forbid UE to measure CLI in UL subband when UL subband is confined within DL BWP.

Regarding UE-to-UE CLI measurement/reporting, at least Method#1, i.e., RSSI measurement in DL subband should be supported, through which the non-uniform CLI can be obtained. In addition, Method#2, i.e., RSRP measurement within UL subband can also be considered in order to identify the aggressor. From this point of view, there may be no motivation for further supporting Method#3, i.e., RSSI measurement within UL subband.

Observation 3: RSSI measurement in DL subband is helpful for obtaining inter-subbands interference with a non-uniform feature. And RSRP measurement within UL subband is benefit to identify the aggressor identity.

Proposal 25: UE-to-UE CLI measurement/reporting in both of RSSI measurement in DL subband and RSRP measurment in UL subband should be supported.

In addition, only wideband CLI measurement and reporting is supported in Rel-16 CLI. It may fail to reflect the changes of interference in different frequency resources. Therefore, it is necessary to introduce a finer frequency domain measurement granularity, for example, subband CLI measurement and reporting.

Observation 4: Wideband CLI measurement and reporting may fail to reflect the changes of inter-subband interference in different frequency resources.

For supporting subband CLI measurement and reporting, the current mechanism of subband CQI/PMI measurement and reporting can be considered as a starting point. A further research direction is impact of introduction of the UL subband on existing mechanism, for example, configuration and determination of the measurement subband size, and measurement reporting overheads reduction, etc.

Proposal 26: Regarding UE-to-UE CLI measurement/reporting, subband CLI measurement and reporting should be supported,

- Further study on configuration and determination of the measurement subband size and measurement reporting overheads reduction, etc.

As we discussed in Section 4.1.1 of our companion contribution under AI9.3.3 [6], UE is difficult to derive the reception timing accurately for UE-to-UE CLI measurement without any information exchange. This issue also applies to SBFD system. Similar approach can be considered for UEs under SBFD as well.

Proposal 27: Timing alignment solution on measurement RS transmission for UE-to-UE CLI should be considered in Rel-18.

- For example, exchange timing related information for reception of measurement RS.

Reference signals with different frequency densities

Reference signals may be needed to measure the interference levels between UEs or to estimate channel for downlink transmission. According to the analysis in Section 3.2.1, different areas with different frequency isolation from the aggressor lead to different interference levels. To obtain more accurate CLI measurement or channel estimation results with less reference signal overhead, reference signals can be configured with different frequency densities in different areas. For example, as shown in Figure 9, the DL subband is divided into three areas. The Area 1 is closest to the UL subband in frequency domain, in which a higher interference with faster changes can be expected, thus the reference signal in the first part can be configured with higher frequency density. The Area 3 is far away from the UL subband in the frequency domain, the interference level may be low and flat in this area. Thus, the reference signal in this area can be configured with lower frequency density.

Proposal 28: Different frequency densities can be configured for reference signals (e.g., RS for UE-to-UE CLI measurement, RS for downlink transmission channel estimation) transmitted in different areas with different interference levels.

Power control based solution

Similar to gNB self-interference, the uplink transmissions in the UL subband generate UE-to-UE CLI with different interference levels due to different frequency domain isolation between the uplink transmission and the DL subband. Furthermore, the maximum interference level that an uplink transmission is allowed to generate is also related to whether there is a downlink transmission located within the time domain areas where uplink transmissions are scheduled or configured in UL subband.

As an example shown in Figure 10, UL subband is divided into three areas in the frequency domain. A higher transmission power can be used for uplink transmission in Area 1 as it is far away from DL subband and a lower interference can be expected. For Area 2 and Area 3, a lower transmission power can be used for uplink transmission in them as they are closed to DL subband.

Figure 10: Different allowed maximum power for UL transmission in different areas

Therefore, similar method as described in section 4.1.2 can be used for UE-to-UE CLI handling. That is, resources with different interference levels can be divided into multiple areas and each area is mapped with a dedicated power control parameter set. The resources contained in each area can be semi-statically configured via RRC or indicated by DCI. The DCI format for indicating the area is similar to that for UL cancellation, where multiple resource indication fields correspond to multiple resource sets one by one respectively. The power control parameters set can be either open-loop power control parameter (e.g., P0, alpha) or closed-loop power control parameter (e.g., TPC table). The transmission power of an UL transmission will be determined according to the power control parameters set corresponding the area where the UL transmission is located.

Proposal 29: Regarding UE-to-UE CLI, UL subband resources can be divided into multiple areas and each area is mapped with a dedicated power control parameter set for suppressing the UE-to-UE CLI with different interference levels.

- The resources contained in each area can be semi-statically configured by RRC or indicated by DCI.

gNB-to-gNB CLI handling

Timing misalignment

As we discussed in Section 3.3 of our companion contribution under AI9.3.3 [6], timing misalignment between gNBs is observed. Without any further enhancements, inter-symbol and inter-slot gNB-gNB inter-subband interference will impact the UL reception in the victim gNB.  In addition, it is impossible to measure the channel state information between aggressor gNB and victim gNB accurately with the uncertain timing misalignment especially if the timing misalignment is larger than the CP.

The timing difference between gNBs is mainly caused by three parts, i.e., T1, T2 and T3.

- T1: DL Tx timing difference between the aggressor and the victim. As defined in [5], the maximum absolute deviation in frame start timing between any pair of TDD cells on the same frequency that have overlapping coverage areas is 3 µs, i.e., T1 ≤ 3us.

- T2: Transmission delay between the aggressor and the victim. About 1.67 µs is required under the deployment scenario of Urban Macro with 500 m inter-BS distance.

- T3: The UL symbol is shifted by NTA_offset (13 us) in the TDD system in order to allow some time for gNB to perform UL-to-DL transition without impacting the UL/DL transmission.

Since the major part of the timing misalignment between gNBs is the NTA_offset . Similar approach as discussed in section 3.1.1 in this contribution can be applied to address this issue. If the NTA_offset is set as 0 for the UL subband for the gNBs, then this issue can be addressed.

Proposal 30: For avoiding inter-slot/inter-symbol gNB-gNB inter-subband interference and for accurate gNB-gNB channel state measurement, two values of  can be defined for UL transmission in UL subband and UL slot, respectively.

- for UL transmission in UL subband

- for UL transmission in UL slot

Reference signals with different frequency densities

Similar as UE-to-UE inter-subband CLI, reference signals may be needed to measure the interference levels between gNBs or to estimate channel for uplink transmission. Then, reference signals can also be configured with different frequency densities in different areas with non-uniform CLI.

Proposal 31: Different frequency densities can be configured for reference signals (e.g., RS for gNB-to-gNB CLI measurement, RS for uplink transmission channel estimation) transmitted in different areas with different interference levels.

Information exchange across gNBs

In RAN1#112 meeting, the following agreement about information exchange across gNBs for gNB-to-gNB CLI handling was made [5]:

Agreement Study the benefit of knowledge among gNBs of configurations such as - SBFD time/frequency configuration

From our perspective, SBFD time/frequency configuration, including time and frequency domain configuration of UL subband, exchanged between gNBs can help them for determining the measurement behavior and interference coordination mechanism for handling the CLI specific for SBFD. For example, CLI measurement of specific for SBFD needs only to be performed on an SBFD resource, and coordinated scheduling of time-frequency domain resource also depends on determining of the SBFD resource.

Proposal 32: Time and frequency domain configuration of UL subband should be exchanged between gNBs.

Conclusion

In this contribution, we provide our analysis for subband non-overlapping full duplex for possible solutions with the following proposals and observations.

SBFD framework and configuration

Proposal 1: In addition to the RB-set based solution, RAN1 further studies the BWP-based solution for SBFD configuration.

Proposal 2: RAN1 studies the flexible subband/resource with the following potential alternatives.

- Alt.1:  subband that can be used to transmit UL and DL is considered as flexible subband.

- Alt.2: subband that configured in the flexible symbols is considered as flexible subband.

- Alt.3: part of the resources within the UL subband is configured as flexible resource, within which DL transmission and UL transmission are allowed.

Proposal 3: Regarding the UL suband configuration, both cell-common configurations and UE-specific configurations are needed.

- Example of cell-common UL subband configurations: frequency/time resource of the UL subband, PUSCH/PUCCH/RACH /SCS/CP configuration for IDLE UE or for UE without UE-specific configuration, etc

- Example of UE-specific UL subband configurations: PUSCH/PUCCH/RACH/SCS/CP configuration for CONNECTED UE

- FFS: Whether UE-specific bandwidth configuration of UL subband is needed or not.

Proposal 4: For semi-static configuration of subband time locations for SBFD operation, the following two options are studied:

- Option 1: the time domain window (e.g., period, offset and length) can be configured by RRC for SBFD operation for better compatibility with less impact on legacy UE and procedures.

- Option 2: the period for subband is determined by the periodicities of TDD UL-DL pattern(s) configured in TDD-UL-DL-ConfigCommon and TDD-UL-DL-ConfigDedicated

Proposal 5: For SBFD operation, guard band can be reserved between DL subband and UL subband explicitly or implicitly.

- UE is not expected to transmit uplink transmission in the guard band.

- UE is allowed to receive DL transmission in the guard band.

Proposal 6: RAN1 further studies the following alternatives for UL subband configuration.

- Alt.1: Each UL BWP can be associated with up to one UL subband. More than one UL subband for different UL BWPs configured in one carrier can be configured, while up to one UL subband can be activated at one time per UE.

- Alt.2: One UL subband is configured in one carrier. The UL subband is activated only if the UL subband is confined within the activated UL BWP.

Proposal 7: In addition to the semi-static UL subband configuration, RAN1 supports explicitly/implicitly updating the SBFD symbols of the UL subband dynamically.

Proposal 8: For a SBFD aware UE semi-statically configured with UL subband in a SBFD symbol configured as DL in TDD-UL-DL-ConfigCommon, the following is agreed as baseline in the RAN1 study:

- The SBFD symbol can be converted to the DL symbol and DL receptions are allowed in the DL symbol converted from the SBFD symbol

Proposal 9: For a SBFD aware UE semi-statically configured with UL subband in a SBFD symbol configured as F in TDD-UL-DL-ConfigCommon, the following is agreed as baseline in the RAN1 study:

- The SBFD symbol can be converted to the F symbol and DL receptions are allowed in the F symbol converted from the SBFD symbol

Proposal 10: For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) are allowed or not in a symbol configured as DL in TDD-UL-DL-ConfigCommon based on the following option 2:

    - Option 2: DL receptions outside semi-statically configured DL subband(s) are allowed

For SBFD-aware UEs, further study whether DL receptions outside semi-statically configured DL subband(s) and UL transmissions outside semi-statically configured UL subband are allowed or not in the symbol configured as flexible in TDD-UL-DL-ConfigCommon based on the following option 2:

    - Option 2: DL receptions outside semi-statically configured DL subband(s) are allowed

      - UL transmissions outside the semi-statically configured UL subbands are not allowed

Proposal 11: RAN1 considers supporting some urgent and necessary DL transmissions (including reference signals and SSB) to be transmitted in the resource containing the UL subband.

Proposal 12: Regarding confliction of Case 1: UL vs. DL, consider the following potential solution.

- Option 1: When DL transmission and UL transmission have the same priority and both of them are DG or CG, if DL transmission is scheduled in DL subband and UL transmission is scheduled in UL subband, UL transmission scheduled in UL subband is always prioritized. If both DL transmission and UL transmission are scheduled in the UL subband, DL transmission is always prioritized.

- Option 2: When DL transmission and UL transmission have different priorities, DL/UL transmission with high priority always survives.

- Option 3: When dynamic DL (or UL) transmission and semi-static UL (or DL) transmission have the same priority and conflict in the time domain, the dynamic transmission is prioritized.

Proposal 13: RAN1 studies UE collision handling at least for the following cases.

- Case 1: UL vs. DL

- Case 2: Semi D vs. UL

- Case 3: SSB vs. UL

- Case 4: Type 0 CSS vs. UL

Proposal 14: The SRS/PUCCH/PUSCH/PDSCH/PDCCH resources on SBFD symbols and non-SBFD symbols in different slots can be shared or configured separately.

Proposal 15: Regarding SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols in different slots, UL power control parameters and beam/spatial relation can be shared or configured separately.

Proposal 16: Regarding UL transmission hopping across SBFD symbol and non-SBFD symbol, the following alternatives can be studied to configure the UL transmission hop within UL subband.

- Alt.1: Separate frequency hopping parameters are configured for UL transmission in SBFD symbol and non-SBFD symbol

- Alt.2: An additional frequency hopping offset is introduced for the UL transmission in the UL subband to adjust the UL transmission hop location such that it can be confined with the UL subband.

Proposal 17: Study impact and potential enhancements for scheduled/configured UL transmissions and scheduled/configured DL receptions across SBFD symbols and non-SBFD symbols, including at least the following:

- PUCCH/PUSCH/PDSCH/PDCCH can be mapped to SBFD symbols and non-SBFD symbols in the same slot if configured.

Proposal 18: The different repetitions of the UL transmission/DL reception can be transmitted/received in the SBFD symbol/slot and in the non SBFD symbol/slot.

- The UL transmission and DL reception include the following:

  - PDSCH/PUSCH/PUCCH repetitions

  - SPS PDSCH/CG PUSCH

  - TBoMS

  - Multi-PUSCH/PDSCH scheduled by a single DCI

  - Periodic/semi-persistent SRS/CSI-RS/PUCCH

  - PDCCH

Proposal 19: The frequency resource determination for the periodic transmission occasions across SBFD symbols and non-SBFD symbols should be studied and the following alternatives can be considered.

- Alt.1: Separate indication of the frequency resource for the periodic transmission occasion in SBFD symbols and non-SBFD symbols

- Alt. 2: Shared indication of the frequency resource for the periodic transmission occasion in SBFD symbols and non-SBFD symbols

- Alt 2-1: The same frequency resource is indicated

- Alt 2-2: The different frequency resources are indicated

Proposal 20: Symbol level configuration for the UL subband should be supported, and a slot containing both SBFD and non SBFD symbols should also be supported.

Proposal 21: RAN1 further studies the potential enhancements for initial access in the UL subband at least for the triggered events related to the RRC_CONNECTED mode.

CLI handling

Observation 1: The potential backward compatibility problem exists if  is set for all UL transmission.

Proposal 22: For avoiding inter-slot and inter-symbol self-interference under SBFD, two values of  can be defined for UL transmission in UL subband and UL slot, respectively.

- for UL transmission in UL subband

- for UL transmission in UL slot

Observation 2: The uplink transmissions in the UL subband are subject to different interference levels due to different frequency domain isolation between the uplink transmission and the DL subband

Proposal 23: UL subband resources can be divided into multiple areas and each area is mapped with a dedicated power control parameter set for compensating the different levels of inter-subband interference.

- The resources contained in each area can be semi-statically configured via RRC or indicated by DCI.

Proposal 24: UE-to-UE CLI measurement and reporting mechanisms for non-uniform inter-subband interference should be discussed in AI 9.3.2.

Observation 3: RSSI measurement in DL subband is helpful for obtaining inter-subbands interference with a non-uniform feature. And RSRP measurement within UL subband is benefit to identify the aggressor identity.

Proposal 25: UE-to-UE CLI measurement/reporting in both of RSSI measurement in DL subband and RSRP measurment in UL subband should be supported.

Observation 4: Wideband CLI measurement and reporting may fail to reflect the changes of inter-subband interference in different frequency resources.

Proposal 26: Regarding UE-to-UE CLI measurement/reporting, subband CLI measurement and reporting should be supported,

- Further study on configuration and determination of the measurement subband size and measurement reporting overheads reduction, etc.

Proposal 27: Timing alignment solution on measurement RS transmission for UE-to-UE CLI should be considered in Rel-18.

- For example, exchange timing related information for reception of measurement RS.

Proposal 28: Different frequency densities can be configured for reference signals (e.g., RS for UE-to-UE CLI measurement, RS for downlink transmission channel estimation) transmitted in different areas with different interference levels.

Proposal 29: Regarding UE-to-UE CLI, UL subband resources can be divided into multiple areas and each area is mapped with a dedicated power control parameter set for suppressing the UE-to-UE CLI with different interference levels.

- The resources contained in each area can be semi-statically configured by RRC or indicated by DCI.

Proposal 30: For avoiding inter-slot/inter-symbol gNB-gNB inter-subband interference and for accurate gNB-gNB channel state measurement, two values of  can be defined for UL transmission in UL subband and UL slot, respectively.

- for UL transmission in UL subband

- for UL transmission in UL slot

Proposal 31: Different frequency densities can be configured for reference signals (e.g., RS for gNB-to-gNB CLI measurement, RS for uplink transmission channel estimation) transmitted in different areas with different interference levels.

Proposal 32: Time and frequency domain configuration of UL subband should be exchanged between gNBs.

Reference

- RP-221352, “Revised SID: Study on evolution of NR duplex operation”, CMCC (Moderator)

- 3GPP TSG RAN WG1 #109-e, “RAN1 Chair’s Notes”

- 3GPP TSG RAN WG1 #110-e, “RAN1 Chair’s Notes”

- R1-2210317, Summary #4 of subband non-overlapping full duplex, Moderator (CATT).

- R1-2212735, Summary #3 of subband non-overlapping full duplex, Moderator (CATT).

- R1-2300341, Discussion of enhancements on dynamic/flexible TDD, ZTE, China Telecom.

- R1-2301935, Summary #3 of subband non-overlapping full duplex, Moderator (CATT).

- 3GPP TSG RAN WG1 #112, “RAN1 Chair’s Notes”