3GPP TSG RAN WG1 #112bis-e                                                      R1-2302760

E-Meeting, April 17th – April 26th, 2023

Source: 	ZTE

Title: 	Discussion on power domain enhancements

Agenda item:	9.14.2

Document for:	Discussion/Decision

Introduction

For MPR/PAR reduction, RAN1 has provided LLS results of techniques for MPR/PAR reduction schemes in RAN1#112 [1], and sent LS to RAN4. The main remaining evaluation effort in RAN1 is about DMRS design [2]. For increasing UE power high limit for CA and DC, the progress in RAN1 is slow due to potential RAN4 dependency. In this contribution, we provide our view on the remaining issues for power domain enhancements.

Increasing UE power higher limit for CA/DC

In RAN1#112, the following agreement regarding increasing UE power higher limit for CA/DC was reached as follow. With this, RAN1 will only focus on increasing gNB awareness of UE’s Tx power.

Agreement Further discussions in RAN1 concerning means to facilitate higher power transmissions in CA and DC, if applicable, can target increasing gNB awareness of UE’s Tx power, e.g., PHR reporting enhancement such as current power class, power class change, or application of P-MPR by UE (subject to RAN4’s input). - FFS: details.

In RAN4#106, it was agreed that the delivery of high-power UL across all bands is dependent on SAR/MPE considerations at the UE during CA/DC operation [3]. In our view, it means the discussion on information exchange needed between the UE and gNB being discussed in RAN1 is not dependent on RAN4 discussion on increasing UE power high limit for CA and DC, e.g., whether/how to introduce new high-power limits for different band combinations. In this sense, RAN1 can proceed the discussion on increasing gNB awareness of UE’s Tx power without subjecting to RAN4’s inputs.

Observation 1: According to RAN4 LS, RAN1 can proceed the discussion on increasing gNB awareness of UE’s Tx power without subjecting to RAN4’s inputs.

According to RAN4 LS [3], RAN4 has discussed several possible solutions (Issue 5 in Topic#2 in [4]) for information exchange needed between the UE and gNB to improve scheduling and network performance, which are also listed below.

- Issue 5: Whether and how PHR reporting enhancement should be considered for FR1 carriers <Recommended WF> - RAN4 discussion will focus on the following solutions that have been proposed in this meeting: - Power class fallback ΔPPowerClass with aperiodic PHR. - Report power-class fallback ΔPPowerClass in the PHR per serving cell, any power-class change, fallback or return to declared power class, should trigger an aperiodic PHR. This also includes FDD PC2. - Report power-class fallback ΔPPowerClass,CA in the multi-entry PHR for the BC; any BC power-class change, fallback or return to advertised BC power class, should also trigger an aperiodic PHR. - For EN-DC report power-class fallback ΔPPowerClass,EN-DC in the multi-entry PHR for the BC. - Power class being used by the UE. Because reporting ΔPPowerClass must be a huge burden for both UE and network. - For single band HPUE operation, PC being used by a UE must be able to be reported per serving cell. - For UL inter band CA HPUE operation, PC being used by a UE must be able to be reported per serving cell per band within a band combination as well as CA PC being used CA for the band combination itself. - The sustainable duty cycle over a certain duration that would prevent triggering a power class fallback at the UE, as well as period of applicability of the ∆PPowerClass report. - Introduce a scheme for a UE to report uplink symbol evaluation period and starting timing. - Enhance the current power headroom reporting framework to enable P-MPR reporting (via MPE field) for FR1 carriers.

In our view, this is mainly caused by the ambiguity of the evaluation period for UE power class fallback in TS 38.101. Different UE vendors may have different understandings and implementations about when and how long the evaluation period is, which makes UE transmit power unclear to gNB. Basically, power class fallback mechanism based on duty cycle is broken.

If a UE supports a different power class than the default UE power class for the band and the supported power class enables the higher maximum output power than that of the default power class: - if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 50% (The exact evaluation period is no less than one radio frame); or - if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or - if the IE P-Max as defined in TS 38.331 [7] is provided and set to the maximum output power of the default power class or lower; - shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified in clause 6.2.4;

Observation 2: gNB is unable to know the exact UE transmit power due to the ambiguity of when and how long the evaluation period for UE power class fallback is implemented.

With above, we believe it is important for gNB to know more accurate UE transmit power so as to make more accurate UL power control and adaptive modulation and coding (AMC). For instance, under existing PHR reporting mechanism, gNB may not be able to determine the range of UE transmit power due to unknown of current UE power class and MPR etc, according to the power calculation in Section 6.2.4 in TS 38.101. More importantly, gNB would not be able to know how long the UE will be able to use this power for the subsequent transmissions. To address the ambiguity issue, we prefer the following solutions based on the discussion in RAN4.

- Alt 1. PHR reporting enhancement with a certain duration for the applicability of one among {the fallback power class ∆PPowerClass (potentially with a finer granularity), the default power class, or Pc,max}. In case the duration is applied for the fallback power class ∆PPowerClass or the default power class is chosen, gNB will know the duration applying the fallback power class and to recover the supported power class after the reported duration. In case the duration is applied for Pc,max, gNB will know the exact power available for a certain period from UE side.

  - Note that, it is not preferred to specify a certain duration for the applicability of the power class other than the default UE power class (e.g., PC2). This could imply that gNB should ensure that the percentage of uplink symbols transmitted in the certain duration cannot be larger than the UE supported duty cycle, which would cause power fallback. This would cause scheduling restrictions at gNB side.

- Alt 2. Introduce a scheme for a UE to report uplink symbol evaluation period and starting timing. With this alternative, similarly, gNB will know the duration applying the fallback power class and the time to recover the supported power class after the duration.

Proposal 1: To address the ambiguity of evaluation period for UE power class fallback, support one of the following alternatives.

- Alt 1. PHR reporting enhancement with a certain duration for the applicability of one among {the fallback power class ∆PPowerClass (potentially with a finer granularity), the default power class, or Pc,max}.

- Alt 2. Introduce a scheme for a UE to report uplink symbol evaluation period and starting timing.

MPR/PAR reduction schemes

In RAN1 #112 meeting, evaluation results on ΔSNR, ΔCM, ΔPAPR and OBO were summarized and included in the LS sent to RAN4. The main remaining evaluation effort in RAN1 is about DMRS design. The corresponding agreements made regarding the DMRS design are listed as following:

Agreement If FDSS-SE is supported in Rel-18, RAN1 to further study the following approaches for DMRS, when the DMRS sequence length before extension of the sequence, if any, is larger than or equal to 30: - Approach A – the DMRS sequence is extended: A DMRS sequence is generated considering the number of PRBs in the inband (no extension). The sequence length depends on the number of PRBs in the inband. Two sequence types can be considered: - A.1: The sequence is a Type 1 DMRS sequence. - A.2: The sequence is a Type 2 DMRS sequence. FFS: how the sequence is extended. - Approach B – the DMRS sequence is not extended: A DMRS sequence based on type 1 or type 2 DMRS sequence is generated considering the number of PRBs in the inband + extension. The sequence length depends on the number of PRBs in the inband + extension. Note: if type 2 is used then both the number of PRBs in the inband and the number of PRBs in the inband+extension must be valid DFT sizes as per NR specification Performance metrics considered for the study are PAPR, CM[, and OBO] for DMRS and 10% BLER SNR for data (to measure channel estimation accuracy). Agreement If FDSS-SE is supported in Rel-18, and RB allocations resulting in DMRS sequence length smaller than 30 before extension of the sequence, if any, are supported, RAN1 to study at least the following approaches: - Approach A – the DMRS sequence is extended: A DMRS sequence is generated considering the number of PRBs in the inband (no extension). The sequence length depends on the number of PRBs in the inband. Two sequence types can be considered: - A.1: The sequence is obtained by DFT transformation of an existing DMRS sequence, e.g., Type 1 DMRS sequence. - A.2: The sequence is a Type 1 or Type 2 DMRS sequence. FFS: how the sequence is extended. - Approach B – the DMRS sequence is not extended: A DMRS sequence based on type 1 or type 2 DMRS sequence is generated considering the number of PRBs in the inband + extension. The sequence length depends on the number of PRBs in the inband + extension. Note: if type 2 is used then both the number of PRBs in the inband and the number of PRBs in the inband+extension must be valid DFT sizes as per NR specification Note: Other sequences are not precluded for Approach A and Approach B. Performance metrics considered for the study are PAPR, CM [, and OBO] for DMRS and 10% BLER SNR for data (to measure channel estimation accuracy).

In our view, different DMRS designs do not impact the LLS directly. So, we focus on the PAPR/CM evaluation for the candidate solutions for DMRS according to the working assumptions made in last meeting (see in the Appendix) in this section. Since the MCS dose not impact the CM/PAPR performance, the configurations for DMRS with FDSS-SE according to the working assumptions can be simplified as Table-1.

Table-1. The RB configurations for DMRS with FDSS-SE.

| No spectrum extension | With spectrum extension |  |  |
| --- | --- | --- | --- |
| #PRBs | #PRBs before extension | #PRBs after extension | Spectrum extension factor |
| 8 | 6 | 8 | 1/4 |
| 40 | 30 | 40 | 1/4 |
| 6 | 4 | 6 | 1/3 |
| 36 | 32 | 36 | 1/9 |

Evaluation results in case of sequence length before extension larger than or equal to 30

In this subsection, we provide the evaluation results in case the sequence length before extension is larger than or equal to 30, i.e., the number of PRBs after extension is 8, 36 and 40.

As an example, the DMRS signal structures of different approaches for 8 PRBs (with SE = 1/4) are provided below.

- For Approach A: the sequence length for inband is 36 (=6*12/2), the extension length on each side would be 6 assuming symmetric extension is used for spectrum extension.

  - Type 1 DMRS: the length for ZC root sequence is 31, and only 30 root sequences for this case.

    - Cyclic extension (i.e. Option 1b): {s25, s26,s27,...,s30, s0,s1,...,s30,s0,s1,...,s4, s5,s6,...,s10}

    - Symmetric extension (i.e. Option 1c): {s30, s0,s1,...,s4, s0,s1,...,s30,s0,s1,...,s4, s0,s1,...,s5}

      - This is Option 1c discussed in RAN1#111.

  - Type 2 DMRS

    - Cyclic extension is the same as symmetric extension: {s30, s31,...,s35, s0,s1,...,s35, s0,s1,...,s5}

- For Approach B: the sequence length for (inband + extension) is 48 (=8*12/2)

  - Type 1 DMRS: the length for ZC root sequence is 47: {s0,s1,...,s30,s0,s1,...,s46,s0}

  - Type 2 DMRS: {s0,s1,...,s47}

The detailed results for number of PRBs 8, 36 and 40 are shown in Table-2, Table-3 and Table 4 below. The evaluation figures are also provided in the appendix. Note that for Type 1 DMRS sequences, there are only 30 and 60 root CAZAC sequences defined in current specification for sequences with length of smaller than 30 and larger than or equal to 30 respectively. So, the CCDF@1% for CM may not be able to obtain, and we provide the max/mean CM instead. For PAPR, the time domain sequence for the DMRS sequence is firstly obtained. And then calculate the peak-to-average for each sample in the time domain sequence. CCDF is then obtained for each DMRS sequence. The final results shown below is the average of the CCDF of multiple DMRS sequences.

Table-2. PAPR/CM evaluation results for 8RBs (after extension) with SE=1/4

|  |  | PAPR CCDF@1% | CM CCDF@1% | Mean CM | Max CM |
| --- | --- | --- | --- | --- | --- |
| Type 1 | Approach B, No FDSS | 4.13 | - | 1.0194 | 2.1071 |
|  | Approach B, FDSS | 3.28 | - | 1.0665 | 1.5361 |
|  | Approach A, Cyclic extension, FDSS | 2.04 | - | -0.007 | 0.1509 |
|  | Approach A, Symmetric extension, FDSS | 3.77 | - | 0.8911 | 1.3504 |
| Type 2 | Approach B, No FDSS | 3.51 | 0.7646 | 0.2425 | 1.2656 |
|  | Approach B, FDSS | 1.29 | -0.6888 | -0.7954 | -0.6079 |
|  | Approach A, FDSS | 1.55 | -0.6643 | -0.7353 | -0.6154 |

Table-3. PAPR/CM evaluation results for 36RBs (after extension) with SE=1/9

|  |  | PAPR CCDF@1% | CM CCDF@1% | Mean CM | Max CM |
| --- | --- | --- | --- | --- | --- |
| Type 1 | Approach B, No FDSS | 4.35 | - | 0.8706 | 2.1921 |
|  | Approach B, FDSS | 3.39 | - | 1.1725 | 1.5468 |
|  | Approach A, Symmetric extension, FDSS | 3.04 | - | 0.7964 | 0.9919 |
|  | Approach A, Symmetric extension, FDSS | 3.79 | - | 1.1900 | 1.3853 |
| Type 2 | Approach B, No FDSS | 3.50 | 0.4852 | 0.2408 | 0.8347 |
|  | Approach B, FDSS | 1.29 | -0.7522 | -0.7954 | -0.7158 |
| - | Approach A, FDSS | 1.26 | -0.8071 | -0.8361 | -0.7874 |

Table-4. PAPR/CM evaluation results for 40RBs (after extension) with SE=1/4

|  |  | PAPR CCDF@1% | CM CCDF@1% | Mean CM | Max CM |
| --- | --- | --- | --- | --- | --- |
| Type 1 | Approach B, No FDSS | 4.50 | - | 0.9072 | 2.3199 |
|  | Approach B, FDSS | 3.40 | - | 0.1388 | 1.6601 |
|  | Approach A, Cyclic extension, FDSS | 2.56 | - | 0.2584 | 0.3254 |
|  | Approach A, Symmetric extension, FDSS | 4.25 | - | 1.2828 | 1.7057 |
| Type 2 | Approach B, No FDSS | 3.50 | 0.4769 | 0.2425 | 0.6197 |
|  | Approach B, FDSS | 1.28 | -0.7547 | -0.7953 | -0.7097 |
| - | Approach A, FDSS | 1.56 | -0.7057 | -0.7366 | -0.6761 |

Evaluation results in case of sequence length before extension less 30

In this subsection, we provide the evaluation results in case the sequence length before extension is less than 30, i.e., the number of PRBs after extension is 6.

The DMRS signal structures of different approaches for 6 PRBs (with SE = 1/3) are provided below.

- For Approach A: the sequence length for inband is 24 (=4*12/2), and the extension length on each side would be 6.

  - Type 1 DMRS: the length for CG-CAZAC root sequence is 24. DFT and no DFT operation are performed respectively. Here, we denote {S0,S1,...,S23}= DFT{s0,s1,...,s23}

    - No DFT, Sequence after extension: {s18,s19,...,s23,s0,s1,...,s23,s0,s1,...,s5}

    - DFT, Sequence after extension: {S18,S19,...,S23,S0,S1,...,S23,S0,S1,...,S5}

  - Type 2 DMRS

    - Sequence after extension: {s18,s19,...,s23,s0,s1,...,s23,s0,s1,...,s5}

- For Approach B: the sequence length for (inband + extension) is 36 (=6*12/2)

  - Type 1 DMRS: the length for ZC root sequence is 31: {s0,s1,...,s30,s0,s1,...,s4}

  - Type 2 DMRS: {s0,s1,...,s35}

The detailed results are shown in Table-5 below. The evaluation figures are also provided in the appendix.

Table-5. PAPR/CM evaluation results for 6 RBs (after extension) with SE=1/3

|  |  | PAPR CCDF@1% | CM CCDF@1% | Mean CM | Max CM |
| --- | --- | --- | --- | --- | --- |
| Type 1 | Approach B, No FDSS | 3.77 | - | 0.8826 | 1.3198 |
|  | Approach B, FDSS | 3.05 | - | 0.7239 | 1.0307 |
|  | Approach A, DFT, FDSS | 2.13 | - | 0.0671 | 0.3795 |
|  | Approach A, No DFT, FDSS | 3.75 | - | 1.2338 | 1.6599 |
| Type 2 | Approach B, No FDSS | 3.52 | 0.8548 | 0.2379 | 1.2718 |
|  | Approach B, FDSS | 1.28 | -0.6604 | -0.7949 | -0.4833 |
|  | Approach A, FDSS | 1.73 | - | -0.5076 | -0.4428 |

Summary of evaluation results for DMRS designs

The results shown in above subsections are further summarized in Table -6. The CM/PAPR evaluation results for QPSK-FDSS-SE of data are also included in this table for reference.

Table-6. Evaluation summary for PAPR/CM results for both data and DMRS

|  |  |  | PAPR CCDF @1% |  |  |  | CM CCDF @1% |  |  |  | Mean CM |  |  |  |  | Max CM |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 6RBs | 8RBs | 36RBs | 40RBs | 6RBs | 8RBs | 36RBs | 40RBs | 6RBs | 8RBs | 36RBs | 40RBs | 6RBs |  | 8RBs | 36RBs | 40RBs |
| Type 1 | Approach A | Cyclic Ext | 2.13[1] | 2.04 | 3.04 | 2.56 | - | - | - | - | 0.07 | -0.01 | 0.80 | 0.26 | 0.38 |  | 0.15 | 0.99 | 0.33 |
|  |  | Symmetric Ext | 3.75[2] | 3.77 | 3.79 | 4.25 | - | - | - | - | 1.23 | 0.89 | 1.19 | 1.28 | 1.66 |  | 1.35 | 1.39 | 1.71 |
|  | Approach B | No FDSS | 3.77 | 4.13 | 4.35 | 4.50 | - | - | - | - | 0.88 | 1.02 | 0.87 | 0.91 | 1.32 |  | 2.11 | 2.19 | 2.32 |
|  |  | FDSS-SE | 3.05 | 3.28 | 3.39 | 3.40 | - | - | - | - | 0.72 | 1.07 | 1.17 | 0.14 | 1.03 |  | 1.54 | 1.55 | 1.66 |
| Type 2 | Approach A | FDSS-SE | 1.73 | 1.55 | 1.26 | 1.56 | - | -0.66 | -0.81 | -0.71 | -0.51 | -0.74 | -0.84 | -0.74 | -0.44 |  | -0.62 | -0.79 | -0.68 |
|  | Approach B | No FDSS | 3.52 | 3.51 | 3.50 | 3.50 | 0.85 | 0.76 | 0.49 | 0.47 | 0.24 | 0.24 | 0.24 | 0.24 | 1.27 |  | 1.27 | 0.83 | 0.62 |
|  |  | FDSS-SE | 1.28 | 1.29 | 1.29 | 1.28 | -0.66 | -0.69 | -0.75 | -0.75 | -0.79 | -0.80 | -0.80 | -0.80 | -4.88 |  | -0.61 | -0.72 | -0.71 |
| Note: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| [1]: DFT is applied for the length-24 sequence. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| [2]: DFT is not applied for the CG-CAZAC length-24 sequence. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

As can be seen in Table-5, both PAPR and CM performance for Type 2 DMRS sequence are better than Type 1 DMRS sequences. We can also find that Type 2 DMRS sequence with either Approach A or Approach B, as well as Type DMRS sequence with Approach A + cyclic extension can achieve better CM/PAPR performance than PUSCH data. That means, the bottleneck of CM/PAPR performance is data transmission if above-mentioned candidates are adopted. But Type 2 DMRS sequence with approach A ensures the same extension operation between DMRS and data which can reduce UE complexity for FDSS operations.

Observation 3: Both the PAPR and CM performance of Type 2 DMRS sequence are better than that of Type 1 DMRS sequences

Observation 4: The bottleneck of CM/PAPR performance is data transmission instead of DMRS.

Observation 5: Type 2 DMRS sequence with approach A ensures the same extension operation between DMRS and data which can reduce UE complexity for FDSS operations.

With above observations, we have the following proposal.

Proposal 2: If non-transparent FDSS with spectrum extension is supported, adopt Type 2 DMRS sequence with Approach A.

Conclusion

Based on above analysis, we have the following observations and proposals.

## Increasing UE power higher limit for CA/DC

Observation 1: According to RAN4 LS, RAN1 can proceed the discussion on increasing gNB awareness of UE’s Tx power without subjecting to RAN4’s inputs.

Observation 2: gNB is unable to know the exact UE transmit power due to the ambiguity of when and how long the evaluation period for UE power class fallback is implemented.

Proposal 1: To address the ambiguity of evaluation period for UE power class fallback, support one of the following alternatives.

- Alt 1. PHR reporting enhancement with a certain duration for the applicability of one among {the fallback power class ∆PPowerClass (potentially with a finer granularity), the default power class, or Pc,max}.

- Alt 2. Introduce a scheme for a UE to report uplink symbol evaluation period and starting timing.

## MPR/PAR reduction schemes

Observation 3: Both the PAPR and CM performance of Type 2 DMRS sequence are better than that of Type 1 DMRS sequences

Observation 4: The bottleneck of CM/PAPR performance is data transmission instead of DMRS.

Observation 5: Type 2 DMRS sequence with approach A ensures the same extension operation between DMRS and data which can reduce UE complexity for FDSS operations.

Proposal 2: If non-transparent FDSS with spectrum extension is supported, adopt Type 2 DMRS sequence with Approach A.

Reference

RAN1#112, R1-2302081, LS to RAN4 for results of the LLS performance evaluation of MPR/PAR reduction solutions, RAN1.

R1-2301873, Final FL summary of power domain enhancements (AI 9.14.2), Moderator (Nokia, Nokia Shanghai Bell)

RAN1#112bis-e, R1-2302270, Reply LS on enhancements to realize increasing UE power high limit for CA and DC, RAN4, NTT DOCOMO INC

RAN#106, R4-2303560, WF on coverage enhancement part 1, Huawei, HiSilicon.

Appendix A – Evaluation assumption

Working Assumption (in RAN1 #112)

The following set of configurations is for companies’ consideration for the comparison of the performance of DMRS with FDSS-SE.

| No spectrum extension |  | With spectrum extension |  |  |  |
| --- | --- | --- | --- | --- | --- |
| #PRBs | MCS | #PRBs before extension | #PRBs after extension | MCS | Spectrum extension factor |
| 8 | 0 [only QPSK] | 6 | 8 | 1 [only QPSK] | ¼ |
| 8 | 6 | 6 | 8 | 8 | ¼ |
| 40 | 2 | 30 | 40 | 3 | ¼ |
| 40 | 6 | 30 | 40 | 8 | ¼ |
| [6 | 3 | 4 | 6 | 5 | 1/3] |
| [36 | 7 | 32 | 36 | 8 | 1/9] |

- FR1 4GHz Urban scenario is prioritized.

- The following filters are for companies’ consideration for the calibration of the performance of DMRS with FDSS-SE

- 3-tap (0.28 1 0.28)

- [Truncated RRC (0.5, 0.1667) or 2-tap (1 0.28)]

- Note1: Considered metrics are PAPR/CM, 10% BLER SNR of data for the considered DMRS configuration (for measuring impact of channel estimation accuracy)[, and OBO]

- Note2: companies are encouraged to consider a receiver which at least makes use of the extension for the decoding (e.g., MRC)

- Note3: The values above serve as a common basis, but any other configuration can be studied by companies.

Appendix B – Evaluation results

Figure B-1. The CCDF of PAPR and CM for 8 RBs with SE=1/4.

Figure B-2 the CCDF of PAPR and CM for 36 RBs with SE=1/9.

Figure B-3. The CCDF of PAPR and CM for 40 RBs with SE=1/4.

Figure B-4. The CCDF of PAPR and CM for 6 RBs with SE=1/3.