3GPP TSG-RAN WG1 #112bis-e	R1-2302763

e-Meeting, April 17th – April 26th, 2023

Title:  	Discussion on UE features for MC enhancements

Source:  	ZTE

Agenda item:  	9.17.10

Document for:	Discussion/Decision

Introduction

After RAN1#112 meeting, the WI “multi-carrier enhancements” has been completed from RAN1 perspective. In this contribution, we provide our initial views on UE features for multi-cell scheduling and UL Tx switching among 3/4 bands, respectively.

UE features for multi-cell scheduling

In this section, we provide analysis for the UE capabilities for multi-cell scheduling.

Issue 1: The scheduling of multi-cell scheduling

Since multi-cell scheduling is a new feature in NR, whether the UE supports multi-cell scheduling or not should be reported by the UE. Since the co-scheduled cells within a set of cells can be in different bands, the report granularity should be per BC report.

For multi-cell scheduling, the following agreements have been reached on the number of co-scheduled cells and the number of sets.

Agreement Confirm the following working assumption reached in RAN1#110 meeting. Working Assumption - The maximum number of co-scheduled cells by a DCI format 1_X in Rel-18 is 4. - The maximum number of co-scheduled cells by a DCI format 0_X in Rel-18 is 4. - FFS: The maximum number of configurable cells for co-scheduling Agreement For a set of cells which is configured for multi-cell scheduling, up to 4 cells within the set of cells are supported. - A DCI format 0_X/1_X can schedule PUSCH(s)/PDSCH(s) on a combination of co-scheduled cells among the same set of cells. Agreement Following is supported in Rel-18 multi-cell scheduling - A UE can be configured one or multiple sets of cells with each set configured for multi-cell scheduling using DCI format 0_X/1_X. - Up to 4 sets of cells can be configured per PUCCH group. - When multiple sets of cells are configured, - a cell in one set of cells can’t be included in another set of cells. - n_CI value is independently configured for each set of cells. - reference cell for counting DCI size and BD/CCE of DCI format 0_X/1_X is independently determined for each set of cells. - search space configuration of DCI format 0_X/1_X is independently configured for each set of cells. - DCI size of DCI format 0_X is independently determined for each set of cells. - DCI size of DCI format 1_X is independently determined for each set of cells. - The multiple sets of cells can be scheduled by DCI format 0_X/1_X from different scheduling cells. - Up to N sets of cells can be configured and respectively scheduled by DCI format 0_X/1_X from a same scheduling cell. - The value of N is reported as UE capability. - An indicator is included in the DCI to indicate the scheduled set of cells, - The size of the indicator is equal to ceil(log2(N)), where N is the number of sets of cells. - Unique n_CI value is configured for each set of cells.

It was agreed that the maximum number of co-scheduled cells by a single DCI format 0_X/1_X is 4 and up to 4 sets of cells can be configured for a UE. Therefore, the number of the co-scheduled cells by a single DCI format 0_X/1_X shall be reported by the UE. Since the number of sets of cells that can be configured with the same scheduling cell should be reported by the UE as agreed, up to 4 sets of cells per PUCCH group can be guaranteed by gNB configuration. For multi-cell scheduling for downlink, all the Type-1, Type-2, Type-3 codebook are supported. If a UE support multi-cell scheduling for downlink, the corresponding HARQ feedback should also be supported since HARQ feedback is always needed for downlink scheduling.

Proposal 1: For multi-cell scheduling cell scheduling with single DCI, it should be per BC report with the following three components.

- The support of multi-cell scheduling for the band combination.

- The number of co-scheduled cells by a single DCI format 0_X/1_X, including the candidate value 2, 3, and 4.

- The number of sets that can be scheduled by the same scheduling cell, including the candidate value 1, 2, 3, and 4.

Issue 2: RBG size for multi-cell scheduling

For FDRA indication, a larger granularity was introduced to reduce the overhead of FDRA field as shown below.

Agreement - A new RBG size configuration “Configuration 3” is added with the following values and only used for DCI format 0_X/1_X for RA type 0. - RBG size is configured per BWP per cell. - Independent RA type configuration is applied per BWP per cell for multi-cell scheduling DCI. Table 5.1.2.2.1-1 / Table 6.1.2.2.1-1: Nominal RBG size P Bandwidth Part Size; Configuration 1; Configuration 2; Configuration 3. 1 – 36; 2; 4; 8. 37 – 72; 4; 8; 16. 73 – 144; 8; 16; 32. 145 – 275; 16; 16; 32. Agreement New RRC parameter of RBG granularity for RA type 1 can be configured per BWP per cell for DCI format 0_X/1_X with same value range applicable for DCI 0_2/1_2.

Therefore, the UE should be report whether it supports the new configuration of the RBG size or not. It should be per UE reporting since it is not related to the band.

Proposal 2: The support of new configuration of the RBG size should be reported by the UE and the type should be per UE reporting.

UE features for UL Tx switching among 3/4 bands

In this section, we provide analysis for the UE capabilities for Rel-18 UL Tx switching.

Issue1: Whether a separate UE feature to indicate the support of UL Tx switching among 3/4bands is needed?

RAN2 has been agreed to reuse the Rel-16/17 band combination list for Rel-18 UL Tx switching. However, RAN2 is discussing the following two approaches regarding the feature set for UL Tx switching. If Approach 1 is adopted, then it seems a separate UE capability to indicate whether UE supports UL Tx switching among 3/4 bands is not necessary because it can be implicitly indicated by the feature set row. In other words, if 3/4 FeatureSetUplink are reported in one row in FSC for the 3/4 UL bands, then UE supports UL Tx switching among 3/4 bands. However, if Approach 2 below is adopted, then a separate UE feature to indicate the support of UL Tx switching among 3/4 bands is needed.

Approach 1: the 3/4 FeatureSetUplink are reported in one row in FSC for the 3/4 UL bands involved in Rel-18 UL Tx switching; Approach 2: the FeatureSets reported for Rel-16/17 Tx switching between 2 bands can be combined to indicate UL capabilities on the 3/4 UL bands for Rel-18 UL Tx switching;

Overall, we have the following proposal.

Proposal 3: Regarding whether a separate UE feature to indicate the support of UL Tx switching among 3/4bands is needed, consider the following two alternatives.

- Alt.1: Not needed. The support of UL Tx switching among 3/4 bands is implicitly indicated by the feature sets indication for UL Tx switching.

- Alt.2: Needed. It is a per BC UE capability.

Issue2: Which band pair is switchedUL and which band pair is dualUL?

Based on the following agreements, this issue will be decided by RAN2 and RAN2 has agreed to have per band pair indication for switchedUL and dualUL.

Agreements (RAN1)  Ask RAN2 to consider following alternatives for UE capability reporting about the supported UL Tx switching options - o Alt.1: report {switchedUL, dualUL, both} for each band pair in the band combination - o Alt.2: report {switchedUL, dualUL, both} for the band combination and report supported band pair for concurrent transmission for the band combination - § Note：If there is no report on the supported band pair(s) for concurrent transmission while the UE reports “dualUL” or “both” for the band combination, gNB may assume that the UE supports concurrent transmission on all the band pairs within the band combination - o Alt.3: report {dualUL} for each band pair in the band combination - § Note: Within the band combination, the UE shall be capable of being operated in switched UL mode for all band pairs Agreements (RAN2): - For UE capability of switching options, introduce a per-band-pair UE capability to report supported switching options for Rel-18 UL Tx switching. - Configure {switchedUL, dualUL} for combination(s) of serving cells (i.e., for each band pair in the band combination) - For RRC configuration to clarify ambiguous Tx state, RAN2 should introduce an RRC configuration that associates a band to another band which the unused Tx chain is switched to when the switch is from concurrent transmission on two bands to 1 Tx transmission on another band. - For UE capability of 2-port UL transmission, RAN2 reuse the per-FS UL-MIMO UE capability (no spec change).

Observation 1: Regarding the UE feature for indication of switchedUL and dualUL for each band pair, it is introduced by RAN2.

Issue3: which band/carrier supports 2-port transmission?

RAN1 agreed that there is no restriction on number of bands supporting up to 2 ports UL transmission for both switched UL and dual UL and for both 3 bands and 4 bands. However, no new UE capability is needed since UE can already report its MIMO capability via legacy UE capabilities.

Agreement There is no restriction on number of bands supporting up to 2 ports UL transmission for both switched UL and dual UL and for both 3 bands and 4 bands. - It is up to UE capability to support 2 ports UL transmission on none/some/all of the 3 or 4 bands - Note: UE with only 1 Tx chain is not expected to perform UL Tx switching (no spec impact)

Observation 2: No need to introduce new UE capability to indicate the number of ports for PUSCH transmission for Rel-18 UL Tx switching.

Issue4: Duration of the switching period

Regarding the duration of the switching period, RAN4 has agreed that for Rel-18 UE, for a band pair within a band combination supporting Tx switching among 3/4 bands, the switching period reported by UE for Rel-18 3/4-band Tx switching can be the same or different from the switching period for Rel-16/17 2-band switching operations. The UE capability can be decided by RAN2 and RAN4.

Issue 1: Exact value of Tx switching period for each band pair RAN4 discussed the exact value of Tx switching period for each band pair in the band combination, and has agreed that: - For Rel-18 UE, for a band pair within a band combination supporting Tx switching among 3/4 bands, the switching period reported by UE for Rel-18 3/4-band Tx switching can be the same or different from the switching period for Rel-16/17 2-band switching operations. - Note 1: the set of candidate values is still the same, i.e., {35 us, 140 us, 210 us}, according to the agreement in RAN4 #104e. - Note 2: here the band pair is a pair of bands within which there is a switching with a switching period.

Proposal 4: Regarding the UE capability for duration of the switching period, it is to be introduced by RAN2/RAN4.

Issue5: Minimum separation time

RAN1 has agreed the following for minimum separation time. This UE capability should be discussed in RAN1. Based on the initial discussion in RAN1#112 meeting, the UE capability can be per BC type.

Agreement Confirm the working assumption with following updates (working assumption) If two uplink switching are triggered and UL transmissions involved in the two uplink switching are on more than 2 bands within any two consecutive reference slots, then the time duration between the start of all transmission(s) after the first uplink switching and the start of all transmission(s) after the second uplink switching within the two reference slots is expected to be not less than a minimum separation time - The minimum separation time is a maximum of X us and the switching gap required for the second uplink switching. - X us is subject to UE capability with a value set of {0us, 500us}

Proposal 5: Introduce a per BC UE capability for value X for determination the minimum separation time.

Issue6: Advanced UE capability

RAN4 has agreed to introduce an advanced UE capability to indicate whether UE is able to transmit with the unchanged Tx chain during the switching period of another Tx chain.

Issue 3: Impact from switching of one Tx chain on the other Tx chain Scenario of one band with the number of Tx chain unchanged due to switching When one of the two Tx chains is triggered to switch from one band (named “band A”) to another band (name “band B”), the other Tx chain is maintained on a different band (named “band C” or “band D” in the case of 4-band) and the number of Tx chain on band C or band D is unchanged due to the switching, RAN4 agreed the granularity of the optional UE capability to allow UL transmission on the band with the number of Tx chain unchanged during UL switching as follows: - Per band (only for the band(s) in the band combination but not included in the pair of bands before and after switching) for each pair of bands before and after switching in each band combination.

Proposal 6: Regarding the UE capability to indicate whether UE is able to transmit with the unchanged Tx chain during the switching period of another Tx chain, it is to be introduced by RAN2/RAN4.

Conclusion

In this contribution, we provide our analysis for UE capabilities for Rel-18 MC WI.

UE features for multi-cell scheduling

Proposal 1: For multi-cell scheduling cell scheduling with single DCI, it should be per BC report with the following three components.

- The support of multi-cell scheduling for the band combination.

- The number of co-scheduled cells by a single DCI format 0_X/1_X, including the candidate value 2, 3, and 4.

- The number of sets that can be scheduled by the same scheduling cell, including the candidate value 1, 2, 3, and 4.

Proposal 2: The support of new configuration of the RBG size should be reported by the UE and the type should be per UE reporting.

UE features for UL Tx switching among 3/4 bands

Proposal 3: Regarding whether a separate UE feature to indicate the support of UL Tx switching among 3/4bands is needed, consider the following two alternatives.

- Alt.1: Not needed. The support of UL Tx switching among 3/4 bands is implicitly indicated by the feature sets indication for UL Tx switching.

- Alt.2: Needed. It is a per BC UE capability.

Observation 1: Regarding the UE feature for indication of switchedUL and dualUL for each band pair, it is introduced by RAN2.

Observation 2: No need to introduce new UE capability to indicate the number of ports for PUSCH transmission for Rel-18 UL Tx switching.

Proposal 4: Regarding the UE capability for duration of the switching period, it is to be introduced by RAN2/RAN4.

Proposal 5: Introduce a per BC UE capability for value X for determination the minimum separation time.

Proposal 6: Regarding the UE capability to indicate whether UE is able to transmit with the unchanged Tx chain during the switching period of another Tx chain, it is to be introduced by RAN2/RAN4.