3GPP TSG-RAN WG1 #112bis-e	R1-2302765

e-Meeting, April 17th – April 26th, 2023

Title 	:  Discussion on UE features for BWP without restriction

Source 	:  ZTE

Agenda item	:  9.17.15

Document for	:  Discussion/Decision

Introduction

In RAN#99 meeting, the following WID is approved for BWP without restriction. In this contribution, we provide our initial views for the potential UE feature from RAN1 perspective.

- The work item includes following objectives: - For Option A - Study and specify if any clarifications of the existing requirements are needed, e.g., applicability of requirements, conditions of gap configuration etc. (RAN4) - For Option B-1-1 - Specify support of BM/RLM/BFD based on SSB outside the active BWP without interruptions (RAN4, RAN2, RAN1) - For Option C - Specify support of BM/RLM/BFD based on NCD-SSB within active BWP for non-RedCap UEs (RAN4, RAN2, RAN1) - For Option B-1-2 - Specify support of BM/RLM/BFD based on SSB outside the active BWP with interruptions with the following conditions (RAN4, RAN2, RAN1): - The UE shall be allowed to use B-1-2 only if there is no CSI-RS, no NCD SSB and no CD SSB configured for RLM/BM/BFD in the active BWP of the corresponding carrier(s) to be measured; and - UE shall support option (C) NCD-SSB (subject to IoDT availability). - The interruption related requirements will be decided and specified in RAN4. The expected RAN2 impacts are the RRC configuration signalling for the above options, and the capability signalling aspects.

Discussion

Clarification of bwp-WithoutRestriction

The above WID is trying to address the issue of UE capability “bwp-WithoutRestriction”, where UE is allowed to be configured with BWP without including SSB for PCell/PSCell/SCell. However, it is not clear whether the “SSB” refers to CD-SSB or NCD-SSB. This uncertainty will impact the further discussion about new UE capabilities for this WID, thus it is worth clarifying it first.

| bwp-WithoutRestriction Indicates support of BWP operation without bandwidth restriction. The Bandwidth restriction in terms of DL BWP for PCell and PSCell means that the bandwidth of a UE-specific RRC configured DL BWP may not include the bandwidth of CORESET #0 (if configured) and SSB. For SCell(s), it means that the bandwidth of DL BWP may not include SSB. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

Based on our understanding,

- For non-RedCap UE, the “SSB” in UE capability “bwp-WithoutRestriction” refers to CD-SSB only since the UE capability “bwp-WithoutRestriction” has been in place since Rel-15.

- However, Redcap UE already supports BWP without CD-SSB but with NCD-SSB. Thus, for RedCap UE, the “SSB” in UE capability “bwp-WithoutRestriction” refers to both CD-SSB and NCD-SSB. Then in this case, RedCap UE has two level of indication as following:

- Indicating support of supportOfRedCap-r17 but not support bwp-WithoutRestriction: CD-SSB/NCD-SSB has to be configured within the active BWP.

- Indicating support of both supportOfRedCap-r17 and bwp-WithoutRestriction: UE supports active BWP without CD-SSB/NCD-SSB. UE may need to perform BM/RLM/BFD based on CSI-RS for example.

Thus, we have the following proposal.

Proposal 1: Regarding UE capability “bwp-WithoutRestriction”

- For Redcap UE, clarifying that “SSB” in UE capability “bwp-WithoutRestriction” refers to both CD-SSB and NCD-SSB.

- For non- Redcap UE, clarifying that “SSB” in UE capability “bwp-WithoutRestriction” refers to CD-SSB but not NCD-SSB.

Analysis of new UE capabilities

Option A: Perform BM/RLM/BFD based on CSI-RS within active BWP

Based on the previous working group discussion, it seems that companies have common understanding that the spec for Option A is completed from function perspective. It may or may not need any clarifications of the existing requirements. But in any case, new UE feature for option A is not needed from our perspective.

Observation 1: From RAN1 perspective, no new UE feature for option A is needed.

| maxNumberCSI-RS-BFD Indicates maximal number of CSI-RS resources across all CCs, and across MCG and SCG in case of NR-DC, for UE to monitor PDCCH quality. In this release, the maximum value that can be signalled is 16. If the UE includes the field in an FR1 band, it shall set the same value in all FR1 bands. If the UE includes the field in an FR2 band, it shall set the same value in all FR2 bands. The UE supports a total number of resources equal to the maximum of the FR1 and FR2 value, but no more than the FR1 value across all FR1 serving cells and no more than the FR2 value across all FR2 serving cells. It is mandatory with capability signalling for FR2 and optional for FR1. | Band | CY | N/A | N/A |
| --- | --- | --- | --- | --- |
| csi-RS-RLM Indicates whether the UE can perform radio link monitoring procedure based on measurement of CSI-RS as specified in TS 38.213 [11] and TS 38.133 [5]. This parameter needs FR1 and FR2 differentiation. If the UE supports this feature, the UE needs to report maxNumberResource-CSI-RS-RLM. This applies only to non-shared spectrum channel access. For shared spectrum channel access, csi-RS-RLM-r16 applies. | UE | Yes | No | Yes |  |
| beamManagementSSB-CSI-RS Defines support of SS/PBCH and CSI-RS based RSRP measurements. The capability comprises signalling of - maxNumberSSB-CSI-RS-ResourceOneTx indicates maximum total number of configured one port NZP CSI-RS resources and SS/PBCH blocks that are supported by the UE to measure L1-RSRP as specified in TS 38.215 [13] within a slot and across all serving cells (see NOTE). On FR2, it is mandatory to report >=8; On FR1, it is mandatory with capability signalling to report >=8. - maxNumberCSI-RS-Resource indicates maximum total number of configured NZP-CSI-RS resources that are supported by the UE to measure L1-RSRP as specified in TS 38.215 [13] across all serving cells (see NOTE). It is mandated to report at least n8 for FR1. - maxNumberCSI-RS-ResourceTwoTx indicates maximum total number of two ports NZP CSI-RS resources that are supported by the UE to measure L1-RSRP as specified in TS 38.215 [13] within a slot and across all serving cells (see NOTE). - supportedCSI-RS-Density indicates density of one RE per PRB for one port NZP CSI-RS resource for RSRP reporting, if supported. On FR2, it is mandatory to report either "three" or "oneAndThree"; On FR1, it is mandatory with capability signalling to report either "three" or "oneAndThree". - maxNumberAperiodicCSI-RS-Resource indicates maximum number of configured aperiodic CSI-RS resources across all serving cells (see NOTE). For FR1 and FR2, the UE is mandated to report at least n4. NOTE: If the UE sets a value other than n0 in an FR1 band, it shall set that same value in all FR1 bands. If the UE sets a value other than n0 in an FR2 band, it shall set that same value in all FR2 bands. The UE supports a total number of resources equal to the maximum of the FR1 and FR2 value, but no more than the FR1 value across all FR1 serving cells and no more than the FR2 value across all FR2 serving cells. | Band | Yes | N/A | FD |

Option B-1-1: SSB outside active BWP without interruption

The option B-1-1 can be summarized as following.

- Option B) Perform BM/RLM/BFD based on SSB outside active BWP

  - Option B-1) UE’s capability not requiring additional measurement gap for BM/RLM/BFD

    - Option B-1-1) Using larger BW covering SSB outside active BWP without interruptions

The motivation is to address the issue when UE reports bwp-WithoutRestriction, the new UE capability for option B-1-1 can be a per UE capability since the existing bwp-WithoutRestriction is already a per band capability.

| bwp-WithoutRestriction Indicates support of BWP operation without bandwidth restriction. The Bandwidth restriction in terms of DL BWP for PCell and PSCell means that the bandwidth of a UE-specific RRC configured DL BWP may not include the bandwidth of CORESET #0 (if configured) and SSB. For SCell(s), it means that the bandwidth of DL BWP may not include SSB. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

Proposal 2: Introduce the following UE capability for option B-1-1.

| Index | Feature group | Components | Type | Prerequisite feature groups | Mandatory/ Optional |
| --- | --- | --- | --- | --- | --- |
| B-1-1 | Perform BM/RLM/BFD based on SSB outside active BWP without interruption | Perform BM/RLM/BFD based on SSB outside active BWP. - UE’s capability not requiring additional measurement gap for BM/RLM/BFD - Using larger BW covering SSB outside active BWP without interruptions | per UE | 6-1a | Optional |

Option C: NCD-SSB for non-RedCap UEs

Option C requires UE to support of BM/RLM/BFD based on NCD-SSB within active BWP for non-RedCap UEs. Similar as the following UE capability for RedCap UE, a per UE capability can be introduced for non-RedCap UEs.

| supportOfRedCap-r17 Indicates that the UE is a RedCap UE with comprised of at least the following functional components: - Maximum FR1 RedCap UE bandwidth is 20 MHz; - Maximum FR2 RedCap UE bandwidth is 100 MHz; - Support of RedCap early indication based on Msg1, MsgA (if UE indicated support of twoStepRACH-r16) and Msg3 for random access; - Separate initial UL BWP for RedCap UEs; - Separate initial DL BWP for RedCap UEs; - UE-specific RRC-configured DL BWP with CD-SSB or NCD-SSB; - NCD-SSB based measurements in RRC-configured DL BWP. A RedCap UE shall set the field to supported. | UE | CY | No |
| --- | --- | --- | --- |

Proposal 3: Introduce the following UE capability for option C.

| Index | Feature group | Components | Type | Prerequisite feature groups | Mandatory/ Optional |
| --- | --- | --- | --- | --- | --- |
| C | Support of active BWP without CD-SSB but with NCD-SSB for non-RedCap UEs | - Support of active BWP without CD-SSB but with NCD-SSB for non-RedCap UEs | UE | 6-1a | Optional |

Option B-1-2: SSB outside active BWP with interruption

The details of option B-1-2 can be summarized as following.

  - support of BM/RLM/BFD based on SSB outside the active BWP with interruptions with the following conditions

    - The UE shall be allowed to use B-1-2 only if there is no CSI-RS, no NCD SSB and no CD SSB configured for RLM/BM/BFD in the active BWP of the corresponding carrier(s) to be measured; and

  - UE shall support option (C) NCD-SSB (subject to IoDT availability).

Proposal 4: Introduce the following UE capability for option B-1-2.

| Index | Feature group | Components | Type | Prerequisite feature groups | Mandatory/ Optional |
| --- | --- | --- | --- | --- | --- |
| B-1-2 | Perform BM/RLM/BFD based on SSB outside active BWP with interruption | support of BM/RLM/BFD based on SSB outside the active BWP with interruptions with the following conditions - The UE shall be allowed to use B-1-2 only if there is no CSI-RS, no NCD SSB and no CD SSB configured for RLM/BM/BFD in the active BWP of the corresponding carrier(s) to be measured; and | per UE | 6-1a and Option C | Optional |

Conclusion

In this contribution, we provide our analysis on UE features for BWP without restriction with the following proposals and observations.

Proposal 1: Regarding UE capability “bwp-WithoutRestriction”

- For Redcap UE, clarifying that “SSB” in UE capability “bwp-WithoutRestriction” refers to both CD-SSB and NCD-SSB.

- For non- Redcap UE, clarifying that “SSB” in UE capability “bwp-WithoutRestriction” refers to CD-SSB but not NCD-SSB.

Observation 1: From RAN1 perspective, no new UE feature for option A is needed.

Proposal 2: Introduce the following UE capability for option B-1-1.

| Index | Feature group | Components | Type | Prerequisite feature groups | Mandatory/ Optional |
| --- | --- | --- | --- | --- | --- |
| B-1-1 | Perform BM/RLM/BFD based on SSB outside active BWP without interruption | Perform BM/RLM/BFD based on SSB outside active BWP. - UE’s capability not requiring additional measurement gap for BM/RLM/BFD - Using larger BW covering SSB outside active BWP without interruptions | per UE | 6-1a | Optional |

Proposal 3: Introduce the following UE capability for option C.

| Index | Feature group | Components | Type | Prerequisite feature groups | Mandatory/ Optional |
| --- | --- | --- | --- | --- | --- |
| C | Support of active BWP without CD-SSB but with NCD-SSB for non-RedCap UEs | - Support of active BWP without CD-SSB but with NCD-SSB for non-RedCap UEs | UE | 6-1a | Optional |

Proposal 4: Introduce the following UE capability for option B-1-2.

| Index | Feature group | Components | Type | Prerequisite feature groups | Mandatory/ Optional |
| --- | --- | --- | --- | --- | --- |
| B-1-2 | Perform BM/RLM/BFD based on SSB outside active BWP with interruption | support of BM/RLM/BFD based on SSB outside the active BWP with interruptions with the following conditions - The UE shall be allowed to use B-1-2 only if there is no CSI-RS, no NCD SSB and no CD SSB configured for RLM/BM/BFD in the active BWP of the corresponding carrier(s) to be measured; and | per UE | 6-1a and Option C | Optional |

Reference

- RP-230805, New WID: Complete the specification support for BandWidth Part operation without restriction in NR, RAN#99.