3GPP TSG-RAN WG1 Meeting #112bis-e	Tdoc R1-2302769

e-Meeting, 17th April – 26th April, 2023

Agenda Item:	9.3.1

Source:	Ericsson

Title:	Evaluation of NR duplex evolution

Document for:	Discussion, Decision

# Table of Contents

Table of Contents	2

1	Introduction	4

2	Methodology for gNB TX and RX modelling for SBFD self-interference mitigation study	5

2.1	Examples of gNB Transmitter Modeling	6

2.1.1	FR1 Power amplifier, digital predistortion (DPD) and net effect modelling	6

2.1.2	FR1 Crest factor reduction (CFR)	8

2.2	Examples of gNB Antenna and Interference Channel	11

2.2.1	FR1 Self-interference leakage within a sector	11

2.2.2	FR2 Self-interference leakage within a sector	13

2.2.3	FR1 Interference leakage between sectors	15

2.2.4	FR2 Interference leakage between sectors	18

2.3	Transmit Beam Nulling	21

2.4	Homodyne/Heterodyne gNB Receiver Modeling	23

2.4.1	Low noise amplifier (LNA) model	25

2.4.1.1	FR1 models	26

2.4.1.2	FR2 models	27

2.4.2	Reciprocal mixing of phase noise	27

2.4.2.1	FR1 models	28

2.4.2.2	FR2 models	29

2.4.3	Analog filtering and analog to digital converter (ADC)	30

2.4.4	Digital filtering	31

2.5	Example digital self-interference cancellation solution for FR1 homodyne/heterodyne type base station receivers	32

2.6	 Summary of Modelling Discussion	33

3	System Level Evaluation	34

3.1	Scenarios for evaluation	34

4	System Level Evaluation Methodology	36

4.1	Performance Metrics	36

4.1.1	Coverage	36

4.2	Modelling for system level simulations	38

4.2.1	FR1 Receiver model in SLS	38

4.3	Differences in RAN1 and RAN4 SLS assumptions	40

5	System level evaluations	41

5.1	Scenarios for system level simulations	41

5.2	System level evaluation in FR1	45

5.2.1	Case 1: Single Operator Urban Macro	45

5.2.1.1	Alt.2 (Static TDD{DDDSU}, SBFD{XXXXU})	45

5.2.1.1.1	Resource utilization per direction	45

5.2.1.1.2	Coverage	46

5.2.1.1.3	User Throughput	47

5.2.1.1.4	Latency	47

5.2.1.2	Alt.4 (Static TDD{DDDSU}, SBFD{XXXXX})	48

5.2.1.2.1	Resource utilization per direction	48

5.2.1.2.2	Coverage	49

5.2.1.2.3	User Throughput	50

5.2.1.2.4	Latency	50

5.2.1.3	Alt.3 (Static TDD{DDSUU}, SBFD{XXXXU})	51

5.2.1.3.1	Resource utilization per direction	51

5.2.1.3.2	Coverage	51

5.2.1.3.3	User Throughput	52

5.2.1.3.4	Latency	52

5.2.2	Case 4: Two Operator Urban Macro	53

5.2.2.1	Alt.2 (Network 1: Static TDD{DDDSU}, Network 2: SBFD{XXXXU})	53

5.2.2.1.1	Resource utilization per direction	53

5.2.2.1.2	Coverage	55

5.2.2.1.3	User Throughput	56

5.2.2.1.4	Latency	57

5.2.2.2	Alt.4 (Network 1: Static TDD{DDDSU}, Network 2: SBFD{XXXXX})	58

5.2.2.2.1	Resource utilization per direction	58

5.2.2.2.2	Coverage	59

5.2.2.2.3	User Throughput	60

5.2.2.2.4	Latency	61

5.2.2.3	Alt.3 (Network 1: Static TDD{DDDSU}, Network 2: SBFD{XXXXU} / Static TDD{DDSUU})	62

5.2.2.3.1	Resource utilization per direction	62

5.2.2.3.2	Coverage	64

5.2.2.3.3	User Throughput	64

5.2.2.3.4	Latency	66

5.2.3	Case 1: Single Operator Indoor Office	67

5.2.3.1 	Alt.2 (Static TDD{DDDSU}, SBFD{XXXXU})	67

5.2.3.1.1	Resource Utilization	67

5.2.3.1.2	User Throughput	68

5.2.3.1.3	Latency	69

5.2.3.2 	Alt.4 (Static TDD{DDDSU}, SBFD{XXXXX})	71

5.2.3.2.1	Resource Utilization	71

5.2.3.2.2	User Throughput	72

5.2.3.2.3	Latency	73

5.2.3.3 	Alt.3 (Static TDD{DDSUU}, SBFD{XXXXU})	74

5.2.3.3.1	Resource Utilization	74

5.2.3.3.2	User Throughput	75

5.2.3.3.3	Latency	76

5.3	System level evaluation in FR2-1	77

5.3.1	Case 1: Single Operator Dense Urban Macro	77

5.3.1.1	Alt.2 (Static TDD{DDDSU}, SBFD{XXXXU})	77

5.3.1.1.1	Resource utilization per direction	77

5.3.1.1.2	Coverage	78

5.3.1.1.3	User Throughput	78

5.3.1.1.4	Latency	79

5.3.1.2	Alt.3 (Static TDD{DDSUU}, SBFD{XXXXU})	80

5.3.1.2.1	Resource utilization per direction	80

5.3.1.1.2	Coverage	80

5.3.1.1.3	User Throughput	81

5.3.1.1.4	Latency	81

5.4	Summary of the system level evaluations	82

Conclusion	82

References	86

Annex A	87

A.1 GMP coefficients for ~2 GHz GaAs PA at 491.52 MHz sampling rate	87

A.2 GMP coefficients for DPD	87

A.3 GMP coefficients for net effect of DPD and PA	88

Annex B:	Parameters for System level evaluations	89

B.1	FR1 Urban Macro Scenario	89

B.2	FR1 Indoor Scenario	91

B.3	FR2 Dense Macro Scenario	94

Annex C: Table of settings for evaluation cases	97

C.1	FR1_Case1 Urban Macro	97

C.2	FR1_Case4 Urban Macro	98

C.3	FR1_Case1 Indoor Office	99

C.4	FR2_Case1 Dense Macro	100

# 1	Introduction

The objective of this study is to identify and evaluate the potential enhancements to support duplex evolution for NR TDD in unpaired spectrum.

In this study, the followings are assumed:

Duplex enhancement at the gNB side

Half duplex operation at the UE side

No restriction on frequency ranges

The detailed objectives are as follows:

Identify applicable and relevant deployment scenarios (RAN1).

Develop evaluation methodology for duplex enhancement (RAN1).

Study the sub-band non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4).

Identify possible schemes and evaluate their feasibility and performances (RAN1).

Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1).

Consider intra-sub-band CLI and inter-sub-band CLI in case of the sub-band non-overlapping full duplex.

Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1).

Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4).

Study the feasibility of and impact on RF requirements considering the self-interference, the inter-sub-band CLI, and the inter-operator CLI at gNB and the inter-sub-band CLI and inter-operator CLI at UE (RAN4).

Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression.

Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

Note: For potential enhancements on dynamic/flexible TDD, utilize the outcome of discussion in Rel-15 and Rel-16 while avoiding the repetition of the same discussion.

The Rel-18 study item on evolution of NR duplex operation contains the following list of objectives related to both sub-band non-overlapping full duplex (SBFD) and dynamic TDD, with the RAN1 objectives highlighted in yellow.

In this paper, we address the following aspects of the SID in the context of both SBFD and dynamic TDD:

- Deployment scenarios

- Evaluation methodology

  - Including radio and antenna modelling aspects

- Performance evaluation results at both link level and system level

# 2	Methodology for gNB TX and RX modelling for SBFD self-interference mitigation study

For supporting legacy UE, the carrier bandwidth (BW) must be one of the existing BW provided in RAN4 spec TS 38.104. It is also beneficial to have the UL subband BW being one of the existing BW. This could be beneficial in terms of sourcing hardware components to support SBFD. It can also be beneficial in terms of reusing existing L1/L2 specs and procedures if the UL subband BW conforms with one of the existing BW. On the other hand, there is no need for the BWs of the DL subbands to be anything special. The full carrier BW is available to the gNB and the gNB simply needs to avoid transmitting DL signals in the UL subband during a mixed direction slot. For instance, if a DL-UL-DL (DUD) SBFD subband configuration is considered, a PDCCH or a PDSCH can use resources from both DL subbands.

A SBFD carrier shall have a carrier BW and a UL subband BW consistent with one of the existing supported carrier BW in RAN4 specs.

Following the above SBFD carrier configuration principle, we consider the following two SBFD carrier examples for FR1 and FR2, respectively.

- FR1 SBFD structure example: a 30 kHz SCS 100 MHz carrier (273 available RBs) split for SBFD DUD configuration as 40-20-40 MHz.

  - RB split: 106(DL) – 5 (ISGB) – 51 (UL) – 5 (ISGB) – 106(DL)

    - 51 RBs for 30 kHz SCS is an existing UL transmission BW configuration in FR1

  - The inter-subband guard band (ISGB): ≥ (defined minimum GB for 20 MHz + defined minimum GB for 40 MHz)

- FR2 SBFD structure example: a 120 kHz SCS 200 MHz carrier (132 available RBs) split for SBFD DUD configuration as 75-50-75 MHz.

  - RB split: 47(DL) – 3 (ISGB) – 32 (UL) – 3 (ISGB) – 47 (DL)

    - 32 RBs for 120 kHz SCS is an existing UL transmission BW configuration in FR2

  - The ISGB ≥ (defined minimum GB for 50 MHz + estimated minimum GB for 75 MHz)

The FR1 and FR2 SBFD structure examples are illustrated in Figure 1 and Figure 2, respectively. CEGB in the figures corresponds to channel edge guard band.

Figure 1: Example FR1 SBFD DUD carrier structure

Figure 2: Example FR2 SBFD DUD carrier structure

## 2.1	Examples of gNB Transmitter Modeling

Representing the various non-linearities in the transmit chain faithfully is essential to studying the impact of gNB self-interference cancellation as well as gNB-gNB cross link interference (CLI) for both SBFD and dynamic TDD. This is vital since both intra-operator and inter-operator CLI is due to spectral leakage, both inter-sub-band and inter-channel, due to these non-linearities. In this section we propose a generic modelling approach that can be used for both link-level and system-level evaluations. From this foundation, additional details, and modelling of other transmitter components such as the DAC noises and phase noises can be further incorporated if needed.

Figure 3: Summary of initial example of FR1 gNB transmitter modeling.

### 2.1.1	FR1 Power amplifier, digital predistortion (DPD) and net effect modelling

TR 38.803 Annex A lists several PA models suitable for link level studies. For instance, the input, x(n), and output, yPA(n), relationship of a memory PA can be characterized by a generalized memory polynomial (GMP):

yPAn=k∈Kal∈LaaklPAxn-lxn-l2k+k∈Kbl∈Lbm∈MbklmPAxn-lx(n-m)2k

An example model for commercially available GaAs PA for ~2 GHz was given in TR 38.803 Annex A with Ka=Kb=8, La=5, Lb=2, and M=1. The total number of parameters is hence 8×7=56. Its AM-AM and AM-PM responses are given in TR 38.803 and copied in Figure 4 for ease of reference.

(a) AM-AM characteristic						(b) AM-PM characteristic

Figure 4: Characteristics of ~2 GHz GaAs PA (copied from TR 38.803).

The expected input sampling rate is 307.2 MHz for the PA model parameters given in TR 38.803. To apply this model to an NR signal of 100 MHz with a sampling rate of 491.52 MHz (corresponds to four times oversampling of a NR OFDM signal with 30 kHz subcarrier spacing and 4096-point DFT size, i.e., 491.52 MHz =4×4096×30 kHz), new PA model parameters need to be derived. The corresponding derived sets of parameters aklPA and bklmPA are provided in Annex A.1 below. The total number of parameters is 56, which is same as those for the PA model in TR 38.803.

It is well known that the nonlinear responses of the PA can generate substantial out-of-band emissions. To mitigate these issues while maintaining PA output efficiency, several common practices are adopted in a typical modern base station.

- A DPD is designed to pre-process the input signals to the PA to linearize the net responses of going through both DPD and PA.

- CFR techniques are deployed to increase PA power efficiency.

The input/output relationship of a DPD can also be described by a GMP:

yDPDn=k∈KaDPDl∈LaDPDaklDPDxn-lxn-l2k

For the above PA, a DPD with KaDPD=8 and LaDPD=5 can be designed to satisfy the base station ACLR requirements. The complete set of parameters aklDPD is provided in Annex A.2 below. The AM-AM response of passing through both DPD and PA is shown in Figure 5, where the DPD and PA are modelled separately. Comparing to the AM-AM response shown in Figure 4, it can be observed that the PA has become substantially linearized by use of a DPD. Note that to compensate for the PA’s compression of the input signal for normalized input powers close to 1 as shown in Figure 4, the DPD expands the input signal. The DPD correspondingly can linearize input magnitude of at most around -2 dB relative to the peak magnitude of 1.

Figure 5: AM-AM characteristics of explicit and net effect modelling of DPD and PA.

One can observe that it may not be necessary or desirable to perform simulations using separate models for the DPD and PA in the link level studies. This is because what matters to the ACLR, and leakage issue is the fidelity and representativeness of the net effect of both devices rather than those of the individual devices. Further, specific PA and DPD implementations may vary significantly from company to company. However, what can be common is that all these implementations are careful trade-offs of complexity/costs/compliance. Hence, it may be more feasible for RAN1 to agree on a net effect model that captures the essential behaviors of a DPD and PA combination with compliance to the base station ACLR requirements.

Since DPD is designed to linearize a nonlinear PA, the net effect of both devices is substantially more linear as discussed in the above. That is, such net effect can also be captured by a GMP model as

ynetn=k∈Kanetl∈Lanetaklnetxn-lxn-l2k

with fewer parameters than those for the PA alone. This can be quite beneficial to speeding up link level simulations. For instance, we found the net effect can be captured with the Kanet=8 and Lanet=5 and the complete set of 40 parameters aklnet is provided in Annex A.3 below. The AM-AM response of the net effect model is also shown in Figure 5. It can be observed that the (one-step) net effect model captures the behaviors of passing through the DPD and the PA quite well.

It is not necessary to perform link level simulations using separate models for DPD and PA.

Adopt a net effect model for link-level simulations that captures the essential behaviors of a realistic DPD and PA combination with compliance to the base station ACLR requirements. This requires input from RAN4.

### 2.1.2	FR1 Crest factor reduction (CFR)

It is well known the OFDM signal can be approximately viewed as a zero-mean complex Gaussian signal for which the magnitude follows a Rayleigh distribution:

CCDFx(n)≈exp-xn2σ2

The peak power to average power ratio (PAPR) of an example OFDM signal is shown in Figure 6(a) and is used as an illustration to demonstrate the effects of the models to be discussed below. It can be observed that the PAPR is around 9.5 dB as expected (measured at CCDF value 10-4).

(a) PAPR of OFDM signal			(b) PAPR of CFR processed OFDM signal

Figure 6: Example of PAPR of OFDM signal without or with CFR processing.

With such characteristics, inputs with large magnitudes to the DPD and PA (or their equivalent net effect) models can still drive into the nonlinear region and cause substantial out-of-band emissions due to the long tail of the Rayleigh distribution. One approach to avoid these issues is to back off the input signal power such that the probability of unacceptably large magnitude becomes negligible. This however comes at substantial reduction of the transmit power. For instance, if the signal is scaled such that the mean sample power to the DPD and PA is around -12.7 dB, the spectra of such a signal going through the different models discussed in the above can be observed in Figure 7. The ACLR of this power back off approach will indeed satisfy the base station ACLR requirements, but with an unacceptable cost in amplifier efficiency.

Figure 7: Example of spectra of OFDM signals with power back-off (mean sample power = -12.7 dB).

A better approach, which is used in practice, is to suppress the PAPR of the OFDM signal to control the probability of unacceptably large magnitudes. A simple single-stage CFR model can be as follows:

- Apply hard clipper on the signal:

xn=xn,if x(n)≤LLej∠x(n),otherwise

where, for an example of clipping at 5.7 dB PAPR, the parameter L=105.7/10⋅avgxn2. CFR, regardless of the exact approach used, generates both distortions to the signal and out-of-band emissions as shown in Figure 8. It is therefore necessary to apply filtering after the hard clipper to suppress the out-of-band emissions.

- Apply bandpass filter with, e.g., 25 dB stop band suppression, on the hard clipper output as shown in Figure 8.

A simple single-stage CFR would facilitate the modelling and selection of algorithms while it provides necessary characteristics of the distortion which is induced in UL PRBs which needs to be considered when additional possible cancellation schemes are considered.

After applying the CFR processing, the PAPR of the OFDM signal is reduced to around 7.9 dB (measured at CCDF value 10-4) as shown in Figure 6(b). The error vector magnitude (EVM) for the OFDM signal after CFR is roughly 3%, where EVM is a measure of in-band distortion.

Figure 8: Example of spectra of OFDM signals before, during and after CFR processing.

The benefits of such CFR processing become clear when comparing the CFR processed example shown in Figure 9 with the power back-off example shown in Figure 7. With CFR processing, it is now possible to raise the input signal power to -10.9 dB, which represents a transmit power gain of almost 2 dB compared to the previous example – a significant increase. The ACLR of this CFR processed example will satisfy the base station ACLR requirements (see purple curve in Figure 9).

Figure 9: Example of spectra of OFDM signals with CFR processing (mean sample power = -10.9 dB).

Adopt a simple crest factor processing model, e.g., hard clipping + bandpass filtering, that captures the essential behaviors of a BS design to increase transmit power. This requires input from RAN4.

## 2.2	Examples of gNB Antenna and Interference Channel

### 2.2.1	FR1 Self-interference leakage within a sector

The total self-interference on RX port n originating from TX ports m=0, 1, …M can be modeled as tapped delay lines,

snRxt=p=0P-1m=0M-1hpn,m ∙smTxt-τpn,m

where smTx(t) and snRx(t) are the transmitted and received signals, respectively, at time sample t, where hpn,m is the complex channel coefficient for tap p, and where τpn,m is the propagation delay for tap p.

As discussed above, the gNB antenna system may be equipped with sub-arrays in accordance with Section 5.2.3.2.4 of TR 38.803 v14.3.0 [1], in which case the individual elements within a TX or RX sub-array are assumed to be connected via fixed (complex-valued) weights, and each TX model chain in the previous section is used to drive one sub-array which corresponds only to one polarization. One can then let each TX or RX port in the self-interference channel model represent one sub-array as illustrated in Figure 10. This reduces channel model computational complexity and can also allow for capturing of non-linear coupling effects between elements within a sub-array if needed. Beamforming can then be performed by applying appropriate TX and RX weights on the sub-arrays.

The self-interference channel should be modeled as a set of tapped delay lines directly from TX sub-array ports to RX sub-array ports.

Figure 10: Illustration of self-interference channel modeling for panels with sub-arrays

Appropriate values for the channel coefficients and delays remain to be determined. There may be contributions both from direct propagation from TX ports to RX ports and from reflections from the environment. For the direct propagation path, it is crucial that the channel coefficients are based on realistic setups supported by real measurements or high-fidelity electromagnetic (EM) evaluations. For example, one needs to capture the fact that RX ports closer to the TX panel may have lower isolation than those farther apart, and that the channel coefficients may be frequency dependent.

Self-interference channel coefficients should be based on realistic setups supported by real measurements or high-fidelity electromagnetic (EM) evaluations.

In the following, we provide example isolation results from high-fidelity EM evaluations for the structure shown in Figure 11 where the TX and RX panels are as shown in Figure 10 and isolation mechanism/material is situated between the two antenna panels.

Figure 11: Illustration of TX elevation beam steering angle θ. The right antenna panel refers to TX while the left one to RX. In this illustration, the horizontal beam is at boresight (i.e., azimuth angle ϕ=0°).

We consider the coupling between the TX panel to the RX sub-arrays closest to the isolation structure. We fix the TX horizontal beam direction at the boresight (i.e., ϕ=0°) but consider the TX elevation beam angles at 0º, -15º and 15º. We provide the coupling magnitudes across a one GHz range for the sub-arrays at the same or cross polarization from the TX beam in Figure 12 and Figure 13, respectively. It can be observed that

- The coupling magnitudes vary significantly across different frequencies and can differ up to 30 dB.

- The coupling magnitudes are affected by the TX beam directions. The coupling is lower when the TX beam is pointing horizontally at boresight and rises higher when the TX beam is tilting either upward or downward. The coupling magnitudes can vary up to 20 dB between the three vertical beam directions consider here.

- With horizontal TX beam, the TX panel to RX subarrays couplings at 4 GHz can be around -80 dBc but can differ between the different RX subarrays by up to 20 dB.

  - Furthermore, we see a variation across the frequency range by up to 15 dB. This is an important point since, different operators will own spectrum in different parts of the band, and the band can be quite wide, e.g., band n77 which has bandwidth of 900 MHz. It is not practical to optimize the antenna design for every operator with different holdings in each band. Hence some carriers will experience good isolation, and others not as good.

- With TX beam pointing 15º upward or downward, the TX panel to RX subarrays couplings at 4 GHz rise to around -65 dBc. The differences between different RX subarrays are around 10 dB.

For both system and link level assessment of SBFD, proper modelling of advanced antennas as well as modelling of beamforming impact on the BS TX to RX isolation should be considered.

| - 0 degree (boresight) | - 15 degrees (TX steers main beam toward RX) | - -15 degrees (TX steers main beam away from RX) |
| --- | --- | --- |

Figure 12: TX panel to RX sub-array coupling magnitude curves considering co-polarized ports. Each curve represents the coupling magnitude of the TX panel to a single RX sub-array. Each sub-figure corresponds to a specific elevation angle. The RX sub-arrays are those closest to the isolation structure.

| (a) 0 degree (boresight) | (b) 15 degrees (TX steers main beam toward RX) | (c) -15 degrees (TX steers main beam away from RX) |
| --- | --- | --- |

Figure 13: TX panel to RX sub-array coupling magnitude curves considering cross-polarized ports. Each curve represents the coupling magnitude of the TX panel to a single RX sub-array. Each sub-figure corresponds to a specific elevation angle. The RX sub-arrays are those closest to the isolation structure.

### 2.2.2	FR2 Self-interference leakage within a sector

For FR2 self-interference leakage analysis, we perform Full-electromagnetic simulations performed to analyze the power leakage from the TX panel to RX branches considering a carrier frequency at 30 GHz. The performed simulations assume a gNB antenna with separate panels for transmission and reception. Each panel consists of an antenna array comprising 8 rows and 24 columns of 2x1 dual-polarized sub-arrays. The vertical separation between elements is 0.7 wavelengths and the horizontal separation is 0.5 wavelengths. The separation between the TX and the RX panels is set to 10 cm, i.e., approximately 10 wavelengths at 30 GHz. The simulation models assume that the antenna panels share a common ground plane, and that no radome is present. The presence of a radome is expected to introduce losses and possibly create additional coupling between the TX and RX.

In FR2, Isolation structures based on multi-layer choke structures are relatively simple to implement and tend to provide good isolation performance. The comparison of isolation performance of the choke-based antenna and reference model that uses no isolator structure is shown in Figure 21 for the boresight case and both polarization ports. One can observe that

- The coupling magnitude results indicates that the reference model provides an isolation level of 70 dB over approximately 4 GHz. It is remarkable that the isolation at boresight is much larger than the previously considered FR1 reference model. This is because the high dimensionality of the FR2 array as compared to that of the FR1 antenna, which yields a very sharp far-field beam that reduces the power leakage toward the RX.

- For the choke-based antenna isolator structure, the co-polarized port provides an isolation of 80 dB with a bandwidth support of roughly 3 GHz, while the cross-polarized port offers the same isolation level but over a slightly larger bandwidth.

- Choke-based antenna isolator structure provide enough isolation improvement to keep the isolation below -80 dB over some bandwidth for this FR2 antenna.

| - Co-polarized ports | - Cross-polarized ports |
| --- | --- |

Figure 14: Comparison of the coupling magnitude between the choke-based and reference antenna models. Each curve represents the coupling between the TX panel and a single RX branch.

It is important to remark that the high dimensionality of the TX panel, the 10 cm separation, and the isolation structure are the main elements in achieving 80 dB in isolation. The FR1 antenna models did not have as many antenna elements in the TX panel, nor the relative panel-to-panel distance of 5 wavelengths.

An SBFD antenna should satisfy the isolation requirements also within a beam tilting range. Figure 14 (a) shows the isolation performance of the choke-based model for co-polarized ports and varying steering angle, and Figure 14 (b) presents the same metric for cross-polarized ports. These simulation results suggest that

- Choke-based antenna isolator supports 80 dB within the ±15 degrees tilting range over a bandwidth of approximately 4 GHz. By contrast, the FR1 antenna model suffers a significant loss in isolation performance when steering in the same range, as seen in Figure 12 and Figure 13. This is because the FR1 antenna has smaller dimensions, therefore producing broader radiation beams as compared to the FR2 antenna model considered in this section.

- The comparison between Figure 14 (a) and Figure 14 (b) indicates that the cross-polarized antenna provides a slightly larger isolation than the co-polarized ports.

| - boresight | - +15 toward RX | - -15 away from RX |
| --- | --- | --- |

Figure 15: Coupling magnitude of the choke-based antennas for varying steering angle θ and co-polarized RX ports.

| - boresight | - +15 toward RX | - -15 away from RX |
| --- | --- | --- |

Figure 16: Coupling magnitude of the choke-based antennas for varying steering angle θ and cross-polarized RX ports.

For FR2, using a structure with RF chokes, 80dB of isolation is achievable over a reasonable bandwidth. Unlike FR1, the isolation does not vary with beam steering.

### 2.2.3	FR1 Interference leakage between sectors

Sectorized site deployments are typically used to provide full coverage in the horizontal plane. This means that up to three antennas will be mounted close together on the mast as illustrated by the photos of representative WA and MR commercial sites in Figure 17. As can be seen in these site deployments, the edge-to-edge distance dH between antennas in different sectors can be quite small, e.g., on the order of 40 cm.

Figure 17: Representative WA and MR commercial sectorized sites showing that the edge-to-edge distance dH between antennas in different sectors can be quite small.

We evaluate the inter-sector interference via detailed electromagnetic (EM) simulations with a simplified 3-sector site setup shown in Figure 18.

- Top view of site

- Side view of site				(c) Side view of site (actually simulated)

Figure 18: Simplified 3-sector site setup used in EM simulations of inter-sector interference. Note that to save simulation time, only half of the SBFD array (Tx panel or Rx panel) is explicitly modeled in each sector. This results in an underestimate of the inter-sector interference since in reality, the received interference power observed at Sector 3 is due not only to the Tx panel in Sector 1, but also due to the Tx panel in Sector 2 (not explicitly modeled). To compensate, we adjust the interference level upwards by 3 dB.

The simplified setup has the following characteristics

- Each antenna panel (Tx or Rx) contains an array of 8 x 8 array of cross-polarized antenna elements  128 elements in total

  - To save simulation time, only one panel (Tx or Rx) of the SBFD antenna array is modeled in each sector:

    - Tx panel in Sector 1

    - Rx panels in Sectors 2 and 3

    - The locations of the non-modeled panels are indicated as dashed lines in Figure 18

  - Inter-sector interference is measured at the Rx panel in Sector 3 due to transmissions from the Tx panel in Sector 1. The received power is summed over all antenna elements in the Rx panel.

    - This results in an underestimate of the inter-sector interference since in reality the received interference power observed at Sector 3 is due not only to the Tx panel in Sector 1, but also due to the Tx panel in Sector 2 (not explicitly modeled).

    - To compensate, we adjust the received interference power upwards by 3 dB

- Edge-to-edge horizontal distance between sectors is dH = 40 cm (4 wavelengths at 3 GHz)

- Edge-to-edge vertical separation of Tx and Rx panels is dV = 30 cm (3 wavelengths at 3 GHz)

- Vertical antenna element spacing = 0.7λ

- Horizontal antenna element spacing = 0.5λ

- Currently, sub-arrays are not modeled

  - This is done to simplify the simulations; however, in reality sub-arrays are the norm in AAS products.

- Center frequency = 3 GHz

Back-lobes as well as side/grating lobes from the beam-forming array in one sector creates interference to other sectors within the site. We find that the interference level depends on the azimuth and elevation beam steering directions. Figure 19 illustrates how the backlobes from one sector interact with the other co-site sectors for the example of a beam steered at 30º in azimuth.

Figure 19: Illustration of inter-sector interference from EM simulations

To evaluate how coupling depends on azimuth and elevation beam steering direction, we evaluate the following combinations:

- Fix the elevation beam direction at boresight (i.e.,θ=0°) and vary the azimuth beam direction angles ϕ as 0º, 30º and 60º.

- Fix the azimuth beam direction at boresight (i.e., ϕ=0°) and vary the elevation beam direction angles θ as 0º, -15º and -30º.

- In order to reduce the simulation time, we did exhaustively evaluate all combinations of azimuth and elevation beam directions. What is missing is combinations with non-zero (non-boresight) directions for both azimuth and elevation, e.g., (ϕ,θ) = (30º, -15º).

Figure 20, shows the inter-sector isolation based on the site setup described above.

(a) Azimuth beam steering ϕ for fixed θ=0°		(b) Elevation beam steering θ for fixed ϕ=0°

Figure 20: Inter-sector Tx-panel-to-Rx-panel coupling magnitude accounting for interference from 2 sectors for different beam steering angles in azimuth and elevation. Panel-to-Rx port coupling magnitude is 10*log10(P) dB lower where P is the number of Rx ports.

One can see a large variation in coupling magnitude (-75 to -55 dB) depending on the azimuth and elevation beam steering angles as well as well as variations across the frequency band. We point out that these values are based on a summation of received power across the antenna elements of the Rx panel, i.e., panel-to-panel isolation. Typically, however, it is customary to quote the isolation from a panel to Rx port basis, instead. Assuming a port corresponds to a sub-array of 4 co-polarized antenna elements in the vertical dimension, the number of Rx ports is 128 / 2 / 4 = 16. Hence the coupling magnitude should be adjusted downward by 10*log10(16) = 12 dB. This results in a range of panel-to-Rx-port coupling magnitude values of -87 to -67 dB.

The results shown here do not catch the full extent of isolation variation. Specifically, the isolation can degrade if one considers also combinations of azimuth and elevation beamforming both with non-zero angles, e.g., (ϕ,θ) = (30º, -15º).

We point out that other realistic effects that are not modeled here may degrade isolation further, for example:

- re-radiation from the electronics on the backside of the antennas

- practical site aspects such as equipment and other metallic objects in between sectors that can act as PIM sources

- presence of sub-arrays (the norm in AAS products), that result in higher side lobes and grating lobes, thus reducing achievable isolation.

- presence of radomes

The above effects are not necessarily solved simply by moving antennas further apart.

For both system level and link level assessment of SBFD, proper modelling of advanced antennas as well as modelling of beamforming impact on the inter-sector TX to RX isolation needs be considered. For the simple exemplary site setup we have simulated for FR1, we see Tx-panel-to-Rx-port isolation values in the range of 67 to 87 dB depending on the azimuth and elevation beam steering directions and the frequency within the band. These values would most likely degrade if other realistic effects are included, e.g., electronics on the backside of the antenna, equipment and other metallic objects between sectors in a practical site, the presence of sub-arrays, and the presence of radomes.

### 2.2.4	FR2 Interference leakage between sectors

Sectorized site deployments are typically used to provide full coverage in the horizontal plane also for FR2. We evaluate the inter-sector interference via detailed electromagnetic (EM) simulations with a simplified 3-sector site setup shown in Figure 21.

Figure 21: Simplified 3-sector site setup used in EM simulations of inter-sector interference. Note that to save simulation time, only half of the SBFD array (Tx panel or Rx panel) is explicitly modeled in each sector. This results in an underestimate of the inter-sector interference since in reality, the received interference power observed at Sector 3 is due not only to the Tx panel in Sector 1, but also due to the Tx panel in Sector 2 (not explicitly modeled). To compensate, we adjust the interference level upwards by 3 dB.

The simplified setup has the following characteristics

- Each antenna panel (Tx or Rx) contains an array of 8 x 8 array of cross-polarized antenna elements  128 elements in total

  - To save simulation time, only one panel (Tx or Rx) of the SBFD antenna array is modeled in each sector:

    - Tx panel in Sector 1

    - Rx panels in Sectors 2 and 3

  - Inter-sector interference is measured at the Rx panel in Sector 3 due to transmissions from the Tx panel in Sector 1. The received power is summed over all antenna elements in the Rx panel.

    - This results in an underestimate of the inter-sector interference since in reality the received interference power observed at Sector 3 is due not only to the Tx panel in Sector 1, but also due to the Tx panel in Sector 2 (not explicitly modeled).

    - To compensate, we adjust the received interference power upwards by 3 dB

- Edge-to-edge horizontal distance between sectors is dH = 40 cm (40 wavelengths at 30 GHz)

- Edge-to-edge vertical separation of Tx and Rx panels is dV = 10 cm (10 wavelengths at 30 GHz)

- Vertical antenna element spacing = 0.6λ

- Horizontal antenna element spacing = 0.5λ

- Currently, sub-arrays are not modeled

  - This is done to simplify the simulations; however, in reality sub-arrays are the norm in AAS products.

- Center frequency = 30 GHz

To evaluate how coupling depends on azimuth and elevation beam steering direction, we evaluate the following combinations:

- Fix the elevation beam direction at boresight (i.e.,θ=0°) and vary the azimuth beam direction angles ϕ as 0º, 30º and 60º.

- Fix the azimuth beam direction at boresight (i.e., ϕ=0°) and vary the elevation beam direction angles θ as 0º, -15º and -30º (downtilt)

Figure 22 shows the inter-sector isolation based on the site setup described above.

(a) Azimuth beam steering ϕ for fixed θ=0°	(b) Elevation beam steering θ for fixed ϕ=0°

Figure 22: Inter-sector Tx-panel-to-Rx-panel coupling magnitude accounting for interference from 2 sectors for different beam steering angles in azimuth and elevation. Panel-to-Rx port coupling magnitude is 10*log10(P) dB lower where P is the number of Rx ports.

One can see a large variation in coupling magnitude (-80 to -57 dB) depending on the azimuth and elevation beam steering angles as well as well as variations across the frequency band. The results shown here do not catch the full extent of such effects. As in the previous section for FR1, these values are based on a summation of received power across the antenna elements of the Rx panel, i.e., panel-to-panel isolation. Typically, however, it is customary to quote the isolation from a panel to Rx port basis, instead. Assuming a port corresponds to a sub-array of 2 co-polarized antenna elements in the vertical dimension, the number of Rx ports is 128 / 2 / 2 = 32. Hence the coupling magnitude should be adjusted downward by 10*log10(32) = 15 dB. This results in a range of panel-to-Rx-port coupling magnitude values of -95 to -72 dB.

The results shown here do not catch the full extent of isolation variation. Specifically, the isolation can degrade if one considers also combinations of azimuth and elevation beamforming both with non-zero angles, e.g., (ϕ,θ) = (30º, -15º).

We point out that other realistic effects that are not modeled here may degrade isolation further, for example:

- re-radiation from the electronics on the backside of the antennas

- practical site aspects such as equipment and other metallic objects in between sectors that can act as PIM sources

- presence of sub-arrays (the norm in AAS products), that result in higher side lobes and grating lobes, thus reducing achievable isolation.

- presence of radomes

The above effects are not necessarily solved simply by moving antennas further apart.

For both system level and link level assessment of SBFD, proper modelling of advanced antennas as well as modelling of beamforming impact on the inter-sector TX to RX isolation needs be considered. For the simple exemplary site setup we have simulated for FR2, we see isolation values in the range of 72 to 95 dB depending on the azimuth and elevation beam steering directions and the frequency within the band. These values would most likely degrade if other realistic effects are included, e.g., electronics on the backside of the antenna, equipment and other metallic objects between sectors in a practical site, the presence of sub-arrays, and the presence of radomes.

## 2.3	Transmit Beam Nulling

Beam nulling refers to the TX antenna using degrees of freedom to steer a null, and hence reduce power towards the receiver. Placing nulls in this way will reduce the number of degrees of freedom available for DL performance and hence can reduce DL SNR or reduce MIMO performance. The degree of beam nulling gain will depend on the sacrifice of DL performance.

For RX IM interference to the UL sub-band, it is not fully clear that the interference signal will have the same phase characteristics as the wanted signal and hence that the beam nulling will also act on the interference. However, here we assume that the beam nulling efficiency for the TX signal IM interference into the UL sub-band is the same as for the TX signal itself.

Our antenna isolation evaluation in Section 2.2 has shown that

- The self-interference isolation levels between the TX and RX arrays are dependent on TX beam steering direction. In particular, we have identified that vertically tilted TX DL beams can reduce the self-interference isolation by 10 dB.

- For each TX DL beam steering direction, different RX subarrays may experience different self-interference isolation at different frequencies. It is desirable to ensure the worst self-interference isolation levels of any RX subarrays are no worse than a prescribed upper bound such that the hardware can be dimensioned correctly.

In the following, we present preliminary TX beam nulling evaluation results for a 2x4 antenna array shown below.

In Figure 23, we provide a TX beam nulling evaluation to ensure the worst self-interference isolation for any RX subarray is no worse than a target isolation. In this diagram the desired DL beam is steered to boresight.

- In the upper panel, we show the per RX subarray isolation levels without nulling relative to the target in dashed blue lines. The worst isolation across all RX subarrays without nulling is shown by the solid blue curve.

- In the upper panel, we show the per RX subarray isolation levels with nulling relative to the target in dashed orange lines. The worst isolation across all RX subarrays with nulling is shown by the solid orange curve.

- In the lower panel, we show the losses to the desired TX DL beam.

For this boresight TX DL beam, we observe that

- TX beam nulling can reduce the worst-case self-interference isolation across RX subarrays and across frequencies down by 7 dB.

- However, such beam null reduces the desired TX DL beam gains by up to 1.5 dB across a wide range of frequency. The average desired TX DL beam gain loss across all frequencies is 0.9 dB.

Figure 23: Beam nulling performance when the desired TX beam is boresight

In Figure 24, we provide a similar evaluation as above, except this time the desired DL beam is downtilted by 10 degrees.

- When TX DL beam is tilted toward the RX array, the worst-case self-interference level is 10 dB higher than when the TX DL beam is not tilted toward the RX array.

- Beam nulling is capable of bringing the heightened self-interference levels down to the same target as set for the boresight beam evaluation shown in Figure 23.

- However, the price of this strong beam nulling is rather steep. As shown in the lower panel of Figure 24, the desired TX DL beam gains are reduced by up to 5.5 dB. The average desired TX DL beam gain loss across all frequencies is 4.6 dB.

Figure 24: Beam nulling performance when the desired TX beam is tilted downward

Since DL resources are reallocated for UL use in an SBFD system, DL capacity and DL coverage (because heightened resource utilization on the remaining DL resources increased intercell interference) are already negatively impacted. Applying beam nulling to address the self-interference issues for the UL subbands introduce further direct degradation to the DL coverage and performance. As shown in the above, applying strong TX beam nulling to counter downtilted beams results in large DL losses to the range of more than 5 dB. This loss is a power loss in downlink, but the overall reduction in DL SNR to a UE may be impacted further due to reduced degrees of freedom to perform SU- and MU-MIMO.

The gain from beam nulling increases when the TX beam is steered and the antenna isolation decreases. Thus, beam nulling can to some extent reduce the variation of the overall spatial isolation due to beam steering. It may also reduce the frequency variation. However, there is a const in terms of reduced DL beam gain.

The cost of beam nulling in downlink can be substantial; we have observed up to 5dB DL power loss. There may be further DL losses due to lower degrees of freedom for MIMO operation.

When deciding beam nulling gains, downlink impacts should be considered.

## 2.4	Homodyne/Heterodyne gNB Receiver Modeling

There are various receiver structures used for BS depending on the deployment as well as capabilities such as beam forming for massive MIMO type of BS or support of multiple carriers or bands. We provide a high-level survey of different base station receiver architectures and examine their compatibility with SBFD operations in Section 2.10 of the companion contribution[14][19]. It was observed that the implementation complexity and difficulty of sharp analog filtering that is suitable separating DL and UL subbands with a few RBs of guard frequencies in a SBFD carrier depends strongly on the operating frequency at which such analog filtering takes place. Such analog filtering is more feasible with homodyne type receivers but becomes increasingly infeasible with the heterodyne or direct RF sampling type receivers. The latter consistent with an industry trend toward supporting wide RF bandwidths for multi-carrier/multi-band receivers for at least higher power base stations.

In this section, we discuss an initial modeling approach to homodyne/heterodyne type base station receivers.

A typical BS receiver applies analogue bandpass filtering for the whole 3GPP band in order to reject out of band signals. In-band, digital filtering is used to suppress interferers from carriers of other operators.

Figure 25: Illustration of analog band filtering and digital channelization filtering at BS

The analogue front end of the BS receiver needs to process both the wanted carrier and other unwanted interferers within the band without significant distortion in order that the entire signal can be passed to the digital domain for filtering. Any distortion within the analogue domain that falls into the wanted carrier frequency range will degrade the receiver sensitivity.

According to the Frii formula, the overall noise figure of the receiver is related to the noise ratio of each of the individual components of the analogue processing chain and the gain of the first component propagates to all subsequent stages. It is hence advantageous to achieve as much gain as possible close to the antenna. For this reason, the first component in the receiver chain is a high gain, low noise amplifier (LNA). Also, any filtering that takes place before the LNA needs to be designed very carefully since filter insertion losses will have a significant impact on the noise figure for the overall receive chain.

While not necessarily the industry trend for multi-carrier/multi-band receivers, we summarize the initial examples of homodyne/heterodyne type gNB receiver modelling incorporating the components discussed in this section below in Figure 26. Representing the various non-linearities in the receiver chain faithfully is essential to studying the impact of gNB self-interference cancellation as well as gNB-gNB CLI for both SBFD and dynamic TDD. This is vital since both intra-operator and inter-operator CLI is due to spectral leakage, both inter-sub-band and inter-channel, due to these non-linearities. In this section we propose a generic modelling approach that can be used for both link-level and system-level evaluations. From this foundation, additional details and modelling of other receiver components such as the AGC and ADC noises can be further incorporated if needed.

The details of the digital cancellation are explored in the next section.

Figure 26: Summary of initial examples of homodyne/heterodyne type BS receiver modeling chain.

### 2.4.1	Low noise amplifier (LNA) model

All amplifiers have an upper output power limit and can suffer from intermodulation distortion. For output power levels not sufficiently backed off from the upper limit, the intermodulation distortion appears as spectral regrowth outside the bandwidth of the signal to be amplified as discussed in Section 3.1 and shown below:

Figure 27: An illustration of the spectral regrowth outside the bandwidth of a signal

A basic third-order model for the LNA at a BS receiver chain can be characterized by the complex baseband representation of the output voltage (excluding the harmonic term far away from the carrier at fc) where g is the voltage gain of the amplifier, vi is the input voltage, vo is the output voltage, and the (real) coefficient a characterizes the 3rd order non-linearity:

vo=g∙vi-3a4vi2vi

The unwanted IM3 distortion power is given by

IM3=3a42Pi3

where Pi=vi2 is the input power. That is, for every 1 dB increase in the input power, the IM3 power increases by 3 dB. As a result, the third order term will intercept the linear term when the input power is at

4g3a≜IIP32

as illustrated in Figure 28.

Figure 28: Illustration of simple third-order modelling of a BS receiver LNA.

Using the above notations, one can relate the gain-normalized IM3 interference power, input power, and IIP3 as follows:

IM3dB=3⋅PidB-2⋅IIP3dB

IIP3dB=(3⋅PidB-IM3dB)/2

where IIP3dB≜20log10IIP3.

#### 2.4.1.1	FR1 models

To further substantiate the discussion of SBFD implementation challenges and potential solutions, we consider in the following three representative BS classes with three different levels of transmit powers. These representative transmit powers are selected based on the RAN4 defined power limits for the local area (LA) and medium range (MR) BS classes and based on the system evaluation assumptions for this SI for the wide area (WA) BS class.

- LA BS with 24 dBm power

- MR BS with 38 dBm power

- WA BS with 53 dBm power

The reference sensitivity levels are also different for the three classes of BS in RAN4 specs. Taking 20 MHz with 30 kHz sub-carrier spacing as representative UL bandwidth for SBFD feasibility analysis:

- LA BS Prefsens is -87.6 dBm

- MR BS Prefsens is -90.6 dBm

- WA BS Prefsens is -95.6 dBm

RAN4 spec TS 38.104 stipulates BS performance requirements for receiver intermodulation. Two interfering signals are placed on the side of the desired signal such that third order intermodulation powers fall in the desired signal bandwidth. For example, from table of Table 7.7.2-2 of TS 38.104:

- Desired signal of 20 MHz BW centered at fc

- First interfering CW signal centered at f1=fc+17.395

- Second interfering DFTS-OFDM signal (5 MHz BW) centered at f2=fc+27.5

The center frequency of their third order intermodulation falls at 2f1-f2=fc+7.29. Given the 5 MHz interferer bandwidth, this intermodulation distortion falls entirely within the desired signal bandwidth. One can estimate the minimum required LNA IIP3dB for the different BS classes as shown in the following table:

Table 1 Minimal LNA IIP3dB parameter estimation based on RAN4 receiver intermodulation requirements.

|  |  | WA BS | MR BS | LA BS |
| --- | --- | --- | --- | --- |
| A | RAN4 REFSENS | -95.6 | -90.6 | -87.6 |
| B | RAN4 RX intermodulation interferer power | -52 | -47 | -44 |
| C | RAN4 allowed desensitization | 6 | 6 | 6 |
| D | Implied INR = 10*LOG10(10^(C/10)-1) | 4.7 | 4.7 | 4.7 |
| E | Implied gain-normalized distortion = A+D | -90.9 | -85.9 | -82.9 |
| F | Estimated minimum IIP3dB = (3*B-E)/2 | -32.6 | -27.6 | -24.6 |

In addition to the distortion powers, the presence of these blockers also causes gain compression of the desired signal. Implementation improvements are typically added to avoid gain compression and the introduction of further distortions, particularly for high peak-to-average-ratio (PAPR) waveforms like the OFDM. The implementation also needs to consider also other RAN4 blocker and adjacent channel selectivity requirements (listed in Table 4) that have interferers at up to 9 dB higher power. Such implementation improvements come with higher complexity and energy consumption and are designed with careful tradeoff of performance gains and cost analysis. Typical implementation improvements are in the range of 5 – 10 dB. In the following, we make an optimistic assumption of 10 dB implementation improvement to arrive at the modeling parameters for the LNA. These values are used in the companion contribution [14][19] for the analysis of self-interference due to RX LNA spectral regrowth based on current BS requirements.

|  |  | WA BS | MR BS | LA BS |
| --- | --- | --- | --- | --- |
| G | Assumed IIP3dB improvement | 10 | 10 | 10 |
| H | Assumed typical IIP3dB | -22.6 | -17.6 | -14.6 |

Adopt a third order representation model in RAN1 studies to capture the essential behaviors of typical high-gain low noise amplifiers (LNA) in BS receiver chains.

#### 2.4.1.2	FR2 models

In the previous section for FR1 we illustrate the dimensioning of the LNA where the IIP3 is derived based on RAN4 intermodulation (IM) requirements, and then a 10 dB implementation improvement is added to the result. In contrast, for FR2, such an approach leads to quite a low IIP3 compared to what is typically used in commercial products. Here we assume a typical IIP3 of -30 dBm.

### 2.4.2	Reciprocal mixing of phase noise

A receiver includes a down-conversion mixer, which multiplies an LO with the RF input signal. The resulting output in the frequency domain is a convolution of the LO and RF spectra. If the RF input consists of a low power wanted signal and a high-power interferer, then the phase noise spectrum of the LO will be superimposed on the interferer with the interferer power level, and part of the resulting distortion will fall into the wanted signal. This is known as reciprocal phase noise mixing.

Using the phase noise models [7][8] discussed in RAN4 Rel-17 as examples, the phase noise power spectral densities are illustrated in Figure 29.

(a) 3.5 GHz							(b) 28 GHz

Figure 29: Illustration of 3.5 GHz and 28 GHz phase noise models discussed in RAN4 Rel-17.

Assuming there is a guard band of BWGB between the DL and UL subbands. The weighting function for integrating the reciprocal mixing interference power from the DL subbands to the center sub-carrier in the UL subband is

WRM,0f=ΠfBWUL+2BWGB+2BWDL-ΠfBWUL+2BWGB

where the rectangular function Πx=1 if |x|≤1/2 and 0 otherwise. The weighting function for integrating the reciprocal mixing interference power from the DL subbands to the left edge sub-carrier (i.e., sub-carrier index -N/2) in the UL subband is

WRM,-N/2f=Πf-BWUL/2BWUL+2BWGB+2BWDL-Πf-BWUL/2BWUL+2BWGB=WRM,0f-BWUL/2

In general, for any UL sub-carrier with index k∈-N/2, …,N/2-1, where N is the number of sub-carriers in the UL subband, the weighting function is given by

WRM,kf=WRM,0f+k⋅SCS

The reciprocal mixing interference power from the DL subbands to the sub-carrier k is given by

IRM,k=-∞∞ΦPNfWRM,kfdf

Summing over all UL sub-carriers, the total interference powers from the DL subbands to the UL subband is given by

IRM=k∈-N/2, …,N/2-1IRM,k

#### 2.4.2.1	FR1 models

Assuming 30 kHz SCS and 5 RB of guard band in a 40-20-40 MHz SBFD carrier, the weighting functions for center sub-carrier is illustrated on the left of Figure 30. This weighting function for the left edge UL sub-carrier is illustrated on the right of Figure 30.

Figure 30: Weighting function for computing the reciprocal mixing interference power from the DL subband to one UL sub-carrier in a 40-20-40 MHz SBFD carrier. The bandwidth of the guard band is assumed to be BWGB=1.8 MHz (5 RBs).

For 30 kHz SCS, the interference power from the DL subbands to each of the 20 MHz UL sub-carriers is provided in Figure 31.

Figure 31: Reciprocal mixing interference to different sub-carriers in the UL subband (at different offset from UL center frequency) from the DL subbands in a 40-20-40 MHz FR1 SBFD carrier.

Summing over all UL sub-carriers, the total interference powers from the DL subbands to the UL subband are given by the following table:

Table 2 Total Interference powers from DL subband to UL subband in a SBFD carrier in FR1

| Phase noise model | Interference caused by reciprocal mixing of phase noise [dBc] |
| --- | --- |
| R4-2011494 (UE, low-cost BS) | -60.3 |
| R4-2010186 DM=5 dB (UE, medium quality BS) | -64.7 |
| R4-2010186 DM=0 dB (high quality BS) | -69.7 |

The interference power caused by reciprocal mixing of phase noise in a 40-20-40 MHz SBFD carrier is around -60 to -70 dBc depending on BS implementation.

The above values are used in the companion contribution [14][19] for the analysis of interference in the UL subband due to reciprocal mixing.

Adopt phase noise modelling in RAN1 studies to capture the distortion introduced by high power leakage from the DL sub-bands into the UL sub-bands. The phase noise models in TR 38.803 or those provided by RAN4 during the Rel-17 phase can be adopted as baseline models.

#### 2.4.2.2	FR2 models

Assuming 120 kHz SCS and 3 RB of guard band in a 75-50-75 MHz FR2 SBFD carrier, the interference power from the DL subbands to each of the 50 MHz UL sub-carriers is provided in Figure 32.

Figure 32: Reciprocal mixing interference to different sub-carriers in the UL subband (at different offset from UL center frequency) from the DL subbands in a 75-50-75 MHz FR2 SBFD carrier.

Summing over all UL sub-carriers, the total interference powers from the DL subbands to the UL subband are given by the following table:

Table 3 Total Interference powers from DL subband to UL subband in a SBFD carrier in FR2

| Phase noise model | Interference caused by reciprocal mixing of phase noise [dBc] |
| --- | --- |
| R4-2011494 (UE, low-cost BS) | -44.0 |
| R4-2010186 DM=5 dB (UE, medium quality BS) | -49.5 |
| R4-2010186 DM=0 dB (high quality BS) | -54.5 |

### 2.4.3	Analog filtering and analog to digital converter (ADC)

The analog to digital converters (ADC) for a BS are designed to handle a dynamic range covering weak UL signals and the potential adjacent channel blockers without losing the standard required sensitivity. For example, for FR1, the RAN4 spec TS 38.104 further stipulates BS performance requirements for in-band narrowband blocker, adjacent channel selectivity, and in-band general blocker. For allowance of 6 dB desensitization, the RAN4 test powers for these blockers or adjacent channels are listed in the following table.

Table 4 RAN4 blocker and adjacent channel selectivity test interference power levels.

|  |  | WA BS | MR BS | LA BS |
| --- | --- | --- | --- | --- |
| A | RAN4 REFSENS | -95.6 | -90.6 | -87.6 |
|  | RAN4 in-band narrowband blocker power | -49 | -44 | -41 |
|  | RAN4 adjacent channel interfering signal power | -52 | -47 | -44 |
|  | RAN4 in-band general blocker power | -43 | -38 | -35 |

Note that, as illustrated in Figure 33, the narrow-band blocker and adjacent channel test cases, being closest to the desired signal, are most relevant to the SBFD discussion.

Figure 33: Illustration of RAN4 BS performance requirements for adjacent channel selectivity (ACS), in-band general blocking and in-band narrowband blocking conditions.

Assuming an antenna array spatial isolation of 80 dB is feasible for all three BS classes, the direct leakage from the TX frontend in the DL sub-bands can be estimated in the following table.

Table 5: Example IM3 distortion in the UL sub-band caused by SBFD DL sub-bands assuming 80 dB spatial isolation and typical existing BS LNA components. (All powers are gain normalized.)

|  |  | WA BS | MR BS | LA BS |
| --- | --- | --- | --- | --- |
| A | RAN4 REFSENS | -95.6 | -90.6 | -87.6 |
| I | TX power | 53 | 38 | 24 |
| J | Antenna isolation | -80 | -80 | -80 |
| K | TX power in DL sub-bands = I+J | -27 | -42 | -56 |

As can be seen, depending on the BS class, the interfering power in the DL-subbands can be significantly higher than the RAN4 blocker levels listed above. For example, for the WA BS class, interfering power can be 22 dB or more higher than current RAN4 blocker performance test powers.

To address this, one possibility increase the bit width of the ADC. To cover an increase of 22 dB dynamic range; however, this has large impact on power consumption which roughly doubles with each additional added bit.  Another possibility is to introduce analog filtering before the ADC. The implementation challenge and feasibility of this approach depends on the base station receiver architecture choices. We discuss feasibility aspects for both ADC and analog filtering in Section 2 of our companion contribution [19] for both FR1 and FR2.

Adopt modelling of analog filtering, if present, in RAN1 link level studies to capture potential impacts to digital cancellation feasibility and performance.

### 2.4.4	Digital filtering

Digital channel filtering after ADC is typically implemented to reduce adjacent channel interference in the desired channel or, in the SBFD case, UL sub-band. Assuming an antenna array spatial isolation of 80 dB is feasible for all three BS classes, the direct leakage from the TX frontend in the DL sub-bands are estimated in the Table 5 and can be 22 dB or more higher than current RAN4 blocker performance test powers.

In-band ACS requirement can be fulfilled several possible solutions:

- Strong analog filtering leaving ADC and digital channel filtering unchanged from typical BS implementation.

- Analog filtering and larger bit width ADC and sharper digital channel filtering.

  - There is a wide range of possible suppression allocations between analog and digital filtering.

Adopt explicit digital filtering models in RAN1 link level studies to capture potential impacts to digital cancellation feasibility and performance.

## 2.5	Example digital self-interference cancellation solution for FR1 homodyne/heterodyne type base station receivers

In modern FR1 MIMO base station, TX beamforming is performed digitally in the frequency domain to direct the beams at different PRBs to different directions for the scheduled UEs allocated to different PRBs. In fact, even for a single UE, different beamforming weights for different PRBs are also needed to align with the UE’s frequency domain channel responses. Such frequency-selective beamforming weights manifest in the time domain as if the signals have been through multi-tap channels already. With multi-layer SU-MIMO and MU-MIMO, the baseband signal into the TX chain is the superposition of more than one beamformed signals targeting the multiple co-scheduled UEs. The superposition of these signals then goes through the transmitter chains with the components discussed in Section 2.1, where many of the components introduce further multi-tap and nonlinear responses.

In Figure 34, we provide an initial example of digital cancellation solutions. The purpose of the discussion is two folds.

- First is to flesh out the essential signals/connections between the TX antenna array and the RX antenna array for further link level study/discussion.

- Second is to estimate how such cancellation solutions scale with the antenna array sizes and the effective lengths of multi-tap channel responses.

Figure 34: An initial generic example of digital self-interference cancellation implementation for FR1 homodyne/heterodyne receivers. The red “AF” box represents an analog subband filter for the SBFD operations.

To support DPD, current FR1 gNB TX chains implement a coupler to collect the RF output for reference. The coupled signal is attenuated and down converted. An anti-aliasing filter is applied to remove the unwanted image before the signal is digitized. The digitized signal is fed into the DPD for adapting its coefficients.

On the RX side, as discussed in the previous sections, analog sub-band filters (shown in the figure as a red “AF” box) are needed to suppress the leaked TX powers in the DL sub-bands. Depending on the receiver architectures and the SBFD use cases, low pass or band pass filters are needed. Such filters need to be much sharper than normal anti-aliasing filters after down conversion since there is only a small guard frequency gap between the DL and UL sub-bands.

The digital cancellation fabric takes the analog coupled signal from each of the TX chains and applies the same analog sub-band filtering as those in the RX chains before digitization. For a 100 MHz carrier, the sampling rate is at 122.88 MHz. Hence, one time sample corresponds to 2.44 m. To cover potential strong reflectors within even a small radius, the adaptive filters need to keep track of several taps if no oversampling is used. In reality, delays of the reflected signals are generally not aligned with the sampling grid perfectly. Oversampling is likely needed to obtain adequate performance. The filter lengths will scale with the oversampling rate accordingly. The analog filters also add to the effective lengths of the overall channel responses.

The digital cancellation fabric takes inputs from NT TX chains and keeps a memory of the most recent L input values for each of these inputs. The fabric applies one set of filter weights on these NTL values to produce one cancellation signal to be used by one RX chain. To serve NR RX chains, NR sets of filter weights are needed. Effectively, for each new sampling time, the cancellation fabric multiplies the stored inputs from the TX chains by a NR×NTL matrix to obtain NR cancellation samples. To obtain the NR×NTL filter weights, the digital cancellation fabric needs inputs from the NR RX chains as well.

The complexity of digital self-interference cancellation scales with the product of (1) the number of TX chains, (2) the number of RX chains and (3) the effective length of the multi-tap response of the environment and the analog RX frontends.

Note that the digital self-interference cancellation solutions may need to address not only the direct TX leakage from the TX array to the RX array. As discussed in the above, additional self-interferences can be caused by the interaction of the high received powers in the DL sub-bands and RX nonlinearity.

## 2.6	 Summary of Modelling Discussion

In the previous sections, we provide an initial set of example models for the TX chains, antenna coupling, and RX chains for typical BS implementations. One important purpose of providing these examples is to illustrate what is needed for a realistic assessment at the link-level of various interference suppression/cancellation approaches, e.g., Tx beam nulling, digital-IC, etc, to determine how many dBs of suppression is practically achievable. This can serve as input to verify/justify assumptions used in system-level evaluations. This is needed since the LS reply from RAN4 on interference modeling [14] provides too wide a range on the digital IC suppression value (0 – 50 dBc) and beam nulling/isolation value (0 – 40 dBc) to be useful – see table below from LS. This ranges needs to be narrowed down, and suitable link-level evaluations can provide that.

Table 6 value range of RSIC

| Parameter | FR1(Frequency Range 1) | FR2(Frequency Range 2) |
| --- | --- | --- |
| Spatial isolation | 50~80dBc | 80-120 dBc |
| Frequency isolation | 45 dBc | 22.5~30 dBc |
| Beam nulling /isolation | 0~40 dBc | 0~40 dBc |
| Digital IC | 0~50 dBc | 0~50 dBc |
| Overall RSIC capability | 95 ~185 dBc | 102.5~ 205 dBc |
| NOTE1: Other isolation schemes could be discussed further. NOTE 2: Both transmitter leakage to the RX sub-band and interference arising from receiver imperfections need to be considered. Receiver imperfections may reduce the RSIC to be lower than the RSIC considering transmitter leakage alone. RAN4 will assess impact of Rx impairments on the RSIC capability. But the RSIC model can potentially be simplified to address impact from both aspects together. |  |  |

Agreement

RAN1 agrees link-level simulations (LLS) may be performed for various purposes related to SBFD performance and feasibility in both FR1 and FR2, interested companies may perform LLS at least for the following purpose:

- To evaluate coverage performance

  - Option 1: Take link level evaluation methodology in TR 38.830 (i.e., LLS + Link budget analysis) as starting point to evaluate the coverage performance (e.g., MPL, MCL, MIL) for SBFD considering inter-gNB/sector interference and self interference.

  - Other options (e.g. SLS as a tool to obtain the coverage metric) are not precluded

  - Details on LLS including but not limited to impact of different BS antennas to channel reciprocity / BF

- FFS:

  - To evaluate advanced receivers and realistic demodulation performance

  - To evaluate UE-UE CLI mitigation performance

  - To evaluate gNB-gNB CLI mitigation performance

  - To evaluate feasibility and performance of self-IC accounting for realistic non-linearities in the gNB transmit and receive chains

  - Details on LLS including but not limited to impact of different BS antennas to channel reciprocity / BF

According to the below RAN1 agreement from last meeting, RAN1 agrees that link-level simulations (LLS) may be performed for various purposes related to SBFD performance and feasibility in both FR1 and FR2. Interested companies may perform LLS at least for the purposes of evaluating coverage performance. While this purpose is useful and important, we believe that should not be the only purpose of LLS as suggested by the FFS bullets in the agreement. As we have demonstrated in previous sections, additional important purposes are Tx beam nulling, digital-IC, etc. Based on this  we propose the following:

RAN1 further agrees that interested companies may perform link-level simulations (LLS) for the purposes of evaluating SBFD performance and feasibility in both FR1 and FR2 including evaluation of the following:

  - Self-interference suppression/cancellation accounting for realistic non-linearities in the gNB transmit and receive chains.

  - Transmit beam nulling accounting for realistic non-linearities in the gNB transmit chain.

# 3	System Level Evaluation

## 3.1	Scenarios for evaluation

At previous RAN1 meetings, the focus for deployment scenarios was primarily centered on high priority cases, specifically Case 1 (Single Operator), Case 3-2 (Two-layer), and Case 4 (Multi-operator). Based on the agreements reached regarding simulation scenarios, a comprehensive summary table of system-level simulation scenarios can be made as follows

| Deployment Scenarios | No. of Layers | UE clustering | Case 1 |  | Case 4 |  | Case 3-2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Indoor office | 1 |  | o | o |  |  |  |  |
| Urban Macro | 1 | o | o |  | o |  |  |  |
| Dense Urban Macro layer | 1 | o | (o) | o |  | o |  |  |
| Dense Urban Micro layer | 1 |  | (o) | (o) |  |  |  |  |
| Dense Urban with 2-layer | 2 |  |  |  |  |  | o |  |
| Urban Macro + Indoor office/factory | - |  |  |  | o |  | o |  |

|  | FR1 |
| --- | --- |
|  | FR2 |
| o | Agreed |
| (o) | Optional |

Table 7 System level simulation scenarios for NR duplex evolution

It can be observed that there were several cases agreed for evaluations for Case 1 in both FR1 and FR2, although some of them were designated as optional. Most companies provided isolated simulation results for indoor office environments in either FR1 or FR2, while a few companies offered simulation results for Urban Macro and 2-layer scenarios. In real deployment cases, the indoor networks are not isolated, and the Urban Macro networks are not operated by a single operator.

In essence, it may be sufficient to evaluate isolated indoor deployments and single operator outdoor deployments to comprehend the maximum gains attainable through the deployment of SBFD, but to understand the realistic gains attainable by SBFD, two operator scenarios and heterogeneous cases for both FR1 and FR2 needs to be considered. RAN1 to further down-select scenarios where SBFD performance improvements may be realistically possible and can be simulated/evaluated by participating entities.

For instance, the Dense Urban 2-layer network and Urban Macro with indoor office/factory scenarios are similar use cases, except that the Dense Urban Micro layer is located outdoors while the indoor office/hotspot is located indoors. The latter scenario appears to be more practical, as it involves an indoor office or factory with UL dominant traffic coexisting with an Urban Macro network with DL dominant traffic outdoors. Additionally, the indoor network provides adequate isolation from the macro network, thereby reducing interference due to penetration loss. As concluded in the RIM CLI study, an indoor-macro network can avoid performance degradation if the indoor-indoor scenario is carefully laid out and parameterized. This conclusion applies to SBFD cases as well. However, the study also identified significant BS-BS interference in Micro-Micro cases. As a result, the Dense Urban network with 2-layer deployment, which deploys a Macro-Micro with 3 Micro sites, is deemed unsuitable for 2-layer scenario study due to the higher power of Macro BSs and the potential for further interference. Therefore, we think it is better to focus on heterogeneous 2-Layer Scenario B. An exemplary 2-Layer Scenario B is provided in Figure 35.

Figure 35: An exemplary 2-Layer Scenario B

It is important to note that the Inter-Site Distance (ISD) for the Dense Urban Macro layer is 200m, but an agreement has been reached for 2-Layer Scenario B with an ISD of 500m. This distance of 500m is deemed too long to have any significant impact on the indoor system and needs to be aligned with the other Dense Urban 2-layer scenario. As a result of this change, there will be substantial impact of the Urban Macro deployment on the indoor system. Previously, it was agreed to study an isolated indoor office scenario, whereas in this case, the Urban Macro serves to generate interference to the indoor system and this proposal enables it.

RAN1 to further down-select scenarios where SBFD performance improvements may be realistically possible and can be simulated/evaluated by participating entities.

Dense Urban with 2-layer system has an ISD of 200m, the same needs to be used for the HetNet with Urban Macro and Indoor deployment.

RAN1 to agree that for evaluation of SBFD deployment 2-layer Scenario B for Case 3-2, Case 4 and dynamic/flexible TDD in FR1 (HetNet with Urban Macro and Indoor) consider the following.

- Layer 1: Urban Macro

  - Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around, ISD=200m.

# 4	System Level Evaluation Methodology

## 4.1	Performance Metrics

### 4.1.1	Coverage

One of the proposed benefits of the SBFD technology is the enhancement of uplink coverage, particularly in macro-outdoor deployments. To evaluate this benefit, a low percentile user throughput, such as the 5th percentile, can be utilized as a metric. To further quantify the improvement in coverage, a coverage metric is proposed to be derived based on the mean user throughput generated by system simulations as a function of the logarithmic path loss between the gNB and UE. This can be achieved by constructing a scatter plot of mean user throughput and path loss and averaging the values at similar path loss values to obtain a single curve of average user throughput versus path loss. Subsequently, the corresponding coverage metric can be derived by determining the path loss value for which a specified average bitrate (such as 10 Mbps for downlink and 1 Mbps for uplink as specified in Table A.1-1 of 38.830) can be maintained. This coverage metric will be referred to as "10 Mbps Coverage" for downlink and "1 Mbps Coverage" for uplink in FR1 and “25 Mbps Coverage” for DL and “5 Mbps Coverage” for UL in FR2, respectively.

Agreement

RAN1 agrees link-level simulations (LLS) may be performed for various purposes related to SBFD performance and feasibility in both FR1 and FR2, interested companies may perform LLS at least for the following purpose:

- To evaluate coverage performance

  - Option 1: Take link level evaluation methodology in TR 38.830 (i.e., LLS + Link budget analysis) as starting point to evaluate the coverage performance (e.g., MPL, MCL, MIL) for SBFD considering inter-gNB/sector interference and self interference.

  - Other options (e.g. SLS as a tool to obtain the coverage metric) are not precluded

  - Details on LLS including but not limited to impact of different BS antennas to channel reciprocity / BF

- FFS:

  - To evaluate advanced receivers and realistic demodulation performance

  - To evaluate UE-UE CLI mitigation performance

  - To evaluate gNB-gNB CLI mitigation performance

  - To evaluate feasibility and performance of self-IC accounting for realistic non-linearities in the gNB transmit and receive chains

  - Details on LLS including but not limited to impact of different BS antennas to channel reciprocity / BF

Figure 36: An example illustration of how to determine the "X Mbps Coverage" metric. The red curve is a moving average smoothing (using a window of 10% of the samples) of the blue average user throughput versus path loss samples. X Mbps coverage metric is then defined as the path loss value corresponding to the chosen the smoothed user throughput value (e.g. X = 1 Mbps).

In RAN1 #111 meeting, the issue of defining a coverage metric was discussed and multiple proposals were presented. It was ultimately decided that the coverage metric would be derived through link budget analysis (Option 1), although alternative options were not ruled out. However, it is worthy to note that the proposed coverage metric based on system level simulations (Option 2) would provide a more realistic estimation of achievable coverage due to the inclusion of real-time cross-link interference (CLI) and beamforming in the simulation, as opposed to a link budget analysis that only considers path loss and penetration loss. Furthermore, the method for modeling self-interference and other CLI in link level simulations for the purpose of estimating signal-to-interference-and-noise ratio (SINR) is uncertain which requires further discussion in our opinion. Therefore, we propose to include Option 2 as additional method for defining the coverage metric.

A coverage metric based on the pathloss corresponding to a given bit rate is a good metric for system level simulations as it considers realistic beamforming and CLI (Option 2), unlike the MPL obtained from link budget analysis (Option 1 and Option 3).

RAN1 to adopt the proposed methodology for calculating coverage metric as the target path loss corresponding to a certain (smoothed) average bit rate determined from system simulations: 10Mbps for DL and 1Mbps for UL in FR1 and 25 Mbps for DL and 5 Mbps for UL in FR2. This is called “10 Mbps coverage” for DL, “1 Mbps coverage” for UL, etc. (Option 2 in the proposal discussed in RAN1 #110)

## 4.2	Modelling for system level simulations

### 4.2.1	FR1 Receiver model in SLS

In RAN1 #111, an agreement was made regarding inter-site gNB-gNB CLI. This agreement assumes that increased power level of the blocker in the band entails reduced selectivity in the receiver. It is important to consider that the reaction of the receiver to interfering signals has multiple sources but here two components are mentioned; (i) power from the interferer arising due to non-ideal spectral confinement of the receiver bandwidth; this can be improved by, e.g., digital filtering, and is a proportional to the blocker power and (ii) non-linearities in the analogue parts of the receiver producing intermodulation products from the interfering signal into the wanted signal bandwidth. (ii) depends on the total power into the receiver and is manifested as an increase of the inband distortion is interferer power dependent. This means that the ability of the receiver to differentiate between signals of different frequencies and effectively eliminate unwanted signals will be impacted by the total power level of the interfering signal. The power level in the receiver is comprised of power from all interference sources in a SBFD BS and not only co-channel inter-subband CLI. These sources include inter-site, inter-sector, and self-interference, all of which pass through the same radio receiver which covers a wide band including co-channel and adjacent channels. The impact of the interfering signals on the receiver's noise floor is dependent on the total combined power level. When the power level is low, the inter-site and inter-sector interference can be suppressed according to the ACS. Self-interference may also be further suppressed if the downlink and uplink are time-aligned. However, as the power level of the combined interference approaches the blocking level, receiver third-order intermodulation distortions become increasingly significant. When the power is high enough, the interference will rapidly increase, and the noise floor will also rise due to the Automatic Gain Control (AGC) reducing the input gain. If the interference power exceeds the maximum level, the analog-to-digital converter may become saturated.

Moreover, it is imperative to model the non-linearities outlined in section 2 in a realistic manner for system level simulations, as these can induce distortions in the receiver's wanted signal bandwidth, particularly when the AGC reaches its minimum gain, and the receiver enters a cut-off stage.

In RAN1#111 an LS was sent to RAN4 and in RAN4#106 RAN4 provided the following response

Answer from RAN4: From RAN4 perspective, the following model is provided for simulation purpose: RAN4 can confirm RAN1 can assume ICSBS (in channel selectivity) is given by the value of gNB ACS. The noise figure model is provided as below: X-axis: Total received power is the linear sum of all received power, including wanted signal, self-interference, inter-gNB interference and inter-sector interference. Y-axis: noise figure The values of A, B, C and D: A = -43dBm B = -25dBm C = 5dB D = 14dB If the total received power is larger than B, the receiver will be blocked.

In [20], a piecewise linear model for the receiver behaviour is proposed to capture the impact to the receiver noise figure. Similarly, we can model the sensitivity degradation of the signal due to high blocker power including nonlinearities and distortion based on a piecewise linear model as shown below.

Sensitivity degradation (dB)

Total average input power (dBm)

P1

P2

P3

k1

k2

Figure 37: Blocking sensitivity degradation profile model

This proposed model can broadly be divided into three regions:

- Region 1 up to threshold P1: In this region, the receiver AGC is set to its maximum gain. The receiver response is primarily composed of linear components and the distortion is negligible. The output signal power is proportional to the input signal power.

- Region 2 from threshold P1 to threshold P3: In this region, the AGC settings are adapted (lowered from maximum gain) based on total received input signal power, and the receiver response starts to get influenced by noise rise and increasing distortion. Beyond threshold P2, the AGC gain cannot be reduced further and the IIP3 distortion dominates.

- Region 3 from threshold P3: At threshold P3, the receiver is saturated (totally blocked), and there is no meaningful output. The distortion is maximum.

We observe that the agreed RAN4 model follows the same structure, but with only one slope. Judging from the slope in the RAN4 model, it is our understanding that only the slope k1 is model and thus P2 = P3. In Table 8, the RAN4 model parameters together with the model parameter values proposed by us in RAN1#112 for comparison are listed. Note that even if it is not explicitly stated in the LS reply, our understanding is that this model is only applicable for FR1.

Table 8 Proposed piece-wise linear model parameter values for FR1

| Parameter | Value | RAN4 model |
| --- | --- | --- |
| Noise figure (F) | 5 dB | 5 dB |
| Total average input power threshold 1 (P1) | -57.9 dBm | -43 dBm |
| Total average input power threshold 2 (P2) | -41.7 dBm | N/A |
| Sensitivity degradation slope 1 (k1) | 0.4 dB/dBm | 0.5 dB/dBm |
| Sensitivity degradation slope 2 (k2) | 3.0 dB/dBm | N/A |
| Total average input power blocking limit (P3) | -32.5 dBm | -25 dBm |

RAN1 to adopt the calculation and reporting of the statistic Pblocker’, considering the total power at the receiver as derived from the system level simulations.

RAN1 to adopt the piecewise linear model proposed by RAN4 to model the receiver blocking and distortions caused by non-linearities in the receiver for FR1. Send an LS to  RAN4 to confirm the understanding that a model for FR2 will be provided as well.

If 1 dB desense is assumed to model self-interference, then the self-interference power input to the model should be the value assumed to get 1 dB desense.

In addition to the adaptation of the noise floor according to the receiver blocking model above, interference due to ACLR and ACS should be added for the inter-sector and inter-site interference (together with 1 dB of desensitization for the self-interference). This is already agreed to be performed in RAN1.

## 4.3	Differences in RAN1 and RAN4 SLS assumptions

RAN1 and RAN4 are performing system level evaluations for evaluating the performance gains of SBFD. However, the assumptions, are different in certain aspects. RAN4 and RAN1 works pursue different objectives, so it is reasonable to accept certain deviation in the assumptions. However, we consider that these deviations should be motivated in TR38.858, and where possible, alignment should be pursued to facilitate comparability. Regarding these differences we believe there is room for some improvement to get closer. Some of these differences are listed in the following table, together with suggestions for motivating the deviation or proposals for alignment.

Table 9 A few differences in RAN1 and RAN4 SLS assumptions

| Parameters | RAN1 SLS | RAN4 SLS | Comments |
| --- | --- | --- | --- |
| Deployment | Urban Macro: 7 sites with wrapping Indoor: 6 TRPs per operator | Urban Macro: 19 sites with wrapping Indoor: 6 TRPs per operator, total 12 TRPs | For Urban Macro, either option can be assumed. However, considering that, differently from RAN4, RAN1 simulations often include small scale fading, 19 sites may significantly increase simulation time, so that our preference is to keep the agreement in RAN1 |
| Noise Floor | RAN1 assumes a constant -96 dBm noise floor regardless of the BS class, with 5 dB noise figure for all BSs | In RAN4, it is based on the REFSENS for different BS classes. | Better to align with RAN4 approach. Specifically, the noise figures consider for FR1 are: 5dB in UMa BS, 13 dB in InH BS, and 9 dB at the UE. For FR2, 10 dB for BS. |
| SBFD configuration | DUD | DUD and DU | Keep the agreement in RAN1. |

As previously noted, RAN1 assumes a 5 dB noise figure for all BSs while RAN4 assumes different noise figures based on the BS class. It may be beneficial for RAN1 to adopt the RAN4 agreement as it pertains to the derived RSI value based on a 1 dB desense. Additionally, it may be worth considering aligning the gNB transmit power with RAN4 as much as possible.

RAN1 should prioritize the alignment of system-level simulation parameters with RAN4, focusing on parameters that are critical for obtaining reliable conclusions for the TR. Deviations in assumptions should be justifiable and documented in TR 38.858.

RAN1 to agree adopting RAN4's noise figure values for different BS classes.

# 5	System level evaluations

## 5.1	Scenarios for system level simulations

In this contribution, we provide evaluation results for single operator and multi-operator deployment cases in FR1 and single operator cases in FR2.

The implementation of SBFD technology promises improved coverage and reduced latency. However, it comes at the cost of increased complexity as discussed in section 2. In our prior contributions, to gain a deeper understanding of the tradeoff between these benefits and the costs, we proposed exploring an alternative scenario that utilizes a static TDD configuration pattern of DUDDU, which features an additional U slot compared to the standard DDDDU pattern. This choice was motivated by the analysis presented in the table below, which highlights that the UL/DL resource ratio for the reference static TDD system is the not the same as SBFD network in Alt.2. Therefore, we devised a new reference static TDD system using the  proposed static TDD 2UL for fair comparison between SBFD and STDD. The proposed system may provide same benefits as SBFD; improved coverage and reduced latency by leveraging the additional UL resources, especially the second U slot in the TDD configuration. The reduction of guard time from the D slot to the U slot is also done to make the UL/DL resource ratio as close to SBFD in Alt. 2 for a fair comparison.

However, in this contribution we provide results for Alt. 3 which also has two UL slots, and the resource ratio is close to the SBFD network in Alt. 3. In accordance with the agreement for Alt.3 that the UL subband is 25% of the total channel bandwidth, it is noteworthy that a different SBFD configuration than Alt. 2/Alt. 4 is used for calculations.

Table 10	Number of effective DL and UL resources in various cases

| Duplex case |  | Number of effective DL resources | Number of effective UL resources | ULDL resource ratio | UL SubbandChannel BW% | UL resource ratio per TDD period |
| --- | --- | --- | --- | --- | --- | --- |
| Alt. 2 | Static TDD: DDDSU | 3*14+12 = 54 symbols | 14 symbols | 25.93% |  | 20.6% |
|  | SBFD: XXXXU with 104:55:104 resource blocks | 208/273 * (3*14 + 12) = 41.1 symbols | 55/273 * (3*14 + 12) + 14 = 24.9 symbols | 60.58% | 20.14% | 37.72% |
| Static TDD 2UL: DUDDU with switching in U slot |  | 3*14 = 42 symbols | 2*12 = 24 symbols | 57.14% |  | 36.36% |
| Alt. 3 | Static TDD: DDSUU | 2*14+12 = 40 symbols | 2*14 = 28 symbols | 70% |  | 41.17% |
|  | SBFD: XXXXU with 98:67:98 resource blocks | 196/273 * (3*14 + 12) = 38.77 symbols | 67/273 * (3*14 + 12) + 14 = 27.25 symbols | 72 % | 24.5% | 41.28% |
| Alt. 4 | Static TDD: DDDSU | 3*14+12 = 54 symbols | 14 symbols | 25.93% |  | 20.6 % |
|  | SBFD: XXXXX with 104:55:104 resource blocks | 208/273 * (5*14) = 53.3 symbols | 55/273 * (5*14)= 14.1 symbols | 26.45% | 20.14% | 20.92 % |
| Alt. 1 | Static TDD: DDDSU | 3*14+12 = 54 symbols | 14 symbols | 25.93% |  | 20.6% |
|  | SBFD: DXXXU with 104:55:104 resource blocks | 208/273 * (2*14 + 2*12) = 39.62 symbols | 55/273 * (2*14 + 12) + 14 = 22.06 symbols | 55.68% | 20.14% | 35.76% |

It is noteworthy that SBFD in Alt. 2/Alt. 1 have more UL resources than the reference static TDD assumed for comparison. Thus, any potential gains in UL for SBFD in Alt.2 need to consider that it might be due to the increased UL resources available for Alt. 2/Alt. 1. Alt.3 and Alt.4, on the other hand have similar UL resources for the reference static TDD network and the SBFD network.

Alt. 1 and Alt.2 SBFD configurations have more UL resources than the reference static TDD pattern it is compared with. Any potential gains in UL for these alternatives need to consider this impact first before drawing conclusions.

In addition to the above, to enhance clarity and scientific rigor, we have included two sets of simulation results, labeled as “Realistic " and "“Optimistic” assumptions" for FR1 Urban Macro scenario. The former is based on the feasibility study in section 2.2 for WA BS, which determines realistic and achievable numbers for self-interference and inter-sector suppression levels. The latter assumes the best-case scenario for inter-sector isolation and follows RAN1's agreement of 1 dB desensitization for self-interference isolation levels. These simulation results aim to provide reliable and relevant conclusions to guide future discussions and decisions in RAN1.

It is important to note that the terms “Realistic" and “Optimistic” do not necessarily indicate whether the assumptions are feasible or not. The feasibility of the assumptions depends on the specific deployment scenario and the level of uncertainty associated with the parameters being assumed which requires a feasibility study in RAN4.

Furthermore, simulations are also done for SBFD antenna configuration Option 2 , and Option 3 denoted as ‘Same gain’ and ‘Same area’ in the plots, respectively. “Realistic” and “Optimistic” assumptions are denoted as ‘Real’ and ‘Opti’ in the plots, respectively.

Table 11	Interference suppression assumptions for system level simulations

| Frequency range | Type | Self-interference suppression level | Tx beam nulling | Inter-sector interference suppression level | Receiver model |  |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 Urban Macro | Realistic assumptions | 70 dB (Antenna isolation) | 10 dB | 75 dB (Spatial isolation) | RAN4 model |  |
|  | Optimistic assumptions | RAN1 agreement for self-interference suppression based on 1 dB desensitization |  | 93 dB (Spatial isolation) | RAN4 model |  |
| FR2 Dense Urban | Realistic assumptions | 80 dB (Antenna isolation) | 10 dB | 75 dB (Spatial isolation) |  | No model used |
|  | Optimistic assumptions | RAN1 agreement for self-interference suppression based on 1 dB desensitization |  | 98 dB (Spatial isolation) |  | No model used |

SBFD antenna configuration Option 2, which has double the antenna elements for SBFD when compared to reference static TDD, is the best-case scenario for SBFD.

SBFD antenna configuration Option 3, which has same number of antenna elements for SBFD when compared to reference static TDD, and only half the TxRUs can be used realistically, it is the practical case for SBFD.

The simulation results obtained from the “Realistic” assumptions can be considered as a more realistic estimation of the performance of SBFD in real-world scenarios, while the results from the “Optimistic” assumptions reflect the best-case scenario for SBFD’s potential performance gains.

Considering the above discussion, we will present simulation results in FR1 for Static TDD with different TDD configurations and SBFD in two different deployment scenarios: Indoor and Urban Macro. We also present FR2 simulation results in Dense Urban Macro, additionally. The table below provides a detailed list of simulation results with different options that is presented in this contribution.

Table 12	Deployment Scenarios considered for system level simulations in single operator and multi-operator cases for FR1 and FR2-1

| Deployment Scenarios | Case 1 | Case 4 |
| --- | --- | --- |
| Indoor office | - FR1 only - Large and short packet size - Realistic assumptions only - SBFD Same gain (Option 2) and SBFD Same area (Option 3) - Network 1 (N1): - SBFD Alt.2 - SBFD Alt.4 - SBFD Alt.3 - DTDD with DL/UL dominant static TDD pattern | - Not applicable |
| Urban Macro | - FR1 only - Large packet size - Realistic and Optimistic assumptions - SBFD Same gain (Option 2) and SBFD Same area (Option 3) - Network 1 (N1): - Baseline static TDD DDDDU - SBFD Alt.2 - SBFD Alt.4 - SBFD Alt.3 - DTDD with DL/UL dominant static TDD pattern | - FR1 only - Grid Shift 0% and 100% - Large packet size - Realistic assumptions only - SBFD Same gain (Option 2) only - N1: Static TDD DDDDU - N2: - Baseline static TDD DDDDU - SBFD Alt.2 - SBFD Alt.4 - SBFD Alt.3 - DTDD with DL/UL dominant static TDD pattern - Static TDD Alt. 3 DDDUU |
| Dense Urban Macro layer | - FR2-1 only - Large packet size - Realistic and Optimistic assumptions - SBFD Same gain (Option 2) and SBFD Same area (Option 3) - Network 1 (N1): - Baseline static TDD DDDDU - SBFD Alt.2 - SBFD Alt.3 - DTDD with DL/UL dominant static TDD pattern |  |

It is noted that the DL/UL dominant static TDD pattern for DTDD network simulated is as follows:

- DL dominant static TDD UL/DL configuration: {DDDSU}, where S=[12D:2G:0U]

- UL dominant static TDD UL/DL configuration: {SUUUU}, where S=[12D:2G:0U]

This is different from the RAN1 agreement for UL-dominant static TDD pattern where DSUUU was agreed. In this proposed format we provide extra U slot to the UL dominant static TDD UL/ DL configuration. It is worthy to note that, the total resources for DL or UL is not fixed for DTDD as it changes depending on the TDD pattern chosen. Therefore, it may not be straightforward to calculate Type-2 RU for DTDD network, so we plot Type-1 RU for DTDD networks in all the simulations within the same Type-2 RU figure for SBFD networks.

## 5.2	System level evaluation in FR1

For evaluation of SBFD in single operator networks, we conducted simulations in Urban Macro and Indoor deployments in FR1. For two operator networks, we conducted simulations in Urban Macro deployment.

For the SBFD networks, we assume both Option 2 and Option 3 from the agreement on antenna modelling in which the number of antenna elements for SBFD network is twice that of the Static TDD network and same as that of the static TDD network, respectively. In addition, we simulated three different alternatives of SBFD configurations: Alt. 2 (XXXXU), Alt. 4 (XXXXX) and Alt.3 (XXXXU with 25% UL subband ratio).

### 5.2.1	Case 1: Single Operator Urban Macro

#### 5.2.1.1	Alt.2 (Static TDD{DDDSU}, SBFD{XXXXU})

##### 5.2.1.1.1	Resource utilization per direction

Figure 38: Comparison of Type-2 RU in DL (top) and UL (bottom) at low, medium, and high loads for single operator Urban Macro in FR1 deploying static TDD{DDDSU}, SBFD{XXXXU} (Alt. 2) and Type-1 RU for DTDD networks.

In the above figure, resource utilization per direction (UL, DL) , Type-2 RU as agreed in RAN1 is plotted for the three scenarios with different assumptions is shown. Three example network traffic load points are chosen based on the reference static TDD system.

The resource utilization per direction plots provides valuable insight into the behavior of various networks. Specifically, for SBFD networks, it is noted that the mean DL utilization of SBFD networks exceeds that of the static TDD network for all load points in DL, regardless of the SBFD network antenna elements nor the realistic or optimistic assumptions. Conversely, the mean utilization for UL in the SBFD networks has decreased in comparison to the reference static TDD network. As mentioned previously, this is due to an increase in the number of resources available for UL in the SBFD network deploying Alt. 2, in addition to that U slot aligned with the reference static TDD network. The trade-off of resources from DL to UL in SBFD network provides additional UL transmission opportunities, which is a defining characteristic of these Alt. 2 network configurations.

On the other hand, for DTDD network with DL/UL dominant TDD patterns, the DL utilization is usually higher than its own UL utilization. This is due to the split in traffic ratio between DL and UL, where the former is more. Furthermore, the utilization for both DL and UL is usually lower than the reference static TDD network for all loads. This is because the total resources for DL or UL is not fixed for DTDD as it changes depending on the TDD pattern.

##### 5.2.1.1.2	Coverage

Figure 39: Comparison of “10 Mbps Coverage” in DL (left) and “1 Mbps Coverage” in UL (right) at low, medium, and high loads for single operator Urban Macro in FR1 deploying static TDD{DDDSU}, SBFD{XXXXU} (Alt. 2) and DTDD networks.

In the figure above, the coverage metric is based on the “10 Mbps Coverage” and “1 Mbps Coverage” for DL and UL, respectively, which are indicative of larger MPL leading to better coverage. At all load levels, there does not seem to a significant difference in performance between “realistic” and “optimistic” assumptions, although there is a slight increase for UL coverage at medium and high loads.

At low loads, the SBFD networks exhibit similar DL coverage as the static TDD network, due to the same output powers and beamforming gains, and due to the fact the SBFD network does not suffer from high cross link interference (CLI). However, at medium and high loads, the SBFD network slightly reduces its coverage performance due to the elevated network interference, particularly intra-network co-channel CLI. Furthermore, there is not a significant loss in DL coverage at low loads for SBFD network with same antenna elements as Static TDD network. However, this loss increases as the load increases as the SBFD network with “Same Area” uses half the number of antenna elements for DL than static TDD in the SBFD slot.

Regarding UL coverage gains, there are marginal gains of ~3dB in UL coverage at low loads and almost no gains at medium and high loads for SBFD networks over static TDD network. The UL coverage of SBFD network with same antenna area as static TDD also diminishes as the load increases. This is evident due to the fact that only half the number of antenna elements are usable by the SBFD network.

Whereas, the DTDD network provides similar coverage in DL and UL for all loads as the Static TDD network.

##### 5.2.1.1.3	User Throughput

Figure 40: Comparison of 5%ile, 50%ile and 95%ile Average- UPT for DL (top) and UL (bottom) at low, medium, and high loads for single operator Urban Macro in FR1 deploying static TDD{DDDSU}, SBFD{XXXXU} (Alt. 2) and DTDD networks.

The user throughput plots shown in Figure 40, allow to make several observations. Firstly, the SBFD XXXXU networks have lower DL cell-edge, median, and 95%ile throughputs than a static TDD network across all load points. This is due to the reduced DL resources in the SBFD networks in comparison to the reference static TDD network, hindering their ability to achieve high DL user throughputs at low loads. At medium and high loads, the SBFD networks suffer from higher network interference as the network resource utilization levels increase and diminishes DL performance. For example, the cell-edge DL throughput of a SBFD network with double-sized antenna is 65% lower than the reference static TDD network and it increases to about 84% for SBFD network with same antenna size as a static TDD network.

A SBFD network with double-sized antenna has more DL cell-edge, median and 95%ile throughputs than a SBFD network with same antenna size as Static TDD network. Similar to coverage enhancements, there is no significant difference in performance of SBFD networks with “realistic” and “optimistic” assumptions.

An SBFD network XXXXU with double-sized antenna provides better UL median and cell-edge user throughputs than a reference static TDD network, particularly at low loads. For instance, the median UL user throughput increases by ~72%, and the cell-edge user throughput increases by up to ~218% at low loads. However, these gains decrease with increased traffic loads, and at high loads, the UL median and cell-edge user throughputs are like the reference static TDD network, meaning that the interference in the UL subband is so high that they can hardly serve any throughput. Assuming “Optimistic” assumptions improves the UL cell-edge, median and 95%ile throughput for SBFD XXXXU network but only slightly.

DTDD network also performs as well as an SBFD network with same area at low loads, but the gains diminish as the load increases owing to the inter-network CLI.

##### 5.2.1.1.4	Latency

Figure 41 displays the latency results, which follow similar trends as the user throughput.

In DL, the worst-case latency (95%ile latency) increases with load and SBFD antenna configuration and is generally more for SBFD networks. The optimistic assumptions allow better isolation between Tx and Rx in the SBFD gNBs, so this enables more UL to be transmitted in the SBFD slots at high loads than the SBFD network with realistic assumptions. Therefore, one can note that the optimistic case DL latency is higher than the realistic for medium and high loads. Furthermore, the latency of an SBFD network with double-sized antenna is smaller than the latency of the SBFD network with same antenna size as a static TDD network.

Figure 41: Comparison of 5%ile, 50%ile and 95%ile latency for DL (top) and UL (bottom) at low, medium, and high loads for single operator Urban Macro in FR1 deploying static TDD{DDDSU}, SBFD{XXXXU} (Alt. 2) and DTDD networks.

In UL, SBFD XXXXU offers comparable performance to STDD for all load levels, with optimistic assumptions resulting in slightly reduced latency compared to STDD. There is a latency reduction of about 37% at low loads for the best-case latency (5%ile) situation. The SBFD network, when operating under realistic assumptions, exhibits high latencies even in the best-case and median latency scenarios, because of the considerable interference in the UL subbands for all slots as the load increases. However, the situation improves significantly when the network has better isolation, which is reflected in the substantial reduction in latency observed under optimistic assumptions.

#### 5.2.1.2	Alt.4 (Static TDD{DDDSU}, SBFD{XXXXX})

##### 5.2.1.2.1	Resource utilization per direction

Figure 42: Comparison of Type-2 RU in DL (top) and UL (bottom) at low, medium, and high loads for single operator Urban Macro in FR1 deploying static TDD{DDDSU}, SBFD{XXXXX} (Alt. 4) and Type-1 RU for DTDD networks.

For the SBFD XXXXX networks , DL utilization is almost same as the reference static TDD network at low and medium but increases at high loads. For SBFD network with “Realistic” assumptions, UL utilization is significantly reduced due to the increase in inter-sector interference and self-interference resulting from assuming realistic isolation values at high loads, and slightly reduced at low and medium loads. Conversely, the SBFD XXXXX network with “Optimistic” assumptions shows a noticeable increase in UL utilization at medium and high loads compared to other networks.

##### 5.2.1.2.2	Coverage

Figure 43: Comparison of “10 Mbps Coverage” in DL (left) and “1 Mbps Coverage” in UL (right) at low, medium, and high loads for single operator Urban Macro in FR1 deploying static TDD{DDDSU}, SBFD{XXXXX} (Alt. 4) and DTDD networks.

For an SBFD network with all SBFD slots (XXXXX), it exhibits similar DL coverage as the static TDD system across all loads. This is because of having similar DL resources in the reference system and the SBFD network. UL coverage gains are marginal about 3 dB at low loads, but quickly diminish as the load increases and at high loads, an SBFD XXXXX network with realistic assumptions, is not able to provide any UL coverage at 1 Mbps target data rate.

##### 5.2.1.2.3	User Throughput

Figure 44: Comparison of 5%ile, 50%ile and 95%ile Average- UPT for DL (top) and UL (bottom) at low, medium, and high loads for single operator Urban Macro in FR1 deploying static TDD{DDDSU}, SBFD{XXXXX} (Alt. 4) and DTDD networks.

Similar to coverage improvements, there are improvements in UL at low loads for the cell-edge and median throughput, but it quickly diminishes as the load increases and at high loads, there is hardly any cell-edge throughput for realistic assumptions of SBFD networks. This is because, at high loads, all the SBFD slots are heavily interfered and is unusable for UL transmissions due to gNB-gNB CLI.

##### 5.2.1.2.4	Latency

Figure 45: Comparison of 5%ile, 50%ile and 95%ile latency for DL (top) and UL (bottom) at low, medium, and high loads for single operator Urban Macro in FR1 deploying static TDD{DDDSU}, SBFD{XXXXX} (Alt. 4) and DTDD networks.

#### 5.2.1.3	Alt.3 (Static TDD{DDSUU}, SBFD{XXXXU})

Alt. 3 is an important use case to study, as this allows same resources ratio for UL and DL , between reference static TDD and SBFD, when compared to Alt. 2.

##### 5.2.1.3.1	Resource utilization per direction

Figure 46: Comparison of Type-2 RU in DL (top) and UL (bottom) at low, medium, and high loads for single operator Urban Macro in FR1 deploying static TDD{DDSUU}, SBFD{XXXXU} (Alt. 3) and Type-1 RU for DTDD networks.

##### 5.2.1.3.2	Coverage

Figure 47: Comparison of “10 Mbps Coverage” in DL (left) and “1 Mbps Coverage” in UL (right) at low, medium, and high loads for single operator Urban Macro in FR1 deploying static TDD{DDDSU}, SBFD{XXXXU} (Alt. 3) and DTDD networks.

The behavior of Coverage in Alt3 SBFD networks is similar to the Alt 4 case in single operator, as the resources are similar between the reference static TDD and SBFD networks.

##### 5.2.1.3.3	User Throughput

Figure 48: Comparison of 5%ile, 50%ile and 95%ile Average- UPT for DL (top) and UL (bottom) at low, medium, and high loads for single operator Urban Macro in FR1 deploying static TDD{DDSUU}, SBFD{XXXXU} (Alt. 3) and DTDD networks.

##### 5.2.1.3.4	Latency

Figure 49: Comparison of 5%ile, 50%ile and 95%ile latency for DL (top) and UL (bottom) at low, medium, and high loads for single operator Urban Macro in FR1 deploying static TDD{DDSUU}, SBFD{XXXU} (Alt. 3) and DTDD networks.

For single operator Urban Macro scenario in FR1, UL performance gains of SBFD network in terms of coverage, latency and cell-edge user throughputs decrease as the load in the network increases.

For single operator Urban Macro scenario in FR1, the proposed DTDD network provides comparable performance as an SBFD network in terms of coverage, latency, and cell-edge user throughput in both DL and UL without having to deal with the hardware-complexity of SBFD network.

For single operator Urban Macro scenario in FR1, the UL performance gains for an SBFD network with all SBFD slots (XXXXX) in cell-edge user throughput is on par with SBFD with XXXXU at low loads but the performance drops considerably at medium and high loads due to significant interference limitations in all the SBFD slots. Further, even with optimistic assumptions for self-interference and inter-sector suppression, there is no noteworthy improvement in UL performance of an SBFD XXXXX network.

### 5.2.2	Case 4: Two Operator Urban Macro

Based on the results of our system level simulations for single operator scenarios, we have concluded that an SBFD network equipped with double-sized antennas and an SBFD configuration of XXXXU provides better UL coverage and significantly higher UL cell-edge user throughputs at low traffic loads compared to a static TDD network. In real-world scenarios, there are typically multiple operators sharing the same frequency band which may impact the performance gains obtained at low loads for single operator scenario. Furthermore, while the use of an “Optimistic” set of assumptions resulted in a slight improvement in SBFD network performance, the assumption of 1 dB of desense due to self-interference is likely to be challenging in practice for wide area BS and requires further study in RAN4. Therefore, in the two operator simulations for Urban Macro we assume “Realistic” assumptions that considers real-world scenarios. In addition, we consider both 0% and 100% grid shift for two operator scenarios. It should be noted that a 100% grid shift scenario may be considered overly optimistic as it is common for operators to share the same mast. As a result, a 100% grid shift scenario could be considered favorable to SBFD since there won't be interference challenges arising from co-sited inter-operator interference.

In addition, we observed that an SBFD network with SBFD configuration XXXXX performs very poorly in terms of UL cell-edge throughout and coverage at medium and high loads. This is primarily due to higher network interference at high loads leading to reduced performance in UL. Therefore, it may be more beneficial to explore other SBFD configurations like XXXXU in real-world deployment scenarios when compared to a reference static TDD pattern of DDDDU.

Therefore, it is necessary to assess the real-world performance gain of SBFD by conducting system level simulations of two-operator scenarios (Case 4) using “Realistic" assumptions to determine if there are any gains for SBFD. In the simulations, operator 1 is always a legacy Static TDD network with DDDDU pattern, while operator 2 contains a baseline reference static TDD configuration whose performance is compared with different flavors of SBFD and DTDD networks.

It should be noted that only the performance metrics for operator 2 is shown in the plots. This gives a good comparison of the performance gains of various SBFD networks in comparison with a legacy static TDD network. It is also worthy to note that the traffic loading is assumed symmetrical in both operators; this means that both operators have low, medium, and high loads at the same time in the simulation. In real-world application, the traffic loading of one operator is unknown to the other operator, therefore, this case could be considered favorable to SBFD.

#### 5.2.2.1	Alt.2 (Network 1: Static TDD{DDDSU}, Network 2: SBFD{XXXXU})

##### 5.2.2.1.1	Resource utilization per direction

Figure 50: Comparison of Type-2 RU in DL (top) and UL(bottom) for Network 2 deploying static TDD{DDDSU}, SBFD{XXXXU} (Alt. 2) and Type-1 RU for DTDD coexisting with Network 1 in FR1 deploying baseline static TDD at low, medium, and high loads in both networks with 0% grid shift.

Figure 51: Comparison of Type-2 RU in DL (top) and UL(bottom) for Network 2 deploying static TDD{DDDSU}, SBFD{XXXXU} (Alt. 2) and Type-1 RU for DTDD, coexisting with Network 1 in FR1 deploying baseline static TDD at low, medium, and high loads in both networks with 100% grid shift.

The RU behavior is quite like the single operator case where there is an increase in DL utilization for SBFD networks and reduction in UL utilization when compared to static TDD network in both grid shifts.

##### 5.2.2.1.2	Coverage

Figure 52: Comparison of “DL 10 Mbps Coverage” (left) and “UL 1 Mbps Coverage” (right) for Network 2 deploying static TDD{DDDSU}, SBFD{XXXXU} (Alt. 2) or DTDD coexisting with Network 1 deploying baseline static TDD at low, medium, and high loads in both networks with 0% grid shift.

Figure 53: Comparison of “10 Mbps Coverage” for DL(left) and “1 Mbps Coverage” (right) for UL Network 2 deploying static TDD{DDDSU}, SBFD{XXXXU} (Alt. 2) or DTDD coexisting with Network 1 deploying baseline static TDD at low, medium, and high loads in both networks with 100% grid shift.

In the figure, we compare the DL and UL coverage of an SBFD network with a static TDD network at low, medium, and high traffic loads. It is evident that both networks provide similar DL coverage at low loads since they are equipped with equal output powers and beamforming gains. However, as the load increases, the DL coverage slightly diminishes for both networks. In UL, the SBFD same gain network provides slightly increased coverage at low loads but provides similar coverage as the static TDD network for medium and high loads. This is because the UL-only slot in XXXXU is used while the SBFD slots are affected by inter-network interference, which increases as the load increases. It is essential to evaluate the coverage improvements in conjunction with throughput improvements, which will be discussed in the following sub-section.

##### 5.2.2.1.3	User Throughput

The 5%ile, 50%ile and 95%ile  Average- UPT for DL and UL are plotted for all the cases in the following figure.

Figure 54: Comparison of 5%ile, 50%ile and 95%ile Average- UPT for DL (top) and UL (bottom) for Network 2 deploying static TDD{DDDSU}, SBFD{XXXXU} (Alt. 2) or DTDD coexisting with Network 1 deploying baseline static TDD at low, medium, and high loads in both networks with 0% grid shift.

Figure 55: Comparison of 5%ile, 50%ile and 95%ile Average- UPT for DL (top) and UL (bottom) for Network 2 deploying static TDD{DDDSU}, SBFD{XXXXU} (Alt. 2) or DTDD coexisting with Network 1 deploying baseline static TDD at low, medium, and high loads in both networks with 100% grid shift.

Similar as for the single operator case, the SBFD have lower 5%tile, 50%ile and 95%ile DL throughput compared to the static TDD baseline. This is because the amount of DL resources of the SBFD networks have decreased in comparison to the reference static TDD network (refer Table 10), thus preventing them from achieving high DL user throughputs at low load. At high loads, the cell-edge DL throughput is quite low compared to the baseline static TDD network for both grid shifts.

For UL cell-edge throughput, the conclusions from single operator case still holds when the load in the networks are low for 100% grid shift. For high and medium loads, the UL performance in terms of cell-edge throughput gains compared to static TDD for both the SBFD networks diminish. This is due to the additional CLI from the neighboring static TDD network on the SBFD slot. It should be noted that the performance of the SBFD networks is lower bounded by that of a static TDD network, because in case the SBFD slots are heavily interfered, all UL traffic can be scheduled on the UL slot at the end. The overall performance of 100% grid shift is only slightly better than 0% grid shift for cell-edge throughput. Median and 95%ile throughput is more for 100% grid shift as it does not have to deal with interference from co-sited BSs.

##### 5.2.2.1.4	Latency

Figure 56: Comparison of 5%ile, 50%ile and 95%ile latency for DL (top) and UL (bottom) for Network 2 deploying static TDD{DDDSU}, SBFD{XXXXU} (Alt. 2) or DTDD coexisting with Network 1 deploying baseline static TDD at low, medium, and high loads in both networks with 0% grid shift.

Figure 57: Comparison of 5%ile, 50%ile and 95%ile latency for DL (top) and UL (bottom) for Network 2 deploying static TDD{DDDSU}, SBFD{XXXXU} (Alt. 2) or DTDD coexisting with Network 1 deploying baseline static TDD at low, medium, and high loads in both networks with 100% grid shift.

The figure above provides an overview of DL and UL latency in various networks. The results show that for DL, SBFD XXXXU networks have slightly higher worst-case latency compared to the reference static TDD network for low loads, and significantly higher latency for medium and high loads, especially for same area SBFD network. This can be attributed to increased DL utilization at higher loads. Additionally, the best-case latency for these networks is also worse than the reference static TDD network, but only by a small margin (1-3ms) at low and medium loads, but increases to a big margin at high loads.

In terms of UL latency, SBFD XXXXU networks have worst-case latencies comparable to the reference static TDD network at high loads, and better at low and medium loads. Additionally, the best-case latency for these networks is also better than the reference static TDD network for all loads.

#### 5.2.2.2	Alt.4 (Network 1: Static TDD{DDDSU}, Network 2: SBFD{XXXXX})

##### 5.2.2.2.1	Resource utilization per direction

Figure 58: Comparison of Type-2 RU in DL (top) and UL(bottom) for Network 2 deploying static TDD{DDDSU}, SBFD{XXXXX} (Alt. 4) and Type-1 RU for DTDD, coexisting with Network 1 in FR1 deploying baseline static TDD at low, medium and high loads in both networks with 0% grid shift.

Figure 59: Comparison of Type-2 RU in DL (top) and UL(bottom) for Network 2 deploying static TDD{DDDSU}, SBFD{XXXXX} (Alt. 4) and Type-1 RU for DTDD, coexisting with Network 1 in FR1 deploying baseline static TDD at low, medium and high loads in both networks with 100% grid shift.

The RU plots for SBFD networks with XXXXX configuration is similar to the reference static TDD network in DL for all loads and considerably lower in UL at high loads. This is because UL at high loads need to be served in the SBFD slots, but these slots are heavily interfered at high loads.

##### 5.2.2.2.2	Coverage

Figure 60: Comparison of “10 Mbps Coverage” for DL(left) and “1 Mbps Coverage” (right) for UL Network 2 deploying static TDD{DDDSU}, SBFD{XXXXX} (Alt. 4) or DTDD coexisting with Network 1 deploying baseline static TDD at low, medium and high loads in both networks with 0% grid shift.

Figure 61: Comparison of “10 Mbps Coverage” for DL(left) and “1 Mbps Coverage” (right) for UL Network 2 deploying static TDD{DDDSU}, SBFD{XXXXX} (Alt. 4) or DTDD coexisting with Network 1 deploying baseline static TDD at low, medium and high loads in both networks with 100% grid shift.

Similar to single operator case, the UL coverage drops considerably at high loads for SBFD networks when compared with reference static TDD network.

##### 5.2.2.2.3	User Throughput

In the following analysis, the 5%ile, 50%ile and 95%ile Average-UPT  plot for DL and UL are plotted for all the cases in the following figure.

Figure 62: Comparison of 5%ile, 50%ile and 95%ile Average- UPT for DL (top) and UL (bottom) for Network 2 deploying static TDD{DDDSU}, SBFD{XXXXX} (Alt. 4) or DTDD coexisting with Network 1 deploying baseline static TDD at low, medium and high loads in both networks with 0% grid shift.

Figure 63: Comparison of 5%ile, 50%ile and 95%ile Average- UPT for DL (top) and UL (bottom) for Network 2 deploying static TDD{DDDSU}, SBFD{XXXXX} (Alt. 4) or DTDD coexisting with Network 1 deploying baseline static TDD at low, medium and high loads in both networks with 100% grid shift.

The cell-edge, median and 95%ile Average-UPT in DL for SBFD network XXXXX is quite like the reference static TDD at all loads. This is because, they have the same number of resources in both the systems. However, for UL, there is gains for cell-edge, median and 95%ile Average-UPT at low loads but the gains vanish quickly as the load increases, for reasons mentioned before.

##### 5.2.2.2.4	Latency

Figure 64: Comparison of 5%ile, 50%ile and 95%ile latency for DL (top) and UL (bottom) for Network 2 deploying static TDD{DDDSU}, SBFD{XXXXX} (Alt. 4) or DTDD coexisting with Network 1 deploying baseline static TDD at low, medium and high loads in both networks with 0% grid shift.

Figure 65: Comparison of 5%ile, 50%ile and 95%ile latency for DL (top) and UL (bottom) for Network 2 deploying static TDD{DDDSU}, SBFD{XXXXX} (Alt. 4) or DTDD coexisting with Network 1 deploying baseline static TDD at low, medium and high loads in both networks with 100% grid shift.

The performance of latency is also similar to the throughput performance explained in the previous sub-section.

#### 5.2.2.3	Alt.3 (Network 1: Static TDD{DDDSU}, Network 2: SBFD{XXXXU} / Static TDD{DDSUU})

##### 5.2.2.3.1	Resource utilization per direction

Figure 66: Comparison of Type-2 RU in DL (top) and UL(bottom) for Network 2 deploying static TDD, static TDD{DDSUU} (Alt. 3), SBFD{XXXXU} (Alt. 3) and Type-1 RU for DTDD, coexisting with Network 1 in FR1 deploying baseline static TDD at low, medium and high loads in both networks with 0% grid shift.

Figure 67: Comparison of Type-2 RU in DL (top) and UL(bottom) for Network 2 deploying static TDD, static TDD{DDSUU} (Alt. 3), SBFD{XXXXU} (Alt. 3) and Type-1 RU for DTDD, coexisting with Network 1 in FR1 deploying baseline static TDD at low, medium and high loads in both networks with 100% grid shift.

In the above figure, resource utilization per direction (UL, DL) is plotted. First, for all load points, SBFD and static TDD Alt.3 have higher mean DL utilization than a static TDD network coexisting with a static TDD network for both 0% and 100% grid shift. Static TDD Alt.3 has almost the same mean resource utilization in DL as a same gain SBFD network. On the other hand, the mean utilization for UL has decreased for SBFD network and Static TDD Alt. 3 network in comparison with reference static TDD network.

##### 5.2.2.3.2	Coverage

Figure 68: Comparison of “10 Mbps Coverage” for DL(left) and “1 Mbps Coverage” (right) for UL Network 2 deploying static TDD, static TDD{DDSUU} (Alt. 3), SBFD{XXXXU} (Alt. 3) or DTDD coexisting with Network 1 deploying baseline static TDD at low, medium and high loads in both networks with 0% grid shift.

Figure 69: Comparison of “10 Mbps Coverage” for DL(left) and “1 Mbps Coverage” (right) for UL Network 2 deploying static TDD, static TDD{DDSUU} (Alt. 3), SBFD{XXXXU} (Alt. 3) or DTDD coexisting with Network 1 deploying baseline static TDD at low, medium and high loads in both networks with 100% grid shift.

##### 5.2.2.3.3	User Throughput

In the following analysis, the 5%ile, 50%ile and 95%ile user throughput plot for DL and UL are plotted for all the cases in the following figure.

Figure 70: Comparison of 5%ile, 50%ile and 95%ile Average- UPT for DL (top) and UL (bottom) for Network 2 deploying static TDD, static TDD{DDSUU} (Alt. 3), SBFD{XXXXU} (Alt. 3) or DTDD coexisting with Network 1 deploying baseline static TDD at low, medium and high loads in both networks with 0% grid shift.

Figure 71: Comparison of 5%ile, 50%ile and 95%ile Average- UPT for DL (top) and UL (bottom) for Network 2 deploying static TDD, static TDD{DDSUU} (Alt. 3), SBFD{XXXXU} (Alt. 3) or DTDD coexisting with Network 1 deploying baseline static TDD at low, medium and high loads in both networks with 100% grid shift.

##### 5.2.2.3.4	Latency

Figure 72: Comparison of 5%ile, 50%ile and 95%ile latency for DL (top) and UL (bottom) for Network 2 deploying static TDD, static TDD{DDSUU} (Alt. 3), SBFD{XXXXU} (Alt. 3) or DTDD coexisting with Network 1 deploying baseline static TDD at low, medium and high loads in both networks with 0% grid shift.

Figure 73: Comparison of 5%ile, 50%ile and 95%ile latency for DL (top) and UL (bottom) for Network 2 deploying static TDD, static TDD{DDSUU} (Alt. 3), SBFD{XXXXU} (Alt. 3) or DTDD coexisting with Network 1 deploying baseline static TDD at low, medium and high loads in both networks with 100% grid shift.

For two operator scenarios in FR1, the conclusions on performance gains of SBFD networks are similar to the conclusions from single operator analysis.

### 5.2.3	Case 1: Single Operator Indoor Office

We conducted system simulation for indoor scenario to evaluate system performance for the three different Alternatives, Alt. 2, 3, 4 as described in Table 12. Furthermore, for each Alt. we simulate two sets of schemes based on big packet sizes and small packet sizes which are shown side-by-side to show performance gains of SBFD due to the packet lengths (Big packet sizes: 0.5Mbyte for DL and 0.125Mbytes for UL, Small packet sizes: 4Kbytes for DL and 1Kbyte for UL). For small packet sizes of Alt.2, with focus on UL latency improvement, we also simulate static TDD with 2 UL slots where one UL slot is in earlier position (i.e., DUDDU) whose UL resource amount is almost the same with UL resource amount of Alt.2 SBFD for fair comparison.

In the simulation, data latency is measured from user data arrival at the transmitter and decoding finish at the receiver, accounting for scheduling delay, encoding delay, transmission time and decoding delay. For UL transmission, delays due to SR and UL grant transmission are also considered. Infinite control channel resource is assumed to avoid DCI and SR resource contention.

There are currently two pending issues related to assumptions for indoor use case, namely whether to use “Realistic" or “Optimistic” assumptions. The “Optimistic” assumptions, which are based on a RAN1 agreement of using 1 dB dense for self-interference suppression, may not be accurate for indoor case since this agreement assumes a fixed noise figure of 5 dB for all BS classes. Additionally, indoor deployments do not require sectorization, which eliminates the need for inter-sector isolation. As a result, more realistic values for spatial isolation are assumed and receiver blocking is enabled using the proposed model for the indoor simulations. It is noteworthy that the "optimistic" scenario of 1 dB desense for indoor corresponds to approximately 120 dB suppression. However, the "realistic" assumptions we make assume 75 dB antenna isolation including 5 dB transmission beam nulling, and 42 dB adjacent channel interference rejection (ACIR) component, which already satisfies the 1 dB desense requirement and beyond.

#### 5.2.3.1 	Alt.2 (Static TDD{DDDSU}, SBFD{XXXXU})

##### 5.2.3.1.1	Resource Utilization

.

Figure 74: Comparison of Type-2 RU in DL(top) and UL (bottom) at low, medium, and high loads for Indoor Network in FR1 deploying static TDD{DDDSU}, SBFD{XXXXU} (Alt. 2) and Type-1 RU for DTDD for big packet sizes.

Similar to the results for Urban Macro, the resource utilization for SBFD networks increase for DL compared to the reference static TDD network, especially for medium and high loads due to the lack of resources for DL at those loads. On the contrary, the RU for SBFD network in UL is lower than reference static TDD. The RUs for DTDD are generally lower for DTDD since we plot type-1 RU for DTDD. The trends are similar for big and small packet sizes.

Figure 75: Comparison of Type-2 RU in DL(top) and UL (bottom) at low, medium, and high loads for Indoor Network in FR1 deploying static TDD{DDDSU}, static TDD 2UL{DUDDU}, SBFD{XXXXU} (Alt. 2) and Type-1 RU for DTDD for small packet sizes.

##### 5.2.3.1.2	User Throughput

Figure 76 and Figure 77 show that both SBFD XXXXU (and static TDD 2UL DUDDU for small packets) offer significantly better cell-edge user throughput, median throughput, and 95%ile throughput compared to the reference static TDD network in UL, albeit at a significant cost of reduced DL cell-edge user throughput, median throughput, and 95%ile throughput because of more UL resource allocation in the former two cases. It is worth noting that at high load, cell-edge user DL throughput for SBFD XXXXU is very poor compared to the reference static TDD. Moreover, DTDD performs particularly well for both DL and UL at low load for big packets. For small packets, DTDD performs not as good due to the limitation that we only have two patterns to select (DDDSU and SUUUU) which are not optimize for small packets in UL since either UL resource is scarce for UL transmissions (for DDDSU) or DL resource is spare for UL grant transmission opportunities (for SUUUU).

Figure 76: Comparison of 5%ile, 50%ile and 95%ile Average- UPT for DL (Top) and UL (Bottom) at low, medium, and high loads for Indoor Network in FR1 deploying static TDD{DDDSU}, SBFD{XXXXU} (Alt. 2) and DTDD for big packet sizes.

Figure 77: Comparison of 5%ile, 50%ile and 95%ile Average- UPT for DL (Top) and UL (Bottom) at low, medium, and high loads for Indoor Network in FR1 deploying static TDD{DDDSU}, static TDD 2UL{DUDDU}, SBFD{XXXXU} (Alt. 2) and DTDD for small packet sizes.

##### 5.2.3.1.3	Latency

When comparing UL latency with the reference static TDD, it can be observed that SBFD (and Static TDD 2UL DUDDU for small packet sizes) demonstrate significantly reduced UL latency at all three-system load conditions. Meanwhile DL latency deteriorates clearly at high load scenario, which is the consequence of increased resource utilization and higher interference. For big packets, DTDD performs quite well for both DL and UL at low load.

Furthermore, it can be observed that SBFD and static TDD 2UL DUDDU networks demonstrate comparable median and cell-edge user latency in both DL and UL.

Figure 78: Comparison of 5-th%ile, 50%ile and 95%ile worst case latency mean latency at low, medium and high loads Indoor Network in FR1 deploying static TDD{DDDSU}, SBFD{XXXXU} (Alt. 2), and DTDD for big packet sizes.

Figure 79: Comparison of 5-th%ile, 50%ile and 95%ile worst case latency mean latency at low, medium and high loads Indoor Network in FR1 deploying static TDD{DDDSU}, static TDD 2UL{DUDDU}, SBFD{XXXXU} (Alt. 2), and DTDD for small packet sizes.

#### 5.2.3.2 	Alt.4 (Static TDD{DDDSU}, SBFD{XXXXX})

##### 5.2.3.2.1	Resource Utilization

Figure 80: Comparison of Type-2 RU in DL(top) and UL (bottom) at low, medium, and high loads for Indoor Network in FR1 deploying static TDD{DDDSU}, SBFD{XXXXX} (Alt. 4) and Type-1 RU for DTDD for big packet sizes.

Figure 81: Comparison of Type-2 RU in DL(top) and UL (bottom) at low, medium, and high loads for Indoor Network in FR1 deploying static TDD{DDDSU}, SBFD{XXXXX} (Alt. 4) and Type-1 RU for DTDD for small packet sizes.

The resource utilization for SBFD XXXXX networks increase for both DL and UL compared to the reference static TDD network, especially for medium and high loads. The reason is due to higher interferences while the UL/DL resource splitting are similar for static TDD and SBFD. The trends are similar for large and small packet sizes.

##### 5.2.3.2.2	User Throughput

Figure 82 shows that for big packet sizes, SBFD XXXXX offers almost the same cell-edge user throughput, median throughput, and 95%ile throughput compared to the reference static TDD network. Any slightly improvement in the UL leads to slightly degrade in DL for SBFD. Moreover, DTDD performs particularly well for both DL and UL at all loads for big packets. For small packet size SBFD offers some gains for indoor scenario thanks to less interference issues. However, as we show in section 5.2.3.1, a simpler scheme STDD 2UL DUDDU could offer similar gains.

Figure 82: Comparison of 5%ile, 50%ile and 95%ile Average- UPT for DL (Top) and UL (Bottom) at low, medium, and high loads for Indoor Network in FR1 deploying static TDD{DDDSU}, SBFD{XXXXX} (Alt. 4) and DTDD for big packet sizes.

Figure 83: Comparison of 5%ile, 50%ile and 95%ile Average- UPT for DL (Top) and UL (Bottom) at low, medium, and high loads for Indoor Network in FR1 deploying static TDD{DDDSU}, SBFD{XXXXX} (Alt. 4) and DTDD for small packet sizes.

##### 5.2.3.2.3	Latency

When comparing UL latency with the reference static TDD for big packet sizes, it can be observed that SBFD XXXXX does not offer any significant gain in term of latency. In fact, the SBFD with same area has worst 5% latencies that rapidly increase with load. For small packet sizes, SBFD provide some gains. However, as we mentioned above a simpler scheme STDD 2UL DUDDU could offers similar gains.

Figure 84: Comparison of 5-th%ile, 50%ile and 95%ile worst case latency mean latency at low, medium and high loads Indoor Network in FR1 deploying static TDD{DDDSU}, SBFD{XXXXX} (Alt. 4) and DTDD for big packet sizes.

Figure 85: Comparison of 5-th%ile, 50%ile and 95%ile worst case latency mean latency at low, medium and high loads Indoor Network in FR1 deploying static TDD{DDDSU}, SBFD{XXXXX} (Alt. 4) and DTDD for small packet sizes.

#### 5.2.3.3 	Alt.3 (Static TDD{DDSUU}, SBFD{XXXXU})

##### 5.2.3.3.1	Resource Utilization

Figure 86: Comparison of Type-2 RU in DL(top) and UL (bottom) at low, medium, and high loads for Indoor Network in FR1 deploying static TDD{DDSUU}, SBFD{XXXXU} (Alt. 3) and Type-1 RU for DTDD for big packet sizes.

Figure 87: Comparison of Type-2 RU in DL(top) and UL (bottom) at low, medium, and high loads for Indoor Network in FR1 deploying static TDD{DDSUU}, SBFD{XXXXU} (Alt. 3) and Type-1 RU for DTDD for small packet sizes.

The resource utilization for SBFD networks slightly increases for both DL and UL compared to the reference static TDD network, especially for medium and high loads due to more interferences. The trends are similar for both large and small packet sizes.

##### 5.2.3.3.2	User Throughput

Figure 88 shows that for big packet sizes, SBFD XXXXU with “same gain” antenna offers slightly better cell-edge user throughput, median throughput, and 95%ile throughput compared to the reference static TDD network, albeit at a slight cost of reduced DL cell-edge user throughput, median throughput, and 95%ile throughput. For “Same area” antenna, SBFD XXXXU performs worse than STDD at medium and high loads both in DL and UL. Moreover, DTDD performs particularly well for both DL and UL at low and medium loads for big packets.

For small packet sizes, SBFD XXXXU offers some gains for UL due to more frequent UL resource. However, as we mentioned above, a simpler scheme STDD 2UL DUDDU could offers similar gains.

Figure 88: Comparison of 5%ile, 50%ile and 95%ile Average- UPT for DL (Top) and UL (Bottom) at low, medium, and high loads for Indoor Network in FR1 deploying static TDD{DDSUU}, SBFD{XXXXU} (Alt. 3) and DTDD for big packet sizes.

Figure 89: Comparison of 5%ile, 50%ile and 95%ile Average- UPT for DL (Top) and UL (Bottom) at low, medium, and high loads for Indoor Network in FR1 deploying static TDD{DDSUU}, SBFD{XXXXU} (Alt. 3) and DTDD for small packet sizes.

##### 5.2.3.3.3	Latency

When comparing UL latency with the reference static TDD for big packet sizes, it can be observed that SBFD XXXXU does not offer any significant gain in term of latency. For small packet sizes, SBFD provides some latency gains. However, as we mentioned above a simpler scheme STDD 2UL DUDDU could offers similar gains.

Figure 90: Comparison of 5-th%ile, 50%ile and 95%ile worst case latency mean latency at low, medium and high loads Indoor Network in FR1 deploying static TDD{DDSUU}, SBFD{XXXXU} (Alt. 3) and DTDD for big packet sizes.

Figure 91: Comparison of 5-th%ile, 50%ile and 95%ile worst case latency mean latency at low, medium and high loads Indoor Network in FR1 deploying static TDD{DDSUU}, SBFD{XXXXU} (Alt. 3) and DTDD for small packet sizes.

Based on the above results for Alt.2, Alt.3, Alt.4, we have the following observation.

FR1 Indoor simulation results show that

- For big packets: When part of DL resource is shifted to UL (compared to reference static TDD Alt.2), SBFD Alt. 2 and static TDD 2UL networks provide better user throughput and latency in the UL compared to the reference static TDD (Alt. 2) network, at the cost of decreased DL performances. When DL/UL resource splitting are similar (compared to reference static TDD Alt. 3,4), SBFD Alt. 3, 4 do not provide meaningful gains compared to static TDD in both UL and DL. Moreover, dynamic TDD offers quite good performance (i.e., similar or even better) compared to SBFD and static TDD for all Alternatives.

- For small packets: SBFD (all Alternatives) offer some gains in term of throughput and latency compared to reference static TDDs. However, a simpler static TDD with 2 UL slots DUDDU would offer similar gains.

## 5.3	System level evaluation in FR2-1

### 5.3.1	Case 1: Single Operator Dense Urban Macro

#### 5.3.1.1	Alt.2 (Static TDD{DDDSU}, SBFD{XXXXU})

##### 5.3.1.1.1	Resource utilization per direction

Figure 92: Comparison of Type-2 RU in DL (top) and UL (bottom) at low, medium, and high loads for single operator Dense Urban Macro in FR2-1 deploying static TDD{DDDSU}, SBFD{XXXXU} (Alt. 2) and Type-1 RU for DTDD.

The RU behavior in FR2 is similar to FR1 cases.

##### 5.3.1.1.2	Coverage

Figure 93: Comparison of “25 Mbps Coverage” in DL (left) and “5 Mbps Coverage” in UL (right) at low, medium, and high loads for single operator Dense Urban Macro in FR2-1 deploying static TDD{DDDSU}, SBFD{XXXXU} (Alt. 2) and DTDD networks.

The coverage is the same for all networks considered in both DL and UL. This is an indication that perhaps the FR2 scenario may not be coverage limited. This needs to be viewed together the UPT improvements shown in the next sub-section.

##### 5.3.1.1.3	User Throughput

Figure 94: Comparison of 5%ile, 50%ile and 95%ile Average- UPT for DL (top) and UL (bottom) at low, medium, and high loads for single operator Dense Urban Macro in FR2-1 deploying static TDD{DDDSU}, SBFD{XXXXU} (Alt. 2) and DTDD networks.

It is evident that there are some improvements in UL cell-edge, median and 95%ile throughput of SBFD networks at the expense of DL performance in the corresponding metrics compared to a reference static TDD network. This is since there are unequal resources in DL and UL for both the systems. Furthermore, there is no impact on the UPT from the MPL plot above. This suggests that there may not be any power-limited UEs in the FR2 dense macro deployment.

##### 5.3.1.1.4	Latency

Figure 95: Comparison of 5%ile, 50%ile and 95%ile latency for DL (top) and UL (bottom) at low, medium, and high loads for single operator Dense Urban Macro in FR2-1 deploying static TDD{DDDSU}, SBFD{XXXXU} (Alt. 2) and DTDD networks.

The performance of latency in both DL and UL is similar to other FR1 simulations for Alt. 2 SBFD networks.

#### 5.3.1.2	Alt.3 (Static TDD{DDSUU}, SBFD{XXXXU})

##### 5.3.1.2.1	Resource utilization per direction

Figure 96: Comparison of Type-2 RU in DL (top) and UL (bottom) at low, medium, and high loads for single operator Dense Urban Macro in FR2-1 deploying static TDD{DDSUU}, SBFD{XXXXU} (Alt. 2) and Type-1 RU for DTDD.

##### 5.3.1.1.2	Coverage

Figure 97: Comparison of “25 Mbps Coverage” in DL (left) and “5 Mbps Coverage” in UL (right) at low, medium, and high loads for single operator Dense Urban Macro in FR2-1 deploying static TDD{DDDSU}, SBFD{XXXXU} (Alt. 2) and DTDD networks.

##### 5.3.1.1.3	User Throughput

Figure 98: Comparison of 5%ile, 50%ile and 95%ile Average- UPT for DL (top) and UL (bottom) at low, medium, and high loads for single operator Dense Urban Macro in FR2-1 deploying static TDD{DDDSU}, SBFD{XXXXU} (Alt. 2) and DTDD networks.

##### 5.3.1.1.4	Latency

Figure 99: Comparison of 5%ile, 50%ile and 95%ile latency for DL (top) and UL (bottom) at low, medium, and high loads for single operator Dense Urban Macro in FR2-1 deploying static TDD{DDDSU}, SBFD{XXXXU} (Alt. 2) and DTDD networks.

## 5.4	Summary of the system level evaluations

Based on the feasibility analysis and system level simulations in this contribution, it appears that the deployment of simpler schemes such as static TDD Alt. 3 or semi-static DTDD in the Urban Macro scenario with high power levels can achieve similar improvements in UL latency and coverage as an SBFD network. However, it's worth noting in scenarios where power levels are lower and the feasibility is not too challenging, SBFD may prove to be more beneficial, particularly in isolated use cases such as indoors. However, more comprehensive analysis and simulations are needed to fully determine its potential benefits in various deployment scenarios.

For FR1 single operator urban macro scenario, UL coverage is the key metric for potential improvement. The simulated alternatives (Alt2, Alt4, Alt3) show that SBFD only offers marginal UL coverage gains (~3 dB) when compared with the corresponding static TDD schemes at low load. The 5% UL average-UPT results across the three alternatives also indicate similar performance gains.

For FR1 two-operator urban macro scenario, UL coverage gains, if any, are only at low loads. To achieve this gain in a SBFD urban macro network, it is necessary to have same loads in coexisting network of another operator in the same frequency band, which is not realistic.

For FR1 two-operator urban macro scenario, UL gains for SBFD network in terms of cell-edge throughput, latency and coverage quickly diminish as the load increases.

For higher power BS class in Urban Macro scenario, system level simulations have shown that there is little to no improvement in UL coverage or cell-edge throughput performance by deploying an SBFD network as opposed to using a simple scheme such as static TDD 2UL or a semi-static DTDD in both single and multi-operator scenario.

For isolated indoor deployments, system level simulations show that similar UL latency and cell-edge throughput improvements can be achieved by deploying an SBFD network as well as using simple schemes such as static TDD 2UL. However, there is a need to align and ensure the scenario assumed for Indoor is realistic by deploying, for example, an Urban Macro layer.

For FR2 single operator Dense Macro scenario, there are UL performance gains in terms of cell-edge, median and 95%ile throughput and latency for SBFD network at low loads, and slightly lower gains at medium and high loads when compared to a reference static TDD network. However, it is not possible to make any meaningful conclusions as the scenario does not seem to be coverage limited.

# Conclusion

In the previous sections we made the following observations:

Observation 1	It is not necessary to perform link level simulations using separate models for DPD and PA.

Observation 2	For FR2, using a structure with RF chokes, 80dB of isolation is achievable over a reasonable bandwidth. Unlike FR1, the isolation does not vary with beam steering.

Observation 3	The gain from beam nulling increases when the TX beam is steered and the antenna isolation decreases. Thus, beam nulling can to some extent reduce the variation of the overall spatial isolation due to beam steering. It may also reduce the frequency variaion. However, there is a const in terms of reduced DL beam gain.

Observation 4	The cost of beam nulling in downlink can be substantial; we have observed up to 5dB DL power loss. There may be further DL losses due to lower degrees of freedom for MIMO operation.

Observation 5	When deciding beam nulling gains, downlink impacts should be considered.

Observation 6	The interference power caused by reciprocal mixing of phase noise in a 40-20-40 MHz SBFD carrier is around -60 to -70 dBc depending on BS implementation.

Observation 7	Adopt explicit digital filtering models in RAN1 link level studies to capture potential impacts to digital cancellation feasibility and performance.

Observation 8	The complexity of digital self-interference cancellation scales with the product of (1) the number of TX chains, (2) the number of RX chains and (3) the effective length of the multi-tap response of the environment and the analog RX frontends.

Observation 9	Dense Urban with 2-layer system has an ISD of 200m, the same needs to be used for the HetNet with Urban Macro and Indoor deployment.

Observation 10	A coverage metric based on the pathloss corresponding to a given bit rate is a good metric for system level simulations as it considers realistic beamforming and CLI (Option 2), unlike the MPL obtained from link budget analysis (Option 1 and Option 3).

Observation 11	Alt. 1 and Alt.2 SBFD configurations have more UL resources than the reference static TDD pattern it is compared with. Any potential gains in UL for these alternatives need to consider this impact first before drawing conclusions.

Observation 12	SBFD antenna configuration Option 2, which has double the antenna elements for SBFD when compared to reference static TDD, is the best-case scenario for SBFD.

Observation 13	SBFD antenna configuration Option 3, which has same number of antenna elements for SBFD when compared to reference static TDD, and only half the TxRUs can be used realistically, it is the practical case for SBFD.

Observation 14	The simulation results obtained from the “Realistic” assumptions can be considered as a more realistic estimation of the performance of SBFD in real-world scenarios, while the results from the “Optimistic” assumptions reflect the best-case scenario for SBFD’s potential performance gains.

Observation 15	For single operator Urban Macro scenario in FR1, UL performance gains of SBFD network in terms of coverage, latency and cell-edge user throughputs decrease as the load in the network increases.

Observation 16	For single operator Urban Macro scenario in FR1, the proposed DTDD network provides comparable performance as an SBFD network in terms of coverage, latency, and cell-edge user throughput in both DL and UL without having to deal with the hardware-complexity of SBFD network.

Observation 17	For single operator Urban Macro scenario in FR1, the UL performance gains for an SBFD network with all SBFD slots (XXXXX) in cell-edge user throughput is on par with SBFD with XXXXU at low loads but the performance drops considerably at medium and high loads due to significant interference limitations in all the SBFD slots. Further, even with optimistic assumptions for self-interference and inter-sector suppression, there is no noteworthy improvement in UL performance of an SBFD XXXXX network.

Observation 18	For two operator scenarios in FR1, the conclusions on performance gains of SBFD networks are similar to the conclusions from single operator analysis.

Observation 19	FR1 Indoor simulation results show that

-	For big packets: When part of DL resource is shifted to UL (compared to reference static TDD Alt.2), SBFD Alt. 2 and static TDD 2UL networks provide better user throughput and latency in the UL compared to the reference static TDD (Alt. 2) network, at the cost of decreased DL performances. When DL/UL resource splitting are similar (compared to reference static TDD Alt. 3,4), SBFD Alt. 3, 4 do not provide meaningful gains compared to static TDD in both UL and DL. Moreover, dynamic TDD offers quite good performance (i.e., similar or even better) compared to SBFD and static TDD for all Alternatives.

-	For small packets: SBFD (all Alternatives) offer some gains in term of throughput and latency compared to reference static TDDs. However, a simpler static TDD with 2 UL slots DUDDU would offer similar gains.

Observation 20	For FR1 single operator urban macro scenario, UL coverage is the key metric for potential improvement. The simulated alternatives (Alt2, Alt4, Alt3) show that SBFD only offers marginal UL coverage gains (~3 dB) when compared with the corresponding static TDD schemes at low load. The 5% UL average-UPT results across the three alternatives also indicate similar performance gains.

Observation 21	For FR1 two-operator urban macro scenario, UL coverage gains, if any, are only at low loads. To achieve this gain in a SBFD urban macro network, it is necessary to have same loads in coexisting network of another operator in the same frequency band, which is not realistic.

Observation 22	For FR1 two-operator urban macro scenario, UL gains for SBFD network in terms of cell-edge throughput, latency and coverage quickly diminish as the load increases.

Observation 23	For higher power BS class in Urban Macro scenario, system level simulations have shown that there is little to no improvement in UL coverage or cell-edge throughput performance by deploying an SBFD network as opposed to using a simple scheme such as static TDD 2UL or a semi-static DTDD in both single and multi-operator scenario.

Observation 24	For isolated indoor deployments, system level simulations show that similar UL latency and cell-edge throughput improvements can be achieved by deploying an SBFD network as well as using simple schemes such as static TDD 2UL. However, there is a need to align and ensure the scenario assumed for Indoor is realistic by deploying, for example, an Urban Macro layer.

Observation 25	For FR2 single operator Dense Macro scenario, there are UL performance gains in terms of cell-edge, median and 95%ile throughput and latency for SBFD network at low loads, and slightly lower gains at medium and high loads when compared to a reference static TDD network. However, it is not possible to make any meaningful conclusions as the scenario does not seem to be coverage limited.

Based on the discussion in the previous sections we propose the following:

Proposal 1	A SBFD carrier shall have a carrier BW and a UL subband BW consistent with one of the existing supported carrier BW in RAN4 specs.

Proposal 2	Adopt a net effect model for link-level simulations that captures the essential behaviors of a realistic DPD and PA combination with compliance to the base station ACLR requirements. This requires input from RAN4.

Proposal 3	Adopt a simple crest factor processing model, e.g., hard clipping + bandpass filtering, that captures the essential behaviors of a BS design to increase transmit power. This requires input from RAN4.

Proposal 4	The self-interference channel should be modeled as a set of tapped delay lines directly from TX sub-array ports to RX sub-array ports.

Proposal 5	Self-interference channel coefficients should be based on realistic setups supported by real measurements or high-fidelity electromagnetic (EM) evaluations.

Proposal 6	For both system and link level assessment of SBFD, proper modelling of advanced antennas as well as modelling of beamforming impact on the BS TX to RX isolation should be considered.

Proposal 7	For both system level and link level assessment of SBFD, proper modelling of advanced antennas as well as modelling of beamforming impact on the inter-sector TX to RX isolation needs be considered. For the simple exemplary site setup we have simulated for FR1, we see Tx-panel-to-Rx-port isolation values in the range of 67 to 87 dB depending on the azimuth and elevation beam steering directions and the frequency within the band. These values would most likely degrade if other realistic effects are included, e.g., electronics on the backside of the antenna, equipment and other metallic objects between sectors in a practical site, the presence of sub-arrays, and the presence of radomes.

Proposal 8	For both system level and link level assessment of SBFD, proper modelling of advanced antennas as well as modelling of beamforming impact on the inter-sector TX to RX isolation needs be considered. For the simple exemplary site setup we have simulated for FR2, we see isolation values in the range of 72 to 95 dB depending on the azimuth and elevation beam steering directions and the frequency within the band. These values would most likely degrade if other realistic effects are included, e.g., electronics on the backside of the antenna, equipment and other metallic objects between sectors in a practical site, the presence of sub-arrays, and the presence of radomes.

Proposal 9	Adopt a third order representation model in RAN1 studies to capture the essential behaviors of typical high-gain low noise amplifiers (LNA) in BS receiver chains.

Proposal 10	Adopt phase noise modelling in RAN1 studies to capture the distortion introduced by high power leakage from the DL sub-bands into the UL sub-bands. The phase noise models in TR 38.803 or those provided by RAN4 during the Rel-17 phase can be adopted as baseline models.

Proposal 11	Adopt modelling of analog filtering, if present, in RAN1 link level studies to capture potential impacts to digital cancellation feasibility and performance.

Proposal 12	RAN1 further agrees that interested companies may perform link-level simulations (LLS) for the purposes of evaluating SBFD performance and feasibility in both FR1 and FR2 including evaluation of the following:

a.	Self-interference suppression/cancellation accounting for realistic non-linearities in the gNB transmit and receive chains.

b.	Transmit beam nulling accounting for realistic non-linearities in the gNB transmit chain.

Proposal 13	RAN1 to further down-select scenarios where SBFD performance improvements may be realistically possible and can be simulated/evaluated by participating entities.

Proposal 14	RAN1 to agree that for evaluation of SBFD deployment 2-layer Scenario B for Case 3-2, Case 4 and dynamic/flexible TDD in FR1 (HetNet with Urban Macro and Indoor) consider the following.

	Layer 1: Urban Macro

o	Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around, ISD=200m.

Proposal 15	RAN1 to adopt the proposed methodology for calculating coverage metric as the target path loss corresponding to a certain (smoothed) average bit rate determined from system simulations: 10Mbps for DL and 1Mbps for UL in FR1 and 25 Mbps for DL and 5 Mbps for UL in FR2. This is called “10 Mbps coverage” for DL, “1 Mbps coverage” for UL, etc. (Option 2 in the proposal discussed in RAN1 #110)

Proposal 16	RAN1 to adopt the calculation and reporting of the statistic Pblocker’, considering the total power at the receiver as derived from the system level simulations.

Proposal 17	RAN1 to adopt the piecewise linear model proposed by RAN4 to model the receiver blocking and distortions caused by non-linearities in the receiver for FR1. Send an LS to  RAN4 to confirm the understanding that a model for FR2 will be provided as well.

Proposal 18	If 1 dB desense is assumed to model self-interference, then the self-interference power input to the model should be the value assumed to get 1 dB desense.

Proposal 19	RAN1 should prioritize the alignment of system-level simulation parameters with RAN4, focusing on parameters that are critical for obtaining reliable conclusions for the TR. Deviations in assumptions should be justifiable and documented in TR 38.858.

Proposal 20	RAN1 to agree adopting RAN4's noise figure values for different BS classes.

# References

RP-213591, New SI: Study on evolution of NR duplex operation, December 2021.

3GPP TR 38.803, “3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on new radio access technology: Radio Frequency (RF) and co-existence aspects (Release 14)”, v14.3.0, March 2022.

Singh et. al., “Full Duplex Radios: Are we there yet?”, HotNets 2020.

3GPP TR 38.828, Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR; (Release 16).

R1-2207462, "Sub-band non-overlapping full duplex," Ericsson, RAN1#110, Aug 2022.

R1-2207463, "Potential enhancements of dynamic TDD," Ericsson, RAN1#110, Aug 2022.

R1-2204109, "Inputs needed from RAN4 to facilitate RAN1 study," Ericsson, RAN1#109-e, May 2022.

R4-2010176, “On phase noise for 52.6-71 GHz”, Ericsson

R4-2011494, “On PN model for 52.6~71GHz”, Huawei

J. Stott, “The effects of phase noise in COFDM,” BBC Research & Development Technical Review, Summer, 1998.

AI 9.3.1 R1-2205540 Summary# of email discussion on evaluation of NR duplex evolution, RAN1 #109-e, May 2022.

R1-2204106, “Evaluation of NR duplex evolution”, Ericsson, May 2022.

AI 9.3.1 R1-2208270 Summary#6 on evaluation on NR duplex evolution, RAN1 #110, Toulouse, August 2022.

R1-2209175, “Subband nonoverlapping full duplex”, Ericsson, October 2022.

R1-2208347, "Reply LS on interference modelling for duplex evolution," RAN4, RAN1#110bis-e, October 2022.

R4-2109872, “Draft LS to ITU-R and CEPT on extension of IMT array antenna model to support sub-array structures”, Ericsson, Nokia, Qualcomm

R4-2108080, “LS to ITU-R and CEPT on extension of IMT array antenna model to support sub-array structures”, Ericsson, Nokia, Qualcomm

R1-2210779, AI 9.3.1, “Final summary on evaluation on NR duplex evolution”, RAN1 #110bis-e, October 2022.

R1-2300908, “Subband non-overlapping full duplex”, Ericsson, February 2023.

R1-2212283, “On the evaluation methodology for NR duplexing enhancements”, Nokia, Nokia Shanghai Bell, November 2022.

# Annex A

## A.1 GMP coefficients for ~2 GHz GaAs PA at 491.52 MHz sampling rate

aklPA with rows k∈[0,1,…,7] and columns l∈[-2,-1,0,1,2]:

Columns 1 through 3

-0.018625 +   0.010388i     0.023333 -  0.0087798i      0.98622 -    0.18951i

0.054357 -   0.088831i    -0.090219 +    0.27003i       1.4755 +    0.65625i

-0.12455 -   0.017389i     -0.21213 -    0.15655i      -17.835 -    0.36305i

0.21638 +    0.66855i       2.8252 -     0.2232i        104.9 -     28.214i

-0.50435 -     2.3545i      -10.033 +     1.1268i      -358.38 +     172.31i

0.62463 +     4.0633i       18.287 -     2.2362i       700.58 -     426.41i

-0.41658 -     3.3421i      -16.322 +     1.9937i      -722.13 +     479.27i

0.12892 +     1.0357i         5.62 -    0.64864i       299.41 -     200.73i

Columns 4 through 5

-0.038271 +   0.057208i      0.05687 -   0.047196i

-0.06683 -    0.23028i    -0.087356 +      0.217i

2.6685 -     1.8173i    -0.055794 -     0.7882i

-18.377 +     11.387i       1.1306 +     3.5817i

52.78 -     30.504i      -2.6675 -     11.211i

-79.138 +     43.326i       2.8947 +     19.236i

60.339 -     31.287i      -1.5249 -     16.556i

-18.351 +     8.9437i      0.30771 +     5.5925i

bkl0PA with rows k∈[0,1,…,7] and columns l∈-1,1:

0.023333 -  0.0087798i    -0.038271 +   0.057208i

-0.60131 +    0.97361i     -0.19392 +    0.52764i

8.3245 -     10.179i      0.18517 -     3.2744i

-66.4 +     51.766i       2.1001 +     27.044i

283.41 -     146.66i       2.6785 -     124.41i

-622.14 +     241.98i       -40.94 +     281.93i

665.85 -     217.24i       85.013 -     304.59i

-273.99 +     80.807i      -52.474 +     125.19i

## A.2 GMP coefficients for DPD

aklDPD with rows k∈[0,1,…,7] and columns l∈[-2,-1,0,1,2]:

Columns 1 through 3

0.022471 -  0.0078921i    -0.043712 -   0.016952i      0.98993 +     0.2369i

-0.11504 +   0.094754i      0.30463 -    0.27514i     -0.41542 -      2.164i

0.61892 -    0.20048i       -2.526 +    0.77428i       6.3179 +     15.297i

-2.1807 +    0.41401i       17.271 -     3.5562i      -20.414 -     64.192i

2.4173 -    0.26032i      -72.797 +     12.622i      -13.665 +     157.79i

8.1116 -     2.0655i       171.24 -     25.741i       171.94 -     249.03i

-23.955 +     7.1712i      -209.54 +     24.822i      -286.08 +     262.85i

17.8 -      6.778i       103.15 -     7.9965i       151.32 -     140.44i

Columns 4 through 5

0.089525 -    0.13533i    -0.058866 +   0.041949i

-0.18446 +    0.36701i      0.21464 -    0.21315i

-0.55708 -    0.73642i      -1.2971 +     1.2718i

8.508 +     6.3938i       7.4914 -     8.3733i

-19.38 -     41.588i      -32.321 +     32.588i

1.9616 +     124.32i       81.761 -     67.305i

42.859 -     170.52i       -106.2 +     68.492i

-38.085 +     87.918i       54.323 -     26.754i

## A.3 GMP coefficients for net effect of DPD and PA

aklnet with rows k∈[0,1,…,7] and columns l∈[-2,-1,0,1,2]:

Columns 1 through 3

-0.0014656 -  0.0023163i    0.0012144 +  0.0096127i       1.0151 -   0.021398i

0.016921 +    0.02074i    -0.033944 -   0.060851i     -0.13734 +   0.092064i

-0.19421 -   0.090327i      0.70051 +    0.46802i     0.053178 +     1.0772i

1.2719 -    0.37999i      -7.2355 -    0.68417i       7.9188 -     22.487i

-3.6967 +     4.8521i       37.145 -     5.8753i      -66.051 +     153.74i

3.9652 -      16.55i      -99.665 +     25.306i        250.5 -     481.88i

1.2339 +     23.785i       134.36 -      36.61i      -450.95 +     708.65i

-4.4834 -     12.473i      -70.523 +     18.559i        304.9 -      395.4i

Columns 4 through 5

-0.015418 +   0.010133i    0.0069273 -  0.0004492i

0.15998 +  0.0074072i    -0.060129 -   0.060582i

-1.4886 +     0.3234i      0.52252 +    0.51057i

11.254 -     7.8223i      -2.8348 -      1.121i

-62.593 +     48.408i       8.8972 -     1.7227i

198.44 -     135.61i      -15.718 +     9.5199i

-314.86 +     177.91i       13.792 -     9.4002i

189.75 -     88.448i      -4.3876 +    0.17987i

# Annex B:	Parameters for System level evaluations

## B.1	FR1 Urban Macro Scenario

|  | Parameters | Value |
| --- | --- | --- |
| Deployment | Scenario | UMa, Hexagonal layout, 7 BS per operator, 3 sectors per site, with wrapping |
|  | ISD | 500 m |
|  | BS height | 25 m |
|  | UE height | 1.5 m |
|  | Number of active UEs | 840 DL/UL only |
|  | UE to BS min 2D distance | 35 m |
|  | UE to UE min 2D distance | 1 m |
|  | UE distribution (in cluster) | Clustered (80%) |
|  | UE cluster number per macro cell | 2 |
|  | Cluster radius | 25 m |
|  | Minimum distance between macro TRP to Indoor/UE cluster center | 60 m |
|  | Minimum distance between two UE cluster centers | 50 m |
|  | Indoor/outdoor | 100% indoor in cluster 100% outdoor in cars outside cluster |
|  | Grid shift for two networks | 100% |
| System parameters | Carrier Frequency | 4 GHz |
|  | Duplex Type | STDD: DDDSU, DDSUU SBFD: XXXXU and XXXXX |
|  | Channel bandwidth | 100 MHz |
|  | Available resource blocks | 273 for STDD Alt.2,4: 104:55:104 (DUD) for SBFD Alt.3: 98:67:98 (DUD) for SBFD |
|  | Switching time | DL->UL: 2OS in the D slot SBFD DL->UL: 2OS in the SBFD slot |
|  | Numerology | 14 OFDM symbols slot SCS = 30kHz |
|  | UE attachment | Based on RSRP on port 0 |
| Channel model | gNB-UE | Macro-to-UE: UMa in TR 38.901 gNB-UE O2I penetration loss: 80% low-loss model, 20% high-loss model |
|  | gNB-gNB | Macro-to-Macro: UMa in TR 38.901 LOS probability: If the 2D distance between two Macro gNBs are less than or equal to the ISD, set the LOS probability to 0.75; Otherwise, reuse gNB-to-UE LOS probability equation in TR 38.901. |
|  | UE-UE | UE-to-UE: UMi-Street canyon in TR 38.901 Penetration loss between UEs follows Table A.2.1-12 in TR38.802 |
| BS | Antenna configuration | (M,N,P,Mg,Ng;Mp,Np) = (12,8,2,1,1;4,8), (dH,dV) = (0.7λ, 0.5λ) |
|  | Antenna element gain | 6.4 dBi |
|  | Antenna element and sub-array model | TR 38.803, Table 5.2.3.2.4-3: Macro urban |
|  | Subarray electrical down tilt | 3 deg |
|  | Mechanical down tilt | 6 deg |
|  | Beamforming method | Frequency domain |
|  | Panel HW assumptions | Same antenna gain Option 2 (Method 2-1) Same antenna area Option 3 (Method 3-1) |
|  | Max gNB Tx Power | 53 dBm for TDD, Same PSD for SBFD |
|  | Noise figure | 5 dB |
|  | Max modulation | 256 QAM |
| UE | UE antenna configuration | Tx: (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1), (dH,dV) = (N/A, N/A)λ Rx: (M,N,P,Mg,Ng;Mp,Np) = (1,2,2,1,1;1,2), (dH,dV) = (0.5, N/A)λ |
|  | Antenna model | isotropic |
|  | Antenna element gain | 0 dBi |
|  | Max UE TX Power | 23 dBm |
|  | UE power control | Sec. 9.1 TR36.942 P0= -80 dBm, alpha = 0.8 |
|  | Noise figure | 9 dB |
|  | Max modulation | 256 QAM |
| Traffic | DL/UL FTP packet size | 0.5Mbyte for DL and 0.125 Mbytes for UL |
|  | Traffic split | To reach target utilizations |
|  | Target Resource utilization | <10%, 20-40%, >50% |
| Intra-network | Spatial isolation for self-interference (Tx panel à Rx port) | “Realistic”: 80 dB including 10 dB Tx beam nulling with 3dB DL loss “Optimistic”: 1 dB desense for self-interference suppression |
|  | Inter-sector Isolation (sector<->sector) Tx panel à Rx port | “Realistic”: 75 dB “Optimistic”: 93 dB |
|  | Digital cancellation | 0 dB |
|  | UE-UE “ACIR” SBFD operator | 18 dB (ACS=33dB, ACLR=18dB based on IBE model) |
|  | gNB-gNB “ACIR” SBFD operator | 42 dB (ACLR 45dB, ACS 46 dB) |
| Inter-network | BS-BS ACIR (adjacent channels) | 42 dB |
|  | UE-UE ACIR (adjacent channels) | 28 dB |
|  | BS-UE ACIR (adjacent channels) | 33 dB |
|  | UE-BS ACIR (adjacent channels) | 30 dB |
|  | Co-sited isolation | 57 dB |
| Piecewise linear blocking model (RAN4 model) | Total input average power threshold 1 (P1) | -43 dBm |
|  | Total input average power threshold 2 (P2) | N/A |
|  | Sensitivity degradation slope 1 (k1) | 0.5 dB/dBm |
|  | Sensitivity degradation slope 2 (k2) | N/A |
|  | Total input average power blocking limit (P3) | -25 dBm |
| Latency related assumptions | DL transmission preparation time (BS) | 1 slot |
|  | DL decoding latency (UE) | 1 slot |
|  | UL transmission preparation time (UE) | 1 slot |
|  | UL decoding latency (BS) | 1 slot |
|  | Scheduling Request (SR) transmission opportunity | Available in every UL or SBFD slot |
|  | DCI transmission opportunity | Available in every DL or SBFD slot |

## B.2	FR1 Indoor Scenario

|  | Parameters | Value |
| --- | --- | --- |
| Deployment | Scenario | Indoor Hotspot (InH), single operator, 12 BS, (Scenario indoor-office in TR38.901) |
|  | ISD | 20 m |
|  | BS height | 3 m, ceiling mounted |
|  | UE height | 1.5 m |
|  | Number of active UEs | 240 DL/UL only |
|  | UE to BS min 2D distance | 0 m |
|  | UE to UE min 2D distance | 1 m |
| System parameters | Carrier Frequency | 4 GHz |
|  | Duplex Type | STDD: DDDSU, DDSUU, and DUDDU SBFD: XXXXU and XXXXX |
|  | Channel bandwidth | 100 MHz |
|  | Available resource blocks | 273 for STDD Alt.2,4: 104:55:104 (DUD) for SBFD Alt.3: 98:67:98 (DUD) for SBFD |
|  | Switching time | DL->UL: 2OS in the D slot SBFD DL->UL: 2OS in the SBFD slot For “Static TDD 2UL” DUDDU: 2OSs are in the U slot. |
|  | Numerology | 14 OFDM symbols slot SCS = 30kHz |
|  | UE attachment | Based on RSRP on port 0 |
| Channel model | gNB-UE | InH-Office in TR 38.901 |
|  | gNB-gNB | InH-Office in TR 38.901 |
|  | UE-UE | InH-Office in TR 38.901 |
| BS | Antenna configuration | (M,N,P,Mg,Ng;Mp,Np) =(4,4,2,1,1;4,4), (dH,dV) = (0.5λ, 0.5λ) |
|  | Antenna element gain | 5 dBi |
|  | Antenna element and sub-array model | R4-2109872, Table 1-2: Macro urban |
|  | Subarray electrical down tilt | N/A |
|  | Mechanical down tilt | 90 deg (pointing to the ground) |
|  | Beamforming method | Frequency domain |
|  | Panel HW assumptions | Same antenna gain Option 2 (Method 2-1) Same antenna area Option 3 (Method 3-1) |
|  | Max gNB Tx Power | 24 dBm for TDD, Same PSD for SBFD |
|  | Noise figure | 5 dB |
|  | Max modulation | 256 QAM |
| UE | UE antenna configuration | Tx: (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1), (dH,dV) = (N/A, N/A)λ Rx: (M,N,P,Mg,Ng;Mp,Np) = (1,2,2,1,1;1,2), (dH,dV) = (0.5, N/A)λ |
|  | Antenna model | isotropic |
|  | Antenna element gain | 0 dBi |
|  | Max UE TX Power | 23 dBm |
|  | UE power control | Sec. 9.1 TR36.942 P0= -60 dBm, alpha = 0.6 |
|  | Noise figure | 9 dB |
|  | Max modulation | 256 QAM |
| Traffic | DL/UL FTP packet size | Big files: 0.5Mbyte for DL and 0.125 Mbytes for UL Small files: 4Kbytes for DL and 1Kbyte for UL |
|  | Traffic split | To reach target utilizations |
|  | Target Resource utilization | <10%, 20-40%, >50% |
| Intra-network | Spatial isolation for self-interference (Tx panel Rx port) | 75 dB including 5 dB Tx beam nulling with 3dB DL loss |
|  | Inter-sector Isolation (sector<->sector) Tx panel Rx port | N/A |
|  | Digital cancellation | 10 dB |
|  | UE-UE “ACIR” SBFD operator | 18 dB (ACS=33dB, ACLR=18dB based on IBE model) |
|  | gNB-gNB “ACIR” SBFD operator | 42 dB (ACLR 45dB, ACS 46 dB) |
| Inter-network | BS-BS ACIR (adjacent channels) | 42 dB |
|  | UE-UE ACIR (adjacent channels) | 28 dB |
|  | BS-UE ACIR (adjacent channels) | 33 dB |
|  | UE-BS ACIR (adjacent channels) | 30 dB |
|  | Co-sited isolation | N/A |
| Piecewise linear blocking model (RAN4 model) | Total input average power threshold 1 (P1) | -43 dBm |
|  | Total input average power threshold 2 (P2) | N/A |
|  | Sensitivity degradation slope 1 (k1) | 0.5 dB/dBm |
|  | Sensitivity degradation slope 2 (k2) | N/A |
|  | Total input average power blocking limit (P3) | -25 dBm |
| Latency related assumptions | DL transmission preparation time (BS) | 1 slot |
|  | DL decoding latency (UE) | 1 slot |
|  | UL transmission preparation time (UE) | 1 slot |
|  | UL decoding latency (BS) | 1 slot |
|  | Scheduling Request (SR) transmission opportunity | Available in every UL or SBFD slot |
|  | DCI transmission opportunity | Available in every DL or SBFD slot |

## B.3	FR2 Dense Macro Scenario

|  | Parameters | Value |
| --- | --- | --- |
| Deployment | Scenario | DMa, Hexagonal layout, 7 BS per operator, 3 sectors per site, with wrapping |
|  | ISD | 200 m |
|  | BS height | 25 m |
|  | UE height | 1.5 m |
|  | Number of active UEs | 420 DL/UL only |
|  | UE to BS min 2D distance | 35 m |
|  | UE to UE min 2D distance | 1 m |
|  | UE distribution (in cluster) | Clustered (80%) |
|  | UE cluster number per macro cell | 1 |
|  | Cluster radius | 20 m |
|  | Minimum distance between macro TRP to Indoor/UE cluster center | 55 m |
|  | Minimum distance between two UE cluster centers | 40 m |
|  | Indoor/outdoor | 100% outdoor (no cars) |
|  | Grid shift for two networks | N/A |
| System parameters | Carrier Frequency | 30 GHz |
|  | Duplex Type | STDD: DDDSU, DDSUU SBFD: XXXXU and XXXXX |
|  | Channel bandwidth | 200 MHz |
|  | Available resource blocks | 132 for STDD Alt.2,4: 52:26:52 (DUD) for SBFD Alt.3: 49:32:49 (DUD) for SBFD |
|  | Switching time | DL->UL: 2OS in the D slot SBFD DL->UL: 2OS in the SBFD slot |
|  | Numerology | 14 OFDM symbols slot SCS = 120kHz |
|  | UE attachment | Based on RSRP on port 0 |
| Channel model | gNB-UE | Macro-to-UE: UMa in TR 38.901 gNB-UE O2I penetration loss: 80% low-loss model, 20% high-loss model |
|  | gNB-gNB | Macro-to-Macro: UMa in TR 38.901 LOS probability: If the 2D distance between two Macro gNBs are less than or equal to the ISD, set the LOS probability to 0.75; Otherwise, reuse gNB-to-UE LOS probability equation in TR 38.901. |
|  | UE-UE | UE-to-UE: UMi-Street canyon in TR 38.901 Penetration loss between UEs follows Table A.2.1-12 in TR38.802 |
| BS | Antenna configuration | (M,N,P,Mg,Ng;Mp,Np) = (16,24,2,1,1;8,24), (dH,dV) = (0.5λ, 0.5λ) |
|  | Antenna element gain | 5.1 dBi |
|  | Antenna element model | Am = SLAv = 30 dB 3dB = 3dB = 90 deg |
|  | Subarray electrical down tilt | 3 deg |
|  | Mechanical down tilt | 6 deg |
|  | Beamforming method | Frequency domain |
|  | Panel HW assumptions | Same antenna gain Option 2 (Method 2-1) Same antenna area Option 3 (Method 3-1) |
|  | Max gNB Tx Power | 43 dBm for TDD, Same PSD for SBFD |
|  | Noise figure | 7 dB |
|  | Max modulation | 256 QAM |
| UE | UE antenna configuration | Tx: (M,N,P,Mg,Ng;Mp,Np) = (2,2,1,1,2;1,1), (dH,dV) = (0.5, 0.5)λ Rx: (M,N,P,Mg,Ng;Mp,Np) = (2,2,2,1,2;1,1), (dH,dV) = (0.5, 0.5)λ |
|  | Antenna model | Table A.2.1-8 in TR 38.802 |
|  | Antenna element gain | 5 dBi |
|  | Max UE TX Power | 23 dBm |
|  | UE power control | Sec. 9.1 TR36.942 P0= -80 dBm, alpha = 0.8 |
|  | Noise figure | 10 dB |
|  | Max modulation | 64 QAM |
| Traffic | DL/UL FTP packet size | 0.5Mbyte for DL and 0.125 Mbytes for UL |
|  | Traffic split | To reach target utilizations |
|  | Target Resource utilization | <10%, 20-40%, >50% |
| Intra-network | Spatial isolation for self-interference (Tx panel à Rx port) | “Realistic”: 90 dB including 10 dB Tx beam nulling with 3dB DL loss “Optimistic”: 1 dB desense for self-interference suppression |
|  | Inter-sector Isolation (sector<->sector) Tx panel à Rx port | “Realistic”: 75 dB “Optimistic”: 98 dB |
|  | Digital cancellation | 0 dB |
|  | UE-UE “ACIR” SBFD operator | 15 dB (ACS=33dB, ACLR=15dB based on IBE model) |
|  | gNB-gNB “ACIR” SBFD operator | 22 dB (ACLR 28dB, ACS 23.5 dB) |
| Inter-network | BS-BS ACIR (adjacent channels) | N/A |
|  | UE-UE ACIR (adjacent channels) | N/A |
|  | BS-UE ACIR (adjacent channels) | N/A |
|  | UE-BS ACIR (adjacent channels) | N/A |
|  | Co-sited isolation | N/A |
| Piecewise linear blocking model (RAN4 model) | Total input average power threshold 1 (P1) | N/A |
|  | Total input average power threshold 2 (P2) | N/A |
|  | Sensitivity degradation slope 1 (k1) | N/A |
|  | Sensitivity degradation slope 2 (k2) | N/A |
|  | Total input average power blocking limit (P3) | N/A |
| Latency related assumptions | DL transmission preparation time (BS) | 1 slot |
|  | DL decoding latency (UE) | 1 slot |
|  | UL transmission preparation time (UE) | 1 slot |
|  | UL decoding latency (BS) | 1 slot |
|  | Scheduling Request (SR) transmission opportunity | Available in every UL or SBFD slot |
|  | DCI transmission opportunity | Available in every DL or SBFD slot |

# Annex C: Table of settings for evaluation cases

## C.1	FR1_Case1 Urban Macro

| Key assumptions Sub-cases | Interference modelling (e.g., Self-interference and Co-site: Spatial isolation + digital isolation) |  |  |  | SBFD slot configuration |  |  | BS transmit power |  | SBFD antenna configuration |  | Packet Size |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Self-interference |  | Co-site inter sector |  | Alt.-2: {DDDSU} vs. {XXXXU} | Alt.-4: {DDDSU} vs. {XXXXX} | Alt.-3: {DDSUU} vs. {XXXXU} | 53dBm | 49dBm | Twice area&same TxRUs ('Same Gain’) | Same area&same TxRUs (‘Same area’) | DL: 4Kbytes, UL: 1Kbyte | DL: 0.5Mbytes, UL: 0.125Mbyte |
|  | 70 dB + 10 dB (Tx beam null) | 1 dB deseeds (Total = 148 dBc) | 75 dB | 93 dB |  |  |  |  |  |  |  |  |  |
| SBFD#1_UMa_FR1_Alt.2_Real_SameGain | ○ |  | ○ |  | ○ |  |  | ○ |  | ○ |  |  | ○ |
| SBFD#1_UMa_FR1_ Alt.2_Real_SameArea | ○ |  | ○ |  | ○ |  |  | ○ |  |  | ○ |  | ○ |
| SBFD#1_UMa_FR1_Alt.2_Opti_SameGain |  | ○ |  | ○ | ○ |  |  | ○ |  | ○ |  |  | ○ |
| SBFD#1_UMa_FR1_ Alt.2_Opti_SameArea |  | ○ |  | ○ | ○ |  |  | ○ |  |  | ○ |  | ○ |
| SBFD#1_UMa_FR1_Alt.4_Real_SameGain | ○ |  | ○ |  |  | ○ |  | ○ |  | ○ |  |  | ○ |
| SBFD#1_UMa_FR1_ Alt.4_Real_SameArea | ○ |  | ○ |  |  | ○ |  | ○ |  |  | ○ |  | ○ |
| SBFD#1_UMa_FR1_Alt.4_Opti_SameGain |  | ○ |  | ○ |  | ○ |  | ○ |  | ○ |  |  | ○ |
| SBFD#1_UMa_FR1_ Alt.4_Opti_SameArea |  | ○ |  | ○ |  | ○ |  | ○ |  |  | ○ |  | ○ |
| SBFD#1_UMa_FR1_Alt.3_Real_SameGain | ○ |  | ○ |  |  |  | ○ | ○ |  | ○ |  |  | ○ |
| SBFD#1_UMa_FR1_ Alt.3_Real_SameArea | ○ |  | ○ |  |  |  | ○ | ○ |  |  | ○ |  | ○ |
| SBFD#1_UMa_FR1_Alt.3_Opti_SameGain |  | ○ |  | ○ |  |  | ○ | ○ |  | ○ |  |  | ○ |
| SBFD#1_UMa_FR1_ Alt.3_Opti_SameArea |  | ○ |  | ○ |  |  | ○ | ○ |  |  | ○ |  | ○ |

## C.2	FR1_Case4 Urban Macro

| Key assumptions Sub-cases | Interference modelling (e.g., Self-interference and Co-site: Spatial isolation + digital isolation) |  |  |  |  | SBFD slot configuration |  |  | BS transmit power |  | SBFD antenna configuration |  | Packet Size |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Self-interference |  | Co-site inter sector |  | Inter-operator co-sited | Alt.-2: Ref {DDDSU} Vs. {DDDSU},{XXXXU} | Alt.-4: Ref {DDDSU} vs. {DDDSU} ,{XXXXX} | Alt.-3: Ref {DDDSU} vs. {DDSUU} ,{XXXXU} | 53dBm | 49dBm | Twice area&same TxRUs ('Same Gain’) | Same area&same TxRUs (‘Same area’) | DL: 4Kbytes, UL: 1Kbyte | DL: 0.5Mbytes, UL: 0.125Mbyte |  |
|  | 70 dB + 10 dB (Tx beam null) | 1 dB deseeds (Total = 148 dBc) | 75 dB | 93 dB | 57 dB |  |  |  |  |  |  |  |  |  |  |
| SBFD#4_UMa_FR1_Alt.2_Real_SameGain_0GridShift | ○ |  | ○ |  | ○ | ○ |  |  | ○ |  | ○ |  |  | ○ |  |
| SBFD#4_UMa_FR1_Alt.2_Real_SameGain_100GridShift | ○ |  | ○ |  |  | ○ |  |  | ○ |  | ○ |  |  | ○ |
| SBFD#4_UMa_FR1_ Alt.2_Real_SameArea_0GridShift | ○ |  | ○ |  | ○ | ○ |  |  | ○ |  |  | ○ |  | ○ |  |
| SBFD#4_UMa_FR1_ Alt.2_Real_SameArea_100GridShift | ○ |  | ○ |  |  | ○ |  |  | ○ |  |  | ○ |  | ○ |  |
| SBFD#4_UMa_FR1_Alt.4_Real_SameGain_0GridShift | ○ |  | ○ |  | ○ |  | ○ |  | ○ |  | ○ |  |  | ○ |  |
| SBFD#4_UMa_FR1_Alt.4_Real_SameGain_100GridShift | ○ |  | ○ |  |  |  | ○ |  | ○ |  | ○ |  |  | ○ |
| SBFD#4_UMa_FR1_ Alt.4_Real_SameArea_0GridShift | ○ |  | ○ |  | ○ |  | ○ |  | ○ |  |  | ○ |  | ○ |  |
| SBFD#4_UMa_FR1_ Alt.4_Real_SameArea_100GridShift | ○ |  | ○ |  |  |  | ○ |  | ○ |  |  | ○ |  | ○ |  |
| SBFD#4_UMa_FR1_Alt.3_Real_SameGain_0GridShift | ○ |  | ○ |  | ○ |  |  | ○ | ○ |  | ○ |  |  | ○ |  |
| SBFD#4_UMa_FR1_Alt.3_Real_SameGain_100GridShift | ○ |  | ○ |  |  |  |  | ○ | ○ |  | ○ |  |  | ○ |  |
| SBFD#4_UMa_FR1_ Alt.3_Real_SameArea_0GridShift | ○ |  | ○ |  | ○ |  |  | ○ | ○ |  |  | ○ |  | ○ |  |
| SBFD#4_UMa_FR1_ Alt.3_Real_SameArea_100GridShift | ○ |  | ○ |  |  |  |  | ○ | ○ |  |  | ○ |  | ○ |  |

## C.3	FR1_Case1 Indoor Office

| Key assumptions Sub-cases | Interference modelling (e.g., Self-interference and Co-site: Spatial isolation + digital isolation) |  | SBFD slot configuration |  |  | SBFD antenna configuration |  | Packet Size |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Self-interference |  | Alt.-2: {DDDSU} vs. {XXXXU} | Alt.-4: {DDDSU} vs. {XXXXX} | Alt.-3: {DDSUU} vs. {XXXXU} | Twice area&same TxRUs ('Same Gain’) | Same area&same TxRUs (‘Same area’) | DL: 4Kbytes, UL: 1Kbyte | DL: 0.5Mbytes, UL: 0.125Mbyte |
|  | 75 dB + 5 dB (Tx beam null) +10 dB(Digital IC) (Total = 132 dBc including frequency isolation) | 1 dB desense (Total = 119 dBc) |  |  |  |  |  |  |  |
| SBFD#1_InH_FR1_Alt.2_Real_SameGain_bf | ○ |  | ○ |  |  | ○ |  |  | ○ |
| SBFD#1_InH_FR1_ Alt.2_Real_SameArea_bf | ○ |  | ○ |  |  |  | ○ |  | ○ |
| SBFD#1_InH_FR1_Alt.2_Real_SameGain_sf | ○ |  | ○ |  |  | ○ |  | ○ |  |
| SBFD#1_InH_FR1_ Alt.2_Real_SameArea_sf | ○ |  | ○ |  |  |  | ○ | ○ |  |
| SBFD#1_InH_FR1_Alt.4_Real_SameGain_bf | ○ |  |  | ○ |  | ○ |  |  | ○ |
| SBFD#1_InH_FR1_ Alt.4_Real_SameArea_bf | ○ |  |  | ○ |  |  | ○ |  | ○ |
| SBFD#1_InH_FR1_Alt.4_Real_SameGain_sf | ○ |  |  | ○ |  | ○ |  | ○ |  |
| SBFD#1_InH _FR1_ Alt.4_Real_SameArea_sf | ○ |  |  | ○ |  |  | ○ | ○ |  |
| SBFD#1_InH _FR1_Alt.3_Real_SameGain_bf | ○ |  |  |  | ○ | ○ |  |  | ○ |
| SBFD#1_InH _FR1_ Alt.3_Real_SameArea_bf | ○ |  |  |  | ○ |  | ○ |  | ○ |
| SBFD#1_InH_FR1_Alt.3_Opti_SameGain_sf | ○ |  |  |  | ○ | ○ |  | ○ |  |
| SBFD#1_InH_FR1_ Alt.3_Opti_SameArea_sf | ○ |  |  |  | ○ |  | ○ | ○ |  |

## C.4	FR2_Case1 Dense Macro

| Key assumptions Sub-cases | Interference modelling (e.g., Self-interference and Co-site: Spatial isolation + digital isolation) |  |  |  | SBFD slot configuration |  |  | BS transmit power | SBFD antenna configuration |  | Packet Size |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Self-interference |  | Co-site inter sector |  | Alt.-2: {DDDSU} vs. {XXXXU} | Alt.-4: {DDDSU} vs. {XXXXX} | Alt.-3: {DDSUU} vs. {XXXXU} | 43dBm | Twice area&same TxRUs ('Same Gain’) | Same area&same TxRUs (‘Same area’) | DL: 4Kbytes, UL: 1Kbyte | DL: 0.5Mbytes, UL: 0.125Mbyte |
|  | 80 dB + 10 dB (Tx beam null) | 1 dB deseense | 75 dB | 98 dB |  |  |  |  |  |  |  |  |
| SBFD#1_DMa_FR2_Alt.2_Real_SameGain | ○ |  | ○ |  | ○ |  |  | ○ | ○ |  |  | ○ |
| SBFD#1_DMa_FR2_ Alt.2_Real_SameArea | ○ |  | ○ |  | ○ |  |  | ○ |  | ○ |  | ○ |
| SBFD#1_DMa_FR2_Alt.2_Opti_SameGain |  | ○ |  | ○ | ○ |  |  | ○ | ○ |  |  | ○ |
| SBFD#1_DMa_FR2_ Alt.2_Opti_SameArea |  | ○ |  | ○ | ○ |  |  | ○ |  | ○ |  | ○ |
| SBFD#1_DMa_FR2_Alt.3_Real_SameGain | ○ |  | ○ |  |  |  | ○ | ○ | ○ |  |  | ○ |
| SBFD#1_DMa_FR2_ Alt.3_Real_SameArea | ○ |  | ○ |  |  |  | ○ | ○ |  | ○ |  | ○ |
| SBFD#1_DMa_FR2_Alt.3_Opti_SameGain |  | ○ |  | ○ |  |  | ○ | ○ | ○ |  |  | ○ |
| SBFD#1_DMa_FR2_ Alt.3_Opti_SameArea |  | ○ |  | ○ |  |  | ○ | ○ |  | ○ |  | ○ |