3GPP TSG-RAN WG1 Meeting #112-bis-e	R1-2302776

e-Meeting, April 17th – 26th, 2023

Agenda Item:	9.1.3.2

Source:	Ericsson

Title:	On SRS enhancements targeting TDD CJT and 8 TX operation

Document for:	Discussion

# 1	Introduction

During RAN#94-e, a new WID for Rel-18 MIMO evolution for DL and UL was agreed [1]. Two objectives of the work item that concern SRS enhancements are:

- Study, and if justified, specify enhancements of CSI acquisition for CJT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows:

- Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off.

- SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences.

- Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e., 32.

- Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices.

- Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

# 2 	Discussion

In this contribution, we discuss SRS enhancements for TDD-CJT and 8 Tx operation. Specifically, SRS enhancements targeting 8 TX UL transmissions are discussed in Section 2.1 and SRS enhancements targeting TDD CJT are discussed in Section 2.2.

## 2.1	SRS for 8 TX

### 2.1.1	SRS for 8 Tx without TDM

In RAN1#110bis-e, it was agreed to support an 8-port SRS resource in an SRS resource set with usage ‘antennaSwitching’ or ‘codebook’, for which 8 SRS ports are mapped to each of m∈{1, 2, 4, 8, 10, 12, 14} SRS symbols. A remaining issue, which we shall discuss next, is how to map the 8 ports onto comb offsets and cyclic shifts (CSs). Note that if such a mapping can be found for m=1, it applies also for m>1, as comb offset and CS are the only parts of the SRS resource configuration that is port specific. Indeed, with repetition, the same mapping would be used over all SRS symbols. For frequency hopping and partial sounding, the frequency-domain starting position is the same for all ports.

For multi-port SRS resources, the SRS ports are assigned to CSs on one or more comb offsets. According to existing specification, SRS ports on a same comb offset are mapped to CSs in such a way that they are maximally and equidistantly separated. Furthermore, the maximum number of CSs nSRScs, max is a function of the configured transmission comb KTC, as shown in Table 1.

Table 1	Maximum number of CSs per comb offset, nSRScs, max, as a function of the transmission comb, KTC (reproduced from Table 6.4.1.4.2-1 in 3GPP TS 38.211).

| KTC | nSRScs, max |
| --- | --- |
| 2 | 8 |
| 4 | 12 |
| 8 | 6 |

It has been agreed that for an 8-port SRS resource configured with transmission comb 2, the 8 SRS ports can be mapped to 1 or 2 comb offsets (8 or 4 SRS ports per comb offset). For transmission comb 4, the 8 SRS ports are mapped to 2 comb offsets (4 SRS ports per occupied comb offset). For transmission comb 8, the 8 SRS ports are mapped to 4 comb offsets (2 SRS ports per occupied comb offset). An open question, however, is which comb offsets that will be occupied by an 8-port SRS resource. Our preference, is to make the mapping as similar as possible to the legacy comb-offset mapping method and, therefore, propose:

For KTC=2, SRS ports {1000, 1002, 1004, 1006} are mapped to kTC, and SRS ports {1001, 1003, 1005, 1007} are mapped to kTC, or (kTC+KTC/2) mod KTC for nSRScs∈0, …, nSRScs,max/2-1 and nSRScs∈nSRScs,max2, …, nSRScs,max-1, respectively.

For KTC=4, SRS ports {1000, 1002, 1004, 1006} are mapped to kTC, and SRS ports {1001, 1003, 1005, 1007} are mapped to (kTC+KTC/2) mod KTC .

For KTC=8, SRS ports {1000, 1004} are mapped to kTC, SRS ports {1001, 1005} are mapped to (kTC+KTC/2) mod KTC, SRS ports {1002, 1006} are mapped to (kTC+KTC/4) mod KTC, and SRS ports {1003, 1007} are mapped to (kTC+3KTC/4) mod KTC.

Here, kTC denotes the RRC-configured comb offset and nSRScs denotes the RRC-configured CS. With the above proposal, we comb offset kTCpi for port pi is given by (changes compared to legacy formula are highlighted in blue, green, and red for KTC=2, KTC=4, and KTC=8, respectively):

kTCpi=kTC+KTC2 mod KTCif NapSRS=8, pi∈1001, 1005, and nSRScs,max=6kTC+KTC4 mod KTCif NapSRS=8, pi∈1002, 1006, and nSRScs,max=6kTC+3KTC4 mod KTCif NapSRS=8, pi∈1003, 1007, and nSRScs,max=6kTC+KTC2 mod KTCif NapSRS=8, pi∈1001, 1003, 1005, 1007, and nSRScs,max=12kTC+KTC2 mod KTCif NapSRS=8, pi∈1001, 1003, 1005, 1007, and nSRScs∈nSRScs,max2, …, nSRScs,max-1kTC+KTC2 mod KTCif NapSRS=4, pi∈1001, 1003, and nSRScs,max=6kTC+KTC2 mod KTC if NapSRS=4, pi∈1001, 1003, and nSRScs∈nSRScs,max2, …, nSRScs,max-1kTCotherwise.

We remark that while no agreements have yet been made on port mapping for 8 Tx partially coherent UE with Ng=2 or Ng=4 antenna-port groups in AI 9.1.4.2, it would be preferable that SRS ports that are assumed to be coherent are mapped to a same comb offset whenever possible. E.g., with comb-offset mapping as per the above proposals, for KTC=8 and Ng=4, SRS ports {1000, 1004}, {1001, 1005}, {1002, 1006}, and {1003, 1007} belong to different antenna-port groups.

Another open question is how to extend the CS mapping formula in 3GPP TS 38.211 to support 8 SRS ports. Specifically, according to Rel-15/16 specification, the CS αi (in radians) for SRS port pi is given by:

αi=2πnSRScs,inSRScs,max.

Furthermore, nSRScs,i in the above formula is given by the following:

nSRScs,i=nSRScs+nSRScs,maxpi-1000NapSRSmod nSRScs,max.

Here, nSRScs is the RRC-configured CS and NapSRS is the number of SRS ports.

It is preferred that nSRScs,i is an integer. If this condition is not fulfilled, it is not possible to separate the NapSRS=8 SRS ports by means of simple delay-domain windowing (see, e.g., Figure 7 in [2]), which is desired. It can be easily verified that for KTC=2, the above formula generates integers, but for KTC>2, it does not. Hence, it is necessary to update the mapping between SRS ports and CSs for 8-port SRS resources.

Focusing on transmission comb 8, in Rel-17, it was agreed that the maximum number of CS for KTC=8 is nSRScs, max=6 (see Table 1), which required a new CS mapping formula for 4-port SRS resources (since 4 ports cannot be equidistantly separated over nSRScs,max=6 CSs):

nSRScs,i=nSRScs+nSRScs,maxpi-10002NapSRS2mod nSRScs,maxif NapSRS=4 and nSRScs,max=6nSRScs+nSRScs,maxpi-1000NapSRSmod nSRScs,maxotherwise.

The updated formula ensures that nSRScs,i is integer values also for the case NapSRS=4 and nSRScs,max=6 (note that nSRScs,max=6 is equivalent to KTC=8). This formula can be straightforwardly extended to NapSRS=8, e.g., as follows (changes compared to legacy formula are highlighted in red):

nSRScs,i=nSRScs+nSRScs,maxpi-1000NapSRS22mod nSRScs,maxif NapSRS≥4 and nSRScs,max=6nSRScs+nSRScs,maxpi-1000NapSRS mod nSRScs,maxotherwise.

Note that the above formula reduces to Rel-17 formula for the case NapSRS=4 and nSRScs,max=6.

In Table 2, we show the assigned CSs and comb offsets a 4-port SRS resource RRC-configured with transmission comb 8, comb offset 0, and CS 0. In Table 3, we show the corresponding assignment for an 8-port SRS resource according to the formula above. Here, we have assumed that comb-offset mapping is as per the above proposals.

Table 2	Per-port cyclic shift and comb offset with 4 SRS ports, transmission comb 8, and with legacy mapping.

| Port | Cyclic shift | Comb offset |
| --- | --- | --- |
| 1000 | 0 | 0 |
| 1001 | 0 | 4 |
| 1002 | 3 | 0 |
| 1003 | 3 | 4 |

Table 3	Per-port cyclic shift and comb offset with 8 SRS ports, transmission comb 8, and with extension of legacy mapping.

| SRS port | Cyclic shift | Comb offset |
| --- | --- | --- |
| 1000 | 0 | 0 |
| 1001 | 0 | 4 |
| 1002 | 0 | 2 |
| 1003 | 0 | 6 |
| 1004 | 3 | 0 |
| 1005 | 3 | 4 |
| 1006 | 3 | 2 |
| 1007 | 3 | 6 |

In, e.g., [3], it is mentioned that PAPR of the transmitted signal may be impacted by SRS virtualization. For example, PAPR can be significantly increased (as we shall later demonstrate) by combining, on a same antenna port, two SRS ports that occupy different comb offsets and use the same CS.

Although not explicitly stated, it is commonly assumed that the first and second half of SRS ports is associated with orthogonal polarizations. For example, Rel-15 DL Type-I codebooks as well 4 Tx UL codebooks are designed according to this principle (it is likely that a similar assumption will be adopted for 8 Tx UL codebook design, but that discussion is contained within AI 9.1.4.2). Under this assumption, with the allocation in Table 2 and Table 3, all SRS ports belonging to a same polarization will share the same CS. Hence, if SRS virtualization (e.g., for usage ‘codebook’) is performed over these ports, PAPR can be high.

In [4] it is highlighted that PAPR is a particular concern for 8-port SRS resources with transmission comb 8 as the number of SRS ports is larger than the number of available CSs and, hence, CSs must be reused over multiple comb offsets. A reasonable strategy, which reduces PAPR, is to adopt a new/updated CS mapping that minimizes the reuse of CSs on different comb offsets, e.g., as per the following:

nSRScs,i=nSRScs+nSRScs,maxpi-1000NapSRS22+pi-1000mod(NapSRS2)mod nSRScs,maxif NapSRS≥4 and nSRScs,max=6nSRScs+nSRScs,maxpi-1000NapSRS mod nSRScs,maxotherwise.

The resulting CSs are exemplified in Table 4 for an 8-port SRS resource RRC-configured with transmission comb 8, comb offset 0, and CS 0. Note that the formula above ensures that CS reuse does not occur within the first or second half of SRS ports.

Table 4	Per-port cyclic shift and comb offset with 8 SRS ports, transmission comb 8, and with new mapping.

| SRS port | Cyclic shift | Comb offset |
| --- | --- | --- |
| 1000 | 0 | 0 |
| 1001 | 1 | 4 |
| 1002 | 2 | 2 |
| 1003 | 3 | 6 |
| 1004 | 3 | 0 |
| 1005 | 4 | 4 |
| 1006 | 5 | 2 |
| 1007 | 0 | 6 |

In Figure 1, we show the PAPR for the case when the first and second half of SRS ports, have been combined with equal weight on a same antenna port. The figures show the PAPR for all possible sequence groups u=0, 1,…, 29, and with both the extension of legacy and new CS mapping. We note that new CS mapping results in significantly lower PAPR.

Figure 1	PAPR with extension of legacy and new CS mapping formula, when first (left plot) and second (right plot) half of SRS ports have been virtualized (by combining using equal weights) onto a same antenna port.

Based on the above discussion, we propose the following:

For an 8-port SRS resource, SRS ports are mapped to CSs in a way such that reuse of CSs over comb offsets is minimized.

Note that the PAPR issue exist also for legacy 4-port SRS resources (as mentioned in [4]). A CS mapping according to Proposal 4 could improve the PAPR for 4-port SRS resources as well. Indeed, the new CS mapping formula results in the CSs shown in Table 5 for a 4-port SRS resource.

Table 5	Per-port cyclic shift and comb offset with 4 SRS ports, transmission comb 8, and with new mapping.

| Port | Cyclic shift | Comb offset |
| --- | --- | --- |
| 1000 | 0 | 0 |
| 1001 | 1 | 4 |
| 1002 | 3 | 0 |
| 1003 | 4 | 4 |

We note from Table 5 that a CS is not used on more than one comb offset.

### 2.1.2	SRS for 8 Tx with TDM

In RAN1#112, the following was agreed for SRS TDM:

Agreement (RAN1#112)

For an 8-port SRS resource in an SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s, support the 8 ports equally partitioned into s subsets with each subset having 8/s different ports.

- At least s = 2.

  - FFS: s = 4, s = 8.

- m = 2, 4, 8, 10, 12, 14, and m is a multiple of s.

- Each of the m OFDM symbols has only one subset. Reuse the existing resource mapping designed for 8/s ports on each OFDM symbol.

  - Including frequency-domain resource allocation and mapping to cyclic shifts. FFS port indexing within the subset of 8/s ports.

  - FFS: down selection from existing resource mapping designs

- FFS: which subset of 8/s ports are mapped onto each OFDM symbol.

- FFS: the TDM factor s is configured as an explicit RRC parameter or determined implicitly from other parameters.

For the first FFS, we don’t prefer to split SRS ports unevenly over OFDM symbols. Uneven distribution of SRS ports is not desired as it requires special handling and results in different power for different SRS ports. The only number of SRS ports per OFDM symbol that support all values of m according to the above agreements is 4. Hence, we propose:

Don’t support TDM factor s=4 or s=8.

For the second FFS, we start by noting that, according to above agreements, existing resource mapping designed for 8/s SRS ports will apply to SRS TDM as well. The main benefit with SRS TDM is that UE splits the transmit power over only half (according to Proposal 5) of the SRS ports to increase the coverage. NR supports already several SRS coverage enhancement schemes: repetition, frequency hopping, and partial sounding. These schemes should be supported also for an SRS resource configured with TDM and offer a sufficient set of methods for mapping each subset of SRS ports to OFDM symbols:

Don’t support down selection from existing resource mapping schemes, i.e., support SRS TDM combined with one or more of repetition, frequency hopping, and partial sounding.

For the third SRS, to reuse existing resource mapping designed for 8/s SRS ports, we propose:

For resource mapping within a subset, the 4 SRS ports are {1000, 1001, 1002, 1003}.

For the fourth SRS, a further agreement was reached in RAN1#112:

Agreement (RAN1#112)

For an 8-port SRS resource in an SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s ≥ 2, the m OFDM symbols are adjacent, and select one of the following options regarding the TDM pattern:

- Option 2-1: the s subsets of ports are mapped cyclically as {1, 2, …, s,1, 2, …, s} on the m OFDM symbols.

- Option 2-2: the s subsets of ports are mapped sequentially as {1, …, 1, 2, …, 2, s, …, s} on the m OFDM symbols.

We remark that either of sequential mapping and cyclic mapping could be preferable, depending on the deployment. However, since it has been agreed to reuse existing resource mapping within each subset, and since existing resource mapping (e.g., when computing frequency-hopping offset) assumes adjacent SRS symbols, sequential mapping should be supported for SRS TDM.

Support sequential mapping such that the first and last 4 SRS ports are mapped to OFDM symbols {0, …, m/2 - 1} and {m/2,…, m-1}, respectively.

For the fifth and last FFS, with Proposal 5, only one TDM factor is supported and it does not need to be explicitly configured:

The TDM factor is implicitly set to s=2 when SRS TDM is configured/enabled.

## 2.2	SRS for TDD CJT

### 2.2.1	Cyclic-shift and/or comb-offset hopping

In RAN1#112, the following agreements related to CS and/or comb-offset hopping was made:

Agreement (RAN1#112)

For SRS interference randomization, support:

- Opt. 3: Both cyclic shift hopping and comb offset hopping.

  - At least the two features can be separately configured

- FFS: Combined cyclic shift hopping and comb offset hopping for a UE

- FFS: Separate or combined with SRS sequence group hopping / sequence hopping

- FFS: Associated UE capability

According to the above agreement, at least one of CS hopping and/or comb-offset hopping can be configured for an SRS resource. In the following, we provide details on how to support such hopping in a way that does not incur collisions between CS and/or comb offsets associated with different SRS resources, we also discuss whether/how a combined CS/comb-offset hopping scheme can be supported.

Irrespectively of which hopping scheme that is configured (i.e., one or more of CS hopping, comb-offset hopping, or a joint CS/comb-offset hopping scheme) it must support co-scheduling of legacy UEs on a shared set of RBs, which puts some restriction on the hopping pattern. The simplest way to support such co-scheduling is to restrict the hopping pattern to a subset of the available CSs (and/or comb offsets).

CS and/or comb offset hopping is limited to a subset of CSs and/or comb offsets.

In what follows, we demonstrate, for the case of transmission comb 2 for which there are 8 possible CSs, how co-scheduling of CS-hopping SRS resources with a legacy 4-port SRS resource (that occupies 4 CSs on the same comb offset, according to legacy port mapping, as shown in Figure 2) can be supported.

Figure 2	Legacy 4-port SRS resource (RRC-configured with comb offset 0 and CS 0) over multiple transmission occasions. The mapping from SRS ports to CSs and comb offsets does not vary over time.

In Figure 3, we show two examples of CS hopping on a subset of CSs for which co-scheduling with legacy UE(s) is possible. Specifically, collisions can be avoided by configuring the CS hopping on different comb offset or by restricting the CS hopping to a subset of the available CSs per comb offset. Note that SRS ports 0—3 can belong to either a single 4-port SRS resource, two different 2-port SRS resources, or four different 1-port SRS resources.

Figure 3	SRS resource configured with CS hopping (over 4 CSs), co-scheduled with legacy SRS resource on separate (top part of the figure) or same (bottom part of the figure) comb offset.

To avoid collisions, the set of CSs and/or comb offsets over which the hopping occurs and the hopping pattern within the set of CSs and/or comb offsets must be NW-configured. In legacy NR, the CSs and comb offsets spanned by an SRS resource, and the mapping from SRS ports to these CSs and comb offsets is configured by RRC parameters combOffset and cyclicShift. These parameters can be reused to determine the subset of CSs and/or comb offsets also for the hopping scheme.

For example, in the above examples, to support SRS resources in orthogonal subsets, the legacy 4-port SRS resource is RRC configured with CS 0 and comb offset 0, the first CS-hopping 4-port SRS resource (in the top part of the figure) is configured with CS 0 and comb offset 1, and the second CS-hopping 4-port SRS resource (in the bottom part of the figure) is configured with CS 1 and comb offset 0.

To support hopping for SRS resources with fewer ports than the cardinality of the configured subset, the starting position of the hopping pattern within the subset can also be configured by RRC parameters combOffset and cyclicShift. For example, in the top part of Figure 2, the SRS ports 0—3 could correspond to four different 1-port SRS resources configured with comb offset 1 and CS 0, 2, 4, and 6, respectively.

Based on the above, we propose:

The subset of CSs and/or comb offsets, and the starting position of the hopping pattern within the subset is configured by legacy parameters cyclicShift and combOffset.

It is a possibility that both (separate and/or joint) CS and comb-offset hopping will be supported in NR Rel-18 in the same OFDM symbol. In this case, from a NW perspective, if more than one hopping scheme is supported, it should still be possible to co-schedule UEs supporting different hopping schemes, i.e., SRS ports in SRS resources configured with different hopping schemes should not collide

Again, the solution is to restrict the hopping to a limited set of CSs and/or comb offsets. In this case, the subset of CSs and comb offsets may have to depend on the configured hopping scheme. In Figure 4, we show an example of how 4-port (or less) SRS resources configured with different hopping schemes can be supported in the same RB.

Figure 4	Legacy SRS resource co-scheduled with CS-hopping (over 4 CSs) SRS resource (top part of the figure), with CS-hopping (over 4 CSs) SRS resource and comb-offset-hopping (over 2 comb offsets) SRS resource (middle part of the figure), and with CS-hopping (over 4 CSs) SRS resource, comb-offset-hopping (over 2 comb offsets) SRS resource, and joint CS-and-comb-offset-hopping (over 2 CSs and 2 comb offsets) SRS resource (bottom part of the figure).

Based on the above, we propose:

Combined CS and comb-offset hopping is supported and such that SRS ports in SRS resources configured with different hopping schemes that does not collide in an OFDM symbol also does not collide in any other OFDM symbol.

According to the below RAN1#112 agreement, the hopping pattern within the configured subset is determined by a predefined pseudo-random sequence (specifically, the sequence cn in Clause 5.2.1 of 3GPP TS 38.211):

Agreement (RAN1#112)

For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port, the hopping pattern is determined based on the pseudo-random sequence c(i), initialized with one of the following IDs.

- Option 1: Reuse the SRS sequence identity nIDSRS.

- Option 2: Introduce new ID(s).

  - FFS: the value range, one new ID or two separate new IDs, default ID(s)

To avoid collisions within the configured subset, the pseudo-random sequence can be initialized by the SRS sequence identity nIDSRS. This supports not only non-colliding SRS ports for SRS resources sharing the same SRS sequence. It can also randomize interference for SRS resources configured with different SRS sequences as they will use different hopping patterns. Hence, no new ID is required and we propose:

The hopping pattern is determined by the SRS sequence identity nIDSRS.

A final agreement on CS and/or comb-offset hopping from the RAN1#112 meeting is as follows:

Agreement (RAN1#112)

For SRS comb offset hopping and/or cyclic shift hopping, the time-domain hopping behaviour depends on at least the slot index ns,fμ within a radio frame and OFDM symbol index l', and select at least one of the following options:

- Option 1: Within a slot, hopping based on the repetition factor R and symbol index that is the same across the R repetitions.

- Option 2: Within a slot, hopping based on only the symbol index l'.

- Option 3: No intra-slot hopping.

- FFS: Time domain hopping behaviour further depends on system frame number (SFN) nf.

  - FFS:  reinitialization periodicity of N radio frames or reinitialization based on system frame number.

- FFS: Whether to adopt the same option(s) for comb offset hopping and cyclic shift hopping (if supported separately)

- FFS: At least support reinitialization at the beginning of each radio frame.

The first three bullets, i.e., Option 1—3, addresses whether intra-slotting hopping should be supported (Option 1 or 2) or not (Option 3). In our view, all SRS resource types should be able to benefit from the interference randomization provided by a CS and/or comb-offset hopping scheme. For aperiodic SRS, intra-slot hopping is required and should be supported. Finally, we note that legacy SRS hopping schemes (e.g., frequency hopping) are such that hopping does not occur over repetitions, which enables NW to measure the channel for the same set subcarriers over multiple OFDM symbols (e.g., to track channel aging) while still benefiting from the interference randomization provided by the hopping scheme. Such legacy behaviour should be supported for CS and/or comb-offset hopping as well. Hence, we propose:

Support CS and comb-offset hopping for all resource types, i.e., periodic, semi-persistent, and aperiodic SRS.

Support intra-slot CS and/or comb-offset hopping.

Intra-slot CS and comb offset is the same across R repetitions for CS and/or comb-offset hopping (Option 1).

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	For KTC=2, SRS ports {1000, 1002, 1004, 1006} are mapped to kTC, and SRS ports {1001, 1003, 1005, 1007} are mapped to kTC, or (kTC+KTC/2) mod KTC for nSRScs∈0, …, nSRScs,max/2-1 and nSRScs∈nSRScs,max2, …, nSRScs,max-1, respectively.

Proposal 2	For KTC=4, SRS ports {1000, 1002, 1004, 1006} are mapped to kTC, and SRS ports {1001, 1003, 1005, 1007} are mapped to (kTC+KTC/2) mod KTC .

Proposal 3	For KTC=8, SRS ports {1000, 1004} are mapped to kTC, SRS ports {1001, 1005} are mapped to (kTC+KTC/2) mod KTC, SRS ports {1002, 1006} are mapped to (kTC+KTC/4) mod KTC, and SRS ports {1003, 1007} are mapped to (kTC+3KTC/4) mod KTC.

Proposal 4	For an 8-port SRS resource, SRS ports are mapped to CSs in a way such that reuse of CSs over comb offsets is minimized.

Proposal 5	Don’t support TDM factor s=4 or s=8.

Proposal 6	Don’t support down selection from existing resource mapping schemes, i.e., support SRS TDM combined with one or more of repetition, frequency hopping, and partial sounding.

Proposal 7	For resource mapping within a subset, the 4 SRS ports are {1000, 1001, 1002, 1003}.

Proposal 8	Support sequential mapping such that the first and last 4 SRS ports are mapped to OFDM symbols {0, …, m/2 - 1} and {m/2,…, m-1}, respectively.

Proposal 9	The TDM factor is implicitly set to s=2 when SRS TDM is configured/enabled.

Proposal 10	CS and/or comb offset hopping is limited to a subset of CSs and/or comb offsets.

Proposal 11	The subset of CSs and/or comb offsets, and the starting position of the hopping pattern within the subset is configured by legacy parameters cyclicShift and combOffset.

Proposal 12	Combined CS and comb-offset hopping is supported and such that SRS ports in SRS resources configured with different hopping schemes that does not collide in an OFDM symbol also does not collide in any other OFDM symbol.

Proposal 13	The hopping pattern is determined by the SRS sequence identity nIDSRS.

Proposal 14	Support CS and comb-offset hopping for all resource types, i.e., periodic, semi-persistent, and aperiodic SRS.

Proposal 15	Support intra-slot CS and/or comb-offset hopping.

Proposal 16	Intra-slot CS and comb offset is the same across R repetitions for CS and/or comb-offset hopping (Option 1).

# References

- Samsung, “RP-213598, MIMO evolution for downlink and uplink,” 3GPP TSG RAN meeting #94-e, electronic meeting, December 2021.

- Ericsson, “R1-2110121, Remaining issues for SRS”, 3GPP TSG RAN WG1 meeting RAN1#106bis-e, electronic meeting, October 2021.

- Qualcomm, “R1-2209971, SRS enhancement for TDD CJT and 8 Tx operation,” 3GPP TSG RAN WG1 meeting RAN1#110bis-e, electronic meeting, October 2022.

- Futurewei, “SRS enhancements for TDD CJT and 8TX operation,” 3GPP TSG RAN WG1 meeting RAN1#110bis-e, electronic meeting, October 2022.