3GPP TSG RAN WG1 #112bis-e  													                     R1-2302777

e-Meeting, April 17th – April 26th, 2023

Source: 	Intel Corporation

Title:	Discussions on reply LS on Rel-18 multi-carrier enhancement

Agenda item:	5

Document for:	Discussion and Decision

# Introduction

RAN1 received a reply LS from RAN4, which includes the following questions [1]:

In addition, RAN4 would like to ask RAN1 one question: From RAN1 perspective, is it possible that the two Tx chains are switched concurrently between two different band pairs and with overlapping switching period? Two examples are given below: - Example #1: In the case of 3-band Tx switching, the switching is performed from 1T+1T on band A and B to 2T on band C. - Example #2: In the case of 4-band Tx switching, the switching is performed from 1T+1T on band A and B to 1T+1T on band C and D. ACTION: RAN4 respectfully asks RAN1 and RAN2 to take into account the above information in their work.

In the contribution, we present our view on reply LS on Rel-18 multi-carrier Tx switching.

# Discussion on response to RAN4 LS

Regarding RAN4 question on whether it is possible that the two Tx chains are switched concurrently between two different band pairs and with overlapping switching period, this would depend on gNB scheduling and configuration of UL switching and UE implementation. In some cases, it is more desirable to support the case that two Tx chains are switched concurrently between two bands, which may help reduce the interruption time and thereby improve the system throughput. In our view, from RAN1 perspective, it is possible that the two Tx chains are switched concurrently between two different band pairs and with overlapping switching period.

Proposal 1

- From RAN1 perspective, it is possible that the two Tx chains are switched concurrently between two different band pairs and with overlapping switching period.

# Conclusions

In this contribution, we presented our view on reply LS on Rel-18 multi-carrier Tx switching. Further, we summarize the proposals as follows:

Proposal 1

- From RAN1 perspective, it is possible that the two Tx chains are switched concurrently between two different band pairs and with overlapping switching period.

# References

- R1-2302266, “LS on Rel-18 Multi-carrier enhancement for NR”, February 2023