3GPP TSG RAN WG1 #112bis-e	                                                   	            R1-2302781

e-Meeting, April 17th – April 26th, 2023

Source:	Intel Corporation

Title:	On two TAs for multi-DCI

Agenda item:	9.1.1.2

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss 2-TA enhancement for Rel-18 NR MIMO WI [1]

- Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed.

# Associating TAGs to uplink channels/signals

We reached the following agreement in RAN1#110bis-e:

Agreement For associating TAGs to target UL channels/signals for multi-DCI based multi-TRP operation, support the following: Associate TAG to TCI-state - Associate TAG ID with UL/joint TCI state - For UL transmission, the TAG ID associated with the UL/joint TCI state is utilized - A baseline is UE expects that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESET Pool Index correspond to one TAG - Working Assumption: A UE may report that it supports that the [activated] UL/joint TCI states [of UL signals/channels] associated to one CORESETPoolIndex correspond to both TAGs FFS: on how to handle association when Rel-15/16 spatial relation framework is used for - PUCCH - DG/CG Type 1/Type 2 PUSCH - AP/SP/P SRS

In RAN1#112 the association of TAG to UL/joint TCI state was agreed. A baseline UE capability may assume that all UL signals and channels associated with one CORESETPoolIndex correspond to one TAG. Note that UL/joint TCI states are dynamically associated with CORESETPoolIndex using MAC-CE. Therefore activated UL/joint TCI states should be used for defining a baseline restriction.

The use-case for allowing multiple TAGs corresponding to a CORESETPoolIndex is usage of multi-DCI multi-TRP operation in intra-DU scenarios where backhaul delay is close to ideal. Compared to single-DCI multi-TRP, multi-DCI multi-TRP is more practical as it allows separate scheduling and better usage of PDCCH search space and BD capabilities across the two TRPs. This usage scenario is not new in RAN1 and was the motivation to introduce joint HARQ-ACK feedback in Rel-16. Therefore we have the following proposal:

Proposal-1: Confirm the working assumption - a UE may report that it supports that the activated UL/joint TCI states of UL signals/channels associated to one CORESETPoolIndex to correspond to both TAGs

Consistent with the TAG to TCI state association that is already agreed, for the FFS part when Rel-15/16 spatial relation framework is used, it is natural to associate TAG with PUCCH-SpatialRelationInfo and with SRS-SpatialRelationInfo. This completes the association of PUCCH, PUSCH and SRS to TAGs in FR2.

Proposal-2: When Rel-15/16 spatial relation framework is used, associate TAG with PUCCH-SpatialRelationInfo and with SRS-SpatialRelationInfo

# Indication of candidate cell in PDCCH order

In RAN1#110bis-e and RAN1#111, we reached the following agreements

Agreement RAN1#110bis-e in MIMO For multi-DCI based inter-cell Multi-TRP operation with two TA enhancement, support a mechanism to determine which PRACH configuration (i.e., RACH configuration corresponding to serving cell PCI or an additional PCI) to be used in the RACH procedure triggered by PDCCH order - FFS: Explicit indication or implicit indication through PDCCH order Agreement RAN1#112 in Rel-18 LTM The PDCCH order from the source cell contains the indication of candidate cell. - The reserved bit(s) in DCI format 1_0 for PDCCH order can be used for indication of cell identity

In alignment with LTM agreements, we propose an explicit indication via DCI bit-field

Proposal-3: For inter-cell mTRP support explicit indication by DCI to determine which PRACH configuration to be used in the RACH procedure triggered by PDCCH order

# Per TRP vs cross TRP

In RAN1#111 we reached the following agreement

Agreement For multi-DCI based Multi-TRP operation with two TA enhancement, support the case where a PDCCH order sent by one TRP triggers RACH procedure towards either the same TRP or a different TRP at least for inter-cell Multi-DCI. - FFS: for intra-cell Multi-DCI - FFS: whether there are any restrictions needed - FFS: if cross TRP RACH triggering is an optional feature

According to current specification the SSB that is QCL with the DMRS of the PDCCH order is used as a reference for Tx power determination for PRACH. In the case of cross TRP PRACH transmission it is desirable to switch this Tx power reference to the SSB used for PRACH association.

Further, for RAR reception a UE assumes that the DMRS is QCL with the SSB used for PRACH association. This also should be switched to the SSB that is QCL with the DMRS of the PDCCH order to allow RAR reception always from the same cell.

Proposal-4: For cross TRP PRACH triggering switch the Tx power reference from the SSB QCL with the PDCCH order to the SSB used for PRACH association.

Proposal-5: For cross TRP PRACH RAR reception switch the QCL reference of the DMRS from the SSB used for PRACH association to the SSB that is QCL with the DMRS of the PDCCH order.

Based on the agreement in RAN1#111, cross TRP RACH triggering is supported in inter-cell case. For intra-cell multi-DCI case, we think cross TRP triggering can be supported with the same enhancement.

Proposal-6: Support cross TRP PRACH triggering in both intra-cell and inter-cell multi-DCI case

# TA/TAG identification for Intra-cell Multi-DCI

In RAN1#111 we agreed on the following

Agreement For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement, support at least one of the following alternatives (down selection to be done in RAN1#111): - Alt 1: indicate TAG ID as part of TA command in RAR - Alt 2: indicate TAG ID as part of PDCCH order - Alt 3: divide SSBs into two groups, one for each TRP. If a SSB associated to a RACH procedure belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP. - Alt 4: divide RACH resources into two groups, where for a RACH procedure, if the corresponding RACH resource belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP. - Alt 5: divide preambles into two groups, where for a RACH procedure, if the corresponding preamble belongs to the nth group (n=1,2), then the TA obtained via the RACH procedure corresponds to the nth TRP - Alt 6: TAG ID is associated with CORESETPoolIndex and TAG ID is determined based on the CORESETPoolIndex of PDCCH order - Alt 7: Each TCI state is associated with a TAG ID, and the TAG ID corresponding to RACH triggered by a PDCCH order is determined based on the TCI state used to receive the PDCCH order Note: If Alt 1 or Alt 2 is downselected, then it does not preclude indication of two TAG IDs (if supported)

In the case of intra-cell multi-DCI, we think its sufficient to indicate TAG ID as part of RAR

Proposal-7: for intra-cell multi-DCI support indication of TAG ID as part of TA command in RAR

# Overlapping uplink transmissions

In RAN1#110, we reached the following agreement on overlapping of two UL transmissions:

Agreement For multi-DCI based multi-TRP operation with two TAs, study how to handle overlapping part between two UL transmissions associated with two TAs, where the study includes: - whether to introduce scheduling restriction in overlapping part - whether to introduce dropping rules - whether specification impact is needed, or if the issue can be handled via implementation - whether to allow overlapped transmission in case the UE supports STxMP transmission (if STxMP feature is agreed in NR Rel-18)

The issue of overlap between two UL transmissions is not new, in the current specifications this can occur in the slot boundaries and is resolved in the specifications as shown below:

If two adjacent slots overlap due to a TA command, the latter slot is reduced in duration relative to the former slot. The UE does not change 𝑁𝑇𝐴 during an actual transmission time window for a PUSCH or a PUCCH transmission [6, TS 38.214].

In order to understand the nature of overlap in Rel-15, we can assume a 15kHz SCS, where a TA adjustment step-size is equal to 16*64*Tc = ~0.5us. A TA command is delivered by 6 bits, so at a slot boundary TA adjustment could be +/- 16us (32*0.5 us) which is greater than a CP. Therefore, up to a symbol can be lost due to TA overlap.

We propose to extend the same principle for Rel-18 where overlapping part between two UL transmissions associated with two TAs is handled by reducing the duration of the second transmission. The issue of maximum TA difference is discussed in RAN4. Additionally, we expect that for STxMP transmissions, such relaxation is not needed.

Proposal-8: overlapping between two UL transmissions associated with two TAs is handled by reducing the duration of the second transmission until the maximum TA difference (as will be advised by RAN4). This does not apply to STxMP transmissions.

# Conclusion

In this paper, we discuss our view for two TAs for multi-DCI and summarize our proposals below:

Proposal-1: Confirm the working assumption - a UE may report that it supports that the activated UL/joint TCI states of UL signals/channels associated to one CORESETPoolIndex to correspond to both TAGs

Proposal-2: When Rel-15/16 spatial relation framework is used, associate TAG with PUCCH-SpatialRelationInfo and with SRS-SpatialRelationInfo

Proposal-3: for inter-cell mTRP support explicit indication by DCI to determine which PRACH configuration to be used in the RACH procedure triggered by PDCCH order

Proposal-4: For cross TRP PRACH triggering switch the Tx power reference from the SSB QCL with the PDCCH order to the SSB used for PRACH association.

Proposal-5: For cross TRP PRACH RAR reception switch the QCL reference of the DMRS from the SSB used for PRACH association to the SSB that is QCL with the DMRS of the PDCCH order.

Proposal-6: Support cross TRP PRACH triggering in both intra-cell and inter-cell multi-DCI case

Proposal-7: for intra-cell multi-DCI support indication of TAG ID as part of TA command in RAR

Proposal-8: overlapping between two UL transmissions associated with two TAs is handled by reducing the duration of the second transmission until the maximum TA difference (as will be advised by RAN4). This does not apply to STxMP transmissions.

# References

- RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung, 3GPP TSG RAN #94e, Electronic Meeting, Dec. 6 - 17, 2021