3GPP TSG RAN WG1 #112bis-e                                                                  R1-2302785

e-Meeting, April 17th – April 26th, 2023

Source:	Intel Corporation

Title:	UL precoding indication for multi-panel transmission

Agenda item:	9.1.4.1

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss UL transmission enhancement to precoding indication for multi-panel transmission for Rel-18 NR MIMO WI

- Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation.

# Single-DCI Based STxMP UL Transmission

## Single-DCI based STxMP PUSCH

### SRS Resource Set and Shared Digital Ports

#112 Working Assumption For dynamic switching between STxMP SDM scheme and sTRP transmission, support the following: - For sTRP transmission: The maximal number of layers of sTRP transmission is configured by the maxRank (or Lmax) as in current spec (i.e., Option 1) - For SDM scheme: configure one single maximal number of layers (separate from maxRank (or Lmax) for sTRP) that is applied to the first SRS resource set and the second SRS resource set, separately (i.e., Alt1) - FFS: Whether/How to enable that the total number of used PUSCH antenna ports for the SDM and sTRP is the same. - Note: This corresponds to the case that digital ports are shared between the panels - Note: RAN1 supports both implementations that digital ports are shared or separate among panels

In RAN1#112 a working assumption was agreed as shown above (Option 1 + Alt 1). This is consistent with UE panel structure with the same number of digital ports. Note that a multi-panel UE with different number of digital ports in different panels could perform codebook based Tx operation with Option 1 + Alt 1 using specification transparent virtualization while the gNB only observes the same number of digital ports across all panels. Therefore we propose to confirm the working assumption. Whether/how to enable asymmetric panel structure at the UE side can be further discussed.

Proposal 1. For sDCI based STxMP PUSCH SDM scheme, confirm the working assumption Option-1 + Alt-1. It is FFS whether to support additional specifications for asymmetric UE panel structure.

| Figure 1. Case A1 | Figure 2. Case A2 |
| --- | --- |
| Figure 3.Case B1 | Figure 4.Case B2 |

In RAN1#112 the support of a case where # of sTRP ports = # of SDM ports was discussed. In the following we consider the following antenna architectures.

|  | #sTRP PUSCH ports | #SDM PUSCH ports | Comments |
| --- | --- | --- | --- |
| Case A1 (Figure 1) | 2 | 2+2 | Dedicated digital ports |
| Case A2 (Figure 2) | 2 | 2+2 | Shared digital ports |
| Case B1 (Figure 3) | 4 | 2+2 | Shared digital ports with port selection |
| Case B2 (Figure 4) | 4 | 2+2 | Shared digital ports with port aggregation for full power |

Note that case A1 and case A2 can already be supported by Option-1 + Alt-1 as per the working assumption which includes both shared and dedicated digital ports architecture. We can also observe that the architectures for cases B1 and B2 reduces to Case A1 if a UE virtualizes each panel to 2 ports. This virtualization enables full power operation and higher MCS can be used for 2-layer sTRP operation compared to 4-layer sTRP operation.

We also note that in order to support full power Mode-2 operation, 2 SRS resources in the same resource set can be used with 2 ports and 4 ports each (for codebook-based operation). This can also be used to support case B1 and B2 where a 2 port SRS is used for SDM transmission, and a 4 port SRS is used for sTRP transmission.

Observation 1: Existing working assumption Option-1 + Alt-1 can support both shared and dedicated digital port architecture optimally in the case where #sTRP PUSCH ports is less than #SDM PUSCH ports.

Proposal 2. Support configuring a 2-port and a 4-port SRS resource in the same SRS resource set if there is a need to support the special case of #sTRP PUSCH ports = #SDM PUSCH ports

For single-DCI based STxMP PUSCH SFN scheme, we need to decide whether each panel can transmit up to 2 layers or 4 layers first. We think that the specifications should not limit the number of layers to less than 4 (but UE capability may limit it)

Proposal 3. For sDCI based STxMP PUSCH SFN scheme, support up to 4-layer transmission

### SRI/TPMI Fields Design and Dynamic Switching Between Different Transmission Modes

#112 Working Assumption For dynamic switching between STxMP SDM scheme and sTRP transmission, support the following: - For sTRP transmission: The maximal number of layers of sTRP transmission is configured by the maxRank (or Lmax) as in current spec (i.e., Option 1) - For SDM scheme: configure one single maximal number of layers (separate from maxRank (or Lmax) for sTRP) that is applied to the first SRS resource set and the second SRS resource set, separately (i.e., Alt1) - FFS: Whether/How to enable that the total number of used PUSCH antenna ports for the SDM and sTRP is the same. - Note: This corresponds to the case that digital ports are shared between the panels - Note: RAN1 supports both implementations that digital ports are shared or separate among panels #112 Agreement On dynamic switching between STxMP SFN scheme and sTRP transmission: - The legacy maxRank/Lmax is applied to sTRP transmission - For configuration of SFN, - Alt2: Configure a separate parameter for the maximal number of layers for STxMP SFN #111 Agreement - Support DCI-based dynamic switching between SFN scheme of single-DCI based STxMP PUSCH and sTRP transmission - The DCI field “SRS resource set indicator” is used to indicate the switching between SFN scheme and sTRP transmission. - (Conclusion) There is no consensus to support DCI-based dynamic switching between SFN scheme and SDM scheme

In previous RAN1 meetings, it has been agreed that a 2-bit DCI field is used for dynamic switching between SDM/SFN STxMP PUSCH transmission mode and single-TRP PUSCH transmission mode. And a working assumption has been made to use the legacy maxRank for sTRP transmission and to configure one new maxRank for SDM transmission in STxMP PUSCH.

Table 1. STxMP and sTRP mode SRI field(s) length comparison for dynamic switching

|  | sTRP, NSRS,1 or NSRS,2=4 Lmax; SRI length, in bits. 1; 2. 2/3/4; 4. | sTRP, NSRS,1 or NSRS,2=3 Lmax; SRI length, in bits. 1; 2. 2/3/4; 3. | sTRP, NSRS,1 or NSRS,2=2 Lmax; SRI length, in bits. 1; 1. 2/3/4; 2. |
| --- | --- | --- | --- |
| STxMP, NSRS,1=4, NSRS,2=4 LmaxSTxMP; SRI1+SRI2 length, in bits. 1; 2 + 2 = 4. 2; 4 + 4 = 8. | using only one SRI field in sTRP mode may not be enough for some max number of layers values. (Example: forLmaxSTxMP =1, Lmax=2, only 2 bits are in the first SRI field, but 4 bits are needed for sTRP SRI indication) |  |  |
| STxMP, NSRS,1=4, NSRS,2=3 LmaxSTxMP; SRI1+SRI2 length, in bits. 1; 2 + 2 = 4. 2; 4 + 3 = 7. | using only one SRI field in sTRP mode may not be enough for some max number of layers values. | using only one SRI field in sTRP mode may not be enough for some max number of layers values. |  |
| STxMP, NSRS,1=4, NSRS,2=2 LmaxSTxMP; SRI1+SRI2 length, in bits. 1; 2 + 1 = 3. 2; 4 + 2 = 6. | using only one SRI field in sTRP mode may not be enough for some max number of layers values. If switching from layer comb {1+1} to sTRP mode rank-2 transmission with SRS resource set 1, 4 bits are needed for SRI indication, but there are only 3 bits |  | using only one SRI field in sTRP mode may not be enough for some max number of layers values. |
| STxMP, NSRS,1=3, NSRS,2=3 LmaxSTxMP; SRI1+SRI2 length, in bits. 1; 2 + 2 = 4. 2; 3 + 3 = 6. |  | using only one SRI field in sTRP mode may not be enough for some max number of layers values. |  |
| STxMP, NSRS,1=3, NSRS,2=2 LmaxSTxMP; SRI1+SRI2 length, in bits. 1; 2 + 1 = 3. 2; 3 + 2 = 5. |  | using only one SRI field in sTRP mode may not be enough for some max number of layers values. | using only one SRI field in sTRP mode may not be enough for some max number of layers values. |
| STxMP, NSRS,1=2, NSRS,2=2 LmaxSTxMP; SRI1+SRI2 length, in bits. 1; 1 + 1 = 2. 2; 2 + 2 = 4. |  |  | using only one SRI field in sTRP mode may not be enough for some max number of layers values. |

(Notes: For the convenience of comparison, LmaxSTxMP represents the configured maximal number of layers applied to the 1st and 2nd SRS resource set in STxMP mode, and Lmax represents the configured maximal number of layers applied to single-TRP mode. NSRS,1 and NSRS,2 represents the number of SRS sources in the 1st and 2nd SRS resource sets respectively in STxMP mode.)

As shown in Table 1, the SRI field(s) lengths for STxMP mode and sTRP mode are compared with different maximal number of layers and number of SRS resources configurations. For many configurations, using only one SRI field in single-TRP mode are not enough.

For example, when (NSRS,1=4,NSRS,2=4,  LmaxSTxMP =1,  Lmax>1) or (NSRS,1=4, NSRS,2=3, LmaxSTxMP =1,  Lmax>1), either the length of the 1st or 2nd SRI field is not enough for a sTRP mode with 4 SRS resources, which needs 4 bits for the SRI field.

When (NSRS,1=3, NSRS,2=3, LmaxSTxMP =1,  Lmax>1), either the length of the 1st or 2nd SRI field is not enough for a sTRP mode with 3 SRS resources, which needs 3 bits for the SRI field.

When (NSRS,1=2, NSRS,2=2, LmaxSTxMP =1,  Lmax>1), either the length of the 1st or 2nd SRI field is not enough for a sTRP mode with 2 SRS resources, which needs 2 bits for the SRI field.  But using both the 1st SRI field and the 2nd SRI field to indicate the single SRI for single-TRP mode will not have this issue.

Thus, we have the following proposals.

Proposal 4. For sDCI based STxMP PUSCH SDM scheme, propose using both the 1st SRI field and the 2nd SRI field to indicate the single SRI when switching to single-TRP mode by DCI.

Proposal 5. For sDCI based STxMP PUSCH SFN scheme, both the 1st SRI field and the 2nd SRI field can be used to indicate the single SRI when switching to single-TRP mode by DCI.

Furthermore, there is one special case, i.e., (NSRS,1=4,NSRS,2=2, LmaxSTxMP =1,  Lmax>1),  that even using both SRI fields for single-TRP SRI indication is not enough to indicate the SRI for single-TRP mode, which are highlighted in red in Table 1. In this case, the total number of bits for the two SRI fields is 3, but 4 bits are needed to indicate the SRI for single-TRP mode.

In order to solve this issue, several methods can be considered – one is that gNB simply avoids this case via implementation, or an additional bit can be added or we can reduce the resolution of the SRI table when the total number of codepoints is not enough for single-TRP SRI.

Table 2. TPMI field length comparison

| Configurations | Length of TPMI field for codebookSubset = (FullyAnd…, PartialAnd…, NonCoherent) | Reference in TS38.212 |
| --- | --- | --- |
| 4 antenna ports, DFTsOFDM (or CP-OFDM, maxRank = 1), ul-FullPowerTransmission= absent/fullpowerMode2/(or fullpower) | [28, 12, 4] codepoints (5, 4, 2) bits | Table 7.3.1.1.2-3 (1st and 2nd TPMI) |
| 4 antenna ports, CP-OFDM, maxRank = 2/3/4, ul-FullPowerTransmission= absent/fullpowerMode2/fullpower | [62, 32, 12] codepoints (6, 5, 4) bits | Table 7.3.1.1.2-2 (1st TPMI) |
| 4 antenna ports, DFTsOFDM (or CP-OFDM, maxRank = 1), ul-FullPowerTransmission= fullpowerMode1 | [NA, 16, 5] codepoints (NA, 4, 3) bits | Table 7.3.1.1.2-3A (1st and 2nd TPMI) |
| 4 antenna ports, CP-OFDM, maxRank = 2, ul-FullPowerTransmission= fullpowerMode1 | [NA, 30, 12] codepoints (NA, 5, 4) bits | Table 7.3.1.1.2-2A (1st TPMI) |
| 4 antenna ports, CP-OFDM, maxRank = 3/4, ul-FullPowerTransmission= fullpowerMode1 | [NA, 36, 15] codepoints (NA, 6, 4) bits | Table 7.3.1.1.2-2B (1st TPMI) |
| 2 antenna ports, DFTsOFDM (or CP-OFDM, maxRank = 1), ul-FullPowerTransmission= absent/fullpowerMode2/fullpower | [6, NA, 2] codepoints (3, NA, 1) bits | Table 7.3.1.1.2-5 (1st and 2nd TPMI) |
| 2 antenna ports, CP-OFDM, maxRank = 2, ul-FullPowerTransmission= absent/fullpowerMode2/fullpower | [9, NA, 3] codepoints (4, NA, 2) bits | Table 7.3.1.1.2-4 (1st TPMI) |
| 2 antenna ports, DFTsOFDM (or CP-OFDM, maxRank = 1), ul-FullPowerTransmission= fullpowerMode1 | [NA, NA, 3] codepoints (NA, NA, 2) bits | Table 7.3.1.1.2-5A (1st and 2nd TPMI) |
| 2 antenna ports, CP-OFDM, maxRank = 2, ul-FullPowerTransmission= fullpowerMode1 | [NA, NA, 4] codepoints (NA, NA, 2) bits | Table 7.3.1.1.2-4A (1st TPMI) |

As agreed in previous meeting, for CB-based PUSCH, DCI indicates two TPMI fields, and each TPMI field separately indicates the precoding information and the number of layers conveyed over the SRS ports of the indicated SRS resource in each SRS resource set. There is no dependence between the lengths of the two TPMI fields in SDM STxMP transmission. Thus, the length of the TPMI field for single-TRP mode in current spec is a good reference for the SDM STxMP TPMI field design. The lengths of single-TRP TPMI field for different configurations are shown in Table 2. Similar to what have been observed in SRI field design, Table 2 shows that if only one TPMI field is used for the single TPMI indication when switching from SDM STxMP to single-TRP mode, either the 1st or 2nd TPMI field may not have the enough bits for the scenario that the single-TRP transmission has a larger ‘maxRank’. For example, as shown in the first 2 rows in green in the table, if ‘maxRank1’ = ‘maxRank2’ = 1, ‘maxRank’=2/3/4, ‘fullpowerMode2’ is configured, ‘codebookSubset = fullyAndPartialAndNonCoherent’, and there are 4 antenna ports in both panels, when switching from layer combination {1+1} to single-TRP mode with rank-2 transmission, 6 bits are needed for single-TRP TPMI indication. However, each TPMI field only has 5 bits. On the other hand, the total number of bits is 5+5=10, which is enough for the single-TRP mode TPMI indication with rank 2 and more.

(Notes: For the convenience of comparison, ‘maxRank1’ and ‘maxRank2’ represent the configured maximal ranks applied to the 1st and 2nd SRS resource set respectively in STxMP mode, and ‘maxRank’ is the configured maximal rank applied to single-TRP mode.)

Proposal 6. For sDCI based STxMP PUSCH SDM/SFN scheme, propose using both the 1st TPMI field and the 2nd TPMI field to indicate the single TPMI when switching to single-TRP mode by DCI.

Next, the codepoint design for dynamic switching when SDM STxMP and SFN STxMP are configured may follow Table 3 and Table 4, respectively. In particular, the first two codepoints ‘00’ and ‘01’ can be used to indicate single-TRP mode with different panel. The third codepoint ‘10’ can be used to indicate SDM or SFN STxMP mode and the last codepoint ‘11’ can be reserved.

Proposal 7. For dynamic switching between SDM/SFN scheme of single-DCI based STxMP PUSCH and sTRP transmission, the first two codepoints ‘00’ and ‘01’ can be used to indicate single-TRP mode with the 1st and 2nd SRS resource sets respectively. The third codepoint ‘10’ can be used to indicate SDM/SFN STxMP mode and the last codepoint ‘11’ can be reserved.

Table 3. Dynamic switching between sDCI SDM STxMP and sTRP transmission and SRI/TPMI field mapping when sDCI SDM STxMP is configured

| Codepoint | Transmission mode/SRS resource set | SRI (for both CB and NCB)/TPMI (CB only) field(s) to use |
| --- | --- | --- |
| 00 | s-TRP mode with 1st SRS resource set | Both 1st and 2nd SRI/TPMI fields |
| 01 | s-TRP mode with 2nd SRS resource set | Both 1st and 2nd SRI/TPMI fields |
| 10 | SDM STxMP mode with (TRP1, TRP2 order) 1st SRI/TPMI field: 1st SRS resource set 2nd SRI/TPMI field: 2nd SRS resource set | Both 1st and 2nd SRI/TPMI fields |
| 11 | Reserved | Reserved |

Table 4. Dynamic switching between sDCI SFN STxMP and sTRP transmission and SRI/TPMI field mapping when sDCI SFN STxMP is configured

| Codepoint | Transmission scheme/mode/SRS resource set | SRI (for both CB and NCB)/TPMI (CB only) field(s) to use |
| --- | --- | --- |
| 00 | s-TRP mode with 1st SRS resource set | Both 1st and 2nd SRI/TPMI fields |
| 01 | s-TRP mode with 2nd SRS resource set | Both 1st and 2nd SRI/TPMI fields |
| 10 | SFN STxMP mode with (TRP1, TRP2 order) 1st SRI/TPMI field: 1st SRS resource set 2nd SRI/TPMI field: 2nd SRS resource set | Both 1st and 2nd SRI/TPMI fields (the length of the 2nd SRI/TPMI field depends on the number of layers indicated by the 1st SRI/TPMI field) |
| 11 | Reserved | Reserved |

### PTRS-DMRS Association

#112 Agreement When max 2 PTRS ports are configured for SDM scheme of single-DCI based STxMP PUSCH: - Actual number of PTRS ports in SDM is 2 and 2-bit “PTRS-DMRS association” DCI field is used to indicate the PTRS-DMRS association for the DMRS ports associated with two TMPI/SRI fields. - The MSB indicates the association between PTRS port 0 and the DMRS port(s) associated with the first TPMI/SRI field. - The LSB indicates the association between PTRS port 1 and the DMRS port(s) associated with the second TPMI/SRI field. - Regarding the “ptrs-PortIndex” configured to SRS resource for NCB PUSCH of SDM scheme, the UE ignores the configuration of “ptrs-PortIndex” per SRS resource. FFS: Whether additional RRC configuration is needed for the max number of PTRS ports for SDM transmission #112 Agreement For single-DCI based STxMP SFN scheme, - Alt2: When maxNrofPorts = 2 is configured for PTRS in SFN scheme, the actual number of PTRS port(s) in SFN is determined by the 1st TPMI field for CB or 1st SRI field for NCB - Each PTRS port is transmitted in SFN manner #110be Agreement Support to configure up to 2 PTRS ports for SDM scheme of single-DCI based STxMP PUSCH transmission: - For 2 PTRS ports, study how to use the ‘PTRS-DMRS association’ field in DCI format 0_1 and 0_2 to indicate the PTRS-DMRS association for SDM scheme

In previous RAN1 meetings, we have agreed to configure up to 2 PTRS ports for SDM scheme and when max 2 ports are configured, the actual number of PTRS ports in SDM is 2. In SDM, the MSB and LSB corresponds to PTRS port0 and port1, respectively, where the PTRS port0 is associated with the first TPMI/SRI field and the PTRS port1 is associated with the second TPMI/SRI field.

For SDM scheme and SFN scheme, max 1 PTRS port configured scenario should also be specified. When max 1 PTRS port is configured for SDM scheme, Table 7.3.1.1.2-25 of TS38.212 can be re-used for PTRS-DMRS indication. When max 1 PTRS port is configured for SFN scheme, the PTRS port can be transmitted in SFN manner also, and Table 7.3.1.1.2-25 of TS38.212 can be used for PTRS-DMRS indication.

Proposal 8. When max 1 PTRS port is configured for SDM scheme of single-DCI based STxMP PUSCH, Table 7.3.1.1.2-25 of TS38.212 can be used for PTRS-DMRS indication.

Proposal 9. When max 1 PTRS port is configured for SFN scheme of single-DCI based STxMP PUSCH, the PTRS port can be transmitted in SFN manner, and Table 7.3.1.1.2-25 of TS38.212 can be used for PTRS-DMRS indication.

Furthermore, the DCI field of PTRS-DMRS association for SFN transmission can be legacy 2 bits if configured maxRank is larger than 2, while it can be shortened to 1 bit if the configured maxRank is no more than 2.

Proposal 10. For single-DCI based STxMP SFN scheme, the DCI field of PTRS-DMRS association can be shortened to 1 bit if the configured maximum number of layers is no more than 2.

Another issue is whether additional RRC configuration is needed for the max number of PTRS ports for SDM transmission. The use-case is where a single PTRS port is sufficient for sTRP operation while 2 PTRS ports is necessary for SDM operation. We note, however that for partial-coherent and non-coherent CB based UL transmission, the association between PUSCH antenna ports and PTRS ports is semi-static, i.e., PUSCH antenna port 1000 and 1002 in indicated TPMI(s) share PT-RS port 0, and PUSCH antenna port 1001 and 1003 in indicated TPMI(s) share PT-RS port 1 - therefore always indicating 1 actual PTRS port for sTRP using DCI may not be possible. However, this is not a critical issue.

Proposal 11. For single-DCI based STxMP PUSCH SDM transmission, additional RRC configuration for the max number of PTRS ports for SDM transmission may be beneficial but not critical.

# Multi-DCI Based STxMP UL Transmission

## Multi-DCI based STxMP PUSCH Transmission

#112 Agreement For multi-DCI based STxMP PUSCH+PUSCH, study enhancements of the UCI multiplexing rule to address the case that one PUCCH overlaps with two overlapped PUSCHs of STxMP PUSCH+PUSCH. #111 Conclusion (mDCI, no PUCCH+PUCCH) There is no consensus on the support STxMP PUCCH+PUCCH in multi-DCI based mTRP system #110be Agreement Support STxMP PUSCH+PUSCH transmission in multi-DCI based system in Rel-18. - Two independent PUSCHs associated with different TRPs can be transmitted by a UE simultaneously in same active BWP. - The total number of layers of these two PUSCHs is up to 4. - FFS: whether the number of layers of each of these two PUSCHs is up to 2.

In previous RAN1 meeting, we have agreed to study enhancements of the UCI multiplexing rule to address the case that one PUCCH overlaps with two overlapped PUSCHs of STxMP PUSCH+PUSCH.

In Rel-18 multi-DCI based STxMP UL transmission, there are different PUCCH and PUSCH collision scenarios, which can be identified as follows.

- Scenario-1: a single PUSCH collision with a single PUCCH transmission (single PUSCH + single PUCCH) but the PUSCH and PUCCH are targeting different TRPs.

- Scenario-2: a single PUCCH collision with STxMP PUSCH (single PUCCH + STxMP PUSCH).

Note that according to previous RAN1 conclusion, there is no consensus on the support STxMP PUCCH+PUCCH in multi-DCI based mTRP system. Scenario-2 is already agreed to be studied in last RAN1 meeting. we think that PUSCH + PUCCH transmission scenario-1 also needs to be addressed.

As for the UCI multiplexing and collision handling rules for multi-DCI based STxMP UL transmission, we think that in principle legacy rules can be applied for UCI handling within a panel separately. In scenario-2 (single PUCCH + STxMP PUSCH) UCI could be multiplexed on a PUSCH only if both are associated with the same joint/UL TCI state (shown in Figure 6). Same principle can be used for scenario-1 where UCI multiplexing is applied if PUSCH and UCI is associated with the same joint/UL TCI state. It is FFS whether to perform any specification work for the error cases – e.g. joint/UL TCI state associated with PUCCH does not match with those associated with any of the PUSCH.

Figure 5. Single panel PUCCH collides with multi-panel PUSCH

Proposal 12. For multi-DCI based STxMP UL transmission, UCI multiplexing according to legacy is applied if PUSCH and UCI is associated with the same joint/UL TCI state. This is applicable to both PUCCH + PUSCH and PUCCH + STxMP PUSCH cases. It is FFS whether any specification is needed for error cases e.g. joint/UL TCI state associated with PUCCH and PUSCH does not match.

## Multi-DCI based STxMP PUCCH Transmission

111 Conclusion There is no consensus on the support STxMP PUCCH+PUCCH in multi-DCI based mTRP system WID #94e for MIMO Evolution for Downlink and Uplink, Objective 6 Study, and if needed, specify the following items to facilitate simultaneous multi-panel UL transmission for higher UL throughput/reliability, focusing on FR2 and multi-TRP, assuming up to 2 TRPs and up to 2 panels, targeting CPE/FWA/vehicle/industrial devices (if applicable) - UL precoding indication for PUSCH, where no new codebook is introduced for multi-panel simultaneous transmission - The total number of layers is up to four across all panels and total number of codewords is up to two across all panels, considering single DCI and multi-DCI based multi-TRP operation. - UL beam indication for PUCCH/PUSCH, where unified TCI framework extension in objective 2 is assumed, considering single DCI and multi-DCI based multi-TRP operation - For the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC.

In last RAN1 meeting, a conclusion was made that there is no consensus on the support of STxMP PUCCH+PUCCH in multi-DCI based mTRP system. However, this conclusion contradicts the Objective 6 in the WID for MIMO evolution which states that for the case of multi-DCI based multi-TRP operation, only PUSCH+PUSCH, or PUCCH+PUCCH is transmitted across two panels in a same CC. As discussed in last RAN1 meeting, the major concern is the UCI multiplexing issue when multi-DCI based STxMP PUCCH + PUCCH is supported. However, we think applying the legacy collision handling rule to each panel could handle most of the scenarios. Thus, we prefer to support the multi-DCI STxMP PUCCH+PUCCH transmission.

Proposal 13. Support multi-DCI based STxMP PUCCH+PUCCH transmission.

# Power Headroom Report for STxMP

106e Agreement For PHR reporting related to M-TRP PUSCH repetition, support Option 4 as UE optional capability for a UE that supports mTRP PUSCH, - Option 4: Calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs. 106e Agreement For option 4, support the following: - When PHR MAC-CE is reported in slot n, for a CC that is configured with mTRP PUSCH repetition, second PHR value is determined as, - If the first PHR value is actual PHR (based on Rel. 15/16) corresponding to a repetition among mTRP PUSCH repetitions associated with a given TRP, the second PHR value, select Alt. 2A - Alt.2A: Is actual only when a repetition associated with the other TRP is transmitted in slot n. Otherwise, it is virtual. - If there are multiple repetitions associated with the other TRP in slot n, the earliest one in slot n is selected. - If the first PHR value is actual PHR (based on Rel. 15/16) but not corresponding to a repetition among mTRP PUSCH repetitions (corresponds to sTRP PUSCH), select Alt. 1B - Alt1B: a second PHR value is reported as virtual PHR. - If the first PHR value is virtual, select Alt. 1C - Alt1C: a second PHR value is reported as virtual PHR. - Note: It was agreed that when second PHR is virtual, it is calculated based on a set of default power control parameters defined for the other TRP (that is not associated with the first PHR) - Note: It was agreed that the above is applicable to both single entry and multi-entry PHR reports

In Rel-17 multi-TRP PUSCH repetition, two PHRs associated with the two TRPs are reported to the BS if the UE has such capability. Similarly, for STxMP PUSCH transmission, two PHRs corresponding to the two panels/TRPs should be reported. In other words, the PHR report for STxMP PUSCH transmission should be panel-specific.

Proposal 14. The PHR report for STxMP PUSCH transmission should be panel specific.

In current specification, the PHR report can be triggered by different events such as PHR timer expires, path loss changing more than threshold, reconfiguration, etc. And the triggering condition is not TRP specific. For example, there is only one phr-PeriodicTimer and one phr-ProhibitTimer.

In multi-DCI based STxMP PUSCH transmission, the time domain overlap scenario can be different if partial overlapping PUSCH is supported. PHR reporting can be triggered in different scenarios, such as 1) only one panel is scheduled for STxMP transmission, and 2) both panels are scheduled for STxMP transmission. For the second scenario, two PHRs should be transmitted. While for the first scenario, there can be two options, one option is reporting one PHR of the scheduled transmission, the other option is reporting one PHR of the scheduled transmission and one virtual PHR for panel that has not been scheduled for transmission. In our view, the second option is more preferrable since the PHR triggering is not panel/TRP specific and this option is more aligned with the Rel-17 PHR reporting for multi-TRP repetition.

Proposal 15. For multi-DCI based STxMP PUSCH transmission, when PHR report is triggered,

- if two panels are scheduled for STxMP transmission, two actual PHRs corresponding to the two panels/TRPs can be reported,

- if only one panel is scheduled for STxMP transmission, one PHR of the scheduled transmission and one virtual PHR for panel that has not been scheduled for transmission can be reported.

For single-DCI based STxMP PUSCH transmission, the two PUSCHs from two panels are always fully overlapped in time domain. Thus, two PHRs corresponding to the two panels/TRPs can be reported when the PHR report is triggered.

Proposal 16. For single-DCI based STxMP PUSCH transmission, two PHRs corresponding to the two panels/TRPs can be reported when the PHR report is triggered.

# UE Panel Association for STxMP

In previous RAN1 meetings and discussions, we have the following FL proposals and agreements.

#112 STxMP FL Proposal 2-4: For the multi-DCI based STxMP PUSCH+PUSCH transmission - Study the scenario where being capable of supporting STxMP PUSCH+PUSCH transmission might change to being incapable of supporting STxMP PUSCH+PUSCH, for example due to channel condition variation. - FFS: whether that is a valid scenario to deal with. - FFS: whether/how to deal with this scenario. - FFS: UE behaviour and system behaviour to deal with this scenario FFS: how to identify good Tx beam pairs for single-DCI based STxMP and multi-DCI based STxMP #108e Unified TCI Agreement On Rel.17 enhancements to facilitate UE-initiated panel activation and selection, support the UE reporting a list of UE capability value [sets] - Each UE capability value [set] comprises the max supported number of SRS ports - Any two capability values [sets] are different - Whether the UE capability value [set] can be common across all BWPs/CCs in same band or BC can be discussed in UE feature session

| Figure 6: UL STxMP Tx - L1-RSRP with panel selection (2Rx) | Figure 7: UL single TRP Tx - L1-RSRP using all panels (4Rx) |
| --- | --- |

We support the motivation for the NW to be informed on whether channel condition permits a UE to support STxMP PUSCH+PUSCH transmission. Channel condition is used in a general form to include scenarios due to UE rotation or switching power on/off at the UE side. It is equally important for the NW to know a UE moving from sTRP to STxMP mode and vice-versa. This can be achieved by the following steps:

- Step-0: UE reports ability to support STxMP transmission along with a UE capability value set that comprises of the max supported number of SRS ports (two capability values sets may have the same number of SRS ports)

- Step-1: gNB transmits SSBs/CSI-RSs in different CSI resource sets using different TRPs.

- Step-2: In a non- groupBasedBeamReporting setting (Figure 7), the UE measures the received the SSB/CSI-RS from all panels and reports L1-RSRP along with a UE capability value set index (panel-ID) in a CSI report. In a groupBasedBeamReporting setting (Figure 6), the UE measures the SSB/CSI-RS received from the best selected panel and reports L1-RSRPs where each L1-RSRP is associated with a value set index (panel-ID) indicating which panel is used for L1-RSRP measurement.

- Step-3: The NW could determine from the UCI reports whether the UE is suitable for STxMP transmission and also can determine pairs of TCI states that are compatible for STxMP.

Proposal 17. To support the scenario where a UE transitions between STxMP operation and sTRP operation, enhance the Rel-17 UE capability value set reporting to the case of symmetric panels with corresponding value set reporting in L1-RSRP reports (with groupBasedBeamReporting turned on/off).

# Conclusion

The main proposals in this paper are outlined as follows.

Proposal 1. For sDCI based STxMP PUSCH SDM scheme, confirm the working assumption Option-1 + Alt-1. It is FFS whether to support additional specifications for asymmetric UE panel structure.

Observation 1: Existing working assumption Option-1 + Alt-1 can support both shared and dedicated digital port architecture optimally in the case where #sTRP PUSCH ports is less than #SDM PUSCH ports.

Proposal 2. Support configuring a 2-port and a 4-port SRS resource in the same SRS resource set if there is a need to support the special case of #sTRP PUSCH ports = #SDM PUSCH ports

Proposal 3. For sDCI based STxMP PUSCH SFN scheme, support up to 4-layer transmission

Proposal 4. For sDCI based STxMP PUSCH SDM scheme, propose using both the 1st SRI field and the 2nd SRI field to indicate the single SRI when switching to single-TRP mode by DCI.

Proposal 5. For sDCI based STxMP PUSCH SFN scheme, both the 1st SRI field and the 2nd SRI field can be used to indicate the single SRI when switching to single-TRP mode by DCI.

Proposal 6. For sDCI based STxMP PUSCH SDM/SFN scheme, propose using both the 1st TPMI field and the 2nd TPMI field to indicate the single TPMI when switching to single-TRP mode by DCI.

Proposal 7. For dynamic switching between SDM/SFN scheme of single-DCI based STxMP PUSCH and sTRP transmission, the first two codepoints ‘00’ and ‘01’ can be used to indicate single-TRP mode with the 1st and 2nd SRS resource sets respectively. The third codepoint ‘10’ can be used to indicate SDM/SFN STxMP mode and the last codepoint ‘11’ can be reserved.

Proposal 8. When max 1 PTRS port is configured for SDM scheme of single-DCI based STxMP PUSCH, Table 7.3.1.1.2-25 of TS38.212 can be used for PTRS-DMRS indication.

Proposal 9. When max 1 PTRS port is configured for SFN scheme of single-DCI based STxMP PUSCH, the PTRS port can be transmitted in SFN manner, and Table 7.3.1.1.2-25 of TS38.212 can be used for PTRS-DMRS indication.

Proposal 10. For single-DCI based STxMP SFN scheme, the DCI field of PTRS-DMRS association can be shortened to 1 bit if the configured maximum number of layers is no more than 2.

Proposal 11. For single-DCI based STxMP PUSCH SDM transmission, additional RRC configuration for the max number of PTRS ports for SDM transmission may be beneficial but not critical.

Proposal 12. For multi-DCI based STxMP UL transmission, UCI multiplexing according to legacy is applied if PUSCH and UCI is associated with the same joint/UL TCI state. This is applicable to both PUCCH + PUSCH and PUCCH + STxMP PUSCH cases. It is FFS whether any specification is needed for error cases e.g. joint/UL TCI state associated with PUCCH and PUSCH does not match.

Proposal 13. Support multi-DCI based STxMP PUCCH+PUCCH transmission.

Proposal 14. The PHR report for STxMP PUSCH transmission should be panel specific.

Proposal 15. For multi-DCI based STxMP PUSCH transmission, when PHR report is triggered,

- if two panels are scheduled for STxMP transmission, two actual PHRs corresponding to the two panels/TRPs can be reported,

- if only one panel is scheduled for STxMP transmission, one PHR of the scheduled transmission and one virtual PHR for panel that has not been scheduled for transmission can be reported.

Proposal 16. For single-DCI based STxMP PUSCH transmission, two PHRs corresponding to the two panels/TRPs can be reported when the PHR report is triggered.

Proposal 17. To support the scenario where a UE transitions between STxMP operation and sTRP operation, enhance the Rel-17 UE capability value set reporting to the case of symmetric panels with corresponding value set reporting in L1-RSRP reports (with groupBasedBeamReporting turned on/off).

# References

- 3GPP RAN1, Chairman’s Note, 3GPP RAN1 #111, November 2022.

- RP-213598, “New WID: MIMO Evolution for Downlink and Uplink”, TSG RAN Meeting #94e, Samsung