3GPP TSG RAN WG1 #112bis-e  													                     R1-2302788

e-Meeting, April 17th – April 26th, 2023

Source: 	Intel Corporation

Title:	Dynamic switching between DFT-s-OFDM and CP-OFDM waveform

Agenda item:	9.12.3

Document for:	Discussion and Decision

# Introduction

At the RAN1#112 meeting, the following agreements and conclusions were made regarding dynamic switching between DFT-s-OFDM and CP-OFDM waveform [1]:

Agreement

For single TB scheduled by single DCI, support new 1-bit field for dynamic waveform indication from UL scheduling DCI.

Note: no change of the current size alignment procedure between UL DCI and DL DCI.

Conclusion

There is no consensus to support “Dynamic waveform switching to PUSCH transmissions with a Type 2 configured grant” in R18.

Agreement

- Dynamic waveform switching in R18 is not applicable to PUSCH transmissions with a Type 1 configured grant.

Conclusion

The dynamic waveform indication in a DCI containing a dynamic uplink grant applies only to PUSCH transmission(s) corresponding to the dynamic uplink grant.

In the contribution, we present our views on dynamic switching between DFT-s-OFDM and CP-OFDM for PUSCH transmission. Our views on PRACH coverage enhancement and power domain enhancement are described in our companion contributions [2] and [3], respectively.

# Discussion on dynamic waveform indication

## DCI size alignment between CP-OFDM and DFT-s-OFDM

As defined in NR, PUSCH transmission can be configured with either DFT-s-OFDM or CP-OFDM waveform via RRC signalling. In general, for cell edge UEs with poor channel conditions, it is more appropriate to utilize DFT-s-OFDM waveform for PUSCH transmission for better coverage due to low PAPR. On the contrary, for cell center UEs with high SNR, CP-OFDM waveform is more beneficial given the fact that it can support UL MIMO with high rank transmission so as to provide high data rate and improve system capacity.

To fully exploit the benefit of DFT-s-OFDM or CP-OFDM waveform for PUSCH transmission in a timely manner, dynamic switching of waveform type can be supported for PUSCH transmission. At the RAN1#112 meeting, it was agreed that for single TB scheduled by single DCI, a new 1-bit field for dynamic waveform indication from UL scheduling DCI is supported [1]. In the DCI for scheduling PUSCH, given that some DCI fields for CP-OFDM and DFT-s-OFDM waveform may have different sizes, UL DCI payload size needs to be determined to avoid ambiguity between UE and gNB. In particular, the following DCI field size may be different for CP-OFDM and DFT-s-OFDM waveforms:

- Precoding information and number of layers

- Antenna ports

- PTRS-DMRS association

- SRS resource indicator (SRI)

- DMRS sequence initialization

At the RAN1#112 meeting, the following options were discussed for DCI size alignment between CP-OFDM and DFT-s-OFDM waveform when dynamic waveform switching is configured in the DCI format 0_1 or 0_2 [4].

FL proposal 2-4r2: RAN1 to select one of the following Options for DCI size alignment when DWS indication is configured in DCI format 0_1 or 0_2: Option 1: Per-format alignment - Step 1: Determine field sizes of DCI addressed to C-RNTI based on the waveform indicated by DWS field. - Step 2: If DCI is addressed to CS-RNTI, align field sizes to that of DCI addressed to C-RNTI based on the waveform indicated by DWS field using the existing C-RNTI/CS-RNTI alignment clause. - Select between: - Alt. A: - By specification, at least one field preceding NDI has size that depends on waveform (e.g. FDRA). - Note: This implies DWS field must be decoded before finding position of NDI field. - Note: This implies DWS field must be positioned before the first field that depends on waveform. - Impact on procedure for PDCCH validation of CG type 2: if DWS field value does not match waveform RRC-configured in ConfiguredGrantConfig for the CG, and NDI=0, it is an error case. - Alt. B: - By specification, no field preceding NDI has size that depends on waveform. - Note: It seems that it is currently the case for DCI format 0_1/0_2 in R17 (may need further checking) - Note: This means DWS field does not need to be decoded before finding position of NDI field. - Note: This means DWS field can be positioned after NDI, but before the first field that depends on waveform. - No impact on procedure for PDCCH validation of CG type 2. UE can ignore value of DWS field in the validation. - Step 3: Apply padding at the end of the DCI, if necessary, to match the size of the largest DCI size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). Option 2: Per-field alignment - Step 1: For C-RNTI, apply padding to each field, if necessary, to match the size of the largest field size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). - Step 2: If DCI is addressed to CS-RNTI, align field sizes to those of C-RNTI according to existing alignment clause between CS-RNTI and C-RNTI. - Note: DWS field does not need to be decoded before finding position of NDI field or any other field. - Note: No constraint on position of DWS field in DCI (but positioning before any field that has waveform-dependent interpretation may help implementation). Option 3: Per-field alignment only for fields preceding NDI field - Step 1: For fields preceding NDI, apply per-field alignment as per Steps 1 and 2 of Option 2. - Note: This means DWS field does not need to be decoded before finding position of NDI field. - Step 2: For fields following NDI, determine field sizes based on DWS as per Steps 1 and 2 of Option 1. - Step 3: Apply padding at the end of the DCI, if necessary, to match the size of the largest DCI size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). - Note: Option 3 has same features as Option1/Alt-B with respect to impact on PDCCH validation and positioning of DWS field. In case all fields preceding NDI are guaranteed to have size independent of waveform by specification, Option 3 may not be relevant. Option 4: Per-field alignment only for DCI addressed to CS-RNTI - For DCI addressed to C-RNTI: - Step 1: Determine size of fields based on DWS indication. - Note: This implies DWS field must be positioned before the first field that depends on waveform. - Step 2: Apply padding at the end of the DCI, if necessary, to match the size of the largest DCI size between (DWS=CP-OFDM) and (DWS=DFT-S-OFDM). - For DCI addressed to CS-RNTI: - Apply padding to each field, if necessary, to match the size of the corresponding field for the DCI addressed to C-RNTI and (DWS=CP-OFDM). - Note: This means DWS field does not need to be decoded before finding position of NDI field in the case of CS-RNTI. - Note: Field introduced in future specs needs to have size for CP-OFDM at least equal to size for DFT-S-OFDM, or otherwise cannot be used for CS-RNTI and NDI=1.

Figure 1 illustrates the per field or per DCI format alignment for dynamic waveform switching. For these options, existing mechanisms on DCI size alignment between DCI format addressed to C-RNTI and CS-RNTI can be reused. Further, if FDRA field size alignment can be ensured for different waveforms, Option 3 can be viewed as Alt. B in Option 1. In this case, dynamic waveform switching field can be placed after NDI field without ambiguity between UE and gNB side.

Figure 1. Per field or per DCI format alignment for dynamic waveform switching

Note that Option 1 with per DCI format based alignment for different waveforms may be simpler in terms of implementation complexity. In particular, gNB or UE does not need to check the individual field between CP-OFDM and DFT-s-OFDM waveform when constructing the DCI format. Additional benefit of per DCI format alignment may be overhead DCI size reduction. However, as only 1 bit for dynamic waveform indication is included in the DCI format 0_1 or 0_2, the DCI size reduction may not be expected, especially when considering that the UL DCI size is typically smaller than that for DL DCI size.

One drawback for per DCI format alignment is the placement of dynamic waveform indication field. Depending on different options, dynamic waveform indication field may need to be placed before NDI field or waveform dependent field to avoid a misunderstanding between gNB and UE side. In addition, considering the potential support of dynamic waveform switching for multi-cell scheduling, this mechanism may result in additional complexity in the placement of dynamic waveform indication field and DCI size alignment.

For Option 2, the basic principle of per field alignment is similar to what was defined for DCI size alignment between DCI addressed to C-RNTI and CS-RNTI. For this option, dynamic waveform indication field may be placed independent of NDI or waveform dependent field, which may be more flexible on the DCI format design and for forward compatible design. Similar design principle was also supported for BWP switching, where per field size alignment between current active BWP and target BWP was defined.

Based on the discussions above, it may be more desirable to support Option 2 with per field alignment for DCI size alignment between CP-OFDM and DFT-s-OFDM waveform when dynamic waveform switching is configured in the DCI format 0_1 or 0_2. Note that typically, for these DCI fields, the size for CP-OFDM waveform is larger than that for DFT-s-OFDM waveform. In this case, to ensure a common understanding between gNB and UE, DCI size can be matched assuming CP-OFDM waveform for dynamic waveform switching.

Proposal 1:

- DCI size alignment between CP-OFDM and DFT-s-OFDM waveform when dynamic waveform switching is configured in the DCI format 0_1 or 0_2, per field alignment (Option 2) is supported.

## Dynamic waveform switching for multi-PUSCH scheduling

In Rel-16 NR-U, multi-PUSCH scheduling was defined to allocate consecutive PUSCH transmissions so as to avoid unnecessary LBT. Similarly, in Rel-17 FR2-2, multi-PUSCH scheduling was further extended to support non-consecutive time domain resource allocation for multiple PUSCH transmissions within a cell.

To fully exploit the benefit of DFT-s-OFDM or CP-OFDM waveform for PUSCH transmission in a timely manner, dynamic waveform switching can be applied for multi-PUSCH scheduling. Further, given that multiple PUSCH transmissions in the scheduled slots typically experience similar channel conditions, some DCI fields including frequency domain resource allocation can be commonly applied for all the scheduled PUSCHs for multi-PUSCH scheduling. Following this design principle, the new 1-bit field for dynamic waveform indication can be included in the DCI for multi-PUSCH scheduling, which can be commonly applied for all the scheduled PUSCHs.

Proposal 2:

- Dynamic waveform switching is supported for multi-PUSCH scheduling.

  - New 1-bit field for dynamic waveform indication is applied for all the scheduled PUSCHs.

Further, in Rel-18, multi-cell scheduling was introduced, where a single DCI can be used to schedule PUSCHs in different cells. The main motivation of multi-cell scheduling is to reduce PDCCH blocking probability. As PUSCH can be either scheduled by single cell scheduling or by multi-cell scheduling, the concept of dynamic waveform switching can be straightforwardly applied for multi-cell scheduling.

Note that different types of DCI fields were introduced for multi-cell scheduling [5]. In particular, for Type-1A field, a single field indicates common information to all the co-scheduled cells; for Type-1B field, a single field indicates separate information to each of co-scheduled cells via joint indication; for Type-1C field, a single field indicates information to only one of co-scheduled cells; for Type 2 field, separate fields are included for each of the co-scheduled cells.

For dynamic waveform indication, given that channel conditions for different carriers can be vastly different, it may not be desirable to consider Type-1A field for multi-cell scheduling. One straightforward option is to consider Type 2 field for dynamic waveform indication in the DCI format 0_X, but at the cost of large DCI overhead. In our view, further discussion is needed on the type of dynamic waveform indication in the DCI format 0_X with the consideration of scheduling flexibility and DCI overhead.

Proposal 3:

- Dynamic waveform switching is supported for multi-cell PUSCH scheduling.

  - FFS: type of dynamic waveform indication field in the DCI format 0_X.

# Application of dynamic waveform switching

At the RAN1#112 meeting, it was agreed that dynamic waveform switching in Rel-18 is not applicable to Type 1 CG-PUSCH. Further, it was concluded that there is no consensus to support dynamic waveform switching for Type 2 CG-PUSCH in Rel-18 [1]. Note that the following scenarios need to be further considered on whether dynamic waveform switching can be applied:

- PUSCH scheduled by DCI format 0_0

- Msg3 PUSCH transmission and retransmission

In current specification, when PUSCH is scheduled by DCI format 0_0, the default waveform type configured by msg3-transformPrecoder is applied for the PUSCH transmission. Given that DCI format 0_0 is mainly targeting fallback operation, it may not be preferrable to further increase the payload size for DCI format 0_0 when utilizing explicit indication for dynamic waveform switching. In our view, dynamic waveform switching enhancement is not applicable to PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI. In this regard, for PUSCH scheduled by DCI format 0_0, the existing mechanism for waveform type indication can be reused.

Proposal 4:

- Dynamic waveform switching enhancement is not applicable to PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI.

For Msg3 PUSCH initial transmission or retransmission, whether DFT-s-OFDM or CP-OFDM waveform is applied is determined according to configured parameter msg3-transformPrecoder. In Rel-17 NR coverage enhancement WI, repetition is supported for Msg3 PUSCH during 4-step RACH procedure, which mainly aims to improve the coverage. Note that for cell edge UEs that need coverage enhancement, it is expected that DFT-s-OFDM waveform can be employed so as to provide better coverage for Msg3 transmission.

However, even for UEs with poor channel conditions, UE still needs to follow the cell-specifically configured parameter msg3-transformPrecoder to determine the waveform type. In case the network configures CP-OFDM waveform as default waveform type, this would certainly degrade the Msg3 performance for these UEs.

To improve the reliability of Msg3 transmission, it may be more beneficial to incorporate each UE’s own channel conditions, instead of relying on cell specific parameter, to determine the waveform type during 4-step RACH procedure. Hence, in our view, similar to the PUSCH transmission scheduled by non-fallback DCI, dynamic waveform switching enhancement is applicable for Msg3 PUSCH initial transmission and retransmission.

Proposal 5:

- Dynamic waveform switching enhancement is applicable for Msg3 PUSCH initial transmission and retransmission.

# Conclusions

In this contribution, we presented our views on dynamic switching between DFT-S-OFDM and CP-OFDM for PUSCH transmission. Further, we summarize the proposals as follows:

Proposal 1:

- DCI size alignment between CP-OFDM and DFT-s-OFDM waveform when dynamic waveform switching is configured in the DCI format 0_1 or 0_2, per field alignment (Option 2) is supported.

Proposal 2:

- Dynamic waveform switching is supported for multi-PUSCH scheduling.

  - New 1-bit field for dynamic waveform indication is applied for all the scheduled PUSCHs.

Proposal 3:

- Dynamic waveform switching is supported for multi-cell PUSCH scheduling.

  - FFS: type of dynamic waveform indication field in the DCI format 0_X.

Proposal 4:

- Dynamic waveform switching enhancement is not applicable to PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI.

Proposal 5:

- Dynamic waveform switching enhancement is applicable for Msg3 PUSCH initial transmission and retransmission.

# References

- Chairman’s notes, RAN1#112 Meeting, March 2023

- R1-2302818, “Discussions on PRACH coverage enhancement”, Intel Corporation, April 2023

- R1-2302787, “Discussions on power domain enhancement”, Intel Corporation, April 2023

- R1-2302222, “Summary #4 on dynamic switching between DFT-S-OFDM and CP-OFDM”, InterDigital, February 2023

- Chairman’s notes, RAN1#110 Meeting, August 2022