3GPP TSG RAN WG1 #112b-e																			R1-2302789

e-Meeting, April 17th – 26th, 2023

Source:	Intel Corporation

Title:	General aspects of AI/ML framework for NR air interface

Agenda item:	9.2.1

Document for:	Discussion and Decision

# Introduction

A new study item on Artificial Intelligence (AI) / Machine Learning (ML) for NR air interface has been approved in [1]. One of the study item objectives includes terminology and description to identify common and specific characteristics for the AI/ML framework investigations and life cycle management (LCM). In this contribution we discuss general framework and LCM characteristics based on past discussions.

# General framework diagram

Figure 1: Functional framework

The general framework diagram above is modified from the RAN3 functional framework. The terminology agreed in RAN1 (shown below) is used to describe the different components:

| Terminology | Description |
| --- | --- |
| Data collection | A process of collecting data by the network nodes, management entity, or UE for the purpose of AI/ML model training, data analytics and inference |
| Model training | A process to train an AI/ML Model [by learning the input/output relationship] in a data driven manner and obtain the trained AI/ML Model for inference |
| Model Inference | A process of using a trained AI/ML model to produce a set of outputs based on a set of inputs |
| Model validation | A subprocess of training, to evaluate the quality of an AI/ML model using a dataset different from one used for model training, that helps selecting model parameters that generalize beyond the dataset used for model training. |
| Model testing | A subprocess of training, to evaluate the performance of a final AI/ML model using a dataset different from one used for model training and validation. Differently from AI/ML model validation, testing does not assume subsequent tuning of the model. |
| Model transfer | Delivery of an AI/ML model over the air interface, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model. |
| Model monitoring | A procedure that monitors the inference performance of the AI/ML model |
| Model activation | enable an AI/ML model for a specific function |
| Model deactivation | disable an AI/ML model for a specific function |
| Model switching | Deactivating a currently active AI/ML model and activating a different AI/ML model for a specific function |

Proposal-1: Support a modification of the RAN3 functional framework (37.817) by using components according to the agreed terminology in RAN1

# Data collection for training

Data collection for training is a critical aspect for practical deployments. Depending on the use-cases, data is generated at the NW and/or at the UE. Data-collection could be in real-time or offline. It is also a possibility that data generated at the UE is transported to the NW. The data collection entity could be a 3GPP entity or a non-3GPP entity, a non-3GPP entity could be a UE-side or a NW side entity. It is obvious that for a non 3GPP entity interfaces need not be specified, but RAN1 can discus and specify enhancements to facilitate data-collection at the UE and at the NW.

In order to facilitate UE-side data collection, NW could share assistance information like gNB panel/beam/virtualization information relevant to CSI-RS transmission to the UE. This allows the UE (or UE-side) to collect data specific to a scenario/site/antenna configuration. Similarly in order to facilitate NW-side data collection UE could share panel/beam/virtualization information to the NW.

Proposal-2: Support assistance information from NW to UE and UE to NW to facilitate data collection for training at the UE and at the NW respectively

# Model: logical vs physical, model-ID

Physical Model: An algorithm description, that, in conjunction with one or more UE specific customizations like quantization, compilation uniquely defines a practical AI-ML implementation. As an example, such a description is necessary for the purposes of transfer, training or development of a model.

Logical Model: An algorithm description that is sufficient for UE and gNB to have common understanding for a specific signaling purpose. Many aspects of the algorithm that has no bearing on signaling specification can be transparent to this model. As an example, such a description is sufficient for the purposes of a gNB monitoring a one-sided model (UE sided) through specification defined signaling. In general, we believe RAN1 specifications should primarily refer to logical models.

It is obvious that a logical model can be associated or implemented via one or more physical models. Model identification applies to both physical and logical models.

The physical or logical model context (usage) could be envisioned as follows:

| Usage | Physical model | Logical model |
| --- | --- | --- |
| Model identification process | X | X |
| Model development and training | X |  |
| Model transfer/delivery (parameters or structure + parameters) | X |  |
| UE capability reporting | X | X |
| Model inference |  | X |
| Model performance monitoring |  | X |
| Model selection, activation, deactivation, switching, fallback |  | X |

As we can see from the above, in different stages of LCM different interpretations of a model (physical or logical) makes natural sense. Therefore model-ID as associated with model identification process is associated with a physical model – and does not make sense to be used in the context of usages relevant for a logical model.

Further, the uniqueness of a model-ID should be considered carefully. From an air-interface perspective it seems perfectly feasible for the NW to use the same model-ID to identify different physical models (for different UEs for example) since the meta-information for the model clarifies the distinction to the NW.

Proposal-3: Interpretation of a “model” (physical or logical) depends on context of usage. Model-ID from a model identification process could be associated with a physical or a logical model. From RAN1 perspective there is no need for uniqueness (global or otherwise) of such a model-ID

# NW-side model development and training

Model development and training could potentially occur at a 3GPP or a non-3GPP node, it can be a NW-side, a UE-side or a neutral-side server. Model training, particularly training on a developed model could occur in an NW-side node. One of the key benefits of such an approach is to customize a model to the NW environment with readily available data. The NW could naturally collect channel data that reflects its own hardware (antenna) and local environment (room, shopping mall, bus-station etc.). A NW trained model (or model parameters) can be expected to achieve consistent performance with a model that is smaller in size and complexity compared to a model that has to be generalized across many unseen scenarios. Model transfer/delivery to the UE can happen through 3GPP or non-3GPP mechanisms.

Proposal-4: Model training, particularly after an initially developed model is acquired, should be supported at a NW-side server that allows customization of a model to the local environment. Subsequent model transfer/delivery to the UE can happen through 3GPP or non-3GPP mechanisms

In terms of model format, an open-format model (like ONNX) obviously allows interoperability and NW-side training of a model – it has been argued this can be undesirable as it leads to disclosure of model implementation. However, it is possible that a proprietary format may be recognized by a closed group of UE/NW vendors that allows training at the NW-side or a model parameter set trained at the UE-side is hosted at the NW. Therefore we have the following proposal

Proposal-5: For model transfer in a proprietary format support a NW to update the parameters of a model at the UE – (including indirect means involving a UE-side server if needed)

# Model identification process

Model identification is a process involving 3GPP signaling or otherwise through which the NW acquires meta information associated with a model (and a model-ID is associated) – this could be a physical model if model transfer/delivery is applicable and if not, a logical model. The model identification process is distinct from model transfer but it is a step that is required for model transfer. A model identification process involves a model structure and parameters – including the case of a model structure with a set of default parameters (not trained).

Model transfer (i.e. either model parameters or model + parameters) from UE to NW requires prior knowledge of model identification at the NW – this allows a NW to match a physical model with a UE (“seen” model at the UE). In the case where a parameter update occurs from the NW to the UE directly (without a new model identification process) it may be considered as an update of the existing model and the NW and UE may re-synchronize in terms of the associated meta-information (if needed). This is sufficient for LCM. We don’t see a need to consider a variant of currently defined model identification procedure from NW to UE.

Proposal-6: Model identification is a process through which NW acquires meta information associated with a physical or a logical model. There is no need to define another model identification process from NW to UE

# Model ID based and functionality-based LCM

Functionality based-LCM

Functionality based LCM is based on the current structure for UE capability reporting and UE feature management by the gNB. It does not require an identification procedure and consequently it does not apply to model transfer/delivery. In other words, it is not applicable of collaboration level-z. In this case the NW manages the LCM at the granularity of functionality while the UE is responsible for model-level LCM. It is applicable to 1-sided models only because for 2-sided models, the NW needs to be aware at the granularity of model level.

Functionality will be defined in the specification and any UE that advertises the support of such a function is treated exactly in the same way by a gNB. A gNB is unaware of any additional information regarding the implementation or underlying models of such a functionality.

Model-ID based LCM

A model-ID based LCM requires a model identification process where meta information associated to a model is acquired by the NW. Subsequent to this, a UE capability type reporting may be used to report model availability at the UE. This fundamentally differs from current UE feature framework as it allows a gNB to be aware of “meta-information” specific to an implementation of a feature in a UE. This allows a gNB to interact with two UEs differently even though they support the same feature and is indistinguishable in the current UE feature framework.

Proposal-7: A model-ID based LCM requires a model identification process – in contrast, a functionality based LCM is not associated with an identification procedure, it is not applicable for model transfer/delivery (collaboration level-z) and it is not applicable to two-sided models

Applicable conditions and meta-information

Applicable conditions for functionality and meta-information for model identification - both of them provides information on the details of the underlying model (input/output/scenarios/performance etc.). However applicable condition for functionality is not based on identification – the application conditions may be dynamically updated (site-specific for example) or not but still not requiring an identification process. In this way it is very much like current UE features or a more dynamic extension of it. There is no need to define applicable condition separately - the description of functionality naturally includes applicable condition – it can be static or dynamically changed/reported.

Meta information, on the other hand, contains description that is not pre-defined in the specification and it is information that is disclosed using an identification process (to the NW) for the purposes of LCM.

Comparing model-ID based LCM and functionality based LCM

With the above understanding, we think the two LCM approaches requiring identification (model-ID based) and without requiring identification (functionality based) can be developed separately. How they work with each other or relate to each other is not a critical issue to be addressed at this point.

| Model-ID based LCM | Functionality based LCM |
| --- | --- |
| NW controls LCM at physical model or logical model granularity | NW controls LCM at functionality level |
| Requires model identification process | No model identification process |
| Applicable with or without model transfer/delivery | Not applicable for model transfer/delivery |
| Applicable to both 2-sided and 1-sided models | Not applicable to 2-sided models |

Proposal-8: we suggest to develop model-ID based LCM and functionality based LCM procedures separately – it is not critical to address how they work with each other at this time

# Conclusions

Proposal-1: Support a modification of the RAN3 functional framework (37.817) by using components according to the agreed terminology in RAN1

Proposal-2: Support assistance information from NW to UE and UE to NW to facilitate data collection for training at the UE and at the NW respectively

Proposal-3: Interpretation of a “model” (physical or logical) depends on context of usage. Model-ID from a model identification process could be associated with a physical or a logical model. From RAN1 perspective there is no need for uniqueness (global or otherwise) of such a model-ID

Proposal-4: Model training, particularly after an initially developed model is acquired, should be supported at a NW-side server that allows customization of a model to the local environment. Subsequent model transfer/delivery to the UE can happen through 3GPP or non-3GPP mechanisms

Proposal-5: For model transfer in a proprietary format support a NW to update the parameters of a model at the UE – (including indirect means involving a UE-side server if needed)

Proposal-6: Model identification is a process through which NW acquires meta information associated with a physical or a logical model. There is no need to define another model identification process from NW to UE

Proposal-7: A model-ID based LCM requires a model identification process – in contrast, a functionality based LCM is not associated with an identification procedure, it is not applicable for model transfer/delivery (collaboration level-z) and it is not applicable to two-sided models

Proposal-8: we suggest to develop model-ID based LCM and functionality based LCM procedures separately – it is not critical to address how they work with each other at this time

# References

| - [1] | - RP-213599 “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,” Qualcomm, December 6 – 17, 2021. |
| --- | --- |
| - [2] | - Chairman notes of RAN1 #112 meeting |