3GPP TSG RAN WG1 #112b-e			      R1-2302790

e-Meeting, April 17th – 26th, 2023

Source: 	Intel Corporation

Title: 	Evaluation for CSI feedback enhancements

Agenda item:	9.2.2.1

Document for: 	Discussion and Decision

# Introduction

A new study item on Artificial Intelligence (AI) / Machine Learning (ML) for NR air interface has been approved in [1]. One of the study objectives includes the analysis of solutions for CSI feedback enhancements:

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project

In this contribution, we present a set of evaluation results for spatial-frequency domain CSI compression using a two-sided AI/ML model sub use case with different deployment scenarios along with generalization performance study for different channels and scenarios for that sub use case as well as evaluation results for Layer-common/Layer-specific models and different ground truth CSI quantization assumptions for data collection.

Evaluation for spatial-frequency domain CSI compression using two-sided AI/ML model

For evaluation of AI/ML applications spatial-frequency domain CSI compression with two-sided AI/ML model is used and the performance of this AI/ML model is compared with that of Rel-16 Enhanced Type II PMI codebook (eType II) with respect to the square generalized cosine similarity (SGCS) and UE throughput. The same testing dataset is used to test the performance of the AI/ML model as well as the eType II codebook.

## AI/ML model description

In this paper the AI/ML model implementation is based on the transformer architecture [2]. The AI/ML model uses channels generated from the SLS as input after a pre-processing step which is discussed in more details in the following sections.

### Data Pre- and Post-processing

The NF×NR×NT channel matrix is generated by using SLS, where NF is the number of PRBs for a given BW, NR is the number of receive antenna ports at the UE and NT is the number of transmit antenna ports at the gNB. Then it is converted to an input matrix V of size NS×2NT as follows. First, the covariance of the channel H over each sub-band (4 PRBs) is calculated and then, for layer k, the kth strongest eigen-vectors is taken.

The size of the complex matrix V is NS×NT where NS denotes the number of sub-bands. The input matrix V of the AI/ML model is constructed by concatenating the real and imaginary values of each element of matrix V in the column dimension making V of size NS×2NT.

The output from the AI/ML model is an estimate of the input matrix, V, which is an estimate of the eigenvector of channel covariance RH.

### CSI Autoencoder Architecture

The autoencoder (AE) AI/ML model used in this paper is based on a transformer architecture which uses the multi-head attention module from [2]. At the encoder, the input matrix is passed through a dense embedding layer which expands the column dimension from 2NT to 6×2NT. After positional encoding, the data is passed through cascaded multi-head attention blocks and finally through a dense layer into a uniform quantizer. At the decoder, a dequantizer is first used followed by a dense layer, embedding and positional encoding. This is followed by a series of cascaded multi-head attention modules and a dense output layer with no activation.

Figure 1: Transformer Based CSI Autoencoder

### Quantization

A uniform B-Bit quantizer is used in the AE as shown in Figure 3. The quantizer is non-trainable i.e., the gradients of the backpropagation during training are passed through the quantizer without any change. A fully connected layer preceding the quantizer reduces the total number of inputs to the quantizer to K channels by compressing it by a factor of β. The total number of inputs to the fully connected layer is given by 2×NS×NT where NT=N1×N2×P where N1,N2 are the antenna ports in horizontal and vertical planes and P is the number of polarizations. The FC layer outputs K channels where K=⌊2NSNT/β⌋. The quantizer uniformly quantizes each of the K inputs to produce KB feedback bits.

Figure 2: Uniform Quantization in CSI Autoencoder

The de-quantizer and FC layer in the decoder reverses the operations of the encoder quantizer and FC layer respectively. In this paper, a noiseless wireless channel is assumed i.e., during testing there is no corruption of the feedback bits.

## Intermediate metrics

The AI/ML model is trained on 540,000 samples with 15000 validation samples and tested on 45000 samples. The output of the AI/ML model H is then compared to the original channel eigenvector H to evaluate SGCS.

In the following sections, the performance of AI/ML CSI for different SLS channel models are shown for the case of Layer 1 and Layer 2 transmission for different overhead feedback bits.

### Indoor Hotspot (InH)

In Indoor Hotspot scenario, the AI/ML easily outperforms the eType II CSI for both high and low overhead values as seen in Table 1 and Figure 3. Layer 1 performs significantly better than Layer 2 for the low overhead bits and is comparable for higher overhead bits.

Table 1: AI/ML Performance for Indoor Hotspot

| TX Ports (NT) | Overhead Bits (KB) | Cosine Similarity (ρ) : Layer 1 | Cosine Similarity (ρ) : Layer 2 |
| --- | --- | --- | --- |
| 32 | 48.00 | 0.8336 | 0.719 |
|  | 62.00 | 0.8643 | 0.7624 |
|  | 96.00 | 0.9154 | 0.8337 |
|  | 128.00 | 0.9402 | 0.88 |
|  | 168.00 | 0.9555 | 0.9198 |
|  | 224.00 | 0.9676 | 0.9471 |
|  | 278.00 | 0.99767 | 0.9594 |

Figure 3: Cosine Similarity of InH for AI/ML AE vs Rel-16 eType II Codebook for layer 1 and layer 2

### Dense Urban Macro

For Dense Urban Macro with both outdoor and indoor UEs, the AI-ML transformer model outperforms eType II codebook for low and high feedback overhead bits and both the layers (layer 1 and layer 2). With AI-ML model, Layer 1 performs better than Layer 2 with higher SGCS values for same overhead feedback bits.

Table 2: AI/ML Performance for Dense Urban Macro with 80% Indoor and 20% Outdoor UEs

| TX Ports (NT) | Overhead Bits (KB) | Cosine Similarity (ρ) : Layer 1 | Cosine Similarity (ρ) : Layer 2 |
| --- | --- | --- | --- |
| 32 | 48.00 | 0.676 | 0.5017 |
|  | 62.00 | 0.7103 | 0.529 |
|  | 96.00 | 0.7862 | 0.6124 |
|  | 128.00 | 0.8431 | 0.7143 |
|  | 168.00 | 0.8786 | 0.7438 |
|  | 224.00 | 0.8992 | 0.8156 |
|  | 278.00 | 0.9166 | 0.8415 |

Figure 4: Cosine Similarity of Dense Urban Macro for AI/ML AE vs Rel-16 eType II Codebook for layer 1 and layer 2

### Dense Urban Micro

For Dense Urban Micro As shown in Table 3 and Figure 5, the AI-ML model outperforms the eType II CSI cases for both Layer 1 and layer 2. As seen with InH and Dense Urban Macro scenarios, Layer 1 performs better than Layer 2.

Table 3: AI/ML Performance for Dense Urban Micro with 80% Indoor and 20% Outdoor UEs

| TX Ports (NT) | Overhead Bits (KB) | Cosine Similarity (ρ) : Layer 1 | Cosine Similarity (ρ) : Layer 2 |
| --- | --- | --- | --- |
| 32 | 48.00 | 0.7044 | 0.508 |
|  | 62.00 | 0.7208 | 0.561 |
|  | 96.00 | 0.7944 | 0.6384 |
|  | 128.00 | 0.8603 | 0.7273 |
|  | 168.00 | 0.8897 | 0.7928 |
|  | 224.00 | 0.915 | 0.8482 |
|  | 278.00 | 0.932 | 0.86 |

Figure 5: Cosine Similarity of Dense Urban Macro for AI/ML AE vs Rel-16 eType II Codebook for layer 1 and layer 2

- ML based Autoencoder can outperform Rel-16 eType II codebook for Layer 1 and Layer 2 case in all overhead regimes for InH, Dense Urban Macro and Dense Urban Micro deployments.

## SLS performance

CSI accuracy is a critical factor for the performance of MU-MIMO system since it has significant impact on the inter-layer interference suppression capability as well as link adaptation for MU-MIMO transmission. Thus, higher accuracy of AI-ML CSI comparing to the eType II PMI codebook observed based on intermediate metrics (SGCS) results should translate to system performance gains.

System level simulations (SLS) allows to assess the system performance for CSI feedback enhancements considering different factors like different UE radio link conditions, realistic traffic, MU-MIMO transmission and scheduling, link adaptation, etc. Also, considering that rank adaptation is used, impact of CSI accuracy for multiple layers will be considered for the performance results in true proportion.

To assess the performance of spatial-frequency domain CSI compression using two-sided AI model, SLS evaluations were done for Dense Urban Macro scenario with FTP1 traffic model. Neural networks pre-trained separately per layer for each overhead point were used for AI-ML CSI with training dataset aligned with the channel model used for the SLS evaluations. The detailed evaluation assumptions are captured in the Appendix of the tdoc.

### Dense Urban Macro

SLS evaluation results for average UE throughput and cell-edge UE throughput are presented in Figure 5 and Figure 6 respectively for different overhead (corresponding to the maximum PMI payload) for AI-ML CSI with AI-ML model presented in Section 2.1 of this contribution. Results for different parameter combinations supported for eType II PMI codebook are presented for performance comparison. Performance gain relative to the first parameter combination of eType II PMI codebook is represented above each performance/overhead point.

Figure 6: Average UE throughput for AI/ML CSI and Rel-16 eType II PMI codebook

Figure 7: Cell-edge UE throughput for AI/ML CSI and Rel-16 eType II PMI codebook

As it can be observed from the above evaluation results, AI/ML CSI provides significant performance gains comparing to the eType II PMI codebook. Depending on the overhead, 5% - 13% gain in average UE throughput and 9% - 17% gain in cell-edge UE throughput is observed for AI/ML CSI comparing to the eType II PMI codebook configuration with similar overhead. Also, up to ~100 bits overhead reduction can be achieved using the AI-ML CSI with the same performance as for eType II PMI codebook.

- Up to 13% gain for average UE throughput and up to 17% gain for cell-edge UE throughput can be achieved for AI-ML CSI comparing to eType II PMI codebook

- Up to ~100 bits overhead reduction can be achieved using the AI-ML CSI with the same performance as for eType II PMI codebook

## Generalization performance

The performance of an AI-ML model depends not only on the model implementation (e.g. pre-/post-processing and neural network structure) but also on the datasets used for training and testing (inference) of the neural network. If there is a mismatch in the statistical properties between the datasets used for training and inference, then some performance loss is expected. The ability to apply the network for different channel statistics (e.g. scenario. configuration) can be measured by generalisation performance. At the last RAN1 meeting it was agreed to consider the following 3 cases to evaluate the generalisation performance.

- Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A

- Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/                                                                                                                                                                                                                                                                                                                                                                                                                                                                  Configuration#B

- Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration, e.g.,  Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B.

### Dense Urban Macro/Micro

The 3 cases for AI/ML model generalization performance was evaluated for UMa and UMi scenarios with the autoencoder neural network described in section 2.1 above. For the evaluation neural network with the same parameters was trained on 3 datasets: UMa, UMi and UMa + UMi. Mixed dataset (UMa + UMi) has equal distribution of channel matrixes (50% UMa and 50% UMi)

The SGCS results for different PMI reporting overhead (62 bits, 128 bits and 278 bits) are presented in figure 8 for inference on UMa dataset for Layer 1 and figure 9 for inference on UMa dataset for Layer 2.

Figure 8: SGCS values for the autoencoder trained on different datasets with UMa/UMi channel matrixes and tested on dataset with UMa channel matrixes for Layer 1

Figure 9: SGCS values for the autoencoder trained on different datasets with UMa/UMi channel matrixes and tested on dataset with UMa channel matrixes for Layer 2

The SGCS results for different PMI reporting overhead (62 bits, 128 bits and 278 bits) are presented in figure 8 for inference on UMi dataset for Layer 1 and figure 9 for inference on UMi dataset for Layer 2.

Figure 10: SGCS values for the autoencoder trained on different datasets with UMa/UMi channel matrixes and tested on dataset with UMi channel matrixes for Layer 1

Figure 11: SGCS values for the autoencoder trained on different datasets with UMa/UMi channel matrixes and tested on dataset with UMi channel matrixes for Layer 2

As it can be seen from the above evaluation results, there is a very small performance loss for autoencoder with misaligned datasets for training and inference (Case 2) comparing to autoencoder with aligned datasets (Case 1). If dataset with both UMa and UMi channel models is used for training (Case 3) then performance loss is negligible compared to Case 1.

- If dataset with both UMa and UMi channel models is used for training (Case 3) then performance loss is negligible compared to training and testing on aligned dataset (Case 1)

## Layer-common and layer-specific models

At the last RAN1 meeting it was agreed to evaluate layer specific and layer common model setting:

- Layer specific : Separated AI/ML models are trained per layer value and applied for corresponding layers to perform individual inference

- Layer common : A unified AI/ML model is trained and applied for each layer to perform individual inference

The SGCS results for different PMI reporting overhead (48 bits, 62 bits, 96bits,128 bits, 168 bits, 224 bits and 278 bits) are presented in figure 12 for training and inference on UMa dataset for layer common and layer specific models. Since models applied on layers are not changed across different ranks, the AI-ML models are rank-common.

Figure 12: SGCS values for the autoencoder trained on UMa dataset for Layer common and Layer specific models

From the above figure, it can be seen that the performance is very similar for layer specific and layer common models for layer 1 and layer 2.

- Performance is comparable for layer common and layer specific models for both layer 1 and layer 2 evaluations.

## Ground truth CSI quantization

Ground truth CSI quantization corresponds to the CSI feedback with higher accuracy comparing to the existing PMI codebook. This feature can be used for data collection and model performance monitoring at the network side. At least two options are considered for ground truth CSI quantization.

- Option 1: Scalar quantization with X bits per scalar value (e.g. floating point or fixed point formats)

- Option 2: eType II PMI codebook with new parameter values and quantization

In order to evaluate the impact of ground truth CSI quantization for data collection, AI-ML models were trained with dataset quantized by using option 1 and option 2 presented above. The following options were considered.

- Float32 floating point value representation (baseline), 26624 bits

- Fixed-point scalar quantization with 8 bits per scalar value, 6656 bits

- eType II PMI with L = 16, pv = 1, beta = 1, 4 bits amplitude and phase, 3329 bits

- eType II PMI with L = 16, pv = 1/2, beta = 1, 4 bits amplitude and phase, 1803 bits

- eType II PMI with L = 16, pv = 1/2, beta = 1/2, 4 bits amplitude and phase, 1131 bits

- eType II PMI with L = 8, pv = 1/2, beta = 1/2, 4 bits amplitude and phase, 588 bits

SGCS results for CSI compression model inference labelled as CSI compression loss are presented below for AI-ML models trained with different quantization of training dataset. SGCS values corresponding to the dataset compression (ground truth CSI quantization) are also presented in the figure below.

Figure 13: SGCS values for different ground truth quantization options

As it can be seen from the above results, smaller SGCS values for dataset quantization with smaller number of bits are correlated with the SGCS loss due to training dataset quantization (ground truth CSI quantization). Based on the presented results it can be observed that 8 times reduction of the number of bits required for ground truth channel reporting for data collection can be done without performance loss. Higher compression ratio can be potentially achieved by optimizing the ground truth quantization method.

- At least 8 times reduction of the number of bits required for ground truth CSI quantization for data collection comparing to float32 scalar quantization format can be done without performance loss.

Conclusion

In this contribution, we provided our views on the aspects of EVM related to AI/ML-based CSI enhancement. In summary, we have following proposals and observation:

Observation 1:

- ML based Autoencoder can outperform Rel-16 eType II codebook for Layer 1 and Layer 2 case in all overhead regimes for InH, Dense Urban Macro and Dense Urban Micro deployments

Observation 2:

- Up to 13% gain for average UE throughput and up to 17% gain for cell-edge UE throughput can be achieved for AI-ML CSI comparing to eType II PMI codebook

- Up to ~100 bits overhead reduction can be achieved using the AI-ML CSI with the same performance as for eType II PMI codebook

Observation 3:

- If dataset with both UMa and UMi channel models is used for training (Case 3) then performance loss is negligible compared to training and testing on aligned dataset (Case 1)

Observation 4:

- Performance is comparable for layer common and layer specific models for both layer 1 and layer 2 evaluations

Observation 5:

- At least 8 times reduction of the number of bits required for ground truth CSI quantization for data collection comparing to float32 scalar quantization format can be done without performance loss

References

- RP-213599 “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,” Qualcomm, December 6 – 17, 2021.

- “Attention Is All You Need”, A. Vaswani et. al., arXiv:1706.03762v5, Dec 2017.

Appendix

System level simulation evaluation assumptions for Dense Urban Macro and Indoor Hotspot scenarios can be found in the tables below (used for dataset generation for training and intermediate metrics calculation as well).

Table 4 System level simulation assumptions for Dense Urban Macro, Dense Urban Micro and Indoor Hotspot

| Parameter |  | Value |
| --- | --- | --- |
| Frequency Range |  | 4GHz |
| Inter-BS distance |  | 200m for Dense Urban Macro/Micro 20m, for Indoor Hotspot |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | For Dense Urban Macro/Micro: 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ For Indoor Hotspot: 32 ports: (8,8,2,1,1,4,4), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 2 Rx: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |
| BS Tx power |  | 44dBm for Dense Urban Macro/Micro 23dBm for Indoor Hotspot |
| BS antenna height |  | 25m for Dense Urban Macro 10m for Dense Urban Micro 3m for Indoor Hotspot |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Bandwidth |  | 20 MHz (52 PRB) |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 30kHz |
| CSI feedback |  | CSI periodicity is 5 ms, CSI delay is 4 slots, CSI subband size is 4 PRBs |
| Traffic model |  | FTP1 with 0.5 Mb packet size, ~70% resource utilization |
| UE distribution for Dense Urban Micro/Macro |  | 80% indoor (3km/h), 20% outdoor (30km/h) |