3GPP TSG RAN WG1 #112b-e                                                         R1-2302791

e-Meeting, April 17th – 26th, 2023

Source: 	Intel Corporation

Title: 	On other aspects on AI/ML for CSI feedback

Agenda item:	9.2.2.2

Document for: 	Discussion and Decision

# Introduction

The new study item on Artificial Intelligence (AI) / Machine Learning (ML) for NR air interface has been approved in [1]. One of the study objectives includes the analysis of solutions for CSI feedback enhancements:

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact. Use cases to focus on: - Initial set of use cases includes: - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1] - Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98 - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project

In this contribution, we express our views on potential specification impact focusing on spatial-frequency domain CSI compression using two-sided AI model sub-use case and CSI prediction using one-sided AI model at the UE side sub-use case.

Discussion

## Spatial-frequency domain CSI compression using two-sided AI model

### Model performance monitoring and data collection

Enhanced Type II PMI codebook supports multiple parameter combinations {L, pv, beta}, where each parameter combination corresponds to different overhead and performance for a given channel. Depending on the channel statistics (e.g. LoS/NLoS, number of clusters, delay spread etc.) the optimal parameter combination can be selected for a UE which allow to optimize CSI reporting overhead. Performance for the parameter combination corresponding to larger L, pv and beta is generally better comparing to the parameter combinations with lower parameter values. Performance of AI-ML model for CSI also depends on many aspects including pre/post-processing, alignment of training dataset and actual channel realisation for CSI inference, neural network structure and quantization, etc. Thus, the proper AI-ML model should be configured for a UE to achieve the best performance and overhead tradeoff.

In order to select the proper AI-ML model for CSI reporting, model performance monitoring can be used. At the RAN1#110b-e meeting the following agreements were made on the model performance monitoring.

Agreement In CSI compression using two-sided model use case, study potential specification impact for performance monitoring including: - NW-side performance monitoring: NW monitors the performance and make decisions of model activation/ deactivation/updating/switching - UE-side performance monitoring: UE monitors the performance and reports to Network, NW makes decisions of model activation/ deactivation/updating/switching Agreement In CSI compression using two-sided model use case, further study potential specification impact related to assistance signaling and procedure for model performance monitoring. Agreement In CSI compression using two-sided model use case, further study at least the following options for performance monitoring metrics/methods: - Intermediate KPIs as monitoring metrics (e.g., SGCS) - Eventual KPIs (e.g., Throughput, hypothetical BLER, BLER, NACK/ACK). - Legacy CSI based monitoring: schemes using additional legacy CSI reporting - Other monitoring solutions, at least including the following option: - Input or Output data based monitoring: such as data drift between training dataset and observed dataset and out-of-distribution detection

As it can be seen from the above agreements, at least eventual KPI and intermediate KPI are considered as monitoring metrics for AI-ML model performance monitoring. However, it is not clear how this metrics is calculated. If the metrics for model performance monitoring is calculated based on one model configured for a UE, then it is not possible to decide whether to re-configure the AI-ML model or continue to use the same AI-ML model. In order to select the AI-ML model with the best performance for CSI, testing of multiple AI-ML models with the measured channel can be considered.

Observation 1:

- Model performance monitoring based on intermediate KPI or eventual KPI calculated based on one AI-ML model is not giving enough information for proper configuration of AI-ML Model

Proposal 1:

- Testing of different AI-ML models with the measured channel should be considered for model performance monitoring

At the last RAN1 meeting the following agreement was made on intermediate KPIs based model performance monitoring.

Agreements In CSI compression using two-sided model use case, further study the necessity, feasibility, and potential specification impact for intermediate KPIs based monitoring including at least: - NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side. - UE-side monitoring based on the output of the CSI reconstruction model, subject to the aligned format, associated to the CSI report, indicated by the NW or obtained from the network side. - Network may configure a threshold criterion to facilitate UE to perform model monitoring. - UE-side monitoring based on the output of the CSI reconstruction model at the UE-side - Note: CSI reconstruction model at the UE-side can be the same or different comparing to the actual CSI reconstruction model used at the NW-side. - Network may configure a threshold criterion to facilitate UE to perform model monitoring. - FFS: Other solutions, e.g., UE-side uses a model that directly outputs intermediate KPI. Network-side monitoring based on target CSI measured via SRS from the UE. Note: Monitoring approaches not based on intermediate KPI are not precluded Note: the study of intermediate KPIs based monitoring should take into account the monitoring reliability (accuracy), overhead, complexity, and latency.

According to the above agreement RAN1 will study several options for model performance monitoring. All the agreed options are based on channel measured based on CSI-RS at the UE, which means that even for network-based model performance monitoring, feedback of the measured channel is required. However, UL reference signals (i.e., SRS) can be used to estimate the model performance since at least delay and angular reciprocity exists for UL and DL channels. Encoder model (CSI generation part of the two-sided model) is required at the gNB in order to calculate an intermediate metrics (e.g., GCS) based on SRS. Since it is possible that gNB controls the used encoder at the UE, the assumption that encoder model is available at the gNB may be valid.

Considering that model performance monitoring doesn’t require reporting of channel measurements from the UE and it may have lower RAN1 specification impact, we propose to study NW-side monitoring based on the target CSI from SRS and output CSI obtained from SRS measurements using the two-sided model (assuming that CSI generation part of the model is known at the gNB).

Proposal 2:

- For CSI compression using two-sided model use case, further study the necessity, feasibility, and potential specification impact of intermediate KPIs based monitoring for

  - NW-side monitoring based on the target CSI from SRS and output CSI obtained from SRS measurements using the two-sided model (assuming that CSI generation part of the model is known at the gNB)

The following agreement was made at the RAN1#110b-e meeting for the co-existence of legacy PMI codebooks and AI-ML based CSI reporting.

Agreement In CSI compression using two-sided model use case, further study potential specification impact related to potential co-existence and fallback mechanisms between AI/ML-based CSI feedback mode and legacy non-AI/ML-based CSI feedback mode.

In our view the new AI-ML based CSI reporting can be fitted to the existing CSI framework as new codebook type or as new CSI quantity. In this case co-existence and fallback can be done via the existing features. For example, two CSI Report Config parameters can be configured with one CSI Report Config corresponding to CSI report with a PMI codebook and another corresponding to AI-ML based CSI feedback. If CSI with PMI codebook outperforms the CSI with AI-ML model based on network performance monitoring procedure, CSI report with PMI codebook can be triggered/activated/used.

Proposal 3:

- Co-existence and fallback mechanism between AI/ML-based CSI feedback mode and legacy non-AI/ML-based CSI feedback mode should be based on existing CSI framework

  - AI/ML based CSI report can be configured by using CSI-ReportConfig with the corresponding measurements and trigger states (for aperiodic CSI) configuration

One of the aspects which is related to the training of the neural network is data collection. In our view, at least for initial study of the AI/ML CSI sub use cases it can be assumed that existing NR features are considered for data collection (e.g. SRS, CSI-RS, CSI reporting). Some enhancements may be needed to support data collection using CSI reporting. New codebook design corresponding to the ground truth CSI quantization can be used for data collection. Since the data collection is not sensitive to larger latency between the measurements and the report, relaxed timing requirements (CSI computation time at the UE) can be considered. Also, reporting of CSI corresponding to multiple time instances for measurements in one CSI report can be used.

Proposal 4:

- Consider existing NR features as baseline for data collection (e.g., SRS, CSI-RS, CSI reporting)

  - The following enhancements can be considered: new codebook design (ground-truth CSI quantization), relaxed timing requirements, reporting of CSI for multiple CSI instances in one CSI report

### AI/ML model input and inference

At the RAN1#110 meeting the following agreements were made w.r.t. AI/ML model input and inference.

Agreement In CSI compression using two-sided model use case, further study potential specification impact on CSI report, including at least - CSI generation model output and/or CSI reconstruction model input, including configuration(size/format) and/or potential post/pre-processing of CSI generation model output/CSI reconstruction model input. - CQI determination - RI determination Agreement In CSI compression using two-sided model use case, further study potential specification impact on output CSI, including at least - Model output type/dimension/configuration and potential post processing

Since the neural network is trained assuming a particular algorithm for pre-processing, it doesn’t make sense to have separate configuration/specification of pre-processing algorithm and neural network (including neural network weights considering that the same neural network may be trained with different pre-processing algorithms). Thus, if an AI/ML model is configured at the UE for inference, information on pre-processing for that model should be provided to the UE (e.g. specified, configured, downloaded, transferred etc.). Since the pre-processing and neural network are highly related it is important to provide clear definition of pre-processing algorithm. To describe pre-processing at least linear transforms (DFT across different dimensions), down selection of matrix elements and normalization operations should be defined.

Proposal 5:

- It is expected that AI/ML model is trained assuming a particular pre/post processing

  - If an AI/ML model is configured at the UE for inference, information on pre-processing for that model should be provided to the UE (e.g. specified, configured, downloaded etc.)

  - Pre/post-processing may include at least linear transforms (DFT across different dimensions), down selection of matrix elements and normalization

The parameters for the model input can be defined similar to parameters L and M for the Enhanced Type II PMI codebook, where parameter L controls the number of inputs for spatial domain and parameter M controls the number of inputs for frequency domain, so the dimensions of input matrix are L × M.

In some cases information from the pre-processing step is required to be reported in CSI, otherwise the precoding matrix can not be reconstructed at the gNB. For example, if oversampling (SD basis rotation factor) of Spatial Domain (SD) vectors is used and/or selection of SD or Frequency Domain (FD) vectors is used at the pre-processing step.

Proposal 6:

- The dimensions of the input are defined by parameters similar to parameters L/M for Enhanced Type II PMI codebook (considering that input corresponds to the neural network input after pre-processing)

  - In some cases, information from pre-processing step shall be reported by the UE together with CSI bits generated by the neural network (e.g., selected basis vectors, basis rotation factor, etc.)

### CQI and RI reporting

For spatial-frequency domain CSI compression using two-sided AI model, PMI calculation approach is redesigned. However, for CQI and RI calculation it is still possible to use legacy principles. Thus, RI can be used to indicate the number of layers recommended for the PDSCH transmission and number of layers for PMI, CQI can be used to indicate the spectral efficiency of the effective channel (i.e., recommended modulation and coding scheme).

Proposal 7:

- Consider existing principles for RI and CQI for spatial-frequency domain CSI compression using two-sided AI model sub-use case

The following agreement was made at the last RAN1 meeting on CQI determination for CSI compression using two-sided model use case.

Agreement In CSI compression using two-sided model use case, further study the following options for CQI determination in CSI report, if CQI in CSI report is configured. - Option 1: CQI is NOT calculated based on the output of CSI reconstruction part from the realistic channel estimation, including - Option 1a: CQI is calculated based on target CSI with realistic channel measurement - Option 1b: CQI is calculated based on target CSI with realistic channel measurement and potential adjustment - Option 1c: CQI is calculated based on legacy codebook - Option 2: CQI is calculated based on the output of CSI reconstruction part from the realistic channel estimation, including - Option 2a: CQI is calculated based on CSI reconstruction output, if CSI reconstruction model is available at the UE and UE can perform reconstruction model inference with potential adjustment - Note: CSI reconstruction part at the UE can be different comparing to the actual CSI reconstruction part used at the NW. - Option 2b: CQI is calculated using two stage approach, UE derive CQI using precoded CSI-RS transmitted with a reconstructed precoder. - Other options are not precluded - Note1: feasibility of different options should be evaluated - Note2: Gap analyses between the UE side CQI calculation results and the NW side results, as well as the impact on the scheduling performance should be evaluated - Note3: Complexity of CQI calculation needs to be evaluated, including the computing complexity and potential RS/signaling overhead

Option 1b from the above agreement assumes that CQI is calculated based on target CSI with realistic channel measurement and potential adjustment. CQI adjustment is needed to compensate mismatch of precoding matrix assumed for CQI calculation and the actual precoding matrix in CSI (i.e., PMI). However, it is not clear from the agreement how to define adjustment for CQI. Several options can be considered. For example, CQI adjustment can be configured via higher layers (RRC or MAC CE).

Alternatively, CQI adjustment can be determined based on mismatch with reference CQI value, where reference CQI value can be determined from preceded CSI-RS transmission. In this case, the mechanism to determine the CQI adjustment can be referred as CQI calibration.

If there is some prior knowledge for the mismatch of target CSI and CSI reconstruction output (e.g., NMSE or GCS), the mismatch can be modelled by adding AWGN with pre-defined power based on the prior knowledge to the target CSI for CQI calculation.

Proposal 8:

- The following alternatives for CQI adjustment determination can be considered for Option 1b CQI determination

  - CQI adjustment is configured via higher layers

  - CQI adjustment is determined by the UE based on reference CQI (e.g., measured from precoder CSI-RS)

  - CQI is calculated using precoding matrix corresponding to the target CSI with added AWGN

## CSI prediction using one-sided AI model at the UE side

At the last RAN1 meeting the following agreement was made for CSI prediction using one-sided AI model at the UE side sub-use case.

Agreement Time domain CSI prediction using UE sided model is selected as a representative sub-use case for CSI enhancement. Note: Continue evaluation discussion in 9.2.2.1. Note: RAN1 Defer potential specification impact discussion at 9.2.2.2 until the RAN1#112b-e, and RAN1 will revisit at RAN1#112b-e whether to defer further till the end of R18 AI/ML SI. Note: LCM related potential specification impact follow the high level principle of other one-sided model sub-cases.

There is a parallel discussion on CSI prediction in Rel-18 MIMO WI, where UE-side channel prediction support is enabled for new PMI codebooks, where codebook enhancements as well as assistance signalling and CQI determination mechanism are specified. Considering the above, CSI prediction with AI/ML model at the UE side shall be discussed in application to Rel-18 CSI enhancements for high/medium mobility to avoid redundant work on similar issues.

Proposal 9:

- CSI prediction with AI/ML model at the UE side shall be discussed in application to Rel-18 CSI enhancements for high/medium mobility

The CSI prediction performance depends on many factors including wireless channel properties, UE speed, properties of reference signals (CSI-RS) and AI-ML model used for CSI prediction at the UE. In general case all the above factors can not be known at the gNB side. Thus, acceptable prediction performance cannot be guaranteed for all the cases which may lead to degradation of system performance if channel prediction is enabled. To avoid the performance loss, model performance monitoring is required for CSI prediction using one-sided AI model at the UE side.

For the model performance monitoring, calculation of intermediate metrics may be considered. In this case, in order to calculate the intermediate metrics, AI-ML model shall be applied to predict CSI for a time instance in which CSI-RS measurements are available. Model performance monitoring for CSI prediction is illustrated in Figure 1 below.

Figure 1. Model performance monitoring for CSI prediction

In the above figure intermediate metrics (e.g., GCS) is calculated based on measured channel H(n+3P) and predicted channel Hp(n+3P), where P corresponds to the CSI-RS periodicity. We propose to study model performance monitoring based on GCS calculated from the measured CSI-RS and predicted channel at the UE side.

Proposal 10:

- Study model performance monitoring based on intermediate metrics (e.g., GCS) calculated from the measured CSI-RS and predicted channel at the UE side

Conclusion

In this contribution, we have provided our views on sub use cases for AI/ML CSI including potential specification impact. The following proposals and observations were made.

Observation 1:

- Model performance monitoring based on intermediate KPI or eventual KPI calculated based on one AI-ML model is not giving enough information for proper configuration of AI-ML Model

Proposal 1:

- Testing of different AI-ML models with the measured channel should be considered for model performance monitoring

Proposal 2:

- For CSI compression using two-sided model use case, further study the necessity, feasibility, and potential specification impact of intermediate KPIs based monitoring for

  - NW-side monitoring based on the target CSI from SRS and output CSI obtained from SRS measurements using the two-sided model (assuming that CSI generation part of the model is known at the gNB)

Proposal 3:

- Co-existence and fallback mechanism between AI/ML-based CSI feedback mode and legacy non-AI/ML-based CSI feedback mode should be based on existing CSI framework

  - AI/ML based CSI report can be configured by using CSI-ReportConfig with the corresponding measurements and trigger states (for aperiodic CSI) configuration

Proposal 4:

- Consider existing NR features as baseline for data collection (e.g., SRS, CSI-RS, CSI reporting)

  - The following enhancements can be considered: new codebook design (ground-truth CSI quantization), relaxed timing requirements, reporting of CSI for multiple CSI instances in one CSI report

Proposal 5:

- It is expected that AI/ML model is trained assuming a particular pre/post processing

  - If an AI/ML model is configured at the UE for inference, information on pre-processing for that model should be provided to the UE (e.g. specified, configured, downloaded etc.)

  - Pre/post-processing may include at least linear transforms (DFT across different dimensions), down selection of matrix elements and normalization

Proposal 6:

- The dimensions of the input are defined by parameters similar to parameters L/M for Enhanced Type II PMI codebook (considering that input corresponds to the neural network input after pre-processing)

  - In some cases, information from pre-processing step shall be reported by the UE together with CSI bits generated by the neural network (e.g., selected basis vectors, basis rotation factor, etc.)

Proposal 7:

- Consider existing principles for RI and CQI for spatial-frequency domain CSI compression using two-sided AI model sub-use case

Proposal 8:

- The following alternatives for CQI adjustment determination can be considered for Option 1b CQI determination

  - CQI adjustment is configured via higher layers

  - CQI adjustment is determined by the UE based on reference CQI (e.g., measured from precoder CSI-RS)

  - CQI is calculated using precoding matrix corresponding to the target CSI with added AWGN

Proposal 9:

- CSI prediction with AI/ML model at the UE side shall be discussed in application to Rel-18 CSI enhancements for high/medium mobility

Proposal 10:

- Study model performance monitoring based on intermediate metrics (e.g., GCS) calculated from the measured CSI-RS and predicted channel at the UE side

References

- RP-213599 “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface,” Qualcomm, December 6 – 17, 2021.