3GPP TSG RAN WG1 #112b-e			     R1-2302799

e-Meeting, April 17th – 26th, 2023

Source:	Intel Corporation

Title:	On the Co-channel Coexistence between LTE and NR Sidelink

Agenda item:	9.4.2

Document for:	Discussion and Decision

Introduction

In 3GPP TSG RAN meeting #94, a new work item related to NR Sidelink (SL) evolution was approved. The work item description was updated during RAN#99 [1] in order to explicitly capture the case of different SCS support between NR and LTE:

- Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any [RAN1, RAN2, RAN4] - Reuse the in-device coexistence framework defined in Rel-16 as much as possible - Note, RAN1 continues the work on dynamic resource pool sharing based on existing agreements and WID with high priority for Type A devices and operating combination A - RAN1 is tasked to support only 15 and 30 kHz SCSs for dynamic resource pool sharing. Existing RAN1 agreements for dynamic resource pool sharing apply to support of 30 kHz. - For NR PSCCH/PSSCH transmissions in 30kHz SCS, NR SL UE selects in MAC layer at least the first of NR SL slots overlapping with an LTE SL subframe, and can select the subsequent overlapping NR SL slot in MAC layer - No change to the R16/17 resource allocation procedure in PHY due to this restriction - The existing SL slot structure from Rel-16 is unchanged - The starting symbol of the first of the overlapping NR SL slots is assumed to be aligned with the first symbol of the LTE SL subframe - For NR SL with 15/30kHz SCSs, NR SL UE avoids selecting resources for PSCCH/PSSCH transmissions where the corresponding PSFCH transmission occasions overlap with LTE SL reservations in time domain - Note, this is inline with Option 1-2 in the working assumption made in RAN1#112. No other options from the working assumption need to be considered. - Mode 2 operation only

In this contribution, the aspects related to open issues for coexistence of LTE V2X and NR V2X in co-channel scenarios are discussed, while our view related to other Rel-18 SL aspects are provided in our companion contributions [2]-[4].

# Discussion

## PSFCH Issue

One of the important issues to be resolved by RAN1 for LTE-NR co-existence is to decide how to avoid impact of PSFCH transmissions in NR on PSCCH/PSSCH transmissions in LTE, especially given that PSFCH transmission may not be easily detected with the RSRP and RSSI mechanisms available in LTE. Furthermore, the transmission of the PSFCH only utilizes the last symbols of a subframe/slot, thus introducing AGC mismatch and channel estimation mismatch issues.

During RAN1#112, the following agreement was made:

Working assumption - For dynamic resource pool sharing, select one option between 1-1 and 1-2, and select one option between 3-1 and 4-1-a, and define how the two selected options can be used in the operation: - Option 1-1: Avoiding PSFCH transmissions overlapping with LTE SL resources reserved for LTE SL transmissions in the time domain is performed by the UE transmitting PSFCH - FFS whether/how to define condition(s) under which the UE transmitting PSFCH drops its PSFCH transmission overlapping with the LTE SL resources reserved for LTE SL transmission - Option 1-2: Avoiding PSFCH transmissions overlapping with LTE SL resources reserved for LTE SL transmissions in the time domain is ensured by the UE transmitting PSSCH - FFS whether/how to define condition(s) of selecting NR SL candidate resources for NR SL PSCCH/PSSCH - Option 3-1: Additional PSFCH periodicity(ies) is introduced - Alt 3-1-a: 10 logical slots - Alt 3-1-b: 8 logical slots - Alt 3-1-c: 5 logical slots - FFS: Alignment between PSFCH periodicity and LTE logical subframes should be ensured by proper configuration - Option 4-1: PSFCH resources and LTE SL resources are TDMed - Alt 4-1-a: PSFCH resources is confined within the guard symbol of the LTE SL subframe in the time domain - Note: selecting a single option is not precluded

Option 1-1 vs Option 1-2:

- In our view, it is an unfortunate situation that one between options 1-1 and 1-2 needs to be selected. Those are naturally complementary and work perfectly together.

- Regardless of RAN#99 discussion, if one option needs to be picked, it should be 1-2 since 1-1 could not handle NACK-only groupcast feedback case due to being susceptible to DTX-to-ACK errors when PSFCH is dropped.

- Considering RAN#99 discussion on support of higher SCS, Option 1-2 is already agreed for the case of 15/30 kHz SCS for LTE and NR. Effectively, this means the selection between Option 1-1 and Option 1-2 has already been made.

Option 3-1 vs Option 4-1-a:

- Option 4-1-a may not be feasible in case of 15/15 kHz SCS combination and may require physical layer changes in 15/30 kHz SCS case. Further, RAN#99 decision recommends not to consider other solutions for 15/30 kHz case.

- Option 3-1 specification impact is limited by introducing new PSFCH periodicities. Note that alignment of PSFCH periodicity and LTE logical subframes may not be possible to be ensured in case of 8 slots periodicity.

Observation 1:

- Considering that Option 1-1 does not work for NACK-only feedback case and the decision in RAN#99 for the case of 15/30 kHz SCS, Option 1-2 should be selected between 1-1 and 1-2.

- Considering feasibility issues for supporting 4-1-a in 15/15 kHz SCS case, Option 3-1 should be selected between 3-1 and 4-1-a.

Proposal 1:

- Support combination of Option 1-2 and Option 3-1

Regarding the details of Option 1-2 support, it needs to be discussed and decided how to apply the overlapping criteria. A new RSRP threshold for PSFCH overlap with LTE needs to be introduced. The following properties of the PSFCH RSRP threshold set can be defined:

- The RSRP threshold set can take into account two different PSFCH overlapping types with LTE as illustrated in Figure 1. When partial overlap is observed, higher RSRP threshold may be utilized since PSFCH impact on LTE transmission is already minimized.

Figure 1 - Illustration of different types of PSFCH overlap with LTE in time domain.

- The RSRP threshold set should consider whether PSFCH carries feedback for initial transmission on retransmission. It is a well-known phenomenon that resources for retransmission are reserved by UEs, but may be unused when ACK is received. Also, in ACK/NACK cases, retransmission feedback may be more important than the initial transmission feedback when PDB is tight. These considerations motivate introducing different RSRP thresholds between initial transmission and retransmission.

Proposal 2:

- For Option 1-2, support (pre-)configuration of RSRP thresholds for checking overlap with LTE transmission

  - Per priority pair,

  - Per overlap type in time domain,

  - Per initial transmission or retransmission.

- For Option 3-1, the following PSFCH periodicities are additionally introduced to existing 1,2,4 logical slots: 5, 8, and 10.

## Use of Information Shared by LTE SL Module

### Other LTE UEs transmissions

During RAN#112, further decision was made to utilize LTE sensing information at NR PHY layer, assuming it is integrated into resource exclusion procedures described in TS 38.214, Section 8.1.4:

Agreement In NR SL resource (re)selection procedure, option 1 is adopted for how to determine candidate resource set for NR SL considering the LTE SL reserved resources by other LTE SL UE - Option 1: The PHY layer of NR SL module excludes NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE when the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold, where the SL RSRP threshold is derived based on LTE SL priority of other LTE SL UE and NR SL priority for NR SL transmission - For the list of the above initial SL RSRP threshold, down-select one of followings: - Alt 1: NR SL RSRP threshold list (pre)configured in a NR SL resource pool - Alt 2: LTE SL RSRP threshold list (pre)configured in a LTE SL resource pool - Alt 3: SL RSRP threshold list separately (pre)configured for dynamic resource pool sharing - Alt 4: SL RSRP threshold list separately (pre)configured for dynamic resource pool sharing - A different SL RSRP threshold list may be (pre)configured for selecting single slot resources in NR SL slots with NR PSFCH - For the LTE SL periodic reserved resources by other LTE SL UE, - For determining the above LTE SL reserved resources, the time-and-frequency resources of LTE SL reserved resources by other LTE SL UE are repeated Q times according to the LTE SL resource reservation period - FFS details including at least whether the formula of Q in Section 14.1.1.6 in TS 36.213 or the formula of Q in Section 8.1.4 in TS 38.214 is used - The PHY layer of NR module applies the above procedure in Step 6 in Section 8.1.4 of TS 38.214 - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of which NR SL periodic resources are overlapping with LTE SL reserved resources by other LTE SL UE when the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold according to condition c in Step 6 in Section 8.1.4 of TS 38.214 - Note: It is assumed that the information relevant to LTE SL reserved resources by other LTE SL UE used in the above procedure is shared from LTE SL module to NR SL module - FFS whether/how LTE SL RSRP is applied

LTE PSCCH and LTE PSSCH are multiplexed in frequency domain, have different PSD and may potentially be adjacent or non-adjacent. Considering this, a singe PSCCH+PSSCH transmission may be split onto two transmissions before performing the search of the overlapping NR sub-channels.

Proposal 3:

- At least when LTE V2X is configured with non-adjacent resource pool option, LTE PSCCH and LTE PSSCH are split into two different reserved resources and treated independently in NR resource exclusion.

It needs to be discussed how the RSRP measured in LTE may be comparable in the NR module. There are three main points to consider:

- Support of different sub-carrier spacing on LTE and NR. Since RSRP is defined as a linear average over resource elements in both LTE and NR, when the same power is allocated in the same bandwidth of LTE or NR, it results in a different RSRP measured when different SCS is employed. For example, when NR operates in 30 kHz, the compatible RSRP measurement would be twice larger. The following equation can be used:

RSRPNR=SCSNRSCSLTERSRPLTE

- Support of non-adjacent resource pool configuration in LTE. This case is quite different from NR resource pool structure. One way to handle it is to split LTE PSCCH and LTE PSSCH parts onto two different resources and consider them separately in NR resource exclusion. Then, associated RSRP for the two parts needs to be carefully recalculated considering PSCCH power boosting and that LTE RSRP is measured on PSSCH DMRS:

RSRPPSCCH=RSRPPSSCH100.3

- Even for adjacent resource pool case, RSRP may be recalculated to better represent the average RSRP value considering the LTE PSCCH PSD boosting:

RSRPavg=RSRPPSSCHNPSCCH100.3+NPSSCHNPSCCH+ NPSSCH

Proposal 4:

- LTE RSRP measurement is scaled before being used in NR module according to the following rules:

  - If NR uses higher SCS, LTE RSRP is scaled proportionally to the sub-carrier bandwidth RSRPNR=SCSNRSCSLTERSRPLTE;

  - If LTE transmission is split into two separate portions for LTE PSCCH and LTE PSSCH, the RSRP on PSCCH part is calculated as RSRPPSCCH=RSRPPSSCH100.3;

  - FFS if average RSRP is used for LTE RSRP.

Between the alternatives of RSRP threshold configuration, Alt.3 looks the more flexible among them. It may realize different fairness between LTE and NR accessing the same resources. Even though the RSRP threshold set may be separately configured, for LTE RSRP to be better tractable in NR module, the above conversions need to be employed.

Alt.4 targets intentionally increasing loading in NR slots containing PSFCH so that this energy is accounted in LTE RSSI measurements. However, this option works only for cases when NR periodicity matches 100 ms base periodicity of LTE. Otherwise, the increased loading in PSFCH slots may only degrade NR performance w/o helping LTE performance.

Proposal 5:

- Alt.3 is adopted for RSRP threshold configuration for LTE resource exclusion in NR module.

One specific feature which is not present in NR but is present in LTE, is the usage of RSSI measurement to rank resources which do not have associated RSRP measurements, i.e., reserving SCIs received. To support it in NR, a given single-subframe LTE resource which has an associated sidelink RSSI measurement may be converted to an NR SCI with a default priority, default periodicity and an associated RSRP measurement, which is a function of RSSI measurement. RSSI to RSRP conversion needs to consider that RSSI also captures interference and noise, and is defined per symbol, not per RE. If it can be allowed for a UE to convert RSSI to RSRP considering appropriate scaling, it may capture the effect of RSSI to a large extent.

Proposal 6:

- LTE sub-channels with RSSI measurement are converted to NR SCIs with a default priority, default periodicity, and with RSSI measurement converted to approximate RSRP measurement considering sub-channel bandwidth.

### Non-monitored LTE subframes

The following proposal was discussed in RAN1#112 but was not agreed:

Draft proposal 2-3(II): In NR SL resource (re)selection procedure, down-select one of followings for how to determine candidate resource set for NR SL considering the LTE SL resources associated with non-monitored subframe of LTE SL module - Option 1: - The PHY layer of NR SL module excludes NR SL candidate resources overlapping with LTE SL resources associated with non-monitored subframe of LTE SL module - For determining the LTE SL resources associated with non-monitored subframe of LTE SL module, - All the LTE SL resources in the non-monitored subframes are assumed to be repeatedly reserved Q times for each LTE SL resource reservation period (pre)configured in a LTE SL resource pool - FFS details including at least whether the formula of Q in Section 14.1.1.6 in TS 36.213 or the formula of Q in Section 8.1.4 in TS 38.214 is used - The PHY layer of NR module applies the above procedure in Step 5 in Section 8.1.4 of TS 38.214 - Note: For periodic reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of which NR SL periodic resources are overlapping with LTE SL resources associated with non-monitored subframe of LTE SL module according to Step 5 in Section 8.1.4 of TS 38.214 - Note: It is assumed that the information relevant to LTE SL resources associated with non-monitored subframe of LTE SL module used in the above procedure is shared from LTE SL module to NR SL module - Option 2: - The PHY layer of NR SL module does not perform NR SL candidate resource exclusion based on the information regarding LTE SL resources associated with non-monitored subframe of LTE SL module

Although it may be natural to follow the same way how the non-monitored subframes are excluded in NR and LTE, there is always a consideration of excessive time resource exclusion which leads to candidate resource set starvation. In Rel-16, a work-around captured in step 5a of section 8.1.4 was introduced to reset the candidate resource set when excessive exclusion is detected. However, the issues may be relaxed before being faced by at least two ways, in addition to always excluding all periodicities:

- Alt.1 Do not exclude non-monitored LTE subframes,

- Alt.2 Exclude LTE subframes related to non-monitored subframes only according to a subset of periodicities.

- Alt.3 Exclude LTE subframes related to non-monitored subframes according to all configured periodicities of LTE.

Proposal 7:

- The subframes non-monitored on LTE sensing window are converted to NR slots not monitored in the sensing window.

- Periodicities for exclusion of non-monitored slots are a separate sub-set of LTE configured periodicities.

### Own LTE module’s transmissions

At this stage we think that consideration of own’s LTE module transmissions in NR module resource selection as optimization, since Rel-16 in-device co-existence feature can handle this case.

Observation 2:

- Rel-16 in-device co-existence framework provides mechanisms to resolve overlap of LTE and NR transmissions and/or receptions and separate consideration of this scenario in Rel-18 is not required.

## Timeline of Information Shared by LTE SL Module

The following was agreed with respect to timeline for sharing LTE information with NR SL module:

Agreement For dynamic resource pool sharing, the NR SL module is expected to use the information shared by the LTE SL module to the NR SL module which is known by NR SL module at the latest T ms prior to slot n (as defined in clause 8.1.4 of TS 38.214), to determine a set of resources for its own (re)transmission. - T is defined using - T≤Tmax ms, and is based on UE implementation, according to the Rel-16 NR SL timeline for in-device coexistence. - FFS: Value of Tmax - FFS: any discussion on the earliest information, if needed Agreement Based on the agreement in RAN1#110bis-e, the value of Tmax = 4 ms.

By carefully reading the agreement, it appears that Tmax is an equivalent of a processing delay of LTE shared information. However, it does not capture whether the LTE shared information is as recent as T or Tmax. Leaving it up to UE implementation, similar to Rel-16 IDC short-term solutions, does not work since it is now not only internal to a UE, but affects the system performance and fairness of the distributed sidelink communication.

To fix this issue, additional deadline is needed. In our understanding, it may be described in the following manner using additional ‘Tshare’ timing:

- The information shared by the LTE SL module corresponds to the sensing information that would be used by the LTE SL module for resource selection in moment n – Tshare. The value of Tshare is implementation specific, but the smaller the value, the better the system performance. It may be discussed as part of capability or fixed in the specification. However, at least Tshare equal to Tmax (=4ms) may be considered as one candidate.

Observation 3:

- The agreements on T and Tmax introduce an equivalent of a processing delay of LTE shared information to consider in the NR resource selection once this information is available in NR module, however, do not capture any age or relevance of the LTE shared information in moment n-T.

Proposal 8:

- The information shared by the LTE SL module corresponds to the sensing information that would be used by the LTE SL module for resource selection not earlier than in moment n - Tshare, where Tshare is FFS and may include Tshare = Tmax.

## Higher SCS

In RAN#99, a decision was made to support 30 kHz SCS on NR co-existing with LTE in 15 kHz SCS, as cited in introduction section 1. The solution aims to minimize PHY layer impact from the feature. However, there are at least two aspects which need to be discussed.

According to RAN#99 agreement, the resource selection is performed in MAC layer so that at least the first slot of the two overlapping with LTE subframe is utilized. The two aspects arising with this consideration are:

- Issue 1: MAC layer is unaware of numerology considerations and also of co-existence between LTE and NR in general, since so far, the feature was only impacting PHY layer. In our view, in this case MAC layer needs to receive from PHY indication of allowed starting slots for selected resources, so that MAC layer is still transparent to numerologies.

- Issue 2: The additional restriction imposed by the mix of numerologies, similarly to non-preferred resource set exclusion, may increase probability that it is impossible to select resources for transmission. In this case, it may be either left for UE implementation or may be allowed for a UE to drop transmission.

Proposal 9:

- In case of NR operating in 30 kHz SCS, for a ach candidate resource NR PHY layer shares with MAC layer information on whether a candidate resource overlaps with the first LTE subframe symbol or not.

Proposal 10:

- It is up to UE implementation to handle the cases when the additional restriction on starting positions of selected resources leads to impossibility to select a transmission resource.

Lastly, it may need to be clarified, whether the subsequent resource may be selected only with the same frequency allocation, or it may have a different frequency allocation. In general, the AGC issue may also be observed if the frequency allocation is changed, due to frequency selectivity of the propagation channel. The issue may not be critical in some scenarios but may impact the performance in other scenarios.

Proposal 11:

- In case of NR 30 kHz SCS, RAN1 to clarify whether/when a UE may select a subsequent resource not overlapping with the first symbol of LTE subframe, which has the same or different frequency allocation from the preceding resource.

## Considerations on LTE and NR Module Requirements

In prior RAN4 meeting, the assumptions to take in terms of RF requirements for the study on co-channel co-existence between LTE sidelink and NR sidelink have been discussed. In this matter, RAN4 converged that they will only study the LTE sidelink and NR sidelink co-channel coexistence scenarios for the case where LTE sidelink and NR sidelink transmit via TDM manner, but could receive simultaneously in the same channel for a single UE, and they have sent a related LS to RAN1 and RAN2 [5], with the expectation that either RAN1 or RAN2 could provide an assumption and more information on how the NR and LTE module may relate.

While RAN1 has defined the concept of type A device as a device containing both LTE SL and NR SL modules, and which may use the sensing and resource reservation information shared by the LTE SL module, the design for co-existence has been transparent from the details on how the LTE and NR module relate to each other, and furthermore it has assumed that simultaneous reception from both modules would be possible.  Therefore, due to the fact that RAN1 is developing solutions which are transparent from RAN4 requirements and possible assumptions in terms of RF and baseband requirements, decision on this specific level of details should be taken by RAN4. However, RAN1 should confirm with RAN4 on the following assumptions:

- RAN1 has assumed that simultaneous reception from both modules in the same channel from a single UE is possible.

- As per WID description, RAN1 has prioritized dynamic resource pool sharing based on operating combination A (i.e., mode 2 to mode 4 co-existence) and using type A devices, where Rel-16 in-device co-existence is assumed together with slot alignment between the two modules.

Observation 4:

- Regarding the assumption to take on how the LTE and NR module relate with each other, RAN1 has indeed assumed that simultaneous reception from both modules in the same channel from a single UE is possible, while any other specific details in terms of RF and baseband requirements are expected to be discussed by RAN4. Furthermore, as per WID description RAN1 has prioritized dynamic resource pool sharing based on operating combination A (i.e., mode 2 to mode 4 co-existence) and using type A devices, where Rel-16 in-device co-existence is assumed together with slot alignment between the two modules.

Conclusions

In this contribution, we discussed several aspects related to coexistence between NR SL and LTE SL, and made the following proposals and observations:

Observation 1:

- Considering that Option 1-1 does not work for NACK-only feedback case and the decision in RAN#99 for the case of 15/30 kHz SCS, Option 1-2 should be selected between 1-1 and 1-2.

- Considering feasibility issues for supporting 4-1-a in 15/15 kHz SCS case, Option 3-1 should be selected between 3-1 and 4-1-a.

Proposal 1:

- Support combination of Option 1-2 and Option 3-1

Proposal 2:

- For Option 1-2, support (pre-)configuration of RSRP thresholds for checking overlap with LTE transmission

  - Per priority pair,

  - Per overlap type in time domain,

  - Per initial transmission or retransmission.

- For Option 3-1, the following PSFCH periodicities are additionally introduced to existing 1,2,4 logical slots: 5, 8, and 10.

Proposal 3:

- At least when LTE V2X is configured with non-adjacent resource pool option, LTE PSCCH and LTE PSSCH are split into two different reserved resources and treated independently in NR resource exclusion.

Proposal 4:

- LTE RSRP measurement is scaled before being used in NR module according to the following rules:

  - If NR uses higher SCS, LTE RSRP is scaled proportionally to the sub-carrier bandwidth RSRPNR=SCSNRSCSLTERSRPLTE;

  - If LTE transmission is split into two separate portions for LTE PSCCH and LTE PSSCH, the RSRP on PSCCH part is calculated as RSRPPSCCH=RSRPPSSCH100.3;

  - FFS if average RSRP is used for LTE RSRP.

Proposal 5:

- Alt.3 is adopted for RSRP threshold configuration for LTE resource exclusion in NR module.

Proposal 6:

- LTE sub-channels with RSSI measurement are converted to NR SCIs with a default priority, default periodicity, and with RSSI measurement converted to approximate RSRP measurement considering sub-channel bandwidth.

Proposal 7:

- The subframes non-monitored on LTE sensing window are converted to NR slots not monitored in the sensing window.

- Periodicities for exclusion of non-monitored slots are a separate sub-set of LTE configured periodicities.

Observation 2:

- Rel-16 in-device co-existence framework provides mechanisms to resolve overlap of LTE and NR transmissions and/or receptions and separate consideration of this scenario in Rel-18 is not required.

Observation 3:

- The agreements on T and Tmax introduce an equivalent of a processing delay of LTE shared information to consider in the NR resource selection once this information is available in NR module, however, do not capture any age or relevance of the LTE shared information in moment n-T.

Proposal 8:

- The information shared by the LTE SL module corresponds to the sensing information that would be used by the LTE SL module for resource selection not earlier than in moment n - Tshare, where Tshare is FFS and may include Tshare = Tmax.

Proposal 9:

- In case of NR operating in 30 kHz SCS, for a ach candidate resource NR PHY layer shares with MAC layer information on whether a candidate resource overlaps with the first LTE subframe symbol or not.

Proposal 10:

- It is up to UE implementation to handle the cases when the additional restriction on starting positions of selected resources leads to impossibility to select a transmission resource.

Proposal 11:

- In case of NR 30 kHz SCS, RAN1 to clarify whether/when a UE may select a subsequent resource not overlapping with the first symbol of LTE subframe, which has the same or different frequency allocation from the preceding resource.

Observation 4:

- Regarding the assumption to take on how the LTE and NR module relate with each other, RAN1 has indeed assumed that simultaneous reception from both modules in the same channel from a single UE is possible, while any other specific details in terms of RF and baseband requirements are expected to be discussed by RAN4. Furthermore, as per WID description RAN1 has prioritized dynamic resource pool sharing based on operating combination A (i.e., mode 2 to mode 4 co-existence) and using type A devices, where Rel-16 in-device co-existence is assumed together with slot alignment between the two modules.

References

- RP-230077, “WID revision: NR sidelink evolution”, OPPO, March 2023.

- R1-2302797, “On the Channel Access Mechanisms for SL Operating in Unlicensed Spectrum”, Intel Cooperation, e-meeting, April 2023.

- R1-2302798, “On the Physical Layer Enhancements for SL Operating in Unlicensed Spectrum “, Intel Corporation, e-meeting, April 2023.

- R1-2302800, “On Sidelink Operation in FR2 Licensed Spectrum”, Intel Cooperation, e-meeting, April 2023.

- R1-2302271 (R4-2303718), “LS on co-channel coexistence”, 3GPP TSG-RAN WG4 Meeting #106, Athens Greece, February/March 2023, RAN4.

- Annex A – List of Agreements

This section captures all the agreements made during the past RAN1 meetings:

RAN1 #109e: Agreement For co-channel coexistence in Rel-18, no changes in the LTE SL specifications are allowed. Agreement For co-channel coexistence in Rel-18, Rel-16/17 simulation assumptions are reused for evaluation of solutions, except for the UE dropping model. - FFS: UE dropping model Agreement For the study of co-channel coexistence solutions in Rel-18, the combination of operational modes Mode 2 NR SL with Mode 4 LTE SL (Combination A) is considered with high priority. - FFS: Whether/how to support Mode 1 NR SL + Mode 4 LTE SL (Combination B) and/or Mode 2 NR SL + Mode 3 LTE SL (Combination C). Agreement For evaluation of co-channel coexistence solutions in Rel-18, support the inclusion of dual module devices with NR+LTE modules using the following UE dropping models: - UE Dropping Model A: The distance between 1 LTE SL module and 1 NR SL module are maintained as zero to model a co-located dual module device. The inter-device distance between any two adjacent devices in the same lane, which may be either a single module or a dual module device, is modified by doubling the time in the upper limit, resulting in max{2 meter, an exponential random variable with the average of the speed * 4sec}. - UE Dropping Model B: The distance between 1 LTE SL module and 1 NR SL module are maintained as zero to model a co-located dual module device. The inter-device distance between any two adjacent devices in the same lane, which may be either a single module or a dual module device, is maintained the same as current assumptions, i.e., max{2 meter, an exponential random variable with the average of the speed * 2sec}. Companies should mention the UE dropping model and the distribution of each device type (single/dual module) used in their simulation assumptions. Agreement Feasibility of semi-static resource pool partitioning and dynamic resource sharing as possible solutions for co-channel coexistence are to be studied. Agreement For studying the feasibility of dynamic resource sharing as a possible solution for co-channel coexistence, - For device type A, the NR SL module uses the sensing and resource reservation information shared by the LTE SL module. - FFS details on how the NR SL module uses this information. - FFS details on how the LTE SL module shares the information to the NR SL module, exact information shared, timeline etc. - FFS: Whether/how to define other method(s) for device type A to be aware of resources being occupied by LTE SL. - FFS: Whether/how device type B should be supported. RAN1 #110: Working assumption Co-channel coexistence between LTE SL and NR SL is supported for device type A. Device type A contains both LTE SL and NR SL modules. For device type A, the NR SL module may use the sensing and resource reservation information shared by the LTE SL module. Conclusion For co-channel coexistence in Rel-18, RAN1 concludes that the TDM-based semi-static resource pool partitioning based on Rel-16/17 specifications is one possible solution to ensure co-channel coexistence between LTE-V UEs and NR-V UEs. - Note: The LTE and NR resource pools do not overlap in time with each other in the TDM-based semi-static resource pool partitioning. - Note 2: Rel-16 in-device coexistence framework can ensure alignment between the slot boundary of the NR SL time slot and the subframe boundary of the LTE SL subframe - FFS: potential enhancements for synchronization can be further investigated Agreement For co-channel coexistence in Rel-18, dynamic resource pool sharing is studied, with the following constraints: - NR SL resource pool is configured with 15 kHz SCS. - FFS support of NR SL resource pool configured with higher SCS, including other solutions to overcome the AGC issue caused by the differing SCSs between the NR SL and LTE SL resource pools - For NR PSFCH (if configured), at least the following alternatives are studied: - Alt 1: Avoid PSFCH transmission in time slots that overlap with subframes used for LTE SL transmissions. - FFS: Avoiding PSFCH transmissions can be performed by the UE transmitting PSFCH and/or the UE transmitting PSSCH. - Alt 2: NR SL UEs use a periodically repeating set of PSFCH slots. - FFS: periodicities of the set. RAN1 #110b-e: Agreement For dynamic resource pool sharing, the candidate information shared by the LTE SL module to the NR SL module may include one or more of the following parameters, to be down-selected: - Time and frequency locations of reserved resources by other LTE UEs, determined based on decoded SCIs - SL RSRP measurement results - Resource reservation periods based on decoded SCI and for own LTE SL transmissions - Priority based on decoded SCI and for own LTE SL transmissions - Time and frequency location of resources used for own LTE SL transmissions - Candidate resource set SA or SB - SL RSSI measurements - LTE logical subframe related information - Resources corresponding to half-duplex subframes which are not monitored by the LTE SL UE Agreement For dynamic resource pool sharing, the NR SL module uses the information shared by the LTE SL module to the NR SL module to determine the set of resources for its own transmission. - FFS: which layer carries out the resource determination: PHY layer or MAC layer. Agreement For dynamic resource pool sharing, where the NR SL module uses the candidate information shared by the LTE SL module to the NR SL module, continue studying the following alternatives: - Alt 1: The LTE SL module provides the NR SL module with the candidate information (excluding at least the candidate resource sets SA or SB) - The NR SL module identifies a set of resources based on information shared by the LTE SL module. - FFS: how to identify the set of resources - The NR SL module excludes these identified resources from its own candidate resource set when performing the resource (re)selection procedure. - The exclusion process is performed in the PHY layer. - Note: implementation of Alt 1 should not have specification impact to LTE - Alt 2: The LTE SL module provides the NR SL module with the candidate resource sets SA or SB shared by the LTE SL module - The LTE PHY SL module is provided information from the higher layer to generate a candidate resource set SA or SB. The resource set SA or SB is then shared to NR SL module. - The NR SL module performs an intersection operation with the candidate resource set received from the LTE SL module and the candidate resource set generated by the NR SL module. - FFS: how to handle the case where this results in an insufficient set of resources - The intersection operation is performed in the MAC layer. - FFS: How to handle NR V2X parameter settings that are not supported by LTE V2X, e.g., periodicities, sub-channel sizes, etc - Note: implementation of Alt 2 should not have specification impact to LTE - In the next meeting strive to decide between the two alternatives Agreement For dynamic resource pool sharing, the NR SL module is expected to use the information shared by the LTE SL module to the NR SL module which is known by NR SL module at the latest T ms prior to slot n (as defined in clause 8.1.4 of TS 38.214), to determine a set of resources for its own (re)transmission. - T is defined using - T≤Tmax ms, and is based on UE implementation, according to the Rel-16 NR SL timeline for in-device coexistence. - FFS: Value of Tmax - FFS: any discussion on the earliest information, if needed RAN1 #111: Agreement Based on the agreement in RAN1#110bis-e, the value of Tmax = 4 ms. Agreement For dynamic resource pool sharing, the NR SL module uses the candidate information shared by the LTE SL module to the NR SL module, where - The NR SL module excludes resources based on the shared information from its own candidate resource set when performing the resource (re)selection procedure in the PHY layer. - FFS how to exclude resources at least based on the time and frequency locations of LTE SL transmissions that have been indicated in the shared candidate information. - FFS how the exclusion is performed according to clause 8.1.4 of TS 38.214. - FFS: whether/how NR SL module excludes resources not belonging to the generated LTE SL’s candidate resource set SB from its own candidate resource set. RAN1 #112: Agreement In NR SL resource (re)selection procedure, option 1 is adopted for how to determine candidate resource set for NR SL considering the LTE SL reserved resources by other LTE SL UE - Option 1: The PHY layer of NR SL module excludes NR SL candidate resources overlapping with LTE SL reserved resources by other LTE SL UE when the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold, where the SL RSRP threshold is derived based on LTE SL priority of other LTE SL UE and NR SL priority for NR SL transmission - For the list of the above initial SL RSRP threshold, down-select one of followings: - Alt 1: NR SL RSRP threshold list (pre)configured in a NR SL resource pool - Alt 2: LTE SL RSRP threshold list (pre)configured in a LTE SL resource pool - Alt 3: SL RSRP threshold list separately (pre)configured for dynamic resource pool sharing - Alt 4: SL RSRP threshold list separately (pre)configured for dynamic resource pool sharing - A different SL RSRP threshold list may be (pre)configured for selecting single slot resources in NR SL slots with NR PSFCH - For the LTE SL periodic reserved resources by other LTE SL UE, - For determining the above LTE SL reserved resources, the time-and-frequency resources of LTE SL reserved resources by other LTE SL UE are repeated Q times according to the LTE SL resource reservation period - FFS details including at least whether the formula of Q in Section 14.1.1.6 in TS 36.213 or the formula of Q in Section 8.1.4 in TS 38.214 is used - The PHY layer of NR module applies the above procedure in Step 6 in Section 8.1.4 of TS 38.214 - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of which NR SL periodic resources are overlapping with LTE SL reserved resources by other LTE SL UE when the SL RSRP value associated with the LTE SL reserved resources is higher than a SL RSRP threshold according to condition c in Step 6 in Section 8.1.4 of TS 38.214 - Note: It is assumed that the information relevant to LTE SL reserved resources by other LTE SL UE used in the above procedure is shared from LTE SL module to NR SL module - FFS whether/how LTE SL RSRP is applied Working assumption - For dynamic resource pool sharing, select one option between 1-1 and 1-2, and select one option between 3-1 and 4-1-a, and define how the two selected options can be used in the operation: - Option 1-1: Avoiding PSFCH transmissions overlapping with LTE SL resources reserved for LTE SL transmissions in the time domain is performed by the UE transmitting PSFCH - FFS whether/how to define condition(s) under which the UE transmitting PSFCH drops its PSFCH transmission overlapping with the LTE SL resources reserved for LTE SL transmission - Option 1-2: Avoiding PSFCH transmissions overlapping with LTE SL resources reserved for LTE SL transmissions in the time domain is ensured by the UE transmitting PSSCH - FFS whether/how to define condition(s) of selecting NR SL candidate resources for NR SL PSCCH/PSSCH - Option 3-1: Additional PSFCH periodicity(ies) is introduced - Alt 3-1-a: 10 logical slots - Alt 3-1-b: 8 logical slots - Alt 3-1-c: 5 logical slots - FFS: Alignment between PSFCH periodicity and LTE logical subframes should be ensured by proper configuration - Option 4-1: PSFCH resources and LTE SL resources are TDMed - Alt 4-1-a: PSFCH resources is confined within the guard symbol of the LTE SL subframe in the time domain - Note: selecting a single option is not precluded Agreement In NR SL resource (re)selection procedure, down-select one of followings for how to determine candidate resource set for NR SL considering the LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission - Option 1: The PHY layer of NR SL module excludes NR SL candidate resources in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission - For the LTE SL periodic resources selected to be used for LTE SL module’s own LTE SL transmission, - For determining the above LTE SL selected resources, the time-and-frequency resources of LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission are repeated according to the LTE SL resource reservation period and LTE SL resource reselection count - The PHY layer of NR module applies the above procedure in Step 5 in Section 8.1.4 of TS 38.214 - Note: For periodic resource reservation of NR SL transmission, the PHY layer of NR SL module further excludes NR SL candidate resources of which NR SL periodic resources are in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission according to Step 5 in Section 8.1.4 of TS 38.214 - Note: When the PHY layer of NR module cancels the above procedure according to Step 5a in Section 8.1.4 of TS 38.214, UE selects either LTE SL transmission or NR SL transmission according to Rel-16 NR SL in-device coexistence rule - FFS: whether the above procedure is applied based on the priority of LTE SL transmission and the priority of NR SL transmission - Note: It is assumed that the information relevant to LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission used in the above procedure is shared from LTE SL module to NR SL module - Option 2: The PHY layer of NR SL module determines NR SL candidate resources regardless of whether the NR SL candidate resources are in a NR SL slot overlapping with LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission - UE selects either LTE SL transmission or NR SL transmission according to Rel-16 NR SL in-device coexistence rule