3GPP TSG RAN WG1 #112b-e			   R1-2302800

e-Meeting, April 17th – 26th, 2023

Source:	Intel Corporation

Title:	On Sidelink Operation in FR2 Licensed Spectrum

Agenda item:	9.4.3

Document for:	Discussion and Decision

# Introduction

In 3GPP TSG RAN meeting #94, a new work item related to NR Sidelink (SL) evolution was approved, whose description has been recently updated during RAN#98-e [1] to study enhancements needed to support SL operation in FR-2 licensed spectrum. In this matter, the related WID currently includes the following objective:

- Study enhanced sidelink operation on FR2 licensed spectrum [RAN1, RAN2] - Focus only on updating the evaluation methodology for commercial deployment scenario in 4Q 2022. [RAN1] - Study is limited to the support of sidelink beam management (including initial beam-pairing, beam maintenance, and beam failure recovery, etc) by reusing existing sidelink CSI framework and reusing Uu beam management concepts wherever possible. [RAN1, RAN2] - Beam management in FR2 licensed spectrum considers sidelink unicast communication only.

In the context of NR SL in FR2, during the past RAN1 meeting [2] the agreements made are captured in the Annex A section. In this contribution several aspects related to the operation of SL in FR-2 licensed spectrum are discussed, while our views on other aspects of Rel.18 NR SL evolution are provided in our companion contributions [3]-[5].

Introduction

The main purpose of Sidelink Beam Management (SL BM) is to find transmit and receive beams for the communication between two devices that enable communication as well as fulfil QoS criteria, which also includes: i) the maintenance of these beams until they are no longer needed, and ii) detecting and responding to the case that communication with the selected beam fails. In the following, it is assumed that each transmission beam corresponds to a reception beam, similar to Rel-17 unified TCI framework in case of joint DL/UL TCI state, thus it is not required that the transmit and the receive beams of a device are trained separately.

The task of selecting a beam from a codebook could follow any of the beam sweeping procedures illustrated in Figure 1. SL FR2 is targeting any device category, but for illustration purpose the devices are represented by cars. In Procedure 1 both the transmit and receive beams are changed and tested in one procedure. If there are MT transmit beams and MR receive beams, this means that MT ⸱MR beam pairs need to be tested. This has the advantage that both beamforming gains of the transmit and receive antenna panels can be utilized during this procedure without prior knowledge. However, at the same time there is a disadvantage of the overhead associated with the testing of all these beam pairs. As for Procedures 2 and 3, either the transmit or the receive beam is changed, while the beam at the other side is kept the same. This can either be employed by a fixed beam or a quasi-omnidirectional beam. It is worth noting that using a quasi-omnidirectional beam reduces the coverage as the beamforming gain of the antenna panel on one of the sides cannot be utilized. Both the initial beam-pairing as well as the beam maintenance can use any of these procedures.

| Procedure 1 |  |
| --- | --- |
| Procedure 2 | Procedure 3 |

Figure 1 - Three solution directions for SL beam sweeping.

For SL beam management, the following high-level topics need to be considered for study:

- Design assumptions: There are number of design assumptions that need to be made for SL FR2. Most of them are based on Rel-16/17 assumptions for sidelink communication.

- Initial beam-pairing: This procedure is the most important to establish a connection between two UEs for communicating in SL FR2. Role assignment (initiator or responder) as well as the connection to the related reference signals need to be established. Also, the criteria for the beam reports, the beam reports’ contents, timing of the beam report and their communication interface need to be defined.

- Beam maintenance: For beam maintenance there are two important aspects: i) triggering conditions and timing, and ii) procedure to be used.

- Beam failure and recovery: It is important that devices detect when beam alignment is failing or is about to fail. Depending on the scenario as well as other system parameters, different solutions are possible.

- Reference signals for SLBM: One of the key aspects is the definition of reference signals for beam management. They need to be defined for all beam management tasks.

- Resource allocation enhancements: Both network-controlled, and UE-autonomous resource allocation modes should be supported. Rel-16/17 resource allocation design may not be suitable for SL FR2 communication aspects. It is beneficial to revisit some of the solutions to optimize the performance for SL FR2.

These topics are discussed one-by-one in the subsequent sections.

# Design Assumptions

There are a few fundamental system assumptions, which are important to be highlighted and made before RAN1 can move forward. Most of these are related to physical layer design assumptions made during Rel-16 and Rel-17 sidelink discussions. Additional assumptions beyond the ones made during Rel-16 and Rel-17 SL discussions are related to the frequency range as well as the simultaneous multi-panel transmission and reception capability for SL FR2 UEs.

In our understanding the following Rel-16/17 design assumptions should be reused for SLBM design:

- Flat hierarchy among users at the physical layer: This can be understood that all devices are equal in terms of their ability to start any procedure related to SLBM.

- Design considers both mode-1 and mode-2 operation: In our understanding both network and UE autonomous resource operation need to be supported for SLBM.

- No coexistence of mode-1 and mode-2 in the same resource pool: Like the Rel-16/17 SL design no coexistence of these two operation modes is considered at the physical layer.

    - The following Rel-16 design assumptions are reused for the SL FR2 beam management design:

      - Flat hierarchy among users at the physical layer

      - Both mode-1 and mode-2 operation support

      - No coexistence of mode-1 and mode-2 in the same resource pool

In addition, it is worthwhile to discuss about multi-panel considerations for UEs. In our understanding a spherical coverage in all directions is impossible with a single panel of a device. Simultaneous multi-panel transmission and/or reception can significantly reduce the overhead associated with initial beam-alignment, beam maintenance, as well as beam recovery and therefore it is worthwhile to consider it during this study item.

    - Study both, UEs with and without simultaneous multi-panel transmission and reception.

During the last RAN1 meeting [2] the assumption of beam correspondence was extensively discussed. In our understanding assuming beam correspondence reduces the complexity as well as the probability of the system failing to establish a connection. This is especially true for the case for SL FR2 standalone operation. Since the time of this study item is rather limited, it would be practical for the group to focus on systems with beam correspondence assumption, while during normative phase design without beam correspondence could be considered assuming UEs with different capabilities.

    - Prioritize UEs with beam correspondence for the study of SL FR2 BM.

Like in the legacy UL/DL case, the procedures illustrated in Figure 1 should be also defined for SL. These procedures can be used to design the different operations to align and maintain beams. It needs to be noted that the beam operations can be performed per panel for UEs operating with multiple panels simultaneously.

    - Define the following procedures for the study for SL BM:

      - P1: Both a Tx UE and a Rx UE perform beam sweeping,

      - P2: A Tx UE performs beam sweeping,

      - P3: A Rx UE performs beam sweeping.

# Initial Beam-Pairing and Directional Discover

For beam sweeping operation it is essential that each device knows which role it performs during the beam sweeping. In this context we define the roles of ‘initiator’ and ‘responder’ as the device transmitting during the initial beam sweeping and the device receiving during the initial beam sweeping, respectively. The example provided in Figure 2 illustrates the case that two devices take the role of the initiator for procedure 1. As both are transmitting, they might not know about each other presence. In a system that uses device autonomous resource selections this means that a directional discovery including initial beam-pairing can only be performed by different devices taking the role of the initiator and responder based on a randomized procedure. The procedure of initial beam-pairing would need to be performed before any communication; thus, the high energy footprint would be present even for the case where no other device with SL FR2 capability is present.

Figure 2 - Role collision during initial beam-pairing.

Based on these considerations it is worthwhile to study which information can be available at the different devices before the initial beam-pairing. These considerations are not limited to only RAN1 considerations; thus, it is worthwhile to exchange with RAN2 about this topic, especially for SL FR2 mode-2 operation. The whole process of information exchange to assist initial beam-pairing plus the initial beam-paring itself as well as the following message exchange can be considered under the concept of directional discovery.

- Study which information can be used to assist the initial beam-pairing.

After the responding UE has detected the best beam for itself as well as the best beam for the initiating UE, this information needs to be reported to the initiating UE. It is also possible that the initiating UE is detecting this information by itself. Therefore, RAN1 needs to study how it can be ensured that both UEs have the same information about the optimal beams after the initial beam pairing.

    - Study which information exchange is necessary to ensure that both UEs have the same understanding of the optimal beam at both devices.

Before initial beam alignment there is directional communication possible. To optimally utilize the beamforming gain at both UEs it is essential that the beam on both sides of the communications is trained. For the case of both UEs having beam correspondence it is possible to perform the procedure P1 once with any of the UEs being the transmitter and using the corresponding beams for the transmission of information in the other direction. In the case without beam correspondence this process needs to be performed for each communication direction.

    - Procedure P1 where both the Tx and the Rx UE perform beam sweeping is utilized for initial beam pairing.

In our understanding before initial beam pairing, it is only possible to use wide beams at both UEs. This means that the coverage may be severely limited, and it is in many cases unlikely that a connection can be established before the initial beam pairing. Therefore, especially for the case of SL FR2 standalone operation the PC5 unicast connection should be established after the initial beam pairing. As described above in the case that coordination information can be exchanged through other communication interfaces it is possible to assists in the initial beam pairing. However, in our understanding exchanging this coordination information for initial beam pairing does not necessarily mean establishing a unicast connection.

    - A PC5 unicast link is established after initial beam pairing for SL FR2 standalone operation.

# Beam Maintenance

Regarding beam maintenance there are two major directions that need to be studied. First the conditions that trigger beam maintenance need to be studied. In this context it is important that after the triggering both participating UEs know the beam maintenance is active and know how to operate in this event.

    - Study which conditions may trigger beam maintenance.

When beam maintenance is triggered and after both UEs are informed, it is important to know how beam maintenance is performed. As SL FR2 beam management targets a wide set of use-cases, different system profiles and channel conditions will require different techniques for beam maintenance. For example, in a high mobility scenario it is required that beam maintenance is performed more often compared to a lower mobility scenario.

    - Study which techniques can be utilized for beam maintenance.

# Beam Failure and Recovery

In contrast to beam management for the Uu link there is no physical layer hierarchy between different UEs, this means that beam failure and recovery can be detected by either UE. Thus, RAN1 should study the conditions to detect and declare the beam failure. In our understanding the legacy definition for Radio Link Failure (RLF) can be a starting point for these conditions.

    - Study the conditions to detect and declare a sidelink beam failure.

After the beam failure is detected by one of the devices, it is necessary that the other device also knows about this state to start the beam recovery. It is possible that this information is transmitted to the other device. It is also possible that through system design the beam failure is always detected simultaneously by both UEs.

    - Study how two UEs can get a mutual agreement on the beam recovery procedure.

In our understanding the sidelink radio link failure (SL RLF) should serve as a starting point for beam failure detection. In our understanding the requirements for beam failure detection are similar compared to RLF, thus the framework can be reused. As in our understanding the timeframe for beam failures might be shorter compared to RLF. Thus, the current RLF timelines need to be adjusted to fit the expected timeline of beam failures.

    - Sidelink RLF should be used as the starting point for beam failure.

# Beam Reporting

In our understanding at different stages of the system the beam report might be reported through different interfaces and for different purposes. For example, SL RRC is only available after a unicast connection is established. In addition, it is also important to support case where the beam report for the initial beam alignment is transmitted on another interface than SL FR2 PC5 as at this stage without established beams no communication might yet be possible.

    - SL Beam reporting can utilize the following containers:

      - SL MAC CE,

      - SL RRC,

      - Signalling between the UEs through the network.

# Reference Signals for SLBM

In the Uu link the SSB as well as CSI-RS are used for beam management. During the Rel-16 SL design, it was already foreseen that the S-SSB may be used for SL FR2 beam sweeping during the initial beam-alignment, thus additional repetitions are configurable for 60 and 120 kHz SCS. However, this means that per S-SSB slot only a single beam pair can be trained. Other reference signal definitions can be studied as well. They should target enabling a beam-alignment with a lower overhead relative to utilizing S-SSB transmissions.

    - Study if other reference signals besides the ones present in Rel-16 S-SSB can be used for initial beam alignment.

In addition, to the reference signals for the initial beam-alignment it is also necessary to consider the requirements of beam maintenance and beam recovery. It is possible to use S-SSB for this purpose, but it is also possible that other reference signals can be used for this purpose.

    - Study which reference signals to use for beam maintenance and beam recovery.

# Resource Allocation Enhancements

In our understanding the resource allocation designed for Rel-16 NR SL mainly targets avoiding collisions and enabling fair access to the channel in a congested radio environment. Due to utilizing directional beamformed transmissions, for NR SL FR2 the system is no longer always limited by interference at the physical layer. Furthermore, communication in FR2 can be less reliable as beams can fail at any moment. The Rel-16 resource allocation design is thus targeting a system with different consideration. We therefore think to support SL FR2 operation it is essential to also adapt the resource allocation for it, specifically the mode 2 resource allocation can benefit from incorporating the specific channel characteristics of systems operating in FR2. Thus, to improve the system performance it is desirable to study how the resource allocation can be changed to consider these changes in system design assumptions.

    - Study resource allocation enhancements for SL FR2 considering the changed system assumption and changed interference environment.

Conclusion

In this contribution, we have provided our views on the Study of Sidelink Communication in FR2. In summary, we have the following list of proposals:

    - The following Rel-16 design assumptions are reused for the SL FR2 beam management design:

      - Flat hierarchy among users at the physical layer

      - Both mode-1 and mode-2 operation support

      - No coexistence of mode-1 and mode-2 in the same resource pool

    - Study both, UEs with and without simultaneous multi-panel transmission and reception.

    - Prioritize UEs with beam correspondence for the study of SL FR2 BM.

    - Define the following procedures for the study for SL BM:

      - P1: Both a Tx UE and a Rx UE perform beam sweeping,

      - P2: A Tx UE performs beam sweeping,

      - P3: A Rx UE performs beam sweeping.

- Study which information can be used to assist the initial beam-pairing.

    - Study which information exchange is necessary to ensure that both UEs have the same understanding of the optimal beam at both devices.

    - Procedure P1 where both the Tx and the Rx UE perform beam sweeping is utilized for initial beam pairing.

    - A PC5 unicast link is established after initial beam pairing for SL FR2 standalone operation.

    - Study which conditions may trigger beam maintenance.

    - Study which techniques can be utilized for beam maintenance.

    - Study the conditions to detect and declare a sidelink beam failure.

    - Study how two UEs can get a mutual agreement on the beam recovery procedure.

    - Sidelink RLF should be used as the starting point for beam failure.

    - SL Beam reporting can utilize the following containers:

      - SL MAC CE,

      - SL RRC,

      - Signalling between the UEs through the network.

    - Study if other reference signals besides the ones present in Rel-16 S-SSB can be used for initial beam alignment.

    - Study which reference signals to use for beam maintenance and beam recovery.

    - Study resource allocation enhancements for SL FR2 considering the changed system assumption and changed interference environment.

References

- RP-222806, “WID revision: NR sidelink evolution”, OPPO, e-meeting, Dec. 2022.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 #112, Athens Greece, February/March 2023.

- R1-2302797, “On the Channel Access Mechanisms for SL Operating in Unlicensed Spectrum”, Intel Corporation, e-meeting, April 2023.

- R1-2302798, “On the Physical Layer Enhancements for SL Operating in Unlicensed Spectrum”, Intel Corporation, e-meeting, April 2023.

- R1-2302799, “On the Co-channel Coexistence between LTE and NR Sidelink”, Intel Corporation, e-meeting, April 2023.

- Annex A – List of Agreements

This section captures all the agreements made during the past RAN1 meeting [2]:

RAN1 #112: Agreement For sidelink beam management, RAN1 is to study - how transmit beam(s) training and/or receive beam(s) training is performed - whether and how spatial related information (e.g., TCI, QCL, beam ID, etc) information could be identified - the relationship between PC5 unicast link establishment and sidelink initial beam pairing (e.g., whether initial beam pairing procedure starts before, during or after sidelink unicast link establishment procedure.) Agreement RAN1 is to study sidelink beam measurement and reporting schemes (e.g., periodicity, contents, container, timing, procedure, etc.) for sidelink beam maintenance. Agreement RAN1 is to study sidelink beam indication and switching schemes (e.g., framework, general procedure, contents, signaling, timing, etc.) for sidelink beam maintenance. Agreement RAN1 is to study the information related to a sidelink beam failure instance that the PHY layer provides to the MAC layer.