3GPP TSG RAN WG1 #112bis-e  													                     R1-2302804

e-Meeting, April 17th – April 26th, 2023

Source: 	Intel Corporation

Title:	On DL and UL carrier phase positioning

Agenda item:	9.5.2

Document for:	Discussion and Decision

# Introduction

In Expanded and Improved NR Positioning WI, the following objective has been identified for the support of PRS/SRS bandwidth aggregation [1]:

- Specify physical layer measurements and signalling to support NR DL and UL carrier phase positioning for UE-based, UE-assisted, and NG-RAN node assisted positioning [RAN1, RAN2, RAN3, RAN4]. - Existing DL PRS and UL SRS for positioning are used for NR carrier phase measurements. - Specify measurements that are limited to a single carrier/PFL. - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements without measurement gaps in connected and inactive mode (including PRS measurement period/reporting) and procedures [RAN4].

During RAN1 #112 meeting, the following were agreed [3]:

Agreement To enable UE-based and UE-assisted NR carrier phase positioning (CPP), one or both of the following new measurements should be introduced: - DL carrier phase (CP), which is obtained by a UE measuring the DL PRS signal(s) from a TRP. - FFS: The detailed definition of the DL CP - DL carrier phase difference (CPD), which is the difference of two DL CPs from two TRPs - FFS: The detailed definition of the DL CPD To enable NG-RAN node-assisted NR carrier phase positioning (CPP), the following new measurement should be introduced: - UL carrier phase (CP), which is obtained by a TRP measuring the UL SRS for positioning or MIMO SRS from a UE. - FFS: The detailed definition of the UL CP Agreement NR DL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements (REs) that carry the DL PRS signals configured for the measurement. A RSCP is associated with a specific RF frequency. - FFS: the reference point of the RSCP - FFS: whether/how the measurement timing is defined - Note: the i-th path is used for the sake of definition, whether only the first path or additional paths will be supported is subject to further discussion - Note: Whether to capture the above definition into TS 38.215 depends on whether RAN1 decides to introduce DL carrier phase measurement for NR CPP Agreement For NR DL reference signal carrier phase difference (RSCPD) measurement for NR CPP, the RSCPD is defined as the difference of RSCPs measured from the DL PRS signals from target TRP and reference TRP. - FFS: whether/how to define per path RSCPD - Note: Whether/how to capture the above definition into TS 38.215 depends on whether RAN1 decides to introduce DL carrier phase difference measurement for NR CPP Agreement For NR carrier phase positioning, at least support the following approach: enable a UE/TRP to report carrier phase measurements together with the legacy positioning measurements to LMF - FFS: which legacy positioning measurements among RSTD, RTOA, UE Rx-Tx time difference measurements, gNB Rx-Tx time difference measurements Agreement NR UL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements (REs) that carry the UL SRS signal for positioning purpose configured for the measurement. A UL RSCP is associated with a specific RF frequency. - FFS: the reference point of the UL RSCP - FFS: whether/how the measurement timing is defined - Note: the i-th path is used for the sake of definition, whether only the first path or additional paths will be supported is subject to further discussion - Note: The support of MIMO SRS for positioning is transparent to UE Agreement To support NR carrier phase positioning, further consider the following options: - Option 1: Support a UE/TRP to report the carrier phase measurements of more than one frequency within a PFL/carrier to LMF - NOTE: the frequency can be the carrier frequency or the frequency of a subcarrier - FFS: the details of reporting, e.g., the maximum number of reported frequencies within a PFL/ carrier - Option 2: Introduce and report a new type of UE/TRP measurement based on carrier phase differentials across multiple subcarriers within a PFL/carrier - NOTE: carrier phase differentials across multiple subcarriers within a carrier can be related to time of arrival - Option 3: Support a UE/TRP to optionally report an estimated integer ambiguity and/or search range of the integer ambiguity to LMF - Option 4: Support LMF to provide the expected integer ambiguity range at least for UE-based NR CPP in the positioning assistance data. Agreement - Rel-17 LOS/NLOS indication (when indicated) applies for the carrier phase measurement(s) in the same report.

In the contribution, we present our views on introducing support for DL and UL carrier phase positioning (CPP) in NR systems.

# Defining carrier phase measurements and their reporting

## DL CPP

Considering the decisions from RAN1 #112, for DL CPP, it is currently open whether to introduce DL RSCP or DL RSCPD or both as metrics for reporting DL CP measurements.

Towards this, we note that the choice of DL RSCPD should be somewhat natural when considering the impact from initial phase at UE receiver. With the differential nature of the reporting quantity, DL RSCPD offers resilience to initial phase at UE receiver and can thereby avoid an increased dependency on use of PRUs for calibration of the UE Rx initial phase.

Further, in our view, DL/UL CPP should be specified also as independent positioning methods. In such a case, a UE may be requested to provide measurement reports for DL RSCPD and UE Rx-Tx time difference as an example of a hybrid positioning approach. One concern with introduction of DL RSCPD as independent positioning method is regarding the resolution of integer ambiguity. When combined with legacy positioning methods, the pseudorange may be estimated using a legacy positioning method to aid integer ambiguity resolution (IAR). Given that multiple options for IAR are under consideration, it can be further discussed under what conditions and along with what associated information (for IAR) DL RSCPD may be supported as independent positioning method.

Proposal 1

- Support DL RSCPD reporting as an independent positioning method.

  - FFS: Exact conditions and associated assistance information to resolve integer ambiguity.

Last meeting it was also left open as to the combination of RSCP measurement reporting and particular legacy positioning methods.

In addition to DL RSCPD, it may be considered further whether to also define DL RSCP reporting. One of the arguments in its favour is that, if DL CP measurements are requested to be reported only with legacy measurements, then, when combined with legacy UE Rx-Tx time difference measurements, a UE may only report the DL RSCP metric for the target gNB/TRP. However, again, this would require calibration of the UE Rx initial phase which may not be trivial.

Further, in view of mobility impact and cell changes, as for the case of DL RSTD, the LMF may provide a target UE with a reference cell that is independent of the serving cell.

Proposal 2

- Introduce DL RSCPD for DL CPP considering its resiliency to initial phase at UE Rx.

  - FFS: Reporting of RSCP corresponding to a target TRP – e.g., DL RSCPD may be sufficient if reporting of DL CP measurements is defined as an independent positioning method.

- LMF configures a target UE with a reference cell that is independent from serving cell.

Proposal 3

- For DL CPP, support reporting of DL RSCPD along with at least RSTD and, if DL RSCPD is defined as independent positioning method, then also along with UE Rx-Tx time difference measurements.

  - FFS: Other combinations, e.g., if DL RSCP is also supported.

Regarding consideration of defining measurement timing, two aspects should be taken into account:

- For DL RSCPD, to minimize impact from error sources like CFO and frequency drift, Doppler, etc., having the measurements for a target and reference TRPs as close as possible in time is desirable. Thus, for DL RSCPD, measurement timing should be defined for the measurements on the DL PRS from a given TRP and reference TRP, e.g., similar to DL RSTD:

  - The measurements are performed on DL PRS received in a subframe SF_j from target TP j and in a subframe SF_i from reference TP i that is closest in time to subframe SF_j.

- While multiple DL PRS resources may be used, it may be beneficial in some cases to also restrict the measurement within a relatively short time window.

Proposal 4

- For DL CPP,

  - The measurements are performed on DL PRS received in a subframe SF_j from target TP j and in a subframe SF_i from reference TP i that is closest in time to subframe SF_j.

  - Multiple DL PRS resources may be used to determine the carrier phase.

    - LMF may optionally request a target UE to perform measurements that are restricted to within a time window.

During RAN1 #112, it was agreed that the carrier phase is defined with respect to a “specific RF frequency”, but further details were not concluded. Towards this, in our view a default should be defined, and by default this could correspond to the center frequency of a given DL PFL. This leads to aligned reference frequency assumption across TRPs within a DL PFL.

Proposal 5

- For DL CPP, as default, the “specific RF frequency” for the carrier phase corresponds to the center frequency for the DL PFL.

On the reference point for DL RSCP measurements, in our view, a baseline assumption similar to existing use of UE antenna connector for RSTD can be used. While it may be appealing to also consider antenna phase center as the reference point for RSCP measurements for higher accuracy, its benefits depend on appropriate modelling and reporting of the PCO, and thus, at this stage, may be deprioritized for potential future optimization.

Proposal 6

- For DL CPP, reference point for DL RSCP measurement is the UE receiver antenna connector is assumed as the baseline.

  - FFS: Further enhancements, including PCO reporting.

Whether or not additional paths may be reported for DL RSCPD was not concluded during last meeting. In our view, considering the target accuracy regime for CPP for the fractional wavelength distance, differentiation of paths within a single wavelength may not be easy and thus, reporting of additional paths may not be very useful in practice. Thus, it is preferred that DL RSCPD is defined only with respect to the first detected path.

Proposal 7

- DL RSCPD is defined only with respect to the first path detected in time.

## UL CPP

For UL CPP, it was already agreed that UL RSCP would be used as the metric for UL carrier phase reporting. Thus, it would be rather essential to utilize additional means, e.g., use of PRUs and double differential methods, to eliminate the effects of Tx and Rx initial phases.

On whether to define UL CPP as an independent positioning method, similar to the case of DL CPP, in our view, UL CPP based on UL RSCP reporting should be introduced as an independent positioning method. Exact conditions related to IAR can be considered further.

Proposal 8

- Support UL RSCP reporting by a gNB as an independent positioning method.

  - FFS: Exact conditions and associated assistance information to resolve integer ambiguity.

Further, on combination with legacy positioning methods, reporting of UL carrier phase along with both UL RTOA and gNB Rx-Tx time difference methods can be requested by an LMF.

Proposal 9

- LMF can request a gNB to report UL RSCP along with UL RTOA and/or gNB Rx-Tx time difference measurements.

Similar to DL RSCP measurements, while multiple SRS (or SRSp) resources may be used, it may be beneficial in some cases to also restrict the measurement within a relatively short time window.

Proposal 10

  - Multiple SRS/SRSp resources may be used to determine the UL carrier phase.

    - LMF may optionally request a gNB to perform measurements that are restricted to within a time window.

On the specific RF frequency an UL RSCP may correspond to, similar to the DL CPP, a default choice should be defined. Either the center frequency of the UL carrier or the RF frequency corresponding to the center of the SRS/SRSp resource on which UL RSCP is measured can be used for this purpose.

Proposal 11

- For UL RSCP, as default, the “specific RF frequency” for the carrier phase corresponds to one of (to be downselected):

  - the center frequency for the UL carrier, or

  - RF frequency corresponding to the center of the SRS/SRSp resource on which UL RSCP is measured.

Also, regarding the reference point for UL RSCP measurements, it would be reasonable to consider the options defined for UL RTOA and gNB Rx-Tx time difference in current specifications as a function of gNB types.

Proposal 12

- For UL CPP, reference point for UL RSCP measurement is assumed as defined for UL RTOA or gNB Rx-Tx time difference as the baseline.

  - FFS: Further enhancements, including PCO reporting.

# Addressing multipath and integer ambiguities

## Multipath mitigation

During the SI phase, it was observed that multipath effects can significantly impact the accuracy of CPP methods and thus, the carrier phase measurements should be defined with a certain level of robustness against multipath effects. Otherwise, in an environment with rich multipath, it is rather unlikely to be able to have any meaningful benefit from using CPP methods.

The following observations and options to address multipath were identified during the SI phase [2]:

The impact of multipath/NLOS on NR carrier phase positioning is evaluated during the study item. Based on the study, it is concluded that multipath/NLOS deteriorates the performance of carrier phase positioning, and it is necessary to consider multipath mitigation for NR carrier phase positioning. The effectiveness of the following multipath mitigation methods for NR carrier phase positioning is studied: - Identification and separation of the first path and other paths. - Reporting of the carrier phase of the first path, and optionally, the additional paths. - The use of LOS/NLOS indication for the carrier phase measurements. - NOTE: Rel-17 LOS/NLOS indicator can be considered as a starting point. - The report of other channel information, such as existing RSRP/RSRPP.

Towards this, we think the most straightforward and baseline option would be to limit the carrier phase measurements to the first path detected in time.

Further, considering the strict scoping of the normative work as defined in the WID, we think among other methods, only the option of LOS/NLOS indication for the carrier phase measurements can be considered by reusing the related Rel-17 feature.

Thus, we have the following proposal.

Proposal 13

- In consideration of multipath environments, adopt the following solutions:

  - Measurements: For both DL and UL CPP, limit reporting of carrier phase to that of the first path detected in time.

  - Assistance information: Rel-17 LOS/NLOS indication can be the baseline.

In a sense, both of the above have already been agreed during RAN1 #112 meeting. In particular, for LOS/NLOS indication, it was agreed only in the context of reporting of carrier phase along with legacy positioning methods:

Agreement - Rel-17 LOS/NLOS indication (when indicated) applies for the carrier phase measurement(s) in the same report.

Considering that in our view, DL/UL CPP should also be introduced as independent positioning methods, LOS/NLOS indication should also be introduced as part of DL RSCPD or UL RSCP as independent positioning report. Further, towards this, the existing design (using “hard” and “soft” information as quantifiers) should be taken as the baseline.

Proposal 14

- Introduce LOS/NLOS reporting as optional assistance information along with reporting of CP(D) measurements for DL/UL CPP as independent positioning method.

  - Reuse R17 LOS/NLOS reporting quantity/granularity as baseline.

## Integer Ambiguity Resolution (IAR)

The issue of Integer Ambiguity (IA) and its necessary resolution was identified during the SI as a critical factor to enable CPP methods in practice. Although it is acknowledged that the WID does not explicitly include normative work scope for detailed investigations for potential methods for Integer Ambiguity Resolution (IAR) as implementation-based methods are assumed for IAR, some assistance information would be essential for UE-assisted and NG-RAN-assisted CPP methods. For UE-based CPP, the problem of IAR may be left up to UE implementation.

Observation 1

- Although for some cases like UE-based positioning, IAR may be left up to implementation, such may not be feasible for all cases, e.g., for UE-/NG-RAN-assisted positioning. Thus, some additional information to facilitate IAR should be considered.

During RAN1 #112 meeting, the following options were identified for further consideration:

Agreement To support NR carrier phase positioning, further consider the following options: - Option 1: Support a UE/TRP to report the carrier phase measurements of more than one frequency within a PFL/carrier to LMF - NOTE: the frequency can be the carrier frequency or the frequency of a subcarrier - FFS: the details of reporting, e.g., the maximum number of reported frequencies within a PFL/ carrier - Option 2: Introduce and report a new type of UE/TRP measurement based on carrier phase differentials across multiple subcarriers within a PFL/carrier - NOTE: carrier phase differentials across multiple subcarriers within a carrier can be related to time of arrival - Option 3: Support a UE/TRP to optionally report an estimated integer ambiguity and/or search range of the integer ambiguity to LMF - Option 4: Support LMF to provide the expected integer ambiguity range at least for UE-based NR CPP in the positioning assistance data.

As discussed in the previous section, reporting of RSCP/RSCPD measurements with legacy positioning measurements can serve to aid in IAR. This is always an option that may be available whenever carrier phase measurements are reported along with legacy measurements.

Observation 2

- Reporting of CP/CPD measurements along with legacy positioning methods can help with Integer Ambiguity Resolution (IAR).

Next, we observe that, in theory, both Options 1 and 2 may be utilized in a similar manner, e.g., at the LMF, to resolve the integer ambiguity. Option 1 would require reporting or configuration of reference RF frequencies for each of the multiple measurements of the carrier phase (RSCP/RSCPD). On the other hand, Option 2 reports a more direct quantity for estimates of the pseudorange that is equivalent to a TOA metric. Further, the reported metric can be defined following RSTD type definition, likely with finer granularity, and multiple such metrics could be reported by a UE or gNB/TRP to an LMF. Considering the more direct definition of the reported quantity as it relates to the pseudorange, Option 2 is slightly preferred.

Option 3 would be a straightforward option for the measuring entity (UE/gNB) to report, and should be considered for optional reporting to the LMF.

However, Option 4 (“Support LMF to provide the expected integer ambiguity range at least for UE-based NR CPP in the positioning assistance data”) may have limited usefulness in practice. As noted in Observation 1, for UE-based NR CPP, it may be left up to UE implementation, e.g., to utilize legacy measurements like RSTD to estimate the IA range. Thus, Option 4 may be deprioritized.

Proposal 15

- For integer ambiguity resolution (IAR), introduce the following options in addition to the option of reporting RSCP/RSCPD with legacy positioning metrics,

  - Option 2: Introduce and report a new type of UE/TRP measurement based on carrier phase differentials across multiple subcarriers within a PFL/carrier.

    - NOTE: carrier phase differentials across multiple subcarriers within a carrier can be related to time of arrival

  - Option 3: Support a UE/TRP to optionally report an estimated integer ambiguity and/or search range of the integer ambiguity to LMF.

# On PRUs for calibration

First, we note some relevant RAN1 decisions from RAN1 #112 meeting [3]:

Conclusion Current RAN1 specifications do not support a mechanism to ensure simultaneous measurements/transmissions (e.g. in the same slot(s)) for multiple UEs, including a target UE and a PRU. Conclusion A PRU can report its location and associated uncertainty as is the case for other UEs. It is not necessary to always include the PRU location information with the PRU measurements in the same report. The PRU location information and measurements should be decoupled, where decoupled means that the PRU location information is determined independently of the reported measurements, even if the PRU location information and the PRU measurements would be included in the same report. Agreement RAN1 will continue discussions on what enhancements to LPP, NRPPa, and/or RAN signaling are necessary to support simultaneous measurements of the same DL-PRS for multiple UEs, including a target UE and a PRU; and to support simultaneous transmission of SRS for multiple UEs, including a target UE and a PRU. RAN1 will include the outcome in an LS to SA2 and RAN2, and cc RAN3. Note: Enhancements might or might not have RAN1 specification impact.

In the context of DL/UL CPP, the need to have simultaneous measurements on DL PRS at a target UE and PRU for DL CPP and “simultaneous” (implying sufficiently-close-in-time) transmissions of SRS/SRSp from a target UE and PRU are quite critical as was already observed during the SI phase.

Thus, in the context of CPP, we make the following observations leading to the identified objectives as below.

Observation 3

- Considering the agreed definition for UL RSCP as an absolute metric, for calibration of initial RX phase at a TRP, it would be essential to ensure transmission of SRS from a PRU sufficiently close-in-time to that from a target UE.

  - Objective 1: LMF should be able to direct a serving gNB to schedule such “sufficiently-close-in-time” transmission of SRS/SRSp from a target UE and PRU.

  - Objective 2: LMF should be able to direct NG-RAN nodes to perform measurements corresponding to these SRS/SRSp resources.

Observation 4

- For DL RSCP(D), initial phase calibration of the TX generators for a target (and reference) TRP(s) would require using measurements on the same DL PRS that are performed simultaneously by a UE with known coordinates (PRU) and a target UE.

  - Objective 3: LMF should be able to request a UE (target UE, PRU) to perform measurements on specific DL PRS resources or at least on DL PRS resources occurring within an indicated time window.

These functions are currently not supported in current specifications.

Towards the above purposes in context of CPP, and also, in the general context of calibration of timing, ARP location, phase related errors via network (NW)-based solutions using PRUs, it would be necessary to enhance NRPPa and LPP signalling to enable the above functionalities.

On Objective 1, we note that it may be reasonable to also consider allowing some flexibility to a serving gNB to decide the exact subframe/slots for scheduling SRS/SRSp in view of overall RAN resource management. Accordingly, instead of requesting particular reference subframe/slots for SRS/SRSp scheduling, an LMF may indicate to a gNB a reference time window and in response, a serving gNB may decide on specific subframe/slots to serve as reference slots for SRS/SRSp scheduling and report back with such information to the LMF.

Thus, we note the following possibilities to realize Objective 1:

- Option 1: LMF may optionally indicate to a serving gNB a subframe and slot in the subframe as a reference slot in which SRS/SRSp may be scheduled for transmission from a UE.

- Option 2: LMF may optionally indicate to a serving gNB a time window within which a reference slot for SRS/SRSp transmission from a UE should be scheduled.

  - a gNB, upon receiving an indication from LMF with a time window within which a reference slot for SRS/SRSp transmission from a UE should be scheduled, is expected to determine a suitable reference slot for SRS/SRSp transmission and indicate the selected reference slot to the LMF.

Details of how the reference subframe/slot may be indicated can be studied further.

For Objectives 2 and 3, LMF may provide reference time windows to an NG-RAN node (gNB/TRP) or UE (respectively) for performing the measurements. The reference time windows may be defined with respect to a reference time. For Objective 2, the reference time may correspond to the SFN initialization time. For Objective 3, the reference time determination may need additional consideration in RAN1. For instance, it could correspond to the time when the message is received, or with respect to a subframe/slot of a reference cell, etc.

Additionally, it may be considered further, if there may be scenarios in which the gNB/TRP or UE may respond back as part of a measurement report on whether the indicated reference resources for measurements were utilized or if there were differences (e.g., more/fewer/different resources were used).

Proposal 16

- To enable simultaneous (sufficiently-close-in-time) SRS/SRSp transmissions from a target UE and a PRU, the following objectives are pursued for potential NRPPa enhancements:

  - Objective 1: LMF should be able to direct a serving gNB to schedule such “sufficiently-close-in-time” transmission of SRS/SRSp from a target UE and PRU.

  - Objective 2: LMF should be able to direct NG-RAN nodes to perform measurements corresponding to these SRS/SRSp resources.

- RAN1 to discuss further on approaches to address these objectives for eventual recommendations to RAN2/SA2.

Proposal 17

- To enable simultaneous measurements on same DL PRS by a target UE and a PRU, the following objective is pursued for potential LPP enhancements:

  - Objective 3: LMF should be able to request a UE (target UE, PRU) to perform measurements on specific DL PRS resources or at least on DL PRS resources occurring within an indicated time window.

- RAN1 to discuss further on approaches to address the above objective for eventual recommendations to RAN2/SA2.

# Conclusions

In this contribution, we presented our views on support of DL and UL CPP in NR systems. We summarize the discussion and our views via the observations and proposals as follows:

Observation 1

- Although for some cases like UE-based positioning, IAR may be left up to implementation, such may not be feasible for all cases, e.g., for UE-/NG-RAN-assisted positioning. Thus, some additional information to facilitate IAR should be considered.

Observation 2

- Reporting of CP/CPD measurements along with legacy positioning methods can help with Integer Ambiguity Resolution (IAR).

Observation 3

- Considering the agreed definition for UL RSCP as an absolute metric, for calibration of initial RX phase at a TRP, it would be essential to ensure transmission of SRS from a PRU sufficiently close-in-time to that from a target UE.

  - Objective 1: LMF should be able to direct a serving gNB to schedule such “sufficiently-close-in-time” transmission of SRS/SRSp from a target UE and PRU.

  - Objective 2: LMF should be able to direct NG-RAN nodes to perform measurements corresponding to these SRS/SRSp resources.

Observation 4

- For DL RSCP(D), initial phase calibration of the TX generators for a target (and reference) TRP(s) would require using measurements on the same DL PRS that are performed simultaneously by a UE with known coordinates (PRU) and a target UE.

  - Objective 3: LMF should be able to request a UE (target UE, PRU) to perform measurements on specific DL PRS resources or at least on DL PRS resources occurring within an indicated time window.

Proposal 1

- Support DL RSCPD reporting as an independent positioning method.

  - FFS: Exact conditions and associated assistance information to resolve integer ambiguity.

Proposal 2

- Introduce DL RSCPD for DL CPP considering its resiliency to initial phase at UE Rx.

  - FFS: Reporting of RSCP corresponding to a target TRP – e.g., DL RSCPD may be sufficient if reporting of DL CP measurements is defined as an independent positioning method.

- LMF configures a target UE with a reference cell that is independent from serving cell.

Proposal 3

- For DL CPP, support reporting of DL RSCPD along with at least RSTD and, if DL RSCPD is defined as independent positioning method, then also along with UE Rx-Tx time difference measurements.

  - FFS: Other combinations, e.g., if DL RSCP is also supported.

Proposal 4

- For DL CPP,

  - The measurements are performed on DL PRS received in a subframe SF_j from target TP j and in a subframe SF_i from reference TP i that is closest in time to subframe SF_j.

  - Multiple DL PRS resources may be used to determine the carrier phase.

    - LMF may optionally request a target UE to perform measurements that are restricted to within a time window.

Proposal 5

- For DL CPP, as default, the “specific RF frequency” for the carrier phase corresponds to the center frequency for the DL PFL.

Proposal 6

- For DL CPP, reference point for DL RSCP measurement is the UE receiver antenna connector is assumed as the baseline.

  - FFS: Further enhancements, including PCO reporting.

Proposal 7

- DL RSCPD is defined only with respect to the first path detected in time.

Proposal 8

- Support UL RSCP reporting by a gNB as an independent positioning method.

  - FFS: Exact conditions and associated assistance information to resolve integer ambiguity.

Proposal 9

- LMF can request a gNB to report UL RSCP along with UL RTOA and/or gNB Rx-Tx time difference measurements.

Proposal 10

  - Multiple SRS/SRSp resources may be used to determine the UL carrier phase.

    - LMF may optionally request a gNB to perform measurements that are restricted to within a time window.

Proposal 11

- For UL RSCP, as default, the “specific RF frequency” for the carrier phase corresponds to one of (to be downselected):

  - the center frequency for the UL carrier, or

  - RF frequency corresponding to the center of the SRS/SRSp resource on which UL RSCP is measured.

Proposal 12

- For UL CPP, reference point for UL RSCP measurement is assumed as defined for UL RTOA or gNB Rx-Tx time difference as the baseline.

  - FFS: Further enhancements, including PCO reporting.

Proposal 13

- In consideration of multipath environments, adopt the following solutions:

  - Measurements: For both DL and UL CPP, limit reporting of carrier phase to that of the first path detected in time.

  - Assistance information: Rel-17 LOS/NLOS indication can be the baseline.

Proposal 14

- Introduce LOS/NLOS reporting as optional assistance information along with reporting of CP(D) measurements for DL/UL CPP as independent positioning method.

  - Reuse R17 LOS/NLOS reporting quantity/granularity as baseline.

Proposal 15

- For integer ambiguity resolution (IAR), introduce the following options in addition to the option of reporting RSCP/RSCPD with legacy positioning metrics,

  - Option 2: Introduce and report a new type of UE/TRP measurement based on carrier phase differentials across multiple subcarriers within a PFL/carrier.

    - NOTE: carrier phase differentials across multiple subcarriers within a carrier can be related to time of arrival

  - Option 3: Support a UE/TRP to optionally report an estimated integer ambiguity and/or search range of the integer ambiguity to LMF.

Proposal 16

- To enable simultaneous (sufficiently-close-in-time) SRS/SRSp transmissions from a target UE and a PRU, the following objectives are pursued for potential NRPPa enhancements:

  - Objective 1: LMF should be able to direct a serving gNB to schedule such “sufficiently-close-in-time” transmission of SRS/SRSp from a target UE and PRU.

  - Objective 2: LMF should be able to direct NG-RAN nodes to perform measurements corresponding to these SRS/SRSp resources.

- RAN1 to discuss further on approaches to address these objectives for eventual recommendations to RAN2/SA2.

Proposal 17

- To enable simultaneous measurements on same DL PRS by a target UE and a PRU, the following objective is pursued for potential LPP enhancements:

  - Objective 3: LMF should be able to request a UE (target UE, PRU) to perform measurements on specific DL PRS resources or at least on DL PRS resources occurring within an indicated time window.

- RAN1 to discuss further on approaches to address the above objective for eventual recommendations to RAN2/SA2.

# References

- RP-223549, “New WID on Expanded and Improved NR Positioning”, Intel Corporation, CATT, Ericsson, December 2022.

- TR 38.859, “Study on expanded and improved NR positioning (Release 18)”, December 2022.

- Chairman’s Notes, 3GPP RAN1 #112 meeting.