3GPP TSG RAN WG1 Meeting #112bis-e	R1-2302805

e-Meeting, April 17th – 26th, 2023

Source: 	Intel Corporation

Title:	On support of LPHAP

Agenda item:	9.5.3

Document for:	Discussion and Decision

# Introduction

At the RAN1 # 112 meeting, the following were agreed with regards to SRS configuration in inactive mode for LPHAP device [1]:

Agreement For SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state, support the following: - An SRS positioning validity area consists of cells configured in the same band and the same carrier, and the following parameters with respect to BWP information of SRS for positioning configuration are commonly applied across cells within the validity area: - BWP parameters - locationAndBandwidth - subcarrierSpacing - cyclicPrefix Agreement For SRS for positioning configuration in multiple cells for a UE in RRC_INACTIVE state, at least the following parameters in SRS for positioning configuration are commonly configured across cells within the validity area: - srs-PosConfig - SRS-PosResourceSet - srs-PosResourceSetId - srs-PosResourceIdList - resourceType - SRS-PosResource - srs-PosResourceId - transmissionComb - resourceMapping - freqDomainShift - freqHopping - groupOrSequenceHopping-r16 - resourceType - FFS: whether sequenceId is configured commonly across cells or per cell Agreement From RAN1 perspective, it is feasible to configure SRS positioning validity area-specific TA timer (e.g., with larger values) for a UE in RRC_INACTIVE state. Details can be up to RAN2. - For TA validation, use of area-specific RSRP change threshold is feasible - FFS: which RS is the reference RS for the RSRP change threshold Agreement For the determination of UL timing to transmit SRS for positioning by UEs in RRC_INACTIVE state within the SRS positioning validity area: - For the UL timing advance, further study the following options, including the DL reference timing for each option: - Option 1: UE maintains the TA obtained from the last serving cell within the validity area - Option 2: UE autonomously adjusts the TA - FFS: how the UE adjusts the TA, e.g. up to UE implementation, or based on the TA from the last serving cell and the DL time difference measurement of SSBs from the last serving cell and the new camping cell. - FFS: whether there is RAN1 specification impact - Option 3: UE maintains multiple TA values, e.g. UE obtains TA using RACH Agreement For the spatial relation of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state, further study the following options: - Option 1: Spatial relation information is absent in the configuration - FFS: different approaches for down selection at least include the following: - 1a: UE transmits SRS for positioning resources using different spatial domain transmission filter - 1b: UE transmits SRS for positioning resource(s) using a fixed spatial domain transmission filter - FFS criterion on UE determination of the fixed spatial domain transmission filter (e.g., up to UE implementation, based on a selected SSB of the camping cell, based on the configured path-loss RS such as SSB, etc.) - Option 2: Spatial relation information is provided in the configuration - FFS details on the configuration and corresponding UE behavior, including whether the information is configured for all or subset of cells - FFS signaling to configure the spatial relation information, e.g., via SRS activation message. - Note: UE power consumption needs to be taken into account - FFS validity criteria of spatial relation for the configured RS and UE behavior if it determines that the validity criteria of spatial relation for the configured RS is not met, if any, to avoid frequent RRC connection for SRS (re)configuration. Agreement For the power control of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state, further study the following options: - Option 1: Pathloss RS is absent in the configuration. - FFS criterion on UE determination of pathloss RS (e.g., up to UE implementation, UE selects an SSB as the pathloss RS based on DL measurements from multiple SSBs of cells within the validity area, etc.). - Option 2: Pathloss RS is provided in the configuration - FFS details on configuration (e.g., pathloss RS is configured as a cell-agnostic DL RS, pathloss RS is configured as a fixed SSB within the validity area, etc.), including whether the information is configured for all or a subset of cells within the validity area - FFS signaling to configure the pathloss RS, e.g., via SRS activation message. - Note: UE Power consumption needs to be taken into account - FFS: Whether p0 and alpha can be commonly configured across cells within the validity area. FFS validity criteria of pathloss RS and UE behavior if it determines that the validity criteria of pathloss RS is not met., if any, to avoid frequent RRC connection for SRS (re)configuration.

In this contribution, we discuss SRS configuration enhancements that can be suitable for LPHAP use case.

# Discussion on multi-cell SRS configuration

Due to mobility of the UE, cell reselection may occur and SRS configuration in the serving cell becomes invalid when validity criteria is not satisfied. In Rel-17, SRS validity criteria includes conditions related to spatial relationship, pathloss RS and TA. For UL positioning service, low power high accuracy positioning (LPHAP) device requires to transmit SRS periodically. According to existing legacy procedure, UE has to enter RRC connected state in new cell to obtain the new SRS configuration. Cf. Figure 1, which illustrates need for new SRS configuration as UE moves to new cell.

Figure 1: Cell reselection for UE requires new SRS configuration in legacy procedure

This could result in large increase in power consumption depending on the frequency of validity criteria failure and cell reselection events. Hence, one of the objective regarding support of LPHAP in Rel-18 is to consider a positioning validity area comprising multiple cells and allow the UE to keep using existing SRS configuration when UE moves to a new cell in RRC inactive state. To this end, UE can be provided a SRS configuration with common parameters that can be valid across multiple cells within the validity area. In our view, the serving cell shall provide the determined SRS configuration to neighbor cells and negotiate for resource reservation. Then the serving cell can indicate to UE which cells are included in the validity area. The UE may continue SRS transmission if the camping cell is within the validity area.

## SRS activation/configuration request

When cell reselection happens out of the positioning validity area, the UE should stop the SRS transmission as in legacy system. In order to maintain the continuity of positioning procedure, the UE should request to the network for SRS configuration update. To this end, we think mechanisms are needed to provide UE with new SRS configuration potentially staying in RRC inactive state in the new cell.

We think Msg-3 based RRC-resume-request can be used to trigger SRS configuration request in the new cell after cell reselection occurs out of the positioning validity area. Then new gNB could provide new SRS configuration/activation in a RRC release message. From power consumption perspective, it is beneficial for the new gNB to maintain the UE in RRC_INACTIVE after UE context retrieval if the RRC resume request is for SRS update only.

Proposal 1: UE could use Msg-3 based RRC-resume-request to trigger SRS configuration/activation request when cell reselection occurs outside of validity area.

- Details of the procedure is up to RAN2.

- Note: The UE can only use Msg-3 based RRCResumeRequest to trigger SRS configuration request if allowed by the network.

## Common SRS configuration parameters

In RAN1 # 112, quite a few SRS configuration parameters were agreed to be commonly configured across the cells within the validity area. However, there was no conclusion on whether sequenceID as part of SRS-PosResource can be commonly configured or not. There were proposals on considering the configuration of sequenceID on a per cell basis. The concern was on SRS capacity and whether assuming a common sequence ID for a UE in the area puts restriction on the size of the validity area comprising only a limited number of cells. On the contrary, having a sequenceID on a per cell basis implies gNB would have to blindly monitor multiple SRS configurations for a UE, something which was recommended to be avoided by RAN2 during SI. In our view, this issue does not critically require optimization since SRS resources may be orthogonalized in time and frequency as well and having a common sequenceID across multiple cells within validity area can work fine, subject to proper network implementation.

Proposal 2: SequenceID can be configured commonly across the cells in a validity area.

## Handling PHY issues of multi-cell SRS configuration

During Rel-18 SI, RAN2 has agreed on the study of SRS positioning validity area for LPHAP where the configured SRS is applicable across multiple cells as follows and raised some concern on potential issues in physical layer, such as interference, timing alignment, spatial relation, and which SRS parameters can be valid across multiple cells, etc. and sent an LS to RAN1.

Agreement: Proposal 3 (modified): RAN2 agree to study enhancements on SRS configuration (12/15). Further study the following candidate enhancements on SRS configuration, including the possible interference and changes of spatial relations problems. - a) Validity area mechanism; (12/13) - b) SRS update mechanism; (10/13) - c) Pre-configure multiple SRS, which could include broadcast transmission of configurations with UE-specific determination along with the network of a configuration; (9/13) FFS if item c would require network nodes to measure multiple SRS configurations for the same UE simultaneously. LS to RAN1 to ask them about interference issues with SRS configurations across multiple cells and about the validity of SRS parameters.

In this section, we discuss how these aspects can be handled at PHY.

### Interference handling

RAN1 responded that SRS positioning configuration for LPHAP across multiple cells is feasible from RAN1’s perspective after checking the potential issues of interference, timing alignment (depending on uplink synchronization conditions), spatial relation, and power control, with or without potential enhancements depending on deployment conditions. Next, we discuss whether any new procedure or behavior needed to handle the PHY issues when SRS configuration is expected to be valid across multiple cells.

Since the SRS configuration needs to be valid across multiple cells, some resource reservation and coordination across the cells within the positioning validity area may be needed so that interference can be avoided in the cells within the positioning validity area. Moreover, positioning SRS can be received and measured by multiple neighboring cells on the same frequency and it is expected that the interference issue can be handled by gNB implementation. We do not see any specification-based enhancement or special handling needed for this issue.

Proposal 3: Interference issues, if any, due to the configuration of SRS across multiple cells within a validity area can be handled by gNB implementation.

### UL timing advance

For the UL timing advance, following options are considered for further study:

- Option 1: UE maintains the TA obtained from the last serving cell within the validity area

- Option 2: UE autonomously adjusts the TA

- Option 3: UE maintains multiple TA values, e.g. UE obtains TA using RACH

For the scenarios where the geometric and channel situation may not change significantly, adjacent cells may have similar coverage with similar cell radius and small range, e.g., indoor warehouse scenario. UE may reuse the TA from the last serving cell where the UE obtained a TA value and can use it when UE reselects to the new cell, i.e., same TA within the list of cells as long as validity criteria is maintained. Note that for timing advance, in Rel-17 SRS transmissions in RRC_INACITVE state, the SRS transmission is based on a valid TA with respect to the DL synchronization to the serving cell to avoid UL interferences to the serving cell. RAN2 agreed that the TA timer mechanism and RSRP-based TA validation mechanism that was defined for CG-SDT are reused for SRS transmission in RRC_INACTIVE state. By reusing TA of  the last serving cell where UE obtained the TA value, the TA validity criterion (e.g., TimeAlignmentTimer, RSRP threshold) can also be reused, which has better robustness. Note that RAN1 has already concluded that it is feasible to configure SRS positioning validity area-specific TA timer (e.g., with larger values) and area-specific RSRP change threshold for a UE in RRC_INACTIVE state.

According to Option 2, UE would determine TA, such as based on RSTD measurements based on old and new cell. In this process, TA value can be potentially updated when cell reselection occurs. However, RSTD measurement accuracy may not be good, since it is based on narrow SSB BW. Also, adjusting TA dynamically based on distance to camping cell may cause increased and likely more unpredictable interference to the neighboring cells. Further, for timing based positioning methods, it would be desirable to minimize UE-autonomous changes to transmit timing. Regarding Option 3, UE needs to obtain new TA value (such as from Msg 2) every time cell reselection occurs. Note that one of the motivation of the objective in the WID was to avoid frequent reconfiguration and not use RACH procedure every time cell selection occurs, as it causes increased power consumption at the UE.  In conclusion, Option 2 may result in unpredictable UE behavior and anyway it is up to UE implementation. Option 3 could potentially result in increased power consumption than Option 1.

Observation 1: UE maintaining a single TA (e.g., Option 1) would potentially result in lower power consumption compared to Option 3 where TA is updated by RACH and more predictable UE behavior compared to using multiple TAs (such as in Option 2) within the validity area.

Observation 2: Depending on the deployment/size of validity area such as cells having small range and similar coverage, using TA from a previous cell where UE obtained the TA value is expected to work fine to manage UL interference.

Proposal 4: Adopt the following option for UL timing advance:  UE maintains TA from the last serving cell where UE obtained the TA value as long as the TA validity criteria is met.

Propose 5: Reuse Rel-17 TA validity criteria with the assumption that area-specific TA timer (e.g., with larger values), and area-specific RSRP change threshold can be applied.

### Spatial relation

During RAN1 # 112, the following two options regarding spatial relation of an SRS for positioning configuration in multiple cells for UEs in RRC_INACTIVE state are listed for further study:

  - Option 1: Spatial relation information is absent in the configuration

  - Option 2: Spatial relation information is provided in the configuration

From our perspective, both options can be supported. Depending on deployment conditions, one or the other can be adopted by the NW. For example, for FR 1, spatial relation can be configured and employing a wider beam for SRS transmission seems viable, such as considering limited coverage. Accordingly, the legacy procedure with existing validity criteria can be reused.  Certainly, if the UE is not able to accurately measure the corresponding RS for pathloss reference, SRS transmission can be suspended. Depending on the deployment scenario and size of the cells, and potential trajectory of the movement of the LPHAP device, frequent validity criteria failure may be avoided. Note that the spatial relation can be valid across multiple cells in FR1 for scenarios wherein analog beamforming is not applied. Hence, whether update of the spatialRelationInfoPos parameter is needed or not may be primarily limited to FR2. In this case, spatial relation information may not be common across the cells and it may be absent in the configuration.

If spatial relation information is absent in the configuration (i.e., Option 1), two approaches are considered:

  - Option 1a: UE transmits SRS for positioning resources using different spatial domain transmission filter

  - Option 1b: UE transmits SRS for positioning resource(s) using a fixed spatial domain transmission filter

According to Option 1a, UE may perform transmit side beam forming over the SRS resources where the number of transmissions can be up to the number of SSBs used in the cell. However, depending on the SRS periodicity and number of beams, this could result in increased power consumption.

On the other hand, in Option 1b, UE could assume correspondence to the best SSB beam in the new camped cell and perform transmission with repetitions with assumption of receiver side beam sweeping at the gNB over the SRS resources. This option also causes increased UE power consumption compared to Option 2. However, Option 1b may result in lower power consumption when compared against Option 1a if number of SRS resources is less than the number of SSBs supported in the camping cell. UE may transmit SRS over the number of configured SRS resources using fixed spatial domain filter in Option 1b. On the other hand, if only a subset of the beams are considered for sweeping, then power consumption of Option 1a can also be reduced.

Proposal 6: Support both options related to configuration of spatial relation information:

  - Option 1: Spatial relation information is absent in the configuration

  - Option 2: Spatial relation information is provided in the configuration and it is valid across the cells in the validity area.

Proposal 7: Further discuss power consumption aspects of the following options when spatial relation is absent in the configuration.

  - Option 1a: UE transmits SRS for positioning resources using different spatial domain transmission filter

  - Option 1b: UE transmits SRS for positioning resource(s) using a fixed spatial domain transmission filter

In our view, the validity criteria for spatial relation applies regardless of cell reselection, i.e., it is only related to UE mobility. Hence, as long as the configured parameters are valid, i.e., UE is able to accurately measure of the configured RS indices, a common UL positioning SRS across multiple cells can still work.

Proposal 8: If spatial relation information is configured, Rel-17 validity criteria of spatial relation for SRS transmission can be reused.

### Pathloss RS information

Similar to the Spatial relation information, two options are considered for pathloss RS as well:

  - Option 1: Pathloss RS is absent in the configuration.

  - Option 2: Pathloss RS is provided in the configuration

From our perspective, both options can be supported. Depending on deployment conditions, one or the other can be adopted by the NW.

If the pathloss RS is absent in the configuration, UE may select the best SSB as the pathloss RS based on DL measurements from multiple SSBs. UE can perform this operation every time cell reselection occurs. Alternatively, UE may keep using the detected SSB as pathloss RS from the last serving cell where UE obtained the SRS configuration. Although in the latter option, gNB may know the cell which is assumed by the UE for using an SSB as pathloss RS, RSRP measurement at the UE is still unknown at the gNB in inactive state. Both options can be considered.  In any case, legacy procedure can be reused, i.e., if pathloss estimate cannot be accurately measured, UE may stop SRS transmission. Hence, robustness of SRS transmission can be still based on Rel-17 validity criteria.

If pathloss RS information is provided, we think it can be configured as a fixed SSB within the validity area applying to all the cells.  If validity criteria fails for the configured SSB, a fallback procedure can be considered such as assume the detected SSB of the new camping cell to avoid using RACH/RA-SDT procedure to obtain the information and increase power consumption. Moreover, as mentioned above, geometric properties and range of the cells within the validity area are expected to be similar, and we think there is no critical need to configure P0 and alpha in a cell-specific manner.

Proposal 9: Support both options related to configuration of pathloss RS information:

  - Option 1: Pathloss RS is absent in the configuration.

  - Option 2: Pathloss RS is provided in the configuration as a fixed SSB and it is valid across the cells in the validity area.

Proposal 10: If pathloss RS is absent in the configuration, UE may select the best SSB as the pathloss RS based on DL measurements from multiple SSBs. Following options can be considered:

  - UE keeps using the detected SSB as pathloss RS from the last serving cell where UE obtained the SRS configuration as long as Rel-17 validity criteria is met.

  - UE updates pathloss RS reference every time UE camps on a new cell and may assume the detected SSB as the reference for pathloss RS. For this option, validity criteria does not apply.

Proposal 11:  Pathloss RS is configured and is valid across cells within the validity area if UE determines that the validity criteria of pathloss RS defined for UEs in RRC_INACTIVE state in Rel-17 is met.

- If not met, some fallback behavior can be assumed such as the detected SSB of new camping cell can be assumed as pathloss RS.

# Conclusions

In this contribution, we discussed several aspects related to SRS configuration in multiple cells and we have the following observations and proposals.

Observation 1: UE maintaining a single TA (e.g., Option 1) would potentially result in lower power consumption compared to Option 3 where TA is updated by RACH and more predictable UE behavior compared to using multiple TAs (such as in Option 2) within the validity area.

Observation 2: Depending on the deployment/size of validity area such as cells having small range and similar coverage, using TA from a previous cell where UE obtained the TA value is expected to work fine to manage UL interference.

Proposal 1: UE could use Msg-3 based RRC-resume-request to trigger SRS configuration/activation request when cell reselection occurs outside of validity area.

- Details of the procedure is up to RAN2.

- Note: the UE can only use Msg-3 based RRCResumeRequest to trigger SRS configuration request if allowed by the network.

Proposal 2: SequenceID can be configured commonly across the cells in a validity area.

Proposal 3: Interference issues, if any, due to the configuration of SRS across multiple cells within a validity area can be handled by gNB implementation.

Proposal 4: Adopt the following option for UL timing advance:  UE maintains TA from the last serving cell where UE obtained the TA value as long as the TA validity criteria is met.

Propose 5: Reuse Rel-17 TA validity criteria with the assumption that area-specific TA timer (e.g., with larger values) and area-specific RSRP change threshold can be applied.

Proposal 6: Support both options related to configuration of spatial relation information:

  - Option 1: Spatial relation information is absent in the configuration

  - Option 2: Spatial relation information is provided in the configuration, and it is valid across the cells in the validity area.

Proposal 7: Further discuss power consumption aspects of the following options when spatial relation is absent in the configuration.

  - Option 1a: UE transmits SRS for positioning resources using different spatial domain transmission filter

  - Option 1b: UE transmits SRS for positioning resource(s) using a fixed spatial domain transmission filter

Proposal 8: If spatial relation information is configured, Rel-17 validity criteria of spatial relation for SRS transmission can be reused.

Proposal 9: Support both options related to configuration of pathloss RS information:

  - Option 1: Pathloss RS is absent in the configuration.

  - Option 2: Pathloss RS is provided in the configuration as a fixed SSB and it is valid across the cells in the validity area.

Proposal 10: If pathloss RS is absent in the configuration, UE may select the best SSB as the pathloss RS based on DL measurements from multiple SSBs. Following options can be considered:

  - UE keeps using the detected SSB as pathloss RS from the last serving cell where UE obtained the SRS configuration as long as Rel-17 validity criteria is met.

  - UE updates pathloss RS reference every time UE camps on a new cell and may assume the detected SSB as the reference for pathloss RS. For this option, validity criteria does not apply.

Proposal 11:  Pathloss RS is configured and is valid across cells within the validity area if UE determines that the validity criteria of pathloss RS defined for UEs in RRC_INACTIVE state in Rel-17 is met.

- If not met, some fallback behavior can be assumed such as the detected SSB of new camping cell can be assumed as pathloss RS.

# References

- RAN1 # 112 Chairman Notes