3GPP TSG RAN WG1 #112b-e			      R1-2302812

e-Meeting, April 17th – 26th, 2023

Source: 	Intel Corporation

Title: 	On coverage enhancement for NR NTN

Agenda item:	9.9.1

Document for: 	Discussion and Decision

# Introduction

The work item description (WID) for NR NTN enhancements was revised at the RAN#98-e meeting [1]. The revised WID include the following objectives and assumptions for coverage enhancements for NR NTN.

The following reference scenario is considered for the definition of uplink coverage enhancements for NTN: parameter set-1 for LEO-1200 satellite operating at Line of Sight (LOS) and commercial smartphones with -5.5 dBi antenna gain and 3 dB polarisation loss (per antenna port). Note: It is understood that the enhancements defined for LEO can also apply to GEO and MEO scenarios as appropriate. No additional work is expected for MEO/GEO. The targeted services are VoIP using AMR 4.75 kbps and data transmission services with Low data rate of 3 kbps. The detailed objectives are for NTN: - To specify PUCCH enhancements for Msg4 HARQ-ACK (e.g. repetition) [RAN1, RAN4] - To specify if necessary, enhancements to the Rel-17 procedures for DMRS bundling for PUSCH taking into account NTN-specifics (e.g. time-frequency pre-compensation) [RAN1]

In this contribution, we express our views on coverage enhancements for NR NTN including PUCCH enhancements for Msg4 HARQ-ACK and enhancements for PUSCH DMRS bundling.

Discussion

## PUCCH enhancements for Msg4 HARQ-ACK

As it is stated in the WID [1], coverage enhancements for PUCCH are considered only for transmission of HARQ-ACK in response to Msg4 PDSCH. In the specification this case can be referred as PUCCH resource indicated by a DCI format 1_0 with CRC scrambled by TC-RNTI. PUCCH transmission when dedicated PUCCH resource configuration is not provided may correspond to HARQ-ACK for a PDSCH transmission which does not carry Msg4. Considering the above, separate discussion is needed to decide if enhancements are required for transmission when dedicated PUCCH resource configuration is not provided. Thus, we propose to discuss coverage enhancements for PUCCH resource indicated by a DCI format 1_0 with CRC scrambled by TC-RNTI first. The need for PUCCH coverage enhancements when dedicated PUCCH resource configuration is not provided can be discussed further, if needed.

Proposal 1:

- RAN1 to discuss PUCCH for Msg4 HARQ-ACK which corresponds to PUCCH resource indicated by a DCI format 1_0 with CRC scrambled by TC-RNTI

  - The need of PUCCH enhancements for other cases shall be discussed separately

Considering that the PUCCH repetition for Msg4 HARQ-ACK should cover a broad range of NTN deployments with different satellite orbits and given that signal received from different UEs may have different SNR values, a mechanism for cell-specific and/or UE-specific configuration/indication of the corresponding PUCCH repetition factor is required. At the RAN1#111 meeting the following working assumption was agreed on the procedure and signaling for repetition factor determination.

Working assumption For PUCCH repetition for Msg4 HARQ-ACK, - One or more repetition factors may be configured via SIB - If only one repetition factor is configured via SIB and if the value is one of {[1], 2, 4, 8}, UE capable of PUCCH repetition for Msg4 HARQ-ACK can perform repetition with the repetition factor - FFS: whether UE requests repetition or indicates repetition capability - If multiple factors from {1, 2, 4, 8} are configured via SIB, PUCCH repetition for Msg4 HARQ-ACK may be dynamically determined and indicated by gNB - FFS: whether UE requests repetition or indicates repetition capability - FFS: whether repetition factor is indicated by UE - FFS: UE behavior when repetition factor is not configured via SIB - FFS: whether one or more UE capabilities are needed for the above is for further discussion

As it can be seen from the above working assumption, both cell-specific configuration and UE-specific indication are supported for repetition factor determination for PUCCH HARQ-ACK repetition in response to Msg4 PDSCH.

For the cell-specific configuration of the repetition factor, there is no need to support configuration of repetition factor equal to 1 (no repetition) since PUCCH HARQ-ACK repetition in response to Msg4 PDSCH feature can be enabled or disabled by the gNB in a simpler way. The PUCCH repetition can be considered as disabled if the corresponding information element is not indicated (absent) in the SI. Thus, we propose to support 3 candidate values {2, 4, 8} for cell-specific configuration of repetition factor in the SI.

For the UE-specific indication of the repetition factor, it is agreed as working assumption that multiple repetition factors from {1, 2, 4, 8} are configured via SIB. At the same time, there is no benefit for configuring a subset of values for repetition factors (i.e. 3 values or 2 values instead of 4 values). Thus, we propose to support configuration of 4 values for repetition factor {1, 2, 4, 8} in the SI in case of UE-specific indication of the repetition factor.

Proposal 2:

- For PUCCH repetition for Msg4 HARQ-ACK, one or multiple repetition factors may be configured via SIB

  - For configuration of one repetition factor (cell-specific configuration), the candidate values are {2, 4, 8}

  - For configuration of multiple repetition factors (UE -specific indication), four repetition factors {1, 2, 4, 8} are configured as candidate values for dynamic indication

  - gNB can enable or disable the repetition feature via SIB, the repetition feature is considered disabled if the information element which configures candidate values, or one value of the repetition factor is not indicated (absent) in the SI

At the last RAN1 meeting the following working assumptions were made on the repetition request or capability report indicated by the UE and the corresponding RSRP threshold to determine whether UE should indicate the repetition request or not when the number of repetitions is configured by SIB.

Working assumption For PUCCH repetition for Msg4 HARQ-ACK, discuss the following options as container of the [repetition request or capability report] indicated by UE. - Option A: PRACH preamble and/or occasion - FFS: whether PRACH resource partitioning is needed for indication of [repetition request or capability report] - FFS: whether or not indication of repetition factor is assumed - Note: the relation with R18 NR coverage enhancements for PRACH may need to be considered in future meetings - Option B: Higher layer signaling in Msg3 PUSCH - FFS: which signaling is used - Note: if higher layer signaling is preferred in RAN1, the feasibility will be asked to RAN2. - Option C: Physical layer signaling in Msg3 PUSCH - FFS: which signaling is used, e.g. DMRS ports Working assumption For PUCCH repetition for Msg4 HARQ-ACK, - A RSRP threshold can be configured via SIB at least when the number of repetitions is configured by SIB. - If the RSRP threshold is configured and the configured RSRP threshold is smaller than X, - UE capable of PUCCH repetition for Msg4 HARQ-ACK transmits repetition request if measured RSRP is lower than a RSRP threshold. - If the RSRP threshold is not configured, or if the configured RSRP threshold is X, - UE capable of PUCCH repetition for Msg4 HARQ-ACK reports the capability of PUCCH repetition for Msg4 HARQ-ACK - FFS: value of X (the maximum configurable value of the RSRP threshold) - Down-select one from the following alternatives for the RSRP threshold. - Alt A: The same RSRP threshold as R17 Msg3 repetition (i.e., rsrp-ThresholdMsg3-r17) is used. - Alt B: New RSRP threshold is introduced. - Note: UE incapable of PUCCH repetition for Msg4 HARQ-ACK transmits neither repetition request nor capability report

Option A and Option C for the indication of repetition request or capability report from the UE correspond to additional constraints and changes at the physical layer. For Option A, gNB should allocate additional PRACH resources either with separate PRACH preambles in case of shared ROs or separate ROs, which may not be desirable in term of spectrum efficiency. For Option C, gNB needs to perform blind decoding of Msg3 PUSCH according to used DMRS port or other physical layer parameters (e.g., scrambling), which would increase implementation complexity at the receiver. The only drawback of Option B is increased payload size for Msg3 PUSCH which should not be significant. Thus, we propose to consider Option B for repetition request or capability report indication. The feasibility of this solution can be assessed by RAN2 as it is stated in the working assumption above.

The mechanism to configure RSRP threshold to determine whether UE should indicate the repetition request was defined in Rel-17 Coverage Enhancements WI for request of Msg3 PUSCH repetition. If PUCCH repetition for Msg4 HARQ-ACK is configured, the information element specified for indication of RSRP threshold for Msg4 PUSCH (rsrp-ThresholdMsg3-r17) can be used for PUCCH repetition. However, due to different SNR requirements for PUSCH and PUCCH reception at the gNB, we prefer to support new RSRP threshold indicated in SIB specifically for PUCCH HARQ-ACK repetition in response to Msg4 PDSCH.

Proposal 3:

- Support higher layer signaling in Msg3 PUSCH for indication of UE capability/UE request for PUCCH HARQ-ACK repetition in response to Msg4 PDSCH (Option B)

- New RSRP threshold is introduced for UE request for PUCCH repetition (i.e., the RSRP value is separately configured from the RSRP value for Msg 3 PUSCH repetition)

The following agreement was made at the last RAN1 meeting on the dynamic indication of repetition factor for PUCCH HARQ-ACK in response to Msg4 PDSCH.

Agreement For PUCCH repetition for Msg4 HARQ-ACK, discuss the following alternatives for dynamic indication of repetition factor from gNB. - Alt 1: Field in DCI scheduling the Msg4 PDSCH - Alt 1-1: One or two bits of the existing field - Alt 1-1a: MCS field - Alt 1-1b: PUCCH resource indicator field (e.g., with repetition factor configuration per PUCCH resource) - Alt 1-1c: HARQ process number filed - Alt 1-1d: DAI field - Alt 1-1e: PDSCH-to-HARQ_feedback timing indicator field - Alt 1-2: New field with one or two bits - Alt 2: Field in DCI scheduling Msg3 PUSCH - PUCCH repetition factor is indicated jointly with Msg3 repetition factor by using a pre-defined/configured relationship between PUCCH repetition factor and Msg3 repetition factor - Note: it is assumed that there is impact on DCI design - Alt 3: CRC scrambling of DCI scheduling the Msg4 PDSCH - One or two CRC bits other than bits scrambled by TC-RNTI is used for the dynamic indication, etc. - Alt 4: Implicit mapping between Msg4 HARQ ACK repetition factor and indication of Msg3 PUSCH repetition with no re-interpreted field / new field (i.e. no change to DCI design)

For Msg3 PUSCH initial transmission and retransmission, the number of repetitions is indicated by repurposing MCS in the RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI, respectively. Similar solution can be used for PUCCH in response to Msg4 PDSCH. For the UE-specific indication of the repetition factor for PUCCH in response to Msg4 PDSCH, the number of repetitions can be indicated via DCI format 1_0 with CRC scrambled by TC-RNTI. Since the repetition factor indication requires only 2 bits, repurposing of DCI bits can be considered. Given that the MCS field is not related to PUCCH and determines the MCS for PDSCH transmission, full range of MCS may be needed due to different DL coverage level comparing to UL coverage. Thus, DAI field which has no purpose according to the current specification (it is reserved) can be used for indication of the repetition factor for PUCCH in the DCI.

Proposal 4:

- Support dynamic indication via DCI for repetition factor of PUCCH using DAI field (reserved) in DCI scheduling the Msg4 PDSCH

The following conclusion was agreed at the RAN#110b-e meeting on the repetition slot counting for the PUCCH in response to Msg4 HARQ-ACK.

Conclusion For PUCCH repetition for Msg4 HARQ-ACK, - The existing mechanism on repetition slot counting (as in section 9.2.6 of TS 38.213) can be applied. - FFS: whether specification update to apply the existing mechanism to PUCCH repetition for Msg4 HARQ-ACK is needed.

Whether to support specification update to apply the existing mechanism to PUCCH repetition for Msg4 HARQ-ACK is for further study according to the above conclusion. As section 9.2.6 of TS 38.213 is for PUCCH repetition in RRC connected mode, both tdd-UL-DL-ConfigurationDedicated and tdd-UL-DL-ConfigurationCommon are used to determine the available slot in that case. However, for PUCCH in response to Msg4, only tdd-UL-DL-ConfigurationCommon can be used to determine the available slot. Thus, we propose to consider specification update to support available slot counting for PUCCH repetition for Msg4 HARQ-ACK.

Observation 1:

- Section 9.2.6 of TS 38.213 is for PUCCH repetition in RRC connected mode, and both tdd-UL-DL-ConfigurationDedicated and tdd-UL-DL-ConfigurationCommon are used to determine the available slot in that case

Proposal 5:

- Specification update needs to be considered to support available slot counting based on existing principles (as in section 9.2.6 of TS 38.213) for the repetition of PUCCH in response to Msg4 HARQ-ACK

One of the features which may further improve the coverage performance for PUCCH repetition are DMRS bundling and inter-slot frequency hopping. DMRS bundling is specified for PUSCH as an optional UE feature with the corresponding UE capability signalling. Since PUCCH repetition for Msg4 HARQ-ACK corresponds to initial access, this feature may not be supported for all the UEs and additional UE capability report before Msg4 transmission is not preferred. Thus, DMRS bundling for PUCCH repetition should not be supported for HARQ-ACK in response to Msg4 PDSCH.

For inter-slot frequency hopping, in our understanding it may not require additional specification effort as it is already supported for PUCCH repetition with dedicated configuration. Thus, inter-slot frequency hopping can be considered to further improve the coverage performance for PUCCH for Msg4 HARQ-ACK.

Proposal 6:

- Do not support DMRS bundling for PUCCH repetition with HARQ-ACK in response to Msg4 PDSCH

- Inter-slot frequency hopping is supported for PUCCH repetition with HARQ-ACK in response to Msg4 PDSCH

Assumption applied for spatial domain filter at the UE for PUCCH HARQ-ACK in response to Msg4 PDSCH is specified in section 9.2.1 of TS38.213. According to the specification, the UE transmits the PUCCH using the same spatial domain transmission filter as for a PUSCH transmission scheduled by a RAR UL grant. In order to allow coherent combining at gNB receiver, same Tx beam should be applied for all the repetitions of PUCCH transmission carrying HARQ-ACK in response to Msg4 PDSCH. Also, there is no need to change the mechanism of PUCCH resource determination for HARQ-ACK in response to Msg4. Thus, we propose to support same Tx beam and same PUCCH resource (determined by PRI in the DCI and starting CCE) for all PUCCH repetitions.

Proposal 7:

- Same Tx beam and same PUCCH resource are applied for all the repetitions of PUCCH transmission with HARQ-ACK in response to Msg4 PDSCH

## DMRS bundling for PUSCH

The following agreement and conclusions were made at the RAN1#111 meeting on PUSCH DMRS bundling support for NR NTN.

Agreement For NTN-specific PUSCH DMRS bundling, - Discuss further the need of enhancement in consideration of at least the following: - Phase difference due to timing drift and/or doppler shift. - e.g., whether/how long a UE can meet phase continuity requirement specified as Table 6.4.2.5-1 in 38.101-1 in consideration of frequency error within ± 0.1 PPM specified in section 6.4.1 of 38.101-5 and timing error specified in Table 7.1C.2-1 of 38.133, whether RAN1 should introduce enhancement to meet the requirement and/or recommend RAN4 to update the requirement or UE should pre-compensate phase difference by UE implementation, etc. - An event which causes power consistency and phase continuity not to be maintained. - e.g., whether the new event is necessary to determine actual TDW(s) from each nominal TDW or the existing specification can work without any specification change or whether such event may not occur depending on implementations, etc. - Note: baseline performance for legacy UEs can include antenna switching Conclusion For the study of NTN-specific PUSCH DMRS bundling, RAN1’s understanding is that Phase variation due to constant frequency error within ± 0.1 PPM specified in section 6.4.1 of 38.101-1 does not have impact on the phase continuity requirement for two adjacent slots specified as Table 6.4.2.5-1 in 38.101-1, according to annex F.9 and F.4 of 38.101-1. Conclusion RAN1 concluded that PUSCH DMRS bundling with sufficient TDW size should be applicable in NTN to meet the performance requirement for VoIP - FFS: How to determine TDW size, including UE capability. - Note: The above does not mean the performance requirements will be satisfied with DMRS bundling

For the duration of actual TDW, a UE should maintain phase continuity and power consistency to allow joint channel estimation across multiple slots at the receiver. For the NTN, the capability to maintain the phase continuity and power consistency also depends on the UE implementation for autonomous TA adjustment and frequency adjustment. Thus, the maximum TDW duration supported by the UE should consider the impact of UE autonomous TA adjustment and frequency adjustment. This aspect should be reflected in the UE capability report for the maximum TDW duration.

Proposal 8:

- UE capability related to maximum TDW duration should consider impact of UE autonomous adjustment of TA and frequency for PUSCH transmission

For NB-IoT/eMTC NTN segment size for autonomous UE pre-compensation adjustment is configured by the network. At the RAN1#110 meeting, it was agreed to support UE capability on the gaps for segmented UL transmission for both eMTC and NB-IoT NTN. The gaps may be required to allow low-complexity implementation for adjustment of time/frequency shift for UL transmission at the UE. Considering that NR devices allow higher UE complexity compared to eMTC/NB-IoT devices, there is no need to support additional gaps to adjust the UE pre-compensation values. Moreover, support of symbol-level or slot-level gaps may reduce coverage performance of PUSCH transmission if the limit of 20 ms transmission is considered which is not aligned with the Rel-18 NR NTN work item objectives.

Proposal 9:

- Additional gaps to adjust the UE pre-compensation values are not supported for NR NTN

There are some events which have an impact on actual TDW duration for PUSCH DMRS bundling. Different events are supported including PUSCH dropping/cancellation based on collision rules, DL slot or DL reception/monitoring based on semi-static DL/UL configuration for unpaired spectrum, gap between two PUSCH/PUCCH transmissions exceeding 13 symbols (normal CP) or 11 symbols for (extended CP), UL beam switching or different sets of power control parameters for multi-TRP operation, UL TA adjustment, frequency hopping, etc. There are some candidates for new events for PUSCH DMRS bundling in NTN. For example, new common TA and satellite ephemeris application and/or update of the epoch time can be considered as new events. Thus, we propose to study whether to introduce new events for PUSCH DMRS bundling in NTN.

Proposal 10:

- RAN1 to study if new event is needed to consider new common TA and satellite ephemeris application and/or update of the epoch time

Conclusion

In this contribution, we expressed our views on coverage enhancements for NR NTN including PUCCH enhancements for Msg4 HARQ-ACK and enhancements for DMRS bundling for PUSCH. The following proposals and observations were made.

Proposal 1:

- RAN1 to discuss PUCCH for Msg4 HARQ-ACK which corresponds to PUCCH resource indicated by a DCI format 1_0 with CRC scrambled by TC-RNTI

  - The need of PUCCH enhancements for other cases shall be discussed separately

Proposal 2:

- For PUCCH repetition for Msg4 HARQ-ACK, one or multiple repetition factors may be configured via SIB

  - For configuration of one repetition factor (cell-specific configuration), the candidate values are {2, 4, 8}

  - For configuration of multiple repetition factors (UE -specific indication), four repetition factors {1, 2, 4, 8} are configured as candidate values for dynamic indication

  - gNB can enable or disable the repetition feature via SIB, the repetition feature is considered disabled if the information element which configures candidate values, or one value of the repetition factor is not indicated (absent) in the SI

Proposal 3:

- Support higher layer signaling in Msg3 PUSCH for indication of UE capability/UE request for PUCCH HARQ-ACK repetition in response to Msg4 PDSCH (Option B)

- New RSRP threshold is introduced for UE request for PUCCH repetition (i.e., the RSRP value is separately configured from the RSRP value for Msg 3 PUSCH repetition)

Proposal 4:

- Support dynamic indication via DCI for repetition factor of PUCCH using DAI field (reserved) in DCI scheduling the Msg4 PDSCH

Observation 1:

- Section 9.2.6 of TS 38.213 is for PUCCH repetition in RRC connected mode, and both tdd-UL-DL-ConfigurationDedicated and tdd-UL-DL-ConfigurationCommon are used to determine the available slot in that case

Proposal 5:

- Specification update needs to be considered to support available slot counting based on existing principles (as in section 9.2.6 of TS 38.213) for the repetition of PUCCH in response to Msg4 HARQ-ACK

Proposal 6:

- Do not support DMRS bundling for PUCCH repetition with HARQ-ACK in response to Msg4 PDSCH

- Inter-slot frequency hopping is supported for PUCCH repetition with HARQ-ACK in response to Msg4 PDSCH

Proposal 7:

- Same Tx beam and same PUCCH resource are applied for all the repetitions of PUCCH transmission with HARQ-ACK in response to Msg4 PDSCH

Proposal 8:

- UE capability related to maximum TDW duration should consider impact of UE autonomous adjustment of TA and frequency for PUSCH transmission

Proposal 9:

- Additional gaps to adjust the UE pre-compensation values are not supported for NR NTN

Proposal 10:

- RAN1 to study if new event is needed to consider new common TA and satellite ephemeris application and/or update of the epoch time

References

- RP-223534 Revised WID: NR NTN (Non-Terrestrial Networks) enhancements, Thales