3GPP TSG RAN WG1 #112bis-e  													                     R1-2302818

e-Meeting, April 17th – April 26th, 2023

Source: 	Intel Corporation

Title:	Discussions on PRACH coverage enhancement

Agenda item:	9.12.1

Document for:	Discussion and Decision

# Introduction

At the RAN1#112 meeting, the following conclusions, working assumptions and agreements were made regarding PRACH coverage enhancement [1]:

Agreement

For multiple PRACH transmissions with same Tx beam, gNB can configure one or multiple values for the number of multiple PRACH transmissions.

- If multiple values are configured, PRACH resources differentiation between multiple PRACH transmissions with different number of multiple PRACH transmissions is supported.

- FFS: details

Working Assumption

For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, at least support that multiple PRACH are transmitted on separate ROs.

- Note: Separate RO means that the RO is separated with single PRACH transmission.

- FFS: whether Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) can be reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning.

Working Assumption

For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, support that multiple PRACH are transmitted with separate preamble on shared ROs.

- Note: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission.

- FFS: whether Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) can be reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning.

Conclusion

For multiple PRACH transmissions within one RACH attempt, they are only transmitted over ROs associated with the same SSB/CSI-RS.

Note: This applies for multiple PRACH transmissions with same Tx beam, and also applies for multiple PRACH transmissions with different Tx beam (if supported).

Agreement

For multiple PRACH transmissions with same Tx beam in one RACH attempt, transmission power ramping is not applied within one RACH attempt.

Agreement

For multiple PRACH transmissions with same Tx beam, only one RAR window is supported for RAR monitoring for one RACH attempt.

  - FFS: the start position of the RAR window.

  - FFS: RA-RNTI.

Agreement

For multiple PRACH transmissions with same Tx beam, "RO group" is assumed for multiple PRACH transmissions with separate preamble on shared ROs and/or multiple PRACH transmissions on separate ROs, and one RO group consists of valid RO(s) for a specific number of multiple PRACH transmissions.

Note 1: All ROs in one RO group is associated with the same SSB(s).

Note 2: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission.

Note 3: whether/how to define “RO group” in specification will be discussed separately

Note 4: Valid RO(s) refers to what is defined in existing specification

FFS: whether and how to address collision between valid ROs for multiple PRACH transmissions and other existing ROs for legacy single PRACH transmission or other features, e.g., 2-step RACH.

FFS: the time span of RO group.

FFS: whether and how ROs can be shared between different RO groups for different number of multiple PRACH transmissions.

FFS: other details

Agreement

Support {2, 4, 8} for the number of multiple PRACH transmissions with same Tx beams.

Note: It is summarized by FL that for the same number of PRACH transmissions per source,

- 1 source [Ericsson] shows that: Multiple PRACH transmitted by beam sweeping, where a UE has no prior knowledge of channel and sweeps Tx beams across 360 degrees horizontally and 180 degrees vertically, outperforms multiple PRACH transmissions with the same Tx wide beam (omni direction) by at least 1 dB, provided gNB configures only one SSB and receives PRACH with a wide beam.

- 3 sources [ZTE, Nokia, vivo] show that: A gain from about 1~3 dB of beam sweeping is observed if a UE is able to direct at least one of its Tx beams in the right direction or to narrow down the azimuth and/or zenith range of 360 degrees and/or 180 degrees for beam sweeping compared with multiple PRACH transmissions with the same Tx wide beam.

- 1 source [Huawei] shows that: compared to the same wide beam for multiple PRACH transmission, if different Tx beams are finer beams, then 3.9~5 dB gains are observed assuming that only one PRACH occasion with the best detected SINR is selected at the gNB reception, where the beam gain of fine beam is 4 times that of wide beam.

- 1 source [vivo] shows that: The performance of PRACH repetition with beam sweeping among beams far apart is 3 dB worse than PRACH repetition with single best beam

- 1 source [vivo] shows that: The performance of PRACH repetition with beam sweeping among beams in the directions close to the best Tx beam is 1dB worse than PRACH repetition with single best beam.

- 1 source [vivo] shows that: PRACH repetition via random beam directions performs 1 dB worse than PRACH repetition with omni beam.

In the contribution, we present our views on PRACH coverage enhancement. Our views on power domain enhancement and dynamic switching between DFT-S-OFDM and CP-OFDM waveform are described in our companion contributions [2] and [3], respectively.

# Multiple PRACH transmissions with same beams

## Use cases for multiple PRACH transmissions

During Rel-17 NR coverage enhancement study item phase, PRACH was identified as one of the bottleneck channels that need coverage enhancement [4]. Given that PRACH transmission is the very first step during RACH procedure, PRACH performance is critical for the system operation, especially when considering initial access and beam failure recovery. In order to improve the PRACH coverage performance during 4-step RACH procedure, one straightforward approach is to employ repetitions with the same Tx beams for the transmission of PRACH.

Note that the PRACH coverage enhancement applies to short PRACH formats and can also apply to other formats when applicable. Considering that only one PRACH sequence is included for PRACH format 0, multiple PRACH transmissions may also apply for the transmission of PRACH format 0 to improve the coverage for FR1. Hence, in our view, multiple PRACH transmissions can be applied for all short PRACH formats and PRACH format 0.

Further, it would be more reasonable to support multiple PRACH transmissions for both CFRA and CBRA. For CFRA, dedicated PRACH preamble can be configured for multiple PRACH transmissions. This can help in improving the PRACH performance for UEs with poor channel condition, which may be critical for certain scenarios, e.g., beam failure recovery.

Proposal 1

- Multiple PRACH transmissions are applied for all short PRACH formats and PRACH format 0.

- Multiple PRACH transmissions are applied for both CFRA and CBRA.

## Differentiation of multiple PRACH transmissions

At the RAN1#112 meeting, the following working assumptions were made [1]:

- For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, at least support that multiple PRACH are transmitted on separate ROs.

- For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, support that multiple PRACH are transmitted with separate preamble on shared ROs.

Similar to the request of Msg3 repetition as defined in Rel-17 coverage enhancement, either separate ROs or shared ROs with separate PRACH preambles can be supported to differentiate between single PRACH transmission and multiple PRACH transmissions. Depending on the deployment scenario or the number of cell edge UEs in the network, gNB may flexibly select one of these two options to differentiate single and multiple PRACH transmissions. For instance, when the number of UEs with poor channel conditions is limited, gNB may only configure shared ROs with separate PRACH preambles for the differentiation of single and multiple PRACH transmissions so as to reduce the RO overhead and thereby improve the spectrum efficiency. Hence, in our view, the working assumption as mentioned above should be confirmed to differentiate the multiple PRACH transmissions with single PRACH transmission.

Proposal 2

- RAN1 to confirm the following working assumptions

  - For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, at least support that multiple PRACH are transmitted on separate ROs.

  - For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, support that multiple PRACH are transmitted with separate preamble on shared ROs.

Further, at the RAN1#112 meeting, it was agreed that gNB can configure one or multiple values for the number of multiple PRACH transmissions for multiple PRACH transmissions with same Tx beam. Further, if multiple values are configured, PRACH resource differentiation between multiple PRACH transmissions with different number of multiple PRACH transmissions is supported [1]. The main motivation of introducing more than one values for the number of multiple PRACH transmissions is to allow flexible system operation and efficient PRACH resource management.

In case multiple number of multiple PRACH transmissions are configured, similar to differentiation of single PRACH transmission and multiple PRACH transmissions, PRACH resource partitioning can be employed to differentiate different number of multiple PRACH transmissions. Based on PRACH resource partitioning, each PRACH resource may correspond to one value of multiple PRACH transmission. In this regard, after successful detection of PRACH preambles in the configured resources, gNB may combine multiple PRACH detection statistics to improve the performance. Note that in order to avoid fragmented PRACH resources and minimize the impact on the legacy system, it is preferable to limit the set of values for multiple PRACH transmissions or only allow separate preamble on shared PRACH resources to differentiate different number of multiple PRACH transmissions.

Figure 1 illustrates PRACH resource partitioning when shared PRACH occasions are configured for single and multiple PRACH transmissions. In the figure, different PRACH preambles associated with the same SSB index can be allocated for multiple PRACH transmissions with number of PRACH transmissions.

Figure 1. Preambles allocated for PRACH with different number of PRACH transmissions

When multiple number of multiple PRACH transmissions are configured, after a certain number of PRACH retransmissions for multiple PRACH transmissions on a given repetition level, and if UE still does not successfully receive RAR, UE may increase the number of multiple PRACH transmissions so as to improve the success rate for RACH procedure. The maximum number of PRACH retransmission for multiple PRACH transmissions may be configured by higher layers for proper control of system operation.

Proposal 3

- To differentiate multiple PRACH transmission with different number of multiple PRACH transmissions, only shared PRACH occasions with separate preamble are supported.

- Repetition level ramping is supported when more than one number of multiple PRACH transmissions is configured.

At the RAN1#111 meeting, it was agreed that SSB-RSRP threshold(s) are used to determine the number of PRACH transmissions at least for the first RACH attempt [5]. In this case, when the measured SSB-RSRP is lower than an RSPR threshold, UE may trigger multiple PRACH transmissions with same Tx beam. Further, if multiple number of multiple PRACH transmissions are configured for multiple PRACH transmissions, multiple thresholds may need to be defined accordingly for proper operation.

For UEs that support both multiple PRACH transmissions and Msg3 PUSCH repetitions, a common SSB-RSRP threshold may be applied for multiple PRACH transmissions and request of Msg3 PUSCH repetitions. In this case, after successfully detecting the multiple PRACH transmission, gNB may have good knowledge of channel condition of UE and can inform UE to transmit Msg3 PUSCH with appropriate number of repetitions.

Proposal 4

- Multiple SSB-RSRP thresholds are used to determine the number of multiple PRACH transmissions if multiple values are configured.

- A common SSB-RSRP threshold may be applied for multiple PRACH transmissions and request of Msg3 PUSCH repetitions

## Frequency hopping for multiple PRACH transmissions

At the RAN1#110b-e meeting, it was agreed that at least ROs located at different time instances can be utilized for the multiple PRACH transmissions with same Tx beam [6]. It is FFS whether the starting RB of ROs can be different at different time instances for multiple PRACH transmissions and multiple PRACH transmissions can be located in the same time instance, e.g., for UEs with multiple Tx chains.

In our view, for multiple PRACH transmissions with same Tx beam, frequency hopping can be employed in conjunction with multiple PRACH transmissions in order to exploit the benefit of frequency diversity and hence further improve the detection performance. Depending on the number of time domain PRACH occasions in a PRACH slot, frequency hopping within or across the PRACH slot may be employed for multiple PRACH transmission.

Figure 2 illustrates PRACH detection performance with repetitions and frequency hopping. In the simulations, 0.1% false alarm probability was assumed for PRACH format 0 and 700 MHz carrier frequency. The detailed simulation assumptions are provided in Table 2 in the Appendix. From the figure, it can be observed that in case of 2 and 4 repetitions, an additional 2.5dB performance gain can be achieved when frequency hopping is employed for PRACH transmission.

Figure 2. PRACH detection performance with repetitions and frequency hopping

Observation 1

- For 2 and 4 PRACH repetitions with frequency hopping, ~2.5dB performance gain can be achieved compared to PRACH repetitions without frequency hopping.

Further, it was discussed at the RAN1#111 meeting whether simultaneous PRACH transmissions using multiple Tx chains can be supported for multiple PRACH transmission with same Tx beam. This highly depends on UE capability on whether UE is equipped with multiple Tx chains and without power sharing. However, during initial access, gNB may not know the UE capability and hence may not be able to perform coherent combining on the PRACH detection statistic. In this regard, performance degradation may be expected compared to the case when multiple PRACH transmissions are multiplexed in the TDM manner. In our view, multiple PRACH transmissions are not located at the same time instances.

Proposal 5

- For multiple PRACH transmission with same Tx beam,

  - Frequency hopping is supported for the multiple PRACH transmissions.

  - Multiple PRACH transmission are not located at the same time instances.

## Association between SSB and RO group

At the RAN1#110b-e meeting, it was agreed that same PRACH preamble during multiple PRACH transmissions in one RACH attempt [6]. The main motivation is to allow gNB to perform combining on multiple detection statistics for better performance. It is for further study on whether different preambles can be used during multiple PRACH transmissions. Note that at least for CBRA procedure, if supported, cell specific PRACH sequence pattern across different transmissions needs to be aligned between gNB and UE side so as to ensure the PRACH coverage enhancement. In this regard, if two UEs select a same preamble index in the first PRACH transmission, consistent preamble collision will still happen for multiple PRACH transmissions. In our view, considering the additional specification impact and no clear benefit compared to same preamble, different preambles during multiple PRACH transmission in one attempt are not supported.

Proposal 6

- Different preambles during multiple PRACH transmission in one RACH attempt are not supported.

At the RAN1#112 meeting, it was agreed that "RO group" is assumed for multiple PRACH transmissions with separate preamble on shared ROs and/or multiple PRACH transmissions on separate ROs, and one RO group consists of valid RO(s) for a specific number of multiple PRACH transmissions [1].

Following the concept of RO group, it is more desirable to reuse the existing mapping rule between SSB and RO group in order to minimize the specification impact. In case separate preamble on shared ROs are configured to differentiate single and multiple PRACH transmissions, the ROs used for the PRACH repetitions can be the ones mapped to the same SSB after PRACH occasions are associated with all the SSBs. In this case, depending on the configuration of the number of PRACH occasions associated with an SSB, PRACH repetitions may be transmitted in non-consecutive slots. Figure 3 illustrates one example of association between SSB and RO group for shared ROs for multiple PRACH transmissions.

Figure 3. Association between SSB and RO group for shared ROs

Further, in case separate ROs are employed to differentiate single and multiple PRACH transmissions, a proper SSB to RO mapping ratio, e.g., N < 1 may be configured so that one SSB can be associated with multiple ROs. In this case, the multiple consecutive ROs can be formed as a RO group depending on the number of multiple PRACH transmissions. Figure 4 illustrates one example of association between SSB and RO group for separate ROs for multiple PRACH transmissions. In the figure, 4 consecutive TDM’ed ROs are formed as a RO group, which is associated with an SSB.

Figure 4. Association between SSB and RO group for separate ROs

Proposal 7

- Existing association rule between SSB and RO can be reused for association between SSB and RO group.

- In case of separate preamble on shared ROs for differentiation of single and multiple PRACH transmissions, ROs used for multiple PRACH transmissions can be the ones mapped to a same SSB after ROs are associated with all the SSBs.

- In case of separate ROs for differentiation of single and multiple PRACH transmissions, consecutive ROs in time domain can be formed as a RO group.

## RAR monitoring window and RA-RNTI determination

During 4-step RACH procedure, UE monitors PDCCH for scheduling Msg2 transmission in a RAR monitoring window, which starts at the first symbol of the earliest CORESET the UE is configured to receive PDCCH for Type1-PDCCH CSS, that is at least one symbol after the last symbol of the PRACH occasion corresponding to the PRACH transmission. When multiple PRACH transmissions with same Tx beam are employed during 4-step RACH procedure, starting positioning of the RAR monitoring window needs to be determined accordingly.

At the RAN1#112 meeting, it was agreed that only one RAR window is supported for RAR monitoring for one RACH attempt for multiple PRACH transmissions with same Tx beam [1]. This mechanism can be beneficial to minimize the specification impact compared to the option with one RAR window per each PRACH transmission.

To further determine the RAR monitoring window, one option is to consider the first PRACH occasion corresponding to multiple PRACH transmission, which can help reduce the latency for 4-step RACH procedure. This option, however, may require gNB to perform opportunistic PRACH detection by attempting to detect the PRACH in each PRACH transmission occasion. Alternatively, UE may monitor the PDCCH for scheduling RAR after the last symbol of the last PRACH occasion corresponding to the multiple PRACH transmissions. In this case, gNB can perform combining for detection statistics on all PRACH transmission occasions so as to ensure the PRACH detection performance.

Based on the discussions above, considering the detection performance and implementation complexity, it would be more desirable to determine the RAR monitoring window based on the last PRACH occasion corresponding to the multiple PRACH transmissions.

Proposal 8

- RAR window is determined based on the last PRACH occasion corresponding to the multiple PRACH transmissions.

As defined in TS 38.321, RA-RNTI for a single PRACH transmission is determined as follows [7]:

The RA-RNTI associated with the PRACH occasion in which the Random Access Preamble is transmitted, is computed as: RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id where s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14), t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80), where the subcarrier spacing to determine t_id is based on the value of μ specified in clause 5.3.2 in TS 38.211 [8] for μ = {0, 1, 2, 3}, and for μ = {5, 6}, t_id is the index of the 120 kHz slot in a system frame that contains the PRACH occasion (0 ≤ t_id < 80), f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8), and ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier).

For multiple PRACH transmission with same Tx beam, RA-RNTI determination may need to be updated to avoid ambiguity between gNB and UE on Msg2. As mentioned above, for RAR monitoring window, it is more desirable to determine the RAR monitoring window based on the last PRACH occasion corresponding to the multiple PRACH transmissions. To follow same design principle, RA-RNTI can be determined based on the last RO corresponding to the multiple PRACH transmissions.

Proposal 9

- For multiple PRACH transmissions with same Tx beam, RA-RNTI is determined based on the last RO corresponding to the multiple PRACH transmissions.

## Power control for multiple PRACH transmissions

At the RAN1#112 meeting, it was agreed that for multiple PRACH transmissions with same Tx beam in one RACH attempt, transmission power ramping is not applied within one RACH attempt [1]. Given that multiple PRACH within one RACH attempt are only transmitted over ROs associated with the same SSB/CSI-RS, it is straightforward to apply same transmit power for all the multiple PRACH transmission within one RACH attempt. In this regard, transmit power determined for the first PRACH transmission may be applied for the subsequent PRACH transmission during multiple PRACH transmissions.

Similar to the power control mechanism as defined for single PRACH transmissions, for the next RACH attempt for multiple PRACH transmission, UE may apply the transmit power ramping so as to improve the PRACH detection performance. A separate power ramping step size may be configured for multiple PRACH transmissions compared to single PRACH transmission.

Proposal 10

- Same transmit power is applied for multiple PRACH transmissions within one RACH attempt.

- Power ramping is supported for multiple PRACH transmissions across RACH attempts.

# Multiple PRACH transmissions with different beams

At the RAN1#111 meeting, it was agreed to further study UE uses different TX beams to transmit the multiple PRACH over ROs associated with the same SSB/CSI-RS [1]. Note that for multiple PRACH transmission with different beams, the following two scenarios can be considered:

- Scenario 1: SSB is transmitted with a wider beamwidth, while multiple PRACH can be transmitted with narrower beam sweeping.

- Scenario 2: during initial access, UE that is not capable of full beam correspondence may perform PRACH with beam sweeping.

For the Scenario 1, a hierarchical beam structure can be employed at gNB for system operating at mmWave bands, where SSB may be transmitted using a wider beamwidth to cover a larger area and CSI-RS can be transmitted using a narrower beam based on beam refinement procedure for higher data rate. When UE attempts to access the network, the UE will select one SSB with RSRP above threshold and transmit the PRACH using the Tx beam based on association between SSB and PRACH occasion.

According to current NR specification, UE only transmits a PRACH with single Tx beam and attempts to monitor RAR within a monitoring window. If the PRACH is not detected by gNB, the UE needs to wait for next PRACH opportunity for PRACH transmission with same or different Tx beams after the RAR monitoring window. To reduce the access latency and improve the reliability of the PRACH detection during 4-step RACH procedure, the UE may transmit multiple PRACH transmissions with narrower beam sweeping.

Figure 5 illustrates PRACH transmissions with narrow beam sweeping during initial access. Note that all these narrower beams may be linked to the same SSB with wider beamwidth. In this case, same procedure as defined for multiple PRACH transmission with same Tx beams can be applied.

Figure 5. PRACH transmissions with narrow beam sweeping

Observation 2

- When SSB is transmitted with wide beam, PRACH transmission with narrow beam sweeping may help reduce access latency and improve reliability of PRACH detection.

For the Scenario 2, as defined in UE feature list and illustrated in Table 1, UE that fulfils the beam correspondence requirement with the uplink beam sweeping shall set the bit to 0 [8]. This indicates that during initial access, UE may not support full beam correspondence.

Table 1. UE feature group for beam correspondence

| 2-20 | Beam correspondence | Support Beam correspondence | beamCorrespondenceWithoutUL-BeamSweeping | MIMO-ParametersPerBand | No | Applicable only to FR2 | Beam correspondence means each Tx port can be beamformed in a desirable direction but does not imply setting phase across ports. | Mandatory with capability signalling - UE that fulfils the beam correspondence requirement without the uplink beam sweeping shall set the bit to 1 - UE that fulfils the beam correspondence requirement with the uplink beam sweeping shall set the bit to 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

When UE is not capable of full beam correspondence, Tx and Rx beam are not fully aligned, which indicates that the selected Rx beam for the corresponding SSB may not be the most suitable Tx beam for PRACH transmission. If UE utilizes the single Tx beam for PRACH transmission, gNB may not be able to detect the PRACH preamble due to Tx and Rx beam mis-alignment. In this case, it may be more beneficial to employ the PRACH beam sweeping at UE side before the start of RAR monitoring window to improve the detection performance and reduce the access latency, especially when considering the scenario that narrow beam is applied for SSB transmission and PRACH detection.

Observation 3

- When UE is not capable of full beam correspondence, it may be more beneficial to employ the PRACH beam sweeping at UE side to improve the detection performance and reduce the access latency.

Note that for multiple PRACH transmission with different beams, two options can be further considered, which depends on whether the ROs used for the multiple PRACH transmissions are associated with same or different SSBs. It is evident that when multiple PRACH transmissions are associated with different SSBs, substantial specification impact, e.g., the SSB and RO association, determination of QCL assumption for RAR monitoring, etc., can be expected. On the contrary, in case of multiple PRACH transmissions on ROs associated with same SSB, the design for multiple PRACH transmissions with same Tx beam can be largely reused, which can help minimize specification effort.

Based on the discussions above, considering the specification impact, and the benefits of access latency reduction and improved detection performance, in our view, at least the multiple PRACH transmission on ROs associated with same SSB/CSI-RS can be supported for multiple PRACH transmissions with different Tx beams in Rel-18.

Proposal 11:

- For multiple PRACH transmissions with different Tx beams, the multiple PRACH transmission on ROs associated with same SSB/CSI-RS can be supported in Rel-18.

# Conclusions

In this contribution, we presented our views on PRACH coverage enhancement. Further, we summarize the observations and proposals as follows:

Observation 1

- For 2 and 4 PRACH repetitions with frequency hopping, ~2.5dB performance gain can be achieved compared to PRACH repetitions without frequency hopping.

Observation 2

- When SSB is transmitted with wide beam, PRACH transmission with narrow beam sweeping may help reduce access latency and improve reliability of PRACH detection.

Observation 3

- When UE is not capable of full beam correspondence, it may be more beneficial to employ the PRACH beam sweeping at UE side to improve the detection performance and reduce the access latency.

Proposal 1

- Multiple PRACH transmissions are applied for all short PRACH formats and PRACH format 0.

- Multiple PRACH transmissions are applied for both CFRA and CBRA.

Proposal 2

- RAN1 to confirm the following working assumptions

  - For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, at least support that multiple PRACH are transmitted on separate ROs.

  - For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, support that multiple PRACH are transmitted with separate preamble on shared ROs.

Proposal 3

- To differentiate multiple PRACH transmission with different number of multiple PRACH transmissions, only shared PRACH occasions with separate preamble are supported.

- Repetition level ramping is supported when more than one number of multiple PRACH transmissions is configured.

Proposal 4

- Multiple SSB-RSRP thresholds are used to determine the number of multiple PRACH transmissions if multiple values are configured.

- A common SSB-RSRP threshold may be applied for multiple PRACH transmissions and request of Msg3 PUSCH repetitions

Proposal 5

- For multiple PRACH transmission with same Tx beam,

  - Frequency hopping is supported for the multiple PRACH transmissions.

  - Multiple PRACH transmission are not located at the same time instances.

Proposal 6

- Different preambles during multiple PRACH transmission in one RACH attempt are not supported.

Proposal 7

- Existing association rule between SSB and RO can be reused for association between SSB and RO group.

- In case of separate preamble on shared ROs for differentiation of single and multiple PRACH transmissions, ROs used for multiple PRACH transmissions can be the ones mapped to a same SSB after ROs are associated with all the SSBs.

- In case of separate ROs for differentiation of single and multiple PRACH transmissions, consecutive ROs in time domain can be formed as a RO group.

Proposal 8

- RAR window is determined based on the last PRACH occasion corresponding to the multiple PRACH transmissions.

Proposal 9

- For multiple PRACH transmissions with same Tx beam, RA-RNTI is determined based on the last RO corresponding to the multiple PRACH transmissions.

Proposal 10

- Same transmit power is applied for multiple PRACH transmissions within one RACH attempt.

- Power ramping is supported for multiple PRACH transmissions across RACH attempts.

Proposal 11:

- For multiple PRACH transmissions with different Tx beams, the multiple PRACH transmission on ROs associated with same SSB/CSI-RS can be supported in Rel-18.

# References

- Chairman’s notes, RAN1#112 Meeting, March 2023

- R1-2302787, “Discussions on power domain enhancement”, Intel Corporation, April 2023

- R1-2302788, “Dynamic switching between DFT-S-OFDM and CP-OFDM”, Intel Corporation, April 2023

- TR 38.830, “Study on NR coverage enhancements”.

- Chairman’s notes, RAN1#111 Meeting, November 2022

- Chairman’s notes, RAN1#110b-e Meeting, October 2022

- TS 38.321, “NR: Medium Access Control (MAC) protocol specification”

- TS 38.322, “NR: User Equipment (UE) feature list”.

# Appendix

Table 2. Simulation assumption for PRACH repetition

| Parameters | Values |
| --- | --- |
| Carrier frequency | 700MHz |
| PRACH formats | Format 0 |
| UE antenna configuration | 1Tx |
| gNB antenna configuration | 2Rx |
| Channel model | TDL-C 300ns with 3km/h |
| Timing offset | 0 |
| Frequency offset | 0 |
| Performance metrics | 1% PRACH misdetection probability 0.1% false alarm probability |