3GPP TSG RAN WG1 #112bis						                 R1-2302824

e-Meeting, Apr. 17th – 26th, 2023

Agenda Item:	5

Source:	InterDigital, Inc.

Title:	Discussion on RAN2 LS on comparison of SL-RSRP and SD-RSRP measurements

Document for:	Discussion and Decision

# Introduction

RAN1 has been asked (along with RAN4) to respond to an LS from RAN2 [1].  In this contribution, perform some analysis of the questions asked by RAN2 and provide a draft LS response in the appendix.

Discussion

RAN2 has sent an LS to RAN1/RAN4 with the following text/questions:

1. Overall Description: During the work on session continuity enhancements within NR_SL_relay_enh work item a new measurement report trigger has been proposed yet with no consensus in RAN2: Event Z2: The measured RSRP of a candidate L2 U2N Relay UE becomes an offset better than measured RSRP of the serving L2 U2N Relay UE This new event requires either the comparison of SD-RSRP (RSRP measurement on broadcast PC5 link used for discovery messages) measurements from different UEs, or the comparison of SL-RSRP (RSRP measurement on unicast PC5 link) and SD-RSRP measurements, or the comparison of SL-RSRP measurements from different UEs. During the discussion some companies expressed concerns on direct comparison of SD-RSRP and SL-RSRP and the comparison of SL-RSRP measurements form different UEs due to the power control mechanism applied on PC5 unicast links, while other companies think the network can handle such comparison by using an appropriate offset in the measurement event. Before agreeing to the introduction of this measurement report trigger, RAN2 would like to ask feedback from RAN1 and RAN4 on the comparisons of RSRP measurements: Q1: Can the comparison of SL-RSRP and SD-RSRP measurement be used for the purposes of triggering a measurement report? Q2: Can the comparison of SL-RSRP measurements from different UEs be used for the purposes of triggering a measurement report?

Power control for discovery transmission, and data transmission in unicast uses different mechanisms.  Power control for discovery uses the Uu path loss, whereas power control for unicast uses both the Uu path loss and the SL path loss.  Based on this difference, any comparison needed for the purposes of triggering a measurement report would require the network to be aware of the difference in path loss in order to configure an appropriate offset.  We consider each of the two scenarios mentioned by RAN2 separately.

SL and SD Comparison (Q1)

In this scenario, the remote UE has a unicast link with the source relay UE but may not have a unicast link with the potential target relay UE.  The source relay UE is in RRC_CONNECTED, and the target relay UE may be in either RRC_IDLE/RRC_INACTIVE or RRC_CONNECTED.  The remote UE needs to compare SL-RSRP of the source relay UE with SD-RSRP of the target relay UE based on an offset configured by the network.  This offset should take into account:

- SL path loss between the remote UE and the source relay UE

  - This could be known by the network if the network configured SL RSRP measurements at the remote UE

- Uu path loss of the source relay UE

  - This could be known by the network based on Uu measurements reported by the source relay UE

- Uu path loss of the target relay UE

  - If the target relay UE is in RRC_CONNECTED, this could be known by the network based on Uu measurements reported by the target relay UE

  - If the target relay UE is in RRC_IDLE/RRC_INACTIVE, the network configures the target relay UE with a Uu path loss range allowing it to transmit discovery message.  Therefore, the relay UE can know this to within a range/error that is itself determined by the network.

Observation 1: For SL-RSRP and SD-RSRP comparison, if the target relay UE is in RRC_CONNECTED, comparison is possible for the purposes of triggering a measurement report as the network can determine an appropriate offset in the trigger from knowledge of the Uu path loss of the source and target relay UEs and the SL path loss between source and remote UE.

Observation 2: For SL-RSRP and SD-RSRP comparison, if the target relay UE is in RRC_IDLE/RRC_INACTIVE, comparison can be performed within an error range that is known to the network because the network can configure the range of possible Uu path loss of the target relay UE for transmission of the discovery.

SL and SL Comparison (Q2)

In this scenario, the remote UE has a unicast link with the source relay UE and a unicast link with the potential target relay UE.  Since the measurements of both source and target relay UE are the same (i.e., SL-RSRP), they can be directly compared.

Observation 3: For SL-RSRP and SL-RSRP comparison, because the same quantities are being compared for both relays, direct comparison is possible.

Based on these observations, we think RAN1 should respond to the LS from RAN2.

Proposal 1: Send a response LS with following answers (also draft response LS in the appendix):

Q1: 	Can the comparison of SL-RSRP and SD-RSRP measurement be used for the purposes of triggering a measurement report?

Response: From RAN1 perspective:

- if the target relay UE is in RRC_CONNECTED, comparison is possible for the purposes of triggering a measurement report as the network can determine an appropriate offset in the trigger from knowledge of the Uu path loss of the source and target relay UEs and the SL path loss between source and remote UE.

- if the target relay UE is in RRC_IDLE/RRC_INACTIVE, comparison can be performed within an error range that is known to the network because the network can configure the range of possible Uu path loss of the target relay UE for transmission of the discovery.

Q2: 	Can the comparison of SL-RSRP measurements from different UEs be used for the purposes of triggering a measurement report?

Response: From RAN1 perspective because the same quantities are being compared for both relays, direct comparison for the purposes of triggering a measurement report is possible.

# Conclusion

In this contribution, we have discussed and propose the following:

Proposal 1: Send a response LS with following answers (also draft response LS in the appendix):

Q1: 	Can the comparison of SL-RSRP and SD-RSRP measurement be used for the purposes of triggering a measurement report?

Response: From RAN1 perspective:

- if the target relay UE is in RRC_CONNECTED, comparison is possible for the purposes of triggering a measurement report as the network can determine an appropriate offset in the trigger from knowledge of the Uu path loss of the source and target relay UEs and the SL path loss between source and remote UE.

- if the target relay UE is in RRC_IDLE/RRC_INACTIVE, comparison can be performed within an error range that is known to the network because the network can configure the range of possible Uu path loss of the target relay UE for transmission of the discovery.

Q2: 	Can the comparison of SL-RSRP measurements from different UEs be used for the purposes of triggering a measurement report?

Response: From RAN1 perspective because the same quantities are being compared for both relays, direct comparison for the purposes of triggering a measurement report is possible.

# References

- R1-2302824 - Discussion on RAN2 LS on comparison of SL-RSRP and SD-RSRP measurements

# Appendix – Proposed LS Text to RAN2

3GPP TSG-RAN WG1 Meeting #112bis-e	R2-230xxxx

Online, April 17, 26, 2023

Title:	[DRAFT] Response LS on Comparison of SL-RSRP and SD-RSRP measurements

Response to:	R1-2302824

Release:	Release 18

Work Item:	NR_SL_relay_enh

Source:	InterDigital [TSG RAN WG2]

To:	TSG RAN WG2, TSG RAN WG 4

Cc:

Contact Person:

#### Name:	Moon-il Lee

E-mail Address:	moonil.lee@interdigital.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 thanks RAN1 for their LS R1-2302824.  RAN1 discussed the questions sent by RAN1 and has the following answers.

Q1: 	Can the comparison of SL-RSRP and SD-RSRP measurement be used for the purposes of triggering a measurement report?

Response: From RAN1 perspective:

- if the target relay UE is in RRC_CONNECTED, comparison is possible for the purposes of triggering a measurement report as the network can determine an appropriate offset in the trigger from knowledge of the Uu path loss of the source and target relay UEs and the SL path loss between source and remote UE.

- if the target relay UE is in RRC_IDLE/RRC_INACTIVE, comparison can be performed within an error range that is known to the network because the network can configure the range of possible Uu path loss of the target relay UE for transmission of the discovery.

Q2: 	Can the comparison of SL-RSRP measurements from different UEs be used for the purposes of triggering a measurement report?

Response: From RAN1 perspective because the same quantities are being compared for both relays, direct comparison for the purposes of triggering a measurement report is possible.

2. Actions:

To RAN WG2

ACTION: 	RAN2 should inform RAN1 if they have any further questions.

3. Date of Next TSG-RAN WG1 Meeting:

RAN2#113	from 2023-05-22	to 2023-05-26		Incheon

RAN2#114	from 2023-08-21	to 2023-08-25		Toulouse