3GPP TSG RAN WG1 #112bis-e			R1-2302825

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.2.3.1

Source:	InterDigital, Inc.

Title:	Discussion for evaluation on AI/ML for beam management

Document for:	Discussion and Decision

# Introduction

In RAN#94-e [1], the study item for AI/ML has been approved for NR Air Interface. In this contribution, we discuss evaluation methodology, KPI and target scenarios for AI/ML beam management and provide preliminary evaluation results for AI/ML aided beam prediction.

# Discussions

## Evaluation methodology

For AI/ML, need of specification enhancements should be well justified with appropriate baseline and KPIs with proper target scenarios. In addition, complex specification enhancements should be avoided unless benefits are well justified. In this section, we provide our views on evaluation methodologies including baseline, KPIs and target scenarios for AI/ML aided beam management.

### Baseline

Discussion on levels of inter-node coordination and information exchange is the most important aspect for evaluation as different levels of coordination and information exchange generally provide different levels of performance benefits. In RAN#93-e [2] and RAN#94-e [3], the following levels of inter-node coordination and information exchange were discussed during the meetings.

0a) No collaboration framework: AI/ML algorithms purely implementation based and not requiring air-interface changes. 0b) No collaboration framework with modified Air-Interface catering to efficient implementation-based AI/ML algorithms. 1) Inter-node assistance to improve the respective nodes’ AI/ML algorithms. This would apply to UEs getting assistance from gNBs (for training, adaptation, etc.) and vice-versa. This level does not require model exchange between network nodes. 2) Joint ML operation between UEs and gNBs. This level requires AI/ML model instruction or exchange between network nodes.

It should be noted that legacy beam management with Rel-17 without AI/ML algorithms is not an appropriate baseline as implementation-based AI/ML operation is available for UE and gNB implementations. On the other hand, implementation-based AI/ML algorithm (level 0a)) could be an appropriate baseline to accurately evaluate the benefits of AI/ML with specification enhancements as implementation based AI/ML algorithm shows actual achievable performance without specification enhancements.

Observation 1: Legacy beam management with Rel-17 without AI/ML algorithms is not an appropriate baseline as implementation-based AI/ML operation is available for UE and gNB implementations.

Proposal 1: ‘No collaboration framework: AI/ML algorithms purely implementation based and not requiring air-interface changes’ could be an appropriate baseline to accurately evaluate the benefits of AI/ML with specification enhancements.

### KPIs

In RAN1#110bis-e [4], the following agreements and working assumptions were made on possible KPI options:

Agreement - The options to evaluate beam prediction accuracy (%): - Top-1 (%): the percentage of “the Top-1 genie-aided beam is Top-1 predicted beam” - Top-K/1 (%): the percentage of “the Top-1 genie-aided beam is one of the Top-K predicted beams” - Top-1/K (%) (Optional): the percentage of “the Top-1 predicted beam is one of the Top-K genie-aided beams” - Where K >1 and values can be reported by companies. Agreement - For DL Tx beam prediction, the definition of Top-1 genie-aided Tx beam considers the following options - Option A, the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx and Rx beams - Option B, the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s) - FFS on specific Rx beam(s) - Note: specific Rx beams are subset of all Rx beams Agreement - For DL Tx-Rx beam pair prediction, the definition of Top-1 genie-aided Tx-Rx beam pair considers the following options: - Option A: The Tx-Rx beam pair that results in the largest L1-RSRP over all Tx and Rx beams - Option B: The Tx-Rx beam pair that results in the largest L1-RSRP over all Tx over all Tx beams with specific Rx beam(s) - FFS on specific Rx beam(s) - Note: specific Rx beams are subset of all Rx beams Working Assumption For both BM-Case1 and BM-Case 2, the following table is adopted as working assumption for reporting the evaluation results. Table X. Evaluation results for [BM-Case1 or BM-Case2] without model generalization for [DL Tx beam prediction or Tx-Rx beam pair prediction or Rx beam prediction] Company A; ……. Assumptions; Number of [beams/beam pairs] in Set A. Number of [beams/beam pairs] in Set B. Baseline scheme. AI/ML model input/output; Model input. Model output. Data Size; Training. Testing. AI/ML model; [Short model description]. Model complexity. Computational complexity. Evaluation results [With AI/ML / baseline]; [Beam prediction accuracy (%)]; [KPI A]. [KPI B] …. [L1-RSRP Diff]; [Average L1-RSRP diff] …. [System performance]; [RS overhead Reduction (%)/ RS overhead]. [UCI report]. [UPT] …. To report the following in table caption: - Which side the model is deployed Further info for the columns: - Assumptions - Number of beams/beam pairs in Set A - Number of beams/beam pairs in Set B - Baseline scheme, e.g., Option 1 (exhaustive beam sweeping), Option 2(based on measurements of Set B), or baseline described by companies - Other assumptions can be added later based on agreements - Model input: input type(s) - Model output: output type(s), e.g., the best DL Tx and/or Rx beam ID, and/or L1-RSRPs of N beams(pairs) - Dataset size, both the size of training/validation dataset and the size of test dataset - Short model description: e.g., CNN, LSTM - Model complexity, in terms of “number of model parameters” and/or size (e.g. Mbyte)”, and - Computational complexity in terms of FLOPs - Evaluation results: agreed KPIs, with AI/ML / with baseline scheme (if applicable) Note: To report other simulation assumptions, if any. Working assumption - For the evaluation of the overhead for BM-Case1, further study the following two metrics for potential down selection: - Option A: RS overhead reduction, FFS for potential down selection: - Option 1: RS OH reduction%=1-NM - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement - where M is the total number of beams (pairs) to be predicted - Option 2: RS OH reduction%=1-NM - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML - Where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme - Companies report the assumption on beam sweeping - Option 3: RS OH reduction%=1-N+PM - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement - where M is the total number of beams (pairs) to be predicted - FFS the following alternatives consider different targets (e.g., beam or beam pair) for prediction: - Alt1: P is the number of Top-K selected beams (pairs) for beam sweeping (if applicable) - Alt2: P is the number of Top-K selected beams (pairs) not in Set B for beam sweeping (if applicable) - Alt3: P is the number of beams used for beam sweeping to get the best Rx beam (if applicable) - Companies report the assumption on beam sweeping - Other options can be reported by companies - Option B: RS overhead, FFS for potential down selection: - Option 1: RS OH = N, - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement - Option 2: RS OH = N + P - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement - FFS the following alternatives consider different targets (e.g., beam or beam pair) for prediction: - Alt1: P is the number of Top-K selected beams (pairs) for beam sweeping (if applicable) - Alt2: P is the number of Top-K selected beams (pairs) not in Set B for beam sweeping (if applicable) - Alt3: P is the number of beams used for beam sweeping to get the best Rx beam (if applicable) - Companies report the assumption on beam sweeping - Other options can be reported by companies

In addition, the following agreements were made in RAN1#111 [5]:

Agreement - For the evaluation of the overhead for BM-Case1, adoption the following metrics: - RS overhead reduction, - Option 1: RS OH reduction%=1-NM - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML - where M is the total number of beams (pairs) to be predicted - Option 2: RS OH reduction%=1-NM - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML, including the beams (pairs) required for additional measurements before/after the prediction if applicable - Where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme, including the beams (pairs) required for additional measurements before/after the prediction if applicable - Companies report the assumption on additional measurements Agreement - For the evaluation of the overhead for BM-Case2, adoption the following metrics: - RS overhead reduction, - Option 2: RS OH reduction%=1-NM - where N is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML, including the beams (pairs) required for additional measurements before/after the prediction if applicable - Where M is the total number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for baseline scheme - Companies report the assumption on additional measurements - FFS: Option 3: RS OH reduction%=1-NLM - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement for AI/ML in each time instance - where M is the total number of beams (pairs) to be predicted for each time instance - where L is ratio of periodicity of time instance for measurements to periodicity of time instance for prediction - Companies report the assumption on T1 and T2 patterns - Other options are not precluded and can be reported by companies.

In addition, the following agreements were made in RAN1#112 [6]:

Agreement - For DL Tx beam prediction, the definition of Top-1 genie-aided Tx beam is defined as - Option A (baseline): the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx and Rx beams - Option B(optional), the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s) - FFS on specific Rx beam(s) - Note: specific Rx beams are subset of all Rx beams - For DL Tx-Rx beam pair prediction, the definition of Top-1 genie-aided Tx-Rx beam pair is defined as - Option A: The Tx-Rx beam pair that results in the largest L1-RSRP over all Tx and Rx beams - Other options are not precluded and can be reported by companies. - Note: This is only for evaluation discussion Agreement - For AI/ML models, which provide L1-RSRP as the model output, to evaluate the accuracy of predicted L1-RSRP, companies optionally report average (absolute value)/CDF of the predicted L1-RSRP difference, where the predicted L1-RSRP difference is defined as: - The difference between the predicted L1-RSRP of Top-1[/K] predicted beam and the ideal L1-RSRP of the same beam.

For KPIs, the following aspects should be considered to accurately evaluate benefits for AI/ML aided beam management.

- System performance related KPIs

  - Although beam prediction accuracy related KPIs (e.g., Avg. L1-RSRP difference) provide brief information on beam selection, beam prediction accuracy related KPIs do not provide overall insight on actual benefits considering the following aspects:

    - Impact on system performance

      - Obviously, a best beam with X% better L1-RSRP does not imply that system performance is enhanced with X%. The performance benefit could be smaller or larger based on channel characteristics of the selected beam. Having said that, specification enhancements should be justified based on actual system performance benefits such as throughputs not merely based on beam related KPIs such as L1-RSRP difference.

    - UE reporting overhead

      - AI/ML based beam prediction can reduce required UE reporting overheads by predicting an optimized beam opposed to deciding an optimized beam based on UE reporting. As beam prediction related KPIs only show L1-RSRP difference or beam selection accuracy, there’s no way to reflect the reduced overhead for UE reporting via spatial domain prediction.

    - UE throughput considering RS overhead reduction

      - For system performance related KPIs, Avg. and 5% UE throughput should be used for evaluation. Other KPIs such as RS overhead reduction does not provide actual insight. For example, Y% RS overhead reduction does not mean Y% performance gain and the actual performance can be worse if the gain from RS overhead reduction is smaller than the performance loss from beam selection.

    - UCI report

      - UCI report overhead is not a metric for evaluation, but assumption for evaluation. Having said that, UCI report should be revised to UCI report overhead reduction.

- Beam information related KPIs

  - As discussed, beam information related KPI is not a measure to evaluate actual performance benefits from AI/ML based beam prediction, however, beam information related KPI can be used as a temporal measure to have an insight as beam information related KPI requires less computational overhead for evaluation. As shown in the above, three options for evaluating beam prediction accuracy were agreed in RAN1#110bis-e [4].

  - Average L1-RSRP difference of Top-1 predicted beam

    - Difference between estimated L1-RSRP and actual L1-RSRP could be a good option to identify potential benefits of AI/ML aided beam prediction in addition to the approved options for beam prediction accuracy. However, as the main objective of beam prediction is to identify the Top-1 or the Top-K beams based on beam quality, KPIs related to the difference between the estimated L1-RSRP and actual L1-RSRPs of the same beams should not get priority over beam ranking based KPIs (e.g., Top-1 (%), Top-K/1 (%), Top-1/K (%)).

  - Definition of Top-1 genie-aided Tx beam and Tx-Rx beam pair

    - In RAN1#110bis-e [4], two options were agreed for the definition of Top-1 genie-aided beam/beam pair. Option A is to consider all Tx and Rx beams, but Option B is to consider only specific Rx beam(s). Further, in RAN1#112 [6], for beam pair selection, it was agreed to use Option A as the main definition of Top-1 genie aided beam pair. Other options are not precluded and can be reported by companies. It is worth to note that, for option 2, if the genie aided beam is from specific Rx beam(s), evaluated metrics such as beam accuracy does not reflect actual benefits from AI/ML based beam prediction and misleads interpretation of evaluation metric for AI/ML based beam prediction. Therefore, considering Option A for both beam and beam pair selection is more appropriate.

Observation 2: Beam information related KPI is not a measure to evaluate actual performance benefits from AI/ML based beam prediction, however, beam information related KPI can be used as a temporal measure to have an insight as beam information related KPI requires less computational overhead for evaluation.

Observation 3: Specification enhancements should be justified based on actual system performance benefits such as throughputs not based on beam related KPIs such as L1-RSRP difference.

Observation 4: UCI report overhead is not a metric for evaluation, but assumption for evaluation.

Observation 5: Option B (specific Rx beam) does not reflect actual benefits from AI/ML based beam prediction and misleads interpretation of evaluation metric for AI/ML based beam prediction.

Observation 6: KPIs related to comparing L1-RSRP difference with predicted L1-RSRP difference does not reflect the main focus of beam predictions, i.e., to select the Top-1 or Top-K best beams.

Proposal 2: Support system performance related KPIs as mandatory KPIs.

- Support Avg. and 5% UE throughput for system performance KPIs.

Proposal 3: UCI report is reported as assumptions not as KPI.

Proposal 4: KPIs related to the difference between the estimated L1-RSRP and actual L1-RSRPs of the same beams get lower priority over beam ranking based KPIs (e.g., Top-1 (%), Top-K/1 (%), Top-1/K (%)).

Proposal 5: Support beam information related KPIs as optional for temporal measures.

- Support average L1-RSRP difference of Top-1 predicted beam in addition to the agreed options for beam prediction accuracy (%).

Proposal 6: For the definition of Top-1 genie-aided beam/beam pair, consider Option A, i.e., the Top-1 genie-aided Tx beam/beam pair is the Tx beam that results in the largest L1-RSRP over all Tx and Rx beams/beam pairs.

### Evaluation scenarios

Selection of beam pattern types

In RAN1#110bis-e [4], the following agreement was made for spatial/temporal domain beam prediction:

Agreement - Study the following options on the selection of Set B of beams (pairs) - Option 1: Set B is fixed across training and inference - Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/report/measurement during training and/or inference), FFS: - Opt A: Set B is changed following a set of pre-configured patterns - Opt B: Set B is randomly changed among pre-configured patterns - Opt C: Set B is randomly changed among Set A beams (pairs) - The number of beams(pairs) in Set B can be fixed or variable - Note: BM-Case1 and BM-Case2 may be considered for different option. - Other options are not precluded.

Based on the agreement in RAN1#110bis-e, the following agreement was made in RAN1#111 [5],

Agreement - Companies report the pattern of Set B. - Further study the performance with different patterns of set B(s) for fixed Set B (Option 1) and different pre-configured/pre-known patterns of Set B(s) (Option 2A and 2B).

Based on the agreed options for selecting Set B beams/beam pairs, we provide the following analysis.

Table 1. Analysis of different types of patterns for Set B

| Set B Type Quality Metric | Fixed Set B (Type #1) | Set B following a set of preconfigured patterns (Type #2) | Set B is randomly changed among preconfigured patterns (Type #3) | Set B is randomly changed among Set A (all beam pairs) (Type #4) |
| --- | --- | --- | --- | --- |
| Performance | 3 | 2 | 1 | 1 |
| Flexibility of inference input | 0 | 1 | 2 | 3 |
| Range of inputs required for training | 3 (small range is better) | 2 | 1 | 0 |

Score ranges from 0 (worst/lowest) to 3 (best/highest)

- Performance

  - According to the evaluation results, fixed set B (Type #1) shows better performance than variable patterns including fixed preconfigured patterns (Type #2), randomly changed preconfigured patterns (Type #3) and randomly changed patterns among Set A (Type #4).

- Flexibility of inference input

  - In the sense of flexibility, Type #4 provides best flexibility as any of beams can be used as input for inference input as any of beams can be used as input. On the other hand, Type #2 and Type #3 require some limitation as inputs are limited to a set of beams (Set B). Among all the types, Type #1 provides worst flexibility as inference input is always fixed and there could be inference performance degradation when any beam information of fixed Set B is not available.

- Range of inputs required for training

  - For range of inputs required for training, Type #1 is best as Type #1 requires data sets only for the fixed Set B. This means that the amount of data acquisition needed for training/inference of Type #1 is lower. Type #2 and Type #3 require relatively large amount of data acquisition for training/inference as multiple data sets preconfigured or randomly selected among Set B should be used. Type #4 requires largest amount of data acquisition as Type #4 utilizes whole Set #A for training and inference.

Observation 7: For different beam pattern type, each beam pattern type has its own pros and cons for performance, flexibility of inference input and range of inputs required for training.

Proposal 7: Further study benefits of various beam pattern types.

Furthermore, the following agreements were made in RAN1#112 [6], for the case of variable Set B (e.g., different beams (pairs) patterns in each time instance/report/measurement during training and/or inference).

Agreement - For the evaluation of Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/report/measurement during training and/or inference), further study the following options as AI/ML model inputs - Alt 2: Implicit information of Tx beam ID and/or Rx beam ID - E.g., measurements of Set B of beams together with default values (e.g. 0) for the beams not in Set B are used as AI inputs in a certain order/ matrix/ vector. - Detailed assumption can be reported by companies. - Alt 3: Tx beam ID and/or Rx beam ID is used as inputs of AI/ML explicitly - Note: Specification impact can be discussed separately. Agreement - Additionally study the following option on the selection of Set B of beams (pairs) (for Option 2: Set B is variable) - Opt D: Set B is a subset of measured beams (pairs) Set C (including Set B = Set C), e.g. Top-K beams(pairs) of Set C - Companies report the number of pre-configured patterns used in the evaluation for Option 2: Set B is variable if applicable (e.g. Opt A and Opt B)

For the evaluation of Option 2, two alternatives, implicit information and explicit Tx/Rx beam IDs were agreed as AI/ML model inputs. As an AI/ML model implicitly identifies Tx/Rx beam IDs (e.g., based on an order of RSRP value), explicit beam IDs may not be needed as AI/ML model inputs. Instead, implicit information with default values can be used to indicate that a corresponding beam information is not available. However, an efficient procedure for supporting Option 2 may be different for specification impact. For example, instead of reporting RSRP values for every beams and beam pairs, reporting explicit beam IDs and corresponding RSRP values can be more efficient. Given the situation, specification impact should be carefully discussed considering number of beams to be reported and required overheads for reporting corresponding RSRP values.

Observation 8: Alt 2 (implicit information of Tx beam ID and/or Rx beam ID) can be applied for evaluation of Option 2: Set B is variable.

Observation 9: For specification impact, an efficient procedure for supporting Option 2: Set B is variable can be different considering number of beams to be reported and required overheads for reporting corresponding RSRP values.

Proposal 8: For evaluation, Alt 2 (implicit information) can be used. However, specification impact to support Option 2 should be further discussed considering reporting information overheads.

Selecting a subset of measured beams (Set C) as Set B (Option D) can help to reduce the reporting overhead while maintaining beam prediction accuracy only when a proper subset selection method is used. In that sense, applying a proper beam subset selection procedure and an efficient reporting mechanism of the selected subset is crucial to get the advantage of Option D. Otherwise, utilization of Option D with inefficiently selected beam subset for Set B can degrade AI/ML beam prediction performance.

Observation 10: Option D, i.e., selecting Set B out of measured beams (Set C), can be beneficial as Set B can be selected in a way for minimizing performance loss with reduced reporting overhead.

Observation 11: Performance of Option D, i.e., Set B is a subset of measured beams (pairs) Set C (including Set B = Set C), can vary depending on beam subset selection methods.

Proposal 9: Consider Option D, i.e., selecting Set B out of measured beams (Set C), to identify performance benefits and corresponding potential specification enhancements.

Proposal 10: Identify potential methods for selecting Set B out of measured beams (Set C) and evaluate benefits of the identified Set B selection methods to decide an optimized Set B selection method.

Whether Set B is a subset of Set A or not.

- Set B is a subset of Set A

  - As using same beamwidth for all channels and signals is a general implementation within a frequency range, using a subset of Set A as Set B is a reasonable option if Set A and Set B are utilized in a same frequency range. This is especially beneficial when Set A and Set B are in an identical frequency range. However, it is difficult to use a subset of Set A considering different beamwidths in different frequency ranges.

Figure 1. Example of ‘Set B is a subset of Set A’ for BM-Case1

- Set A and Set B are different

  - As discussed, utilizing different beams is not a general option for different FRs. However, it is difficult to apply Alt.1 considering different beamwidths in different frequency ranges. In addition, it should be noted that utilization of wide beam information from a low frequency range has great potential as a low frequency range is more reliable and utilization of wide beam requires much less time and frequency resources for beam management. In our view, association between different frequency ranges should be supported for both between FR1 and FR2-1 and between FR2-1 and FR2-2.

Observation 12: As using same beamwidth for all channels and signals is a general implementation within a frequency range, using a subset of Set A as Set B is a reasonable option if Set A and Set B are utilized in a same frequency range.

Observation 13: It is difficult to use a subset of Set A considering different beamwidths for beam management between different frequency ranges.

Observation 14: Utilization of wide beam information from a low frequency range has great potential as a low frequency range is more reliable and utilization of wide beam requires much less time and frequency resources for beam management.

Proposal 11: Support ‘Set B is a subset of Set A’ when Set A and Set B are utilized in a same frequency range for both temporal/spatial domain prediction.

Proposal 12: Support ‘Set A and Set B are different’ when Set A and Set B are utilized in different frequency ranges for both temporal/spatial domain prediction.

Proposal 13: AI/ML based beam management based on association between different frequency ranges should supported for both between FR1 and FR2-1 and between FR2-1 and FR2-2.

### Impact of quantization errors associated with RSRP reporting

In RAN1#112 [6], following agreements were made on the impact of measurement and quantization errors associated with L1-RSRP measurement used for AI/ML model training and model inference.

Agreement - Further study the impact of quantization error of inputed L1-RSRP (for training and inference) for AI/ML model for beam management. - Existing quantization granularity of L1-RSRP (i.e., 1dB for the best beam, 2dB for the difference to the best beam) is the starting point for evaluation at least for network-sided model. Agreement - Further study on whether/how to evaluate the performance impact with L1-RSRP measurement accuracy.

Based on our evaluation results presented in detail in Section 3.2, we observe 1.5% difference in Top-1 beam prediction accuracy between Case 1 where all the beams are quantized with 1 dB step size, and Case 3 where the beam with the highest L1-RSRP is quantized with 1 dB step size and differential L1-RSRP of the remaining beams (15 beams in the evaluation) are quantized with 2 dB steps size.

Observation 15: Beam prediction accuracy marginally improves when the quantization step size used for beam measurement and reporting is reduced.

However, it is worth to note that UE can only report L1-RSRP of up to maximum of 4 beams according to the current specifications. That is, the UE reports one CRI and one L1-RSRP of a best beam with highest L1-RSRP in a RS resource set. In addition, the UE can additionally report up to 3 more CRIs and corresponding differential L1-RSRPs [7, clause 5.2.1.4.3]. The L1-RSRP of the best beam is reported with 1 dB quantization using 7 bits, and the differential L1-RSRPs of additional 3 beams are reported with 2 dB quantization using 4 bits. This gives the maximum L1-RSRP reporting overhead of 19 bits. If the identical procedure is extended to report more number of beams (e.g., 16 beams or 32 beams) that are required for AI/ML model inference/training, the required beam reporting overhead is significantly increases (e.g., 7 + 15 x 4 = 67 bits for 16 beams and 7 + 31 x 4 = 131 bits for 32 beams). Therefore, alternative methods, that are capable of reporting more beams (e.g., 16 beams or 32 beams) with high accuracy and low reporting overhead, are required to support efficient beam reporting for AI/ML model training/inference. Based on the discussion and the evaluation results, we make the following observations and the proposals.

Observation 16: According to the current specifications, UE can only report L1-RSRP of up to maximum of 4 beams.

Observation 17: If L1-RSRP reporting mechanism in the current specification is extended to report large number of beams, reporting overhead is significantly increased.

Proposal 14: Consider alternative beam reporting methods that can report L1-RSRP of more beams with minimized reporting overheads to support AI/ML based beam predictions.

Proposal 15: Identify and evaluate performance of potential beam reporting methods that can support large number of beams with minimized reporting overhead increment.

### Evaluation assumptions

This section provides our views on evaluation assumptions based on the agreed evaluation assumptions.

- Traffic model

  - For beam information related KPIs, no traffic model is needed to be defined as UE is measuring reference signals not decoding actual PDSCHs.

  - For system performance related KPIs, FTP traffic should be used to reflect practical traffics for the evaluation.

  - For FTP traffic model, FTP model 1 is not an appropriate option as FTP model generates a new UE for each packet. In this case, successful spatial/temporal domain beam prediction is not possible due to lack of the information. Between FTP model 2 and 3, FTP model 3 according to a Poisson process with arrival rate λ is preferred.

- UE distribution

  - For full buffer traffic (if supported), 10 UEs per sector/cell was agreed. For FTP traffic, if number of UEs is increased and small number of packets arrive for each UE, then it is difficult to efficiently evaluate benefits from AI/ML based beam prediction. Given the situation, it is preferred to keep the same number of UEs per cell with the number for full buffer traffic with 50% and 70% Rus.

  - For spatial domain beam prediction, two options were discussed for UE distribution. The first option was to support 80% indoor UEs and 20% outdoor UEs as defined in TR38.901 and the second option was to support 100% outdoor UEs. For time domain prediction, evaluation 100% outdoor UEs could be a reasonable option as baseline UE mobility assumption is 30km/h, however, applying 100% outdoor UEs for 3km/h is not a reasonable evaluation assumption.

Proposal 16: For traffic model, support the following evaluation assumptions:

- For beam information related KPIs, no traffic model is needed to be defined as UE is only measuring reference signals not decoding actual PDSCHs.

- For system performance related KPIs, FTP traffic should be used to reflect practical traffics for the evaluation.

- For FTP traffic model, FTP model 3 is preferred as generating a new UE for each packet (FTP model 1) is not appropriate for evaluating benefits from AI/ML based beam prediction.

Proposal 17: For UE distribution, support the following evaluation assumptions:

- For FTP traffic model, 10 UEs per cell/sector with 50% and 70% Rus is preferred.

- 80% outdoor UEs and 20% indoor UEs for spatial domain beam prediction as defined in TR 38.901 (Option 1).

# Evaluation results

## Spatial Prediction

In this section, we provide evaluation results of spatial beam prediction based on partial RSRP measurements. Out of 64 Tx beams between each UE and serving gNB, 50% of the RSRP measurements are assumed to be available. The AIML model outputs the predicted best beam index.

### UE Dropping Generalization

A ResNet AIML model is trained for spatial beam prediction in two UE dropping scenarios in UMa environment:

- Scenario #1: 100% Outdoor UEs (UE Speed: 3 Kmh)

- Scenario #2: 20% Outdoor UE and 80% Indoor UEs (UE Speed: 3 Kmh)

The input of the model is a fixed set of RSRP measurements (fixed set B)

We consider the following cases for evaluation:

- Case 1-1: Training on data from Scenario #1 and Testing on Data from Scenario #1

- Case 1-2: Training on data from Scenario #2 and Testing on Data from Scenario #1

- Case 1-3: Training on a mixed set of data from Scenario #1 and Scenario #2 and Testing on Data from Scenario # 1

- Case 2-1: Training on data from Scenario #2 and Testing on Data from Scenario #2

- Case 2-2: Training on data from Scenario #1 and Testing on Data from Scenario #2

- Case 2-3: Training on a mixed set of data from Scenario #1 and Scenario #2 and Testing on Data from Scenario #2

The following table summarizes the accuracy results.

Table 2: Spatial Prediction Accuracy of AIML-based Classification (UE Dropping Generalization)

| Case | Top-1 Accuracy | Top-3/1 Accuracy | Top-5/1 Accuracy | Top-1/5 Accuracy |
| --- | --- | --- | --- | --- |
| Case 1-1 | 83.05% | 98.14% | 99.57% | 97.90% |
| Case 1-2 | 79.48% | 97.39% | 99.48% | 95.90% |
| Case 1-3 | 85.19% | 98.29% | 99.71% | 97.57% |
| Case 2-1 | 82.29% | 97.57% | 99.24% | 97.52% |
| Case 2-2 | 71.52% | 91.90% | 95.43% | 94.71% |
| Case 2-3 | 83.14% | 98.10% | 99.19% | 98.38% |

Observation 18: Training an AIML model for a certain environment (e.g., UMa) and a certain UE dropping scenario for a different UE dropping scenario results in degradation in Top-1 accuracy by 4%-11%.

Observation 19: Training an AIML model for a certain environment (e.g., UMa) and a mixed data set from different UE dropping scenarios can be generalized for different UE dropping scenarios.

In addition, Figures 2-a and 2-b provide prediction accuracy within an error margin in dB when testing with UE dropping Scenarios #1 and #2, respectively. Based on the error margin, the best beam selected by AIML model was counted as misdetection only when the RSRP of the selected beam differs from the optimal beam obtained from actual RSRP values by a value greater than the error margin.

| (a) | (b) |
| --- | --- |

Figure 2: Accuracy of spatial beam prediction with error margin for Scenario#1 and Scenario #2

### Beam Pattern Generalization

A ResNet AIML model is trained for spatial beam prediction for multiple beam patterns in UMa environment with mixed UE dropping (i.e., examples come from Scenario#1 and Scenario# 2). We consider a measurement set B consisting of 32 Tx beams, which a subset of set A consisting of 64 Tx beams. We consider the following beam patterns for set B:

|  | Beam is member of set A and set B |
| --- | --- |
|  | Beam is member of set A |

Pattern #1

- Pattern #2: Compliment of Pattern #1

- Pattern #3: Random Pattern (Pattern of available / missing RSRP measurements differ per training/testing example but the missing ratio is fixed)

Figure 3. Beam patterns of Set B for 32 Tx beams

The input of the model is a set of RSRP measurements according to one of the patterns. We consider the following cases for evaluation:

- Case 3-1: Training on data from Pattern #1 and Testing on Data from Pattern #1

- Case 3-2: Training on data from Pattern #2 and Testing on Data from Pattern #1

- Case 3-3: Training on a mixed set of data from Pattern #1 and Pattern #2 and Testing on Data from Pattern # 1

- Case 3-4: Training on data from Pattern #3 and Testing on Data from Pattern #1

- Case 4-1: Training on data from Pattern #2 and Testing on Data from Pattern #2

- Case 4-2: Training on data from Pattern #1 and Testing on Data from Pattern #2

- Case 4-3: Training on a mixed set of data from Pattern #1 and Pattern #2 and Testing on Data from Pattern # 2

- Case 4-4: Training on data from Pattern #3 and Testing on Data from Pattern #2

- Case 5: Training on data from Pattern #3 and Testing on Data from a mixed set of data from Pattern #1 and Pattern #2

The following table summarizes the accuracy results.

Table 3: Spatial Prediction Accuracy of AIML-based Classification (Beam Pattern Generalization, Set B Size = 32)

| Case | Top-1 Accuracy | Top-3/1 Accuracy | Top-5/1 Accuracy | Top-1/5 Accuracy |
| --- | --- | --- | --- | --- |
| Case 3-1 | 84.17% | 98.19% | 99.45% | 97.98% |
| Case 3-2 | 7.83% | 16.88% | 24.62% | 20.00% |
| Case 3-3 | 85.02% | 98.26% | 99.43% | 97.79% |
| Case 3-4 | 87.02% | 98.82% | 99.74% | 99.17% |
| Case 4-1 | 85.10% | 98.71% | 99.62% | 98.19% |
| Case 4-2 | 12.40% | 27.02% | 36.62% | 31.69% |
| Case 4-3 | 84.78% | 98.36% | 99.38% | 97.69% |
| Case 4-4 | 86.05% | 98.83% | 99.76% | 99.17% |
| Case 5 | 86.95% | 98.79% | 99.69% | 99.26% |

Furthermore, we consider a measurement set B consisting of 16 Tx beams, which a subset of set A consisting of 64 Tx beams. We consider the following beam patterns for set B:

- Pattern #4

|  | Beam is member of set A and set B |
| --- | --- |
|  | Beam is member of set A |

- Pattern #5: Shifted Version of Pattern #4

- Pattern #6: Random Pattern (Pattern of available / missing RSRP measurements differ per training/testing example but the number of input RSRPs is fixed)

Figure 4. Beam patterns of Set B for 16 Tx beams

The input of the model is a set of RSRP measurements according to one of the patterns. We consider the following cases for evaluation:

- Case 6-1: Training on data from Pattern #4 and Testing on Data from Pattern #4

- Case 6-2: Training on data from Pattern #5 and Testing on Data from Pattern #4

- Case 6-3: Training on a mixed set of data from Pattern #4 and Pattern #5 and Testing on Data from Pattern # 4

- Case 6-4: Training on data from Pattern #6 and Testing on Data from Pattern #4

- Case 7-1: Training on data from Pattern #5 and Testing on Data from Pattern #5

- Case 7-2: Training on data from Pattern #4 and Testing on Data from Pattern #5

- Case 7-3: Training on a mixed set of data from Pattern #4 and Pattern #5 and Testing on Data from Pattern # 5

- Case 7-4: Training on data from Pattern #6 and Testing on Data from Pattern #5

- Case 8: Training on data from Pattern #6 and Testing on Data from a mixed set of data from Pattern #4 and Pattern #5

The following table summarizes the accuracy results.

Table 4: Spatial Prediction Accuracy of AIML-based Classification (Beam Pattern Generalization, Set B Size = 16)

| Case | Top-1 Accuracy | Top-3/1 Accuracy | Top-5/1 Accuracy | Top-1/5 Accuracy |
| --- | --- | --- | --- | --- |
| Case 6-1 | 74.48% | 94.02% | 97.52% | 95.24% |
| Case 6-2 | 4.38% | 10.40% | 15.57% | 20.33% |
| Case 6-3 | 72.64% | 93.24% | 96.40% | 95.12% |
| Case 6-4 | 69.81% | 94.12% | 97.74% | 95.93% |
| Case 7-1 | 75.29% | 94.81% | 97.64% | 96.21% |
| Case 7-2 | 1.19% | 3.83% | 5.95% | 5.29% |
| Case 7-3 | 72.21% | 93.38% | 97.05% | 95.55% |
| Case 7-4 | 70.71% | 94.60% | 98.02% | 96.76% |
| Case 8 | 69.62% | 93.90% | 97.81% | 96.36% |

Observation 20: Training an AIML model with different beam patterns or random beam patterns can help obtain a model that generalizes over multiple beam patterns without significant degradation in prediction accuracy.

## Evaluation of the Impact of Quantization Errors on Spatial Predictions

In this section, we provide evaluation results for the impact of quantization errors on spatial beam prediction in a UMa scenario with 100% of the UEs are dropped outdoors. We consider a measurement set B consisting of 16 Tx beams, which a subset of set A consisting of 64 Tx beams. Selected beams for set B follow a fixed pattern as shown in the following figure.

|  | Azimuth Angles |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Elevation Angles |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  | Beam is member of set A and set B |
| --- | --- |
|  | Beam is member of set A |

Figure 5. Beam pattern of Set B for quantization errors

The RSRP measurements of set B are quantized using legacy 1 dB step or 2 dB step. The quantized RSRP measurements are used for both training and inference in a ResNet-based AIML model. The AIML model outputs the predicted best beam index.

We consider the following cases for evaluation:

- Baseline: AIML model trained using real RSRP values and tested using real RSRP values.

- Case 1: AIML model trained using quantized RSRP values and tested using quantized RSRP values, where quantization step is 1 dB following legacy quantization.

- Case 2: AIML model trained using quantized RSRP values and tested using quantized RSRP values, where quantization step is 2 dB.

- Case 3: AIML model trained using quantized RSRP values and tested using quantized RSRP values, where legacy differential quantization with 2 dB is applied.

The following table summarizes the accuracy results.

Table 4: Spatial Prediction Accuracy of AIML-based Classification with and without Quantization

| Case | Top-1 Accuracy | Top-3/1 Accuracy | Top-5/1 Accuracy | Top-1/5 Accuracy |
| --- | --- | --- | --- | --- |
| Baseline (No Quantization) | 79.12% | 95.50% | 98.00% | 96.24% |
| Case 1 (1 dB Quantization) | 76.02% | 94.60% | 97.64% | 95.64% |
| Case 2 (2 dB Quantization) | 71.21% | 92.00% | 96.26% | 94.74% |
| Case 3 (2 dB Differential Quantization) | 74.48% | 93.67% | 97.10% | 95.52% |

Observation 21: The more the quantization step is set, the more the degradation in prediction accuracy becomes.

Observation 22: Using 1 dB quantization step results in small degradation in accuracy by 3% compared to baseline (no quantization) for Top-1 accuracy and by <1% for Top-3/1 and Top5/1 accuracies.

In addition, Figure 3 provide prediction accuracy within an error margin in dB. Based on the error margin, the best beam selected by AIML model was counted as misdetection only when the RSRP of the selected beam differs from the optimal beam obtained from actual RSRP values by a value greater than the error margin.

Figure 6. Accuracy of spatial beam prediction with error margin with and without Quantization

## Temporal Prediction

In this section, we provide evaluation results of temporal beam prediction of prediction window frames based on past RSRP measurements obtained during observation window frames. RSRP measurements of observation window frames are obtained at time instants (T1, T2, T3, …, Ti, Ti+1, Ti+2, …), where Ti+1 – Ti = 160 ms.

We consider an observation window consisting of RSRP measurements at 3 time instant (Ti-2, Ti-1, Ti) and a prediction window in which we predict the best beam in 2 time instants (Ti+1, Ti+2). We compare the best beam prediction accuracy for the following UE speed generalization cases.

A transformer-based AIML model is trained to capture temporal correlations between elements in a sequence beam information from the observation window and predict the best beam in prediction window instants. Two UE mobility speeds in UMa environment were considered:

- Speed Scenario #1: 100% Outdoor UEs with UE Speed = 30 Kmh

- Speed Scenario #2: 100% Outdoor UEs with UE Speed = 60 Kmh

We consider the following cases for evaluation:

- Case T1-1: Training on data from Speed Scenario #1 and Testing on Data from Speed Scenario #1

- Case T1-2: Training on data from Speed Scenario #2 and Testing on Data from Speed Scenario #1

- Case T1-3: Training on a mixed set of data from Speed Scenario #1 and Scenario #2 and Testing on Data from Speed Scenario # 1

- Case T2-1: Training on data from Speed Scenario #2 and Testing on Data from Speed Scenario #2

- Case T2-2: Training on data from Speed Scenario #1 and Testing on Data from Speed Scenario #2

- Case T2-3: Training on a mixed set of data from Speed Scenario #1 and Scenario #2 and Testing on Data from Speed Scenario #2

Accuracy results are summarized in the table below.

Table 5: Temporal Prediction Accuracy of AIML-based Classification (UE Speed Generalization)

| Case | 1st Instant of Prediction Window |  |  | 2nd Instant of Prediction Window |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Top-1 Accuracy | Top-5/1 Accuracy | Top-1/5 Accuracy | Top-1 Accuracy | Top-5/1 Accuracy | Top-1/5 Accuracy |
| Case T1-1 | 60.51% | 95.35% | 76.05% | 61.29% | 95.35% | 77.02% |
| Case T1-2 | 30.21% | 63.40% | 59.92% | 22.64% | 55.15% | 48.85% |
| Case T1-3 | 60.51% | 95.69% | 76.55% | 59.61% | 94.91% | 75.77% |
| Case T2-1 | 50.32% | 86.20% | 65.58% | 58.05% | 89.70% | 74.47% |
| Case T2-2 | 21.22% | 52.00% | 47.79% | 16.67% | 41.70% | 39.53% |
| Case T2-3 | 49.96% | 87.29% | 65.30% | 58.26% | 90.27% | 74.89% |

Observation 23: Training an AIML model with examples from different UE speeds can help obtain a model that generalizes for temporal beam prediction for different UE speeds.

# Summary

In this contribution, we discuss evaluation methodology, KPI and target scenarios for AI/ML beam management and provide preliminary evaluation results for AI/ML aided beam prediction. Based on the discussions and evaluation results, we made the following observations and proposals:

Observation 1: Legacy beam management with Rel-17 without AI/ML algorithms is not an appropriate baseline as implementation-based AI/ML operation is available for UE and gNB implementations.

Observation 2: Beam information related KPI is not a measure to evaluate actual performance benefits from AI/ML based beam prediction, however, beam information related KPI can be used as a temporal measure to have an insight as beam information related KPI requires less computational overhead for evaluation.

Observation 3: Specification enhancements should be justified based on actual system performance benefits such as throughputs not based on beam related KPIs such as L1-RSRP difference.

Observation 4: UCI report overhead is not a metric for evaluation, but assumption for evaluation.

Observation 5: Option B (specific Rx beam) does not reflect actual benefits from AI/ML based beam prediction and misleads interpretation of evaluation metric for AI/ML based beam prediction.

Observation 6: KPIs related to comparing L1-RSRP difference with predicted L1-RSRP difference does not reflect the main focus of beam predictions, i.e., to select the Top-1 or Top-K best beams.

Observation 7: For different beam pattern type, each beam pattern type has its own pros and cons for performance, flexibility of inference input and range of inputs required for training.

Observation 8: Alt 2 (implicit information of Tx beam ID and/or Rx beam ID) can be applied for evaluation of Option 2: Set B is variable.

Observation 9: For specification impact, an efficient procedure for supporting Option 2: Set B is variable can be different considering number of beams to be reported and required overheads for reporting corresponding RSRP values.

Observation 10: Option D, i.e., selecting Set B out of measured beams (Set C), can be beneficial as Set B can be selected in a way for minimizing performance loss with reduced reporting overhead.

Observation 11: Performance of Option D, i.e., Set B is a subset of measured beams (pairs) Set C (including Set B = Set C), can vary depending on beam subset selection methods.

Observation 12: As using same beamwidth for all channels and signals is a general implementation within a frequency range, using a subset of Set A as Set B is a reasonable option if Set A and Set B are utilized in a same frequency range.

Observation 13: It is difficult to use a subset of Set A considering different beamwidths for beam management between different frequency ranges.

Observation 14: Utilization of wide beam information from a low frequency range has great potential as a low frequency range is more reliable and utilization of wide beam requires much less time and frequency resources for beam management.

Observation 15: Beam prediction accuracy marginally improves when the quantization step size used for beam measurement and reporting is reduced.

Observation 16: According to the current specifications, UE can only report L1-RSRP of up to maximum of 4 beams.

Observation 17: If L1-RSRP reporting mechanism in the current specification is extended to report large number of beams, reporting overhead is significantly increased.

Observation 18: Training an AIML model for a certain environment (e.g., UMa) and a certain UE dropping scenario for a different UE dropping scenario results in degradation in Top-1 accuracy by 4%-11%.

Observation 19: Training an AIML model for a certain environment (e.g., UMa) and a mixed data set from different UE dropping scenarios can be generalized for different UE dropping scenarios.

Observation 20: Training an AIML model with different beam patterns or random beam patterns can help obtain a model that generalizes over multiple beam patterns without significant degradation in prediction accuracy.

Observation 21: The more the quantization step is set, the more the degradation in prediction accuracy becomes.

Observation 22: Using 1 dB quantization step results in small degradation in accuracy by 3% compared to baseline (no quantization) for Top-1 accuracy and by <1% for Top-3/1 and Top5/1 accuracies.

Observation 23: Training an AIML model with examples from different UE speeds can help obtain a model that generalizes for temporal beam prediction for different UE speeds.

Proposal 1: ‘No collaboration framework: AI/ML algorithms purely implementation based and not requiring air-interface changes’ could be an appropriate baseline to accurately evaluate the benefits of AI/ML with specification enhancements.

Proposal 2: Support system performance related KPIs as mandatory KPIs.

- Support Avg. and 5% UE throughput for system performance KPIs.

Proposal 3: UCI report is reported as assumptions not as KPI.

Proposal 4: KPIs related to the difference between the estimated L1-RSRP and actual L1-RSRPs of the same beams get lower priority over beam ranking based KPIs (e.g., Top-1 (%), Top-K/1 (%), Top-1/K (%)).

Proposal 5: Support beam information related KPIs as optional for temporal measures.

- Support average L1-RSRP difference of Top-1 predicted beam in addition to the agreed options for beam prediction accuracy (%).

Proposal 6: For the definition of Top-1 genie-aided beam/beam pair, consider Option A, i.e., the Top-1 genie-aided Tx beam/beam pair is the Tx beam that results in the largest L1-RSRP over all Tx and Rx beams/beam pairs.

Proposal 7: Further study benefits of various beam pattern types.

Proposal 8: For evaluation, Alt 2 (implicit information) can be used. However, specification impact to support Option 2 should be further discussed considering reporting information overheads.

Proposal 9: Consider Option D, i.e., selecting Set B out of measured beams (Set C), to identify performance benefits and corresponding potential specification enhancements.

Proposal 10: Identify potential methods for selecting Set B out of measured beams (Set C) and evaluate benefits of the identified Set B selection methods to decide an optimized Set B selection method. Proposal 11: Support ‘Set B is a subset of Set A’ when Set A and Set B are utilized in a same frequency range for both temporal/spatial domain prediction.

Proposal 12: Support ‘Set A and Set B are different’ when Set A and Set B are utilized in different frequency ranges for both temporal/spatial domain prediction.

Proposal 13: AI/ML based beam management based on association between different frequency ranges should supported for both between FR1 and FR2-1 and between FR2-1 and FR2-2.

Proposal 14: Consider alternative beam reporting methods that can report L1-RSRP of more beams with minimized reporting overheads to support AI/ML based beam predictions.

Proposal 15: Identify and evaluate performance of potential beam reporting methods that can support large number of beams with minimized reporting overhead increment.

Proposal 16: For traffic model, support the following evaluation assumptions:

- For beam information related KPIs, no traffic model is needed to be defined as UE is only measuring reference signals not decoding actual PDSCHs.

- For system performance related KPIs, FTP traffic should be used to reflect practical traffics for the evaluation.

- For FTP traffic model, FTP model 3 is preferred as generating a new UE for each packet (FTP model 1) is not appropriate for evaluating benefits from AI/ML based beam prediction.

Proposal 17: For UE distribution, support the following evaluation assumptions:

- For FTP traffic model, 10 UEs per cell/sector with 50% and 70% RUs is preferred.

- 80% outdoor UEs and 20% indoor UEs for spatial domain beam prediction as defined in TR 38.901 (Option 1).

# References

- RP-213599, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”,

- RP-211662, “Moderator’s summary for discussion [RAN93e-R18Prep-12] AI/ML”,

- RP-212668, “Moderator’s summary for discussion [RAN94e-R18Prep-08] AI/ML for Air Interface”

- “Final Report of 3GPP TSG RAN WG1 #110bis-e”, 3GPP RAN1 Meeting #110bis-e, e-Meeting, October 10th – 19th, 2022,

- “Final Report of 3GPP TSG RAN WG1 #111”, 3GPP RAN1 Meeting #111, November 14th – 18th, 2022,

- “Final Report of 3GPP TSG RAN WG1 #112”, 3GPP RAN1 Meeting #112, February 27th – 3rd March 2022,

- TS 38.214, “Physical layer procedures for data (Release 17)”, v17.3.0, 2022-09.

# Appendix

## Evaluation assumptions

| Parameter | Values |
| --- | --- |
| Carrier Frequency | 30 GHz |
| Bandwidth | 80 MHz |
| Subcarrier Spacing | 120 kHz |
| Deployment | 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) with ISD = 200 m |
| Channel model | 5G-UMa (TR 38.901) |
| UE Model Parameters |  |
| UE Noise Figure | 10 dB |
| UE Antenna Configuration | (M, N, P, Mg, Ng) = (1, 4, 2, 1, 1), dH = 0.5λ |
| UE Dropping (Spatial Prediction) | - Scenario #1: 100% Outdoor UEs - Scenario #2: 20% Outdoor UEs and 80% Indoor UEs - UE Speed: 3 Km/h |
| UE Dropping (Temporal Prediction) | - 100% Outdoor UEs - UE Speed: 30 Km/h |
| UE mobility modeling (Temporal Prediction) | Procedure A from Section 7.6.3.2 in TR38.900 |
| UE Antenna Height | 1.5 m |
| gNB Model Parameters |  |
| gNB TX power | 23 dBm |
| gNB Antenna Configuration | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH=dV=0.5λ |
| gNB Antenna Radiation Pattern | 3-sector (120 degrees apart from each other) |
| gNB Antenna Height | 25 m |

## AI/ML Spatial Prediction Parameters

| Data Description |  |
| --- | --- |
| Complete Dataset Description | Each sample includes 64 RSRP measurements for beams from one sector to one UE |
| Input Dataset Description | Each sample includes 50%/25% of the RSRP measurements from complete dataset and 50%/75% missing inputs |
| Output Dataset Description | Best beam index |
| Beams Description | 64 Tx beams per sector per TRP. Beams are generated from the combination of beams at 16 azimuth angles and 4 elevation angles |
| Number of Samples | 21,000 Samples |
| Size of Training, Validation, and Testing Set | 70%, 10%, 20% of the dataset |
| ML Model Parameters |  |
| Model Description | A ResNet model to predict the best beam given the available RSRP measurements in the input dataset (Figure A.1) |
| Model Parameters | - Loss Function: Cross entropy - Optimizer: Adam |
| Evaluation | - Top-K Accuracy: If the best genie-predicted beam is among the top-K model-predicted beams - Top-K Reversed Accuracy: If the best model-predicted beam is among the top-K genie-predicted beams - Accuracy within X dB: If the best model-predicted beam is within X dB from the top-1 genie-predicted beam |

Figure A.1: ResNet Model for Spatial Beam Prediction

## AI/ML Temporal Prediction Parameters

| Data Description |  |
| --- | --- |
| Complete Dataset Description | Each sample includes 64 RSRP measurements for beams from one sector to one UE |
| Input Dataset Description (Observation Window) | RSRP measurements from observation window frames at time instants (Ti, Ti+1, Ti+2, …), where Ti+1 – Ti = 160 ms |
| Output Dataset Description (Prediction Window) | Best beam |
| Beams Description | 64 beams per sector per TRP. Beams are generated from the combination of beams at 16 azimuth angles and 4 elevations angles |
| Number of Samples | ~30K Samples |
| Size of Training, Validation, and Testing Set | 70%, 10%, 20% of the dataset |
| ML Model Parameters |  |
| Model Description | A transformer-based model to predict the best beam (or best k-beams) in each time frame of the prediction window (Figure A.2) |
| Model Parameters | - Loss Function: Aggregate cross entropy over the prediction window - Optimizer: Adam |
| Evaluation Metrics | - Top-K Accuracy: If the best genie-predicted beam is among the top-K model-predicted beams - Top-K Reversed Accuracy: If the best model-predicted beam is among the top-K genie-predicted beams |

Figure A.2: Transformer-based Model for Temporal Beam Prediction