3GPP TSG RAN WG1 #112bis-e			R1-2302827

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.11.1

Source:	InterDigital, Inc.

Title:	Discussion on evaluation on LP-WUS

Document for:	Discussion and Decision

# Introduction

In RAN#94-e [1], the study item for LP-WUS has been approved for NR. In this contribution, we discuss evaluation methodology, KPI and target scenarios for LP-WUS.

# Discussions

## KPIs

For KPIs, the following agreement was made in RAN#111 [4].

Agreement For system impact analysis, the following performance metrics are considered to be provided, Performance Metric; Note. System overhead; expressed as percentage of used part of all REs for LP-WUS (including guard band or time or others resource used for LP-WUR if any) among all resources Other assumptions related to the system overhead analysis can be reported, e.g., the LP-WUR raw data rate evaluated in the coverage evaluations.. Capacity impact; Evaluate the system capacity impact due to introducing of LP-WUS Note: it is for UEs which are in connected mode. Definition is the same as in XR TR.. FFS: NW power consumption / Energy Efficiency; [Impact of LP-WUS/WUR operation on gNB energy consumption as performance metric in system impact analysis.]. For power and latency evaluation of the LP-WUS, the following performance metrics definitions provided for future study Performance Metric; Note. Power consumption; Relative power consumption in units. The power consumption includes main radio and LP-WUR. For comparison, the relative power consumption and evaluation period for baseline schemes should also be provided, as well as the power saving gain (i.e., percentage of power consumption reduction of the proposed power saving scheme from the baseline scheme).. Latency; For IDLE/INACTIVE state, - the latency is the time interval between the data arrival time at the gNB and the time of the first PO UE can monitor the paging message - alternatively, if UE is not required to monitor a PO after wake-up, company to report detailed procedure and definition of the latency . In RAN1#111, there are no definitions being precluded - sync/re-sync for main radio is included. UPT; The definition is the same as in [TR38.840] Note: it is for connected mode purpose.. Companies to report baseline scheme, e.g., PO monitoring with i-DRX, e-DRX, with or without PEI Companies to report the power consumption / power saving gain considering the FAR impact, latency considering MDR impact Other performance metrics (e.g., mobility) can be reported by companies (if any)

In RAN1#111 [4], Capacity impact and UPT were agreed as performance metrics while NW power consumption/energy efficiency was still remained as FFS. Although power consumption and energy efficiency are crucial factors, motivation to adopt NW power consumption and energy efficiency as a performance metric is not clear. For example, as deployment scenarios and implementations can be different for each NW vendor, it is difficult to have common understanding on the NW power consumption and energy efficiency.

Proposal 1: NW power consumption/energy efficiency is not adopted as a performance metric.

## Evaluation methodology

In this section, we provide our view on evaluation methodology for LP-WUS.

### Power model for main radio

In RAN1#110bis-e [3], the following agreement was made for power model of ultra-deep sleep state for main radio.

Agreement - The following power models are used ‘Ultra-deep sleep’ power state for main radio for evaluation Power State; Relative Power (unit); Ramp-up and down transition energy (Note1): (unit multiplied by ms); Ramp-up time; Time for sync/re-sync. Ultra-deep sleep; [0.015]; [2000 ~ 40000] - Study to converge on candidate numbers to use for evaluation - FFS: other values and reported by companies. - FFS: down-selection of the values, - companies are encouraged to provide details for down-selection; [400ms], FFS: 100ms; X. Note1: - Ramp-up time may consist of the procedure for [main radio hardware tune on e.g., boot, memory load and etc.], - Time for sync/re-sync consists of the procedure for [main radio to re-synchronization with the serving gNB etc.], - FFS: X and whether/how to have different values depending on other factors, e.g., signal-to-noise ratio - Companies can report the assumption of X in the initial evaluation. - Ramp up and down energy includes power for ramp-up and ramp-down. Energy consumption for sync/re-sync is separately calculated. - The total time for main radio transition from ultra-deep sleep to active/micro sleep state is the sum of ramp-up time and time for sync/re-sync. - FFS whether/how to define ramp-down time, whether to separately describe the ramp-down energy consumption Note 2: the power state transitions in this table refer to transitions between ultra deep sleep state and active / micro sleep state. Note 3: The values inside of ‘[ ]’ are to be used as starting point of future study on LP-WUS

Based on the agreement, we provide the following discussions:

Relative power

The relative power value 0.015 was proposed based on the agreed relative power unit of Low Power High Accuracy Positioning (LPHAP) evaluation. From the two options of LPHAP, the first option was used as the first option assumes a universal usage of normal UEs while the second option is assuming UEs with only positioning functionality. In RAN1#110bis-e [3], the following additional agreement was made for LPHAP:

Agreement For the LPHAP study only: - For the power consumption model of the ultra-deep sleep type, adopt the following option (i.e. revision of option 1 from previous agreement): - The relative power unit: 0.015 - Additional transition energy: 10000 - Note: Power consumption analysis from individual companies with additional transition energy of 5000 can be optionally evaluated and captured in the TR. - Total transition time: 400ms - Note: Power consumption analysis from individual companies with Option 2 (revised from previous agreement) can be optionally evaluated and captured in the TR. - Option 2 additional transition energy is revised from 450 to 480. - Note: No new device type is expected based on ultra-deep sleep power modeling.

As the relative power value for LPHAP is confirmed as shown in the above, the relative power value for ultra-deep sleep should be confirmed as well.

Proposal 2: Confirm the relative power value 0.015 for Ultra-deep sleep of MR.

Ramp-up/down transition energy and ramp-up time

In RAN1#112 [5], the following agreement on ramp-up/down transition energy and ramp-up time was made:

Agreement For evaluation, at least for FR1 MR ultra-deep sleep state, (Ramp-up and down transition energy, ramp-up time) is as follows, - Alt 1: (15000, 400ms) - Alt 2: ([40000], [800ms]) Company to report which alternative they use for which use cases.

As depicted above, two alternatives were agreed for (ramp-up and down transition energy, ramp-up time). The original intention for supporting two alternatives to support different use cases. For example, while Alt 1 can be used for IoT use case, Alt 2 can be used for eMBB and wearable use cases. However, the reason for supporting different ramp-up/down transition energy and ramp-up time is not clear. For example, supporting longer ramp-up time for eMBB case is not reasonable as eMBB UEs generally have more complex implementation than IoT use case. In addition, as the additional transition energy for LPHAP was agreed as 10000, ultra-deep sleep state with 15000 for ramp-up/down transition energy already provides enough degree of energy consumption for ramp-up and down in ultra-deep sleep state.

Proposal 3: For ramp-up/down transition energy and ramp-up time of MR, support Alt 1 (15000, 400ms) and deprioritize Alt 2 ([40000], 800ms)).

Total time for sync/re-sync

The following agreement on time for sync/re-sync was made in RAN1#111 [4]:

Agreement For MR, at least for FR1 evaluation, - Number of SSBs for sync/re-sync for MR is up to 10 - Companies to report timeline and energy consumption Companies to provide feasibility analysis for transition time and transition energy with aim to converge to one or two set of values in RAN1#112

For total time for sync/re-sync, two alternatives were discussed in RAN1#111. The first alternative is to support 3 – 10 SSBs and the second alternative is to support 1 – 3 SSBs as total time for sync/re-sync. If main radio is totally powered off and needs to achieve time and frequency synchronization the first alternative (3 – 10 SSBs) can be a reasonable choice. On the other hand, there can be potential implementations which possibly utilize LP-WUS as a reference. In this case, supporting 1 – 3 SSBs can be a possible option. In that regard, supporting up to 10 SSBs was agreed as total time for sync/re-sync at least for FR1. In our view, keeping the same option for FR2 is a reasonable choice.

Proposal 4: For total time for sync/re-sync of MR, support up to 10 SSBs for FR2 as well as FR1.

### Power model for LP-WUR

In RAN1#111 [4], the following agreement was made for ultra-deep sleep state.

Agreement The following power model for LP-WUR is used for evaluation for FR1, Power State; Relative Power (unit); Transition energy: (unit multiplied by ms); Ramp-up time TLR, ramp-up (ms). Off; 0.001; [TLR, ramp-up *(PON+POFF)/2]; TLR, ramp-up = FFS, and company to report TLR, ramp-up FFS: Relation between Receiver architecture and its relative power and value of TLR, ramp-up. On; 0.005/0.01/0.02/0.03/0.05/0.1/0.2/0.5/1/2/4 FFS: If other values are needed. FFS: whether further categorization/sub-categorization is needed and how.

Transition energy

In RAN1#112 [5], a proposal to update transition energy as TLR, ramp-up *(PON – POFF)/2. While the current transition energy equation is a value based on average of on state power and off state power multiplied by ramp up time, the proposal is to define the transition energy as additional energy on top of the off state. In our understanding, the transition energy means total energy during the ramp-up time not the additional energy consumption based on the off state.

Proposal 5: Confirm the current definition of transition energy (i.e., TLR, ramp-up *(PON – POFF)/2) for LP-WUR.

Relative power for LP-WUR on state

In the receiver architecture discussions for LP-WUR, the three candidate receiver architectures were agreed:

Agreement Study at least the following three types of receiver architectures for LP-WUR: - Architecture with RF envelope detection - Heterodyne architecture with IF envelope detection - Homodyne/zero-IF architecture with baseband envelope detection - Note: The details of each type of receiver architecture are discussed separately. - Note: Above receiver architectures are considered suitable for OOK modulation. Some of the architectures can be applicable for other modulations such as FSK.

To reflect the agreed receiver architectures, multiple categories of relative power unit for LP-WUR on state should be introduced. For example, the following categories can be a starting point for further discussion with potential down selection:

- Cat 1: 0.01 and 0.05

- Cat 2: 0.1 and 0.5

- Cat 3: 1, 2 and 4

Proposal 6: For relative power unit of LP-WUR on state, define three categories of candidate values. The candidate values for each category can be further discussed and the following categorization can be a starting point.

- Cat 1: 0.01 and 0.05

- Cat 2: 0.1 and 0.5

- Cat 3: 1, 2 and 4

### Noise figure for LP-WUR

As well as the relative power unit, multiple categories of noise figure assumption should be defined based on the candidate receiver architectures. In RAN1#110bis-e [3], the following candidate noise figure values were agreed:

Agreement - For LP-WUS coverage evaluation, the noise figure of LP-WUR is - Options : [9, 12, 15, 18, 21, 24], Other values can be reported by companies - FFS: how to determine the NF option. - The values provided is for the purpose of studying coverage of LP-WUS, and it can be further revisited depending on the receiver architecture discussion.

Based on the agreed values for noise figure, the following categorization can be a starting point point for further discussion with potential down selection:

- Cat 1: 9 and 12 dB

- Cat 2: 15 and 18 dB

- Cat 3: 21 and 24 dB

Proposal 7: For noise figure of LP-WUR on state, define three categories of candidate values. The candidate values for each category can be further discussed and the following categorization can be a starting point.

- Cat 1: 9 and 12 dB

- Cat 2: 15 and 18 dB

- Cat 3: 21 and 24 dB

### MDR/FAR assumption

For MDR/FAR assumption, the following conclusion was made in RAN1#112 [5]:

Conclusion: The FAR definition does NOT include the impact of the falsely alarmed for wake-up due to the detection of a LP-WUS which is intended to wake-up/alarm the LP-WUR of another UE within the same UE group

In addition, the following assumption was agreed for link level evaluation:

- The miss-detection rate (MDR) of LP-WUS [1%] 1%, - The false-alarm rate (FAR) of LP-WUS - [0.1%, 1%, 10%] - Other values are not precluded for studying, reported by companies - Further discuss on the following alternatives for FAR target is determined - Alt 1: FAR target is determined per single WUS attempt/trial, - FFS: Alt 2: FAR target is determined across a reference time duration of one or multiple WUS attempts/trials - Length of reference time duration: [0.32s, 1.28s,10.24s, 20.48s, 2621.44s,10485.76s], other values not precluded - FFS: possible values for reference time durations - Companies to report details of receiver behaviour, e.g., receiver behaviour, how to compute MDR, detection threshold - Companies to report the selected reference time duration values and the associated number of WUS attempts/trials

FAR values

As depicted in the above, 2 target FAR values were agreed with brackets. In addition, 10% FAR was proposed, however, 10% FAR value does not provide reliable operations for activating/deactivating main radio. In that regard, it is preferred to confirm the agreed values for link level evaluation.

Proposal 8: For target FAR values, confirm the agreed values for link level evaluation.

Definition of FAR target

Two alternatives, FAR target per single WUS attempt and FAR target across a reference time duration, were agreed for FAR target definition. While Alt 1 provides a simple FAR target per single WUS attempt/trial, Alt 2 defines an FAR target across a reference time duration. In our view, Alt 2 requires further discussion on possible values for reference time durations as captured as FFS, benefits of Alt 2 are not clear.

Proposal 9: For target FAR definition, support Alt 1 (FAR target is determined across a reference time duration).

### LP-WUR frequency and time errors

For LP-WUR frequency and time errors, the following working assumption was made in RAN1#112 [5]:

Working assumption: - For evaluation of LP-WUR frequency and time errors, the following is used, Parameter; Value. LO XO Oscillator max frequency error [ppm], LO XO Oscillator frequency drift [ppm/s]; option 1: (200, 0.1) option 2: (50, 0.1) option 3: (10, 0.05) option 4: (5, 0.05) Other values are not precluded for studying, reported by companies. RTC max frequency error [ppm]; 20. - Company to report how to use the clocks for LR on/off states - The above clock assumptions for LR assumes the MR is in ‘ultra-deep sleep’ power state. - For Option ¾, cannot be used - FFS applicability when MR is in ultra-deep sleep power consumption state and associated power consumption for LR on state and LR off state, - e.g., option ¾ is not applicable - when MR is in ULPS’ultra-deep sleep state’ with [0.015] power units and LR is in off state or, - when LR monitoring power less than 1 [TBD] power unit, - Note: Assumptions important for achieving performance by option ½/3/4 clock for LR should be declared, including active on/off power, transition energy/ ramp-up time TLR, ramp-up for LR and etc. - If MR is in other state than ULPS’ultra-deep sleep state’(e.g., ‘deep sleep’), the clock running for MR can be used for LR. - assumptions important for achieving performance by using MR clock for LR should be declared - Other clock accuracy options are not precluded. Companies to report options based on a feasibility analysis of clock power consumption and UE power consumption to use the clock accuracy option - Company to report the initial/residual frequency error assumption for the detection of LP-WUS/synchronization signal, - The following are examples for consideration, other approaches are not precluded, - Model 1: - The relationship between a drifted frequency error(ΔF), frequency drift ( F’) over a time (T1) is ΔF = ±F’ * T1+Fr - When ΔF frequency displacement [Fd] reaches max frequency error, it is assumed to be equaled to max frequency error - T1 is the time from the previous frequency synchronization. T1 may take different values depending on the chosen frequency synchronization approach. - Fr is residual frequency error after the previous frequency synchronization. - [Fr=0.] - FFS: [Fr>0 and explanation of its modeling and how it is obtained at each clock synchronization/calibration] - If residual frequency error after the previous synchronization is justified to be zero or marginal compared with Fe should be declared. - FFS: Frequency displacement (Fd), before detection of a current sync signal defined as the difference between ideal frequency and frequency due to 1) clock drifting (ΔF); and 2) residual frequency error from previous sync signal detection synchronization/calibration (Fr), is given as Fd (ppm)=ΔF (ppm) +Fr(ppm). - Model 2: random frequency drifting, FFS details - Company to report the timing drifting error assumption for the detection of LP-WUS/synchronization signal, - The following are examples for consideration, other approaches are not precluded, - Model 1 [R1-2301438] [R1-2301558][R1-1714993]: - The relationship between the maximum frequency error(Fe) and corresponding timing drift( ΔT) over a time(T) is ΔT = ±Fe * T (saturated liner region) - The relationship between a frequency drift( F’), and corresponding timing drift(ΔT) over a time(T) is ΔT = ±0.5 * F’ * T2 ΔT = Fr*T ±0.5 * F’ *T2 (transient region) - The transition between transient and saturated linear region (from synchronization or calibration point/time) occurs at time Ts= [Fe /( 0.5F’)] [Ts= (Fe-Fr)/( F’)] - T is the time from the previous time synchronization. T may take different values depending on the chosen synchronization approach - FFS: Fr ≠ 0, e.g., ΔT = Fr*T ±0.5 * F’ *T2 , Ts= (Fe∓Fr)/( 0.5F’) - FFS: Time error (Te) before detection of a current sync signal is defined as the difference between ideal time of the current sync signal and the time error due to 1) clock time drift (ΔT); and 2) residual time error from previous sync signal detection synchronization/calibration (Tr); Te= ΔT+ Tr - Model 2: random time drifting, FFS details - FFS: Phase noise model

Applicability of agreed power consumption assumptions for Option 3/4

In RAN1#112 [5], whether to apply agreed power consumption assumptions for Option 3/4 was discussed. For high frequency error/drift case, there may be increased power consumption and therefore the agreed power consumption for MR and LP-WUR may not be applicable. However, there is no clear evidence that only Option 3/4 are not applicable with the current assumption. In that regard, applying the current assumptions for Option 3/4 as well as Option 1/2 is preferred.

Frequency error and time drifting error assumptions

For both frequency error and time drifting error, two models were agreed. While model 1 for frequency error and time drifting error provides traditional models with details, model 2 introduces random time/frequency drifting models without details. As the traditional models have been used for various evaluations, it is preferred to support model 1 unless significant issues are identified.

Except the issues discussed in the above, we failed to find other potential issues for confirming the working assumption. Given the situation, it is preferred to confirm the working assumption without FFS bullet on applicability of the agreed power assumptions for Option ¾ and support of model 2 for both frequency error and time drifting error assumptions.

Proposal 10: Confirm the working assumption on time/frequency errors with the following updates.

- Remove the FFS bullet on applicability of the agreed power assumptions for Option 3/4.

- Remove model 2 for both frequency error and time drifting error assumptions.

# Summary

In this contribution, we discuss KPIs and evaluation methodology for LP-WUS. From the discussions, we made the following proposals:

Proposal 1: NW power consumption/energy efficiency is not adopted as a performance metric.

Proposal 2: Confirm the relative power value 0.015 for Ultra-deep sleep of MR.

Proposal 3: For ramp-up/down transition energy and ramp-up time of MR, support Alt 1 (15000, 400ms) and deprioritize Alt 2 ([40000], 800ms)).

Proposal 4: For total time for sync/re-sync of MR, support up to 10 SSBs for FR2 as well as FR1.

Proposal 5: Confirm the current definition of transition energy (i.e., TLR, ramp-up *(PON – POFF)/2) for LP-WUR.

Proposal 6: For relative power unit of LP-WUR on state, define three categories of candidate values. The candidate values for each category can be further discussed and the following categorization can be a starting point.

- Cat 1: 0.01 and 0.05

- Cat 2: 0.1 and 0.5

- Cat 3: 1, 2 and 4

Proposal 7: For noise figure of LP-WUR on state, define three categories of candidate values. The candidate values for each category can be further discussed and the following categorization can be a starting point.

- Cat 1: 9 and 12 dB

- Cat 2: 15 and 18 dB

- Cat 3: 21 and 24 dB

Proposal 8: For target FAR values, confirm the agreed values for link level evaluation.

Proposal 9: For target FAR definition, support Alt 1 (FAR target is determined across a reference time duration).

Proposal 10: Confirm the working assumption on time/frequency errors with the following updates.

- Remove the FFS bullet on applicability of the agreed power assumptions for Option 3/4.

- Remove model 2 for both frequency error and time drifting error assumptions.

# References

- RP-213645, “Study on Low-power Wake-up Signal and Receiver for NR”,

- 3GPP TR 38.840 (v16.0.0): “Study on User Equipment (UE) power saving in NR”,

- “Final Report of 3GPP TSG RAN WG1 #110bis-e”, 3GPP RAN1 Meeting #110bis-e, e-Meeting, October 10th – 19th, 2022,

- “Final Report of 3GPP TSG RAN WG1 #111”, 3GPP RAN1 Meeting #111, November 14th – 18th, 2022,

- “Final Report of 3GPP TSG RAN WG1 #112”, 3GPP RAN1 Meeting #112, February 27th – March 3rd, 2023.