3GPP TSG RAN WG1 #112bis-e                                             R1-2302836

Meeting，April 17th – April 26th, 2023

Source:          TCL Communication

Title:            Discussion on XR specific capacity enhancement techniques

Agenda item:     9.8.1

Document for:    Discussion and Decision

- Introduction

In the RAN #98e-meeting [1], the WID for XR capacity enhancement has been approved, and the scopes for XR were shown in the following:

Specify the enhancements related to power saving: - DRX support of XR frame rates corresponding to non-integer periodicities (through at least semi-static mechanisms e.g. RRC signalling) (RAN2). Specify the enhancements related to capacity: - Multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration (RAN1, RAN2); - Dynamic indication of unused CG PUSCH occasion(s) based on UCI by the UE (RAN1); - BSR enhancements including at least new BS Table(s); (RAN2); - Delay reporting of buffered data in uplink; (RAN2); - Provision of XR traffic assistance information for DL and UL (e.g. periodicity); (RAN2); - Discard operation of PDU Sets (RAN2); Specify the enhancements for XR Awareness (RAN2, RAN3): TBD (detailed objectives will be further clarified at RAN#99 based on the conclusions of TR38.835, and work to be started only after RAN#99)

In last RAN1 meeting [2], several issues for XR specific capacity have been discussed and the following agreements were made:

Agreement

- Multi-PUSCHs CG is supported for Type-1 configured grant.

- Multi-PUSCHs CG is supported for Type-2 configured grant.

Agreement

For determination of the time domain resource allocation of CG PUSCHs associated to a multi-PUSCHs CG, the following alternatives for further study:

- Alt-A: TDRA determination based on repetition framework.

  - Alt-A1: Follow the time domain resource mapping of Type A repetition

    - N configured by higher layers or indicated by activation DCI

    - Single SLIV is determined from TDRA

    - The same SLIV in N PUSCH in consecutive slots per CG period

      - FFS for non-consecutive slots

    - FFS details, including related RRC parameters

  - Alt-A2: Follow the time domain resource mapping of Type B repetition

    - N configured by higher layers or indicated by activation DCI

    - Single SLIV is determined from TDRA

      - The SLIV used for 1st PUSCH per CG period.

    - N consecutive nominal PUSCHs with same duration per CG period

    - Note: N is not necessarily the repetition factor.

FFS details, including related RRC parameters

- Alt-B: TDRA determination based on NR-U framework

    - N and M configured by higher layers

    - Single SLIV is determined from TDRA.

      - The SLIV used for 1st PUSCH per CG period.

    - M consecutive PUSCH TOs with same duration in slot. The M PUSCH TOs are used in N consecutive slots per CG period

    - Note: N and M are configured independently from cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, respectively. M and N configuration is independent from cgRetransmissionTimer configuration.

    - FFS details, including related RRC parameters

- Alt-C: TDRA determination based on single DCI scheduling multiple PUSCHs

  - Alt-C1: Follow Rel-16 single DCI scheduling multiple PUSCHs

    - TDRA configured by pusch-TimeDomainAllocationListForMultiPUSCH-r16 with k2-r16

    - A row of TDRA with N entries determines the time domain resources allocation of N PUSCH TOs per period

      - Note: N PUSCH TOs should be consecutive PUSCH TOs in consecutive slots.

    - FFS details, including related RRC parameters

  - Alt-C2: Follow Rel-17 single DCI scheduling multiple PUSCHs

    - TDRA configured by pusch-TimeDomainAllocationListForMultiPUSCH-r16 with extendedK2-r17

    - A row of TDRA with N entries determines the time domain resources allocation of N PUSCH TOs per period

      - Note: N PUSCH TOs can be non-consecutive PUSCHs and/or in non-consecutive slots.

    - FFS details, including related RRC parameters

Agreement

For the PUSCHs parameters in a multi-PUSCHs CG configuration, the configuration/indication parameters except MCS and FDRA of CG PUSCHs in a multi-PUSCHs CG configuration are the same

- FFS: For MCS and FDRA, study further to decide whether/how to be different.

- FFS: Applicability to type-1 and type-2

- Note: TDRA and HP ID are not in this scope of the above statement.

Agreement

For determination of HARQ process IDs associated to PUSCHs in multi-PUSCHs CG assuming one TB per PUSCH, consider the following alternatives:

- Alt. 1:  The HARQ process ID for the first configured/valid PUSCH in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured, and applying "the period duration divided by X instead of the period duration.

  - The HARQ process ID of the remaining PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period.

  - Alt 1-1; X = 1

  - Alt 1-2: X is the number of configured PUSCHs in a period

  - Alt 1-3: X is provided by RRC configuration.

  - FFS details

- Alt. 2: Support that UE can decide, as in NR-U, the HARQ IDs for the multiple CG PUSCH transmission occasions and indicate the decided HARQ IDs to gNB if multiple HARQ processes are used for the multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration

  - FFS details

- Alt. 3: The HARQ process ID for the configured PUSCHs in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured.

  - FFS on potential enhancements different from previous alternatives

  - Alt 3-1: Note: Same HP ID would be used for all PUSCHs within a period.

    - FFS details

  - Alt 3-2: Note: Different HP ID could be used for all PUSCHs within a period.

    - FFS details

- Alt. 4:  The HARQ process ID for the first configured/valid PUSCH in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured.

  - The HARQ process ID of the remaining PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period

  - FFS on potential enhancements different from previous alternatives

- Alt 5: Support that UE can decide, as in NR-U, the HARQ IDs for the first CG PUSCH transmission occasions and indicate the decided HARQ IDs to gNB if multiple HARQ processes are used for the multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration

  - The HARQ process ID of the remaining PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period

  - FFS details

- Alt 6: FFS other solutions

Note: The case of one TB map to multiple PUSCHs is not considered here.

Agreement

The physical channel that carries the UCI that provides information about unused CG PUSCH transmission occasions is CG PUSCH.

Agreement

Encoding and multiplexing for “the UCI that provides information about unused CG PUSCH transmission occasions” in a CG PUSCH applies encoding and multiplexing procedures for CG-UCI as baseline.

- FFS on details

Agreement

Consider the following alternatives for “the UCI that provides information about unused CG PUSCH transmission occasions” for down-selection or revision

- Alt. 1: “The UCI that provides information about unused CG PUSCH transmission occasions” is defined as a new UCI.

  - FFS on details

- Alt. 2: “The UCI that provides information about unused CG PUSCH transmission occasions” is added as new field(s) to the CG-UCI.

  - FFS on details

- Alt. 3: “The UCI that provides information about unused CG PUSCH transmission occasions” replaces/re-purposes some field(s) of the CG-UCI.

  - FFS on details

Agreement

For dynamic indication of unused CG PUSCH occasion(s) based on a UCI, the following options for further down-scoping with possible revision, are considered for the transmission occasion of the UCI:

- Option 1: A transmitted CG PUSCH, includes the UCI.

  - FFS details

- Option 2: A transmitted CG PUSCH includes the UCI, if it is transmitted in an occasion determined by RRC.

  - FFS details

- Option 3: A transmitted CG PUSCH includes the UCI, if it is transmitted in a pre-defined transmission occasion.

  - FFS details

    - Example of a pre-determined occasion: 1st configured PUSCH TO in a CG period or 1st configured PUSCH TO in a multiple CG periods

- Option 4: A transmitted CG PUSCH includes the UCI, if it is transmitted in a transmission occasion determined satisfying given condition(s).

  - FFS details

    - Examples of a condition: A first transmitted PUSCH in a CG period, or a first PUSCH transmission within a multiple of CG periods.

Other options are not precluded. Proponent companies to provide details.

Agreement

For dynamic indication of unused CG PUSCH transmission occasion(s) based on a UCI, the following options for further down-scoping, are considered for the information provided by the UCI:

- Option 1: The UCI determines the consecutive CG PUSCH TO(s) that are indicated as “unused”

  - Option 1-1: The UCI provides the number of consecutive TO(s) in time domain.

    - Applicable numbers can be determined from information obtained from configuration.

    - FFS details

  - Option 1-2: The UCI provides a time duration/range that includes the consecutive TO(s) in time domain.

    - Applicable time duration/range can be determined from information obtained from configuration

    - FFS details

- Option 2: The UCI determines the CG PUSCH TO(s) that are indicated as “unused” (consecutive/non-consecutive TO(s) in time domain)

  - Option 2-1: The UCI provides a bitmap where a bit corresponds to a TO within a time duration/range. The bit indicates whether the TO is “unused”.

    - Applicable time duration/range can be determined from information obtained from configuration

    - FFS details

  - Option 2-2: The UCI provides a bitmap where a bit corresponds to TOs within a time duration/range. The bit indicates whether all TOs within the time duration/range are “unused”.

    - Applicable time duration/range can be determined from information obtained from configuration

    - FFS details

- FFS whether/how the unused TO(s) can be associated to multiple CG configuration.

- Other options are not precluded. Proponent companies to provide details.

In this contribution, we provide our views on further XR specific capacity enhancements techniques.

- Discussion

Traffic characteristics of XR services were intensively studied in the Rel-17 RAN1 meetings, and the DL/UL traffic models for the video stream and Pose/control for XR and cloud gaming services were agreed. Based on the outcomes of the studies so far, the following characteristics [3] of the XR traffic are identified and should be considered for the potential enhancements of NR to support XR.

- Non-integer periodicity

In RAN1 meetings of Rel-17, it was agreed that 60 frame per second (fps) is baseline for both DL and UL video stream and 30 fps, 90 fps as well as 120 fps can be also optionally evaluated. Based on the arrival time of packet with 30 fps, 60fps, 90fps, 120fps per second, the corresponding periodicities are {33.33ms, 16.67ms, 11.11ms, 8.33ms} respectively. It’s not an integer periodic of symbol or slot.

- Jitter of packet arrival time

As agreed in RAN1 meeting in Rel-17, a truncated Gaussian distribution is used to model the jitter of DL and UL video stream for XR services. The range of jitter is agreed to be [-4, 4]ms (baseline) and [-5, 5]ms (optional). This means the XR packets may arrive at gNB or UE within a time window of total 8ms or 10ms length, and the exact arrival time is not known in advance.

- Low latency and large packet size

To provide good experience of XR, the latency of XR traffic should be as low as possible. As agreed in RAN1 meetings in Rel-17, air interface PDB for DL video stream is as follows.

VR/AR

  - 10ms

CG

  - 15ms

According to the agreed traffic model in Rel-17, mean packet size is very large. Taking AR/VR 60Mbps as example, mean packet size is 125000 bytes. This is very different from R-15/6 URLLC or eMBB services. For URLLC, one of the most challenge is latency, and for eMBB, one of the most challenge is high transmission data. However, for XR services, both high transmission rate and low latency should be satisfied.

- Varying packet size

In Rel-17 RAN1 meeting, parameters of Truncated Gaussian distribution for packet size of DL video stream in case of single stream evaluation has been agreed and is shown below. We can observed that the packet size of XR is not fixed.

- [STD, Max, Min]: [10.5, 150, 50]% of Mean packet size

- Multiple flows

For an XR application, there might be multiple data streams. Multiple streams may have different traffic characteristics, requirements and priorities.

Observation 1: XR services have the following characteristics.

- The non-integer periodicity

- Jitter of packet arrival time

- Low latency and large packet size

- Varying packet size

- Multiple flows

CG enhancements for XR

- Time resource for multiple TOs configured within a CG

It’s has been agreed that multiple PUSCH transmission occasions (TOs) can be configured into a CG configuration to support large packet of XR, the time domain resource allocation (TDRA) for the TOs can refer to current mechanism of resource allocation of PUSCH repetition type A or type B, which can be called as repetition type A like or type B like TDRA for multiple TOs, respectively.

- For the repetition type A like TDRA, up to one TO within a slot and the same time resources allocation is expected for each slot. In other words, the occupied resources within each TOs is expected to have the same starting position and duration length.

- For the repetition type B like TDRA, a set of consecutive symbols is allocated to the multiple TO within a CG configuration, where only the valid UL symbols within these consecutive symbols are used for nominal or actual PUSCH transmission.

Which type allocation in time domain is suitable for multiple TOs configured within a CG for XR? In our views, PUSCH repetition type B like time domain allocation is useful for low latency by allowing back-to-back transmissions in one slot. Moreover, the back-to-back transmissions can also be better to use as many available symbols as possible for uplink transmission, especially for the specific slots in TDD. Thus, the repetition type B like TDRA can be used for multiple TOs configured within a CG period.

Proposal 1: Follow the time domain resource mapping of Type B repetition can be used for multiple TOs within a CG configuration.

- Single SLIV is determined from TDRA

- The SLIV used for 1st PUSCH per CG period.

- N consecutive nominal PUSCHs with same duration per CG period

- Indicate the number of TOs within a CG configuration

In current specification, a CG configuration supports single PUSCH transmission with or without repetitions, when a CG configured with repetition and the number of repetition is large than 1, then multiple transmission occasions can be used for transmitting a TB and the corresponding repetitions can be with a same or different RV(s). In Rel-15, semi-static configured the number of repetition (e.g. Rep k) for CG is supported, to provide the better scheduling flexibility, dynamic indication the number of repetition is also supported for CG in Rel-16 through adding a new column in TDRA. Similar rules can be used to indicate the number of TOs within a CG configuration, e.g. a new column can be added into the TDRA table, when the DCI indicates a row containing the number of multiple TOs, the granted transmission corresponds to the multiple TOs transmission within a CG configuration.

Proposal 2: The number of multiple TOs within a CG configuration can be indicated by TDRA.

In addition, for XR service, high reliability is also a key requirement, to improve the reliability of XR traffic, one of the most straightforward way is enabled repetition of multi-PUSCHs within a CG configuration. As a result, repetition for multiple PUSCHs transmission within a CG configuration need to be considered.

Proposal 3: Repetition for multi-PUSCHs transmissions within a CG configuration can be supported.

- HARQ-ID for multiple TOs within a CG configuration

In current spec, the HARQ-ID of a CG configuration is calculated based on the time location of the first transmission occasion, as the following formula:

HARQ Process ID = [floor(CURRENT_symbol / periodicity)] modulo nrofHARQ-Processes + harq-ProcID-Offset

where CURRENT_symbol = (SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + slot number in the frame × numberOfSymbolsPerSlot + symbol number in the slot), and numberOfSlotsPerFrame and numberOfSymbolsPerSlot refer to the number of consecutive slots per frame and the number of consecutive symbols per slot, respectively as specified in TS 38.211.

In rel-15/16, PUSCH repetitions allow a single transport block transmission over one or multi-slots with a shared HARQ-ID. However, when more than one transport blocks can be allowed to transmit over multiple TOs within a CG configuration, the shared HARQ-ID for multiple TOs within a CG will be leaded ambiguity between gNB and UE. As a results, the HARQ-ID between multiple TOs within a CG configuration need to be determined independently.

To determine the HARQ-ID for multi-PUSCHs within a CG configuration, in our views, the current mechanism can be reused as much as possible to mitigate the impact of spec. A reference HARQ-ID can be calculated based on current formula defined in TS 38.321, then the HARQ process ID of the remaining TOs within the CG period is determined by incrementing the HARQ process ID of the preceding TO in the period.

Proposal 4: For XR, when multiple TOs within a CG is configured, the HARQ-ID for the first configured/valid PUSCH within a CG configuration is determined based on current mechanism which is defined in TS 38.321, then the HARQ-ID of the remaining TOs within a CG period is determined by incrementing the HARQ-ID of the preceding TO within a CG configuration.

UCI using to indicate the unused TOs within a CG configuration

As the XR video traffic is generated periodically and with low latency and vary packet size, to support XR traffic by CG is one straightforward way to reduce the scheduling delay thanks to no SR and BSR procedure. For XR video with large packet, a PUSCH transmission occasion within a CG configuration is not enough to transmit a whole packet, in addition, as the CG resources is semi-statically configured to UE, due to lack information of the XR packet, then gNB need to configure enough number of TOs within a CG period to make sure the XR packet can be carried whole, however, this may be lead resources oversupplied. To avoid resource waste, the un-used TOs within a CG configuration can be indicated to gNB, then the resources reserved for the un-used TOs can be allocated to other UEs. There are two cases which the UE need to indicate the unused TOs to gNB.

Case 1: A set of consecutive/non-consecutive unused TOs are located in the front of a CG period. As XR traffic is pseudo-period arrival, the CG configuration should be matched with the period of XR, however, due to jitter, the traffic arrival of XR will be late than the expected TOs within a CG period, as show in figure 1. 4 PUSCHs in one CG period are configured, and during the period P2, the packet arrival at the third TO when jitter happened, as a results, the front 2 consecutive TOs within CG period P2 are unused TOs.

Figure 1. Variable size and jitter for XR frame size with CG

Case 2: A set of consecutive/non-consecutive unused TOs are located in the end of a CG period, as show in figure 1. 4 PUSCHs in one CG period are configured, during the period P3, 2 TOs are enough to carry the packet of XR, then the remaining 2 TOs are unused TOs.

Proposal 5: Both a set of unused TOs located in the front and the end of a CG configuration can be considered for CG enhanced for XR.

For the unused TOs within a CG configuration indication, in the last RAN1 meeting, it has been discussed and the following potential ways has been proposed:

- Option 1: The UCI determines the consecutive CG PUSCH TO(s) that are indicated as “unused”

  - Option 1-1: The UCI provides the number of consecutive TO(s) in time domain.

  - Option 1-2: The UCI provides a time duration/range that includes the consecutive TO(s) in time domain.

- Option 2: The UCI determines the CG PUSCH TO(s) that are indicated as “unused” (consecutive/non-consecutive TO(s) in time domain)

  - Option 2-1: The UCI provides a bitmap where a bit corresponds to a TO within a time duration/range. The bit indicates whether the TO is “unused”.

  - Option 2-2: The UCI provides a bitmap where a bit corresponds to TOs within a time duration/range. The bit indicates whether all TOs within the time duration/range are “unused”.

For option 1, a set of consecutive CG PUSCH within a CG configuration can be indicated as “unused”, with this case, the un-used TOs should be consecutive in time domain and corresponding, the overhead of UCI will also be low due to the limitation of TOs’ location pattern, however, with this way, it cannot to indicate the un-used TOs which is located with non-consecutive within a CG configuration, the flexibility of the indication is limited. For option 2, a bitmap can be used to indicate the un-used TOs within a CG configuration, with this case, both consecutive and non-consecutive TOs within a CG configuration can be supported, in addition, more flexibility of TOs’ location indication will also can be achieved. Comparing with option 1, the overhead of UCI will be increased. However, as agreed in last RAN1 meeting, the UCI is carried by CG-PUSCH, then the corresponding resources reserved for UCI transmission is minuscule within the PUSCH which is used for large packet transmission. Thus, we prefer option 2.

Proposal 6: The UCI determines the CG PUSCH TO(s) that are indicated as “unused” based on bitmap.

In LTE-LAA, to support Re-transmission adaptive, AUL-UCI have been approved and mainly includes HARQ-ID, RV-ID, NDI, etc. Similar mechanism of LTE-LAA is reused for NR-U, in RAN 1#93, CG-UCI for NR-U has been agreed adopt the principle and structure of LTE-LAA, and the contents of CG-UCI are shown in table 1.

Table 1: Mapping order of CG-UCI fields

| Field | Bitwidth |
| --- | --- |
| HARQ process number | 4 |
| Redundancy version | 2 |
| New data indicator | 1 |
| Channel Occupancy Time (COT) sharing information | log2C if both higher layer parameter ul-toDL-COT-SharingED-Threshold and higher layer parameter cg-COT-SharingList are configured, or if both higher layer parameter ue-SemiStaticChannelAccessConfig and higher layer parameter cg-COT-SharingList are configured, or if higher layer parameter cg-COT-SharingList is configured in frequency range 2-2, where C is the number of combinations configured in cg-COT-SharingList; 1 if higher layer parameter ul-toDL-COT-SharingED-Threshold is not configured, and if higher layer parameter ue-SemiStaticChannelAccessConfig is not configured, and if higher layer parameter cg-COT-SharingOffset is configured; 0 otherwise; If a UE indicates COT sharing other than "no sharing" in a CG PUSCH within the UE's initiated COT, the UE should provide consistent COT sharing information in all the subsequent CG PUSCHs, if any, occurring within the same UE's initiated COT such that the same DL starting point and duration are maintained. |

Since the current CG-UCI is designed for CG transmission in NR-U and some parameters within CG-UCI is not needed in licensed band, as a result, introduces a new UCI for XR in R-18 can be considered.

Proposal 7: A new UCI will be introduced in R-18 for indicating the un-used TOs within a CG configuration.

Other enhancements on capacity performance for XR

- Transmission pattern of CG within an integer periodicity

Comparing with dynamic scheduling mechanism, scheduling time delay can also be decreased thanks to no SR and BSR procedure for CG. In addition, CG transmission mechanism can also reduce the overhead of DCI signaling. As a result, enhanced CG to support XR has been agreed in R-18. In Rel-17 RAN1 meetings, it was agreed that 60 frame per second (fps) is baseline for both DL and UL video stream and 30 fps, 90 fps as well as 120 fps can be also optionally evaluated, so CG transmission mechanism can be considered for periodic XR traffic. In current specification, the periodicity of CG can only be configured as: {1/7ms, 0.5ms ,1ms,…, 320ms, 640ms} for 15kHz, 0.5x {1/7ms,0.5ms, 1ms,…,1280ms} for 30kHz, 0.25x {1/7ms,0.5ms, 1ms, …,2560ms} for 60kHz, 0.125x {1/7ms,0.5ms, 1ms, …,5120ms} for 120kHz. As show in table 2.

Table 2. ConfiguredGrantConfig information element

-- ASN1START

-- TAG-CONFIGUREDGRANTCONFIG-START

ConfiguredGrantConfig ::=           SEQUENCE {

frequencyHopping                    ENUMERATED {intraSlot, interSlot}                                       OPTIONAL,   -- Need S

OPTIONAL,   -- Need R

periodicity                         ENUMERATED {

sym2, sym7, sym1x14, sym2x14, sym4x14, sym5x14, sym8x14, sym10x14, sym16x14, sym20x14,

sym32x14, sym40x14, sym64x14, sym80x14, sym128x14, sym160x14, sym256x14, sym320x14, sym512x14,

sym640x14, sym1024x14, sym1280x14, sym2560x14, sym5120x14,

sym6, sym1x12, sym2x12, sym4x12, sym5x12, sym8x12, sym10x12, sym16x12, sym20x12, sym32x12,

sym40x12, sym64x12, sym80x12, sym128x12, sym160x12, sym256x12, sym320x12, sym512x12, sym640x12,

sym1280x12, sym2560x12

},

configuredGrantTimer                INTEGER (1..64)

……

-- TAG-CONFIGUREDGRANTCONFIG-STOP

-- ASN1STOP

However, the periodicity for XR service is not an integer of a slot or symbol. Therefore, it is not suitable to configure a CG periodicity to match with the XR UL traffic perfectly. If the periodicity of CG is configured to an integer number of slot or symbol, there will be a gap between XR packet arrival time and CG configuration.

Observation 2：There is a gap between XR periodic UL traffic and CG configuration.

To align with the periodicity of CG and XR traffic, when jitter is not considered for XR services, then UE and gNB know the arrival time of XR traffic according to the periodicity. A straightforward way is to introduce a set of non-integer CG periodicity for XR services (such as 8.33ms, 11.11ms, 16.66ms, 33.33 ms), however, non-integer periodicity is not matched with integer symbol or slot and the granularity of symbol or slot is the minimum time unit for data transmission in current specification. To solve the problem, another way can be considered, as show in Figure 2 (take 60 fps as an example), it can be obviously observed that the mismatch issue appears every three arrival intervals. In others words, within 50ms periodicity, there are three packets will be arrived, so pre-defined a fixed transmission pattern of CG within an integer periodicity for XR can be considered.

Figure 2. Mismatch between DL traffic with 30 fps and CG/SPS with 32ms

Proposal 8: A fixed transmission pattern of CG within an integer periodicity for XR can be considered.

- Additional TOs for CG configuration

As agreed in Rel-17 RAN1 meeting, a truncated Gaussian distribution is used to model the jitter of DL and UL video stream for XR services. The range of jitter is agreed to be [-4, 4]ms (baseline) and [-5, 5]ms (optional). This means the XR packets may arrive at gNB or UE within a time window of 8ms or 10ms length, and the exact arrival time is not known in advance. When the XR traffic is arrived before the first TOs within a CG transmission, then UE can transmit a PUSCH on the following TOs within a CG configuration. However, when the XR traffic is arrived after the first TOs within CG transmission occasion, the remaining TOs within a CG configuration may not be enough to carry a packet whole, then scheduling delay will be caused for transmission the remaining portion of the packet if dynamic scheduling mechanism is used. To solve this problem, to enable UE to indicate several additional TOs after the end of configured TO within the CG period or activating more than one CG configurations simultaneously similar as R-16 URLLC can be considered. When more than one CG configurations activation simultaneously for XR is adopt, as agreed in last RAN1 meeting [2], the UCI which used to indicate the un-used TOs within a CG configuration need to carry by CG-PUSCH, thus, there is no CG-UCI to indicate the un-used TOs within the CG configurations that are not chosen by the UE, this will be lead resource waste, to solve this issue, a CG-UCI can be enable to indicate un-used TOs within more than one CG configurations.

Proposal 9: Additional TOs after the end of the configured TO within a CG period and activate more than one CG configurations simultaneously can be considered.

Proposal 10: When more than one CG configuration activation simultaneously, a CG-UCI to indicate un-used TOs within more than one CG configurations can be considered.

- A CG to support multiple XR traffic flows

As discussed in the above, XR services may include multiple data streams, and each data stream may have different traffic characteristics and requirements. For a single CG configuration with multiple TOs, a shared MCS cannot be suitable for different characteristics requirement (e.g. reliability) for multiple flows, to solve this, multiple TOs within a CG configuration with different MCS can be considered.

Proposal 11: Multiple TOs with different MCS within a CG configuration for XR can be considered.

- Conclusion

In this contribution, the following observations and proposals have been made:

Observation 1: XR services have the following characteristics.

- The non-integer periodicity

- Jitter of packet arrival time

- Low latency and large packet size

- Varying packet size

- Multiple flows

Proposal 1: Follow the time domain resource mapping of Type B repetition can be used for multiple TOs within a CG configuration.

- Single SLIV is determined from TDRA

- The SLIV used for 1st PUSCH per CG period.

- N consecutive nominal PUSCHs with same duration per CG period

Proposal 2: The number of multiple TOs within a CG configuration can be indicated by TDRA

Proposal 3: Repetition for multi-PUSCH transmissions within a CG configuration can be supported.

Proposal 4: For XR, when multiple TOs within a CG is configured, the HARQ-ID for the first configured/valid PUSCH within a CG configuration is determined based on current mechanism which is defined in TS 38.321, then the HARQ-ID of the remaining TOs within a CG period is determined by incrementing the HARQ-ID of the preceding TO within a CG configuration.

Proposal 5: Both a set of unused TOs located in the front and the end of a CG configuration can be considered for CG enhanced for XR.

Proposal 6: The UCI determines the CG PUSCH TO(s) that are indicated as “unused” based on bitmap.

Proposal 7: A new UCI will be introduced in R-18 for indicating the un-used TOs within a CG configuration.

Observation 2：There is a gap between XR periodic UL traffic and CG configuration.

Proposal 8: A fixed transmission pattern of CG within an integer periodicity for XR can be considered.

Proposal 9: Additional TOs after the end of the configured TO within a CG period and activate more than one CG configurations simultaneously can be considered.

Proposal 10: When more than one CG configuration activation simultaneously, a UCI to indicate un-used TOs within more than one CG configurations can be considered.

Proposal 11: Multiple TOs with different MCS within a CG configuration for XR can be considered.

References

[1] RP-223502, New WID on XR Enhancements for NR. Nokia, Qualcomm.

[2] 3GPP RAN1 #112 meeting, Chairman notes.

[3] TR 38.838, Study on XR (Extended Reality) Evaluations for NR.