3GPP TSG RAN WG1 #112bis-e			R1-2302848

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.4.1.2

Source: 	Sony

Title:	Discussion on physical channel design framework for SL-unlicensed

Document for:	Discussion

Introduction

In RAN#95-e, Rel-18 NR sidelink evolution has been approved and sidelink on unlicensed spectrum will be studied and specified in order to increase bandwidth for sidelink transmission as described in [1]. In RAN1#112, RAN1 discussed and made some agreements [2].

In this contribution, we discuss our view on physical channel design framework for SL-unlicensed.

Discussion

Slot structure

Regarding AGC symbols for slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission, we have discussed the number of AGC symbols for the 1st and 2nd starting symbols in terms of Tx UE behaviour and Rx UE behaviour respectively.

At RAN1 #111, the following options were agreed for down-selection later:

Agreement For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - Regarding Tx UE behaviour: - If PSCCH/PSSCH transmission starts from 1st starting symbol, down-select one of the followings - Option 1: The PSCCH/PSSCH transmission has 2 symbols for AGC purpose - Option 2: The PSCCH/PSSCH transmission has only 1 symbol for AGC purpose - Option 3: The PSCCH/PSSCH transmission has 1 or 2 symbol(s) for AGC purpose depending on conditions, FFS details - If PSCCH/PSSCH transmission starts from 2nd starting symbol, the PSCCH/PSSCH transmission has only 1 symbol for AGC purpose - Regarding Rx UE behaviour, down-select one of the followings: - Option A: The Rx UE always monitors two AGC symbols in such slot - Option B: The Rx UE monitors two AGC symbols in such slot by default, but could drop monitoring the 2nd AGC symbol at least if it detects a PSCCH/PSSCH transmission starting from the 1st starting symbol - FFS details - Option C: The Rx UE monitors two AGC symbols in such slot by default, but it is up to UE implementation whether to drop monitoring the 2nd AGC symbol - Option D: It is up to UE implementation to monitor 1 or 2 AGC symbol(s) in such slot

Regarding Tx UE behaviour, it was agreed that if PSCCH/PSSCH transmission starts from 2nd starting symbol, the PSCCH/PSSCH transmission has only 1 symbol for AGC purpose. A remaining discussion point is the number of AGC symbol for the PSCCH/PSSCH transmission. Two symbols for AGC purpose ensures that the AGC procedure is effective and robust in various transmission scenarios, minimizing the risk of decoding failure due to inaccurate AGC. Therefore, we prefer to have two AGC symbols for the PSCCH/PSSCH transmission.

Regarding Rx UE behaviour,, at least when the Rx UE detects a PSCCH/PSSCH transmission starting from the 1st starting symbol, only one AGC symbol would be monitored. We think the condition whether to monitor 1 or 2 AGC symbol(s) in a slot does not have to be restricted, and it would be up to Rx UE implementation. Therefore, we support that It is up to UE implementation to monitor 1 or 2 AGC symbol(s) in such slot .

Proposal 1: Regarding Tx UE behaviour, the PSCCH/PSSCH transmission has 2 symbols for AGC purpose if PSCCH/PSSCH transmission starts from 1st starting symbol (Option 1).

Proposal 2: Regarding Rx UE behaviour, it is up to UE implementation to monitor 1 or 2 AGC symbol(s) in a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission (Option D).

Enhancements for SL resource allocation mode 2

Procedure in the case of LBT failure for mode 2

In RAN1#109-e, it was agreed to support UE-to-UE COT sharing for NR-U. Though it is still FFS which SL channels/signals can be transmitted from a COT-shared UE, if PSFCH transmission corresponding to PSSCH within the shared COT is supported and LBT is required for the COT-shared UE before transmitting the PSFCH, a procedure in the case of LBT failure is necessary.

At RAN1#112, the following agreement has been made for the PSFCH transmission.

Agreement To address PSFCH transmission dropping due to LBT failure: - Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission - Down-select one or support both of the followings - Option 1: Such PSFCH occasion(s) are (pre-)configured - Option 2: Such PSFCH occasion(s) are (pre-)configured and dynamically indicated - FFS applicable scenarios, e.g., considering the applicability of COT sharing, MCSt, etc. - FFS other details

Considering a resource efficiency for PSFCH, we propose to support a dynamic indication of the PSFCH resource through a COT sharing information in SCI.

Proposal 3: PSFCH occasion(s) are (pre-)configured and dynamically indicated by COT initiating UE.

Supporting interlaced mapping

In RAN1#109-e, it was discussed that some solutions to meet OCB and PSD requirement for S-SSB transmission in SL-U and agreed to down-select from the following 4 options;

- Option 1: Using interlaced RB transmission

- Option 2: S-SSB multiplexing with other SL transmissions in the same slot

- Option 3: Repetition of S-PSS/S-SSS/PSBCH in frequency domain

- Option 4: S-PSS/S-SSS/PSBCH with wider bandwidth

For NR-U, SSB can be multiplexed with other DL transmissions including DL signals, e.g. CSI-RS, and DL channels, e.g. PDCCH/PDSCH to meet OCB and PSD requirement for SSB in NR-U. On the other hand, for sidelink transmission, S-SSB multiplexing with other SL channels in frequency domain is not beneficial due to a half-duplex restriction.

Considering a specification effort, new structure of S-SSB should be deprioritized at this stage as long as there is any reason.

Therefore, we have thoughted to down-select from either option 1 or 3. At RAN1#110-bis, it was agreed to support either option 1 or 3 and down-select between them. Comparing them, we don’t see any large difference. But since it was agreed that PRB-based interlaced mapping is supported for PSCCH/PSSCH, the same solution might be better to use for S-SSB transmission as well considering specification efforts as long as there are any critical issues. Therefore we slightly support PRB-based interlaced mapping (option 1) for S-SSB transmission in SL-U.

Proposal 4: Using interlaced RB transmission is supported for S-SSB transmission in SL-U.

Conclusion

Proposal 1: Regarding Tx UE behaviour, the PSCCH/PSSCH transmission has 2 symbols for AGC purpose if PSCCH/PSSCH transmission starts from 1st starting symbol (Option 1).

Proposal 2: Regarding Rx UE behaviour, it is up to UE implementation to monitor 1 or 2 AGC symbol(s) in a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission (Option D).

Proposal 3: PSFCH occasion(s) are (pre-)configured and dynamically indicated by COT initiating UE.

Proposal 4: Using interlaced RB transmission is supported for S-SSB transmission in SL-U.

Reference

- RP-220300, “WID revision: NR sidelink evolution,” 3GPP TSG RAN Meeting #95e.

- RAN1 chair’s notes, RAN1#112.