3GPP TSG RAN WG1 #112bis-e			R1-2302851

e-Meeting, April 17th – April 26th, 2023

Agenda Item: 	9.5.1.1

Source: 	Sony

Title: 	Discussion on SL positioning reference signal

Document for: 	Discussion & Decision

# Introduction

A new work item on expanded and Improved NR Positioning was approved in RANP#98e [1], which identify sidelink positioning reference signal as one of the WI topics. In [2] the reference signal design featured for sidelink positioning such as reference sequence generation, multiplexing was discussed. The agreement from the last meeting is given below.

Agreement - SL PRS sequence is generated based on Gold sequence: - where c(i) is a pseudo-random sequence as defined in Clause 5.2.1 of TS 38.211. - For SL PRS sequence generation, the pseudo-random sequence c(i) initialization equation is defined as a function of at least: slot number, symbol number, and a parameter . - The pseudo-random sequence c(i) initialization equation is based on initialization equation as for DL PRS - For SL PRS sequence generation, consider at least the following options to define the parameter , and select one option: - Option 1: is a higher layer configured parameter - Option 2: is based on 12 bits CRC of PSCCH associated with the SL PRS transmission - Option 3: based on a combination of higher layer configured parameter from a configured ID list and 12 bits of CRC of PSCCH associated with the SL PRS transmission - Option 5: is based on 12bits LSB of destination ID - Option 6: is based on 8 bits of source ID + 4 zero bits - Option 7: is based on the CRC field of the 2nd SCI associated with SL PRS transmission, if there is a 2nd SCI defined. - Range of the parameter is: - A SL PFL is not defined. SL positioning RS are defined directly with respect to and contained within a single SL BWP and carrier. - Support SCS values for SL PRS include: - 15 kHz, 30 kHz, 60 kHz for FR1, and 60 kHz, 120 kHz for FR2 - Which SCS values are required, and which ones are optional follow Rel-16 UE capabilities. - For RE-offset sequence for SL PRS, the RE-offset sequences specified for DL PRS are considered as a starting point. - FFS: Exact RE-offset sequences - Comb-based multiplexing of SL PRS from different UEs in a slot is supported at least for dedicated resource pools. - FFS: Comb-based multiplexing of SL PRS from different UEs in a slot for shared resource pools. - For comb-based multiplexing of SL PRS from different UEs, support at least the case wherein a single (M,N) value is possible . - FFS: Whether to support comb-based multiplexing of SL PRS from different UEs in a slot using multiple (M,N) values. - FFS: additional restrictions (if any) due to e.g. the impact of synchronization and IBE interference between UEs - For SL PRS in shared or dedicated resource pools, - at least comb sizes (N) 2, 4 are supported. - Comb size 6 is supported at least in dedicated resource pool - FFS: comb size 6 in shared resource pool - Comb size 1 is supported at least in shared resource pool - FFS: comb size 1 in dedicated resource pool - comb sizes (N) > 12 are not supported. - FFS: support of comb sizes (N) of 8, 12. - For SL PRS in shared and dedicated resource pools, - SL PRS patterns with full staggering are supported. - FFS: whether (M,N)=(6,6) is supported - SL PRS patterns with partial staggering are supported at least for the following (M,N) pairs: - (M, 2) with M = {1} - (M, 4) with M = {2} - FFS: constraints on maximum effective comb size - FFS: support of partial staggering for other comb sizes - FFS: Support of SL PRS patterns with M > N at least with full staggering. - TDM-based multiplexing of SL PRS from different UEs in a slot is supported at least for dedicated resource pools. - FFS: TDM-based multiplexing of SL PRS from different UEs in a slot for shared resource pools. - FFS: Details, including resource granularity and relationship to SCI/PSCCH associated with the SL PRS resources, additional AGC symbols. - FFS: restrictions for the configuration of TDM-based multiplexing of SL PRS from different UEs in a slot, if any - FFS: which resource allocation schemes are applicable - FFS: whether or not this is a separate UE capability - The OLPC framework defined for PSSCH/PSCCH is considered as a starting point for OLPC for SL PRS.

In this contribution, we provide our views on the characteristics of sidelink positioning reference signal.

# Discussion

Sidelink Positioning and ranging requires the transmission of positioning reference signal. During the study item phase, it was concluded to use a comb-based frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS.

The usage of comb-based frequency domain is essential as this would enable more UEs transmit SL-PRS within the same symbol. This would enable sidelink positioning measurement from multiple UE transmitting SL-PRS within at least one symbol. The comb-structure as in the legacy NR positioning can be used as the starting point.

Proposal 1: Support SL-PRS frequency domain with comb structure. The legacy comb-N structure in legacy NR positioning can be used as the baseline.

On the time domain structure of SL-PRS, we prefer to have time-gap on before and after SL-PRS transmission. The time-gap can be used to minimize the interference from/to the adjacent symbol. Furthermore, the time-gap prior to and after the SL-PRS can be used for AGC purpose and possible Rx/Tx retuning, respectively. This is illustrated in Figure 1. We consider one symbol for each gap is sufficient.

Figure 1: Time domain structure of SL-PRS transmission

Proposal 2: Support the time-gap prior to and after the SL-PRS transmission. The first and second gap are used for AGC and Rx/Tx retuning.

The SL-PRS is based on the legacy DL-PRS where a pseudo-random generator is required at the UE transmitting SL-PRS to generate the SL-PRS. There are several parameters which affect the initialization of generating pseudorandom sequence. We consider one of the parameters is the SL-PRS ID (nID,seqSL-PRS). Multiple options of SL-PRS ID generation were given in the last meeting. Among those options, we prefer a nID,seqSL-PRS based on 8 bits of source ID and 4 zero bits. It is important for SL-Rx-UE can regenerate the identical SL-PRS of the intended SL-Tx-UE. This is to be used for the SL positioning measurement. In SL positioning the SL-PRS is generated at the UE Tx side. In order for the SL-Rx-UE to be able to perform positioning measurement by regenerating the transmitted SL-PRS, the SL-Rx-UE needs to know the associated SL-PRS ID which is based on the source ID.

Proposal 3: Support the nID,seqSL-PRS generation is based on bits of source ID and 4 zero bits (also known as option 6).

The transmission of SL-PRS is expected to be in two types of SL resource pool, namely dedicated resource pool and shared resource pool. The bandwidth of the SL-PRS should be reconfigurable and it is up to the gNB to allocate the bandwidth of SL-PRS within a resource pool, regardless of the resource pool type. However, the maximum bandwidth of SL-PRS should be equivalent to the resource pool bandwidth.

Proposal 4: SL-PRS bandwidth within a resource pool is reconfigurable up-to the maximum resource pool bandwidth.

During RAN1#112 meeting, there was a discussion about the multiplexing scheme for SL positioning. It was suggested that both TDM and FDM could be used in dedicated resource pool. This means SL-PRSs from different UEs can be multiplexed either in time-domain and frequency-domain. For FDM, a comb-based structure can be used where the resource for SL-PRS is allocated in a staggered manner. A FDM pattern can be described by comb size N (N is greater than 1) and the number of consecutive symbols M.

Similarly, for TDM, we consider that the resource can be configured using comb-1 structure (where N=1) for M consecutive symbols. Comb-1 means SL-PRS occupies all the sub-carriers in a sub-channel. An example of a slot format of TDMed SL-PRSs with (M, N) = (2, 1) is shown in Figure 2 below. In this format, multiple SL-PRSs from different UE TXs are multiplexed in different time symbols within the slot.

Figure 2. An example of a slot format for TDMed SL-PRSs

Proposal 5: For TDM-based multiplexing, the SL-PRSs can be configured using comb-1 structure with comb size N= 1, M≥1.

Overall, these multiplexing schemes offer flexibility in how SL-PRSs are transmitted and allow for efficient use of resources. It is important to consider the trade-offs between the different schemes and their impact on performance when selecting a particular scheme and configuring resource. In order to support both schemes, it is necessary for the resource scheduler to be aware of the capability of the UE as not all the UEs is capable of both schemes. For example, some Rx UEs may have difficulty achieving an optimal performance when using FDM due to the hardware limitation. Therefore, TDM becomes a preferred solution for these types of UEs.

To address this issue, it is beneficial to have a separate UE’s capability report transmitted to the gNB. This report would provide information about the UE's capabilities on processing TDMed SL-PRS and FDMed SL-PRS, allowing the gNB to configure FDMed or TDMed resources for SL-PRS transmission accordingly. By doing so, the gNB can optimize the resource allocation for each UE, improving overall system performance.

Proposal 6: Support a separate UE’s capability on processing TDM SL-PRS and FDM SL-PRS.

# Conclusion

In this contribution we provide our view on sidelink positioning reference signal. We made the following proposals:

Proposal 1: Support SL-PRS frequency domain with comb structure. The legacy comb-N structure in legacy NR positioning can be used as the baseline.

Proposal 2: Support the time-gap prior to and after the SL-PRS transmission. The first and second gap are used for AGC and Rx/Tx retuning.

Proposal 3: Support the nID,seqSL-PRS generation is based on bits of source ID and 4 zero bits (also known as option 6).

Proposal 4: SL-PRS bandwidth within a resource pool is reconfigurable up-to the maximum resource pool bandwidth.

Proposal 5: For TDM-based multiplexing, the SL-PRSs can be configured using comb-1 structure with comb size N= 1, M≥1.

Proposal 6: Support a separate UE’s capability on processing TDM SL-PRS and FDM SL-PRS.

# References

- RP-213588, “Revised SID on Study on expanded and improved NR positioning”, RANP #94e, December 2021

- RAN1 Chairman’s notes, RAN1#112, February 2023