3GPP TSG RAN WG1 #112bis-e			R1-2302855

e-Meeting, April 17th – April 26th, 2023

Agenda Item: 	9.5.5

Source: 	Sony

Title: 	Discussion on Positioning for RedCap UEs

Document for: 	Discussion & Decision

# Introduction

Positioning of UEs with reduced capabilities (RedCap UE) were studied in the recently closed NR positioning Rel-18 study item. The NR positioning performance evaluation with the legacy Rel-17 and with the potential enhancements were conducted and the results were captured in TR 38.859 [1]. The potential techniques to enhance the performance of RedCap UE were identified.

In [2], agreements and conclusion were made mainly on the feature of DL-PRS Rx Frequency hopping, which are given below.

Conclusion For positioning enhancements for RedCap UEs, only Rx frequency hopping of the DL PRS is supported. The scope for RedCap positioning includes FR1 and FR2. Agreement For RedCap UEs, support at least measurements on DL PRS with Rx frequency hopping using a measurement gap - FFS: details on RedCap UE processing capabilities for DL PRS with Rx frequency hopping and MG - FFS: the use of a single or multiple instances of a MGs - FFS: the use of PPW For Positioning enhancements for redcap UEs for UL SRS Tx and DL PRS Rx frequency hopping, from the RAN1 perspective, short switching time to allow RF retuning between adjacent hops may be beneficial in terms of accuracy and latency performance. - Send an LS to RAN4 requesting feedback on the feasible values for the switching time between hops, at least when numerology and bandwidth for each hops can be the same, and the Tx/Rx antennas used in all hops can be the same. For positioning for RedCap UEs with DL PRS Rx Hopping, the UE hops within a DL PRS resource - FFS: whether there is specification update needed for RAN1 - FFS: remaining details For RedCap UEs, support SRS for positioning frequency hopping by - Using a configuration separate from the existing BWP configuration - FFS: hopping is configured within a SRS resource or across SRS resources

This contribution discusses our views on positioning for RedCap UEs, particularly on positioning enhancement techniques suitable for RedCap UE.

# Discussion

The operation of PRS with frequency hopping for RedCap UE were approved in RAN1#112 meeting. It includes both Rx frequency hopping and Tx frequency hopping for downlink and uplink based positioning, respectively. In ideal case of DL-PRS reception with frequency hopping, the receiver combines DL-PRS in multiple sub-bands (e.g, the sub-band BW is the RedCap UE) to form a virtual wideband DL-PRS reception. A similar approach is also applied for the uplink direction. In reality, there is a random phase offset between the sub-bands depending on the receiver / transmitter characteristics and this lead to the performance loss. In order to compensates the performance loss, we consider using bandwidth stitching to extend the bandwidth for positioning. For the downlink case, this requires gNB to transmit multiple bandlimited PRSs at different frequency and RedCap UE to receive these bandlimited PRSs individually and stitch them into one virtual wideband bandwidth for the PRS measurement. This method may alleviate the performance loss of RedCap UE due to the bandwidth limitation.

Bandwidth stitching requires RedCap UE to measure PRSs on multiple frequency sub-bands. These sub-bands can be transmitted and received in a hopping manner (frequency hopping). It requires RedCap UE to receive and measure different frequency hops in different time windows. Figure 1 below shows an illustration of frequency hopping operation for RedCap positioning. In this example, RedCap UE receives and measures 5 different frequency hops individually. One hop contains 1 PRS resource set in this case. By stitching 5 different 20MHz bandwidth PRSs, the RedCap UE can obtain a virtual wideband PRS signal/transmission. The positioning measurement based this virtual wideband BW is expected to achieve a comparable performance as a native 100MHz bandwidth PRS. The performance of bandwidth stitching has also been evaluated during the SID [1]. The simulation results shows that with combing 5 small hops (with 20MHz each), the performance is comparatively as good as the one with one native wide bandwidth (with 100MHz).

Figure 1. Illustration of frequency hopping and bandwidth stitching for RedCap positioning.

Observation 1: Bandwidth stitching requires RedCap UE to measure PRSs on multiple frequency bands. This can be achieved by frequency hopping.

However, phase offset (PO) becomes an issue in practice. Since the frequency synthesizer is basically a phase locked loop (PLL), every time it switches or hops into a new carrier frequency it introduces a random phase offset relative to the previous carrier. The phase offset may degrade the performance if stitching two carriers with large phase offset.

Observation 2: Frequency hopping between two carriers would introduce a random phase offset (PO) due to the nature of PLL. The frequency offset may degrade the performance gain from bandwidth stitching.

The phase offsets can be estimated and calibrated if two adjacent bands have overlapping frequency tones. The frequency hopping with overlapping tones is also illustrated in figure 1. In this example, two adjacent PRS resource sets share a few overlapping PRBs in frequency domain. The overlapping PRBs (overlap region) can be utilized to estimate the frequency offset of these two adjacent hops, which may potentially help to mitigate the performance loss.

Observation 3: Phase offset can be calibrated if two adjacent frequency bands have overlapping region.

Proposal 1: Support frequency hopping with partial overlap with two adjacent frequency bands to compensate the performance loss due to the phase offset.

The resource allocation is an important consideration for improving the performance of bandwidth stitching. Frequency hopping requires resources allocated at different time-frequency location, it can be with a staggered manner. One factor that can affect the performance is the total length of the measurement occasion. A long time gap between hops should be avoided as it introduces long measurement time and also may introduce additional phase offset due to the Doppler effect. Hence, we consider the frequency hopping operation should be consistent (i.e., uniform time-gap among gaps) and as compact as possile. This can be facilitated by defining a new parameter, so called time gap (T_gap) between two adjcent hops. By selecting the time gap Tgap properly, the benefit of bandwidth stitching operation (i.e., positioning accuracy improvement) can be achieved. In addition to the time gap, the overlapping region on the frequency spam should also be considered. Since bandwidth hopping would introduce an extra phase offset between hops, it is essential to define an overlapping region between two hops, such that the phase offset can be compensated.The overlapping region should be carefully defined, which should not be too large, but sufficient for phase offset estimation. To accomplish this, a parameter called F_ovl can be introduced to indicate the size of frequency allocation that overlap between hops. The size can be in a multiple of sub-carriers (e.g., PRB). Increasing F_ovl enlarges the overlapping region, which leads to better phase offset estimation but a smaller overall bandwidth, while decreasing F_ovl results in a larger bandwidth but less accurate phase offset estimation.

Proposal 2: Introduce two parameters, Tgap (the time gap between two adjacent hops) and F_ovl (the overlap resources in frequency domain), to facilitate bandwidth stitching in the frequency hopping operation.

Proposal 3: These parameters (T_gap and F_ovl) can be configured for each measurement occasion or semi-consistent for multiple occasions.

In principle, these parameters can help in optimizing the performance of bandwidth stitching and improve the accuracy of RedCap UE positioning. The configurability of these parameters offer flexibility in configuring the hopping pattern and provide a balance between phase offset estimation and overall bandwidth usage.

The illustration in Figure 1 shows one of the frequency hopping patterns in staggered manner. The frequency hopping can be fixed but it can also be reconfigurable. A reconfigurable hopping pattern will offer flexibility to the resource allocation arrange by the gNB.

Proposal 4: Further study whether the frequency hopping pattern is fixed or reconfigurable.

UE capability is another aspect which shall be considered in supporting frequency hopping operation for the positioning purpose. In another work-item, NR RedCap UE Rel-18, it is expected the RedCap UE can be operated with limited bandwidth for PDSCH reception (i.e, 5 MHz). Hence, the trend is to reduce the capability of NR RedCap UE Rel-18 in comparison to NR RedCap Rel-17 (i.e, that can be operated with 20 MHz in FR1). Some examples related to the supporting frequency hopping for NR RedCap UE can be:

- A RedCap UE that is not capable to perform frequency hopping. In this case, low positioning estimation accuracy can be expected.

- A RedCap UE that is capable to perform the maximum number of frequency hopping. In this case, high positioning accuracy can be expected.

- A RedCap UE that is capable to perform a few number of frequency hopping. In this case, a medium positioning accuracy can be expected.

Different UE vendor may have different implementation requirements. Hence, providing such capability would provide flexibility to the UE / chipset vendor.

Proposal 5: Support the UE capability parameter to reflect the supported frequency hopping operation for NR RedCap UE. (i.e, by considering the RedCap UE constraints / limitations).

When supporting DL-PRS reception with Rx frequency hopping, a time gap between two adjacent hops to allow the Rx UE to measure PRS and perform carrier switching needs to be considered by gNB. It is important to consider the UE’s processing time when configuring the gap. Therefore, we propose including UE processing time as part of the UE capability report.

Proposal 6: Support the RedCap UE’s processing time for Rx frequency hopping as part of the UE capability.

There was also a discussion on supporting frequency hopping operation within a PRS Processing Window (PPW). PPW concept would enable positioning procedure to be performed without a measurement gap. The motivation is to support low latency positioning. However, low latency aspect of RedCap UE was not evaluated during study item phase. The primary focus was on obtaining accurate positioning. Hence, we propose to down-prioritize the frequency hopping operation within a PRS Processing Window (PPW).

Proposal 7: Down-prioritize the frequency hopping operation within a PRS Processing Window (PPW) for RedCap UE.

In the last meeting, there was a discussion on whether to use a single of multiple MGs for Rx frequency hopping. To avoid the effect of doppler, frequency hopping should be done in a very short time, which may not be feasible when using multiple MGs. Hence, for RedCap positioning, we propose only supporting the use of a single instance of MG for DL PRS with Rx frequency hopping.

Proposal 8: For RedCap UE positioning, only support the use of a single instance of MG for DL PRS with Rx frequency hopping.

In terms of frequency hopping pattern, it has been agreed to use multiple hops within one DL PRS resource. However, this may not be feasible with considering of the DL Tx beam sweeping. Normally, one PRS resource-set may contain multiple resources. Hopping within a DL PRS resource may require a large number of carriers switches. For example, if 1 resource-set comprises 16 resources and for each resource it has 5 hops, the Rx UE needs to switch the hops 79 times (16*5-1). Therefore, it is more feasible to perform hopping in a large scale, such as hopping across multiple PRS resources or hopping across multiple resource-set to reduce the switching frequency.

Proposal 9: Support frequency hopping is configurable across multiple DL PRS resources or resource-sets.

# Conclusion

In this contribution we provide our view on positioning support for RedCap UE. We made the following observations and proposals:

Observation 1: Bandwidth stitching requires RedCap UE to measure PRSs on multiple frequency bands. This can be achieved by frequency hopping.

Observation 2: Frequency hopping between two carriers would introduce a random phase offset (PO) due to the nature of PLL. The frequency offset may degrade the performance gain from bandwidth stitching.

Observation 3: Phase offset can be calibrated if two adjacent frequency bands have overlapping region.

Proposal 1: Support frequency hopping with partial overlap with two adjacent frequency bands to compensate the performance loss due to the phase offset.

Proposal 2: Introduce two parameters, Tgap (the time gap between two adjacent hops) and F_ovl (the overlap resources in frequency domain), to facilitate bandwidth stitching in the frequency hopping operation.

Proposal 3: These parameters (T_gap and F_ovl) can be configured for each measurement occasion or semi-consistent for multiple occasions.

Proposal 4: Further study whether the frequency hopping pattern is fixed or reconfigurable.

Proposal 5: Support the UE capability parameter to reflect the supported frequency hopping operation for NR RedCap UE. (i.e, by considering the RedCap UE constraints / limitations).

Proposal 6: Support the RedCap UE’s processing time for Rx frequency hopping as part of the UE capability.

Proposal 7: Down-prioritize the frequency hopping operation within a PRS Processing Window (PPW) for RedCap UE.

Proposal 8: For RedCap UE positioning, only support the use of a single instance of MG for DL PRS with Rx frequency hopping.

Proposal 9: Support frequency hopping is configurable across multiple DL PRS resources or resource-sets.

# References

- 3GPP TR 38.859 V18.0.0 (2022-12) Study on expanded and improved NR positioning

- RAN1 Chairman’s notes, RAN1#112, February 2023