3GPP TSG RAN WG1 #112bis-e					                                             R1-2302857

e-Meeting, April 17th – April 26th, 2023

Agenda Item:					    9.9.1

Source:	Sony

Title:	On coverage enhancement for NR NTN

Document for:	Discussion

# Introduction

At RAN#99, the work item description [1] for Rel-18 was further updated, focusing on UL coverage enhancement.

Coverage enhancement The Rel-18 NTN objectives are focused on the applicability of the “solutions developed by general NR coverage enhancement” (NR_cov_enh) to NTN, and identifying potential issues and enhancements if necessary, considering the NTN characteristics including large propagation delay and satellite movement. Only NTN-specific characteristics are to be included in this coverage enhancement work, otherwise it should be part of another WI (e.g., UL enhancement of coverage). The following reference scenario is considered for the definition of uplink coverage enhancements for NTN: parameter set-1 for LEO-1200 satellite operating at Line of Sight (LOS) and commercial smartphones with -5.5 dBi antenna gain and 3 dB polarisation loss (per antenna port). Note: It is understood that the enhancements defined for LEO can also apply to GEO and MEO scenarios as appropriate. No additional work is expected for MEO/GEO. The targeted services are VoIP using AMR 4.75 kbps and data transmission services with Low data rate of 3 kbps. The detailed objectives are for NTN: - To specify PUCCH enhancements for Msg4 HARQ-ACK (e.g. repetition) [RAN1, RAN4] - To specify if necessary, enhancements to the Rel-17 procedures for DMRS bundling for PUSCH taking into account NTN-specifics (e.g. time-frequency pre-compensation) [RAN1]

In this contribution, we discuss some potential enhancement on DMRS bundling for PUSCH that can be applied to NTN networks for coverage enhancement.

# DMRS bundling for PUSCH

DMRS bundling enhancement has been further discussed in RAN1#112, where the following observation has been made [2]:

Additionally, the candidate solutions to support DMRS bundling have been further refined [2]:

Looking at the NTN-specific issue, the major impact on the UE phase continuity is the time and frequency of pre-compensation that NTN UEs would need to perform. Due to the fast movement of satellites (especially in LEO constellations), more frequent adjustments of TA can be expected for NTN. This is confirmed by the proposed observation from RAN1#112. Therefore, UE pre-compensation of TA and phase is necessary. However, multiple disruptive events have been specified for Rel-17 DMRS bundling due to feasibility from the UE RF implementation aspect, which includes both UE autonomous TA adjustment and network-commanded TA adjustment. Therefore, it is necessary to enhance the DMRS bundling scheme to accommodate the NTN operation.

For NR NTN, the UE autonomous adjustment of TA is up to UE implementation, which is unknown to the network. Therefore, the network has no control or prediction of the actual TDW length, which makes the gain of performing JCE uncertain. Thus, one possible solution is to introduce an NTN-specific configuration for TDW, which only allows UEs to perform pre-compensation at network-configured time/frequency resources.

In Rel-17, the segmented uplink transmission scheme enables TA adjustment for IoT NTN devices between UL segments while the TA value is maintained within a UL segment. This allows the IoT-NTN eNB to receive the UL transmission predictably. A similar approach (of using UL segments) would also enable NR NTN DMRS bundles to be decoded in a predictable manner by the gNB.

Observation 1: Providing NTN-specific configuration/indication regarding when UE can perform pre-compensation can ensure the network can control the actual TDW size.

Observation 2: Introducing a segmented uplink transmission scheme to NR NTN can indicate to the UE when it can perform pre-compensation.

However, a potential issue with the solution above is that the UE may have to withhold the necessary UE TA and doppler pre-compensation since it may get limited by the potential TDW windows restricting the updates. To overcome this problem, an alternative solution is that the Actual/Nominal TDW is determined based on UE-reported information regarding when it performs pre-compensation. With doing this, the UE can still perform the pre-compensation when it is necessary, as in Rel-17, while the network also has knowledge about the expected actual TDW length before it configures it. Such a scheme and its comparison with the two other options mentioned above is illustrated in Fig. 1.

Proposal 1: Support UE reporting of information regarding when UE performs pre-compensation (e.g., the time location or the periodicity that UE perform pre-compensations) that can cause phase discontinuity and power inconsistency in Rel-18 NR NTN.

Some concerns regarding the UE reporting were also raised in RAN1#112, including the possible delay in starting the TDW window due to waiting for the UE report and heavier uplink overhead caused by frequent reporting [3]. We believe both issues can be appropriately resolved if the UE can report the future locations when UE performs the pre-compensation. Considering the NTN network usually operates under an LoS condition without too much dynamic variation in the channel, and that the UE knows satellite location and trajectory from ephemeris information, a good estimation on the UE side can be assumed to predict future times at which it will need to perform the pre-compensation, as shown in Fig.1.

Observation 3: UE can report the future occasions when it will perform pre-compensation to reduce the frequency of reporting and possible delay on starting the TDW.

Moreover, knowing when the UE will perform the pre-compensation allows the network to even configure a TDW that crosses the pre-compensation event to obtain a larger JCE gain. Considering a more frequent adjustment of TA mentioned earlier, the length of DMRS bundling will be restricted and the gain from the JCE operation will be limited for NTN operation, which may not contribute sufficiently to a coverage enhancement for NTN network and enhancement is therefore needed. Therefore, such an enhancement may become particularly important for NTN operation.

Fig. 1. A comparison of different schemes for TDW configuration with NTN.

Some initial simulation results in Fig. 2 based on a simplified simulation setup show a comparison of the channel estimation error under three different channel estimation schemes. A disruptive event happens in the middle of the transmission, which can be understood as the UE performing pre-compensation that causes a phase jump. The three schemes are illustrated in Fig. l.

Fig. 2. The channel estimation error for each slot index by adopting different estimation schemes.

It can be observed from Fig. 2 that when the network knows the location of a disruptive event, the network can perform a joint channel estimation over it, which reduces the estimation error compared to the case where two separate estimations are performed. Therefore, it is a demonstration that it is feasible to have the actual TDW cross the per-compensation event if the network knows its time location based on the UE report.

Observation 4: By knowing the time location when the UE performs pre-compensation, the channel estimation gain can be improved if the actual TDW crosses the pre-compensation event.

# Conclusions

We have discussed DMRS bundling for NTN coverage enhancement. We make the following observations and proposals:

Observation 1: Providing NTN-specific configuration/indication regarding when UE can perform pre-compensation can ensure the network can control the actual TDW size.

Observation 2: Introducing a segmented uplink transmission scheme to NR NTN can indicate to the UE when it can perform pre-compensation.

Observation 3: UE can report the future occasions when it will perform pre-compensation to reduce the frequency of reporting and possible delay on starting the TDW.

Observation 4: By knowing the time location when the UE performs pre-compensation, the channel estimation gain can be improved if the actual TDW crosses the pre-compensation event.

Proposal 1: Support UE reporting of information regarding when UE performs pre-compensation (e.g., the time location or the periodicity that UE perform pre-compensations) that can cause phase discontinuity and power inconsistency in Rel-18 NR NTN.

# References

- RP-230809	 Revised WID: NR NTN (Non-Terrestrial Networks) enhancements	  Thales

- Chair's notes RAN1#112 9.11 R18 NTN v04

- R1-230xxxx_Summary#3 for 9.11.1 Coverage enhancement for NR NTN_v001_FL