3GPP TSG RAN WG1 #112bis	                         		R1-2302861

eMeeting, 17th – 26th April 2023

Agenda Item: 	9.11.1

Source: 	Sony

Title: 	Evaluation of low-power WUS

Document for: 	Discussion / Decision

# Introduction

A new release 18 study item on low-power wake-up signal and receiver was approved in RANP#94e meeting and revised in RANP#97e, with the following objectives

- Identify evaluation methodology (including the use cases) & KPIs [RAN1] - Primarily target low-power WUS/WUR for power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, controllers) and wearables - Other use cases are not precluded - Study and evaluate low-power wake-up receiver architectures [RAN1, RAN4] - Study and evaluate wake-up signal designs to support wake-up receivers [RAN1, RAN4] - Study and evaluate L1 procedures and higher layer protocol changes needed to support the wake-up signals [RAN2, RAN1] - Study potential UE power saving gains compared to the existing Rel-15/16/17 UE power saving mechanisms, the coverage availability, as well as latency impact of low-power WUR/WUS. System impact, such as network power consumption, coexistence with non-low-power-WUR UEs, network coverage/capacity/resource overhead should be included in the study [RAN1] - Note: The need for RAN2 evaluation will be triggered by RAN1 when necessary.

During RAN1#110e-bi s[2] meeting, the following agreements related to the first and the fifth objectives were made.

For future meetings on LP WUS: Agreement For evaluation, 1 Rx chain for LP-WUS receiver is baseline. Agreement Both RRC IDLE/INACTIVE and CONNECTED modes are to be studied as part of the LP-WUS/WUR SI. - FFS: Further prioritization if needed during the study item. Agreement Take the following power model for main radio for evaluation in LP-WUS/WUR SI, - For IoT and wearable cases, reuse TR38.875 power model as baseline. - For eMBB and other cases, reuse TR38.840 power model as baseline. - Introduce ‘Ultra-deep sleep’ power state for main radio of UEs with LP-WUS receiver - FFS: The details of ‘Ultra-deep sleep’ power state Agreement The following power model for LP-WUR/WUS evaluation is considered, - Relative power unit for LP-WUR ‘off’ state, i.e., the LP-WUR does not perform monitoring: - [0.001] - Relative power unit for LP-WUR ‘on’ state, i.e., the LP-WUR performs monitoring: - [0.005/0.01/0.02/0.03/0.05/0.1/0.2/0.5/1/2/4] - Other values are not precluded to be evaluated. - FFS: Mapping from values to a LP-WUR architecture or LP-WUR mode of operation - No additional transition energy and transition time between ‘on’ and ‘off’ state as start point, FFS any transition energy and transition time if needed. Note1: A unit of power is defined to be the same for main receiver and LP-WUS receiver. Note2: the values provided is for the purpose of studying power saving gain, and the values can be further revisit and categorization depending on the receiver architecture discussion. Note3: For LP-WUR ‘on’ state, more than one values within the above range may be used for evaluation (e.g. for a single LP-WUR architecture) FFS: LP-WUR power consumption values for FR2. Agreement - The following power models are used ‘Ultra-deep sleep’ power state for main radio for evaluation Power State; Relative Power (unit); Ramp-up and down transition energy (Note1): (unit multiplied by ms); Ramp-up time; Time for sync/re-sync. Ultra-deep sleep; [0.015]; [2000 ~ 40000] - Study to converge on candidate numbers to use for evaluation - FFS: other values and reported by companies. - FFS: down-selection of the values, - companies are encouraged to provide details for down-selection; [400ms], FFS: 100ms; X. Note1: - Ramp-up time may consist of the procedure for [main radio hardware tune on e.g., boot, memory load and etc.], - Time for sync/re-sync consists of the procedure for [main radio to re-synchronization with the serving gNB etc.], - FFS: X and whether/how to have different values depending on other factors, e.g., signal-to-noise ratio - Companies can report the assumption of X in the initial evaluation. - Ramp up and down energy includes power for ramp-up and ramp-down. Energy consumption for sync/re-sync is separately calculated. - The total time for main radio transition from ultra-deep sleep to active/micro sleep state is the sum of ramp-up time and time for sync/re-sync. - FFS whether/how to define ramp-down time, whether to separately describe the ramp-down energy consumption Note 2: the power state transitions in this table refer to transitions between ultra deep sleep state and active / micro sleep state. Note 3: The values inside of ‘[ ]’ are to be used as starting point of future study on LP-WUS Agreement For R18 LP-WUS/WUR power evaluation in RRC connected mode, the following can be considered, - XR traffic model with evaluation methodologies and assumptions captured in TR 38.838. - eMBB traffic model with evaluation methodologies and assumptions captured in TR 38.840 - Heartbeat traffic models in 3GPP TR 38.875. - Other models are not precluded. Company to further provide the followings, - Parameters (e.g., frame rate, data rate, jitter range, DRX configurations and etc if needed.) - How to use LP-WUS, e.g., LP-WUS to trigger/adapt PDCCH monitoring - Other details if any Agreement - For LP-WUS coverage evaluation, the noise figure of LP-WUR is - Options : [9, 12, 15, 18, 21, 24], Other values can be reported by companies - FFS: how to determine the NF option. - The values provided is for the purpose of studying coverage of LP-WUS, and it can be further revisited depending on the receiver architecture discussion. Agreement For the performance evaluations of LP-WUS candidate designs, it is assumed that - The miss-detection rate (MDR) of LP-WUS [1%], - The false-alarm rate (FAR) of LP-WUS - [0.1%, 1%, 10%] - Other values are not precluded for studying reported by companies - Note: if LP-WUS for wake-up indication consists of two parts or even multiple parts, the proposed MDR/FAR should take into account the reception performance of the two or more parts jointly - The above values applied in both RRC CONNECTED and IDLE/INACTIVE mode. - FFS FAR requirement based on the study outcome of the impact of FAR on power consumption / power saving gain / system overhead - FFS: Note: FAR should be evaluated both in the absence of gNB transmissions and in the presence of transmissions from gNB. Proponent to provide the details. Agreement For system impact analysis, the following performance metrics are considered to be provided, Performance Metric; Note. System overhead; expressed as percentage of used part of all REs for LP-WUS (including guard band or time or others resource used for LP-WUR if any) among all resources Other assumptions related to the system overhead analysis can be reported, e.g., the LP-WUR raw data rate evaluated in the coverage evaluations.. FFS: Capacity impact; [Evaluate the system capacity impact due to introducing of LP-WUS]. FFS: NW power consumption / Energy Efficiency; [Impact of LP-WUS/WUR operation on gNB energy consumption as performance metric in system impact analysis.]. For power and latency evaluation of the LP-WUS, the following performance metrics are considered to be provided. Performance Metric; Note. Power consumption; Relative power consumption in units. The power consumption includes main radio and LP-WUR. For comparison, the relative power consumption and evaluation period for baseline schemes should also be provided, as well as the power saving gain (i.e., percentage of power consumption reduction of the proposed power saving scheme from the baseline scheme).. Latency; For IDLE/INACTIVE state, the latency is the time interval between the data arrival time at the gNB and the time of the first PO UE can [monitor/detect] the paging message - FFS: if UE is not required to monitor a PO after wake-up, e.g., latency is the time interval between the data arrival time at the gNB and the time UE transmits the PRACH after LP-WUS detection. - sync/re-sync for main radio is included For CONNECTED state, TBD. FFS: UPT; FFS Note: it is for connected mode purpose.. Companies to report baseline scheme, e.g., PO monitoring with i-DRX, e-DRX, with or without PEI Companies to report the power consumption / power saving gain considering the FAR impact , latency considering MDR impact Other performance metrics (e.g., mobility) can be reported by companies (if any) Agreement The following is assumed for RRC IDLE/INACTIVE evaluation, Parameters; Value. i-DRX cycle length; 1.28s and other values not precluded and reported by companies, consider both with PEI/ without PEI. e-DRX cycle length; 20.48s, 61.44s and other values not precluded, company to report which value(s) are used. Note: ‘ultra-deep sleep’ state can be assumed for eDRX whenever necessary for baseline UE. Number of POs in Paging Frame; 1. Number of DRXs per PTW; 4. Number of SSB before PO / PEI; 1, 2 or 3, (used for e.g., AGC adjustment, T/F tracking, serving cell and intra-F measurement) company to report which value(s) are used Note: the assumptions is for MR wakes from ‘Deep sleep’. Sync/re-sync after ultra-deep sleep; companies to report the timeline of sync/re-sync and X value, X is the time for sync/re-sync. RRM Measurement; Company to report whether and how the RRM measurement is assumed, e.g., whether RRM performed by main radio or LP-WUR, whether RRM is relaxed or not.. LP-WUS monitoring; Option 1: continuously monitoring Option 2: discontinuously monitoring, with [T] ms as the period for complete an on-and-off cycle, and [D] ms as the active time for monitoring LP-WUS every cycle.. Traffic; Option 1 (baseline): Per UE paging rate (R_E)= ([1%]) or ([0.1%]) or ([0.01%]) or ([0.001%]) within duration Y, [FFS Y is an i-DRX cycle length or an absolute time duration length] - R_G denotes as the group paging rate and R_E denotes as UE paging rate, and 1-R_G=(1-R_E)^N, where N is the number of UEs in the group, and N is [TBD] - FFS: how (R_G, R_E) for e-DRX derived from FFS: Option 2 (optional): Reusing TR 38.875 heart beat traffic model Model; FTP3. Packet size; 100 Bytes. Mean inter-arrival time; 60s (per UE paging rate≈2%). Model RRC connection phase power consumption as follows, RRC connection duration; [30ms]. Relative energy consumption of RRC connection block (Relative power x ms); [=3000]. Other options are not precluded can be reported by companies.. Others; Reported by companies. Agreement For evaluation of the coverage of LP-WUS, the methodology and assumptions in R17 CovEnh SI (described in TR38.830) is reused as baseline. - MIL is used as the metric for LP-WUS coverage evaluation - urban (2.6GHz/4GHz), rural(700MHz) scenario for FR1 are considered to be evaluated, others (e.g., FR2) are not precluded. Note: For IoT/wearables devices, refer to R17 Redcap SI TR38.875 if the assumptions differ from TR38.830. Companies report any other assumptions which differ from the TR38.875/ TR38.830, e.g., Tx and Rx loss Companies are encouraged to compare LP-WUS with at least PDCCH for paging, PUSCH, others are not precluded. FFS: Target coverage of LP-WUS

During RAN1#111meeting [4], further agreements on performance metrics definition, traffic model and LP-WUR power model were made.

Agreement For system impact analysis, the following performance metrics are considered to be provided, Performance Metric; Note. System overhead; expressed as percentage of used part of all REs for LP-WUS (including guard band or time or others resource used for LP-WUR if any) among all resources Other assumptions related to the system overhead analysis can be reported, e.g., the LP-WUR raw data rate evaluated in the coverage evaluations.. Capacity impact; Evaluate the system capacity impact due to introducing of LP-WUS Note: it is for UEs which are in connected mode. Definition is the same as in XR TR.. FFS: NW power consumption / Energy Efficiency; [Impact of LP-WUS/WUR operation on gNB energy consumption as performance metric in system impact analysis.]. For power and latency evaluation of the LP-WUS, the following performance metrics definitions provided for future study Performance Metric; Note. Power consumption; Relative power consumption in units. The power consumption includes main radio and LP-WUR. For comparison, the relative power consumption and evaluation period for baseline schemes should also be provided, as well as the power saving gain (i.e., percentage of power consumption reduction of the proposed power saving scheme from the baseline scheme).. Latency; For IDLE/INACTIVE state, - the latency is the time interval between the data arrival time at the gNB and the time of the first PO UE can monitor the paging message - alternatively, if UE is not required to monitor a PO after wake-up, company to report detailed procedure and definition of the latency . In RAN1#111, there are no definitions being precluded - sync/re-sync for main radio is included. UPT; The definition is the same as in [TR38.840] Note: it is for connected mode purpose.. Companies to report baseline scheme, e.g., PO monitoring with i-DRX, e-DRX, with or without PEI Companies to report the power consumption / power saving gain considering the FAR impact, latency considering MDR impact Other performance metrics (e.g., mobility) can be reported by companies (if any) Agreement Update the IDLE/INACTIVE state traffic model option 1 as follows and remove traffic model option 2, - The traffic arrival is modeled as a Poisson Arrival Process where inter-arrival times are exponentially distributed, the mean arrival time is P = YREF / RE, REF, where - RE, REF= 1%, 0.1%, 0.01% or 0.001% and YREF = 1.28s - Per group paging probability RG = 1 – (1 – RE)N, where N is the number of UEs in the group - FFS: Value of N - For LP-WUS - Both per group and UE paging can be assumed. Note： - For i-DRX with i-DRX cycle duration Y second, - Per UE paging probability RE = 1 – (1 – RE, REF )Y/YREF - Per group paging probability RG = 1 – (1 – RE)N, where N is the number of UEs in the group - For e-DRX with K i-DRX cycles duration, L PTW duration of L i-DRX cycles, and an i-DRX cycle duration Y second - Per UE paging probability is - RE = 1 – (1 – RE, REF )(K-L)Y/YREF for the first i-DRX cycle within the PTW - RE = 1 – (1 – RE, REF )LY/YREF for each of the remaining L-1 i-DRX cycles within the PTW - Per group paging probability RG = 1 – (1 – RE)N, where N is the number of UEs in the group - L=4 (as agreed in RAN1#110bis) Agreement For MR, at least for FR1 evaluation, - Number of SSBs for sync/re-sync for MR is up to 10 - Companies to report timeline and energy consumption - Companies to provide feasibility analysis for transition time and transition energy with aim to converge to one or two set of values in RAN1#112 Agreement The following power model for LP-WUR is used for evaluation for FR1, Power State; Relative Power (unit); Transition energy: (unit multiplied by ms); Ramp-up time TLR, ramp-up (ms). Off; 0.001; [TLR, ramp-up *(PON+POFF)/2]; TLR, ramp-up = FFS, and company to report TLR, ramp-up FFS: Relation between Receiver architecture and its relative power and value of TLR, ramp-up. On; 0.005/0.01/0.02/0.03/0.05/0.1/0.2/0.5/1/2/4 FFS: If other values are needed. FFS: whether further categorization/sub-categorization is needed and how.

Finally, during RAN#112 [5], the following conclusion and agreements were made.

Conclusion: The FAR definition does NOT include the impact of the falsely alarmed for wake-up due to the detection of a LP-WUS which is intended to wake-up/alarm the LP-WUR of another UE within the same UE group Agreement The following characteristics for target use cases are considered in the study item: - IoT cases including e.g., industrial wireless sensors, controllers, actuators and etc, including the following characteristics, - FFS: latency - primary for small form devices - power-sensitive - static, nomadic or limited mobility - Wearable cases including e.g., smart watches, rings, eHealth related devices, and medical monitoring devices etc., - FFS: latency - primary for small form devices, - power-sensitive - low/medium speed, FFS: high speed - eMBB cases including e.g., XR/smart glasses, smart phones and etc., - FFS: latency - devices form is various and not restricted - power-sensitive - low/medium speed, FFS: high speed Note: other use cases/characteristics are not precluded if any. Agreement For evaluation, at least for FR1 MR ultra-deep sleep state, (Ramp-up and down transition energy, ramp-up time) is as follows, - Alt 1: (15000, 400ms) - Alt 2: ([40000], [800ms]) Company to report which alternative they use for which use cases. Agreement For coverage evaluation, the following is used, Number of RX chains at the UE’s MR antenna elements for UE; Case 1: 1 Rx for Redcap Case 2: 2 Rx Case 3: 4 Rx Company to report which case is being used. Further decision on antenna assumption for coverage is FFS.. Number of RX chains antenna elements for LP-WUR; 1 Rx Note: agreed in RAN1#110bis. Scenario and frequency; Urban: 4GHz (TDD), 2.6GHz (TDD) Rural: 4GHz (TDD), 2.6GHz (TDD), 2GHz (FDD), 700MHz (FDD) Rural with long distance: 700MHz (FDD), 4GHz (TDD). Reference data rates for MR eMBB; Urban: PDSCH 10Mbps, PUSCH 1Mbps Rural: PDSCH 1Mbps, PUSCH 100kbps Rural with long distance: DL 1Mbps, UL 100kbps, 30kbps (optional). Reference PDCCH configuration; SCS; 30kHz for TDD, 15kHz for FDD.. Aggregation level; 8, 16 Company to report which case is being used. Further decision on aggregation level for coverage is FFS.. Payload; 40 bits. CORESET size; 2 symbols, 48 PRBs. Tx Diversity; Reported by companies. BLER; 1% BLER,.. Pathloss model (select from LoS or NLoS); Urban: NloS Rural: NloS and LoS. Bandwidth; 100MHz for 4GHz and 2.6GHz. 20MHz for 2GHz (FDD) 20MHz (optional for 10MHz) for 700MHz. (FDD). Channel model for link-level simulation; TDL-C for NLOS, TDL-D for LOS.. Delay spread; Urban: 300ns, optional: 1000ns and companies to provide descriptions for such scenarios Rural: 300ns Rural with long distance: 30ns. UE velocity; Urban: 3km/h Rural: 3km/h, FFS: 120km/h (optional 30km/h) for outdoor. Number of antenna elements for BS; - Urban: 192 antenna elements for 4GHz and 2.6GHz, (M,N,P,Mg,Ng) = (12,8,2,1,1) (optional) 128 antenna elements for 4GHz, (M,N,P,Mg,Ng) = (8,8,2,1,1) - Rural: 64 antenna elements for 4GHz and 2.6GHz (M,N,P,Mg,Ng) = (8,4,2,1,1) 32 antenna elements for 2GHz (M,N,P,Mg,Ng) = (8,2,2,1,1) - Rural: 16 antenna elements for 700MHz (M,N,P,Mg,Ng) = (4,2,2,1,1). Number of TxRUs for BS; gNB architectures to study: - 2 or 4 TXRUs for 2GHz, 700 MHz - 64TxRUs for 2.6 and 4 GHz. - Optional: 32 TXRUs at 2 GHz gNB modeling in LLS for TDL: - Option 1: 2 or 4 gNB RF chains in LLS. - Option 2 (Optional): Number of gNB RF chains = number of TXRUs in LLS. - Companies can report if and how correlation is modelled.. Note: The descriptions above does not change the agreements for coverage in the RAN1#110-bis.

In this contribution, we discuss aspects related to target use-cases and LP-WUS evaluation and perform initial analysis in term of power consumption, latency and system overhead for an example LP-WUS/WUR design (MC-OOK option 4).

# Use cases

The use of LP-WUR is more beneficial for time-delay critical applications with low traffic intensity. In these types of applications, the device needs to listen to the channel often to fulfill the short time delay requirements. The cost of idle channel listening, due to low-traffic intensity, can become the dominant source of energy consumption in the device.  By using a LP-WUR in addition to a main receiver, where the LP-WUR listens for potential traffic on the channel, we can largely reduce the idle channel listening cost and save significant power also in use-cases with tight wake-up delay requirements.

Three target use cases, i.e., IoT use cases (such as industrial sensors, controllers), wearables and eMBB (such as XR/smart glasses, smart phones) are considered in this study where their characteristics is under discussion. In applications such as XR where the device has tight wake-up delay requirements, but the traffic is relatively high, the main receiver needs to be woken up very often to receive or transmit the actual data. In these types of applications, the energy saving from using a low-power WUR is very limited. However, in IoT use cases (such as industrial sensors, controllers), wearables the devices not only have tight wake-up delay requirements but also are power-sensitive and small form factor devices with low traffic intensity. From the set of target use-cases, we propose to prioritize LP-WUS/WUR IoT use cases (such as industrial sensors, controllers) and wearables as the use of LP-WUR is more beneficial for these types of use-cases.

Additionally, in the previous meeting, it was agreed to study LP-WUS for both RRC IDLE/INACTIVE and CONNECTED modes. As described earlier, using LP-WUS/WUR is more beneficial for scenarios where the traffic is low, and the device needs to be reachable in time quite often. Between the two modes, RRC_IDLE/INACTIVE more closely follows the characteristics described above. We, therefore, propose to prioritize to LP-WUS/WUR for devices operating in RRC_IDLE/INACTIVE mode.

Proposal 1 – Prioritize LP-WUS/WUR for power-sensitive, low-traffic, small form factor devices as in IoT use cases (such as industrial sensors, controllers) and wearables where delay requirement or device reachability in time is short.

Proposal 2 – Prioritize LP-WUS/WUR for devices in RRC_IDLE/INACTIVE.

In the following we focus our LP-WUS evaluation on IoT type and wearable use-cases and for devices in RRC_IDLE/INACTIVE mode.

# Evaluation of LP-WUS

Low-power low-complexity design of a LP-WUR typically leads to a higher noise figure and degraded sensitivity, compared to the main receiver. Depending on the LP-WUR architecture, the level of power consumption vs noise figure and sensitivity can vary. During the previous meetings, a set of values have been agreed for LP-WUR noise figure,

- For LP-WUS coverage evaluation, the noise figure of LP-WUR is

  - Options: [9, 12, 15, 18, 21, 24],

Additionally, LP-WUR “on” power has been modelled with a set of values,

- On power model for LP-WUR, used for evaluation for FR1 are 0.01, 0.05, 0.1, 1, 2, 4.

There is, however, no direct mapping between these “on” power values, noise figure values and the LP-WUR architecture. Additionally, even though the power consumption of LP-WUR is much lower than the main receiver, the associated performance loss compared to the main receiver needs to be taken into account in the design of LP-WUS/WUR scheme. For instance, when transmitting the LP-WUS, transmit power may have to be increased. Another approach is to transmit a longer LP-WUS, for instance by employing spreading, allowing LP-WUS to carry more energy without increased transmit power. The choice of the LP-WUR architecture and its characteristics directly impact the LP-WUS design, its signal length and data rate if also the goal is to reach as close as possible to the coverage of the main receiver when it receives PDCCH. This, in return, has impact on the total system overhead and the amount of energy saving using LP-WUR.

To have a better understanding when evaluating LP-WUS, we propose a one-to-one mapping between the LP-WUR architecture, its noise figure and its power consumption.

Proposal 3 – A one-to-one mapping between the LP-WUR architecture, its noise figure and power consumption for a fair evaluation of LP-WUS/WUR.

# Initial evaluation of LP-WUS for MC-OOK

We perform the evaluation in steps, starting with analysis of power consumption, latency and system overhead for a certain LP-WUS design and LP-WUR architecture. In our analysis we compensate for the LP-WUR higher noise figure and subsequently degraded sensitivity by transmitting a longer LP-WUS, for instance by employing spreading.

## Power consumption evaluation

In this section, we analyze the average UE power consumption for a duty-cycled LP-WUR/WUS scheme. For refence, we compare power consumption of the duty-cycled LP-WUS/WUR scheme with a reference scheme where no wake-up signal is used and the main receiver duty-cycles to monitor the channel for potential paging. The reference scheme is basically a Rel 15/Rel 16 UE in RCC_IDLE/INACTIVE where no PEI is available, operating based on DRX and eDRX. The measure we use is average power consumption, calculated as expected energy per duty-cycle period. In the following, we briefly describe the calculation of energy consumption for the LP-WUS/WUR and reference schemes.

LP-WUS/WUR scheme – In this scheme, the device always duty-cycles using its LP-WUR, unless it detects a WUS and switches on its main receiver for receiving a paging. In our calculation no PEI is assumed since a LP-WUS prior to PEI leads to a 4-step wake-up procedure thereby an extensive wake-up delay. In our view, LP-WUS is basically to replace PEI for IoT type use-cases.

The average power consumption at a high level is based on the summation of the energy consumptions of the MR in Ultra-deep sleep and of the LP-WUR when listening plus the energy consumption when operating the MR, either correctly or as a result of a LP-WUS false-alarm,

Average power consumption = (ultra-deep sleep energy consumption + LP-WUR energy consumption when listening + energy consumption when operating the main receiver + LP-WUS false-alarm probability* energy consumption when operating the main receiver) / (e)DRX cycle.

Energy consumption when operating the main receiver also includes the transition energy costs of preparing the main receiver for reception from deep, light and micro sleep states and receiving paging data such as sync.

Reference Scheme – In this scheme, the UE has only the main receiver for its duty-cycling and no LP-WUS/WUR is used to assist the UE in the DRX cycle. Therefore, the total energy consumption is the sleep energy consumption plus the energy cost of listening for paging, including light sleep and micro sleep transition energy cost and sync energy cost, as well as the extra cost resulting from PDCCH/P-RNTI false-alarm,

Average power consumption = (sleep energy consumption + paging reception energy consumption + 
MR false-alarm probability* paging monitoring energy consumption) / (e)DRX cycle.

The sleep energy is the energy cost of deep sleep or ultra-deep, for DRX or eDRX, respectively.

### Detection performance

For the power consumption evaluation, we use the LP-WUS design, and the simulation results presented in our companion contribution [3].  Based on coverage MIL of 142.4 dB, discussed in [6], 240 kbps Manchester-coded OOK symbol rate and a low-power envelope detector, we end up with a LP-WUR power consumption in the range of 100 uW.  Kasami sequences of equal length M and L of 63 bits are selected to represent 504 cell identities and 16 group identities. This translates to spreading factors of 7 and 16 for the two parts of the 126-bit WUS, which has a total time duration of 0.52 msec, for details see [3]. This parameter settings give us an LP-WUS detection probability of 0.985 and a low false-alarm probability of around 10-5.

Using the simulation results described above and the power model calculation assumptions from the previous meeting, we calculate the power consumption for the two schemes. For details of the assumptions used, see the Appendix.

Power consumption calculation without RRM measurement – Let us now start with comparing power consumption of the reference scheme and the LP-WUS/WUR in Figure 4 for different duty-cycle lengths where a shorter duty-cycle means reduced wake-up delay and better device reachability in time and for three paging rates. In this analysis, we assume the UE is stationary and/or in good coverage and the UE does not need to do any serving cell or neighbor cell measurements. We observe that introducing a LP-WUS/WUR in DRX results in a 6-10 times power saving for a paging rate of 10% and in 36-43 times power saving when the paging rate is reduced to 0.1%. In eDRX, using LP-WUS/WUR we can reach as high as 27 times power saving for a 10% paging rate and this increases to up to 85 times for a 0.1% paging rate. Another benefit is the large reduction in cycle length that can be done at a fixed power consumption. For example, at 0.1 units, the cycle length can be reduced by 290 times, leading to a correspondingly reduced wake-up delays.

Figure 4 - Average power consumption per UE as a function of duty-cycle length

Observation 1 – Introducing a LP-WUS/WUR in DRX results in 6-10 times power saving for paging rate of 10% and in 36-43 times power saving when paging rate is reduced to 0.1%. In eDRX, using LP-WUS/WUR we can reach as high as 27 times power saving for 10% paging rate and this increases to up to 85 times for a 0.1% paging rate.

Observation 2 – Introducing an LP-WUS/WUR allows a large reduction in cycle length at a fixed power consumption. For example, at a power consumption of 0.1 units, the cycle length can be reduced by 290 times, leading to correspondingly reduced wake-up delays.

Power consumption calculation with RRM measurement – Now we compare power consumption for scenarios where the UE also needs to perform measurements. The results are shown in Figure 5 for two measurement periodicities, i.e., every DRX cycle and every 10 DRX cycles, and a paging rate of 0.1%. When calculating the power consumption, we assume measurements cannot be supported by the LP-WUS and the main radio needs to wake-up for measurement purposes. We observe that when the LP-WUS itself does not support mobility measurements, the amount of power saving is limited, and we cannot reach as high a power saving gain as when LP-WUR is also used for measurement purposes, see Figure 4. This also means we cannot have the same reduction in cycle length as can be achieved using an LP-WUR.

Figure 5 - Average power consumption per UE as a function of duty-cycle length, measurements in all cases are performed by the main radio.

Observation 3 – Power saving gain is limited if the main receiver needs to wake up to perform measurements.

Proposal 4 – Consider low-power mechanism to support mobility and cell re-selection mechanism for UEs with LP-WUR.

## Latency

When operating based on a duty-cycled scheme, the average communication delay from when data is available at the gNB until paging/data is correctly received by the UE depends on the sleep duration in the duty-cycle length, contributing error events such as false-alarm or miss-detection, and the transition time before the main receiver is ready for PDCCH reception. At a high level, the average delay can be expressed as

Average delay = ½ sleep time + transition time + signal miss detection × average time for re-transmission

The average communication delay in the LP-WUS/WUR scheme is longer than the baseline scheme, i.e., Rel. 15/Rel. 16 scheme, if both are configured with the same duty-cycle length to fulfill a certain delay requirement. This is since the main radio goes to ultra-deep sleep state to further save power and therefore a longer transition time is needed to wake-up the main radio from ultra-deep sleep. Additionally, any miss-detection error of the LP-WUS results in an extra delay.

When designing the LP-WUS/WUR, these aspects need to be taken into account. While duty-cycled operation is necessary to reduce the total power consumption, in certain scenarios with extremely tight delay requirements, the transition time can become very significant and even the shortest DRX cycle can prevent the UE from meeting the tight delay requirement. The use of an always-on LP-WUR can avoid such issues but on the other hand this type of operation is not suitable for all types of use-case scenarios as it leads to unnecessarily high-power consumption. To address this, the operation of the LP-WUR can be configured as an adaptive operation where the LP-WUR can be adaptively operated according to a duty-cycled or an always-on scheme.

Observation 4 – The operation of LP-WUR based on duty-cycling is necessary to reduce the total power consumption. The long transition time to wake-up the main radio from ultra-sleep time together with sleep time of the duty-cycle can prevent some UEs from meeting the delay requirement.

Proposal 5 – Support an adaptive configuration where the UE, depending on its delay requirement, can operate based on an always-on or a duty-cycle scheme.

## System overhead

Introducing new signaling before for instance paging transmission leads to an increase of system resources. As described earlier, a longer LP-WUS is needed to compensate for the LP-WUR performance loss resulting from its ultra-lower power design. Additionally, the length/amount of information being carried by the LP-WUS also influences the number of resources that need to be used at the transmitter. At the transmitter side the mechanism used to multiplex the LP-WUS in an OFDM-based transmitter with other NR signals has impact on the number of resources used. So in summary, the system overhead depends on the followings

  - The LP-WUR architecture characteristics in terms of power consumption and noise figure as well as LP-WUS bandwidth (as it directly influences the LP-WUS design, its data rate and its length).

  - The technique used to multiplex the LP-WUS in the OFDM transmitter.

For instance, Manchester-coded OOK-based LP-WUS used in the initial analysis in this contribution is generated based on “option OOK-4”. This LP-WUS will require a total 6RBs for its transmission, this including some guard band, to wake up a certain group of UEs when it is OFDM embedded using the mechanism described in [3]. This is translated to 6.5% system overhead compared to the reference scheme, where no LP-WUS is used. Using the same LP-WUS design with a different multiplexing technique, however, can result in different system overhead.

Observation 5 – LP-WUS length, the amount information it carries, and the technique used for its multiplexing in an OFDM transmitter impact the number of resources for LP-WUS transmission and its associated system overhead.

# Conclusion

This document has considered the evaluation of LP-WUS. The following observations are made:

Observation 1 – Introducing a LP-WUS/WUR in DRX results in 6-10 times power saving for paging rate of 10% and in 36-43 times power saving when paging rate is reduced to 0.1%. In eDRX, using LP-WUS/WUR we can reach as high as 27 times power saving for 10% paging rate and this increases to up to 85 times for a 0.1% paging rate.

Observation 2 – Introducing an LP-WUS/WUR allows a large reduction in cycle length at a fixed power consumption. For example, at a power consumption of 0.1 units, the cycle length can be reduced by 290 times, leading to correspondingly reduced wake-up delays.

Observation 3 – Power saving gain is limited if the main receiver needs to wake up to perform measurements.

Observation 4 – The operation of LP-WUR based on duty-cycling is necessary to reduce the total power consumption. The long transition time to wake-up the main radio from ultra-sleep time together with sleep time of the duty-cycle can prevent some UEs from meeting the delay requirement.

Observation 5 – LP-WUS length, the amount information it carries, and the technique used for its multiplexing in an OFDM transmitter impact the number of resources for LP-WUS transmission and its associated system overhead.

The following proposals are made:

Proposal 1 – Prioritize LP-WUS/WUR for power-sensitive, low-traffic, small form factor devices as in IoT use cases (such as industrial sensors, controllers) and wearables where delay requirement or device reachability in time is short.

Proposal 2 – Prioritize LP-WUS/WUR for devices in RRC_IDLE/INACTIVE.

Proposal 3 – A one-to-one mapping between the LP-WUR architecture, its noise figure and power consumption for a fair evaluation of LP-WUS/WUR.

Proposal 4 – Consider low-power mechanism to support mobility and cell re-selection mechanism for UEs with LP-WUR.

Proposal 5 – Support an adaptive configuration where the UE, depending on its delay requirement, can operate based on an always-on or a duty-cycle scheme.

# References

- RP-222644. “Revised SID: Study on low-power Wake-up Signal and Receiver for NR”. RANP#97e. September 2022.

- RAN1#110e-bis chair’s notes

- R1-2302862. “LP-WUS L1 signal design and procedures”. Sony. RAN1#112bis-e. April 2023.

- RAN1#111 chair’s notes

- RAN1#112 chair’s notes

- R1-2300892. “Evaluation of low-power WUS”. Sony. RAN1#112. February 2023.

# Appendix Simulation and calculation assumption

UE power consumption model for main receiver (FR1), from table 6.2-1 in TR 38.875

| Power State | Relative power |
| --- | --- |
| Deep Sleep (PDS) | 0.8 |
| Light Sleep (PLS) | 18 |
| Micro sleep (PMS) | 31 |
| PDCCH-only (PPDCCH) | 50 for same-slot scheduling, 40 for cross-slot scheduling |
| PDCCH + PDSCH (PPDCCH+PDSCH) | 120 |
| PDSCH-only (PPDSCH) | 112 |
| SSB/CSI-RS proc. (PSSB) | 50 |
| Intra-frequency RRM measurement (Pintra) | [60] (synchronous case, N=8, measurement only) [80] (combined measurement and search) |
| Inter-frequency RRM measurement (Pinter) | [60] (neighbor cell search power per freq. layer) [80] (measurement only per freq. layer) Micro sleep power assumed for switch in/out a freq. layer |
| Note: 2 Rx is assumed |  |

Ultra-deep sleep assumption used in the initial calculation

| Power State | Relative Power (unit) | Ramp-up and down transition energy: (unit multiplied by ms) | Ramp-up time |
| --- | --- | --- | --- |
| Ultra-deep sleep | 0.015 | 2000 | 100ms |