3GPP TSG RAN WG1 #112bis								R1-2302862

eMeeting, 17 April – 26 April 2023

Agenda Item 	:	9.11.3

Source 	:	Sony

Title 	:	L1 signal design and procedures for LP-WUS

Document for 	:	Discussion and decision

# Introduction

A new release 18 study item on low-power wake-up signal and receiver was approved in RANP#94e meeting and revised in RANP#97e, with the following objectives

- Identify evaluation methodology (including the use cases) & KPIs [RAN1] - Primarily target low-power WUS/WUR for power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, controllers) and wearables - Other use cases are not precluded - Study and evaluate low-power wake-up receiver architectures [RAN1, RAN4] - Study and evaluate wake-up signal designs to support wake-up receivers [RAN1, RAN4] - Study and evaluate L1 procedures and higher layer protocol changes needed to support the wake-up signals [RAN2, RAN1] - Study potential UE power saving gains compared to the existing Rel-15/16/17 UE power saving mechanisms, the coverage availability, as well as latency impact of low-power WUR/WUS. System impact, such as network power consumption, coexistence with non-low-power-WUR UEs, network coverage/capacity/resource overhead should be included in the study [RAN1] - Note: The need for RAN2 evaluation will be triggered by RAN1 when necessary.

During RAN1#111 meeting, the following agreements were made.

Agreement - Study generation and link performance of multi-carrier (MC)-ASK (including OOK) waveform - study techniques to generate waveform by modulating sub-carriers of CP-OFDM symbol, consider up to M bits transmitted per OFDM symbol, where M is FFS. - Note that above does not preclude DFT-S-OFDMA - Study generation and link performance of multi-carrier (MC)-FSK waveforms - study techniques to generate waveform by modulating sub-carriers of CP-OFDM symbol symbol, consider up to M bits transmitted per OFDM symbol, where M is FFS. - Study link performance of OFDMA-based signals/channels considering at least the existing signal/channel structure (e.g. CSI-RS, SSS) - Other signal/channel structures are not precluded - For next meeting, companies to provide input on aspects to consider that might impact link performance Agreement For the purpose of study, the BW of one LP-WUS is not greater than X (FFS X is 5 or 20) MHz for FR1, study further - whether BW of LP-WUS is configurable (implicitly or explicitly) - size of guard band [FFS: within or outside of BW X], if any - whether there is different X for Idle, Connected, Inactive modes FFS: Whether FR2 is included in the scope of LP-WUS SI Agreement For a UE support LP-WUR in IDLE/INACTIVE mode, - Study how to reduce UE power consumption due to existing RRM measurement requirements at least for mobility support, - study feasibility of RRM measurements performed by LP-WUR, at least for serving/camping cell, based on signals detected by LP-WUR - FFS: measurement metric - FFS: whether and how to identify cell/ tracking area - FFS: need for neighbouring cells - FFS: need for relaxation of existing RRM measurement requirements (for UE)

In RAN#112 meeting, the following agreements were made.

Agreement For MC-ASK waveform generation, where K is size of iFFT of CP-OFDMA, N is number of SCs used by LP-WUS including potential guard-bands, study further - Option OOK-1: Single-bit in 1 OFDM symbol, SCs of LP-WUS are - OOK=1 means all SCs are modulated - OOK=0 means all SCs are zero power (from base-band point of view) - Option OOK-2: Parallel M-bit OOK in frequency domain, - N SCs of LP-WUS is further separated into M segments (M=2 in Figure) possibly with guard-bands in-between and/or around - OOK=1 means all SCs in segment are modulated - OOK=0 means all SCs in segment are zero power (from base-band point of view) - FFS architecture. - Option OOK-3: Multi-tone single-bit OOK - N SCs of LP-WUS is separated into L segments (L=2 on Figure) without guard-bands in-between segment, but possibly around - OOK=1 means 1 sub-carrier (known by UE) of each segment is modulated, rest of SC is zero power (from base-band point of view) - OOK=0 means all SCs in all segments are zero power (from base-band point of view) - FFS architecture - Option OOK-4: Transform M-bit OOK in time domain - N SCs of OOK-1 are generated by a transformation (DFT/Least square) - N’ samples are generated from M-bits - signal modification may or may NOT be used - truncation or other additional modification may or may NOT be used, if not used, N is the same as N’ - N’ can be the same as K - FFS modulated SCs are e.g. QAM symbols, sequences or other signals - Companies to report their assumptions - potential guard-band SCs are zero power (from base-band point of view) - [optionally, 2 additional segments, one always modulated and one always zero power (from base-band point of view) can be transmitted] - Other options are not precluded (e.g. OOK-1 with multiple bits in one OFDM symbol) Agreement Study synchronisation signal used by LP-WUR, if needed, based on - Option 1: aperiodic signal transmitted as part of LP-WUS - FFS: Whether the signal can additionally be transmitted separately from LP-WUS - Option 2: periodic signal transmitted separately from LP-WUS - Option 3: Option1 + Option2 Agreement For M-bit MC-FSK generation study further the following options - Option FSK-1: N SCs of LP-WUS are separated to M pairs of segments with potential guard-bands in-between and around. - segment comprises one sub-carrier or multiple contiguous SCs - in a pair of segments one segment is modulated, other segment is zero power (from base-band point of view) - Option FSK-2: N SCs of LP-WUS are separated to 2^M segments with potential guard-bands in-between and around. - segment comprises one sub-carrier or multiple contiguous SCs - one segment from 2^M segments is modulated, other segments of SCs are zero power (from base-band point of view) - M >0 - N >1 - Study how to generate segment in time domain, e.g. OOK-1 or OOK-4 - Other options are not precluded. Agreement For MC-ASK or MC-FSK waveform generation, SCS of a CP-OFDM symbol used for LP-WUS generation can be the same as SCS used for other NR transmissions in CP-OFDM symbol overlapping in time with, study whether SCS can be different, also study - FDM/TDM multiplexing with other NR transmissions - link performance - impact to legacy UEs - impact on gNB Agreement Study further pros and cons of the following monitoring behaviors of LP-WUR - Option1: Duty cycle, corresponds to LP-WUR switches between ON/OFF states - Option2: Continuous monitoring, corresponds to LP-WUR is ON all the time Agreement Study potential measurement metric used for RRM measurements performed by LP-WUR. - examples of measurement metric are signal quality, signal power, detection rate of LP-WUS/synch signal - companies to report assumption of signal used for measurements

Our initial evaluation of an OFDM/multi-carrier-OOK based LP-WUS L1 design was provided in [2]. This document provides further details on this OFDM-OOK based design and an OFDM sequence-based LP-WUS design. This document also considers L1 and higher layer procedures required to support LP-WUS operation.

# L1 signal design

The low-power wake-up signal (WUS) should be designed to have certain properties:

- The WUS should avoid causing interference to the existing OFDM transmission – this can be achieved by the signal having a similar mapping as OFDM.

- The WUS should be designed so that it can carry a minimum amount of information.

- The total energy consumption of WUR during WUS detection is minimised.

- The total system overhead is minimized,

- Using the WUS to acquire some degree of synchronization when waking up from sleep mode is advantageous both for the WUS detection and also power consumption. We do not need to decode SSBs before detecting the WUS for example.

- The WUS structure should be configurable to support different conditions, e.g., coverage level, data capacity etc.

- The WUS should have high detection probability and minimum false alarm rate even in poor radio conditions.

- The WUS should be uniquely identifiable as emitted from a particular cell.

- The WUS should identify which UE(s) or group of UEs it is targeted at.

The following sub-sections contain analyses of an OOK-based design for LP-WUS and an OFDM-based design for LP-WUS. Both LP-WUS types should be considered in this study item, allowing conclusions on the most suitable LP-WUS form to be made.

# Interference-free OFDM embedding of OOK Design for LP-WUR

When designing LP-WUS, to ensure low power detection and decoding of the WUS, the following properties should be considered:

- non-coherent modulation to allow for the low-power design of the receiver,

- compensation for the wake-up receiver performance (sensitivity) loss due to extreme low-power design of the LP-WUR and co-channel interference for example.

To tailor LP-WUR/WUS scheme to the NR protocol, the LP-WUS transmission needs to be compliant with the transmission principles of NR, i.e., based on OFDM transmission. In other words, to avoid interfering with the OFDM transmission, the LP-WUS needs to be orthogonal in the same way as OFDM sub-carriers are, despite being based on different modulation principles. In the following, we discuss LP-WUS structure, its transmission scheme, and its base-band processing.

LP-WUS design and structure – To allow for the use of low-power low-complexity WUR designs, a simple non-coherent modulation scheme like on-off keying (OOK), frequency shift keying (FSK), or pulse position modulation (PPM) is often used. Among these simple modulation schemes, we choose to consider Manchester coded OOK modulated sequences in this section as they can be detected at extremely low power consumption levels. Different candidate architectures for the reception of OOK based LP-WUS are proposed and they are still under discussion. Low-power low-complexity design of a LP-WUR typically lead to a higher noise figure and degraded sensitivity, compared to the main receiver. Depending on the LP-WUR architecture, the level of power consumption vs noise figure and sensitivity can vary. The degraded sensitivity inevitably leads to higher raw bit-error rates (BERs). To compensate for the resulting high raw BER on the channel, we use longer signal sequences containing more total energy and suitable low power digital base-band processing (DBB), allowing for low detection error, i.e., miss-detection and false-alarm, probabilities. This lengthening of the signals can also be used for carrying additional information, beyond the pure wake-up trigger. In this LP-WUS design, we embed cell identity and a wake-up group identity, which will significantly reduce unnecessary wake-ups. The first part which carries the cell identity can also be used as a preamble, identifying whether there is a WUS on the channel or not. Only after detection of the first part, the UE proceeds with decoding of the second part of the LP-WUS. The sequences are chosen from sequences such as m-sequences with good correlation properties resulting in low miss detection and false-alarm probabilities. Figure 1 shows the general signal structure used in this section of the contribution for an OOK-based LP-WUS containing sequences of length M and L bits, representing cell identity and wake-up group identity, respectively. The address spaces of these two sequences are typically much smaller than the M and L bits in the sequences, providing a degree of spreading that reduces error probabilities. The first part of the LP-WUS can be also used for synchronization purposes.

Figure 1 - low-power wake-up signal (LP-WUS) structure

Proposal 1 – For OOK-based LP-WUS, support LP-WUS designs compensating for LP-WUR performance loss using spreading.

Proposal 2 – For OOK-based LP-WUS, support LP-WUS structure including cell identity and wake-up group identity.

Transmission scheme – To avoid LP-WUS interfering with the OFDM transmission, the OFDM transmitter can be modified to also transmit OOK modulated sequences designed for non-coherent LP-WUR, on a subset of the carriers, thus embedding the LP-WUS correctly and ensuring orthogonality. This scheme is basically based on the same principle as in “MC-OOK option 4” agreed as one of the option to for the generation of MC-OOK waveform in the previous meeting.

Figure 2 - Block diagram of modified OFDM transmitter

Figure 2 shows a block diagram of the modified OFDM transmitter, including the two blocks used to create the embedded WUS. The first block (A) is a sectioning block creating sections of the target LP-WUS and is only used if the length of the LP-WUS becomes longer than one OFDM symbol. The WUS shaping block (B) then shapes each LP-WUS section so that the resulting signal as output from the OFDM modulator becomes as close to the target LP-WUS as possible using the designated sub-carriers. Step (C) to (D) adds CPs to maintain orthogonality in channels with time dispersion.

This transmission scheme has advantages compared to the other MC-OOK waveform generation candidates in terms of system overhead, latency and low-power consumption. Assume the LP-WUS is L bits long with BWx transmission bandwidth. The length of LP-WUS for MC-OOK option4 is L/K times lower than the one in MC-OOK option1 where K is the number of bits transmitted in one OFDM symbol. This results in lower system overhead, delay and total power consumption. For MC-OOK option2 and option3, system overhead and delay can become the same. The LP-WUR for reception of these two waveforms, however, are more complicated and thereby have higher power consumption that the one used for MC-OOK option4 reception. The high LP-WUR power consumption also results in high total power consumption.

Proposal 3 – Support MC-OOK option 4 mechanism to embed LP-WUS within OFDM transmissions without creating interference to other OFDM transmissions.

DBB processing – The employed LP-WUR DBB design, consisting of two detector units used to detect the above LP-WUS design, is shown in Figure 3. The detectors contain a cell identity correlator (CIC) of length M and a wake-up group identity correlator (GIC) of length L. First the CIC is active and whenever its output exceeds the threshold, the cell identity is detected, then the next L incoming bits are sent to the GIC for group identity detection. If the output of the GIC also exceeds its threshold we declare LP-WUS detection, and the main receiver is woken up.

Figure 3 – Digital baseband (DBB) block diagram

Based on coverage MIL of 142.4 dB, discussed in [3], 240 kbps Manchester-coded OOK symbol rate and a low-power envelope detector, we end up with a LP-WUR power consumption in the range of 100 uW and a raw channel BER of 0.15. Using the 0.15 BER and the LP-WUS design described above we simulate the digital baseband processing to find LP-WUS detection characteristics. In this initial performance analysis, Kasami sequences of equal length M and L of 63 bits are selected to represent 504 cell identities and 16 group identities. This translates to spreading factors of 7 and 16 for the two parts of the 126-bit WUS, which has a total time duration of 0.52 msec. With these parameter settings the maximum outputs from the correlators are equal to the sequence length of 63. The receiver operating characteristics for the cell identity detector when changing threshold γ1 from 0 to 62 and keeping γ2=45, is shown in Figure 4.

Figure 4 - Simulated receiver operating characteristics for LP-WUS

There is one curve for the single cell case and one where the LP-WUS interference from neighbour cells is also included. The two curves are, for this parameter setting, essentially identical. The chosen point of operation is at γ1of 46, which gives us an LP-WUS detection probability of 0.985 and a low false-alarm probability of around 10-5. When performing this simulation, we have taken into account a drift compensation, chosen cautiously to be 10 times that of the ultra-deep sleep of the main receiver.

Observation 1 – To compensate for performance loss of LP-WUR due to its ultra-low power design, longer signal sequences containing more total energy and suitable low power digital base-band processing (DBB), allow for low detection error probabilities.

# OFDM-Sequence-based Design

The OFDM-sequence-based LP-WUS can consist of a preamble sequence optionally followed by a data part as illustrated in Figure 5 below.  The optionality of the data part depends on how much signalling information the WUS is expected to carry. The preamble part can use sequences such as ZC, Gold etc sequences with high autocorrelation properties to maximise detection probability and low cross correlation to minimise false detection probability. Consideration should also be given to PAPR of the LP-WUS signal in the choice of sequences. Further, the preamble part is composed of multiple OFDM symbols modulated with the same preamble sequence. Each LP-WUS OFDM symbol is preceded by a cyclic-prefix preferably longer than is used in transmissions to the main receiver. Using the same sequence across multiple OFDM symbols allows detection of the preamble in the presence of significant frequency offsets. The LP-WUS information symbols carry data in a cyclic shift of the OFDM symbol that provides decoding resilience to co-channel interference and other additive noise during detection of the preamble. Some of the information carried in the LP-WUS information symbols can be used to verify the detected preamble thereby lowering significantly the false alarm rate of the WUS detector.

Figure 5 - OFDM Sequence based LP-WUS design

To modulate the sequences either in-band or out of band of the NR main carrier, elements of the sequence are used to modulate each OFDM subcarrier as illustrated in Figure 6. This example shows an LP-WUS sequence composed of 63 elements which uses 8 RBs of the OFDM signal. The same set of subcarriers are modulated with the same sequence for Np OFDM symbols of the preamble and then a further Nd symbols for the information part. Np depends on the minimum operating SINR that is required whilst Nd additionally depends on how many bits are to be carried in the information part.

Figure 6 - : LP-WUS OFDM modulation

For the information symbols part of the LP-WUS, binary modulation such as QAM can be used but, as this is an amplitude modulation, it is quite susceptible to noise and interference. So instead, the data part can use the same sequence as the preamble part wherein data is carried in a cyclic shift or other property or perturbation of the sequence. For example, if b bits of data are to be carried in the LP-WUS data symbols, each symbol is subjected to one of 2b cyclic shifts.

Carrying the data in properties of the sequence changes the data detection into a sequence detection process which is much more immune to noise and interference than amplitude modulations. Sequence detection for the data part requires coherent detection, but the sequences are deployed in a manner that allows easy frequency and time synchronisation at the preamble detection stage. Furthermore, as WUS has to be cell-dependent, different sequences for the WUS preamble can be deployed to different cells.

LP-WUS Design Evaluation

We have evaluated an example LP-WUS signal design. In this example, the LP-WUS signal uses one of 32 possible ZC sequences. Each ZC sequence is 157 elements long and is generated with a unique root that corresponds to a targeted group of UEs. To complete the LP-WUS design, the ZC sequence is multiplied by a PN sequence that uniquely identifies the cell that emits the LP-WUS signal for example by initialising its pseudo-random sequence generator with the cell-ID. In the evaluation, each of the Nd data symbols of the LP-WUS carries 5 bits of information. The 5-bit value identifies the root of the ZC used in the preamble. At the LP-WUR of the UE, this is used to confirm the detected ZC sequence thereby ensuring that the false alarm rate (wherein the WUR detects an LP-WUS targeted at the UE where, in reality, there was none) is effectively zero. Table 1 summarises the test conditions.

| Condition | Values | Notes |
| --- | --- | --- |
| (Np, Nd) | (4,4); (8,4); (16,4); (4,8); (8,8); (16,8) | Number of preamble and data symbols |
| WUS duration(us) | 333.33, 500, 833,3, 666.67, 1000 | Duration of (8,4) and (4,8) is same, 500us. |
| Channels | AWGN, TDL-C @ 3km/h |  |
| SNR (dB) | (-12, -9, -6, -3, 0) | Range for reasonable noise figures |
| SCS | 30kHz |  |
| CP | 8.33us | 25% of symbol duration @ 30kHz SCS |
| BW | 4.71MHz |  |
| Band gap width | 1.485MHz | Maximum (49.5 REs) |

Table 1: Link-layer simulation conditions for LP-WUS

The OFDM sequence-based design of LP-WUS as described in Table 1 was evaluated as part of the design process to gauge the WUS preamble detection capabilities. In all the simulations, the FAR is zero because of the confirmation of the LP-WUS sequence transmitted in the data symbols. The receiver is then evaluated with different values for (Np, Nd), SNR in AWGN and TDL-C at 3km/h speed. The simulated performance is shown in Figure 7 and Figure 8. The ultimate design goal is to minimise both misdetection rate (MDR) and false alarm rate (FAR).

Figure 7: LP-WUS performance in AWGN

Figure 8: LP-WUS performance in TDL-C @ 3km/h

The general trend illustrates that the best values for (Np, Nd) are {(4,8), (8,8), (16,8)} in AWGN and {(4,8), (16,8), (8,8)} in TDL-C channels.  Minimising the number of WUS symbols translates to reducing the amount of resources allocated for the LP-WUS and the amount of processing power needed to detect the WUS at the UE. So for this particular LP-WUS design (Np, Nd) = (8,8) is a good compromise for both channel types. Given the near zero MDR performance at SNR = -3dB, this suggests that for a target MDR of below 5%, a WUR for this LP-WUS design can operate with a noise figure of as much as 12dB.

In this section, we have studied an OFDMA-based LP-WUS design, according to the agreement made in RAN1#111:

- Study link performance of OFDMA-based signals/channels considering at least the existing signal/channel structure (e.g. CSI-RS, SSS) - Other signal/channel structures are not precluded

Proposal 4 – For OFDMA-based LP-WUS, support LP-WUS designs consisting of N WUS preamble / detection symbols and M WUS information symbols. Values of N and M can be chosen for a suitable SNR operating point.

Proposal 5 – For OFDMA-based LP-WUS, support LP-WUS structure including cell identity and wake-up group identity.

# L1 Procedures

In this section, we touch on the following issues related to LP-WUS:

- LP-WUR monitoring behaviours: duty-cycled vs continuous

- Low-power synchronization signal

- Reliability of LP-WUS

LP-WUR monitoring behaviours: duty-cycled vs continuous monitoring

At RAN1#112 Athens, the following agreement was made:

Agreement Study further pros and cons of the following monitoring behaviors of LP-WUR - Option1: Duty cycle, corresponds to LP-WUR switches between ON/OFF states - Option2: Continuous monitoring, corresponds to LP-WUR is ON all the time

In our companion contribution [5], we consider the effect of duty-cycled LP-WUR / LP-WUS operation and observe that :

- The operation of LP-WUR based on duty-cycling is necessary to reduce the total power consumption.

- The long transition time to wake-up the main radio from ultra-deep sleep together with sleep time of the duty-cycle can prevent some UEs from meeting the delay requirement

In contrast, continuous monitoring would have a higher power consumption but would allow for a lower latency. Given that there are more “monitoring opportunities” in the continuous monitoring case, the false alarm rate for continuous monitoring may be higher.

Hence, option 1 and option 2 have the following pros and cons:

| Monitoring behaviour | Pros | Cons |
| --- | --- | --- |
| Option 1: duty-cycled | Lower power consumption | Higher latency |
| Option 2: continuous | Higher power consumption | Lower latency [potentially] higher false alarm rate |

Given that both duty-cycled monitoring and continuous monitoring both have their pros and cons, it should be possible to configure the LP-WUR to support either duty-cycled monitoring or continuous monitoring. Hence, we make the following proposal:

Proposal 6 – Support an adaptive configuration of LR monitoring behaviour, where the UE, depending on its delay requirement, can operate based on a continuous or a duty-cycle scheme.

Minimisation of LP-WUR ON duration

The following agreements were made in RAN1#112 Athens:

Working assumption: - For evaluation of LP-WUR frequency and time errors, the following is used, Parameter; Value. LO XO Oscillator max frequency error [ppm], LO XO Oscillator frequency drift [ppm/s]; option 1: (200, 0.1) option 2: (50, 0.1) option 3: (10, 0.05) option 4: (5, 0.05) Other values are not precluded for studying, reported by companies. RTC max frequency error [ppm]; 20. - Company to report how to use the clocks for LR on/off states

It is our understanding that the RTC and LP-XO will be used as follows:

- LO/XO is used to detect and demodulate the LP-WUS, including signal processing operations. The LO/XO only needs to run during the LP-WUS monitoring window. For continuous monitoring of LP-WUS, the LO/XO would run continuously.

- RTC is used to control the times at which the LR needs to wake up to monitor an LP-WUS monitoring occasion. For continuous monitoring of LP-WUS, the RTC is not needed as the LR is always awake.

Observation 2: RTC is used to control the time at which the LR needs to wake up to monitor the LP-WUS monitoring occasion.

In the FLS for RAN1#112 for AI9.13.1 [6], the FL made the observation that:

The impact of frequency error/drifting will impact the WUS detection performance from following two perspectives - UE may need to start search for sync sequence in advance to handle time drift caused by frequency error - the filter is not placed at the right frequency which results the desired signal not fully included or the adjacent NR signal not fully rejected.

In the following, we discuss how the timing drift impacts the need to start search for the sync sequence in advance. This issue is discussed in AI9.11.3 since the timing drift impacts the need for LP-SS support, where LP-SS operation is a L1 procedure.

Since the LP-WUR is not continuously synchronized with the NR waveform, the free-running LP-WUR clock will build up an ever-increasing timing error with respect to the timing of the NR waveform. The issue is illustrated in Figure 10 where is it assumed that the LP-WUR has a timing drift rate relative to the gNB of Tdrift_rate and the DRX cycle length is TDRX. The timing drift rate corresponds to the clock / oscillator drift. The LP-WUR is assumed to be synchronized to the gNB’s timing at paging occasion A. At the next paging occasion, B, the LP-WUR’s timing has drifted by an amount Tdrift_rate × TDRX. The LP-WUR hence has to turn on early and turn off late in order to ensure that its LP-WUS monitoring window fully covers the gNB’s LP-WUS transmission window (the window of time over which the LP-WUS might be transmitted). The extent to which the LP-WUR turns on early and turns off late can be based on design assumptions about the potential level of inaccuracy of the LP-WUR’s clock. The timing drift, and hence the size of the LP-WUR’s LP-WUS monitoring window, keeps increasing (as shown in paging occasion C) until the LP-WUR re-synchronises with the gNB, for example by decoding an LP-WUS. The increase in length of the LP-WUR’s LP-WUS monitoring window increases the power consumption of the LP-WUR.

Observation 3: The length of the LR’s LP-WUS monitoring window increases due to the timing inaccuracy of the RTC in the duty-cycled monitoring case.

Observation 4: The length of the LR’s LP-WUS monitoring window is minimized when the LP-WUR has recently synchronised to the network.

Figure 10 – Timing error built up at the LP-WUR due to timing drift of the LP-WUR’s low accuracy free-running clock

If LP-SS is not supported in some form, the length of the LP-WUS monitoring window will depend on the RTC accuracy and the interarrival time of paging messages (assuming the LR re-synchronises when it receives an LP-WUS preceding a paging message).

For example, for the case where the UE is paged with a mean inter-arrival time of 128 seconds, and for an RTC accuracy of 20ppm, the LP-WUS monitoring duration (and hence LP-WUR ON duration) will be extended by 2 x 128 x 20e-6 = 5.1ms. For longer inter-arrival times, the LP-WUR ON duration increases more significantly. This increase in LP-WUS monitoring duration is significant in comparison to the length of the LP-WUS monitoring window and the LR ramp-up time.

The effect on LR power consumption of UE LP-WUS monitoring window extension due to lack of synchronization was analyzed. The analysis assumptions are provided in Table 2 and the results are provided in Table 3 and Table 4.

Table 2 – Assumptions for analysis of power consumption impact of LP-WUS monitoring window extension

| Parameter | value |
| --- | --- |
| LR ramp up time (ms) | 1 |
| LR power consumption (unit) | 1 |
| LR power consumption in OFF state (units) | 0.001 |
| LP-WUS monitoring window (ms) | 10 |
| RTC timing error | 20ppm |
| iDRX length (ms) | 1280 |
| LP-SS periodicity (ms) | 1280 |
| Time to receive LP-SS (ms) | 1 |
| Paging traffic model | RE, REF= 1%, 0.1%, 0.01% or 0.001% and YREF = 1.28s |
| Number of UEs per paging group | 10 |

Table 3 – LR power consumption assuming UE-specific paging

|  | LR power consumption (units) |  |
| --- | --- | --- |
| Per UE paging rate, RE_EREF (%) | Without LP-SS | With LP-SS |
| 1 | 0.011 | 0.010 |
| 0.1 | 0.029 | 0.010 |
| 0.01 | 0.209 | 0.010 |

Table 4 – LR power consumption assuming group paging (10 UEs / group)

|  | LR power consumption (units) |  |
| --- | --- | --- |
| Per UE paging rate, RE_EREF (%) | Without LP-SS | With LP-SS |
| 1 | 0.009 | 0.010 |
| 0.1 | 0.011 | 0.010 |
| 0.01 | 0.029 | 0.010 |
| 0.001 | 0.209 | 0.010 |

It is apparent that if the LR re-synchronises itself only on the basis of LP-WUS that are transmitted as a consequence of paging messages, the RTC inaccuracy will increase the LP-WUR ON duration and this will negatively impact LR power consumption. It is hence proposed that LP-SS is supported, since LP-SS allows the LR to re-synchronise independently of LP-WUS associated with paging messages.

Observation 5: LP-SS reduces LR power consumption. The power consumption reduction is most pronounced when the paging rate is lower.

Low power synchronization signal (LP-SS)

At RAN1#112 Athens, the following agreement was made:

Agreement Study synchronisation signal used by LP-WUR, if needed, based on - Option 1: aperiodic signal transmitted as part of LP-WUS - FFS: Whether the signal can additionally be transmitted separately from LP-WUS - Option 2: periodic signal transmitted separately from LP-WUS - Option 3: Option1 + Option2

An LP-SS can provide the following functionalities:

- Minimise the LP-WUR ON duration, as discussed above.

- Simplify LP-WUS decoding as the LR will already be synchronized when it attempts to decode LP-WUS

- Support RRM measurements (such as RSRP and RSRQ-like measurements)

The following aspects of LP-SS need to be considered:

- Timing relation to LP-WUS. In particular, it needs to be considered whether LP-SS is transmitted as:

  - Option 1: aperiodic signal transmitted as part of LP-WUS

  - Option 2: periodic signal transmitted separately to LP-WUS

- Structure of LP-SS: whether LP-SS is sequence-based or is signalled as part of an LP-WUS payload

- Required periodicity assuming clock / oscillator drift

Timing relation of LP-SS to LP-WUS

The paging occasions of different UEs are generally offset relative to one another, although multiple UEs can share the same paging occasion. It is expected that each paging occasion will be indicated by an LP-WUS within an LP-WUS monitoring window, as shown in Figure 9. Assigning an LP-SS prior to every LP-WUS monitoring window would lead to a high system overhead (case B), which is likely to be unacceptable. Hence, it should be possible to use one LP-SS to support synchronization with multiple paging occasions (case C). Furthermore, to allow for scheduling flexibility, it should be possible for the gNB to skip transmission of LP-SS (e.g. to allow other users to be scheduled).

Figure 9 - Potential LP-SS locations relative to LP-WUS monitoring windows and Pos

The benefit of transmitting LP-SS periodically is that the LR knows a window within which it should monitor for LP-SS. If the LR does not observe LP-SS within this window, it understands that the LP-SS won’t occur outside the window and can safely go to sleep again. The LR can hence monitor this window and then go to sleep outside this window, reducing LR power consumption. When LR power consumption is a concern, LP-WUS should be monitored in a duty-cycled fashion, for which a periodic (duty-cycled) LP-SS would be appropriate.

Proposal 7: LP-SS is supported for duty-cycled monitoring of LP-WUS.

When LR power consumption is a lower concern than latency, continuous monitoring of LP-WUS is preferred (see section above). For continuous monitoring, the LR is monitoring the channel anyway and can hence correlate / detect an LP-SS at any time. Hence aperiodic LP-SS transmission is feasible (the LR can detect the LP-SS at any time anyway). In order to minimize the number of channels that the LR has to monitor, it is preferable that the LP-SS is transmitted as a part of LP-WUS.

Proposal 8: For continuously monitored of LP-WUS, consider synchronisation based on an aperiodic signal transmitted as part of LP-WUS.

Structure of LP-SS

It should be possible to monitor LP-SS using the LP-WUR, hence the LP-SS should have a structure that is related to the structure of the LP-WUS. Possible structures for LP-SS that should be further studied include:

- A sequence using the underlying LP-WUS waveform (whether that be an OOK-based, FSK-based or OFDMA-based waveform)

- A bit string that is carried by the LP-WUS. For example, if the LP-WUS conveys an ID of a group of UEs that is to be woken up, one of the group IDs could be reserved to indicate “LP-SS”. If the LP-SS group ID were received, UEs would not have to wake up their main receivers, but could use LP-WUS (that carried the LP-SS group ID) to re-synchronise the LP-WUR timing. An example mapping of group-IDs carried by an LP-WUS could be:

| Group ID | meaning |
| --- | --- |
| 00 | LP-SS |
| 01 | WU for group 1 |
| 01 | WU for group 2 |
| 11 | WU for group 1 and 2 |

Proposal 9: Study the following structures for LP-SS:

- Sequence based LP-SS

- LP-SS signalled as a bit string within the regular LP-WUS

Reliability of LP-WUS

When the LP-WUR only monitors LP-WUS in IDLE mode, the LP-WUR doesn’t know whether the LP-WUS is being received reliably or not. If the UE relies on the LP-WUS to wake it up and the LP-WUS becomes unreliable (for example, due to UE movement or fading), the UE can become unreachable. In such a case, it might be better for the UE to switch to monitoring via the MR. Furthermore, if the gNB simply transmits the LP-WUS with fixed parameters, such as a fixed transmission power, the gNB is unable to control the coverage and performance of the LP-WUS. Hence, RAN1 should study the need to monitor the reliability / performance of the LP-WUS. If necessary, RAN1 can then study mechanisms to allow the UE and / or gNB to monitor the reliability of the LP-WUS.

Proposal 10: RAN1 studies the need to monitor the reliability / performance of the LP-WUS.

# Higher layer signalling to support LP-WUR/WUS

When the UE is configured with DRX operation, the UE needs to also perform cell measurements to ensure the signal strength of the serving cell is above a certain threshold. If the signal strength of the serving cell does not fulfill a certain requirement (e.g., due to mobility), the UE enters cell-reselection evaluation procedure. Currently, the signals available for cell re-selection evaluation procedure, i.e., the synchronization signals (SSB) or reference signals of the neighbor cells, can only be measured by the main radio.

Observation 6 - Currently, the signals available for cell re-selection evaluation procedure, i.e., the synchronization signals (SSB) or reference signals of the neighbor cells, can only be measured by the main radio.

A use of LP-WUR in addition to the main radio can significantly reduce the cost of channel listening. Performing serving cell and neighbor cell measurement using the existing synchronization signal or reference signals, however, makes the power saving resulting from using LP-WUR limited, as shown in [3]. We, therefore, need low-power mechanisms to also support measurement and mobility aspects for UEs with LP-WUR

Proposal 11 – Consider low-power mechanisms such as transmission of low-power reference signal to support mobility and cell re-selection mechanism for UEs with LP-WUR.

# 5	Conclusion

In this contribution, we have discussed our views on low-power WUS design and aspects related to L1 and higher layer signalling. We have considered both an OOK-based LP-WUS signal design and an OFMDA-based LP-WUS signal design. Our observations and proposals are listed below.

Observation 1 – To compensate for performance loss of LP-WUR due to its ultra-low power design, longer signal sequences containing more total energy and suitable low power digital base-band processing (DBB), allow for low detection error probabilities.

Observation 2: RTC is used to control the time at which the LR needs to wake up to monitor the LP-WUS monitoring occasion.

Observation 3: The length of the LR’s LP-WUS monitoring window increases due to the timing inaccuracy of the RTC in the duty-cycled monitoring case.

Observation 4: The length of the LR’s LP-WUS monitoring window is minimized when the LP-WUR has recently synchronised to the network.

Observation 5: LP-SS reduces LR power consumption. The power consumption reduction is most pronounced when the paging rate is lower.

Observation 6 - Currently, the signals available for cell re-selection evaluation procedure, i.e., the synchronization signals (SSB) or reference signals of the neighbor cells, can only be measured by the main radio.

Proposal 1 – For OOK-based LP-WUS, support LP-WUS designs compensating for LP-WUR performance loss using spreading.

Proposal 2 – For OOK-based LP-WUS, support LP-WUS structure including cell identity and wake-up group identity.

Proposal 3 – Support MC-OOK option 4 mechanism to embed LP-WUS within OFDM transmissions without creating interference to other OFDM transmissions.

Proposal 4 – For OFDMA-based LP-WUS, support LP-WUS designs consisting of N WUS preamble / detection symbols and M WUS information symbols. Values of N and M can be chosen for a suitable SNR operating point.

Proposal 5 – For OFDMA-based LP-WUS, support LP-WUS structure including cell identity and wake-up group identity.

Proposal 6 – Support an adaptive configuration of LR monitoring behaviour, where the UE, depending on its delay requirement, can operate based on a continuous or a duty-cycle scheme.

Proposal 7: LP-SS is supported for duty-cycled monitoring of LP-WUS.

Proposal 8: For continuously monitored of LP-WUS, consider synchronisation based on an aperiodic signal transmitted as part of LP-WUS.

Proposal 9: Study the following structures for LP-SS:

- Sequence based LP-SS

- LP-SS signalled as a bit string within the regular LP-WUS

Proposal 10: RAN1 studies the need to monitor the reliability / performance of the LP-WUS.

Proposal 11 – Consider low-power mechanisms such as transmission of low-power reference signal to support mobility and cell re-selection mechanism for UEs with LP-WUR.

References

- RP-222644. “Revised SID: Study on low-power Wake-up Signal and Receiver for NR”. RANP#97e. September 2022.

- R1-2210224. “LP-WUS L1 signal design and procedures”. Sony. RAN1#110bis-e. October 2022.

- R1-2302861. “Evaluation of low-power WUS”. Sony. RAN1#112bis-e. April 2023.

- R1-2302213. “Summary#3  of discussions on L1 signal design and procedure for low power WUS”. Moderator (Nordic Semiconductor). RAN1#112. February 2023.

- R1-2302861. “Evaluation of low power WUS”. Sony. RAN1#112bis_e. April 2023.

- R1-2302251. “FL summary #4 (final) of evaluation on low power WUS”. Moderator (vivo). RAN1#112 Athens. February 2023.