3GPP TSG RAN WG1#112bis-e	    												R1-2302866

e-Meeting, Apr. 17th – 26th, 2023

Agenda Item:	 9.13.1

Source:	Sony

Title:	Discussion on UE capability and RRC signaling for UAV UE beamforming

Document for	Discussion and decision

# Introduction

RAN1#112 reached the following agreements regarding the support of UAV UE beamforming in FR1:

Agreement: RAN1 only considers potential UE UAV beamforming capabilities for FR1 with directional antennas at UE side in Rel-18 if such capabilities don’t impact RAN4 requirements. Agreement: If new UE UAV beamforming capabilities for FR1 with directional antennas at UE side are supported, Rel-17 unified TCI framework is considered as baseline. Agreement: Multi-TRP beamforming is not supported for UAV UEs in FR1 in Rel-18. Agreement: UE UAV beamforming for FR1 based on beam switching among fixed directional antennas is supported in Rel-18. - Note 1: new UE capabilities may not be necessary to support beam switching among fixed directional antennas. - Note 2: no RAN4 specification impact is assumed. - FFS: whether updating (e.g. extending to FR1) legacy UE capabilities is needed, and it is not precluded if it is needed. - FFS: whether/how specification may be impacted.

In this contribution, we present our views on the UE capability signaling to indicate and support UAV UE beamforming capabilities as stated in the objective above.

# Discussion

## On UAV UE beamforming capability in FR1

Using beamforming using antenna switching in UAV UEs can help mitigate interference and improve coverage, capacity, and quality of wireless communication in FR1 frequency bands [1-8]. The proposed UE capability and associated RRC signaling for UAV beamforming in 3GPP RAN1 on UAV can provide a standardized approach for enabling this feature, enhancing the efficiency and reliability of UAV applications and other wireless services and devices. However, the impact of UAV beamforming capabilities on RAN4 requirements needs to be carefully considered and addressed to ensure compatibility and interoperability with existing and future network infrastructure. Therefore, the following agreement was made in the RAN1#112 meeting: “RAN1 only considers potential UE UAV beamforming capabilities for FR1 with directional antennas at UE side in Rel-18 if such capabilities don’t impact RAN4 requirements.”

The main challenge is to ensure that UAV beamforming does not cause harmful interference to ground UEs and does not negatively affect network performance. Therefore, to support all beam management features of FR2 also for FR1, there is a need for RAN4 to study the expected impact of supporting UAV UE beamforming, either as an optional or mandatory capability. Refining beam management procedure and extension for FR1 can be addressed in future 3GPP UAV releases, e.g., Rel-19, with dedicated RAN1, RAN2, and RAN4 involvement. Even for limited beam management, RAN1 needs to decide if UAV beamforming is a mandatory or optional capability.

Proposal 1: RAN1 to decide whether or not UAV beamforming in FR1 is an optional or a mandatory capability.

## Indication of UAV UE Beam Characteristics

In TDD mode, one can assume that UAV UEs are capable of beam correspondence since DL/UL channel reciprocity holds. Under beam correspondence, the UE Tx beam can be determined based on the selected UE Rx beam, thereby reducing UL beam training for legacy beam management in FR1 and avoiding resource overhead and delays. Thus, we make the following observation:

Observation 1: Reusing FR2 beam correspondence in FR1 for UAV UEs will minimize specification efforts and achieve decent performance.

However, FR2, in contrast to FR1, is all about TDD. In FDD mode, the DL and UL channels operate on separate frequency bands, making it even more challenging to achieve channel reciprocity than in TDD mode. Therefore, the assumption of beam correspondence for aerial UEs may not be valid in FDD mode. To achieve beam alignment for uplink transmission, uplink beam management another beam management parameter must be supported, e.g., sweeping the antenna beams. This parameter enables the gNB to determine the best TRP Rx beam by measuring the optimal uplink transmission, which allows the UE to select the ideal UE Tx beam. However, if there is some level of channel reciprocity between the DL and UL channels, beam correspondence may still be utilized for FR1 UAV UEs with beamforming capabilities.

To this end, based on the agreement to use fixed directional antennas instead of phased arrays effectively alleviates the beam squint effect and aids beam correspondence for both TDD and FDD.

It can be noted that for FR2, there is a capability signaling, i.e., beamCorrespondenceWithoutUL-BeamSweeping, where a UE indicates a restriction in its beam correspondence to cope with beam squint. However, to minimize specification impact for FR1 in Rel-18 and based on beam correspondence from fixed directional antennas, UL beam sweeping is not needed.

Proposal 2: Beam correspondence without UL beam sweep should be mandatory for Rel-18 FR1 UAV UEs.

To facilitate beam management in FR1, it is convenient to remove the restriction that some UE capability parameters related to beam management apply only to FR2. Several such parameters for Rel-15/16 have been identified, including PDSCH beam switching, beam switching, A-CSI-RS beam switching timing, configured spatial relations, and active spatial relations. In addition, a Rel-17 parameter related to beam misalignment between the DL source RS in the TCI state and the PL-RS has also been identified. These parameters are helpful to avoid wireless channel degradation. Furthermore, using a narrower uplink beam via this process can reduce uplink interference and enhance system performance, highlighting the necessity of extending this parameter to UAV capabilities in FR1.

However, according to the objectives, the refinement of the beam management procedure needs more input with dedicated RAN1, RAN2, and RAN4 involvement. Due to the limited time budget it was not defined as part of the UAV Rel-18 WI objectives. Therefore, we propose not to consider extending parameters for the beam management procedure from FR2 to FR1 in RAN1 UAV Rel-18 WI. The refinement of the beam management procedure and extension to FR1 can be addressed in future 3GPP UAV releases, e.g., Rel-19, with dedicated RAN1, RAN2, and RAN4 involvement.

Proposal 3: Extend the beam management procedure from FR2 to FR1 for UAV Rel-18 with only selected mandatory parameters.

On the beam capabilities description, the UAV should report the capability of one or multiple antenna configurations. The decision to use wider or directional antennas may depend on the gNB configuration, which is based on the channel conditions and traffic load. For instance, a wider beam antenna configuration is adequate when the UAV is flying low without a LOS, but a directional antenna can be used to manage LOS-interference when the UAV is flying high in the air. To help the gNB control the beam, the UE should report the antenna configurations, such as the number of antennas/beams, beamwidth, beam gain of the main lobe, sidelobe, etc. So the beam or antenna configuration adjustment based on aerial UE capability could vary according to the altitude. The UE’s physical beam properties are crucial and beneficial for network interference management, which is a unique requirement for aerial UEs, unlike the legacy terrestrial UE, where beam properties are transparent to the network.

To enable network-assisted beamforming (i.e., beam-shape selection) for UAV UEs, the network must know the UAV UEs’ beamforming characteristics. Currently, there is no established standard for characterizing beamforming capabilities of UEs in NR RAN, but conformance requirements specified for network infrastructure in FR2 have served a similar purpose in RAN4 [9]. These requirements include parameters such as beam, beam centre direction, beam direction pair, beam peak direction, beamwidth, OTA coverage range, and reference beam direction pair for repeaters, IAB nodes, and gNBs operating in FR2. While this parameter set would be sufficient for the network to fully determine directional transmissions made by UAV UEs, a more limited set of features may be advantageous to realize the benefits of network-assisted beamforming.

To enable effective monitoring of UAV UE orientation and, consequently, the orientation of the UAV beams, it may be necessary for the network to supplement its characterization of the UAV UEs’ beam characteristics. This is particularly important for FR1, as previously discussed, where it can facilitate better network operation. Additionally, the orientation information can be combined with existing UE beam management solutions to enhance network performance in FR2.

Proposal 4: Support FR1 capability information containing the number of beams, beam center directions, and post-antenna connector gain of UAV UEs, each beam with a unique beam identity

Proposal 5: Support a UE should be able to report the UAV UE’s orientation to the network, such as the heading/velocity vector, as part of its location information.

# Conclusion

This contribution discussed the UE capability signaling to indicate UAV beamforming capabilities. We made the following observations and proposals:

Observation 1: Reusing FR2 beam correspondence in FR1 for UAV UEs will minimize specification efforts and achieve decent performance.

Proposal 1: RAN1 to decide whether or not UAV beamforming in FR1 is an optional or a mandatory capability.

Proposal 2: Beam correspondence without UL beam sweep should be mandatory for Rel-18 FR1 UAV UEs.

Proposal 3: Extend the beam management procedure from FR2 to FR1 for UAV Rel-18 with only selected mandatory parameters.

Proposal 4: Support FR1 capability information containing the number of beams, beam center directions, and post-antenna connector gain of UAV UEs, each beam with a unique beam identity

Proposal 5: Support a UE should be able to report the UAV UE’s orientation to the network, such as the heading/velocity vector, as part of its location information.

# References

- 3GPP RP-213600, WID on “NR support for UAV,” RAN#94e.

- 3GPP RP-223545, Revised WID: NR Support for UAV (Uncrewed Aerial Vehicles), RAN#98e.

- 3GPP TR 36.777, “Study on enhanced  LTE support for aerial vehicles,” v15.0.0, Dec. 2017.

- 3GPP RP-172826, WID on “Enhanced LTE Support for Aerial Vehicles,” Dec. 2017.

- Yong Zeng (Editor) et al., “UAV Communications for 5G and Beyond,” Wiley 2020, ISBN: 978-1-119-57569-6.

- Tomasz Izydorczyk et al., “Achieving High UAV Uplink Throughput by Using Beamforming on Board,” IEEE Access, vol. 8, pp. 82528-82538, 2020, doi: 10.1109/ACCESS.2020.2991658.

- R1-2209631, ”UE capability and RRC signaling for UAV beamforming,” RAN1#110bis-e, 10–19 Oct. 2022.

- Ayat Z-H. et al, “Uplink coexistence for high throughput UAVs in cellular networks”, IEEE Globecom, Dec. 2022.

- 3GPP TS 38.106, “NR Repeater Radio Transmission and Reception”, v17.1.0 (2022-06).

- 3GPP R1-2301953.