3GPP TSG RAN WG1 #112bis-e		                                                      R1-2302869

e-Meeting, April 17th – April 26th, 2023

Source:	CATT

Title:	Discussion on timing advance management to reduce latency

Agenda Item:	9.10.2

Document for:	Discussion and Decision

Introduction

In RAN #97 e-meeting, a revised WID on further NR mobility enhancements was agreed [1]. In the WID, timing advance management was included in order to reduce handover interruption time. In RAN1 #112 meeting, the following agreements on timing advance management were achieved [2] .

Agreemment For Rel-18 LTM, Random Access Preamble indices and indication of RACH occasions with the associated SSB indices are configured for each candidate cell. Note: the detailed signalling is left to RAN2 Agreement The PDCCH order from the source cell contains the indication of candidate cell. - The reserved bit(s) in DCI format 1_0 for PDCCH order can be used for indication of cell identity Agreement For PDCCH ordered-RACH for candidate cell(s), RAR reception can be configured/indicated - If reception of RAR is not configured/indicated (without RAR) - TA value of candidate cell is indicated in cell switch command - FFS: whether UE should re-transmit PRACH when reception of RAR is not configured/indicated - FFS: how UE determine the transmit power of subsequent PRACH triggered by PDCCH order - If reception of RAR is configured/indicated (with RAR), FFS - whether RAR is received from serving cell or candidate cell - if RAR is received from candidate cell, whether Type1-PDCCH CSS of the candidate cell is configured to the UE - content of RAR - FFS: signaling for configuration/indication of whether RAR needs to be received - UE can report the support combination of with RAR only and without RAR only, where support of one default scheme is the baseline UE approach for LTM - Send LS to RAN2 and RAN3 to check the feasibility about this agreement - Note: Definition of candidate cells is up to RAN2 Agreement - For PDCCH-order based RACH for TA measurement for candidate cells, legacy CBRA is not supported Agreement On whether UE should initiate re-transmit PRACH when reception of RAR is not configured/indicated, down select one from the following alternatives. - Alt 1: UE autonomous re-transmission of PRACH is not allowed (e.g., by setting the number of allowed PRACH transmission to the minimum value of PreambleTransMax=1) - Alt 2: UE autonomous Re-transmission of PRACH is allowed, - The number of PRACH transmission will be defined e.g. set the times of RACH transmission to the minimum value of PreambleTransMax Agreement If reception of RAR is configured/indicated, RAR contains at least TA of candidate cell. - The maximum number of TA values memorized by UE is a UE capability - FFS: whether other parameters such as UE ID, candidate cell ID etc. is contained in RAR Agreement Whether RAR needs to be received is configured by RRC. Agreement Study at least the following issues on PDCCH-order based PRACH for candidate cell that is not UL serving cell, i.e. without PUCCH/PUSCH configured - Whether gap between the DCI and PRACH longer than timeline defined in spec is needed - Any impact/interruption on UL Tx of serving CCs due to the PRACH Tx Working Assumption UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported. - Corresponding UE capability is to be introduced to support UE-based TA measurement - For a UE reports support of this capability, configuration of UE-based TA measurement is supported - FFS: other impacts on RAN1 spec

In this contribution, timing advance managements are discussed with the following outline:  RACH-based solution for TA acquisition is discussed in section 2, which include TA acquisition and TA indication by PDCCH-ordered RACH. In section 3, RACH-less solution for TA acquisition is discussed. A brief conclusion is given in Section 4.

RACH-based solution

In RAN1 #112 meeting, it has been agreed that for PDCCH ordered-RACH for candidate cell(s), RAR reception can be configured/indicated. One further issue to be discussed is whether UE should initiate re-transmit PRACH if reception of RAR is not configured/indicated(without RAR), there are the following alternatives accordingly:

- Alt1: UE autonomous re-transmission of PRACH is not allowed (e.g., by setting the number of allowed PRACH transmission to the minimum value of PreambleTransMax=1)

- Alt2: UE autonomous Re-transmission of PRACH is allowed,

  - The number of PRACH transmission will be defined e.g. set the times of RACH transmission to the minimum value of PreambleTransMax

In the legacy mechanism, if UE doesn’t receive RAR during ra_responseWindow with a RAPID(Random Access Preamble Identifier) that matches the transmitted preamble, UE will increase the preamble_transmission_counter by 1 and retransmits the PRACH preamble with a ramping up power until PreambleTransMax provided by RRC before random access procedure is completed. In LTM, when RAR is not configured/indicated, UE can’t use the RAR detection as the criterion for autonomous re-transmission of PRACH.  When without RAR is configured, NW may or may not decode the PRACH after UE send PRACH. If NW successfully decodes the PRACH, NW will wait until cell switch command to send TA. Otherwise, NW will send PDCCH order to trigger PRACH again.

Allowing UE autonomous re-transmit PRACH, i.e.Alt2, means to ask UE to repeat the PRACH for the max number before cell switch command even NW has already decoded the RACH successfully, in which case, re-transmitting PRACH is a waste of resource.

Proposal 1: Support Alt1. i.e. UE autonomous re-transmission of PRACH is not allowed if reception of RAR is not configured/indicated. NW will re-trigger PDCCH ordered PRACH on the failure detection of the Preamble sent by UE.

Another issue to be discussed is that for PDCCH ordered-RACH, if reception of RAR is configured/indicated, whether other parameters such as UE ID, candidate cell ID is contained in RAR. In the legacy mechanism, if UE receives a RAR with a RAPID that matches the transmitted preamble during ra_responseWindow. UE will consider that NW has successfully decoded the preamble and estimated the TA. UE will apply the TA included in the MAC PDU to the subsequent uplink transmission. Similar mechanism can be applied to the LTM scenario, where if one PDCCH ordered RACH is triggered, UE will use the ra_responseWindow to determine whether the received RAR comes from the candidate cell and further the TA included in the RAR be applied or not. If multiple PDCCH ordered RACH are triggered simultaneously for TA estimation of the candidate cell. Preamble of different ra_PreambleIndex can be included in each PDCCH order for each candidate cell, if UE receives a RAR with a RAPID that matches the corresponding preamble in the PDCCH order for each candidate cell during ra_responseWindow, UE will apply the TA included in the RAR to the uplink transmission of the corresponding candidate cell after cell switch command.

Proposal 2: Don’t contain UE ID/candidate cell ID in RAR if reception of RAR is configured/indicated. Different ra_PreambleIndex can be included in PDCCH order for different candidate cell, if UE receives a RAR with a RAPID that matches the corresponding preamble in the PDCCH order for each candidate cell during ra_responseWindow, UE will apply the TA included in the RAR to the uplink transmission of the corresponding candidate cell after cell switch command.

On whether RAR is received from serving cell or candidate cell if reception of RAR is configured/indicated，in RAN1#112 meeting, there are following alternatives:

- Alt1: RAR is received from serving cell

- Alt2: RAR is received from candidate cell

Both alternatives can work. For Alt1, after candidate cell receives the preambles sent by UE, TA will be estimated and included in the RAR, the RAR is transferred from candidate cell to serving cell. UE will receive RAR from serving cell, using the DL Rx beam of receiving PDCCH order as those in legacy. Alt1 implementation has a minor impact on the current spec. The only thing to be considered is the backhaul latency between serving cell and candidate cell might lead to the RAR MAC PDU beyond the ra_responseWindow, in such case, the length of ra_responseWindow can be extended.

Fig.1. Illustration of the PDCCH-ordered RACH , RAR is received from serving cell

For Alt2, as illustrated in Fig.2. UE will receive RAR from candidate cell. Considering current spec only defines the UE behaviour to receive RAR when the PDCCH order triggered CFRA is for secondary cell[3], spec change is needed to configure an additional type-1 CSS of candidate cell.

Fig.2. Illustration of the PDCCH-ordered RACH, RAR is received from candidate cell

Proposal 3: Slightly prefer Alt1, i.e. RAR is received from serving cell for its minor impact on spec.

RACH-less solution

TA of the candidate cell can be also obtained by RACH-less solution. In RAN1 #111 meeting, there are the following alternatives:

- Alt1: SRS based TA acquisition

- Alt2: RACH-less mechanism as in LTE

- Alt3: UE based TA measurement(including UE based TA measurement with one TAC from serving cell

For Alt2, RACH-less mechanism as in LTE, i.e. using TA=0 or the latest TA value of the source cell for the candidate cell was proposed to make a conclusion in RAN1 #112 meeting. In our opinion, this mechanism can be achieved by implementation and has no additional impact on the current spec. Considering this, it can be naturally extended to LTM.

Proposal 4: RACH-less mechanism as in LTE can be supported by indicating TA=0 or the latest TA value of the source cell.

For Alt3, following assumption was made in RAN1#112 meeting:

Working Assumption

UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported.

- Corresponding UE capability is to be introduced to support UE-based TA measurement

- For a UE reports support of this capability, configuration of UE-based TA measurement is supported

FFS: other impacts on RAN1 spec

Introducing UE-based TA measurement has a minor impact on spec except a new UE capability introduction and NW configure the TA measurement mode as UE-based TA measurement upon the UE capability reporting. As mentioned by other companies, the merit of Alt3 is reducing overhead and implementation issue due to PRACH on candidate cell.

Proposal 5:  The following working assumption is confirmed

Working Assumption UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported. - Corresponding UE capability is to be introduced to support UE-based TA measurement - For a UE reports support of this capability, configuration of UE-based TA measurement is supported FFS: other impacts on RAN1 spec

Conclusions

In this contribution, we focus on the TA management in L1/L2 based inter-cell mobility, TA acquisition and indication for RACH-based solution as well as RACH-less solution are mainly discussed. We have the following proposals:

Proposal 1: Support Alt1. i.e. UE autonomous re-transmission of PRACH is not allowed if reception of RAR is not configured/indicated. NW will re-trigger PDCCH ordered PRACH on the failure detection of the Preamble sent by UE.

Proposal 2: Don’t contain UE ID/candidate cell ID in RAR if reception of RAR is configured/indicated. Different ra_PreambleIndex can be included in PDCCH order for different candidate cell, if UE receives a RAR with a RAPID that matches the corresponding preamble in the PDCCH order for each candidate cell during ra_responseWindow, UE will apply the TA included in the RAR to the uplink transmission of the corresponding candidate cell after cell switch command.

Proposal 3: Slightly prefer Alt1, i.e. RAR is received from serving cell for its minor impact on spec.

Proposal 4: RACH-less mechanism as in LTE can be supported by indicating TA=0 or the latest TA value of the source cell.

Proposal 5:  The following working assumption is confirmed

Working Assumption UE-based TA measurement (UE derives TA based on Rx timing difference between current serving cell and candidate cell as well as TA value for the current serving cell) is supported. - Corresponding UE capability is to be introduced to support UE-based TA measurement - For a UE reports support of this capability, configuration of UE-based TA measurement is supported FFS: other impacts on RAN1 spec

References

- RP-222332: Revised WID on Further NR mobility enhancements, MediaTek, RAN #97-e meeting, September 12th-16th, 2022.

- RAN1 # 112 meeting, Chairman notes.

- 3GPP TS38.213 v17.1.0.