3GPP TSG-RAN WG1 Meeting #112bis	R1-2302873

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.15.1

Source:	Ericsson, Qualcomm, Thales

Title:	Assumptions for the Self-Evaluation for the Satellite Component of IMT-2020

Document for:	Discussion

# 1	Introduction

RAN#99 agreed on a Study Item for a Self-Evaluation towards the 3GPP submission of a IMT-2020 Satellite Radio Interface Technology [1]. In this contribution we provide basic simulation assumptions to be used for the self-evaluation of peak data rate and peak spectral efficiency.

# 2	Simulation assumptions for peak data rate and peak spectral efficiency

Rel-17 defined two frequency bands for NR NTN, n256 (S-band) and n255 (L-band). Both of these bands are in FR1 and FDD, so there is no need to evaluate TDD or FR2 frequencies.

Table 1 shows simulation assumptions proposed for DL evaluations for the peak data rate and peak spectral efficiency.

Table 1: NR-NTN parameters for DL evaluations

| # Parameters | # Values | # Remarks |
| --- | --- | --- |
| Max. number of layers | 1 |  |
| Highest modulation order | 8 | 256QAM |
| Scaling factor of modulation | 1 |  |
| Max. coding rate Rmax | 948/1024 = 0.9258 |  |
|  | 0 | SCS = 2×15 kHz RAN4 UE UL Tx timing requirements allow only 15 kHz and 30 kHz SCS for NTN [2] |
|  | Based on ITU-R report M.2514, 30 MHz channel BW should be considered. Hence, 160 PRBs are assumed for the purpose of the evaluations |  |
| Overhead assumptions | - PDCCH: CORESET of 24 PRBs (4 CCE; AL 4) in every slot 12 RE/PRB/slot - TRS burst of 2 slots with periodicity of 80 ms and occupies 52 PRBs 12 RE/PRB/80 ms - DMRS: Type 1, Mapping A 12 RE/PRB/slot - CSI-RS: 1 CSI-RS ports with periodicity of 20 ms 1 RE/PRB/20 ms - 1 SS/PBCH blocks (SSB) per 20 ms; one SSB occupies 960 REs = 4 OFDM symbols × 20 PRB × 12 REs/PRB NOTE: If the channel bandwidth is less than TRS bandwidth, the TRS bandwidth is assumed to be equal to the channel bandwidth. |  |

Table 2 shows simulation assumptions for UL evaluations for the peak data rate and peak spectral efficiency.

Table 2: NR-NTN parameters for UL evaluations

| # Parameters | # Values | # Remarks |
| --- | --- | --- |
| Max. number of layers | 1 |  |
| Highest modulation order | 8 | 256QAM |
| Scaling factor of modulation | 1 |  |
| Max. coding rate Rmax | 948/1024 = 0.9258 |  |
|  | 0 | SCS = 2×15 kHz RAN4 UE UL Tx timing requirements allow only 15 kHz and 30 kHz SCS for NTN [2] |
|  | Based on ITU-R report M.2514 30 MHz channel BW should be considered. Hence, 160 PRBs are assumed for the purpose of the evaluations |  |
| Overhead assumptions | - PUCCH: short PUCCH with 1 PRB and 1 symbol in every UL slot 12 RE/slot - DMRS: Type 1, one complete symbol 12 RE/PRB/slot - SRS: 1 symbol with periodicity of 10 ms |  |

Further evaluation assumptions related to the satellite and UE configuration parameters can be found in the ITU-R Report M.2514, as well as TR 38.821 [3].

In order to move forward with the evaluations, the following is proposed:

The evaluation assumptions in the enclosed tables are endorsed.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	The evaluation assumptions in the enclosed tables are endorsed.

# References

RP-230736, New SID, “Study on Self-Evaluation towards the 3GPP submission of a IMT-2020 Satellite Radio Interface Technology”, RAN#99, March 2023.

TS 38.133 v18.0.0, “Requirements for support of radio resource management (Release 18)”, December 2022.

TR 38.821 v16.1.0, “Solutions for NR to support non-terrestrial networks (NTN) (Release 16)”, May 2021.