3GPP TSG RAN WG1 #112bis			R1-2302874

Online Meeting, 17th – 26th Apr 2023

Source:	Panasonic

Title: 	Discussion on Sidelink Positioning Reference Signal

Agenda Item:		9.5.1.1

Document for:	Discussion, Decision

# Introduction

This contribution discusses SL PRS for support of sidelink positioning. With using existing sequence DL-PRS as a starting point, this paper covers PRS operation scenarios, PRS sequence and other PRS parameters.

# Discussion

## PRS operation scenarios

Since the SL-PRS has two schemes, namely Scheme 1 (network centric) and Scheme 2 (UE autonomous) to be applied with or without network coverage, depending on how PRS related parameters are obtained and periodic/aperiodic transmission of PRS, we think the operation scenarios can be categorized as the following table. Whether all cases need to be supported or not may need some discussion.

Table 1. Different PRS Operation scenarios

| Case | PRS related parameters | Transmission of PRS |
| --- | --- | --- |
| Case 1 | Fully (Pre-)configured | Periodic transmission |
| Case 2 | Fully (Pre-)configured | Aperiodic PRS transmission |
| Case 3 | Scheme 1) obtained from gNB | Periodic transmission |
| Case 4 | Scheme 1) obtained from gNB | Aperiodic PRS transmission |
| Case 5 | Scheme 2) obtained from other UE | Periodic transmission |
| Case 6 | Scheme 2) obtained from other UE | Aperiodic PRS transmission |

Proposal 1: The operation scenarios can be categorized depending on "how PRS related parameters are obtained" and "periodic/aperiodic PRS transmission".

The WID requires to specify support for SL PRS bandwidths of up to 100 MHz in FR1 spectrum. We think a singular design regarding to the 100MHz is enough. For spectrum/BWP less than 100MHz, a truncated PRS resource set configuration would be sufficient.

Proposal 2: The design for SL-PRS should be unified and a truncated configuration would be used for spectrum less than 100MHz.

## PRS Sequence

The following were agreed in last RAN1#112 meeting.

Agreement SL PRS sequence is generated based on Gold sequence: rn=121-2⋅c2n+j121-2⋅c2n+1, where c(i) is a pseudo-random sequence as defined in Clause 5.2.1 of TS 38.211. Agreement - For SL PRS sequence generation, the pseudo-random sequence c(i) initialization equation is defined as a function of at least: slot number, symbol number, and a parameter nID,seqSL-PRS. - The pseudo-random sequence c(i) initialization equation is based on initialization equation as for DL PRS Agreement For SL PRS sequence generation, consider at least the following options to define the parameter nID,seqSL-PRS, and select one option: - Option 1: nID,seqSL-PRS is a higher layer configured parameter - Option 2: nID,seqSL-PRS is based on 12 bits CRC of PSCCH associated with the SL PRS transmission - Option 3: based on a combination of higher layer configured parameter from a configured ID list and 12 bits of CRC of PSCCH associated with the SL PRS transmission - Option 5: nID,seqSL-PRS is based on 12bits LSB of destination ID - Option 6: nID,seqSL-PRS is based on 8 bits of source ID + 4 zero bits - Option 7: nID,seqSL-PRS is based on the CRC field of the 2nd SCI associated with SL PRS transmission, if there is a 2nd SCI defined. Agreement Range of the parameter nID,seqSL-PRS is: nID,seqSL-PRS∈0,1,…,4095.

It has been agreed the reference signal sequence would be reusing DL-PRS Gold sequence, while the SL-PRS sequence ID nID,seqSL-PRS, 7 options have been listed for down-selection. In our understanding, The SL-PRS sequence ID should be different for SL UEs with Scheme 1 and Scheme 2 with separated range of values allocated. Among the listed options, we prefer either option 1 or 3 with higher layer configuration. Then by proper higher layer parameter configurations, different sequence ID values can be allocated for Scheme 1 and Scheme 2 at the physical layer.

Proposal 3: The SL-PRS sequence ID needs to be separately allocated for Scheme 1 and Scheme 2

Proposal 4: nID,seqSL-PRS is either a higher layer configured parameter or based on a combination of higher layer configured parameter from a configured ID list and 12 bits of CRC of PSCCH associated with the SL PRS transmission

## Other PRS Parameters

The following were agreed in last RAN1#112 meeting.

Agreement For RE-offset sequence for SL PRS, the RE-offset sequences specified for DL PRS are considered as a starting point. - FFS: Exact RE-offset sequences Agreement - Comb-based multiplexing of SL PRS from different UEs in a slot is supported at least for dedicated resource pools. - FFS: Comb-based multiplexing of SL PRS from different UEs in a slot for shared resource pools. - For comb-based multiplexing of SL PRS from different UEs, support at least the case wherein a single (M,N) value is possible . - FFS: Whether to support comb-based multiplexing of SL PRS from different UEs in a slot using multiple (M,N) values. FFS: additional restrictions (if any) due to e.g. the impact of synchronization and IBE interference between Agreement TDM-based multiplexing of SL PRS from different UEs in a slot is supported at least for dedicated resource pools. - FFS: TDM-based multiplexing of SL PRS from different UEs in a slot for shared resource pools. - FFS: Details, including resource granularity and relationship to SCI/PSCCH associated with the SL PRS resources, additional AGC symbols. - FFS: restrictions for the configuration of TDM-based multiplexing of SL PRS from different UEs in a slot, if any - FFS: which resource allocation schemes are applicable - FFS: whether or not this is a separate UE capability

Regarding to the multiplexing of SL-PRS from different UEs, we think proper multiplexing should be designed to avoid potential over-the-air collisions from different SL UEs. The approach could be from either frequency aspect or time aspect by using different frequency offset or time offset. The time offset or frequency offset can be either by gNB (pre-)configuration, from other SL UE’s signaling, or UE self-generation.

Proposal 5: Proper multiplexing with different frequency offset or time offset should be designed to avoid potential over-the-air collisions from different SL UEs

Proposal 6: The time offset, or frequency offset can be either by gNB (pre-)configuration, from other SL UE’s signaling, or UE self-generation.

Starting from referencing the DL-PRS, we think most of the parameters could re-use the legacy SL/DL designs. The following are some of the parameters may need to be further discussed for SL-PRS because of difference Scheme 1 and Scheme 2.

The assignment of Resource Element Offset, Resource Slot Offset, Resource Symbol Offset, etc. could be up to gNB (for scheme 1) and generated by UE itself (scheme 2) with pre-defined rules. For scheme 2, what kind of rules to be applied need to be further discussed with how the value is assigned to different UEs.

Proposal 7: The assignment of Resource Element Offset, Resource Slot Offset, Resource Symbol Offset, etc. could be up to gNB for scheme 1 and or UE self-generation for scheme 2 according to pre-defined rules.

For SL PRS periodicity, the periodicity could be just same as or multiple times of S-SSB for simplicity.  For SL PRS repetition, the concept can be similar to the LTE blind re-transmission, but with more repetitions. We may either to reuse the DL fixed time-gap concept for simplicity, or to design a one-to-one or one-to-many chain reservation by using SCI (for various time gaps between different transmissions). Periodic cases (PRS Operation scenarios case 1, 3, 5) only need the initial SCI or SCI carry single same value.  Aperiodic cases (PRS Operation scenarios case 2, 4, 6) may need specific designs.

Proposal 8: SL-PRS periodicity could be just same as or multiple times of S-SSB for simplicity.

Proposal 9: SL-PRS repetition (if >2) may be either to reuse the DL fixed time-gap concept for simplicity, or to design a one-to-one or one-to-many chain reservation by using SCI.

Sidelink needs to receive multiple UEs transmitter. Therefore, alignment for each is not required on AFC. For AGC, the sum of multiple UE's signal situation can be different for each of the transmission. Then the past setting of AGC cannot be reused even if the same Tx of PRS is intended to be received. Then QCL used for DL cannot be used. Similar to other SL channels, AGC symbol is necessary for SL-PRS.

Proposal 10: AGC symbol is necessary for SL-PRS.

# Conclusion

In this contribution, followings proposals are made:

Proposal 1: The operation scenarios can be categorized depending on "how PRS related parameters are obtained" and "periodic/aperiodic PRS transmission".

Proposal 2: The design for SL-PRS should be unified and a truncated configuration would be used for spectrum less than 100MHz.

Proposal 3: The SL-PRS sequence ID needs to be separately allocated for Scheme 1 and Scheme 2

Proposal 4: nID,seqSL-PRS is either a higher layer configured parameter or based on a combination of higher layer configured parameter from a configured ID list and 12 bits of CRC of PSCCH associated with the SL PRS transmission

Proposal 5: Proper multiplexing with different frequency offset or time offset should be designed to avoid potential over-the-air collisions from different SL UEs

Proposal 6: The time offset, or frequency offset can be either by gNB (pre-)configuration, from other SL UE’s signaling, or UE self-generation.

Proposal 7: The assignment of Resource Element Offset, Resource Slot Offset, Resource Symbol Offset, etc. could be up to gNB for scheme 1 and or UE self-generation for scheme 2 according to pre-defined rules.

Proposal 8: SL-PRS periodicity could be just same as or multiple times of S-SSB for simplicity.

Proposal 9: SL-PRS repetition (if >2) may be either to reuse the DL fixed time-gap concept for simplicity, or to design a one-to-one or one-to-many chain reservation by using SCI.

Proposal 10: AGC symbol is necessary for SL-PRS.

# Reference

[1] RP-230328, Revised WID on Expanded and Improved NR Positioning, Intel Corporation, RAN#99

[2] TR38.859: Study on expanded and improved NR positioning

[3] TR38.857: Study on NR positioning enhancements

[4] TR38.845: Study on NR positioning support

[5] Draft Report of 3GPP TSG RAN WG1 #109-e v0.3.0, Online meeting, 9th – 20th May 2022

[6] Final Report of 3GPP TSG RAN WG1 #110 v1.0.0, Toulouse, France, 22nd – 26th August 2022

[7] Final Report of 3GPP TSG RAN WG1 #110bis v1.0.0, Online meeting, 10th – 19th October 2022

[8] Final Report of 3GPP TSG RAN WG1 #111 v1.0.0, Toulouse, France, 14th – 18th November 2022

[9] Draft Report of 3GPP TSG RAN WG1 #112 v0.4.0, Athens, Greece, 27th Feb – 3rd Mar 2023