3GPP TSG RAN WG1 #112-bis-e		                                                       R1-2302876

e-Meeting, April 17th – April 26th, 2023

Agenda item:	9.17.15

Source: 	Vodafone, vivo

Title: 	Preliminary RAN1 UE features list for BWP Without Restriction for Rel-18 NR

Document for:	Information

- Introduction

This contribution includes preliminary RAN1 UE features for BWP Without Restriction for Rel-18 NR provided from WI rapporterur.

- NR_BWP_wor

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 53. NR_BWP_wor | 53-1 | Support RLM/BM/BFD measurements based on SSB outside active BWP without interruptions | 1. UE performs RLM/BM/BFD measurements based on SSB without interruptions, where the SSB is outside active DL BWP but is within the bandwidth of the corresponding carrier(s) to be measured. 2. The UE is not expected to receive PDSCH, PDCCH, or CSI-RS outside an active DL BWP. [3. Rules/conditions applicability and corresponding UE behavior is left to RAN4 to decide] | [6-1a] | Yes | n/a | UE cannot support RLM/BM/BFD measurements based on SSB outside active BWP without interruptions | [Per Band] | No | No | n/a | 53-1 is applicable to 6-1, 6-2, 6-3, or 6-4. [UE can at most indicate the support for one between FG 53-1 and FG 53-2.] | Optional with capability signalling |
| 53. NR_BWP_wor | 53-2 | Support RLM/BM/BFD measurements based on SSB outside active BWP with interruptions | 1. UE performs RLM/BM/BFD measurements based on SSB outside the active BWP with interruptions, where the SSB is outside active DL BWP but is within the bandwidth of the corresponding carrier(s) to be measured, only if there is no CSI-RS, no NCD-SSB, and no CD-SSB configured for RLM/BM/BFD in the active DL BWP of the corresponding carrier(s) to be measured. 2. The UE is not expected to receive PDSCH, PDCCH, or CSI-RS outside an active DL BWP. [3. The interruption time and corresponding UE behavior is left to RAN4 to decide] | [6-1a, 1-7, 2-24, 2-31, 53-3] | Yes | n/a | UE cannot support RLM/BM/BFD measurements based on SSB outside active BWP and meet interruptions requirements | Per Band | No | No | n/a | 53-2 is applicable to 6-1, 6-2, 6-3, or 6-4. This feature only applies if there is no CSI-RS, no NCD- SSB, and no CD-SSB configured for RLM/BM/BFD in the active BWP of the corresponding carrier(s) to be measured [UE can at most indicate the support for one between FG 53-1 and FG 53-2.] | Optional with capability signalling |
| 53. NR_BWP_wor | 53-3 | Support RLM/BM/BFD measurements based on NCD-SSB within active BWP | 1. UE performs RLM/BM/BFD measurements based on NCD-SSB, where the NCD-SSB is within the active DL BWP. 2. The UE is not required to perform RLM/BM/BFD measurements with CD-SSB if it is configured with NCD-SSB within the active DL BWP | [6-1a] | Yes | n/a | UE cannot support RLM/BM/BFD measurements based on NCD-SSB within active BWP | Per Band | No | No | n/a | 53-3 is applicable to 6-1, 6-2, 6-3, or 6-4 | Optional with capability signalling |