3GPP TSG RAN WG1 #112bis-e                                               R1-2302879

e-Meeting, April 17th – April 26th, 2023

Source: Google Inc.

Title:	On XR-specific capacity enhancements techniques

Document for: Discussion and Decision

# Introduction

A new Rel-18 Work Item for XR enhancements in NR has been approved in RAN#98-e [1]. Two main enhancements related to the system capacity are to be specified in RAN1 [1]:

-	Multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration

-	Dynamic indication of unused CG PUSCH occasion(s) based on UCI by the UE

In this Tdoc, we propose some techniques to specify the two enhancements above.

# Multiple CG PUSCH transmission occasions

The support of multiple PUSCH occasions in a CG period is beneficial to reduce latency for XR and improve performance.

## Time domain resource allocation of CG PUSCHs associated to a multi-PUSCHs CG

The following agreement has been made in RAN1#112 regarding the time domain resource allocation of CG PUSCHs associated to a multi-PUSCHs CG:

Agreement

For determination of the time domain resource allocation of CG PUSCHs associated to a multi-PUSCHs CG, the following alternatives for further study:

- Alt-A: TDRA determination based on repetition framework.

  - Alt-A1: Follow the time domain resource mapping of Type A repetition

    - N configured by higher layers or indicated by activation DCI

    - Single SLIV is determined from TDRA

    - The same SLIV in N PUSCH in consecutive slots per CG period

      - FFS for non-consecutive slots

    - FFS details, including related RRC parameters

  - Alt-A2: Follow the time domain resource mapping of Type B repetition

    - N configured by higher layers or indicated by activation DCI

    - Single SLIV is determined from TDRA

      - The SLIV used for 1st PUSCH per CG period.

    - N consecutive nominal PUSCHs with same duration per CG period

    - Note: N is not necessarily the repetition factor.

FFS details, including related RRC parameters

- Alt-B: TDRA determination based on NR-U framework

    - N and M configured by higher layers

    - Single SLIV is determined from TDRA.

      - The SLIV used for 1st PUSCH per CG period.

    - M consecutive PUSCH TOs with same duration in slot. The M PUSCH TOs are used in N consecutive slots per CG period

    - Note: N and M are configured independently from cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, respectively. M and N configuration is independent from cgRetransmissionTimer configuration.

    - FFS details, including related RRC parameters

- Alt-C: TDRA determination based on single DCI scheduling multiple PUSCHs

  - Alt-C1: Follow Rel-16 single DCI scheduling multiple PUSCHs

    - TDRA configured by pusch-TimeDomainAllocationListForMultiPUSCH-r16 with k2-r16

    - A row of TDRA with N entries determines the time domain resources allocation of N PUSCH TOs per period

      - Note: N PUSCH TOs should be consecutive PUSCH TOs in consecutive slots.

    - FFS details, including related RRC parameters

  - Alt-C2: Follow Rel-17 single DCI scheduling multiple PUSCHs

    - TDRA configured by pusch-TimeDomainAllocationListForMultiPUSCH-r16 with extendedK2-r17

    - A row of TDRA with N entries determines the time domain resources allocation of N PUSCH TOs per period

      - Note: N PUSCH TOs can be non-consecutive PUSCHs and/or in non-consecutive slots.

    - FFS details, including related RRC parameters

Supporting companies need to clarify how the single DCI scheduling multiple PUSCHs in Alt-C will be adapted for CG configuration with the multiple PUSCH occasions for CG type 1. It seems Alt-C could operate with CG type 2 as the UL DCI can indicate the row of the TDRA with the N entries. However for type-1, it seems the other alternatives are more adequate.

Rel-16 NR-U specification includes the use of CG with multiple CG PUSCH occasions within a period. The Rel-16 NR-U feature should be considered the basis for this XR enhancement in Rel-18.

This feature is already in the specification and we need to just remove the band restriction and introduce some minor modification mainly for the use of the retransmission timer.

The current parameters cg-nrofPUSCH-InSlot and cg-nrofSlots of the RRC parameter structure   ConfiguredGrantConfig defined in 3GPP TS 38.331 could be re-utilized to configure multiple PUSCH transmission occasions per CG period on licensed spectrum for XR, as these parameters are currently only supported for unlicensed spectrum operation when the RRC parameter cg-RetransmissionTimer is configured. Hence, the current restriction that the network can only configure these fields if cg-RetransmissionTimer is configured can be relaxed for the XR traffic.

As captured in TS 38.214 clause 6.1.2.3, the higher layer parameter cg-nrofSlots determines the number of consecutive slots allocated within a given CG period. The cg-nrofPUSCH-InSlot parameter configures the number of consecutive PUSCH allocations within each slot. The first allocation follows the higher layer parameter timeDomainAllocation for Type 1 PUSCH transmission or the higher layer configuration according to [10, TS 38.321], and UL grant received on the DCI for Type 2 PUSCH transmissions. Subsequent allocations have the same length, PUSCH mapping type and are appended without gaps. The same combination of start symbol, length and PUSCH mapping type repeats over the consecutively allocated slots.

The Rel-16 NR-U scheme described above can be extended to XR as is, but further improvements can be made to make it more suitable for the XR traffic.

- The TDRA determination based on NR-U framework should be extended to XR with some enhancements.

The specified NR-U scheme relies on a retransmission timer (cg-RetransmissionTimer) for autonomous retransmissions. If the UE does not receive an acknowledgment for the previous CG retransmission after this timer expires, it considers the transmission to have been unsuccessful and automatically initiates another transmission.

For XR and instead of the NR-U autonomous retransmission scheme (used mainly because the gNB can have difficulty acquiring a COT to send a DCI scheduling the retransmission), the configuredGrantTimer used with the CG legacy scheme could be applied for each HARQ process associated with each PUSCH occasion of the multiple CG PUSCH transmission occasions.  If the CG timer expires and the UE does not receive an UL DCI scheduling a retransmission then the UE can consider the corresponding HARQ transmission as successful and can consider the HARQ process released and could be used for another transmission.

- The legacy configuredGrantTimer can be used for each PUSCH occasion of the multiple CG PUSCH transmission occasions instead of the cg-RetransmissionTimer used in NR-U.

## HARQ Process ID determination

In RAN1#112 meeting, the following agreement has been reached:

Agreement

For determination of HARQ process IDs associated to PUSCHs in multi-PUSCHs CG assuming one TB per PUSCH, consider the following alternatives:

- Alt. 1:  The HARQ process ID for the first configured/valid PUSCH in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured, and applying "the period duration divided by X instead of the period duration.

  - The HARQ process ID of the remaining PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period.

  - Alt 1-1; X = 1

  - Alt 1-2: X is the number of configured PUSCHs in a period

  - Alt 1-3: X is provided by RRC configuration.

  - FFS details

- Alt. 2: Support that UE can decide, as in NR-U, the HARQ IDs for the multiple CG PUSCH transmission occasions and indicate the decided HARQ IDs to gNB if multiple HARQ processes are used for the multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration

  - FFS details

- Alt. 3: The HARQ process ID for the configured PUSCHs in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured.

  - FFS on potential enhancements different from previous alternatives

  - Alt 3-1: Note: Same HP ID would be used for all PUSCHs within a period.

    - FFS details

  - Alt 3-2: Note: Different HP ID could be used for all PUSCHs within a period.

    - FFS details

- Alt. 4:  The HARQ process ID for the first configured/valid PUSCH in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured.

  - The HARQ process ID of the remaining PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period

  - FFS on potential enhancements different from previous alternatives

- Alt 5: Support that UE can decide, as in NR-U, the HARQ IDs for the first CG PUSCH transmission occasions and indicate the decided HARQ IDs to gNB if multiple HARQ processes are used for the multiple CG PUSCH transmission occasions in a period of a single CG PUSCH configuration

  - The HARQ process ID of the remaining PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period

  - FFS details

- Alt 6: FFS other solutions

Note: The case of one TB map to multiple PUSCHs is not considered here.

We don’t support the UE deciding on the HPI for its UL transmission. That design was adopted in NR-U due to a specific need as the gNB may not be able to have a successful LBT and send a DCI with the HARQ process ID. Hence, Alt. 2 and Alt. 5 in the RAN1 agreement above should be precluded.

Alt. 3 and Alt. 4 are proposing to re-use the legacy CG procedure when cg-RetransmissionTimer is not configured. Alt. 4 is proposing to re-use the legacy formula to derive the HPI for the first PUSCH occasion in the CG period and then increment it for the remaining PUSCH occasions in the same period. However, it is not clear how Alt. 4 would operate for the following CG period as using the legacy formula means two consecutive periodicities would have consecutive HPIs and this means there will be overlapping between the HPIs used in the current CG period and the first CG occasion in the following CG period. An offset could be added, but the UE needs to keep incrementing the offset in each CG occasion which is not an optimum solution and not efficient from implementation perspective.

Alt. 3-1 is proposing to use the same HPI for all PUSCHS within a period. This is not optimum as it means if one of the PUSCH transmissions fails the gNB needs to schedule an UL grant to retransmit all the PUSCH occasions, unless another mechanism is introduced to identify which CG occasion has to be retransmitted, which would be over complicating the design.  Alt. 3-2 is proposing to use different HP IDs for all PUSCHS within a period. However, this is not clear using the legacy formula as two consecutive periodicities would have consecutive HPI.

We propose below to adjust the legacy CG formula to take the multiple PUSCH occasions in the CG into consideration.

HPI = [N *floor(CURRENT_symbol / periodicity)   + I ] modulo nrofHARQ-Processes

where the N defines the number of PUSCH occasions in the CG period and I is the index of the PUSCH occasion in the CG period. The index I can be in the range from 0 to N.

This is also equivalent to Alt-1 in the agreement above, as dividing the periodicity by the number of occasions is equivalent to multiplying this quantity floor(CURRENT_symbol / periodicity)  by the number of CG occasions.

- For the HPI determination, Alt-1 should be supported:

- The HARQ process ID for the first configured/valid PUSCH in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured, and applying "the period duration divided by X instead of the period duration.

- For the HPI determination, Alt. 2 and Alt. 5 where the UE is deciding on the HPI for its UL transmission should not be supported

To optimize PDCCH monitoring for retransmissions and reduce the PDCCH signalling overhead and reduce the UE monitoring effort, the UE could be instructed to monitor PDCCH after the transmissions of all PUSCH occasions in the CG period.  A single DCI scheduling multiple PUSCH retransmissions could be used to schedule all failed retransmissions at once.

- Single DCI scheduling multiple PUSCH occasions retransmission should be used to reduce PDCCH signalling overhead and the UE monitoring effort.

Alternatively, DCI format 0_1 scrambled with CS-RNTI to carry a HARQ-ACK bitmap for all HARQ processes transmitted in the CG period (and previous CG periods) could be used and the UE can send the retransmissions in the next CG periods subject to non-expired delay budget. Currently the DFI flag is one bit when cg-RetransmissionTimer is configured and it is set to ‘0’ when activating/releasing type 2 CG transmission and set to ‘1’ to be used as CG-DFI. The DFI flag should be re-utilized for the XR traffic even when cg-RetransmissionTimer is not configured. A new RRC parameter could be defined to activate the DFI flag for the XR traffic.

With the scheme described above, the DCI only reports the HARQ-ACK feedback for the CG-PUSCH occasions and the UE retransmits the failed transmissions using the allocated CG-PUSCH occasions eliminating the need for the dynamic scheduling of the retransmissions.

- DCI format 0_1 scrambled with CS-RNTI and with the DFI flag enabled and set to 1 can be used for the UL XR traffic to carry the HARQ-ACK bitmap for all HARQ processes transmitted in the CG periods.

The UE can also piggy-back a CG-UCI on the CG-PUSCH occasions to signal the associated HARQ Process Identifier and also set the NDI to the pre-defined value to indicate an initial transmission or a retransmission.

In TDD mode and if the existing parameters cg-nrofPUSCH-InSlot and cg-nrofSlots defined for Rel-16 NR-U are to be re-used to configure multiple PUSCH transmission occasions per CG period for the XR traffic on the licensed spectrum, then cg-nrofSlots could be specified to be counted only on UL slots and the counter is not incremented for DL slots. For example, if cg-nrofSlots is equal to 4 and the TDD pattern is DDDUU, then cg-nrofSlots is counted over two successive TDD patterns.

- The design of multiple CG-PUSCH transmission occasions should take into considertation the operation in TDD.

# Dynamic indication of unused CG PUSCH occasion(s)

Providing assistance information to the gNB is always useful. gNB doesn’t have much awareness about the UL AR traffic and further information from the UE about the need to cancel unused CG-PUSCH occasions can help optimize the gNB scheduling. When UL AR packets arrive at the UE modem, the UE can determine the number of the CG-PUSCH occasions required to transmit the packets and can signal this information to the gNB. This allows the network to free some CG-PUSCH occasions and recycle them to schedule other UEs which results in better system capacity and spectral efficiency.

This will help mitigate the limitation of using multiple CG-PUSCH transmission occasions in the CG period for the UL AR traffic. In fact, the resources semi-statically allocated for the UL AR data transmission may become over-allocated for some UL AR frames since the gNB does not receive an accurate BSR in time. This can decrease the overall capacity of the system. To mitigate this limitation, it was agreed in the SI that the UE should be able to dynamically inform the gNB via UCI (e.g., CG-UCI or a new UCI) of any unused PUSCH occasions and the gNB can reallocate the freed resources to other UE transmissions which is expected to increase the system capacity.

## Content of the UCI indicating CG PUSCH TOs as unused

In RAN1#112, the following agreement has been reached for the content of the UCI indicating the CG PUSCH TOs as unused:

Agreement

For dynamic indication of unused CG PUSCH transmission occasion(s) based on a UCI, the following options for further down-scoping, are considered for the information provided by the UCI:

- Option 1: The UCI determines the consecutive CG PUSCH TO(s) that are indicated as “unused”

  - Option 1-1: The UCI provides the number of consecutive TO(s) in time domain.

    - Applicable numbers can be determined from information obtained from configuration.

    - FFS details

  - Option 1-2: The UCI provides a time duration/range that includes the consecutive TO(s) in time domain.

    - Applicable time duration/range can be determined from information obtained from configuration

    - FFS details

- Option 2: The UCI determines the CG PUSCH TO(s) that are indicated as “unused” (consecutive/non-consecutive TO(s) in time domain)

  - Option 2-1: The UCI provides a bitmap where a bit corresponds to a TO within a time duration/range. The bit indicates whether the TO is “unused”.

    - Applicable time duration/range can be determined from information obtained from configuration

    - FFS details

  - Option 2-2: The UCI provides a bitmap where a bit corresponds to TOs within a time duration/range. The bit indicates whether all TOs within the time duration/range are “unused”.

    - Applicable time duration/range can be determined from information obtained from configuration

    - FFS details

- FFS whether/how the unused TO(s) can be associated to multiple CG configuration.

- Other options are not precluded. Proponent companies to provide details.

First, the need to cancel consecutive or non-consecutive CG PUSCH occasions should be discussed.

We assume that in a CG period, an entire Data Burst is transmitted by the UE. A Data Burst can contain multiple PDU Sets and each PDU Set will experience a different jitter. All PDUs of a PDU Set could arrive at the same time at the UE buffer but different PDU Sets could experience different delays and some of them could be impacted by a larger jitter. Hence, some PDU Sets could arrive late at the UE buffer and there could be gaps in the middle for the UL transmission hence the need to cancel some CG occasions in the middle.

It is important to discuss if all the PDU Sets belonging to the same Data Burst arrive at the same time at the UE buffer or if the arrival is staggered and do they all experience the same jitter? (if there is UL jitter). This should be clarified through discussion with SA2 and SA4.

The UE modem may or may not have awareness about the jitter experienced by all PDU Sets of the Data Burst at the start of the CG period. If the UE modem has awareness, then the use of the bitmap is beneficial, if there is no awareness the UE can still use a bitmap and overwrites it as soon as information about the arrival of the PDU Sets is available.

Therefore, we support Option 2-1 which consists of using a bitmap and each bit in the bitmap corresponds to a CG PUSCH TO within the time duration. This option offers good flexibility and more future proof. Option 2-2 could also be supported in the case of large number of CG occasions per CG period to reduce the signalling overhead.

- Support Option 2-1 using a bitmap where a bit corresponds to a TO within a time duration/range.

## UCI transmission occasions

In RAN1#112, the following agreement has been reached for location of the UCI transmission occasions:

Agreement

For dynamic indication of unused CG PUSCH occasion(s) based on a UCI, the following options for further down-scoping with possible revision, are considered for the transmission occasion of the UCI:

- Option 1: A transmitted CG PUSCH, includes the UCI.

  - FFS details

- Option 2: A transmitted CG PUSCH includes the UCI, if it is transmitted in an occasion determined by RRC.

  - FFS details

- Option 3: A transmitted CG PUSCH includes the UCI, if it is transmitted in a pre-defined transmission occasion.

  - FFS details

    - Example of a pre-determined occasion: 1st configured PUSCH TO in a CG period or 1st configured PUSCH TO in a multiple CG periods

- Option 4: A transmitted CG PUSCH includes the UCI, if it is transmitted in a transmission occasion determined satisfying given condition(s).

  - FFS details

    - Examples of a condition: A first transmitted PUSCH in a CG period, or a first PUSCH transmission within a multiple of CG periods.

Other options are not precluded. Proponent companies to provide details.

Option 4 is the most logical one as it allows the UE to send the information as soon as it is ready and with the first transmitted PUSCH occasion.

Option 4 is the most logical choice because it allows the UE to send the information as soon as it is ready and with the first transmitted PUSCH occasion. This means that the UE does not have to wait for a specific time to send the information, and it can send the information as soon as it is available. This can allow enough time for the gNB to recycle the unused CG PUSCH occasions. There was concern about blind decoding in Option 4 if the gNB misses the detection of the first CG PUSCH transmission and will try therefore to decode the following CG-PUSCH transmission with two hypotheses about the presence or not of the UCI. This can happen with low probability.

Compared to Option 2, 3 and 4, Option 1 offers better reliability because even if the gNB misses one of the CG PUSCH occasion it can still retrieve the UCI from other CG PUSCH occasions.

We therefore support the use of Option 1 to have better reliability. However, in order to reduce the signalling overhead and account for the gNB processing time, a time window could be introduced and configured by the gNB and any CG PUSCH TO within the configured time window carries the UCI indicating the unused CG PUSCH occasions.

- Support a modified version of Option 1:

- A transmitted CG PUSCH within a confiurable time window includes the UCI.

## UCI bit-fields

In RAN1#112, the following agreement has been reached for introducing the UCI bit-fields:

Agreement

Consider the following alternatives for “the UCI that provides information about unused CG PUSCH transmission occasions” for down-selection or revision

- Alt. 1: “The UCI that provides information about unused CG PUSCH transmission occasions” is defined as a new UCI.

  - FFS on details

- Alt. 2: “The UCI that provides information about unused CG PUSCH transmission occasions” is added as new field(s) to the CG-UCI.

  - FFS on details

- Alt. 3: “The UCI that provides information about unused CG PUSCH transmission occasions” replaces/re-purposes some field(s) of the CG-UCI.

  - FFS on details

The Rel-16 NR-U CG-UCI could be re-used to dynamically indicate the unused CG PUSCH occasions. New bit-fields could be introduced in the CG-UCI to indicate the PUSCH occasions that the UE is not planning to use.

- Support Alt. 2

- “The UCI that provides information about unused CG PUSCH transmission occasions” is added as new field(s) to the CG-UCI.

# Conclusion

In this contribution, we have presented several proposals to specify the support of multiple CG PUSCH transmission occasions in a CG period:

We made the following proposals:

- The TDRA determination based on NR-U framework should be extended to XR with some enhancements.

- The legacy configuredGrantTimer can be used for each PUSCH occasion of the multiple CG PUSCH transmission occasions instead of the cg-RetransmissionTimer used in NR-U.

- For the HPI determination, Alt-1 should be supported:

- The HARQ process ID for the first configured/valid PUSCH in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured, and applying "the period duration divided by X instead of the period duration.

- For the HPI determination, Alt. 2 and Alt. 5 where the UE is deciding on the HPI for its UL transmission should not be supported

- Single DCI scheduling multiple PUSCH occasions retransmission should be used to reduce PDCCH signalling overhead and the UE monitoring effort.

- DCI format 0_1 scrambled with CS-RNTI and with the DFI flag enabled and set to 1 can be used for the UL XR traffic to carry the HARQ-ACK bitmap for all HARQ processes transmitted in the CG periods.

- The design of multiple CG-PUSCH transmission occasions should take into considertation the operation in TDD.

- Support Option 2-1 using a bitmap where a bit corresponds to a TO within a time duration/range.

- Support a modified version of Option 1:

- A transmitted CG PUSCH within a confiurable time window includes the UCI.

- Support Alt. 2

- “The UCI that provides information about unused CG PUSCH transmission occasions” is added as new field(s) to the CG-UCI.

# References

- RP-223502, “New WID on XR Enhancements for NR” Nokia, Qualcomm, 3GPP TSG RAN Meeting #98-e, Dec. 2022