3GPP TSG-RAN WG1 Meeting #112bis-e	Tdoc R1-2302883

Online, April 17–26, 2023

Agenda Item:	9.2.3.2

Source:	Ericsson

Title:	Discussion on AI/ML for beam management

Document for:	Discussion, Decision

# Introduction

In this contribution, we discuss potential specification impacts of the beam management use case. General aspects on AI/ML functional frameworks, life cycle management (LCM), UE capabilities, and AI/ML model testing, which apply more widely than this use case, are discussed in [2]. Evaluation methodologies for the beam management use case are discussed in [3].

# Discussion

## Feasibility of TX/RX beam prediction

### NW-sided

Proposal 4.2.1: For DL beam pair prediction of BM-Case1 and BM-Case2 with a network-side AI/ML model, study the feasibility and potential spec impacts (if feasible) from the following aspects as a starting point - Whether/How to align the common understanding between NW and UE on the mapping between beam pairs and UE’s associated Rx beams - Whether/How to indicate a beam pair / Tx beam /Rx beam from NW to UE - whether/how Rx beam related information corresponding to a Tx beam reported from UE to NW - Note1: The potential down-selection/prioritization (if any) on the types of beam prediction is a separate discussion - Note2: The performance and spec impacts should be considered. Conclusion Regarding the explicit assistance information from UE to network for NW-side AI/ML model, RAN1 has no consensus to support the following information - UE location - UE moving direction - UE Rx beam shape/direction

Regarding TX/RX beam prediction at the NW-side, it should be highlighted that the NW cannot train a unique beam pair prediction model for each UE RX beam pattern. The gNB simultaneously serves many different UEs and needs to operate the same model for a potential beam pair prediction. Hence, the model needs to be able to generalize to a vast number of UE RX beam shapes and UE locations/orientations. Given the conclusion above regarding explicit assistance information from UE to network, it will be challenging for NW to create such model given the limited knowledge of the UE properties. Before any discussion on beam pair prediction specification impact, the study item should focus on addressing the feasibility given the new conclusion on UE assistance information from RAN1#112. Moreover, in addition to the generalization issue, the overhead for enabling beam pair prediction needs to be considered. Our updated proposal is the following,

For DL beam pair prediction of BM-Case1 and BM-Case2 with a network-side AI/ML model, study the feasibility from the following aspects as a starting point

  - How to generalize to different UE Rx beam shapes/directions

  - How to generalize to different UE orientation/location

  - Overhead in reporting TX/RX beam pairs in set B and potential assistance information

    - Note 1: Potential assistance information to achieve generalization should not disclose proprietary/privacy information.

    - Note 2: Companies are encouraged to provide evaluation results for the agenda item 9.2.3.1 addressing above aspects

### UE-sided

Proposal 4.3.1: For DL beam pair prediction of BM-Case1 and BM-Case2 with a UE-side AI/ML model, UE will report to network the information of predicted Tx beam(s), rather than the predicted beam pair(s), to facilitate the model inference - FFS: additional spec impact compared to DL beam prediction

Regarding UE TX/RX beam predictions, it is unclear why the UE would need to indicate any information of the RX beam information as part of the beam pair predictions. The NW cannot mandate which UE RX beam to use, hence it would be sufficient to investigate potential specification impact if the UE report the predicted Tx beam(s). Proponents of the usability of RX beam information should clarify what is the potential specification impact with a beam pair prediction report from the UE.

Conclude that the specification impact for DL beam pair prediction at UE sided model inference is same as for TX DL beam prediction

# Potential specification impacts

Our discussion around potential specification impacts focuses on the TX-beam prediction scenario.

## NW-sided data collection and training

Proposal 3.2.2: Regarding the training data collection mechanism for NW-side AI/ML model training trained at NW side, study necessity and beam-management-specific potential specification impact (if necessary) from the following additional aspects - Signaling type Mechanism of the reporting, e.g., RRC signaling, L1 signaling, user plane, control plane - Information of timestamp corresponding to the reported data samples, e.g., timestamps, [UE speed], SNR, etc. - Signaling and/or condition(s) to trigger/stop data logging (including buffering) and/or reporting - Signaling and/or condition(s) for trigger/stop reporting - Quantization of the measurement results (e.g., L1-RSRP) - Reporting overhead reduction - Note: non-3GPP based solution is a separate issue.

Proposal 3.2.1: Regarding the training data collection mechanism for NW-side AI/ML model training trained at NW side, study the following options as a starting point for the contents of collected data - Opt.1: UE reports sends M1 L1-RSRPs (corresponding to M1 beams) optionally with the indication of beams (beam pairs) indicators based on the measurement corresponding to a beam set (e.g., Set A, Set A+B, Set B), where M1 can be larger than 4 - FFS: the range of M1 - Opt.2: UE reports sends M2 L1-RSRPs (corresponding to M2 beams) optionally with the indication of beams (beam pairs) indicators based on the measurement corresponding to a beam set (e.g., Set B), reports sends M3 L1-RSRPs (corresponding to M3 beams) optionally with the indication of beams (beam pairs) indicators based on the measurement corresponding to another beam set (e.g., Set A), where M2 (M2>0) and M3 can be larger than 4 - FFS: the range of M2, M3 - Opt.3: UE r reports sends M4 L1-RSRPs (corresponding to M4 beams) optionally with the indication of beams (beam pairs) indicators based on the measurement corresponding to a beam set (e.g., Set B), reports sends M5 beams (beam pair) indicators based on the measurement corresponding to another beam set (e.g., Set A), where M4 can be larger than 4 - FFS: the range of M4, M5 - Other option(s) is not precluded - Note1: From UE perspective, the measurement and reporting related to one beam set may be separate from/transparent to the operations related to another beam set - Note2: Data collection for model training may be implemented by gNB in a transparent way - Note3: Potential down-selection/prioritization will be discussed later - Note4: Overhead, UE complexity and power consumption should be considered for the above options

Model training/retraining is a slow process, where data collection can be performed infrequently relative to L1 timescales when the NW is at low load. Therefore, latency requirement for NW to obtain training data can be relaxed as compared to model inference.

For NW-sided beam prediction, model training requires UE downlink L1-RSRP measurements. It is important to standardize appropriate data collection mechanisms for the UE’s measurements to be reported to the NW over the 3GPP air-interface. This is to ensure that the NW side can efficiently train models by collecting unified data format from different UE/chipset vendors, which is essential for reducing the complexity and data processing cost of the AI/ML model design. In addition, by standardizing data collection via UE report over the air interface, and the data quality can be tested and determined by the 3GPP standard. Moreover, the network may identify a need of collecting extra training data from a specific area or at a specific period of a day or week to improve its AI/ML model design, hence, a standard data collection procedure is required to support the network to collect the training data when needed. Data collection is also a key-aspect of the AI/ML life-cycle, hence it is important to address this stage for the completeness of the study item.

It is necessary to study the training data collection mechanism for NW-side AI/ML model for the completeness of the AI/ML beam prediction use case

Due to the decoupled nature of measurement time and reporting time, a UE can log/store its radio/non-radio measurements together with the meta information (e.g., time stamps, cell ID, and/or UE location) for multiple measurement occasions. The meta information can be seen as “tagging” the measurements with information on where/when the data was collected. After performing the radio measurements and logging the data, the UE can be triggered to report these accumulated data to the NW using an RRC message over the 3GPP air-interface for subsequent offline AI/ML model training at the NW side. This UE reporting over the 3GPP air-interface based training data collection for NW-sided beam prediction has standard impact and it should be studied.

Comparing to the layer-1 CSI reporting method, the RRC message-based data collection solution can enable the NW to collect large amount of measurement data from a UE with reduced signaling overhead and radio resource consumption. A high-level illustration of the data collection procedure is shown in Figure 1.

An RRC-message based approach is best suitable for training data collection for NW-sided beam prediction

The standardized data collection mechanism should enable UE logging and reporting of collected data to the NW

As discussed in our general aspects paper [3], data collection for model training should consider defining data types, data sizes, latency and periodicity, and configuration related requirements. In the following, we discuss these aspects for beam management use cases, and identify the potential standard impacts.

Figure 1 Illustration of training data collection for NW-sided spatial beam prediction use cases

Figure 2 Illustration of training data collection for NW-sided temporal beam prediction use cases

### Measurement occasion configuration

The data collection framework should support UEs collecting data from multiple measurement occasions, and then report the accumulated data to the NW. For the spatial beam prediction use case, a measurement occasion can consist of a single RS resource set, e.g., a set of CSI-RS/SSB resources that are corresponding to the union of Set A and Set B beams.

For the temporal beam prediction use case, a measurement occasion can consist of multiple sets of CSI-RS/SSB resources, with each resource set corresponding to the union of set A and set B beams transmitted at a given time instance. In addition, the time instances configured in a measurement occasion should cover both the observation time instances (related to model input, e.g., K time instances in T1) and the prediction time instances (related to model output, e.g., F time instances in T2).

The minimum measurement window required for a UE to complete one shot measurement of all SSB and CSI-RS beams within set A and set B depends on the DL-RS measurement resource configuration, the TDD DU/UL configuration, and the UE’s capability of performing beam management related measurements. For instance, consider an example where the union of set A and set B consists of 12 wide beams and 192 CSI-RS beams, the TDD configuration is DDDSU with SCS of 120KHz, and UE can measure up to 8 SSB&CSI-RS beams within a slot, the minimum measurement window for a UE to complete one shot measurement of all SSB&CSI-RS beams can be around 192/8/3*4/8 = 4 ms.

For training data collection for NW-sided beam prediction, a measurement occasion configuration needs to be designed to configure/indicate one or multiple sets of SSB/CSI-RS resources for L1-RSRP measurements, where a single set of SSB/CSI-RS resources is associated to a union of the Set A and Set B of beams, as examples shown in Figure 2 and Figure 3.

### Data content, types and data sizes

In general, the collected data for model training can include radio measurement data and non-radio-measurement data. For the beam prediction use cases, the radio measurement data can include L1-RSRP and/or CRI/SSBRI, and the non-radio measurement data can include for example cell-ID, time stamps or other information preserving the UE privacy/proprietary information. For both spatial and temporal beam prediction use cases, it is proposed that the UE logs all measurements performed on CSI-RS/SSB resources associated with set A and set B beams at each measurement occasion. The accumulated data of one or multiple measurement occasions are then reported from the UE to the NW.

The number of bits needed for a UE to report its L1-RSRP measurements depends on how many beams the UE is configured to measure and report. Based on the current standard, 7 or 4 bits are used for reporting a L1-RSRP value. Therefore, the payload size per UE report for the beam management use cases is not expected to be too large (e.g., around 771 bits for L1-RSRP measurements of 192 CSI-RS beams). Non-radio-measurement data may or may not need to be reported, depending on the type of information and whether UE mobility is supported in combination with data collection or not.

Data collection is seldom performed, and the payload size per UE report for the beam management use cases is not expected to be too large (e.g., around 771 bits for L1-RSRP measurements of 192 CSI-RS beams).

Given that the expected data size of the L1-RSRP measurement report is not too large, our proposed update to “Proposal 3.2.1” is the following:

Regarding the training data collection mechanism for NW-side AI/ML model trained at NW side, study the following options as a starting point for the contents of collected data.

  - Opt.1: UE reports M1 L1-RSRPs (corresponding to M1 beams) corresponding to a beam set (e.g., Set A, Set A+B, Set B), where M1 can be larger than 4. FFS: the range of M1

### Measurement occasion interval and logging duration

It is expected that a UE performs measurements at each configured measurement occasion and log the measurement data together with non-radio measurement data (if needed). In this case, the data logging interval is equivalent to the measurement occasion interval, and it is the periodicity for a UE to store measurement results and associated non-radio-measurement data. A UE should also be configured with a data collection duration during which the UE performs radio measurements and data logging.

### Data logging triggers/types

Different events and conditions can be used for triggering a UE to start performing radio measurements and data logging. For instance, a UE can start performing radio measurements and data logging after receiving a data collection related measurement configuration message from the NW. The data logging can also be event-based triggered, e.g., when a UE experience TCI state switches (TX beams switches) above a certain maximum number within a time window, when UE experiences beam failure.

### Data report triggers/types

Different events and conditions can be studied for triggering a UE to report its logged data for the beam management use cases. For instance, a UE can indicate its data availability and report the logged data at the end of the data collection period, or a UE can report its logged data after receiving a request from the NW. A UE can also be configured to report the logged data periodically. Or a UE can be configured to report the logged data when certain conditions are met, e.g., when detecting a beam failure.

### Other aspects - Enhanced mechanism to facilitate efficient data collection

In general, the following aspects are important when the investigating potential data collection, to address how one could:

- Reduce radio resource overhead (cost of over-the-air transmission, number of samples)

- Improve data quality (measurement noise, rare event, data validity, tighter requirements)

Reducing radio resource overhead for data collection can be achieved by means of minimizing the radio measurements and assistance information that needs to be collected for model training/inference/monitoring. For example, by only collecting the relevant samples, and configuring such data collection in certain favourable conditions. One example for collecting relevant data samples is to avoid reporting duplicated samples from the UE, a static UE might experience similar channel conditions over a certain time period and should in such scenario avoid reporting duplicated samples when collecting data for training an AI/ML model. The data quality could be improved via evaluating if for example measurement accuracies and/or the quantization granularity of the measurements can be enhanced, for example tighten the requirements on the L1-RSRP measurement accuracy, as shown in our evaluation paper [3].

### Updated proposal for NW-sided data collection

Given the discussion in this above section, we have the following proposal:

Regarding the training data collection mechanism for NW-side AI/ML model training trained at NW side, study beam-management-specific potential specification impact (if necessary) from the following additional aspects

  - Mechanism of the reporting, e.g., RRC signaling, L1 signaling

    - Measurement RS configuration

    - Signaling and/or condition(s) to trigger reporting logged data

  - Information corresponding to the reported data samples, e.g. non-radio information such as timestamps

  - Signaling and/or condition(s) to trigger/stop data logging (including buffering)

    - candidate values of data collection duration and measurement occasion intervals.

    - Event types to trigger data logging

  - Measurement accuracy requirements

  - Reporting overhead reduction

    - Quantization of the measurement results (e.g., L1-RSRP)

    - Other aspects (e.g. remove duplicated samples, how to collect relevant samples, …)

## UE-sided data collection and training

Agreement Regarding the data collection for AI/ML model training at UE side, study the potential specification impact considering the following additional aspects. - Whether and how to initiate data collection - Configurations, e.g., configuration related to set A and/or Set B, information on association/mapping of Set A and Set B - Assistance information from Network to UE (If supported) - Other aspect(s) is not precluded

Coverage and capacity enhancement operation (CCO) is part of the SON-framework devolved in 3GPP. It refers to the improvement of the coverage and signal quality in a geographical area of the network. Examples of such operations are directing the antennas towards a certain area e.g., via tilting the antennas or via beamforming, focusing the transmitted signal in a specific direction to improve the coverage and reduce the interference in the network. The use of CCO, potentially using reinforcement learning techniques, implies dynamic adaptation of the cell coverage and quality in a certain area. This would hence require a potential UE-sided beam prediction model to adapt to such scenarios.

### When to collect new data

Due to for example CCO, the UE-side might need to collect new data and adapt to new configurations. It is assumed that a new beam is associated with a new unique beam ID. The UE might need to collect new data when:

  - A new configuration/beam ID is present.

  - A configuration/beam ID used in the model is not present (outdated ID)

For a new beam ID, the UE needs to update its model to learn the relations with said beams. The model hence needs to be updated via new data collection, the UE could for example be configured with measuring the new beam ID in addition to the predicted top-K beams. When an old beam ID is not present, two situations can occur, namely when the outdated ID is part of Set A, and when part of set B. For set B, the UE needs to impute the missing beam information, for example using the training data it has already collected. For set A, the UE can use the 2nd strongest beams when the outdated beam ID is predicted to be strongest.

The UE can initiate data collection based on the received configuration/beam ID

### Configurations and assistance information

Currently, the QCL relations are used by the UE regarding how to relate different beams. It is assumed that such information is enough also for collecting measurements on set A and B as a starting point.

## New or enhanced mechanism(s) to facilitate AI/ML inference

In this section, we provide our views on AI/ML inference.

#### NW-sided model inference

Proposal 4.2.2: For BM-Case1 and BM-Case2 with a network-side AI/ML model, study the following additional aspects (including the necessity) to facilitate AI model inference: - Quantization of L1-RSRP measurement results - The default quantization scheme is NR existing quantization for L1-RSRP reporting if no consensus can be achieved on any other quantization scheme(s) - Beam indication of the unmeasured Tx beam(s) (or unmeasured beam pair(s)) from network to UE - Note: This may or may not have specification impact (e.g., legacy mechanism may be reused). - Beam indication of multiple future time instances for BM-Case2 - Report of temporal variance of L1-RSRP/L1-SINR measurements for beams - Note: at least the performance and spec impacts should be considered

Enhanced UE reporting

At RAN1#111, other L1 reporting enhancements in addition to UE reporting more than 4 beams were discussed. As shown in [3], the impact of the UE measurement accuracies severely impacts the prediction performance. If the UE can report more accurate measurements, it would also be possible of studying an increased resolution of the measurement report. Furthermore, the evaluations show how it is possible to utilize a UE reported measurement uncertainty to improve the training of the model.

For NW-sided AI/ML model, RAN4 could study the possibility for tightening requirements on the L1-RSRP measurement accuracies for beam prediction use cases

Regarding an enhanced report of temporal information to improve the predictions for BM-Case-2 at NW-side. One example is the temporal variance, another example is a polynomial approximation according to the figure below. UE can for example reports the polynomial representing the measured RSRP/RSRQ/SINR of the beams in set B for time window T1, this would enable the NW to get an understanding of the UE trajectory.

Figure 3 Example of UE reports a polynomial of the RSRP measurements over 30 CSI-RS/SSB time instances.

The SI evaluations mainly investigate models that uses UE beam measurements as input. However, with the increased capabilities in advanced todays UEs, external sensors could be used to detect presence of static or dynamic blockers (objects or humans). This assistance information could be used at the NW-side AI/ML inference stage, for example NW configures more beam measurements due to the uncertainty caused by the blocker.

For BM-Case1 and BM-Case2 with a network-side AI/ML model, study the following additional aspects (including the necessity) to facilitate AI model inference:

  - Quantization of L1-RSRP measurement results

  - For BM-case 2. Report of compressed value(s) based on temporal sequence of L1-RSRP (e.g. temporal variance or polynomial approximation of L1-RSRP/L1-SINR measurements for beams)

  - UEs to report the L1-RSRP measurement inaccuracy.

  - Assistance information (e.g. blockage probability)

- Note: at least the performance and spec impacts should be considered

- Note: Assistance information should preserve privacy/proprietary information

#### UE-sided model inference

RAN1#112 Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the necessity, feasibility and the potential specification impact (if needed) of the following information reported from UE to network: - Predicted L1-RSRP(s) corresponding to the DL Tx beam(s) or beam pair(s) - Whether/how to differentiate predicted L1-RSRP and measured L1-RSRP - Confidence/probability information related to the output of AI/ML model inference (e.g., predicted beams) - FFS: Definition/content of confidence/probability information - Note: At least the performance and spec impact should be considered

##### Confidence/uncertainty of prediction

Uncertainty is a key notion in AI/ML and uncertainty quantification is a key element for trustworthy and explainable AI/ML. Accuracy quantifies how close a prediction is to the true value and can only be measured once the true value is known. It is often determined as average for many predictions and used to evaluate and compare the predictive power of AI/ML algorithms. Uncertainty, on the other hand, assesses how much a prediction may differ from the true value and can be estimated along with the prediction. There are two inherently different sources of uncertainty, often referred to as aleatoric and epistemic uncertainty.

Epistemic uncertainty describes what the model does not know because training data was not appropriate. It is due to limited data and knowledge. Given enough training samples, epistemic uncertainty will decrease. Epistemic uncertainty can arise in areas where there are fewer samples for training.

Figure 4 Toy example of a one-dimensional model input x, indicating how capabilities in estimating the uncertainty for a given sample depends on the selected ML model

Aleatoric uncertainty is the uncertainty arising from the natural stochasticity of observations. It cannot be reduced even when more data is provided. Aleatoric uncertainty can further be categorized into homoscedastic uncertainty, uncertainty which stays constant for different inputs, and heteroscedastic uncertainty. Heteroscedastic uncertainty depends on the inputs to the model, with some inputs potentially having more noisy outputs than others.

When training an ML model, the selected algorithm will affect the level of uncertainty that can be estimated. In general, a more complex model (e.g. Gaussian process) can estimate the uncertainty more accurately than a simpler model. For example, Figure 4 illustrates the 95% prediction intervals for two models obtained by different algorithms. The first model is derived from a Gaussian process, which can learn the heteroscedastic noise process comprised in the training data. Thus, the size of the prediction interval for this model depends on the input to the model.  Figure 4 shows that the Gaussian process estimates a higher (epistemic) uncertainty for 2≤x≤4, due to the lack of training samples, and a higher (aleatoric) uncertainty for 6≤x≤8, due to the higher noise level. By contrast, the second model is obtained by linear regression with polynomial features, which is a much simpler method. In this case, the size of the prediction interval is constant for all inputs, so it does not depend on the input. Therefore, the uncertainty estimation is much less accurate, since it cannot capture the heteroscedastic noise process in a meaningful way.

Some models might be capable to report a per-input uncertainty, for example model 1 in Figure 4. Another method to estimate a per-input uncertainty is outlined in our evaluation paper [3], where a k-nearest neighbor method is used to mainly estimate the epistemic uncertainty. Further it is shown how such per-input uncertainty can be used to optimize the number of Top-K beams to transmit. Note that other models/methods might only support reporting a constant confidence value (model 2 in Figure 4) for all input samples, which is estimated from the training step. Such models/methods could include the uncertainty in the UE capability report.

Depending on the AI/ML model, confidence can be estimated:

  - During inference, confidence is dependent on the input.

  - During training, confidence is constant for all inputs during inference.

For the input-dependent confidence reporting during UE-sided AI/ML inference, study feasibility and specification impact for the following alternatives:

  - Probability/likeliness of strongest beam for each Top-K beam

  - Confidence interval (e.g. 95th percentile) for L1-RSRP prediction for a predicted beam

For constant confidence reporting for all input (based on the training step) for UE-sided AI/ML inference, study the feasibility and specification impact for the following alternatives:

  - Strongest beam prediction (log-loss, accuracy,
 L1 RSRP error in e.g. 50th and 95th percentile to the genie aided beam)

  - L1-RSRP prediction (e.g. L1-RSRP error in e.g. 50th and 95th percentile for the predicted Top-1/K beam)

##### Enhanced CSI resource/report configuration

When performing a beam switch for a UE, in the current NR specification, an indication of a new QCL relation needs to be signalled from the NW to the UE. The new QCL relation becomes active after a fixed offset time, where the offset time depends on the method and the condition. A fixed offset might lead to signaling congestion, especially when many UEs need to switch beam at the same time. For BM case-2, a flexible TCI switch time offset (compared to fixed timing rules in current spec) indicating the prediction of beam switch to the UE by utilizing AI based BM algorithm. Another aspect is to report an indication of several consecutive TCI is possible. If the predicted best beams are not measured/reported by the UE, the CSI-RS resources associated to the best beams can have unknown TCI states. How to support beam indication for a UE to switch to a predicted beam with unknown TCI state needs to be studied.

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study potential specification impact of AI model inference from the following additional aspects on top of previous agreements

  - Enhanced CSI resource/report configuration, e.g. how to adapt the TCI switch time offsets or configure several TCIs in one configuration.

  - Beam indication for a UE to switch to a predicted beam with unknown TCI state.

##### Assistance information

Proposal 6.3: In order to facilitate AI/ML operations for BM-Case1 and BM-Case2 with UE-side AI model, study the necessity and the potential specification (if needed) of the following aspect on data collection for training, inference and/or monitoring - Explicit or implicit indication information from NW to UE or from UE to NW to ensure the common understanding between NW and UE on the applicable scenarios/configurations

It is expected that AI/ML based beam predictions should be designed using a data driven approach, that is, rather than asking for very detailed beam shape information, an AI/ML model for downlink beam prediction should use the collected radio measurement data from different gNB TX/RX beams.

One potential issue with a data-driven approach to learning the gNB TX/RX beam correlations/properties is that different sites/cells may have different antenna/beam configurations. Moreover, even within the same cell, there can be scenarios where antenna/beam configurations are semi-dynamically adjusted to better fit to the current traffic load situations. To enable a trained AI/ML model at the UE-side to generalize to many different scenarios and/or antenna/beam configurations, a NW antenna/beam configuration ID can be used for the AI/ML model design. For example, a NW antenna/beam configuration ID can represent a specific NW antenna/beam configuration for training data collection. Such configuration ID can be used to provide UE with a consistency in NW behaviour that are valid for a longer period (days/weeks/months), than the consistency assumed for existing CSI-RS IDs for example.

In order to facilitate AI/ML operations for BM-Case1 and BM-Case2 with UE-side AI model, study the necessity and the potential specification (if needed) of the following aspect on data collection for training, inference and/or monitoring:

  - Scenario identification from NW to UE (e.g. antenna/beam configuration IDs)

A configuration ID can be used as a general identifier for the UE scenario. The NW can further indicate the beam IDs used for transmitting the set B of beams. One problem is the signalling overhead for deployments with many unique beams. For example, if the standard would like to capture/support 1024 unique beam IDs, this would require standardizing a 10-bit beam ID field. The number of beam IDs will grow rapidly in scenarios where the NW allocates a new ID for every SSB/CSI-RS beamforming weight update – leading to unnecessarily large control overhead and difficulties for UE-sided data collection and AI/ML model training/retraining. It is important to consider solutions to reduce the potential overhead in signalling beam IDs from NW to UE.

The number of unique beam IDs should be restricted to minimize the signalling overhead of beam IDs

## New or enhanced mechanism(s) to facilitate AI/ML model monitoring

In this section, we provide an initial discussion and examples of the agreed discussion points related to AI/ML model monitoring.

#### Performance metric(s) of model monitoring for AI-BM use cases

In the RAN1 #110bits meeting, the following agreements were made on model monitoring in the “general aspects of AI/ML framework” agenda item for AI PHY:

Agreement Study at least the following metrics/methods for AI/ML model monitoring in lifecycle management per use case: - Monitoring based on inference accuracy, including metrics related to intermediate KPIs - Monitoring based on system performance, including metrics related to system peformance KPIs - Other monitoring solutions, at least following 2 options. - Monitoring based on data distribution - Input-based: e.g., Monitoring the validity of the AI/ML input, e.g., out-of-distribution detection, drift detection of input data, or something simple like checking SNR, delay spread, etc. - Output-based: e.g., drift detection of output data - Monitoring based on applicable condition Note: Model monitoring metric calculation may be done at NW or UE Agreement Study performance monitoring approaches, considering the following model monitoring KPIs as general guidance - Accuracy and relevance (i.e., how well does the given monitoring metric/methods reflect the model and system performance) - Overhead (e.g., signaling overhead associated with model monitoring) - Complexity (e.g., computation and memory cost for model monitoring) - Latency (i.e., timeliness of monitoring result, from model failure to action, given the purpose of model monitoring) - FFS: Power consumption - Other KPIs are not precluded. Note: Relevant KPIs may vary across different model monitoring approaches. FFS: Discussion of KPIs for other LCM procedures

In the RAN1 #112 meeting, the following agreement was made on the performance metric(s) of AI/ML model monitoring in the “Other aspects on AI/ML for beam management” agenda item:

Agreement Regarding the performance metric(s) of AI/ML model monitoring for BM-Case1 and BM-Case2, study the following alternatives (including feasibility/necessity) with potential down-selection: - Alt.1: Beam prediction accuracy related KPIs, e.g., Top-K/1 beam prediction accuracy - Alt.2: Link quality related KPIs, e.g., throughput, L1-RSRP, L1-SINR, hypothetical BLER - Alt.3: Performance metric based on input/output data distribution of AI/ML - Alt.4: The L1-RSRP difference evaluated by comparing measured RSRP and predicted RSRP - Other alternatives are not precluded - Note: At least the performance and spec impact should be considered

For the beam management use cases, the performance metric(s) for AI/ML model monitoring can be categorized into three types, i.e., inference accuracy based, system/link performance metrics based, and data distribution based. In Table 1, we provide examples and compare these different performance metrics-based model monitoring methods.

Table 1 Summary of different performance metrics-based methods for AI/ML BM model monitoring

| Performance metric | Examples | Required data samples for to derive the performance metric | Benefits | Challenges |
| --- | --- | --- | --- | --- |
| Inference Accuracy (Intermediate KPIs) (Alt.1, Alt. 4) | Beam prediction accuracy related KPIs, e.g., Top-K/1 beam prediction accuracy | The measured L1-RSRP values of the predicted top-K/1 beam(s) + the ground truth (i.e., the measured RSRP value of the best beam in set A) | Metric reflects the model performance very well Expected to provide reliable model failure detection | Signalling overhead for collecting ground truth data at UE/NW (RS transmission and/or UE reporting) Frequent monitoring degrades the usability of the model. May not reflect the system performance very well (e.g., a higher prediction accuracy does not necessarily mean a better system KPI) |
|  | The L1-RSRP difference evaluated by comparing measured RSRP and predicted RSRP | The predicted L1-RSRP values of the predicted top-K/1 beam(s) + the ground truth (i.e., the measured L1-RSRP values of these beams) |  |  |
| System/Link performance metric(s) (Alt.2) | Throughput | Throughput values using AI/ML model. Reference throughput values for a non-AI/ML solution. | Metric reflects the system performance Low complexity and signalling overhead | Challenging to identify that the degradation is due to an inaccurate model (inaccurate model monitoring) |
|  | Number of beam failures or/and beam switches within a time window | Number of beam failure instances, number of triggered beam failure recovery procedures, number of beam switches |  |  |
|  | BLER for a hypothetical PDCCH transmission | SSB/CSI-RS beam(s) measurements (used for calculated the BLER of a hypothetical PDCCH transmission at the PHY layer of UE, measurements can be performed on serving beams or configured/dedicated beams); a target BLER (configured by e.g., the network) + hypothetical PDCCH configuration (configured by e.g., the network, or defined in spec) |  |  |
| Data distribution (Alt.3) | Input/output data distribution of AI/ML | The input data can be the measured RSRP values for Set B, and the output data can be the AI/ML model output. | No additional signalling overhead for obtaining input/output data Shorter latency for obtaining data samples for model monitoring Frequent monitoring possible | May not reflect model performance as well as Alt.1 May not reflect system performance as well as Alt. 2 To achieve reliable model failure detection, many samples may be required to calculate statistical metrics. This may lead to - Potential high complexity (computation and memory cost) - Potential long monitoring window, hence, increased latency from model failure occurs to detecting the failure |

#### Monitoring of NW-sided models

The following agreements are made regarding monitoring of NW-side models for beam management use cases:

RAN1 #110 bits Agreement For BM-Case1 and BM-Case2 with a network-side AI/ML model, study the NW-side model monitoring: - NW monitors the performance metric(s) and makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation Agreement Regarding NW-side model monitoring for a network-side AI/ML model of BM-Case1 and BM-Case2, study the potential specification impacts from the following aspects - Beam measurement and report for model monitoring - Note: This may or may not have specification impact. RAN1 #111 Agreement Regarding NW-side model monitoring for a network-side AI/ML model of BM-Case1 and BM-Case2, study the necessity and the potential specification impacts from the following aspects: - UE reporting of beam measurement(s) based on a set of beams indicated by gNB - Signaling, e.g., RRC-based, L1-based - Note: Performance and UE complexity, power consumption should be considered

For NW-sided AI/ML models, it has been agreed that the model performance monitoring is performed at the NW side. Based on the monitoring results, the NW node decides whether to take model LCM actions, e.g., fallback to a default feature/model, deactivate this model, switching to another reporting configuration, etc.

The network can monitor system/link level performance (e.g., throughput, number of beam failures, number of beam switches) of the users whose transmissions/receptions are scheduled/configured based on the NW-sided beam prediction model output. By comparing the system/link level performance statistics to a target system KPI(s), the NW can detect or predict potential system/link performance degradation, thereby, triggering model LCM actions like model fallback and perform error cause analysis if needed. The NW can also detect or predict potential model failure by comparing the system/link level performance statistics of users scheduled using the AI/ML model with the performance statistics of users scheduled using the legacy non-AL/ML scheme. System/link level performance metrics based model monitoring method has low complexity and it does not require additional signalling overhead for monitoring data collection. This approach can be considered as the first step of model monitoring, which can be sufficient for MBB services that do not have stringent latency and reliability requirements.

System/link level performance metrics based model monitoring method has low complexity and low signalling overhead. It can be sufficient for the NW to monitoring the AI-feature performance of users with MBB services if fall back operations are supported.

When detecting/predicting a potential system/link performance degradation, the NW can perform further error cause analysis by e.g., request UE to include other measurements or transmit reference signals so that the NW can use these measurements/signals to estimate the UE’s radio link condition and determine if the error was introduced due to bad wireless link qualities. The network can also trigger inference-accuracy-based model monitoring to check if the NW-sided model is functioning properly, or trigger data distribution based model monitoring to check if there is data drift.

Monitoring of NW-sided models can be done in a step-wise approach by considering different performance metric(s) and associated performance monitoring related KPIs (e.g., latency, complexity, signaling overhead, etc.).

Inference-accuracy-based model monitoring methods require the NW to collect ground truth (i.e., the best beam in set A) so that the NW can calculate the beam prediction accuracy or/and the RSRP difference by comparing predicted values with the corresponding measurement values. Hence, standard should support UE reporting beam measurements for Inference-accuracy-based model monitoring at the NW side. For example, for spatial beam prediction use cases, the data collection for model monitoring should support the UE to report measurements for Set A and Set B of beams. For temporal beam prediction use cases, the data collection for model monitoring should support the UE to report measurements for Set A and Set B of beams for multiple time instances, including both the time instances in the observation window (T1) but also the time instances in the prediction window (T2). To collect sufficient monitoring data for obtaining the inference accuracy metric statistics for model monitoring, the data collection framework discussed in Section 3.1 for model training can be reused here for the purpose of model monitoring. A time window for monitoring data collection needs to be configured, and the length of the time window depends on how many monitoring data samples are needed to enable a reliable model monitoring result. To reduce the monitoring data collection overhead, it is expected that the inference-accuracy based model monitoring should be performed either periodically with a large periodicity, or it can be event triggered, e.g., when predicting/detecting a potential system performance drop. Both L1 fast CSI reporting based and L3 RRC reporting based monitoring data collection methods can be considered.

Study both an RRC-message based and L1 fast CSI reporting-based data collection methods for the NW to monitor NW-sided models, including

- periodic UE reporting and event-triggered UE reporting configuration

  - SSB/CSI-RS resource configuration

  - Monitoring window configuration

Model input/output data distribution-based monitoring method is another method that NW can use to monitor its beam prediction models. As shown in our evaluation paper [3], data-distribution-based method can be used to detect the appearance of UEs with propagation conditions not seen during model training (data drift detection), and an alarm can be sound when the fraction of such UEs increases too much. The NW should collect monitoring data samples (model input/output data) from multiple users to obtain sufficient monitoring data statistics. The conditions and the measurable KPIs to represent input/out data distribution for sounding a model failure alarm should be carefully designed by taking different factors like user distribution, number of active users in the network, computation complexity and memory cost into account, to achieve a good trade-off between model failure detection reliability/accuracy (low false alarm rate and missed detection rate) and latency. The design of input/output data distribution-based model monitoring method for the NW to monitor its model is up to network implementation.

Model input/output data distribution based monitoring methods should be based on data statistics of sufficient input/output data samples collected from the system.

#### Monitoring of UE-sided models

RAN1 #110 bits Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the following alternatives for model monitoring with potential down-selection: - Atl1. UE-side Model monitoring - UE monitors the performance metric(s) - UE makes decision(s) of model selection/activation/ deactivation/switching/fallback operation - Atl2. NW-side Model monitoring - NW monitors the performance metric(s) - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation - Alt3. Hybrid model monitoring - UE monitors the performance metric(s) - NW makes decision(s) of model selection/activation/ deactivation/switching/ fallback operation RAN1 #112 Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding NW-side performance monitoring, study the following aspects as a starting point including the study of necessity: - Configuration/Signaling from gNB to UE for measurement and/or reporting - UE reporting to NW (e.g., for the calculation of performance metric) - Indication from NW for UE to do LCM operations - Other aspect(s) is not precluded - Note1: At least the performance and reporting overhead of model monitoring mechanism should be considered Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding UE-side performance monitoring, study the following aspects as a starting point including the study of necessity and feasibility: - Indication/request/report from UE to gNB for performance monitoring - Note: The indictation/request/report may be not needed in some case(s) - Configuration/Signaling from gNB to UE for performance monitoring - Other aspect(s) is not precluded

For UE-sided AI/ML model for the beam prediction use cases, it is the responsibility of the UE that the model is working correctly. Hence, model performance monitoring is expected to be mainly done at the UE-side. A UE can monitor its data distribution and system/link level performance (e.g., BLER of a hypothetical PDCCH transmission, number of beam failures, number of beam switches) of its own beam prediction feature without any request from the NW. By comparing the system/link level performance statistics to a target system KPI(s), the UE can detect or predict potential system/link performance degradation, thereby, triggering model LCM actions. Mechanisms for supporting UE to perform model fallback based on pre-defined/pre-configured conditions and UE behaviours, UE requesting NW assistance with its model fallback could be studied.

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, conclude that UE is responsible to make sure the model is working correctly.

Study mechanisms for the NW to configure a UE operating an AI/ML based beam prediction model to fallback to a legacy non-ML based beam reporting.

# Conclusions

In the previous sections we made the following observations:

Observation 1	It is necessary to study the training data collection mechanism for NW-side AI/ML model for the completeness of the AI/ML beam prediction use case

Observation 2	An RRC-message based approach is best suitable for training data collection for NW-sided beam prediction

Observation 3	The standardized data collection mechanism should enable UE logging and reporting of collected data to the NW

Observation 4	For training data collection for NW-sided beam prediction, a measurement occasion configuration needs to be designed to configure/indicate one or multiple sets of SSB/CSI-RS resources for L1-RSRP measurements, where a single set of SSB/CSI-RS resources is associated to a union of the Set A and Set B of beams, as examples shown in Figure 2 and Figure 3.

Observation 5	Data collection is seldom performed, and the payload size per UE report for the beam management use cases is not expected to be too large (e.g., around 771 bits for L1-RSRP measurements of 192 CSI-RS beams).

Observation 6	For NW-sided AI/ML model, RAN4 could study the possibility for tightening requirements on the L1-RSRP measurement accuracies for beam prediction use cases

Observation 7	Depending on the AI/ML model, confidence can be estimated:

a.	During inference, confidence is dependent on the input.

b.	During training, confidence is constant for all inputs during inference.

Observation 8	The number of unique beam IDs should be restricted to minimize the signalling overhead of beam IDs

Observation 9	System/link level performance metrics based model monitoring method has low complexity and low signalling overhead. It can be sufficient for the NW to monitoring the AI-feature performance of users with MBB services if fall back operations are supported.

Observation 10	Monitoring of NW-sided models can be done in a step-wise approach by considering different performance metric(s) and associated performance monitoring related KPIs (e.g., latency, complexity, signaling overhead, etc.).

Observation 11	Model input/output data distribution based monitoring methods should be based on data statistics of sufficient input/output data samples collected from the system.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For DL beam pair prediction of BM-Case1 and BM-Case2 with a network-side AI/ML model, study the feasibility from the following aspects as a starting point

o	How to generalize to different UE Rx beam shapes/directions

o	How to generalize to different UE orientation/location

o	Overhead in reporting TX/RX beam pairs in set B and potential assistance information

	Note 1: Potential assistance information to achieve generalization should not disclose proprietary/privacy information.

	Note 2: Companies are encouraged to provide evaluation results for the agenda item 9.2.3.1 addressing above aspects

Proposal 2	Conclude that the specification impact for DL beam pair prediction at UE sided model inference is same as for TX DL beam prediction

Proposal 3	Regarding the training data collection mechanism for NW-side AI/ML model trained at NW side, study the following options as a starting point for the contents of collected data.

	Opt.1: UE reports M1 L1-RSRPs (corresponding to M1 beams) corresponding to a beam set (e.g., Set A, Set A+B, Set B), where M1 can be larger than 4. FFS: the range of M1

Proposal 4	Regarding the training data collection mechanism for NW-side AI/ML model training trained at NW side, study beam-management-specific potential specification impact (if necessary) from the following additional aspects

o	Mechanism of the reporting, e.g., RRC signaling, L1 signaling

	Measurement RS configuration

	Signaling and/or condition(s) to trigger reporting logged data

o	Information corresponding to the reported data samples, e.g. non-radio information such as timestamps

o	Signaling and/or condition(s) to trigger/stop data logging (including buffering)

	candidate values of data collection duration and measurement occasion intervals.

	Event types to trigger data logging

o	Measurement accuracy requirements

o	Reporting overhead reduction

	Quantization of the measurement results (e.g., L1-RSRP)

	Other aspects (e.g. remove duplicated samples, how to collect relevant samples, …)

Proposal 5	The UE can initiate data collection based on the received configuration/beam ID

Proposal 6	For BM-Case1 and BM-Case2 with a network-side AI/ML model, study the following additional aspects (including the necessity) to facilitate AI model inference:

	Quantization of L1-RSRP measurement results

	For BM-case 2. Report of compressed value(s) based on temporal sequence of L1-RSRP (e.g. temporal variance or polynomial approximation of L1-RSRP/L1-SINR measurements for beams)

	UEs to report the L1-RSRP measurement inaccuracy.

	Assistance information (e.g. blockage probability)

Note: at least the performance and spec impacts should be considered

Note: Assistance information should preserve privacy/proprietary information

Proposal 7	For the input-dependent confidence reporting during UE-sided AI/ML inference, study feasibility and specification impact for the following alternatives:

a.	Probability/likeliness of strongest beam for each Top-K beam

b.	Confidence interval (e.g. 95th percentile) for L1-RSRP prediction for a predicted beam

Proposal 8	For constant confidence reporting for all input (based on the training step) for UE-sided AI/ML inference, study the feasibility and specification impact for the following alternatives:

a.	Strongest beam prediction (log-loss, accuracy,  L1 RSRP error in e.g. 50th and 95th percentile to the genie aided beam)

b.	L1-RSRP prediction (e.g. L1-RSRP error in e.g. 50th and 95th percentile for the predicted Top-1/K beam)

Proposal 9	For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study potential specification impact of AI model inference from the following additional aspects on top of previous agreements

a.	Enhanced CSI resource/report configuration, e.g. how to adapt the TCI switch time offsets or configure several TCIs in one configuration.

b.	Beam indication for a UE to switch to a predicted beam with unknown TCI state.

Proposal 10	In order to facilitate AI/ML operations for BM-Case1 and BM-Case2 with UE-side AI model, study the necessity and the potential specification (if needed) of the following aspect on data collection for training, inference and/or monitoring:

	Scenario identification from NW to UE (e.g. antenna/beam configuration IDs)

Proposal 11	Study both an RRC-message based and L1 fast CSI reporting-based data collection methods for the NW to monitor NW-sided models, including

	periodic UE reporting and event-triggered UE reporting configuration

o	SSB/CSI-RS resource configuration

o	Monitoring window configuration

Proposal 12	For BM-Case1 and BM-Case2 with a UE-side AI/ML model, conclude that UE is responsible to make sure the model is working correctly.

Proposal 13	Study mechanisms for the NW to configure a UE operating an AI/ML based beam prediction model to fallback to a legacy non-ML based beam reporting.

# References

RP-213599, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, 3GPP TSG RAN Meeting #94e, Electronic Meeting, Dec. 6 – 17, 2021.

R1-2300178, Discussion on general aspects of AI-ML framework, Ericsson, 3GPP TSG-RAN WG1 Meeting 112, February 27th - March 3rd, 2023.

R1-2302878, Evaluations of AIML for beam management, Ericsson, 3GPP TSG-RAN WG1 Meeting 112-bis-e, April 17–26, 2023 .

# Agreements RAN1#112 (2023/03)

Conclusion For the sub use case BM-Case1 and BM-Case2, “Alt.2: DL Rx beam prediction” is deprioritized. Agreement Regarding the performance metric(s) of AI/ML model monitoring for BM-Case1 and BM-Case2, study the following alternatives (including feasibility/necessity) with potential down-selection: - Alt.1: Beam prediction accuracy related KPIs, e.g., Top-K/1 beam prediction accuracy - Alt.2: Link quality related KPIs, e.g., throughput, L1-RSRP, L1-SINR, hypothetical BLER - Alt.3: Performance metric based on input/output data distribution of AI/ML - Alt.4: The L1-RSRP difference evaluated by comparing measured RSRP and predicted RSRP - Other alternatives are not precluded - Note: At least the performance and spec impact should be considered Conclusion Regarding the explicit assistance information from UE to network for NW-side AI/ML model, RAN1 has no consensus to support the following information - UE location - UE moving direction - UE Rx beam shape/direction Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study the necessity, feasibility and the potential specification impact (if needed) of the following information reported from UE to network: - Predicted L1-RSRP(s) corresponding to the DL Tx beam(s) or beam pair(s) - Whether/how to differentiate predicted L1-RSRP and measured L1-RSRP - Confidence/probability information related to the output of AI/ML model inference (e.g., predicted beams) - FFS: Definition/content of confidence/probability information - Note: At least the performance and spec impact should be considered Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, study potential specification impact of AI model inference from the following additional aspects on top of previous agreements: - Indication of the associated Set A from network to UE, e.g., association/mapping of beams within Set A and beams within Set B if applicable - Beam indication from network for UE reception - Note: The second bullet may or may not have additional specification impact (e.g., legacy mechanism may be reused). Conclusion Regarding the explicit assistance information from network to UE for UE-side AI/ML model, RAN1 has no consensus to support the following information - NW-side beam shape information - E.g., 3dB beamwidth, beam boresight directions, beam shape, Tx beam angle, etc. - Note: Other information (e.g., relative information) of Tx beam(s) preserving sensitive proprietary information is a separate discussion - e.g., some information following the same principle of Rel-17 positioning agreement Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding NW-side performance monitoring, study the following aspects as a starting point including the study of necessity: - Configuration/Signaling from gNB to UE for measurement and/or reporting - UE reporting to NW (e.g., for the calculation of performance metric) - Indication from NW for UE to do LCM operations - Other aspect(s) is not precluded - Note1: At least the performance and reporting overhead of model monitoring mechanism should be considered Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model, regarding UE-side performance monitoring, study the following aspects as a starting point including the study of necessity and feasibility: - Indication/request/report from UE to gNB for performance monitoring - Note: The indictation/request/report may be not needed in some case(s) - Configuration/Signaling from gNB to UE for performance monitoring - Other aspect(s) is not precluded